3GPP TSG RAN WG1 Meeting #98  	R1-1908017

Prague, Czech Republic, 26th – 30th August, 2019

Agenda Item: 7.2.5.4

Source: MediaTek Inc.

Title: Link budget of NR NTN

Document for: Discussion and Decision

# Introduction

A RAN3-led Study Item on Rel-16 enhancements for NR-NTN was approved at RAN Plenary #80 [1]. The study item phase has identified solutions for the considered NR-NTN deployment scenarios [2, 3]. Solutions in the satellite and the UE will require adequate link budget for evaluation of NR NTN.

This contribution aims to discuss link budget for satellite deployment.

# NR NTN Link budget C/N

According to RAN1#97 agreed TP [4], the link budget for on access link can be calculated as follows:

CNR dB=EIRP dBW+GTdBK-k dBWKHz-PLFS dB-PLA dB-PLS dB-PLAD dB-B dBHZ

Where EIRP is effective isotropic radiated power (EIRP), G⁄T is antenna-gain-to-noise-temperature, k is Boltzmann constant and equals to -228.6 dBW/K/Hz,  〖PL〗_FS is free space path loss, 〖PL〗_A is atmospheric path loss due to gases and rain fades, 〖PL〗_S is shadowing margin, 〖PL〗_AD is additional loss, B is bandwidth.

Antenna-gain-to-noise-temperature GT can be derived by [2]

|  | GTdB=GR dBi-Nf [dB]-10log10T0 [K]+Ta K-T0 [K]10-0.1Nf [dB] |  |
| --- | --- | --- |

where GR is receive antenna gain,  Nf is noise figure, T0 is ambient temperature and Ta is antenna temperature. EIRP figure and G/T figure from TP 802.11 are used in the calculation for the link budget as summarized in Table 1-3 for satellite deployments

- LEO @ 600 km, S band and Ka band

- LEO @ 1200 km, S band and Ka band

- GEO @ 35876 km, S band and Ka band

We assumed 30MHz (S band), 400MHz (Ka band) for the system bandwidth. The C/N and corresponding data rates / spectrum efficiency achievable with these parameters are shown in the tables. We used MCS Vs C/N tables from IMT2020 evaluation. We assume reuse factor 3 in the calculation. For UL in S band (handheld UE), we used channel bandwidth of 1 MHz per UE, and provided the aggregated data rates over 10 MHz corresponding to 10 UEs transmitting on UL simultaneously (i.e. system bandwidth/ frequency reuse factor). For other satellite scenarios, the channel bandwidth is set to system bandwidth/ frequency reuse factor [4].

| Link parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Satellite altitude | LEO=600 km |  |  |  |  |
| Elevation angle | 30 degrees |  |  |  |  |
| Satellite-UE distance | 1075.1 km |  |  |  |  |
| DL/UL | Downlink |  | Uplink |  |  |
| Frequency band | S | Ka | S | Ka |  |
| Carrier frequency (GHz) | 2 | 20 | 2 | 30 |  |
| Channel Bandwidth (MHz) | 10 | 133 | 1 | 133 |  |
| Number of RBs | 55 | 740 | 5 | 740 |  |
| SCS (kHz) | 15 | 15 | 15 | 15 |  |
| Losses |  |  |  |  |  |
| Free space path loss (dB) | 159.1 | 179.1 | 159.1 |  | 182.6 |
| Atmospheric loss (dB) | 0.1 | 0.1 | 0.1 |  | 0.1 |
| Polarisation loss (dB) | 3 | 0 | 3 |  | 0 |
| Shadowing margin (dB) | 3 | 0 | 3 |  | 0 |
| Satellite Parameters |  |  |  |  |  |
| EIRP Density (dBW/MHz) | 34 | 4 | - |  | - |
| EIRP per spot (dBW) | 43.95 | 25.24 | - |  | - |
| Noise figure (dB) | - | - | - |  | - |
| Satellite G/T (dB/K) | - | - | 1.1 |  | 13 |
| Terminal Parameters |  |  |  |  |  |
| Terminal type | 3GPP Class 3 |  |  |  |  |
| G/T (dB/K) | -31.62 | 13.87 | - |  | - |
| Antenna type | Omni (linear) | VSAT | Omni (linear) |  | VSAT |
| Receive antenna gain (dBi) | 0 | 39.7 | - |  | - |
| Noise figure (dB) | 7 | 1.2 | - |  | - |
| T0 (K) | 290 | 290 | - |  | - |
| Ta (K) | 290 | 290 | - |  | - |
| Transmit power (dBW) | - | - | -7 |  | 33 |
| Link Budget |  |  |  |  |  |
| C/N (dB) | 5.1 | 7.3 | -1.9 |  | 10.6 |
| HARQ BLER target | 10% | 10% | 10% |  | 10% |
| Spectral efficiency (bps/Hz) | 1.5 | 2.3 | 0.62 |  | 3.1 |
| Max data rates | 13.4 Mbps | 275.7 Mbps | 506 Kbps (5.1 Mbps in 10 MHz) |  | 374.0 Mbps |

Table 1 DL and UL Link budget for LEO=600 km

| Link parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Satellite altitude | LEO=1200 km |  |  |  |  |
| Elevation angle | 30 degrees |  |  |  |  |
| Satellite-UE distance | 1998.9 km |  |  |  |  |
| DL/UL | Downlink |  | Uplink |  |  |
| Frequency band | S | Ka | S | Ka |  |
| Carrier frequency (GHz) | 2 | 20 | 2 | 30 |  |
| Channel Bandwidth (MHz) | 10 | 133 | 1 | 133 |  |
| Number of RBs | 55 | 740 | 5 | 740 |  |
| SCS (kHz) | 15 | 15 | 15 | 15 |  |
| Losses |  |  |  |  |  |
| Free space path loss (dB) | 164.5 | 184.5 | 164.5 |  | 188.0 |
| Atmospheric loss (dB) | 0.1 | 0.1 | 0.1 |  | 0.1 |
| Polarisation loss (dB) | 3 | 0 | 3 |  | 0 |
| Shadowing margin (dB) | 3 | 0 | 3 |  | 0 |
| Satellite Parameters |  |  |  |  |  |
| EIRP Density (dBW/MHz) | 40 | 10 | - |  | - |
| EIRP per spot (dBW) | 49.95 | 31.24 | - |  | - |
| Noise figure (dB) | - | - | - |  | - |
| Satellite G/T (dB/K) | - | - | 1.1 |  | 13 |
| Terminal Parameters |  |  |  |  |  |
| Terminal type | 3GPP Class 3 |  |  |  |  |
| G/T (dB/K) | -31.62 | 13.87 | - |  | - |
| Antenna type | Omni (linear) | VSAT | Omni (linear) |  | VSAT |
| Receive antenna gain (dBi) | 0 | 39.7 | - |  | - |
| Noise figure (dB) | 7 | 1.2 | - |  | - |
| T0 (K) | 290 | 290 | - |  | - |
| Ta (K) | 290 | 290 | - |  | - |
| Transmit power (dBW) | - | - | -7 |  | 33 |
| Link Budget |  |  |  |  |  |
| C/N (dB) | 6.4 | 7.9 | -7.4 |  | -3.0 |
| HARQ BLER target | 10% | 10% | 10% |  | 10% |
| Spectral efficiency (bps/Hz) | 2.1 | 2.5 | 0.62 |  | 1.7 |
| Max data rates | 18.7 Mbps | 299.7 Mbps | 162.0 Kbps (1.6 Mbps in 10 MHz) |  | 201.0 Mbps |

Table 2 DL and UL Link budget for LEO=1200 km

| Link parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Satellite altitude | GEO=35786 km |  |  |  |  |
| Elevation angle | 10 degrees |  |  |  |  |
| Satellite-UE distance | 40581 km |  |  |  |  |
| DL/UL | Downlink |  | Uplink |  |  |
| Frequency band | S | Ka | S | Ka |  |
| Carrier frequency (GHz) | 2 | 20 | 2 | 30 |  |
| Channel Bandwidth (MHz) | 10 | 133 | 1 | 133 |  |
| Number of RBs | 55 | 740 | 5 | 740 |  |
| SCS (kHz) | 15 | 15 | 15 | 15 |  |
| Losses |  |  |  |  |  |
| Free space path loss (dB) | 190.6 | 210.6 | 190.6 |  | 214.2 |
| Atmospheric loss (dB) | 0.1 | 0.1 | 0.1 |  | 0.1 |
| Polarisation loss (dB) | 3 | 0 | 3 |  | 0 |
| Shadowing margin (dB) | 3 | 0 | 3 |  | 0 |
| Satellite Parameters |  |  |  |  |  |
| EIRP density (dBW/MHz) | 59 | 40 | - |  | - |
| EIRP per spot (dBW) | 68.95 | 61.24 | - |  | - |
| Noise figure (dB) | - | - | - |  | - |
| Satellite G/T (dB/K) | - | - | 19 |  | 28 |
| Terminal Parameters |  |  |  |  |  |
| Terminal type | 3GPP Class 3 |  |  |  |  |
| G/T (dB/K) | -31.62 | 13.87 | - |  | - |
| Antenna type | Omni (linear) | VSAT | Omni (linear) |  | VSAT |
| Receive antenna gain (dBi) | 0 | 39.7 | - |  | - |
| Noise figure (dB) | 7 | 1.2 | - |  | - |
| T0 (K) | 290 | 290 | - |  | - |
| Ta (K) | 290 | 290 | - |  | - |
| Transmit power (dBW) | - | - | -7 |  | 33 |
| Link Budget |  |  |  |  |  |
| C/N (dB) | -0.76 | 11.7 | -15.6 |  | -5.9 |
| HARQ BLER target | 10% | 10% | - |  | 10% |
| Spectral efficiency (bps/Hz) | 0.8 | 3.4 | - |  | 3.1 |
| Max data rates | 7.1 Mbps | 407.6 Mbps | - |  | 30.0 Mbps |

Table 3 DL and UL Link budget for GEO=35786 km

Observation 1: Link budget for LEO deployment in S and Ka bands with set 1 parameters is sufficient to meet NR-NTN data rate requirements.

Observation 2: Link budget for GEO deployment in S and Ka bands with set 1 parameters is sufficient to meet NR-NTN data rate requirements, except UL in S band where C/N is below -15 dB.

# Analysis

Performance aspects

The data rates achieved on the DL and UL are consistent with NR user experience data rate requirements of up to 2 Mbps on DL and up to 60 kbps on UL for extreme coverage. The user experienced data rate according to [5] is the minimum data rate required to achieve a sufficient quality experience. The usage scenarios with activity factor and user density are summarized in Appendix from Table B.2-1 in [3]. The maximum number of UEs per beam spot and UE density per km2 can be calculated as

max UEs per beam spot=maxdata rate in beam spotactivity factor × experience data rate

UE density per km2=maxUEs per beam spotπ×beam spot radius2

As an example, consider usage scenario pedestrian for extreme coverage in Appendix. The peak data rate is 2 Mbps (DL, and 60 kbps (UL). We use the activity factor defined in [5] as the percentage value of the amount of simultaneous active UEs to the total number of UEs. The activity factor is 1.5% for this scenario. As an example, consider set 1 parameters for UL in satellite deployment LEO=600 km band with beam spot diameter 50 km and carrier = 2 GHz in S band with frequency re-use 3 [2]. The maximum number of UEs per beam spot at elevation angle 30 degrees is 5622 (=5.06 Mbps / 0.015/60 Kbps). The UE density per km2 is 2.88 (=5622 / (π*252)). The UE densities achievable for the satellite deployments in set 1 for worst case elevation angle are given for DL and UL for frequency re-use 3 in Table 4.

Note: the link budget analysis does not take into account the inter-beam interference. Even assuming frequency re-use 3, the inter-beam interference will result in some loss of performance which needs further evaluation in system level simulator.

| Satellite Link |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Satellite altitude | LEO=600 km |  |  |  |  |
| DL/UL | Downlink |  | Uplink |  |  |
| Frequency band | S | Ka | S | Ka |  |
| Re-use factor | 3 | 3 | 3 |  | 3 |
| System bandwidth / re-use factor | 10 MHz | 133 MHz | 10 MHz |  | 133 MHz |
| Experience user data rate (eMBB pedestrian) | 2 Mbps | 2 Mbps | 60 Kbps |  | 60 Kbps |
| Activity factor | 1.5% | 1.5% | 1.5% |  | 1.5% |
| LEO=600 km, Elevation angle 30 degrees |  |  |  |  |  |
| Data rates per beam spot | 13.4 Mbps | 275.7 Mbps | 5.1 Mbps |  | 374.0 Mbps |
| Max UEs per beam spot | 891 | 9.19 103 | 5.66 103 |  | 415.5 103 |
| Beam spot diameter | 50 km | 20 km | 50 km |  | 20 km |
| UE Density per km2 | 0.45 | 29.25 | 2.88 |  | 1322.75 |
| LEO=1200 km, Elevation angle 30 degrees |  |  |  |  |  |
| Data rates per beam spot | 18.7 Mbps | 299.7 Mbps | 1.6 Mbps |  | 201.0 Mbps |
| Max UEs per beam spot | 623 | 9.99 103 | 1.77 103 |  | 223.3 103 |
| Beam spot diameter | 90 km | 40 km | 90 km |  | 40 km |
| UE Density per km2 | 0.10 | 7.94 | 0.28 |  | 177.7 |
| GEO=35786 km, Elevation angle 10 degrees |  |  |  |  |  |
| Data rates per beam spot | 7.1 Mbps | 407.6 Mbps | - |  | 30.0 Mbps |
| Max UEs per beam spot | 236 | 13.59 103 | - |  | 33.33 103 |
| Beam spot diameter | 250 km | 110 km | 250 km |  | 110 km |
| UE Density per km2 | 0.05 | 1.43 | - |  | 3.51 |

Table 4 UE density per km2 for LEO=600 km, 1200 km and GEO=35786 km

Proposal 1: Includes the formulas for max UEs per beam spot and UE density per km2 in TR 38.811

UL bandwidth aspects

The channel bandwidth may be challenge for the UL link budget [4]. The transmission power is 23 dBm for a handheld UE and 33 dBm for a VSAT UE [2]. Increasing the UL bandwidth increases the noise in gNB receiver, which degrades SNR. To maximize the aggregated UL data rate per beam spot, the gNB scheduler may schedule UEs with a channel bandwidth that is compromise between the per UE data rate and the resource efficiency for each UE transmission. It also depends on the per UE experienced user data rate requirement. For example, eMBB stationary experienced UL data rate requirement is 25 Mbps, which can be achieved with about 50 MHz channel bandwidth in GEO deployment in Ka band (where per UE data rate is 30.61 Mbps). In another example, the eMBB pedestrian experience UL data rate requirement is 60 kbps, which can be achieved with about 180 kbps channel bandwidth in LEO deployment in S band.

Table 5 shows per UE UL data rates and per UL beam data rates for various channel bandwidth in LEO and GEO deployments. A general trend is that smaller UL BW per UE improves C/N value in all scenarios. In case of LEO in S and Ka band, and GEO in Ka band, the smaller UL bandwidth per UE improves the per beam UL data rate. Each UE can transmit at full power in narrower bandwidth channel, which improves C/N and also improves spectral efficiency. However, for GEO in S band, a smaller UL bandwidth per UE cannot improve C/N sufficiently with C/N as low as -8.69 dB observed with minimum channel bandwidth of 180 kHz.

We assumed UE UL transmit power is 23 dBm (i.e. UE is Power Class 3 device). A lower MCS in UL can be considered to allow UE to transmit to gNB. For example, assuming a further 3 dB gain with more aggressive repetitions or slot aggregation the C/N could be in the order of -5.69 dB with a spectral efficiency or around 0.25 bps/Hz. This could allow an UL data rate of around 20 kbps. This is consistent with the usage scenario IoT connectivity for extreme coverage in Appendix. The peak data rate is 2 kbps (DL), and 10 kbps (UL), where IoT connectivity refers to low power wide area service capability.

| LEO=600 km, S band, UL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Channel bandwidth | C/N [dB] | S.E. [bps/Hz] | Per UE data rate | Per beam data rate |
| 180 kHz | 4.94 | 1.7 | 275 kbps | 13.75 Mbps |
| 1 MHz | -2.04 | 0.625 | 506 kbps | 5.06 Mbps |
| 10 MHz | -5.05 | 0.3 | 486 kbps | 2.43 Mbps |
| LEO=1200 km, S band, UL |  |  |  |  |
| Channel bandwidth | C/N [dB] | S.E. [bps/Hz] | Per UE data rate | Per beam data rate |
| 180 kHz | -0.43 | 0.8 | 129 kbps | 6.45 Mbps |
| 1 MHz | -7.42 | 0.2 | 162 kbps | 1.62 Mbps |
| 10 MHz | -10.43 | - | - | - |
| GEO=35786 km, S band, UL |  |  |  |  |
| Channel bandwidth | C/N [dB] | S.E. [bps/Hz] | Per UE data rate | Per beam data rate |
| 180 kHz | -8.69 | - | - | - |
| 1 MHz | -15.67 | - | - | - |
| GEO=35786 km, Ka band, UL |  |  |  |  |
| Channel bandwidth | C/N [dB] | S.E. [bps/Hz] | Per UE data rate | Per beam data rate |
| 10 MHz | 5.8 | 1.8 | 14.58 Mbps | 193.9 Mbps |
| 50 MHz | -1.52 | 0.7 | 30.61 Mbps | 81.4 Mbps |
| 133 MHz | -5.9 | 0.25 | 29.97 Mbps | 29.97 Mbps |

Table 5 Per UE / Per beam UL Data rates for LEO=600 km, 1200 km and GEO=35786 km

Observation 3: Smaller UL BW per UE improves C/N value in all scenarios.

Observation 4: In case of LEO in S and Ka band, and GEO in Ka band, the smaller UL bandwidth per UE improves the per beam UL data rate.

Proposal 2: Smaller UL bandwidth and lower MCS can be considered for use case IoT connectivity for low power wide area service capability at least for IoT UE in GEO in S band.

# Conclusion

In this contribution, we discussed link budget of NR-NTN. We made the following observations and proposals.

Observation 1: Link budget for LEO deployment in S and Ka bands with set 1 parameters is sufficient to meet NR-NTN data rate requirements.

Observation 2: Link budget for GEO deployment in S and Ka bands with set 1 parameters is sufficient to meet NR-NTN data rate requirements, except UL in S band where C/N is below -15 dB.

Proposal 1: Includes the formulas for max UEs per beam spot and UE density per km2 in TR 38.811

Observation 3: Smaller UL BW per UE improves C/N value in all scenarios.

Observation 4: In case of LEO in S and Ka band, and GEO in Ka band, the smaller UL bandwidth per UE improves the per beam UL data rate.

Proposal 2: Smaller UL bandwidth and lower MCS can be considered for use case IoT connectivity for low power wide area service capability at least for IoT UE in GEO in S band.

# References

- RP-181370, Thales,  Study on solutions evaluation for NR to support Non Terrestrial Network, SID, RAN#80, June 2018

- TR38.811 “Study on NR to support non-terrestrial networks”

- TR 38.821 “Solutions for NR to support non-terrestrial networks”

- R1-1907836, TP for Link Budget Analysis in TR 38.821, Nokia, Thales, MediaTek, Ericsson, ZTE, RAN1#97, May 2019

- 3GPP TS 22.261 “Service requirements for the 5G system; Stage 1 (Release 16)”

Appendix

Non-Terrestrial network target performances per usage scenarios [3]

| Usage scenarios | Experience data rate |  | Overall UE density per km2 | Activity factor | Max UE speed | Environment | UE categories |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | DL | UL |  |  |  |  |  |
| Pedestrian | 2 Mbps | 60 kbps | TBD | 1,50% | 3 km/h | Extreme coverage | Handheld |
| Vehicular connectivity | 50 Mbps | 25 Mbps | TBD | N.A. | 250 km/h | Along roads in low population density areas | Vehicular mounted |
| Stationary | 50 Mbps | 25 Mbps | TBD | N.A. | 0 km/h | Extreme coverage | Building mounted |
| Airplanes connectivity | 360 Mbps | 180 Mbps | TBD | N.A. | 1 000 km/h | Open area | Airplane mounted |
| IoT connectivity | 2 kbps | 10 kbps | 400 | 1,00% | 0 km/h | Extreme coverage | IoT |

Set-1 Satellite parameters for System Level Simulations [2]

Table X.1: Set-1 Satellite parameters for System Level Simulations

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35’786 km | 1’200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m |
| Satellite EIRP density |  | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| Satellite beam diameter (Note 2) |  | 250 km | 90 km | 50 km |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 5 m | 0.5 m | 0.5 m |
| Satellite EIRP density |  | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| Satellite beam diameter (Note 2) |  | 110 km | 40 km | 20 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | S-band (i.e. 2 GHz) | 22 m | 2 m | 2 m |
| G/T |  | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 5 m | 0.5 m | 0.5 m |
| G/T |  | 28 dB K-1 | 13 dB K-1 | 13 dB K-1 |
| Satellite RX max Gain |  | 62 dBi | 42 dBi | 42 dBi |
| Note 1: This value is equivalent to the antenna diameter to be used in Sec. 6.4.1 of TR 38.811. Note 2: This beam size refers to the Nadir pointing of the satellite |  |  |  |  |