3GPP TSG RAN WG1 Meeting #98	R1-1908060

Prague, Czech Republic, 26th – 30th August, 2019

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on propagation delay compensation for reference time information delivery

Document for:	Discussion and Decision

# Introduction

An LS on Time Sensitive Networking (TSN) requirements evaluation was received from RAN2#106 meeting [1] with the following action to RAN1:

ACTION: RAN2 respectfully asks RAN1 to answer the questions above and provide any further feedback with respect to propagation delay compensation for reference time information delivery, if any.

In TR 38.825[2], following is the related conclusions of the physical layer aspects of the achievable timing synchronization accuracy:

A timing synchronization error between a gNB and a UE no worse than 540ns is achievable based on the RAN1 agreed evaluation assumptions for Rel-15 NR with 15kHz SCS. It is RAN1´s conclusion, that the synchronization accuracy is improved when using higher SCS. For small service areas with dense small cell deployments a propagation delay compensation by the UE would not be required. The propagation delay compensation needs to be applied by the TSN UEs for larger service areas with more sparse cell deployments (e.g. for inter-site distances >200m the gNB-to-UE timing synchronization accuracy without propagation delay compensation may be worse than 1us).

Note that the RAN1 analysis does not contain the effects of the granularity & accuracy of the absolute timing indication information by the gNB, which are outside of the RAN1 study scope.

This contribution discusses the following questions from RAN2 on propagation delay compensation method:

Q1. What method did RAN1 assume for propagation delay compensation in their synchronization accuracy analysis in IIoT study (as per results captured in TR 38.825), e.g. was it Timing Advance based or based on another method?

Q2. Does RAN1 see the need for specifying any propagation delay compensation requirements or enhancements in order to meet the synchronization requirements as studied in NR Industrial IoT SI?

# Propagation delay compensation method

## Timing Advance based method

In the summary of TSN related evaluations [6], timing synchronization error results from different companies are listed. It can be seen that when evaluation the error with propagation delay compensation, error of indicating granularity of TA command or TAG is considered. So to answer Q1, timing Advance based method is applied for downlink propagation delay compensation.

Answer for Q1: synchronization accuracy analysis as captured in TR 38.825, was based on Timing Advance as the propagation delay compensation method.

In SA1 meeting #84, the requirements for clock synchronization service have been further clarified and updated as follows:

It can be seen that the requirement of 1μs accuracy exists both in the urban macro scenario (electrical power distribution) and indoor scenario (factory automation), which means that 1μs is the time synchronization requirement between different UEs in a cell. So the time error between a gNB and a single UE should be less than 500ns without considering the effects of the granularity and accuracy of the absolute timing indication information provided by the gNB, i.e. assuming perfect timing is sent by the gNB.

The results on maximum timing synchronization error with UE propagation delay compensation are shown in Table 6.3.2.3-2 in [5]. We can see that 2 out of 4 sources show that the requirement cannot be met by just applying the current TA based method.

Table 6.3.2.3-2: Summary of maximum timing synchronization error results 
with UE propagation delay compensation.

|  | 15kHz SCS | 30kHz SCS | 60kHz SCS |
| --- | --- | --- | --- |
| Source R1-1900156 | 488ns | 357.5ns | 276.5ns |
| Source R1-1901334 | 505ns | 371ns | 287.5ns |
| Source R1-1900935 | 472.5ns | 338.5ns |  |
| Source R1-1901252 | 536ns | 438ns | 357ns |

According to the analysis, the total error in TA estimation is composed of several parameters, including:

- BS detecting error (errorBS_detect)

BS decides the value of the TA for a certain UE by detecting the reference signal (e.g. SRS) sent by the UE, so the detecting error impacts the final accuracy of the time synchronization. Simulation results about the detecting performance of SRS in Rel-15 NR show that the detecting accuracy is within about +/- 130ns for 99% confidence with 20MHz bandwidth for TDL-C channel model (i.e. outdoor).

- TA indicating error (errorindicate)

The indicating granularity of TA command causes additional error, which can be as large as half of the indicating granularity. According to 38.213[4], the TA indicating granularity is 16∙64∙Tc/2μ, so the indicating error can be assumed as +/-8∙64∙Tc/2μ.

errorindicate∈-8∙64∙Tc2μ,8∙64∙Tc2μ=[-260ns,260ns] for 15kHz

- Downlink frame timing error (Te)

The downlink frame timing at the UE receiver represents the arrival time of the downlink signal, and is obtained via detecting the downlink signal of the reference cell. The downlink frame timing error also impacts the accuracy of the estimation for downlink propagation delay, because the estimated TA equals the correct TA plus Te without regard to other factors. As defined in TS 38.133 [3], Te has various values under different scenarios.  The values +/-12*64*Tc can be used as typical values for SCS 15kHz.

errorUE_timing∈-391ns,391ns for 15kHz

- Asymmetry between downlink and uplink channel (Asymmetry).

UE estimates the downlink propagation delay as half of the TA value obtained from gNB, which introduces error due to the asymmetry between downlink and uplink propagation delay. Simulation results show that the typical asymmetry between downlink and uplink channel multi-path distribution is about 160ns in outdoor in TDD system. In FDD systems, the situation would be worse since the downlink and uplink signal are transmitted at different carrier frequencies.

In general, the total error of TA estimation is calculated as:

errorTA=errorBS_detect+errorindicate+Te

Since the downlink propagation delay PDL is gotten from the following equation sets:

TA=PDL+PUL
Asymmetry=PDL-PUL

The downlink propagation delay PDL is calculated as:

PDL=(TA+Asymmetry)2.

The error of the downlink propagation delay PDL is:

errorPDLwith comp=Asymmetry+errorBS_detect+errorindicate+Te2

Besides the four above aspects, other factors such as BS timing error (errorBS_timing), UE timing error (errorUE_timing) also impacts the accuracy of the final error of the time synchronization between UE and gNB, the final error of the time synchronization between UE and gNB is calculated as follows:

TUE=TBS+PDL

TUE+Te=(TBS+errorBS_timing)+(PDL+errorPDL)

TUE=TBS+PDL+(errorBS_timing+errorPDL-Te)

TUE=TBS+PDL+(errorBStiming+Asymmetry+errorBSdetect+errorindicate2-Te2)

Form the above analysis, we can see that several parameters could affect the synchronization accuracy between gNB and UE. Therefore, several methods could be considered to improve the error of each parameters correspondingly. For example,

- Possible enhancement for more accurate TA. E.g. increasing the indicating granularity of TA command could decrease TA indicating error (errorindicate), or enable a mechanism for gNB to indicate the current TA adjustment to UE periodically;

- Possible enhancement for smaller Te. By increasing the bandwidth of synchronization signal or DL reference signal could improve downlink frame timing accuracy (Te);

- Possible enhancement for asymmetry between DL/UL propagation delays. E.g. In TDD system, we can shorten the time gap between downlink and uplink signal transmission to increase the correlation of downlink and uplink channel and decrease the asymmetry between downlink and uplink propagation delay.

These methods could be applied together to improve the synchronization accuracy between gNB and UE, only one method may not be enough to meet the synchronization accuracy requirement.

Answer for Q2: RAN1 see the need for specifying propagation delay compensation requirements or enhancements in order to meet the synchronization requirements as studied in NR Industrial IoT SI, for example, increasing the indicating granularity of TA command, considering the asymmetry between downlink and uplink channel.

# Conclusions

In this paper, the propagation delay compensation methods based on Timing Advance are discussed and the following answers are given:

Answer for Q1: synchronization accuracy analysis as captured in TR 38.825, was based on Timing Advance as the propagation delay compensation method.

Answer for Q2: RAN1 see the need for specifying propagation delay compensation requirements or enhancements in order to meet the synchronization requirements as studied in NR Industrial IoT SI, for example, increasing the indicating granularity of TA command, considering the asymmetry between downlink and uplink channel.

# References

R1-1907993, LS to RAN1 on propagation delay compensation for reference time information delivery, RAN2.

3GPP TR 38.825 v1.0.0, “Study on NR Industrial Internet of Things (IoT)”.

3GPP TS 38.133, “Requirements for support of radio resource management (Release 15)”.

3GPP TS 38.213, “Physical layer procedures for control”.

3GPP R1-1903437, "Text proposal for TSN requirements evaluation", Nokia, Nokia Shanghai Bell, Athens, Greece, 25th February – 1st March, 2019

3GPP R1-1901387, "Summary of evaluations on TSN reliability, latency and synchronization accuracy", Nokia, Nokia Shanghai Bell, Taipei, Taiwan, 21-25 January 2019