3GPP TSG RAN WG1 Meeting #98	R1-1908068

Prague, Czech Republic, August 26-30, 2019

Agenda Item:	7.2.8.4

Source:	Huawei, HiSilicon

Title:	Remaining issues on UL MIMO full power transmission with multiple PAs

Document for:	Discussion and Decision

Introduction

In last meetings, two modes have been supported as working assumption for both UE with capability 2 and 3 to support full power transmission [1].

Support following scheme for UL full power Tx for UE capability 2 and 3: - A UE can be configured for one of two modes of full power operation to support ‘Capability 2’ and ‘Capability 3’ subject to UE capability - A UE can be configured by the network to support full power transmission - Mode 1: The UE can be configured with one or more SRS resources with same number of SRS ports (according to Rel-15) within an SRS resource set which usage is set to ‘codebook’ - gNB can configure the UE to use a subset of TPMIs that combine ports in a layer to produce full power transmission. - A new codebookSubset is introduced only for the rank value(s) where full power transmission in UL is not achievable includes the TPMI precoders in fullyAndPartialAndNonCoherent defined in Rel-15 - FFS: At least a subset of the non-antenna selection TPMI precoder(s) is(are) supported - FFS: Additional support of antenna selection TPMI precoders - Note: as non-coherent UE, it is not capable of maintaining relative phase of antenna ports according to TPMI - Mode 2: The UE can be configured with one SRS resource or multiple SRS resources with different number of SRS ports within a SRS resource set which usage is set to ‘codebook’ - UE transmits SRS and PUSCH in same manner, whether antenna virtualization is used or not - Rel-15 codebooks and codebook subsets are used - Note: Antenna selection precoder can be used to enable full power related PA(s) to produce full power transmission for Capability-3 UE. - UL full power Tx is achieved for PUSCH transmission according to indicated SRI and/or TPMI - A set of TPMIs that deliver full power can be signalled by the UE in order to support at least UEcap3, for SRS resource with more than 1 ports, - e.g. For SRI indicating SRS resource with 1 port then single layer PUSCH is transmitted with full power in same manner as single port SRS, if SRI indicating SRS resource with multiple ports is signalled based on Rel-15 MIMO behaviour (transmission rank indicator, TPMI indicator, etc) except the power scaling aspects - The following cases are not precluded - For example, for 4TX on UE side (with 20+20+17+17dBm) virtualized as 2 SRS ports, full uplink power transmission can be enabled by precoder [1 0] or [0 1] - FFS: number of SRS resources supported - 2 - 3 - FFS: for 4 Tx, how many different TPMIs/TPMI groups support full power - FFS: any rules for spatial filter update for the SRS resources with different number ports Note: How to capture the behaviour for ‘Mode 1’ and ‘Mode 2’ in specifications is TBD Note: For single port, there is no SRI and TPMI - Note: Support of Mode 1, Mode 2 have separate UE capability

In addition, different UE capabilities to allow full power UL transmission was agreed in previous meetings as follows [2][3][4][5]:

Agreement Full TX power UL transmission with multiple power amplifier is supported at least for codebook based UL transmission for non-coherent and partial/non-coherent capable UEs - This specification support is a UE optional feature - FFS: Whether this applies for the entire codebook or subset of codebook Agreement Regardless of UE capability 1, 2, or 3, signalling of “UL full power tx capability” is supported for UEs with full power uplink transmission capability - FFS: For UE capability 1, if any other information is necessary - For UE capability 2 and UE capability 3, in addition to signalling “UL full power tx capability”, further information on UE capability are signalled if needed - FFS: Details such as support of UE capability signaling of supported one or group of TPMI precoder(s) for full power transmission, support different number of SRS ports for resources for codebook, and other UE capability signaling can be introduced - FFS: Whether full uplink TX power capability can be explicitly/implicitly derived from the TPMI/TPMI group precoders for full power transmission UEs with full power uplink transmissions are those Rel-16 UEs which can transmit at full power at least for rank1 The signalling of above information does not imply any specific UE PA architecture implementation.

In this contribution, we discuss the remaining issues for the full power transmission.

Full power transmission schemes

Remaining issues for Mode-1

Mode-1 uses a new codebook subset to enable full power transmission. In the following, we discuss which codebook subset should be introduced for Mode-1 for UE PA architecture 2 and 3.

For the case that full power transmission only achieved by using all of PAs, such as 2Tx with only 20dBm PAs, or 4Tx with only 17dBm PAs for power class 3, non-antenna selection precoder should be introduced for supporting full power transmission. For instance, non-antenna selection precoder 1211 can be used for 2Tx with 20dBm PAs to output 23dBm transmission power, and non-antenna selection precoder 121111 can be used for 4Tx with only 17dBm PAs.

However, there are some PA architectures that full power transmission can be achieved by part of PAs, such as 4Tx with 20+20+20+20dBm PAs, antenna selection precoders are sufficient for supporting full power transmission. For example, antenna selection precoder 121010 is used for 4Tx with 20+20+20+20dBm PAs for full power transmission. By the way, in such cases, the full power transmission with antenna selection precoders will be more efficient than non-antenna-selection precoder, since less number of PAs need to be activated for full power transmission in the antenna-selection precoder.

Observation 1: Non-antenna selection precoder can be used for the case of full power transmission achieved by only using all PAs, and antenna-selection precoder can be used for the case of full power transmission achieved by using part of PAs.

In the following, we discuss the exact precoders which should be added in the new subset.

If the UE is only with non-coherent precoding capability, the phase difference between two antennas in a precoder are no impact on the performance, since the phase cannot be guaranteed for the non-coherent UEs. For example, 1211, 121-1, 121j and 121-j are the same for a non-coherent 2Tx UE. So, the precoder 1211 is sufficient in the new codebook subset for full power transmission. Similarly, for 4Tx, there is no difference among the precoders 121010, 1210-10, 1210j01210-j0, so that 121010 is sufficient. However, the precoder 121010 and 120101 may be different since some antennas will be blocked in some cases. In addition, for 4Tx case, the non-antenna selection precoder 1211jj also should be supported for the case that full power achieved by all the PAs for a UE, where  1211jj is selected due to the precoder is included for both OFDM and DFT-s-OFDM. Otherwise, whole codebook is enlarged for OFDM case or DFT-s-OFDM case.

In Figure 1, we show that with antenna selection precoders brings performance benefits. The simulation assumes that UE is with 20+20+20+20 dBm PAs so that full power transmission can be achieved when using precoder 121010 and 120101. Other detailed simulation assumptions can be found in the Appendix B.

Figure 1. Performance of different new codebook subset of rank-1 transmission

For rank 2, it is not necessary to introduce such precoder 12111-1for 2Tx, since 121001 is already full power. For 4Tx, both the precoder 121001100-j and 12111111-1-1 support full power transmission for non-coherent UEs, but introducing 121001100-j is slightly better since the transmission robustness with wide beam width. In Figure 2, we provide a simulation result to show the performance difference between using these two precoders. We can see that precoder 121001100-j performs slightly better than precoder 12111111-1-1 due to that precoder 121001100-j can use a wider beam per rank which can be more robust especially for cell edge with low SINR.

Figure 2. Performance of different new codebook subset of rank-2 transmission

For rank 3, 12100100100001 is sufficient for the new codebook subset.

Based on the above discussion, the new codebook subset for non-coherent UE for full power transmission is summarized in Table 1, which depends on UE capability reporting (as discussed in Section 3).

Table 1 New codebook subset for 2Tx and 4Tx non-coherent UE for full power transmission

|  | Rank 1 | Rank 2 | Rank 3 |
| --- | --- | --- | --- |
| 2Tx Non-coherent UE | 1211 i.e., TPMI=2 | - | - |
| 4Tx Non-coherent UE | 121010, 120101, 1211jj i.e., TPMI=4,8,13 | 121001100-j i.e., TPMI=6 | 12100100100001 i.e., TPMI=1 |

For the UEs with partial coherent precoders, full power transmission already supported for rank>1 cases. In the following, we only discuss the rank 1 precoders in the new codebook subset. Different with non-coherent UE, the phase difference can be maintained between the coherent antennas (1st and 3rd antennas are coherent, and 2nd and 4th antennas are coherent according to the current spec), however cannot be guaranteed for the different groups, i.e., the phase varies between antenna group (1st, 3rd) and group (2nd, 4th). For example, among the precoders 121111, 121j1j, 121-11-1and 121-j1-j, the phase between the coherent antennas are the same (i.e., phase between 1st and 3rd antennas are same), so select one of them is sufficient. The detailed selection can be found in Appendix A, where the precoders with the same color means same phase between the coherent antennas.

The new codebook subset for partial-coherent UE for full power transmission is concluded in Table 2, which depends on UE capability reporting (as discussed in Section 3).

Table 2 New codebook subset for 4Tx partial-coherent UE for full power transmission

|  | Rank 1 |
| --- | --- |
| 4Tx Partial-coherent UE (OFDM) | 121111, 1211jj,1211-1-1, 1211-j-j, i.e., TPMI=12,13,14,15 |
| 4Tx Partial-coherent UE (DFT-s-OFDM) | 12111-1,1211jj, 1211-11,1211-j-j, 121j1j,121jj1, 121j-1-j,1211-j-1, i.e., TPMI=12,13,14,15,16,17,18,19 |

Proposal 1: For Mode-1, the new codebook subsets for full power transmission should include the precoders shown in Table 1 and Table 2 for Non-coherent and Partial coherent UEs, respectively.

Remaining issues for Model-2

Mode-2 enables full power transmission by using port virtualization or port selection.

If the UE have the PA with full power capability, the port selection precoder can be used for enabling full power transmission. For example, for 4Tx, 23dBm+17dBm+17dBm+17dBm, the precoder 121000 can be used for full power transmission. In such cases, one SRS resource is enough.

If there is no full power related PA, full power transmission can be through port virtualization. For example, for 4Tx, 20dBm+20dBm+20dBm+20dBm, the 4 ports can be virtualized as 2 ports: 23dBm+23dBm, then the precoder 1210 and 1201 can enable full power transmission. It is also possible that the 4 antennas are virtualized as only one SRS port for full power transmission. In this case, two or three SRS resources need to be transmitted from UE side. gNB indicates the precoder by SRI and TPMI to full power transmission up to rank 1 or rank 2 or rank 4 for the SRS resources. To give the flexibility for gNB scheduling, 3 resources should be supported for full power transmission.

Proposal 2: For full power transmission, 3 SRS resources should be supported.

For Mode-2, Rel-15 codebook is used. Which precoders used for full power transmission depends on the UE PA architecture. If the precoder is already support full power in Rel-15, full power transmission also can be enabled by those precoders in Rel-16. Thus, for Mode-2, we only need to discuss the precoders did not support full power transmission in Rel-15, but can support full power transmission in Rel-16.

For 2Tx, if the non-coherent UE is with PAs 23dBm+20dBm, then the precoder 1210 is enough for enable full power transmission. If the non-coherent PAs 20dBm+23dBm, the precoder  1201 is for enabling full power transmission. The third case, UE with 23dBm+23dBm, then both of 1210 and  1201 can enable full power transmission and no need to configure 1-port SRS resource for the above three cases. If no PAs is full power related, none of  1210 and 1201 can support full power transmission, the full power transmission need to be enabled through port virtualization via 1-port SRS resource. So, for 2Tx, two precoders (1210 and  1201) will be supported for full power transmission based on UE capability reporting.

For 4Tx, if non-coherent UE with a full power related PA, then at least one or multiple precoders 121000, 120100,120010, 120001 in rank 1 can support full power transmission. Such as 17dBm+23dBm+17dBm+17dBm, then 120100 enables full power transmission, and 23dBm+23dBm+17dBm+17dBm, both 121000 and 120100 can be used for full power transmission.

For rank 2, 1210010000 is for the cases such as UE is with 20dBm+20dBm+17dBm+17dBm or 23dBm+23dBm+17dBm+17dBm; 1210000010 is for cases such as 20dBm+17dBm+20dBm+17dBm, and 1210000001 is for the cases such as 20dBm+17dBm+17dBm+20dBm. In some cases, such as 20dBm+20dBm+20dBm+17dBm, multiple precoders such as 1210010000 and 1210000010 can be used.

For Rank 3, only one precoder 12100100000010 used for full power transmission for non-coherent UE. For rank 4, the current precoder can already support full power transmission.

So, for non-coherent UE with 4Tx, the following precoders can be used for supporting full power transmission, which depends on UE capability reporting as discussed in Section 3.

Table 3: 4Tx codebook subset can be for full power transmission for non-coherent UE

|  | Rank 1 | Rank 2 | Rank 3 |
| --- | --- | --- | --- |
| 2Tx non-coherent UE | 1210, 1201 | - | - |
| 4Tx non-coherent UE | 121000, 120100,120010, 120001 | 1210010000, 1210000010,1210000001 1201000010, 1201000001,1200001001 | 12100100000010 |

Similar discussion as 4Tx non-coherent UEs, the new precoders for supporting full power transmission for rank 2 and 3 is exactly same as non-coherent UEs. For rank-1, in addition to 121000, 120100,120010, 120001, it is also possible to use 121010, 1210-10, 1210j0,1210-j0 for the case of 20dBm+17dBm+20dBm+17dBm, and 120101,12010j,12010-1,12010-j for the case of  17dBm+20dBm+17dBm+20dBm.

So, for the partial coherent UEs, the following precoders in Table-4 can support full power transmission, which depends on UE capability reporting as discussed in Section 3.

Table 4: 4Tx codebook subset can be for full power transmission for partial-coherent UE

|  | Rank 1 | Rank 2 | Rank 3 |
| --- | --- | --- | --- |
| 4Tx Partial-coherent UE | 121000, 120100,120010, 120001 121010, 1210-10, 1210j0,1210-j0 120101,12010j,12010-1,12010-j | 1210010000, 1210000010,1210000001 1201000010, 1201000001,1200001001 | 12100100000010 |

Note that 121010, 1210-10, 1210j0,1210-j0 is a group precoders that corresponds to the same PAs. If one of precoder can enable full power transmission, other precoders in the same group can be used for full power transmission. So, in the UE capability reporting, one of precoders can represent the whole group, which will be further explained in the Section 3.

Proposal 3: For Mode 2, in addition to the precoders that already support full power transmission in Rel-15, the precoders in Table 3 and Table 4 should be supported based on UE’s capability reporting.

UE capability reporting

UE capability reporting aims to let gNB clearly understand how to enable full power transmission. The reported information include not only whether to support full power but also how to schedule full power transmission (i.e., which precoder for full power transmission). Please note that the PA architectures and virtualization solutions are UE’s implementation, which will not be disclosed.

For the UL MIMO transmission, the precoding is based on the TPMI indication (Note that full power transmission already supported for Non-codebook based UL transmission from Rel-15, so here only discuss the full power transmission for codebook based UL transmission). To schedule full power transmission, gNB need to know which precoder/TPMI can support full power transmission. So, in the UE capability reporting, to report the TPMI/TPMI group(s) which can support full power transmission is an easy and direct way to let gNB understand how to enable full power transmission.

By the way, UE PA architecture 1/2/3 (marked as UE capability during the discussion) will not be disclosed by each UE. So, we cannot assume the PA architecture information is known in the design of UE capability reporting. However, whether to use 2Tx or 4Tx codebooks, and whether to use non-coherent/partial-coherent/full-coherent codebook subsets are already reported by UE capability from Rel-15. So, those information is known during the design of UE capability reporting.

Observation 2: PA architecture 1/2/3 cannot be assumed known at gNB in the signaling design.

UE capability reporting for Mode-1

As discussed in Section 2.1, for Mode-1, to support full power, there is introduced new codebook subset for 2Tx and 4Tx, where the introduced new codebook subset can support full power transmission. UE needs to report the TPMI information to let gNB know which precoders are introduced and can support full power.

For 2Tx, it is very easy, only one precoder introduced (as shown in Table-1). So, UE only need to report whether support  1211.

Proposal 4: In Mode-1, for 2Tx, UE only reports whether support  1211 for full power transmission.

For 4Tx non-coherent UE, as shown in Table-1, since different PA architectures corresponds to different precoders for full power transmission. UE with 20+17+20+17 dBm support 121010 for full power transmission, but cannot support full power with 120101. Furthermore, for the other precoders  1211jj, 121001100-j, and 12100100100001, UE also needs to report whether support all the precoders in the new codebook subset or only part of them are supported, which may impact on the DCI signaling design as discussed in Section 4 and UE complexity as well. So, totally 5 bits (bit-map) is required for UE capability reporting for 4Tx non-coherent UEs.

Proposal 5: In Mode-1, for 4Tx non-coherent, UE needs to report whether support following each of 5 precoders for full power transmission.

|  | Rank 1 | Rank 2 | Rank 3 |
| --- | --- | --- | --- |
| 4Tx Non-coherent UE | 121010, 120101, 1211jj i.e., TPMI=4,8,13 | 121001100-j i.e., TPMI=6 | 12100100100001 i.e., TPMI=1 |

Similarly, for 4Tx partial coherent UE, UE needs to report to gNB whether all of the precoders in the new codebook subset are supported or only support part of them. The DCI signaling and UE complexity may be impacted due to the new introduced codebook subset. So, for OFDM case, 4 bits are required for UE capability reporting in 4Tx partial coherent UE, and 8 bits are required for DFT-S-OFDM cases.

Proposal 6: In Mode-1, for 4Tx partial-coherent, UE need to report whether support following each of 4 and 8 precoders for full power transmission for the OFDM case and DFT-s-OFDM cases, respectively.

|  | Rank 1 |
| --- | --- |
| 4Tx Partial-coherent UE (OFDM) | 121111, 1211jj,1211-1-1, 1211-j-j, i.e., TPMI=12,13,14,15 |
| 4Tx Partial-coherent UE (DFT-OFDM) | 12111-1,1211jj, 1211-11,1211-j-j, 121j1j,121jj1, 121j-1-j,1211-j-1, i.e., TPMI=12,13,14,15,16,17,18,19 |

UE capability reporting for Mode-2

In Mode 2, there is no new codebook subset introduced for full power transmission, Rel-15 precoders are reused for full power. So, there is not necessary to report the new codebook subset, only need to figure out which precoder can be full power transmission. Whether port virtualized or not and how virtualize antenna ports is an UE implementation, UE will not to disclose the virtualization information. Actually, in Mode-2, the information of which TPMI/TPMI group(s) can support full power for the UE is sufficient for gNB to schedule full power. For example, if UE report 1210 can support full power to gNB, then gNB will understanding the scheduling with 1210can be full power transmission, no matter the physic antenna/PA architecture is 23+20dBm or 20+20+17+17dBm, and port virtualized or not.

As discussed in Section 2.2, the possible precoders for full power transmission already shown in Table 3 and Table 4. For 2Tx non-coherent UE, only two precoders 1210 and 1201are needed to be reported whether both or one of them can support full power transmission. So, totally 2 bits for bit-map is sufficient.

Proposal 7: In Mode-2, for 2Tx, support 2 bits (bit-map) UE capability reporting for the precoders  1210 and 1201.

For 4Tx non-coherent UE, there are 11 precoders for rank1~rank3 may support full power transmission, which may correspond to different UE PA architectures or virtualization. So, totally 11 bits for bit-map is sufficient for the UE capability reporting.

Proposal 8: In Mode-2, for 4Tx non-coherent UE, support 11 bits (bit-map) UE capability reporting for the following precoders.

|  | Rank 1 | Rank 2 | Rank 3 |
| --- | --- | --- | --- |
| 4Tx Non-Coherent UE | 121000, 120100,120010, 120001 i.e., TPMI=0,1,2,3 | 1210010000, 1210000010,1210000001 1201000010, 1201000001,1200001001 i.e., TPMI=0,1,2,3,4,5 | 12100100000010 i.e., TPMI=0 |

For 4Tx partially coherent UE, since the group of precoders  121010, 1210-10, 1210j0,1210-j0 mapping to the same antennas, so only 1 bit to indicate the precoder/TPMI group can support full power or not. Only 1 bits for the precoder group  120101,12010j,12010-1,12010-j as well. For simplicity, the first precoder/TPMI can be used for representing the TPMI group, i.e., using 121010 and  120101 to represent the above two precoder/TPMI groups. If UE report 121010for full power transmission, then gNB can understand any of precoders in the group   121010, 1210-10, 1210j0,1210-j0 can be for full power transmission. So, totally 10 bits UE capability reporting are sufficient for the case.

Proposal 9: In Mode-2, for 4Tx partial-coherent UE, support 13 bits (bit-map) UE capability reporting for the following precoders.

|  | Rank 1 | Rank 2 | Rank 3 |
| --- | --- | --- | --- |
| 4Tx Partial-coherent UE | 121000, 120100,120010, 120001 121010, 120101 TPMI=0,1,2,3,4,8 | 1210010000, 1210000010,1210000001 1201000010, 1201000001,1200001001 TPMI=0,1,2,3,4,5 | 12100100000010 TPMI=0 |

Overhead analysis for UE capability reporting with TPMI:

For 2Tx, as discussed above, there are only two precoders need to be reported, i.e., 1210 and 1201. So, only 2 bits are needed if with bitmap reporting for the two precoders. For rank-2 in 2Tx, full power can be enabled by any rank-2 precoder, so there is no additional information on full power need to be reported from UE capability. For 4Tx, there are totally 8 bits reporting for non-coherent UEs, and 10 bits for TPMI/TPMI group reporting is needed for 4Tx partial-coherent UEs for full power transmission.

It is worth noting that, for RRC overhead, the CSI (such as type-II) configuration, how many bits for each band/band combination per UE? The mentioned overhead are much more than UE capability reporting 2 or 13 bits for RRC here, while UE report the capability only once in very a long time. So, the overhead should not be an issue.

Observation 3: UE capability reporting is RRC signaling and only once for reporting during the connection, so 13 bits reporting overhead should not be an issue.

DCI impact for TPMI indication

For Mode-2, the Rel-15 codebooks are reused. There is no additional DCI design impact for Mode-2. However, for Mode-1, since new codebook subsets are introduced, the DCI for TPMI indication need to be revised.

For the case of 2Tx DFT-s-OFDM or OFDM case with maxRank=1 as shown in Table 7.3.1.1.2-5 in 38.212, one precoder is added in the new codebook subset, so the totally number of TPMIs for non-coherent UE is from 1 to 2, i.e., 1 more bit is needed for Rel-16 UE for full power transmission.

Table 5. 2Tx non-coherent UE with maxRank=1

| Bit field mapped to index | codebookSubset = nonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |

But for maxRank=2 for OFDM case in Table 7.3.1.1.2-4 in 38.212, there is no additional bits for the new codebook subset, since there is 3 code points in Rel-15, added one code point is the same DCI overhead.

Table 6. 2Tx non-coherent UE with maxRank=2

| Bit field mapped to index | codebookSubset = nonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 |
| 2 | 2 layer: TPMI=0 |
| 3 | 1 layer: TPMI=2 |

For 4Tx non-coherent UE, as discussed in Section 2.1 and Section 3.1, there are multiple precoders are added in the new codebook subset. Which precoders are added in the codebook depends on the UE capability reporting. UE will decide the new codebook subset based on the PA architectures and also considering the implementation complexity. For example, for the non-coherent 4Tx UE, there are totally 5 precoders in the new codebook subset may support full power transmission as shown in Table 1. If the UE is with PA architecture 20+20+20+20dBm, all of the 5 precoders can support full power, so the totally DCI bits for TPMI indication should be increased to 5 bits from 4 bits shown as Table 7. If the UE is with 20+17+20+17dBm PAs, then 120101, i.e., TPMI=8 will be not for full power, so 4 bits is sufficient. By the way, up to UE’s capability reporting, even the UE with 20+20+20+20dBm PAs, UE also possibly reports only rank-1 or rank-2 full power transmission, so the totally DCI overhead is dependent on the UE capability reporting.

For 4Tx partial-coherent UE, there are up to 4 precoders need to be added in the DCI table for the new codebook subset. The exact number of precoders in the new codebook subset depends on the UE capability reporting. In the Table 7, we show the example that all of the precoders are enabled for full power transmission for the case of maxRank is more than 1.

Table 7. 4Tx Partial-/Non-coherent UEs with maxRank=2/3/4

| Bit field mapped to index | codebookSubset = partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 |
| … | … | … | … |
| 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 |
| 10 | 3 layers: TPMI=0 | 10 | 3 layers: TPMI=0 |
| 11 | 4 layers: TPMI=0 | 11 | 4 layers: TPMI=0 |
| 12 | 1 layer: TPMI=4 | 12 | 1 layer: TPMI=4 |
| … | … | 13 | 1 layer: TPMI=8 |
| 19 | 1 layer: TPMI=11 | 14 | 1 layer: TPMI=13 |
| 20 | 2 layers: TPMI=6 | 15 | 2 layer: TPMI=6 |
| … | … | 16 | 3 layer: TPMI=1 |
| 27 | 2 layers: TPMI=13 |  |  |
| 28 | 3 layers: TPMI=1 |  |  |
| 29 | 3 layers: TPMI=2 |  |  |
| 30 | 4 layers: TPMI=1 |  |  |
| 31 | 4 layers: TPMI=2 |  |  |
| 32 | 1 layer: TPMI=12 |  |  |
| 33 | 1 layer: TPMI=13 |  |  |
| 34 | 1 layer: TPMI=14 |  |  |
| 35 | 1 layer: TPMI=15 |  |  |

For 4Tx non-coherent UEs in the DFT-s-OFDM cases, there are up to 3 precoders in the new codebook subset. The DCI overhead is also dependent on the UE capability reporting. For partial-coherent UEs, as shown in Table 2, there are up to 4 and 8 precoders for supporting full power transmission for OFDM and DFT-S-OFDM, respectively.

Table 8. 4Tx partial-/Non-coherent UEs with maxRank=1

| Bit field mapped to index | codebookSubset= partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 1 layer: TPMI=4 | 4 | 1 layer: TPMI=4 |
| … | … | 5 | 1 layer: TPMI=8 |
| 11 | 1 layer: TPMI=11 | 6 | 1 layer: TPMI=13 |
| 12 | 1 layer: TPMI=12 |  |  |
| 13 | 1 layer: TPMI=13 |  |  |
| 14 | 1 layer: TPMI=14 |  |  |
| 15 | 1 layer: TPMI=15 |  |  |
| 16 | 1 layer: TPMI=16 (only for DFTsOFDM) |  |  |
| 17 | 1 layer: TPMI=17 (only for DFTsOFDM) |  |  |
| 18 | 1 layer: TPMI=18 (only for DFTsOFDM) |  |  |
| 19 | 1 layer: TPMI=19 (only for DFTsOFDM) |  |  |

Proposal 10: For Mode-1, the DCI table for TPMI indication should be extended due to the new codebook subset introduced, where the exact precoders in the new codebook subset is based on UE capability reporting.

Summary of discussions

Based on the above discussion, we have the following observations:

Observation 1: Non-antenna selection precoder can be used for the case of full power transmission achieved by only using all PAs, and antenna-selection precoder can be used for the case of full power transmission achieved by using part of PAs.

Observation 2: PA architecture 1/2/3 cannot be assumed known at gNB in the signaling design.

Observation 3: UE capability reporting is RRC signaling and only once for reporting during the connection, so 13 bits reporting overhead should not be an issue.

Also, we have the following proposals:

Proposal 1: For Mode-1, the new codebook subsets for full power transmission should include the precoders shown in Table 1 and Table 2 for Non-coherent and Partial coherent UEs, respectively.

Proposal 2: For full power transmission, 3 SRS resources should be supported.

Proposal 3: For Mode 2, in addition to the precoders that already support full power transmission in Rel-15, the precoders in Table 3 and Table 4 should be supported based on UE’s capability reporting.

Proposal 4: In Mode-1, for 2Tx, UE only reports whether support  1211 for full power transmission.

Proposal 5: In Mode-1, for 4Tx non-coherent, UE needs to report whether support following each of 5 precoders for full power transmission.

|  | Rank 1 | Rank 2 | Rank 3 |
| --- | --- | --- | --- |
| 4Tx Non-coherent UE | 121010, 120101, 1211jj i.e., TPMI=4,8,13 | 121001100-j i.e., TPMI=6 | 12100100100001 i.e., TPMI=1 |

Proposal 6: In Mode-1, for 4Tx partial-coherent, UE need to report whether support following each of 4 and 8 precoders for full power transmission for the OFDM case and DFT-s-OFDM cases, respectively.

|  | Rank 1 |
| --- | --- |
| 4Tx Partial-coherent UE (OFDM) | 121111, 1211jj,1211-1-1, 1211-j-j, i.e., TPMI=12,13,14,15 |
| 4Tx Partial-coherent UE (DFT-OFDM) | 12111-1,1211jj, 1211-11,1211-j-j, 121j1j,121jj1, 121j-1-j,1211-j-1, i.e., TPMI=12,13,14,15, 16,17,18,19 |

Proposal 7: In Mode-2, for 2Tx, support 2 bits (bit-map) UE capability reporting for the precoders  1210 and 1201.

Proposal 8: In Mode-2, for 4Tx non-coherent UE, support 11 bits (bit-map) UE capability reporting for the following precoders.

|  | Rank 1 | Rank 2 | Rank 3 |
| --- | --- | --- | --- |
| 4Tx Non-Coherent UE | 121000, 120100,120010, 120001 | 1210010000, 1210000010,1210000001 1201000010, 1201000001,1200001001 | 12100100000010 |

Proposal 9: In Mode-2, for 4Tx partial-coherent UE, support 13 bits (bit-map) UE capability reporting for the following precoders.

|  | Rank 1 | Rank 2 | Rank 3 |
| --- | --- | --- | --- |
| 4Tx Partial-coherent UE | 121000, 120100,120010, 120001 121010, 120101 | 1210010000, 1210000010,1210000001 1201000010, 1201000001,1200001001 | 12100100000010 |

Proposal 10: For Mode-1, the DCI table for TPMI indication should be extended due to the new codebook subset introduced, where the exact precoders in the new codebook subset is based on UE capability reporting.

- References

3GPP, “RAN1#95 Meeting Chairman’s Notes”, Spokane, USA, November 12th – 16th, 2018.

3GPP, “RAN1 AdHoc#1901 Meeting Chairman’s Notes”, Taipei, January 21th – 25th, 2019.

3GPP, “RAN1#96 Meeting Chairman’s Notes”, Athens, Greece, February 25th – 1st March, 2019.

3GPP, “RAN1#96bis Meeting Chairman’s Notes”, Xi’an, China, April 8th – 12th, 2019.

3GPP, “RAN1#97 Meeting Chairman’s Notes”, Reno, USA, May 13th – 17th, 2019.

- Appendix A Precoding matrix in current spec

Table 1: Precoding matrix  for single-layer transmission using four antenna ports with transform precoding disabled in TS 38.211

| TPMI index | (ordered from left to right in increasing order of TPMI index) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 – 7 |  |  |  |  |  |  |  |  |
| 8 – 15 |  |  |  |  |  |  |  |  |
| 16 – 23 |  |  |  |  |  |  |  |  |
| 24 – 27 |  |  |  |  | - | - | - | - |

Table 2: Precoding matrix  for single-layer transmission using four antenna ports with transform precoding enabled in TS 38.211.

| TPMI index | (ordered from left to right in increasing order of TPMI index) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 – 7 |  |  |  |  |  |  |  |  |
| 8 – 15 |  |  |  |  |  |  |  |  |
| 16 – 23 |  |  |  |  |  |  |  |  |
| 24 – 27 |  |  |  |  | - | - | - | - |

- Appendix B. Simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 4GHz |
| Subcarrier spacing | 15kHz |
| Delay spread | 100ns |
| System band | 5MHz |
| PUSCH RB number | 2 |
| Channel model | TDL-C |
| UE speed | 3km/h |
| UE antenna configurations | 4Tx |
| BS antenna configurations | 4Rx |
| Receiver | MMSE |
| Link adaptation | Fixed rank-1/rank-2 with MCS adaptation |
| Maximum transmission power | 23 dBm |