3GPP TSG RAN WG1 Meeting #98	R1-1908096

Prague, August 26 - August 30, 2019

Agenda Item:	6.2.4.1

Source:	Shanghai Jiao Tong University

Title:	New numerology for PMCH to support rooftop reception

Document for:	Discussion

Introduction

The approved new work item on LTE-based 5G terrestrial broadcast [1] contains the following objectives:

- Specify new numerology(ies) for PMCH for support of rooftop reception in MPMT and HPHT-1 scenarios as described in TR 36.776 [RAN1, RAN4]

  - The new numerology(ies) shall have a cyclic prefix length of at least 300us and a core OFDM symbol duration (excluding cyclic prefix) of at least 2.4ms.

  - The definition of the new numerology, apart from link/system level performance, shall consider factors such as UE complexity and frame structure alignment with the cell acquisition subframes (CAS).

  - This objective comprises specifying associated RS pattern(s), MCS, TBS, UE capabilities for the new numerology(ies).

  - RAN1 should strive to define a single numerology to cover both MPMT and HPHT-1 cases.

In RAN1 #97bis meeting[2], agreements for new numerology to support rooftop reception are made as follow:

Agreement:

Selection of the new numerologies for support of rooftop reception in HPHT-1 and MPMT scenarios is based on the candidates listed in the Table X below.

Table X: New numerologies for support of rooftop reception in MPMT and HPHT-1 scenarios

| Tcp (us) | Tu (ms) | T (ms) | Numerology (kHz) | FFT size | Number of MBSFN subframes per 40ms in MBMS-dedicated carrier (with no gap overhead) | Number of MBSFN subframes per 5ms (with gap overhead in mixed-carrier ) | CP overhead |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 386 | 2.4 | 2.786 | 0.417 | 36864 | 14 | 1 (4.3%) | 13.9% |
| 300 | 2.7 | 3 | 0.370 | 41472 | 13 | 1 (0%) | 10.0% |

In this contribution, we provide different RS patterns for new numerologies and evaluate the performance of these RS patterns. In Section 2, we discuss the design principles of RS pattern of the new numerologies for support of rooftop reception. In Section 3, we provide link level evaluation results. In section 4, we summarize our observations and provide proposals for the way forward.

RS pattern for the new numerology

The RS pattern to be designed is based on the candidate numerologies with 300/2700 and 386/2400 according to the agreement in RAN1 #97bis meeting.

Design principles for RS pattern

For the single-antenna LTE system, the RS structure of a physical resource block (PRB) is studied. For the candidate numerologies, the subframe is 3ms, and the frequency domain is 144 sub-carriers with a width of 0.37 kHz or 0.417kHz. RS should be staggered in the frequency domain.

In the LTE system, in order to guarantee the performance of channel estimation, the RS interval in the frequency domain and the time domain of the LTE system should obey the rules below. Df is used to represent the RS tone separation in the frequency domain and Dt is to represent RS tone separation in the time domain.

，

Where  is the maximum Doppler shift and  is the maximum multipath delay.

Design examples for RS pattern

For the new numerology of the CP length of Tcp=300us and core symbol duration of Tu = 2700us, one example for the RS pattern is shown in figure 1. The Dt is 2 and Df is 3. In such case, EI is Tu/Df =2700/3=900us.

Figure 1: One example for RS pattern of the new numerology.

The values of Df and Dt determine the RS overhead and also the performance of channel estimation. According to Figure 1 and the formula of EI, the larger the Df is, the smaller the RS overhead and EI are.

From the system simulation[3]，the absolute SINR achieved for the 95% locations in MPMT network is plotted in figure 2.

Figure 2: 95th percentile SINR for various numerologies. MPMT fixed rooftop.

The absolute SINR achieved for 95% locations in the HPHT network is summarized by figure 3.

Figure 3: 95th percentile SINR for various numerologies. HPHT1 fixed rooftop.

From the Figure 2 and Figure 3 above, we can see that the performance of Df=2/3/4 is better than performance of Df=6/8 in both MPMT and HPHT-1 fixed rooftop. The RS overhead would be too large if Df=1. Therefore, Df=2/3/4 might be a good tradeoff.

In addition, the larger Dt is the smaller RS overhead is. However, Dt cannot be too large because of the large channel delay for which a dense RS in time domain is usually expected for better channel estimation performance.

Given the analysis above, some combination candidates of Df and Dt as well as the EI length and RS overhead are summarized in Table 1.

Table 1: Parameters of combination candidates for RS pattern

| Df | EI (Tu/Df) (300/2700) | EI (Tu/Df) (386/2400) | Dt | RS overhead |
| --- | --- | --- | --- | --- |
| 3 | 900 | 800 | 1 | 1/3 |
| 2 | 1350 | 1200 | 2 | 1/4 |
| 3 | 900 | 800 | 2 | 1/6 |
| 4 | 675 | 600 | 2 | 1/8 |
| 2 | 1350 | 1200 | 3 | 1/6 |
| 2 | 1350 | 1200 | 4 | 1/8 |

Link -level Performance Evaluations

Link-level simulations were carried out using three different RS patterns. Simulation parameters are summarized in Table 2.

Table 2: parameters for Link-Level simulation

| Parameter | Value |
| --- | --- |
| System bandwidth | 10 MHz |
| Carrier frequency | 700 MHz |
| Numerology: CP/Subcarrier/FFT size | 300us/0.37kHz/41472 |
| Channel model | TDL-B with DS of 35us(MPMT) and 45us(HPHT-1) |
| Number of Tx at Base Station | 1 |
| Number of Rx antennas at UE | 2 |
| Turbo Decoding Algorithm | Max-Log-MAP with a maximum of 8iterations |

In this contribution, we focus our discussion on the numerology of 300/2700. Figure 4 and Figure 5 compare performance over TDL-B channel for different RS patterns with TBS of 42368.

Figure 4: Performance for different combinations of Df and Dt with TBS=42368. MPMT fixed rooftop.

Figure 5: Performance for different combinations of Df and Dt with TBS=42368. HPHT-1 fixed rooftop.

From Figure 4 and Figure 5, it can be seen that:

- For MPMT scenarios, the RS pattern with Df=3, Dt=2 and Df=2, Dt=2 have the almost same performance. When BER=1e-3, the gain of Df=3, Dt=1 is about 1dB compared with Df=2, Dt=2， which benefits from more accurate channel estimation.

- For HPHT-1 scenarios, the RS pattern with Df=3, Dt=2 and Df=2, Dt=2 have the almost same performance. When BER=1e-3, the gain of Df=3, Dt=1 is about 3dB compared with Df=2, Dt=2.

Observation 1: For MPMT scenarios, the RS pattern with Df=3, Dt=2 shows similar BER performance with the RS pattern with Df=2, Dt=2. The RS pattern with Df=3, Dt=1 has the best performance. When BER=1e-3, the gain of Df=3, Dt=1 is about 1dB compared with Df=2, Dt=2.

Observation 2: For HPHT-1 scenarios, the RS pattern with Df=3, Dt=1 has the best performance. When BER=1e-3, the gain of Df=3, Dt=1 is about 3dB compared with Df=2, Dt=2.

For MPMT and HPHT-1 scenarios, the simulation results for the achievable spectral efficiency for rooftop scenario are plotted in figure 6.

(a) MPMT                                  (b) HPHT

Figure 6: Spectral efficiency for different RS patterns.

Observation 3: The RS pattern with Dt=2, Df=3 has the highest spectral efficiency in both MPMT and HPHT-1 scenarios for rooftop reception.

Although RS pattern with Df=3, Dt=1 has the best BER performance, its RS overhead is the largest. In addition, the RS pattern with Dt=2, Df=3 achieves more spectral efficiency.

Proposal 1: The RS pattern for the new numerology shall be selected between Dt=2, Df=3 and Dt=1, Df=3 based on further evaluations of performance and spectral efficiency.

Conclusions

Below is the summary of obervations and proposals:

Observation 1: For MPMT scenarios, the RS pattern with Df=3, Dt=2 shows similar BER performance with the RS pattern with Df=2, Dt=2. The RS pattern with Df=3, Dt=1 has the best performance. When BER=1e-3, the gain of Df=3, Dt=1 is about 1dB compared with Df=2, Dt=2.

Observation 2: For HPHT-1 scenarios, the RS pattern with Df=3, Dt=1 has the best performance. When BER=1e-3, the gain of Df=3, Dt=1 is about 3dB compared with Df=2, Dt=2.

Observation 3: The RS pattern with Dt=2, Df=3 has the highest spectral efficiency in both MPMT and HPHT-1 scenarios for rooftop reception.

Proposal 1: The RS pattern for the new numerology shall be selected between Dt=2, Df=3 and Dt=1, Df=3 based on further evaluations of performance and spectral efficiency.

- References

RP-190732; “New WID on LTE-based 5G terrestrial broadcast”; Qualcomm Inc.; 3GPP TSG RAN #83, Shenzhen, China, March 2019.

RAN1#97bits Chairman notes,Reno,USA.

R1- 1903284；“Evaluation Results for LTE-Based 5G Terrestrial Broadcast”; EBU, BBC, IRT；3GPP TSG RAN WG1 Meeting #96 Athens, Greece 25th February – 1st March 2019.