3GPP TSG RAN WG1 Meeting #98	R1-1908099

Prague, Czech Republic, August 26-30, 2019

Agenda Item:	7.2.13.3

Source:	Huawei, HiSilicon

Title:	Discussion on SCell activation and deactivation

Document for:	Discussion and Decision

# Introduction

In RAN1#96bis meeting and email discussion [1], several issues were discussed including two aspects: one is about reducing SCell activation time and another is about efficient SCell management by providing some form of “dormancy-like behavior” for BWP operation.

In this contribution these two aspects and other CA related issues are further discussed.

# Low latency SCell activation

From current specification in TS 38.213, TS 38.133 and TS 38.321, when a UE receives an activation command in a PDSCH for a SCell in slot n, the SCell activation procedure starts at n+k and completes at the slot with valid CSI reported. The value of  k=k1+3∙Nslotsubframe,μ+1 is defined in RAN1 specification. The actions related to CSI reporting for the SCell being activated is started at n+k. The CQI value of out of range should be reported in the CSI report before valid CSI for the SCell is available for the UE. According to RAN4 requirement for SCell activation delay defined in TS 38.133, a UE shall complete a SCell activation no later than n+ [THARQ + Tactivation_time + TCSI_Reporting], where THARQ is the timing between DL data transmission and acknowledgement corresponding to k1 in k described above. Tactivation_time includes the delay of MAC-CE parsing time, RF warm up, AGC setting and frequency/time synchronization. TCSI_reporting is the delay including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources. The MAC-CE parsing /software application/RF retuning processing time is roughly 3ms.

Firstly, the Tactivation_time defined in RAN4 can be summarized in Table 1 where TSMTC_SCell is the SMTC periodicity of SCell being activated and the value can be configured from 5ms to 160ms. TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated in case of intra-band SCell activation in FR1 and there is roughly 5 ms delay including 3 ms MAC-CE parsing/software application/RF retuning processing time and 2 ms uncertainty of SSB measurement. As can be seen, TSMTC_SCell is the main component of Tactivation_time defined in RAN4.

Table 1 T_activation_time defined in RAN4

|  | FR1 |  |  | FR2 |  |
| --- | --- | --- | --- | --- | --- |
|  | SCell known |  | SCell unknown | at least one active serving cell on that FR2 band | there is no active serving cell on that FR2 band |
|  | SCell measurement cycle =< 160 ms | SCell measurement cycle > 160 ms |  |  |  |
| Tactivation_time | [TSMTC_SCell + 5ms] | [TSMTC_MAX + TSMTC_SCell + 5ms] | [2*TSMTC_MAX + 2*TSMTC_SCell + 5ms] | [TSMTC_SCell + 5ms] | [TBD* TSMTC_SCell + 5ms] |

Secondly, compared with main component of Tactivation_time, delay of SCell activation command only takes a small portion of the overall SCell activation delay. For example, for a known cell with SCell measurement cycle less than 160 ms, the maximum total activation delay would be roughly THARQ + Tactivation_time + TCSI_Reporting = K1 + TSMTC_SCell + 5ms+ uncertainty in acquiring the first available downlink CSI reference resource+Z+ uncertainty in acquiring the first available CSI reporting resources=1ms + 20ms +5ms+4ms=30ms. The latency reduction by introducing L1 activation command compared with MAC-CE used in current network would be about 1~2 ms, which is far from a meaningful optimization from the total delay. For the case of SCell measurement cycle larger than 160 ms or unknown cell, the gain will be even smaller. Further if L1 based activation command is adopted, HARQ feedback needs also to be introduced in order to confirm the reception of activation command, similar as what has been widely used in LTE DL SPS release, then extra latency would be required. There is no obvious benefits to consider L1 based activation command.

Thirdly, in the current CSI framework, periodic CSI-RS (P-CSI), semi-persistent CSI-RS (SP-CSI) and aperiodic CSI-RS (A-CSI) resource can be configured (as well as periodic, semi-persistent and aperiodic CSI reporting). Among them, A-CSI resource triggering/reporting requires network has good knowledge for each UE to decide a proper activation timing, which is not really practical as the cell search and frequency/time tracking implementation are variable for different UEs. Simple P-CSI resource and periodic CSI reporting can be applied after SCell is configured in this case.

Based on the above, we have

Observation 1: During the SCell activation,

- SMTC periodicity is the main component;

- L1 based activation signaling through PDCCH does not seem to provide obvious benefit on latency;

- Periodic CSI-RS and periodic CSI reporting are simple process for frequency/time tracking, measurement and reporting.

Taking LTE as a reference, the SCell activation start delay is 8 subframes and the largest activation processing delay for one normal SCell is 24~34 subframes. The activation processing delay would be about 3 to 4 times compared to the activation start delay. It is also noted that CRS symbols can be used to AGC setting, downlink frequency/time tracking and CSI measurement for SCell activation procedure in LTE. However, as SS/PBCH block symbols are not distributed over the whole system bandwidth and measured in close time interval as CRS symbols in NR, the measurement samples for AGC estimation and downlink timing reference will be less than LTE. That implies that SCell activation in NR may need more measurements symbols in the time domain to ensure the same accurate power gain estimation and timing accuracy.

In NR, the measurement samples for AGC estimation and downlink timing reference is mainly related to the SS/PBCH block measurement timing configuration (SMTC). Considering the cell-specific overhead, SMTC will probably not be configured with the minimum value. 20ms can be seen as a common configuration. Large period of SMTC will result in high latency cell search and time tracking. It should be noted that CSI-RS can also be used to frequency/time tracking for UE besides CSI measurement. And short period CSI-RS for tracking during SCell activation can help to reduce Tactivation_time.

Valid CSI report is the last procedure in SCell activation. TCSI_Reporting reflects the latency of this part processing and includes uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources. Thus the time interval of CSI reference resources and reporting resources would also impact on the SCell activation delay.

Proposal 1: Short interval periodic CSI-RS resource and CSI reporting can be supported for low latency of SCell activation.

# Efficient SCell management

In email discussion [1], for NR, when cross-carrier scheduling is not configured, the following additional implicit/explicit L1 signaling mechanisms which can enable fast adaptation from sparse/no PDCCH monitoring to more frequent PDCCH monitoring on the activated SCells are observed.

  - Potential additional L1 signalling mechanisms include

    - Using DCI on cell x to control PDCCH monitoring on SCell(s)

    - Using DCI on cell x to control CSI measurements on SCell(s)

    - Using DCI on cell x to switch between different configured BWPs on SCell(s)

    - Using implicit association between triggering of active BWP switching on PCell/scheduling cell and triggering of BWP switching on SCell(s).

For NR, when cross-carrier scheduling is configured, it is possible using Rel15 signaling to support activated SCells with no PDCCH monitoring on the SCell and with CSI measurement/reporting. Switching between cross-carrier and self-scheduling for activated SCells is based on RRC reconfiguration. And additional mechanisms such as per BWP configuration of cross-carrier scheduling can enable fast adaptation between cross-carrier and self-scheduling for activated SCells.

## Cross-carrier scheduling

If cross-carrier scheduling is configured, the PDCCH monitoring parameters except the number of PDCCH candidates for the scheduled cell will follow the PDCCH monitoring parameters on the scheduling cell. Though the number of PDCCH candidate for scheduled cell can be configured as zero, it does not make sense to do this in order to enable cross-carrier scheduling operation.

Regarding PDCCH monitoring, frequent PDCCH monitoring would be expected on the scheduling cell. Sparse/no PDCCH monitoring for the scheduled cell could not be applied based on current R15 search space configuration framework for cross-carrier scheduling in this case. Although there is no PDCCH monitoring on the scheduled cell, the PDCCH monitoring for the scheduled cell still exists. The effort for PDCCH monitoring for the scheduled cell is just transferred from scheduled cell to the scheduling cell. Furthermore, CSI measurement or other procedure of “dormancy-like behavior” in BWP on the scheduled SCell are still on operation. RF cannot be switched off for the scheduled cell. Then from UE perspective, the effect of “dormancy-like behavior” in BWP for the scheduled cell may not be significant.

Unless SearchSpace configuration for cross-carrier scheduling is optimized to support separate PDCCH monitoring parameter configuration for the scheduled cell, the same issue still exists even if per BWP configuration of cross-carrier scheduling is introduced.

Observation 2: Without search space configuration optimization for scheduled cell (e.g. separate PDCCH monitoring period), even if per BWP configuration of cross-carrier scheduling is introduced, the benefit of “dormancy-like BWP behaviour” for the SCell may not be significant.

## Additional explicit L1 signalling

Self-scheduling by explicit L1 signalling for control of PDCCH monitoring or CSI measurements on SCell(s) can have various alternatives. It can be found that the PDCCH monitoring parameters are configured in SearchSpace within downlink BWP configuration. CSI measurement configuration is also associated to a BWP in CSI-ResourceConfig. Different BWPs can be configured with different PDCCH monitoring and CSI measurement parameters. Switch between different configured BWPs is an easy way to control both PDCCH monitoring and CSI measurement on SCells.

Observation 3: Switch between different configured BWPs can control both PDCCH monitoring and CSI measurement.

For simplicity, only PDCCH monitoring control on SCell(s) is considered in the following discussion. If a SCell is configured with two BWPs with sparse/no PDCCH monitoring and more frequent PDCCH monitoring respectively, L1 signaling sent on the SCell cannot support fast adaptation from sparse/no PDCCH monitoring BWP to more frequent PDCCH monitoring BWP, where ‘fast’ is estimated relative to the frequent PDCCH monitoring. Therefore, cross-carrier BWP switch indication is needed, which would require some new fields or new interpretation for existing fields for cross-carrier BWP switch indication in DCI. If BWP switch in multiple SCells needs to be supported, a bitmap for target BWP ID in different cells need also to be included. Considerable specification effort for this kind of DCI design are expected and related L1 signaling HARQ feedback also needs to be additionally specified. Other explicit L1 signaling for separate PDCCH monitoring control or CSI measurement control has similar issues.

Observation 4: For additional explicit L1 signalling, new DCI design of cross-carrier BWP switch or separate cross-carrier PDCCH monitoring/CSI measurement control is needed for fast adaptation from sparse/no PDCCH monitoring to more frequent PDCCH monitoring.

## Additional implicit L1 signalling

The intention of implicit L1signaling is to use existing L1 signaling in one cell to control operation on other cell(s) based on implicit association between different cells.

The following solution is kind of implicit L1 BWP switch signaling supporting cross-carrier BWP switch in different cells based on BWP bundle configuration. The meaning of BWP bundle can be seen as an association of BWPs across different cells. The BWP bundle can be configured by RRC as listed in Table 2. The first two BWPs (BWP1 and BWP2) configurations in the table are the current normal BWP configuration. The last row is the BWP bundle configuration in Cell 1.  The BWP bundle here represents an association between BWP2 on Cell 1 and BWP1 on Cell 2. And a BWP ID 3 is assigned to the special BWP bundle configuration.

Table 2 BWP bundle configuration

| Serving cell | Feature | BWP ID | Indication | Note |
| --- | --- | --- | --- | --- |
| Cell 1 | Rel-15 mechanism | 1 | BWP1 on Cell 1 | Normal BWP configuration on Cell 1 |
|  |  | 2 | BWP2 on Cell 1 | Normal BWP configuration on Cell 1 |
|  | Proposed additional approach to Rel-15 | 3 | A BWP bundle: BWP2 on Cell 1 and BWP1 on Cell 2 | BWP bundle configuration with new BWP ID corresponding to a BWP on Cell 1 and a BWP on other cell(Cell 2) |

Based on above BWP bundle configuration, two main scenarios for in-and-out BWP sparse/no PDCCH monitoring are discussed.

Scenario 1: Enable fast adaptation from sparse/no PDCCH monitoring to more frequent PDCCH monitoring BWP

Normal BWP to BWP bundle switching:

Figure 1 Implicit BWP switch with BWP bundle (case A)

It can be assumed that BWP2 is a BWP with no/sparse PDCCH monitoring and BWP 1 is a BWP with frequent PDCCH monitoring on Cell 2. Both BWP1 and BWP2 on Cell 1 are BWPs with frequent PDCCH monitoring. As illustrated in Figure 1, L1 BWP switch signalling is transmitted on Cell 1 indicating target BWP ID=3. According to BWP bundle with BWP ID=3 configuration above, active BWP on Cell 1 change to BWP2 and active BWP ID change to 3. The active BWP change from BWP2 to BWP1 on Cell 2 is implicitly triggered. Fast adaptation from sparse/no PDCCH monitoring to more frequent PDCCH monitoring BWP on Cell 2 is applied with BWP bundle configuration reusing existing L1 BWP switching signalling on Cell 1. Furthermore, BWPs in different cells can switch simultaneously by only one L1 BWP switch signalling, which can further increase transmission efficiency.

Figure 2 Implicit BWP switch with BWP bundle (case B)

As illustrated in Figure 2, if the BWP ID of the current active BWP on Cell 1 is the same as the BWP ID in the BWP bundle configuration for the cell, the actual active BWP Cell 1 is maintained on BWP2 but the active BWP ID is updated to 3. The BWP switch from BWP2 to BWP1 on Cell 2 is triggered by the implicit L1 BWP switching signaling on Cell 1.

BWP bundle to normal BWP switching:

After BWP switch according to BWP ID with BWP bundle, if necessary, the active BWP ID can be changed to normal BWP ID via a BWP switch on Cell 1 with target normal BWP ID (e.g. BWP =2). As shown in Figure 3,  a L1 BWP switch signaling with target BWP ID=2 can be transmitted on Cell 1 to update active BWP ID to normal BWP ID 2. This BWP switch signaling does not trigger BWP change on Cell 1 neither BWP change on Cell 2. The procedure of BWP ID recover is to prepare for the next BWP switch with BWP bundle.

Figure 3 Implicit BWP switch with BWP bundle (case C)

Scenario 2: Adaptation from more frequent PDCCH monitoring BWP to sparse/no PDCCH monitoring

This can be applied via legacy BWP switch signaling, e.g. L1 BWP switch signaling or bwp-InactivityTimer on each cell. Of course BWP switch with BWP bundle can also be applied to support fast simultaneously BWP switch.

Furthermore, if with proper BWP configuration, e.g. same RF related parameters for BWP1 and BWP2 on both Cell 1 and Cell 2, short BWP switch delay even zero delay could be expected.

Based on the above analysis, the following proposal is given:

Proposal 2: BWP switch with BWP bundle configuration can be supported for fast adaptation from sparse/no PDCCH monitoring to more frequent PDCCH monitoring.

# Analysis on use of low latency SCell activation vs. efficient SCell management

## Latency for data transfer

It is known that some traffic patterns are burst in nature which may result short lived connections. Long SCell activation delay will limit utilization of larger spectrum in NR as the UE may have left connected mode already when SCell is activated. It is nature to find some ways to reduce SCell activation delay in order to efficiently and fast utilize large BW based on CA/DC.

The state of SCell activated is for maximum peak data rate while SCell deactivated is for minimized power consumption in CA/DC. An intermediate “dormancy-like behavior” on SCell which can reduce latency for data transfer and keep a moderate power consumption was proposed for efficient spectrum utilization. For simplicity, the behaviour in current SCell activated refers to activated (normal) and “dormancy-like behaviour” on activated SCells refers to activated (dormancy).

Figure 4 Traffic data transfer in CA/DC

The latency for data transfer and power consumption comparison between these two alternatives are discussed in the following subsections. As illustrated in Figure 4, the latency for data transfer from deactivated SCell to activated SCell can be denoted as T1, which can be seen as SCell activation delay. The latency for data transfer from SCell activated (dormancy) to SCell activated (normal) can be denoted as T2, which can be seen as activated dormancy to normal switching delay.

Latency of SCell from deactivated to activated (normal)

Based on the SCell activation procedure described in section 2 and the following assumptions, SCell activation delay in different scenarios can be summarized in Table 3.

Assumptions:

- SCell SCS:15 kHz ;

- K1=1;

- TSMTC_MAX = TSMTC_MAX =20ms (based on RRC configuration);

- TCSI_reporting= Tuncertainty_CSI_RefRes+ TCSI_processing_time+ Tuncertainty_CSI_ReportRes= 4 ms +2 ms + 4 ms = 10 ms (based on RRC configuration and UE capability)

Table 3 SCell activation delay

| Frequency Range | Other conditions | SCell activation delay |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | THARQ | Tactivation_time | TCSI_Reporting | Total |
| FR1, SCell is known | SCell is known, measurement cycle =< 160 ms | 1 ms | 25 ms | 10 ms | 36 ms |
|  | SCell is known, measurement cycle > 160 ms | 1 ms | 45 ms | 10 ms | 56 ms |
| FR1,SCell is unknown | SCell is unknown | 1 ms | 85 ms | 10 ms | 96 ms |

Taking known SCell in FR1 as an example, if the optimized short interval CSI-RS for synchronization and CSI-RS for CM, Tactivation_time  and  TCSI_Reporting could be reduced to about 4 ms and 6 ms respectively. The total SCell activation delay would be reduced to about 10 ms.

Based on the above summary and analysis:

Observation 5: In current specification, SCell activation delay varies from about ten milliseconds to tens of milliseconds based on SCell FR, states and network configurations. Optimized SCell activation delay could be about 10 ms for FR1 known SCell.

Latency of SCell from activated (dormancy) to activated (normal)

There are a lot of discussions on improvement related to ‘dormancy-like behaviour’ on activated SCells in contributions at last meeting [4]. Before we discuss latency of SCell from dormancy to normal in details. It is necessary to make it clear what is ‘dormancy-like behaviour’ on activated SCells. It is RAN1 understanding at this stage that a UE need to do sparse/no PDCCH monitoring while maintaining CSI measurements/reporting for ‘dormancy-like behaviour’.

According to the following RAN2 agreement in LS [5], other activities such as AGC and beam management were mentioned.

Agreement

1	SCell dormant state like LTE euCA will not be introduced in NR.

2	‘dormancy’ behaviour will be studied as a solution for fast return to SCell utilisation for data transfer. The 'dormancy' behaviour implies that the UE stops monitoring PDCCH but continues other activities such as CSI measurements, AGC and beam management. RAN1/4 input required on feasibility and benefit.

Q1: What is the condition of AGC re-setting?

Q2: Is there any difference between CSI-RS for CM (channel measurement) and CSI-RS for BM (beam management)? Could these two procedure be performed based on the same CSI-RS configuration?

If the time duration of activated (dormancy) exceeds a certain value, AGC re-setting may be needed for dormancy. If CSI-RS for CM and CSI-RS for BM cannot perform simultaneously base on same CSI-RS configuration, the activity on BM should be considered for dormancy behaviour.

Proposal 3: Whether activities of AGC and beam management are needed in ‘dormancy-like behaviour’ on activated SCells should be clarified.

The transition from activated dormancy to normal can be based on BWP switching framework. If activities of AGC and BM are not needed from dormancy to normal, the latency would be equal to BWP switching delay defined in RAN4, which is about 1~3 ms.

Observation 6: Activated SCell from dormancy to normal delay could be about a few milliseconds when not considering AGC and BM procedure, similar to BWP switching delay.

## Power consumption

Power consumption in SCell deactivated

When a SCell is deactivate, neither PDCCH monitoring nor SSB/CSI-RS processing is needed. If the PCell and SCell are inter-band CA, RF channel for SCell can be switched off when SCell is de-active. The power state of a SCell in deactivated would be Deep/Light Sleep, thus much lower relative power in this state. Taking the relative power of Light Sleep as an example (reference to Appendix E):

Observation 7: The energy consumption in a deactivated SCell would be 20* Tdeactivated.

Power consumption in SCell activated (dormancy)

According to the definition of the SCell activated (dormancy) and UE power consumption model in Appendix E, the power consumption for SSB or CSI-RS proc could be inevitable for CSI measurement or synchronization. The relative power for SSB or CSI-RS processing is the same as PDCCH-only 100. The energy consumption of a UE in SCell activated (dormancy) depend on the relative power and the time duration for SSB or CSI-RS processing Tdormancy_CSIProc within dormancy Tdormancy.

Observation 8: When activities of AGC and beam management are not considered, the energy consumption of SCell activated (dormancy) would be 100* Tdormancy_CSIProc+20* (Tdormancy-Tdormancy_CSIProc).

According to the above analysis, both of these two approaches have their own pros and cons. SCell from deactivated to activated would have longer delay but lower power consumption while activated SCell from dormancy to normal would have lower delay but higher power consumption. The usage of the two alternatives can be differentiated as follows: dormant BWP switching targets faster adaptation to short-term traffic variations, while SCell activation/deactivation targets long-term traffic variations. This can be up to network scheduling while standard wise improvement on both is expected.

Proposal 4: Improvements on both R15 SCell activation/deactivation procedure and  ‘dormancy-like behaviour’ on activated BWP can be supported in R16.

# Cross-carrier triggering PDCCH order

The intention of carrier aggregation is to aggregate multiple consecutive or non-contiguous carriers into larger bandwidths to support higher data throughput. After a SCell is activated, random access procedure may also be required to be ready for data transmission. Reducing RACH procedure delay can help improve SCell usage efficiency.

After SCell is activated, the UE has obtained downlink synchronization with the cell, so the UE can receive downlink data. However, the UE can perform uplink transmission only if it acquires uplink synchronization with the cell. The UE establishes uplink synchronization with the cell through a random access procedure. If the SCell being activated and other activated cell are not within the same TAG, random access procedure on the SCell is needed for specific TA measurement. The Random Access procedure on a SCell shall only be initiated by a PDCCH order.

Observation 9: In some cases, random access procedure on a SCell is needed and can only be initiated by a PDCCH order.

In current spec, PDCCH order can only be transmitted by DCI format 1_0. And the PDCCH order can only be sent on the SCell which the random access need to be initiated since there is no carrier indication in the DCI format for PDCCH order. However, before a SCell is activated, PDCCH for the SCell is not monitored for the UE. PDCCH order can only be transmitted after the SCell is activated. The latency of UL timing alignment for the SCell would be increased. The random access on a SCell can be shown in Figure 5. The random access delay for the SCell can be seen as the sum of SCell activation delay and random access triggering delay.

Figure 5 Current random access procedure on SCell being activated

Observation 10: Random access delay for a SCell being activated can be seen as the sum of SCell activation delay and random access triggering delay.

In LTE, DCI format 1A is used for the random access procedure initiated by a PDCCH order. And CIF is present within DCI format 1A when it is mapped to USS. Therefore cross-carrier triggering PDCCH order can be supported in LTE.

If cross-carrier triggering PDCCH order can be supported, as shown in Figure 6, PDCCH order for a SCell being activated can be transmitted on PCell to reduce the latency of UL timing alignment for the SCell.

Figure 6 Optimized random access procedure on SCell being activated

Observation 11: Random access delay for a SCell being activated can be reduced to SCell activation delay if cross-carrier PDCCH order can be supported.

If the UE is configured with the CIF for a given serving cell, the UE can use the CIF value from the detected "PDCCH order" to determine the serving cell for the corresponding random access preamble transmission.” If cross-carrier scheduling is not configured, other carrier indication (e.g. ServCellIndex) can also be added in PDCCH order.

To enable PDCCH order for a different cell, CIF or other carrier indication in DCI format for downlink scheduling is needed. To solve this issue, the following two options can be considered:

Option 1: When DCI format 1_0 is mapping to UE-specific search space, CIF or other carrier indication can be included to support cross-carrier triggering PDCCH order.

Option 2: To support cross-carrier triggering PDCCH order by DCI format 1_1.

Proposal 5: Cross-carrier triggering PDCCH order by DCI format 1_0 or DCI format 1_1 should be supported.

# Power saving issue in CA

In RANP #84, the following was agreed in WID of power saving.

Note: The following objective may be considered in RAN#85 pending progress on SCell dormant behaviour and will be re-evaluated in RAN#85 - Study and specify (if agreed) power saving techniques to reduce PDCCH monitoring on activated SCells for CA and DC (EN-DC in particular). - Solutions (e.g. power saving signal/channel, enhancing Rel-15 DRX procedure) are to be discussed - Enhancements to Rel-15 DRX procedure can be studied further if they do not violate the general principle of single DRX configuration per MAC entity

To avoid duplex discussion in power saving and CA/DC enhancement, the SCell monitoring reduction is recommended to be discussed in CA/DC enhancement.

In power saving topic, the PDCCH based power saving signal outside of Active Time can indicate the UE to or not to monitor PDCCH in the following C-DRX cycles. An example is shown in Figure 7, where the power saving signal before the first DRX cycle indicates the UE to monitor PDCCH during the first DRX cycle, and the next power saving signal indicates the UE not to monitor PDCCH during the second DRX cycle. However, the UE behavior when the UE is configured with multiple cells is not clearly defined.

Figure 7 An example of power saving signal outside of Active Time

When a UE is configured with multiple cells, it can be beneficial for the UE to have the same behavior in all cells in some cases. For example, when a UE is configured with two intra-band cells, the UE may use the same RF chain to receive DL signal. If the UE is indicated to “sleep” (i.e. not monitor PDCCH) in a C-DRX cycle on the two cells, the UE can turn off the RF chain to save power. If the UE is only indicated to “sleep” in a C-DRX cycle on one cell, and keep monitoring PDCCH on the other one cell, the UE cannot turn off the RF chain, and little power is saved. However, in some other cases the UE should be allowed to behave differently on different cells. For example, when a UE is configured with two inter-band cells, the UE may use different RF chain for different cell. Then there is no need to restrict the UE to have the same behavior on different cells. In this case, CC grouping can be used, where in a group the UE may have the same behavior, and the UE behaves differently in a different group. Besides the motivation for hardware design, the UE may desire to behave differently on different cells when monitoring PDCCH on PCell to avoid large latency, and not monitoring PDCCH on SCells to save power.

Proposal 6: Support the UE to have different behaviors on different cell in order to reduce PDCCH monitoring on SCell(s).

# Conclusion

According to the above discussions, we have the following observations and proposals:

Observation 1: During the SCell activation,

- SMTC periodicity is the main component;

- L1 based activation signaling through PDCCH does not seem to provide obvious benefit on latency;

- Periodic CSI-RS and periodic CSI reporting are simple process for frequency/time tracking, measurement and reporting.

Observation 2: Without search space configuration optimization for scheduled cell (e.g. separate PDCCH monitoring period), even if per BWP configuration of cross-carrier scheduling is introduced, the benefit of “dormancy-like BWP behaviour” for the SCell may not be significant.

Observation 3: Switch between different configured BWPs can control both PDCCH monitoring and CSI measurement.

Observation 4: For additional explicit L1 signalling, new DCI design of cross-carrier BWP switch or separate cross-carrier PDCCH monitoring/CSI measurement control is needed for fast adaptation from sparse/no PDCCH monitoring to more frequent PDCCH monitoring.

Observation 5: In current specification, SCell activation delay varies from about ten milliseconds to tens of milliseconds based on SCell FR, states and network configurations. Optimized SCell activation delay could be about 10 ms for FR1 known SCell.

Observation 6: Activated SCell from dormancy to normal delay could be about a few milliseconds when not considering AGC and BM procedure, similar to BWP switching delay.

Observation 7: The energy consumption in a deactivated SCell would be 20* Tdeactivated.

Observation 8: When activities of AGC and beam management are not considered, the energy consumption of SCell activated (dormancy) would be 100* Tdormancy_CSIProc+20* (Tdormancy-Tdormancy_CSIProc).

Observation 9: In some cases, random access procedure on a SCell is needed and can only be initiated by a PDCCH order.

Observation 10: Random access delay for a SCell being activated can be seen as the sum of SCell activation delay and random access triggering delay.

Observation 11: Random access delay for a SCell being activated can be reduced to SCell activation delay if cross-carrier PDCCH order can be supported.

Proposal 1: Short interval periodic CSI-RS resource and CSI reporting can be supported for low latency of SCell activation.

Proposal 2: BWP switch with BWP bundle configuration can be supported for fast adaptation from sparse/no PDCCH monitoring to more frequent PDCCH monitoring.

Proposal 3: Whether activities of AGC and beam management are needed in ‘dormancy-like behaviour’ on activated SCells should be clarified.

Proposal 4: Improvements on both R15 SCell activation/deactivation procedure and  ‘dormancy-like behaviour’ on activated BWP can be supported in R16.

Proposal 5: Cross-carrier triggering PDCCH order by DCI format 1_0 or DCI format 1_1 should be supported.

Proposal 6: Support the UE to have different behaviors on different cell in order to reduce PDCCH monitoring on SCell(s).

# References

R1-1905915, “Summary of email discussion [96b-NR-11]”, Ericsson.

3GPP TS38.133-f50.

R1-1814117, “On search space configuration cross-carrier scheduling	”, NTT DOCOMO, Spokane, United States, November 12 – 16, 2018.

R1-1907681, “Summary of Efficient and low latency serving cell configuration/activation/setup”, Ericsson,

R1-1908003, “LS on NR fast Scell activation”,	RAN2, Qualcomm

# Appendix A. RAN1 agreement of search-space configuration for cross-carrier scheduling

Agreements (RAN1#95): For the agreements made in RAN1#94bis as below:

- UE uses the same ID for linkage of the search spaces configured in scheduled and scheduling cell, and ignores the CORESET ID configured in the scheduled cell

RAN1 re-confirms the agreements, with further clarification as below:

- Number of candidates configured in search-space in scheduled cell apply to the search-space of the same index in scheduling cell.

  - Note: If only one BWP is configured on scheduled cell, then number of cross-carrier search-spaces is limited to 10 instead of 40

  - noOfCandidates in SS set(s) of active BWP of the scheduled cell(s) is applied to the corresponding SS set(s) of active BWP of the scheduling cell(s)

  - Note: this is the original RAN1 solution and concerns of RAN2 has been addressed

# Appendix B. cross-carrier scheduling configuration

#### –	CrossCarrierSchedulingConfig

The IE CrossCarrierSchedulingConfig is used to specify the configuration when the cross-carrier scheduling is used in a cell.

CrossCarrierSchedulingConfig information element

-- ASN1START

-- TAG-CrossCarrierSchedulingConfig-START

CrossCarrierSchedulingConfig ::=        SEQUENCE {

schedulingCellInfo                      CHOICE {

own                                     SEQUENCE {                  -- No cross carrier scheduling

cif-Presence                            BOOLEAN

},

other                                   SEQUENCE {                  -- Cross carrier scheduling

schedulingCellId                        ServCellIndex,

cif-InSchedulingCell                    INTEGER (1..7)

}

},

...

}

-- TAG-CrossCarrierSchedulingConfig-STOP

-- ASN1STOP

CrossCarrierSchedulingConfig field descriptions

cif-Presence The field is used to indicate whether carrier indicator field is present (value true) or not (value false) in PDCCH DCI formats, see TS 38.213 [13].

cif-InSchedulingCell The field indicates the CIF value used in the scheduling cell to indicate a grant or assignment applicable for this cell, see TS 38.213 [13]. If cif-Presence is set to true, the CIF value indicating a grant or assignment for this cell is 0.

other Parameters for cross-carrier scheduling, i.e., a serving cell is scheduled by a PDCCH on another (scheduling) cell. The network configures this field only for SCells.

own Parameters for self-scheduling, i.e., a serving cell is scheduled by its own PDCCH.

schedulingCellId Indicates which cell signals the downlink allocations and uplink grants, if applicable, for the concerned SCell. In case the UE is configured with DC, the scheduling cell is part of the same cell group (i.e. MCG or SCG) as the scheduled cell.

# Appendix C. search-space configuration

#### –	SearchSpace

The IE SearchSpace defines how/where to search for PDCCH candidates. Each search space is associated with one ControlResourceSet. For a scheduled cell in the case of cross carrier scheduling, except for nrofCandidates, all the optional fields are absent.

SearchSpace information element

-- ASN1START

-- TAG-SEARCHSPACE-START

SearchSpace ::=                         SEQUENCE {

searchSpaceId                           SearchSpaceId,

controlResourceSetId                    ControlResourceSetId                                        OPTIONAL,   -- Cond SetupOnly

monitoringSlotPeriodicityAndOffset      CHOICE {

sl1                                     NULL,

sl2                                     INTEGER (0..1),

sl4                                     INTEGER (0..3),

sl5                                     INTEGER (0..4),

sl8                                     INTEGER (0..7),

sl10                                    INTEGER (0..9),

sl16                                    INTEGER (0..15),

sl20                                    INTEGER (0..19),

sl40                                    INTEGER (0..39),

sl80                                    INTEGER (0..79),

sl160                                   INTEGER (0..159),

sl320                                   INTEGER (0..319),

sl640                                   INTEGER (0..639),

sl1280                                  INTEGER (0..1279),

sl2560                                  INTEGER (0..2559)

}                                                                                                   OPTIONAL,   -- Cond Setup

duration                                INTEGER (2..2559)                                           OPTIONAL,   -- Need R

monitoringSymbolsWithinSlot             BIT STRING (SIZE (14))                                      OPTIONAL,   -- Cond Setup

nrofCandidates                          SEQUENCE {

aggregationLevel1                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel2                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel4                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel8                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel16                      ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}

}                                                                                                   OPTIONAL,   -- Cond Setup

searchSpaceType                         CHOICE {

common                                  SEQUENCE {

dci-Format0-0-AndFormat1-0              SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-0                           SEQUENCE {

nrofCandidates-SFI                      SEQUENCE {

aggregationLevel1                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16                      ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-1                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-2                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-3                           SEQUENCE {

dummy1                  ENUMERATED {sl1, sl2, sl4, sl5, sl8, sl10, sl16, sl20}          OPTIONAL,   -- Cond Setup

dummy2                  ENUMERATED {n1, n2},

...

}                                                                                           OPTIONAL    -- Need R

},

ue-Specific                             SEQUENCE {

dci-Formats                             ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1},

...

}

}                                                                                                   OPTIONAL    -- Cond Setup

}

-- TAG-SEARCHSPACE-STOP

-- ASN1STOP

SearchSpace field descriptions

common Configures this search space as common search space (CSS) and DCI formats to monitor.

controlResourceSetId The CORESET applicable for this SearchSpace. Value 0 identifies the common CORESET#0 configured in MIB and in ServingCellConfigCommon. Values 1..maxNrofControlResourceSets-1 identify CORESETs configured in System Information or by dedicated signalling. The CORESETs with non-zero controResourceSetId locate in the same BWP as this SearchSpace.

dummy1, dummy2 This field is not used in the specification. If received it shall be ignored by the UE.

dci-Format0-0-AndFormat1-0 If configured, the UE monitors the DCI formats 0_0 and 1_0 according to TS 38.213 [13], clause 10.1.

dci-Format2-0 If configured, UE monitors the DCI format 2_0 according to TS 38.213 [13], clause 10.1, 11.1.1.

dci-Format2-1 If configured, UE monitors the DCI format 2_1 according to TS 38.213 [13], clause 10.1, 11.2.

dci-Format2-2 If configured, UE monitors the DCI format 2_2 according to TS 38.213 [13], clause 10.1, 11.3.

dci-Format2-3 If configured, UE monitors the DCI format 2_3 according to TS 38.213 [13], clause 10.1, 11.4

dci-Formats Indicates whether the UE monitors in this USS for DCI formats 0-0 and 1-0 or for formats 0-1 and 1-1.

duration Number of consecutive slots that a SearchSpace lasts in every occasion, i.e., upon every period as given in the periodicityAndOffset. If the field is absent, the UE applies the value 1 slot, except for DCI format 2_0. The UE ignores this field for DCI format 2_0. The maximum valid duration is periodicity-1 (periodicity as given in the monitoringSlotPeriodicityAndOffset).

monitoringSlotPeriodicityAndOffset Slots for PDCCH Monitoring configured as periodicity and offset. If the UE is configured to monitor DCI format 2_1, only the values 'sl1', 'sl2' or 'sl4' are applicable. If the UE is configured to monitor DCI format 2_0, only the values ′sl1′, ′sl2′, ′sl4′, ′sl5′, ′sl8′, ′sl10′, ′sl16′, and ′sl20′ are applicable (see TS 38.213 [13], clause 10).

monitoringSymbolsWithinSlot The first symbol(s) for PDCCH monitoring in the slots configured for PDCCH monitoring (see monitoringSlotPeriodicityAndOffset and duration). The most significant (left) bit represents the first OFDM in a slot, and the second most significant (left) bit represents the second OFDM symbol in a slot and so on. The bit(s) set to one identify the first OFDM symbol(s) of the control resource set within a slot. If the cyclic prefix of the BWP is set to extended CP, the last two bits within the bit string shall be ignored by the UE . For DCI format 2_0, the first one symbol applies if the duration of CORESET (in the IE ControlResourceSet) identified by controlResourceSetId indicates 3 symbols, the first two symbols apply if the duration of CORESET identified by controlResourceSetId indicates 2 symbols, and the first three symbols apply if the duration of CORESET identified by controlResourceSetId indicates 1 symbol. See TS 38.213 [13], clause 10.

nrofCandidates-SFI The number of PDCCH candidates specifically for format 2-0 for the configured aggregation level. If an aggregation level is absent, the UE does not search for any candidates with that aggregation level. The network configures only one aggregationLevel and the corresponding number of candidates (see TS 38.213 [13], clause 11.1.1).

nrofCandidates Number of PDCCH candidates per aggregation level. The number of candidates and aggregation levels configured here applies to all formats unless a particular value is specified or a format-specific value is provided (see inside searchSpaceType). If configured in the SearchSpace of a cross carrier scheduled cell, this field determines the number of candidates and aggregation levels to be used on the linked scheduling cell (see TS 38.213 [13], clause 10).

searchSpaceId Identity of the search space. SearchSpaceId = 0 identifies the searchSpaceZero configured via PBCH (MIB) or ServingCellConfigCommon and may hence not be used in the SearchSpace IE. The searchSpaceId is unique among the BWPs of a Serving Cell. In case of cross carrier scheduling, search spaces with the same searchSpaceId in scheduled cell and scheduling cell are linked to each other. The UE applies the search space for the scheduled cell only if the DL BWPs in which the linked search spaces are configured in scheduling cell and scheduled cell are both active.

searchSpaceType Indicates whether this is a common search space (present) or a UE specific search space as well as DCI formats to monitor for.

ue-Specific Configures this search space as UE specific search space (USS). The UE monitors the DCI format with CRC scrambled by C-RNTI, CS-RNTI (if configured), and SP-CSI-RNTI (if configured)

| Conditional Presence | Explanation |
| --- | --- |
| Setup | This field is mandatory present upon creation of a new SearchSpace. It is optionally present, Need M, otherwise. |
| SetupOnly | This field is mandatory present upon creation of a new SearchSpace. It is absent otherwise. |

# Appendix D. CSI-ResourceConfig

#### –	CSI-ResourceConfig

The IE CSI-ResourceConfig defines a group of one or more NZP-CSI-RS-ResourceSet, CSI-IM-ResourceSet and/or CSI-SSB-ResourceSet.

CSI-ResourceConfig information element

-- ASN1START

-- TAG-CSI-RESOURCECONFIG-START

CSI-ResourceConfig ::=      SEQUENCE {

csi-ResourceConfigId        CSI-ResourceConfigId,

csi-RS-ResourceSetList      CHOICE {

nzp-CSI-RS-SSB              SEQUENCE {

nzp-CSI-RS-ResourceSetList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need R

csi-SSB-ResourceSetList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId

OPTIONAL  -- Need R

},

csi-IM-ResourceSetList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId

},

bwp-Id                      BWP-Id,

resourceType                ENUMERATED { aperiodic, semiPersistent, periodic },

...

}

-- TAG-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

CSI-ResourceConfig field descriptions

bwp-Id The DL BWP which the CSI-RS associated with this CSI-ResourceConfig are located in (see TS 38.214 [19], clause 5.2.1.2.

csi-ResourceConfigId Used in CSI-ReportConfig to refer to an instance of CSI-ResourceConfig.

csi-RS-ResourceSetList Contains up to maxNrofNZP-CSI-RS-ResourceSetsPerConfig resource sets if resourceType is 'aperiodic' and 1 otherwise (see TS 38.214 [19], clause 5.2.1.2).

csi-SSB-ResourceSetList List of SSB resources used for beam measurement and reporting in a resource set (see TS 38.214 [19], section 5.2.1.2).

resourceType Time domain behavior of resource configuration (see TS 38.214 [19], clause 5.2.1.2). It does not apply to resources provided in the csi-SSB-ResourceSetList.

# Appendix E. UE power consumption model for FR1

TR 38.840-g00

Table 4 UE power consumption model for FR1

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | 1 (Optional: 0.5) |
| Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 |
| Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. | 300 |
| UL | Long PUCCH or PUSCH. | 250 (0 dBm) 700 (23 dBm) |