3GPP TSG RAN WG1 Meeting #98	R1-1908100

Prague, Czech Republic, August 26 – 30, 2019

Title:	[draft] Reply LS on NR fast SCell activation

Response to:	R1-1908003(R2-1908483)

Release:	Release 16

Work Item:	LTE_NR_DC_CA_enh-Core

Source:	Huawei, HiSilicon [RAN1]

To:	RAN2, RAN4

Cc:

Contact Person:

#### Name:

E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: -

1. Overall Description:

RAN1 would like to thank RAN2 for the questions about NR fast SCell activation. Based on discussions during RAN1#98, RAN1 would like to provide following answers.

Q 2:  which part of latency can be reduced via the ‘dormancy’ behaviour and by how much?

[RAN1] RAN1 acknowledged the RAN2 agreements on not introducing SCell dormant state like LTE euCA for NR, and is still discussing the details of the ‘dormancy-like’ behaviour, with the understanding that the SCell is already activated. In this case, the latency of SCell activation, which is the delay of a SCell from deactivated to activated defined in RAN4, will not be affected. The latency of a UE from ‘dormancy’ behaviour to normal data transfer behaviour would be affected, and can be reduced. The exact latency from ‘dormancy’ behaviour to normal data transfer behaviour depends on the detailed design of the ‘dormancy’ behaviour and how the behaviour transition is indicated.

Q 3: if the latency can be reduced, is it feasible to support ‘dormancy’ behaviour from RAN1/RAN4 perspective? If it is feasible, what are expected spec impacts from RAN1/RAN4 perspective?

[RAN1] From RAN1 perspective, it is feasible. Though the details are under discussion, RAN1 foresee that BWP configuration and BWP switching operation may be impacted to support the ‘dormancy-like’ behaviour. Related RRC signalling and/or L1 signalling may need to be designed accordingly.

Q 4: which part of latency can be reduced via temporary RS and by how much?

[RAN1]RAN1 understands the latency potentially reduced via temporary RS is referring to the latency reduction of SCell activation delay, rather than the latency for data transfer for which case the SCell is already activated. RAN1 expects RAN4 to provide response for this question. From RAN1 perspective, it also depends on the details of temporary RS, e.g. aperiodic TRS or CSI-RS with even short-interval compared to that in NR Rel-15.

Q 5: if the latency can be reduced, is it feasible to support temporary RS from RAN1/RAN4 perspective? If it is feasible, what are expected spec impacts from RAN1/RAN4 perspective?

[RAN1] From RAN1 perspective, it is feasible. It depends on the details of temporary RS. For example, if CSI-RS with even short-interval compared to that in NR Rel-15 in introduced, RRC signalling for this configuration may be needed.

2. Actions:

To RAN2 and RAN4:

RAN1 kindly asks RAN2 and RAN4 to take the above information into account for defining mechanisms for fast SCell activation.

3. Date of Next TSG-RAN1 Meetings:

RAN1#98bis			14 - 18 Oct 2019, China, CN

RAN #99			18 - 22 Nov 2019, Reno, US