3GPP TSG RAN WG1 Meeting #98	R1-1908106

Prague, Czech Republic, August 26- 30, 2019

Agenda Item:	7.2.2.1.1

Source:	Huawei, HiSilicon

Title:	Initial access signal and channels in NR unlicensed band

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss NR-U DRS design including SSB pattern and the corresponding CORESET 0, RMSI PDSCH rate matching and DRS composition. Furthermore, extensive PRACH preamble evaluations are provided for ZC sequences with/without repetitions.

NR–Unlicensed DRS design

SSB pattern and CORESET 0

LTE LAA Discovery Reference Signal (DRS) consists of CRS, PSS, SSS and optional CSI-RS. Similarly, introduction of NR-U DRS consisting of at least the SSB burst set was identified beneficial in SI phase. In DC and SA mode, the corresponding RMSI is jointly transmitted with SSB as NR-U DRS for purpose of initial access. Besides, CSI-RS could also be configured as part of NR-U DRS for the purpose of RLM/RRM measurement. As demonstrated in Figure 1, two alternatives on the combination of SSB pattern and CORESET 0 are proposed for NR-U [1].

For alternative 1(legacy SSB pattern) with length-2 CORESET 0, it can only support RMSI PDSCH with at most 4 symbols. Compared with alternative 2 which can support RMSI PDSCH of 5 symbols, fewer time/frequency resources are available for RMSI PDSCH. In NR Rel-15, RMSI payload size is around 1700 bits, and limited time/frequency resources available for transmission could lead to severe RMSI decoding performance degradation and incur undesirable access delay. It was argued that 2 remaining symbols at the end of slot could be used for PUCCH transmission or be allocated as RACH Occasions. However, switching transmission direction in DRS transmission is undesirable since it increases the probability of losing channel occupancy. Configuring CSI-RS at the last 2 symbols separated apart from the QCLed SSB is also not preferred as it requires additional beam switching operation. Furthermore, CORESET 0 associating with the 2nd SSB in a slot starts at OS#6 will impede UE to perform half-slot processing. On the other hand, if length-1 CORESET 0 associating with 2nd SSB in a slot is at OS#7, then the relative position among SSB, CORESET 0 and RMSI PDSCH will be different from the 1st SSB within the same slot, which also increases UE’s decoding complexity.

Alternative 2 allows at most 5 or 6 symbols RMSI PDSCH when length-2 or length-1 CORESET 0 is adopted. More RE could be allocated to RMSI, OSI and paging compared with alternative 1. The gNB could also configure 1 symbol CSI-RS occupying the full initial BWP at OS#6 and OS#13, which is assumed to have the same QCL assumption with the corresponding SSB (CSI-RS transmitted at OS#6 corresponding to SSB 0 and CSI-RS transmitted at OS#13 corresponding to SSB 1) and hence no beam switching back and forth is needed. Besides, alternative 2 also supports length-2 CORESET 0 for each SSB even if 2 SSBs are transmitted in one slot. Furthermore, the symmetric structure between two half-slots could facilitate UE implementation. Considering all discussion above, alternative 2 is preferred to be adopted as SSB pattern for NR-U.

Figure 1.  Illustration of NR-U SSB pattern candidates

Proposal 1: Alternative 2 shall be adopted in NR-U as SSB position and Type-0 PDCCH monitoring in a slot.

- Support Type0-PDCCH in symbol (#0, #1) for length-2 CORESET 0 and symbol (#0) for length-1 CORESET 0 for the first SSB in a slot

- Support Type0-PDCCH in symbol (#7, #8) for length-2 CORESET and symbol (#7) for length-1 CORESET 0 for the second SSB in a slot

RMSI PDSCH rate matching

In NR Rel-15, PDSCH scheduled by Type-0 PDCCH should be contiguous in frequency domain and cannot rate-match around SSBs. However, RMSI PDSCH around SSBs shall be considered in NR-U since it helps to increase the capacity of PDSCH for RMSI/OSI/paging without extending DRS block transmission duration. Assuming only PDSCH scheduled by Type-0/Type-0A/Type-2 PDCCH are located in the same slot as the associated SSB, as shown in the Figure 1, UE is able to know the PDSCH rate-matching  around SSB without additional signaling since time/frequency location of the associated SSB is known at UE side. In this case, contiguous resource allocation scheme in frequency domain can be re-used.  If alternative 2 of SSB position and Type-0 PDCCH monitoring discussed in previous section is adopted as NR-U SSB pattern, at least Type-B PDSCH mapping with 5/6 symbols shall be supported if Type-0 PDCCH and RMSI PDSCH is transmitted in 2nd half slot.. As such, more valid S and L combinations for PDSCH mapping type B shall be introduced to Table 5.1.2.1-1 in [2] .

Three multiplex pattern between SSB and RMSI PDSCH are given in Figure 2.

Pattern 1: RMSI PDSCH rate matching is not allowed, SSB is placed at the center of the initial BWP

Pattern 2: RMSI PDSCH rate matching is not allowed, SSB is placed at the edge of the initial BWP

Pattern 3: RMSI PDSCH rate matching is allowed, SSB could be placed near the edge or the center of the initial BWP

Figure 2.  Illustration of 3 rate-matching pattern candidates using alternative 2

As summarized in Table 1, more REs are available for PDSCH transmission when SSB rate matching is supported. Even compared with pattern 2, as illustrated in Figure 2, up to 51.6% more REs are available using pattern 3 since all REs in the symbol(s) non-overlapped with SSB could be utilized for PDSCH scheduling.

Table 1. Available REs for PDSCH within a DRS block with 30 kHz SCS.

| NR-U SSB pattern | L=4 | L=5 | L=6 | L=12 | L=13 |
| --- | --- | --- | --- | --- | --- |
| Pattern 1 | 672 REs | 840 REs | 1008 REs | 2016 REs | 2184 REs |
| Pattern 2 | 1344 REs (100%) | 1680 REs (100%) | 2016 REs (100%) | 4032 REs (100%) | 4368 REs (100%) |
| Pattern 3 | 1440 REs (114%) | 2016 REs (140%) | 2592 REs (157%) | 6048 REs (200%) | 6624 REs (203%) |

Note: Number in the bracket reflects the percentage of more REs available for PDSCH scheduling using pattern 2 or pattern 3 compared with that of pattern 1 with same PDSCH duration.

Proposal 2: NR-U shall support that RMSI PDSCH scheduled by Type-0 PDCCH is rate-matched around the corresponding SSBs.

If alternative 2 of SSB position and Type-0 PDCCH monitoring is adopted with length-1 CORESET 0, PDSCH DMRS in a DRS burst could be transmitted at OS#1 and OS#8, and RMSI PDSCH could be scheduled as shown in case A of Figure 3. When length-2 CORESET 0 is configured, a combination of RMSI PDSCH DMRS and reference signal in SSB such as PSS/SSS and PBCH DMRS can be employed for purpose of channel estimation since RMSI PDSCH and SSB are multiplexed in frequency domain and occupy the whole initial BWP. In this case, the available resources that could be used for RMSI PDSCH is shown in case B of Figure 3. Otherwise, if RMSI PDSCH DMRS is multiplexed with SSB in frequency domain and channel estimation is solely based on PDSCH DMRS, then the available resources for RMSI PDSCH is fewer compared to the above two cases, which is depicted in case C of Figure 3.

Figure 3.  Illustration of available resources for PDSCH scheduling using alternative 2

Proposal 3: DMRS in both RMSI PDSCH and PBCH can be used to decode RMSI PDSCH when RMSI PDSCH rate matching around SSB is allowed.

DRS composition indication

It was agreed in TR [3] that OSI and paging could be transmitted within NR-U DRS. It is beneficial to reduce the required number of channel access attempts and better fulfill the OCB requirement in NR-U. From another perspective, it is undesirable to always consist of all these channels/signals in NR-U DRS due to limited resources available for the DRS transmission.

In NR, PBCH within SSB can indicate the multiplexing pattern between SSB and RMSI CORESET. When additional signal (i.e. OSI, paging) also be included in DRS, it is intuitive to extend the configuration information in PBCH to carry the indication of inclusion for additional signal and the multiplexing pattern between SSB and those signals multiplexed in DRS.

In NR Rel-15, a set of RBs in symbols of a slot occupied by SSB transmission are not available for PDSCH reception. However, each SSB/DRS transmission in NR-U depends on LBT outcome, which varies from time to time. Therefore, additional dynamic signaling shall be carried in DCI that scheduling PDSCH in order to indicate the actual SSB/DRS transmission time location or transmission time offset introduced by LBT.

Proposal 4: Inclusion of RMSI, OSI and paging in DRS is indicated by PBCH.

In unlicensed spectrum, multiple PLMNs from different operators can share the same channel and coordination between different operators may not happen. This may cause PCI collisions or confusion. Additional information carried in RMSI for both Pcell and Scell, e.g. CGI, could help the gNBs to solve this problem. Furthermore, the probability of occurrence of the above issue depends on the number of NR-U cells of other operators a UE can find in its cell coverage and the number of available channels, which could be very low with limited operators in sparse deployment scenario. Instead of transmitting RMSI, PCI collision/confusion may be solved by other means to reduce the corresponding overhead. For example, a new short identification may be introduced to help UE identifying the cell, which can be included in PBCH. Alternatively, for Scell a UE can switch to Pcell based on kssb carried in PBCH of Scell and report collided PCI and the corresponding channel index of Scell to the gNB. With this information, the gNB can update PCI used in Scell to avoid PCI collision.

Proposal 5: Considering the required resources and its functionality, RMSI transmission in Scell is unnecessary.

NR-Unlicensed PRACH Preamble

Numerical evaluations

In this section, we evaluate and compare the agreed alternatives for NR-U PRACH preamble. Table A-1 in Appendix lists all the evaluated PRACH schemes. We follow the simulation assumptions agreed in [4][5]. To reduce the Cubic Metric (CM) of PRACH schemes with repetition, we modulate each repetition of the ZC sequence with a phase rotation as detailed in Table A-1, where the phase rotation values are cited from [6]. The power backoff is computed as the CM value x measured at PrCM≤x=0.95, where CM=(RCM-1.52)/1.56 and RCM is the raw CM calculated with 2048-point FFT. The results are shown in Figures A-1 to A-3 of Appendix. In particular, the results for 30 kHz SCS are summarized in Table 2 below, and the following observations are made:

Observation 1: Compared to a single ZC sequence of similar length, a repeated length-139 ZC sequence performs worse, and the degradation increases with more repetitions. For 30 kHz SCS:

- The mis-detection probability is ~0.06-0.28 dB worse

- The 95-percentile CM is ~0.20-0.33 dB larger

- The MCL is ~0.11-0.37 dB lower

- The PRACH capacity is ~61-81% smaller

Table 2. Performance of different PRACH schemes with 30 kHz SCS.

| Scheme | NZC | R | SCS*NZC*R (MHz) | Np (dBm) | SNRFD (dB) | Pmax (dBm) | 95% CM (dB) | PTx (dBm) | MCL (dB) | NFDM | PRACH capacity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ZC139 | 139 | 1 | 4.17 | -102.8 | -4.05 | 16.12 | 2.3328 | 16.12 | 122.59 | 4 | 6072 (92 cells) |
| ZC139x2 | 139 | 2 | 8.34 | -99.8 | -7.58 | 19.13 | 2.6561 | 19.13 | 126.07 | 2 | 3036 (46 cells) |
| ZC283 | 283 | 1 | 8.49 | -99.7 | -7.64 | 19.20 | 2.3294 | 19.20 | 126.18 | 2 | 7896 (112 cells) |
| ZC139x4 | 139 | 4 | 16.68 | -96.8 | -10.90 | 22.14 | 2.5305 | 20.87 | 128.14 | 1 | 1518 (23 cells) |
| ZC571 | 571 | 1 | 17.13 | -96.7 | -11.18 | 22.25 | 2.3330 | 21.03 | 128.51 | 1 | 7980 (114 cells) |

It is noted that in Table 2, the 95% CM values are calculated over all the PRACH preambles generated by one scheme, where the number of PRACH preambles generated by repeated length-139 ZC sequences is much less than that generated by a single ZC sequence of similar length. Were the PRACH capacity supported by repeated ZC sequence regarded to be sufficient, one can simply select a subset of preambles for use with the single ZC sequence of similar length. In this case, the 95% CM values of the latter can be much further reduced as shown in Table A-2 in Appendix, which in turn leads to even higher MCL values.

Table 2 shows the unacceptable consequence that a repeated length-139 sequence results in smaller PRACH capacity than in NR Rel-15!

Observation 2: Compared to the NR Rel-15 length-139 ZC sequence without repetition, a repeated length-139 ZC sequence results in ~50-75% smaller PRACH capacity for 30 kHz SCS.

The results for 15 kHz SCS are summarized in Table A-3 in Appendix. Basically, similar trends as those for 30 kHz SCS can be observed.

Observation 3: For 15 kHz SCS, a length-139 ZC sequence with 8 repetitions results in:

- 0.62 dB lower MCL and 93.2% lower PRACH capacity than a length-1151 ZC sequence

- 87.5% lower PRACH capacity than a length-139 ZC sequence

Regarding the timing estimation error, it can be seen from Figures A-1(c) and A-2(c) that a repeated ZC sequence and a single long ZC sequence of similar length have almost the same performance, both having less timing error than the NR Rel-15 length-139 ZC sequence due to increased frequency resource occupation.

Issues of repeated ZC sequence

- PAPR/CM reduction

For the repeated length-139 ZC sequence, it has been proposed to apply different root indices/cyclic shifts/phase rotations to different repetitions so as to improve the CM of the generated PRACH preambles. However, that is an optimization which is very challenging, as so far there is no analytical approach to obtain the optimal combination of root indices/cyclic shifts/phase rotations for different repetitions that achieves the minimum CM. In addition, the optimized values of the root indices/cyclic shifts/phase rotations of different repetitions need to be specified. So far, the optimized CM value of a repeated ZC sequence is still larger than that of a single ZC sequence with similar length, which, plus the slightly worse mis-detection performance and slightly lower maximum transmission power level (i.e., Pmax in Table 2), leads to a lower MCL value than that for the corresponding single long ZC sequence.

Observation 4: The MCL of a repeated length-139 ZC sequence will be worse than that of a single long ZC sequence of similar length, even if some method is used to reduce its CM.

- PRACH capacity

The number of cells (or PRACH occasions) with 64 preambles on a given time-frequency resource is determined by

Ncell=NZC-164NZC/NCS∙NFDM                                                            (1)

where the number of frequency resources NFDM is given in Table 2 and Table A-3 for a given ZC sequence length NZC. In Figure 4, we use (1) to plot Ncell where the cyclic shift values  NCS are taken from Table 4 in Section 3.4.2 and where we use equation (2) in Section 3.4 to compute the maximum supportable cell radius from NCS. The results show that a length-139 ZC sequence which is repeated 2 or 4 times (i.e., the preamble length is LRA=2NZC or LRA=4NZC) has a PRACH capacity loss of 50% and 75%, respectively, over all cell radii compared to that of NR Rel-15. On the other hand, the longer sequences (i.e., LRA=NZC) with NZC=283 and  NZC=571 offer a capacity gain compared to NR Rel-15. Unlicensed networks may not be planned among operators, hence cells in vicinity of each other could end up using the same PRACH preambles. This undesirable situation would occur more often for the sequence repetition schemes since their low PRACH capacity translates into a smaller reuse distance of the preambles.

Figure 4. Number of supported cells with 64 preambles as function of cell radius for single ZC sequence of length LRA=139, 283 and 571, compared to 2x and 4x repeated length-139 ZC sequences. The subcarrier spacing is ∆f=30 kHz, the maximum channel delay is τ=0.87 µs and the number of guard samples is ng=2. The NR Rel-15 PRACH capacity is shown by the blue curve, LRA=139.

Cyclic shift dimensioning and root index ordering

In Sections 3.4 and 3.5, we show that it is straightforward to define the cyclic shifts and the root index ordering for new ZC sequence lengths with the same methods being used in NR Rel-15. A repeated ZC sequence is a form of interlaced waveform, whose auto-/cross correlation functions are no longer ideal, especially when there are gaps between repetitions. Also, it is unclear how to order root indices if the preamble consists of multiple ZC sequences in the frequency domain. Hence new methods are required for specifying the cyclic shifts and the root index ordering. Such methods may also depend on how/whether additional CM reduction (e.g., different root indices/cyclic shifts/phase rotations for different repetitions) is introduced.

Observation 5: New methods would be required for specifying the cyclic shifts and the root index ordering of repeated ZC sequences.

Preamble implementation/detection complexity

A repeated ZC sequence is a new waveform, while a single long ZC sequence is not new since the Rel-15 NR UE already supports ZC sequences with very many lengths for the PRACH, DMRS and SRS, see Table A-4 in Appendix. The candidate lengths 283, 571 and 1151 for the single long ZC sequence are all contained in Table A-4, thus there is no additional UE complexity of adopting any of these lengths.

Observation 6: Repeated ZC sequences are a new waveform while a Rel-15 NR UE is already capable of transmitting a single ZC sequence with one of 99 different sequence lengths ranging from 31 to 1259 in a 20 MHz bandwidth.

In addition, a single long ZC sequence also brings a benefit of reduced gNB detection complexity in the following two aspects

- The detection of PRACH preambles generated by a common ZC root index with different cyclic shifts can share a common power delay profile (PDP) obtained by a single matched filter, where different cyclic shifts correspond to different timing detection window of the PDP. This leads to a reduction of detection complexity by a factor of 1/NZCNCS;

- The detection of PRACH preambles generated by paired root indices (i.e., root r and NZC - r) can share the multiplication operations involved in the matched filter, thanks to the symmetry property of them [7], which further brings a reduction of detection complexity by a factor of 1/2.

As a comparison, the PRACH waveform of the repeated ZC sequence is essentially a new design. So far it is unclear if different PRACH preambles can share certain detection operations, or if the corresponding root indices can be paired for receiver complexity reduction, especially when different root indices/cyclic shifts/phase rotations are adopted to different repetitions. Hence principally we can conclude that a single long ZC sequence has a lower detection complexity than a repeated length-139 ZC sequence.

Summary of evaluation

- The study above is summarized in this observation.

- Observation 7: A repeated length-139 ZC sequence:

- performs worse than a single ZC sequence of similar length in terms of CM, MCL and PRACH capacity

- results in worse PRACH capacity than what NR Rel-15 supports

- needs deployment and specification of sequence manipulations for CM reduction

- needs a new method for specifying the cyclic shifts

- needs a new method for specifying the root index ordering

Therefore, there is no motivation, either from performance point of view, or in terms of specification impact, to adopt a repeated ZC sequence. Regarding the choice between the lengths 571/1151 and 283/571, the results show that larger lengths have ~3 dB higher MCL. The number of PRACH occasions in frequency will be half of that for a longer sequence but overall, due to the longer sequence, there is a net gain in PRACH capacity, as shown in Figure 4. Hence there is no advantage from a sequence that span only 10 MHz, i.e., half of the available 20 MHz.

The WI states that 60 kHz SCS will be supported for control and data channels. A PRACH with 60 kHz SCS is anticipated to be useful for low-latency access and for single-numerology configuration. From the results in Figure A-3 and Table A-3, we have not observed any particular technical prohibitive issue which makes 60 kHz SCS PRACH performance problematic. The existing length-139 ZC sequence could be reused without any further optimizations. Whether 60 kHz SCS for PRACH could be optional or not is a future discussion. Hence, the following proposal is made.

Proposal 6: The NR-U PRACH is based on a single long ZC sequence with the following lengths:

- For 15 kHz SCS: LRA=1151

- For 30 kHz SCS: LRA=571

- For 60 kHz SCS: LRA=139

Cyclic shift dimensioning for ZC sequences of length 283, 571 and 1151

With ZC sequences of new lengths adopted to NR-U PRACH, determining their cyclic shifts is trivial and the same method which was applied for LTE and NR could be reused. Consider a ZC sequence of length NZC used at a given cell radius Rcell, then for orthogonal multiplexing, the cyclic shift, NCS, needs to fulfill

NCS≥2∙Rcellc+τ∙NZC∙∆f+ng	                                                     (2)

where ∙ is the ceiling operator, τ is the maximum channel delay, c is the speed of light, ∆f is the SCS and ng is the number of guard samples to account for TX/RX filtering. The cyclic shifts are limited to 16 values and is signaled by the RRC parameter zeroCorrelationZoneConfig. The number of preamble sequences per ZC root sequence is given by

N=NZCNCS                                                                             (3)

where ∙ is the floor operator. Since NCS is limited to a set of 16 values, N can be smaller for a given cell radius Rcell, than if any integer would have been possible for NCS (i.e., a smaller NCS would have been feasible). This implies that more root sequences will be used in the cell, which results in non-orthogonal preambles. Therefore, the set of NCS values should be chosen so that the number of preambles is as close as possible to what would have been feasible if there were no restrictions on the value of NCS. It has been mathematically proven in [8] that the optimal solution to this problem is

NCSk=NZCNSPR0∙ak+a1-a(ak-1),       k=0,1,…,140,                                                                 k=15,                                       (4)

where ∙ denotes rounding to the closest integer and a is the solution to

NSPRk=NSPR0∙ak+a1-aak-1.                                                                  (5)

More precisely, the cyclic shifts are chosen such that for any cell radius that is located between the ones supported by NCSk and NCSk+1, respectively, the number of preambles is as close as possible to the number of preambles that could be obtained if there were no restrictions on the value of NCS. The NSPRk is a boundary condition for the number of sequences per root sequence and, obviously, setting a large such value results in a small supported cell radius Rcell. The parameter a can be obtained numerically from (5) by setting the boundary conditions NSPR0 and NSPR14. Equation (4) was used for determining the NCS values for unrestricted sets in LTE and NR for the sequence length NZC=839, assuming that it should be possible to multiplex 64 preambles on a single root sequence (i.e., NSPR0=64) and that it should be possible to multiplex at least 2 preambles on a single root sequence (i.e., NSPR14=2). These values result in a=0.8561. The value NCS15=0 denotes that no cyclic shift multiplexing is used, which achieves the maximum Rcell provided by the PRACH format. Table A-5 in Appendix contains the maximum Rcell for each NR Rel-15 PRACH format.

Cyclic shift values for the length-139 ZC sequence

The NCS values for the length-139 ZC sequence in NR are listed in Table 3 together with the supported cell radius for different SCS when multiplexing N preambles according to (2), where τ=0.87 µs (i.e., assuming the CDL-C model with 100 ns desired delay spread) and ng=2. The values marked in green were obtained according to (4) from [9] assuming 3-bit signaling (i.e., 8 NCS values) and the values marked in blue were subsequently manually inserted, since it was later agreed to use a 4-bit table (i.e., 16 NCS values). Notably, the number of NCS values is 8 for the length-139 ZC sequence in LTE. Table 1 shows that in several cases, the cell radius is less than 1 m, however, it was understood that PRACH is used for other applications than initial access (e.g., contention-free random access, beam management etc.) wherein the receive timing is already known and for which it was considered useful with very small NCS values for high multiplexing capacity. It should be noted that there are several large NCS values of the length-139 ZC sequence which result in cell radii which are considerably larger than what would be needed for NR-U with SCS 15 kHz and 30 kHz.

Table 3. Cyclic shift values NCS for the length-139 ZC sequence in NR Rel-15 and corresponding maximum cell radii  Rcell [m] assuming τ=0.87 µs maximum channel delay.

| zeroCorrelation ZoneConfig | NCS | Rcell [m] for 15 kHz SCS | Rcell [m] for 30 kHz SCS | Rcell [m] for 60 kHz SCS | Rcell [m] for 120 kHz SCS |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | Maximum supported by the PUCCH format | Maximum supported by the PUCCH format | Maximum supported by the PUCCH format | Maximum supported by the PUCCH format |
| 1 | 2 | <1 | <1 | <1 | <1 |
| 2 | 4 | 13 | <1 | <1 | <1 |
| 3 | 6 | 157 | 13 | <1 | <1 |
| 4 | 8 | 301 | 85 | <1 | <1 |
| 5 | 10 | 445 | 157 | 13 | <1 |
| 6 | 12 | 588 | 229 | 49 | <1 |
| 7 | 13 | 660 | 265 | 67 | <1 |
| 8 | 15 | 804 | 337 | 103 | <1 |
| 9 | 17 | 948 | 409 | 139 | 4 |
| 10 | 19 | 1092 | 481 | 175 | 22 |
| 11 | 23 | 1380 | 624 | 247 | 58 |
| 12 | 27 | 1668 | 768 | 319 | 94 |
| 13 | 34 | 2171 | 1020 | 445 | 157 |
| 14 | 46 | 3034 | 1452 | 660 | 265 |
| 15 | 69 | 4689 | 2279 | 1074 | 472 |

Cyclic shift values for ZC sequences of length 283, 571 and 1151

We apply (4) and still assume NSPR0=64. However, the maximum cell radius would be excessive for NR-U if NSPR14=2, which is shown in Table A-6 in Appendix. Thereto, a large cell radius is anyway supported for NCS=0, which is shown in Table A-5 in Appendix. A better choice is if we set NSPR14=8, which yields a=0.90162 and the values in Table 4. It can be noted that the supported cell radius (excluding the case of NCS=0) is Rcell≈1100m for a SCS of 15 kHz, which according to Table 3 is comparable to that for the length-139 sequence with 60 kHz SCS and the cell radius is Rcell≈470m for a SCS of 30 kHz, which is comparable to that for the length-139 sequence with 120 kHz SCS. Since 60 kHz and 120 kHz SCS are primarily for small cells, the supported cell radii in Table 4 appear suitable, as they mimic those in Table 3 for 60 kHz and 120 kHz.

Table 4. Cyclic shifts NCS for different ZC sequences lengths NZC and corresponding maximum cell radii Rcell  for 15 kHz or 30 kHz subcarrier spacing, using NSPR0=64 and NSPR14=8 assuming τ=0.87 µs maximum channel delay.

| zeroCorrelation ZoneConfig | NZC=571, Δf=15 kHz |  | NZC=1151, Δf=15 kHz |  | NZC=283, Δf=30 kHz |  | NZC=571, Δf=30 kHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NCS | Rcell [m] | NCS | Rcell [m] | NCS | Rcell [m] | NCS | Rcell [m] |
| 0 | 0 | max | 0 | max | 0 | max | 0 | max |
| 1 | 8 | <1 | 17 | <1 | 4 | <1 | 8 | <1 |
| 2 | 10 | 9 | 20 | 25 | 5 | <1 | 10 | <1 |
| 3 | 11 | 27 | 23 | 51 | 6 | <1 | 11 | <1 |
| 4 | 12 | 44 | 26 | 78 | 7 | <1 | 12 | <1 |
| 5 | 14 | 79 | 29 | 104 | 8 | <1 | 14 | <1 |
| 6 | 16 | 114 | 33 | 138 | 9 | <1 | 16 | <1 |
| 7 | 19 | 167 | 38 | 182 | 10 | 10 | 19 | 18 |
| 8 | 21 | 202 | 44 | 234 | 11 | 28 | 21 | 35 |
| 9 | 24 | 254 | 50 | 286 | 12 | 46 | 24 | 62 |
| 10 | 28 | 324 | 57 | 347 | 14 | 81 | 28 | 97 |
| 11 | 33 | 412 | 67 | 434 | 16 | 116 | 33 | 140 |
| 12 | 40 | 534 | 82 | 564 | 20 | 187 | 40 | 202 |
| 13 | 47 | 657 | 95 | 677 | 23 | 240 | 47 | 263 |
| 14 | 57 | 832 | 115 | 851 | 28 | 328 | 57 | 351 |
| 15 | 71 | 1077 | 143 | 1094 | 35 | 452 | 71 | 473 |

Proposal 7: The following cyclic shifts are used for ZC sequences of length NZC:

- If NZC=283:  NCS∈0,4,5,6,7,8,9,10,11,12,14,16,20,23,28,35

- If NZC=571:  NCS∈0,8,10,11,12,14,16,19,21,24,28,33,40,47,57,71

- If NZC=1151:  NCS∈0,17,20,23,26,29,33, 38,44,50,57,67,82,95,115,143

Root index ordering for ZC sequences of length 283, 571 and 1151

For the length-839 ZC sequence in LTE, the logical root indices are mapped to the sequence roots considering the Cubic Metric (CM). The length-139 ZC sequence was introduced in LTE and there were several proposals on the logical root index mapping [10]-[12] which also considered the CM. However, it was agreed to just follow a complex conjugate pair ordering, since this reduces about 50% detection complexity [7]. Ordering in terms of CM may be even less important for small cells. The same root index ordering was used for the length-139 sequence in NR and it could be straightforwardly applied to any new ZC sequence length, i.e., the mapping is done in pairs of 1, NZC-1,2, NZC-2, 3, NZC-3,…. Equivalently, the sequence root index u could be determined from a logical index i by

ui=yi, i=0,1,…,NZC-2                                                                                                
y2k=k+1, y2k+1=NZC-k-1, k=0,1,…,NZC2                                        (6)

An example of (6) for NZC=283 is contained in Table 5.

Table 5. Logical root index ordering for length-283 ZC sequence.

|  | Sequence number u in increasing order of |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 – 19 | 1 | 282 | 2 | 281 | 3 | 280 | 4 | 279 | 5 | 278 | 6 | 277 | 7 | 276 | 8 | 275 | 9 | 274 | 10 | 273 |
| 20 – 39 | 11 | 272 | 12 | 271 | 13 | 270 | 14 | 269 | 15 | 268 | 16 | 267 | 17 | 266 | 18 | 265 | 19 | 264 | 20 | 263 |
| 40 – 59 | 21 | 262 | 22 | 261 | 23 | 260 | 24 | 259 | 25 | 258 | 26 | 257 | 27 | 256 | 28 | 255 | 29 | 254 | 30 | 253 |
| 60 – 79 | 31 | 252 | 32 | 251 | 33 | 250 | 34 | 249 | 35 | 248 | 36 | 247 | 37 | 246 | 38 | 245 | 39 | 244 | 40 | 243 |
| 80 – 99 | 41 | 242 | 42 | 241 | 43 | 240 | 44 | 239 | 45 | 238 | 46 | 237 | 47 | 236 | 48 | 235 | 49 | 234 | 50 | 233 |
| 100 – 119 | 51 | 232 | 52 | 231 | 53 | 230 | 54 | 229 | 55 | 228 | 56 | 227 | 57 | 226 | 58 | 225 | 59 | 224 | 60 | 223 |
| 120 – 139 | 61 | 222 | 62 | 221 | 63 | 220 | 64 | 219 | 65 | 218 | 66 | 217 | 67 | 216 | 68 | 215 | 69 | 214 | 70 | 213 |
| 140 – 159 | 71 | 212 | 72 | 211 | 73 | 210 | 74 | 209 | 75 | 208 | 76 | 207 | 77 | 206 | 78 | 205 | 79 | 204 | 80 | 203 |
| 160 – 179 | 81 | 202 | 82 | 201 | 83 | 200 | 84 | 199 | 85 | 198 | 86 | 197 | 87 | 196 | 88 | 195 | 89 | 194 | 90 | 193 |
| 180 – 199 | 91 | 192 | 92 | 191 | 93 | 190 | 94 | 189 | 95 | 188 | 96 | 187 | 97 | 186 | 98 | 185 | 99 | 184 | 100 | 183 |
| 200 – 219 | 101 | 182 | 102 | 181 | 103 | 180 | 104 | 179 | 105 | 178 | 106 | 177 | 107 | 176 | 108 | 175 | 109 | 174 | 110 | 173 |
| 220 – 239 | 111 | 172 | 112 | 171 | 113 | 170 | 114 | 169 | 115 | 168 | 116 | 167 | 117 | 166 | 118 | 165 | 119 | 164 | 120 | 163 |
| 240 – 259 | 121 | 162 | 122 | 161 | 123 | 160 | 124 | 159 | 125 | 158 | 126 | 157 | 127 | 156 | 128 | 155 | 129 | 154 | 130 | 153 |
| 260 – 279 | 131 | 152 | 132 | 151 | 133 | 150 | 134 | 149 | 135 | 148 | 136 | 147 | 137 | 146 | 138 | 145 | 139 | 144 | 140 | 143 |
| 280 – 281 | 141 | 142 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |

Proposal 8: The logical root indices i=0,1,2,3,4,5,… are mapped to the sequence root indices as 1, NZC-1,2, NZC-2, 3, NZC-3,….

PRACH formats

The NR PRACH formats were agreed after considerable work and we see no reason to change the basic preamble structure, i.e., a CP followed by repetitions of the sequence (without CP in between) and potentially a Guard Time (GT) at the end. Therefore NR-U PRACH formats based on NR PRACH formats A and B should be the default. In particular there is no reason to change the preamble structure and introduce CPs between the OFDM symbols in order to increase the PRACH capacity, which is an inferior method compared to new sequences. It is known that there are sequences which achieve several orders of magnitude PRACH capacity increase under existing PRACH formats [13]. Introduction of a time period for LBT prior to the preamble in the ROs may need more discussion.

Proposal 9: NR-U PRACH formats are based on NR PRACH formats A and B.

Multiplexing between PRACH and PUSCH/PUCCH

Two cases for multiplexing between PRACH and PUSCH/PUCCH are considered:

- FDM between PRACH and PRB-interlaced PUSCH/PUCCH

This would always have been possible if PRB-interlaced PRACH was adopted. Thus, if the PRACH allocation overlaps with the PUSCH/PUCCH allocation, this type of multiplexing should not be allowed. We see no reason to manipulate and rate match the PUSCH/PUCCH resource allocation to fit around the PRACH. If the PRACH allocation does not overlap with the PUSCH/PUCCH allocation (e.g., the allocations are in different LBT subbands), multiplexing is possible. However, this does not require any particular standardized solution.

- FDM between PRACH and contiguous PUSCH/PUCCH

This could be possible by scheduling PUSCH/PUCCH on resources orthogonal to the PRACH. The potential issue is that, if the allocations are in the same LBT subband, PRACH transmission may be blocked by other UEs’ PUSCH/PUCCH transmissions if the UEs are sufficiently close such that they interfere with each other and the timing advance is larger than the RX-to-TX switch time. The agreed 300 m ISD assumed in the evaluations corresponds to a Round Trip Time (RTT) of 1.2 μs, which is smaller than the expected switch time (e.g., 13 μs and 7 μs for FR1 and FR2, respectively). Hence, the issue will not be severe and the gNB could handle it with implementation specific means.

Proposal 10: No solution with specification impact is adopted for handling multiplexing between PRACH and PUSCH/PUCCH.

Conclusions

In this contribution, we have made the following proposals:

For DRS:

Proposal 1: Alternative 2 shall be adopted in NR-U as SSB position and Type-0 PDCCH monitoring in a slot.

- Support Type0-PDCCH in symbol (#0, #1) for length-2 CORESET 0 and symbol (#0) for length-1 CORESET 0 for the first SSB in a slot

- Support Type0-PDCCH in symbol (#7, #8) for length-2 CORESET and symbol (#7) for length-1 CORESET 0 for the second SSB in a slot

Proposal 2: NR-U shall support that RMSI PDSCH scheduled by Type-0 PDCCH is rate-matched around the corresponding SSBs.

Proposal 3: DMRS in both RMSI PDSCH and PBCH can be used to decode RMSI PDSCH when RMSI PDSCH rate matching around SSB is allowed.

Proposal 4: Inclusion of RMSI, OSI and paging in DRS is indicated by PBCH.

Proposal 5: Considering the required resources and its functionality, RMSI transmission in Scell is unnecessary.

For PRACH:

Proposal 6: The NR-U PRACH is based on a single long ZC sequence with the following lengths:

- For 15 kHz SCS: LRA=1151

- For 30 kHz SCS: LRA=571

- For 60 kHz SCS: LRA=139

Proposal 7: The following cyclic shifts are used for ZC sequences of length NZC:

- If NZC=283:  NCS∈0,4,5,6,7,8,9,10,11,12,14,16,20,23,28,35

- If NZC=571:  NCS∈0,8,10,11,12,14,16,19,21,24,28,33,40,47,57,71

- If NZC=1151:  NCS∈0,17,20,23,26,29,33, 38,44,50,57,67.82.95,115,143

Proposal 8: The logical root indices i=0,1,2,3,4,5,… are mapped to the sequence root indices as 1, NZC-1,2, NZC-2, 3, NZC-3,….

Proposal 9: NR-U PRACH formats are based on NR PRACH formats A and B.

Proposal 10: No solution with specification impact is adopted for handling multiplexing between PRACH and PUSCH/PUCCH.

References

Chairman’s Notes RAN1 Meeting #96b, Xi’an, China, Apr. 8 –Apr. 12, 2019.

TS 38.214, v15.5.0, NR, Physical layer procedures for data, March 2019.

TR 38.889 v16.0.0 Study on NR-based access to unlicensed spectrum, December 2018.

Chairman’s Notes RAN1 Meeting #96, Athens, Greece, Feb. 25 – Mar. 1, 2019.

Chairman’s Notes RAN1 Ad Hoc Meeting, Taipei, Jan. 21-25, 2019.

Intel Corporation, “Initial access signals/channels for NR-unlicensed”, R1-1906782, Reno, Nevada, USA, May. 13 – 17, 2019.

Huawei, “Efficient matched filters for paired root Zadoff-Chu sequences”, R1-071409, St. Julian’s, Malta, March 26-30, 2007.

Huawei, “Multiple values of cyclic shift increment NCS”, R1-072325, Kobe, Japan, May 7 – 11, 2007.

Huawei, HiSilicon, “Cyclic-shift dimensioning and capacity shortfall solution for agreed NR-PRACH formats”, R1-1716686, Nagoya, Japan, Sept. 18 – 21, 2017.

Huawei, “Root Zadoff-Chu sequence order for PRACH format 4 in FS2 TDD”, R1-081029, Sorrento, Italy, Feb. 11 – 15, 2008.

ZTE, Qualcomm Europe, Texas Instruments, “ZC sequence ordering for short RACH”, R1-081069, Sorrento, Italy, Feb. 11 – 15, 2008.

Ericsson, “ZC sequence sorting for RACH preamble format 4”, R1-080895, Sorrento, Italy, Feb. 11 – 15, 2008.

Huawei, HiSilicon, “Capacity shortfall solution for agreed NR PRACH formats”, R1-1719378, Reno, USA, Nov. 27 – Dec. 1, 2017.

- Appendix:

Table A-1 lists all the candidate PRACH schemes for NR-U. Their corresponding ZC sequence lengths NZC, cyclic shift values NCS and frequency resource allocations are detailed in the table. To generate 64 preambles in one cell for each scheme, we take their root indexes in the order of 1, NZC-1, 2, N-2, 3, NZC-3,  and so on, and take their cyclic shift gaps NCS according to the supported cell radius (i.e., 173 m) based on Table 4 in Section 3.4. The cyclically shifted sequences are then repeated by R times (if needed) and mapped to the frequency resources with certain phase rotations according to the RE allocations of one RACH occasion (RO) in the table.

Table A-1. Candidate PRACH schemes for NR-U.

| SCS (kHz) | Scheme | Frequency domain sequence generation |  |  |  | NRB per RO | Bandwidth per RO (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NZC | NCS | R | RE allocation of one RO |  |  |
| 15 | ZC139 | 139 | 8 | 1 | 139 contiguous REs | 12 | 2.085 |
|  | ZC139x4 | 139 | 8 | 4 | 556 contiguous REs with phase rotation [1, 1i, 1i, 1] per repetition | 47 | 8.34 |
|  | ZC571 | 571 | 21 | 1 | 571 contiguous REs | 48 | 8.565 |
|  | ZC139x8 | 139 | 8 | 8 | 1112 contiguous REs with phase rotation [1, 1i, 1i, -1, -1, 1i, 1i, 1] per repetition | 93 | 16.68 |
|  | ZC1151 | 1151 | 38 | 1 | 1151 contiguous REs | 96 | 17.265 |
| 30 | ZC139 | 139 | 12 | 1 | 139 contiguous REs | 12 | 4.17 |
|  | ZC139x2 | 139 | 12 | 2 | 278 contiguous REs with phase rotation [1, 1] per repetition* | 24 | 8.34 |
|  | ZC283 | 283 | 20 | 1 | 283 contiguous REs | 24 | 8.49 |
|  | ZC139x4 | 139 | 12 | 4 | 556 contiguous REs with phase rotation [1, 1i, 1i, 1] per repetition | 47 | 16.68 |
|  | ZC571 | 571 | 40 | 1 | 571 contiguous REs | 48 | 17.13 |
| 60 | ZC139 | 139 | 19 | 1 | 139 contiguous REs | 12 | 8.34 |

*Since [6] doesn’t provide the optimized phase rotation values for the scheme of ZC139x2, we numerically set them at [1, 1], which achieve the minimum 95% CM value among options [1, 1], [1, 1i], [1, -1] and [1, -1i].

Figures A-1 to A-3 show the performance of all the candidate PRACH schemes listed in Table A-1.

(a) 												(b)

(c)

Figure A-1. Performance comparison of PRACH schemes with 15 kHz SCS.

(a) 												(b)

(c)

Figure A-2. Performance comparison of PRACH schemes with 30 kHz SCS.

(a) 												(b)

(c)

Figure A-3. Performance of PRACH scheme with 60 kHz SCS.

Table A-2 lists the 95% CM values of different PRACH schemes in Table A-1 when all or a subset of the generated preambles are considered. For fair comparison, the preamble subset of a single long ZC sequence is selected to either contain a similar number of PRACH preambles, or support the same number of cells, as the corresponding repeated ZC sequence.

Table A-2. The 95% CM values of different PRACH schemes with all or a subset of generated preambles.

| SCS (kHz) | Scheme | Number of Preambles considered | 95% RCM | 95% CM |
| --- | --- | --- | --- | --- |
| 15 | ZC139 | 18768 | 5.1592 | 2.3328 |
|  | ZC139x4 | 4692 (68 cells) | 5.4685 | 2.5311 |
|  | ZC571 | 30780 | 5.1595 | 2.3330 |
|  |  | 4698 | 1.8222 | 0.1937 |
|  |  | 5508 (68 cells) | 2.0170 | 0.3186 |
|  | ZC139x8 | 2346 (34 cells) | 5.6466 | 2.6453 |
|  | ZC1151 | 34500 | 5.1581 | 2.3321 |
|  |  | 2370 | 1.2717 | -0.1591 |
|  |  | 3060 (34 cells) | 1.4466 | -0.0471 |
| 30 | ZC139 | 6072 | 5.1592 | 2.3328 |
|  | ZC139x2 | 3036 (46 cells) | 5.6636 | 2.6561 |
|  | ZC283 | 7896 | 5.1538 | 2.3294 |
|  |  | 3052 | 2.5490 | 0.6596 |
|  |  | 3220 (46 cells) | 2.5823 | 0.6809 |
|  | ZC139x4 | 1518 (23 cells) | 5.4676 | 2.5305 |
|  | ZC571 | 7980 | 5.1595 | 2.3330 |
|  |  | 1526 | 2.0316 | 0.3279 |
|  |  | 1610 (23 cells) | 2.0710 | 0.3532 |
| 60 | ZC139 | 996 | 5.1782 | 2.3450 |

The performance of PRACH schemes with 30 kHz SCS has been summarized in Table 2 in Section 3.1, and that of PRACH schemes with 15 kHz and 60 kHz SCSs is summarized in Table A-3 below.

Table A-3. Performance of different PRACH schemes with 15 and 60 kHz SCS.

| SCS (kHz) | Scheme | LRA | R | SCS*LRA*R (MHz) | Np (dBm) | SNRFD (dB) | Pmax (dBm) | 95% CM (dB) | PTx (dBm) | MCL (dB) | NFDM | PRACH capacity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | ZC139 | 139 | 1 | 2.085 | -105.8 | -3.64 | 13.17 | 2.3328 | 13.17 | 122.25 | 8 | 18768 (272 cells) |
|  | ZC139x4 | 139 | 4 | 8.34 | -99.8 | -10.54 | 19.19 | 2.5311 | 19.19 | 129.12 | 2 | 4692 (68 cells) |
|  | ZC571 | 571 | 1 | 8.565 | -99.7 | -10.74 | 19.31 | 2.3330 | 19.31 | 129.35 | 2 | 30780 (380 cells) |
|  | ZC139x8 | 139 | 8 | 16.68 | -96.8 | -13.91 | 22.20 | 2.6453 | 20.77 | 131.05 | 1 | 2346 (34 cells) |
|  | ZC1151 | 1151 | 1 | 17.265 | -96.6 | -14.38 | 22.35 | 2.3321 | 21.03 | 131.67 | 1 | 34500 (383 cells) |
| 60 | ZC139 | 139 | 1 | 8.34 | -99.8 | -4.50 | 19.13 | 2.3450 | 19.13 | 123.04 | 2 | 1932 (26 cells) |

Table A-4 lists the lengths of ZC sequences that are supported in Rel-15 NR at UE for PRACH, DMRS and the SRS.

Table A-4. Supported ZC sequence lengths for a Rel-15 NR UE for a 20 MHz bandwidth.

| NZC | NZC | NZC | NZC | NZC | NZC | NZC | NZC | NZC | NZC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31 | 151 | 271 | 389 | 509 | 647 | 761 | 919 | 1039 | 1163 |
| 47 | 167 | 283 | 401 | 523 | 659 | 773 | 929 | 1051 | 1171 |
| 59 | 179 | 293 | 419 | 547 | 661 | 787 | 947 | 1063 | 1187 |
| 71 | 191 | 311 | 431 | 563 | 683 | 797 | 953 | 1069 | 1193 |
| 83 | 199 | 317 | 443 | 571 | 691 | 811 | 971 | 1091 | 1201 |
| 89 | 211 | 331 | 449 | 587 | 701 | 827 | 983 | 1103 | 1223 |
| 107 | 227 | 347 | 467 | 599 | 719 | 839 | 991 | 1109 | 1231 |
| 113 | 239 | 359 | 479 | 607 | 727 | 863 | 997 | 1123 | 1237 |
| 131 | 251 | 367 | 491 | 619 | 743 | 887 | 1019 | 1129 | 1259 |
| 139 | 263 | 383 | 503 | 631 | 751 | 911 | 1031 | 1151 | - |

# Table A-5 lists the maximum cell radius for each NR Rel-15 PRACH format.

Table A-5. Maximum cell radii Rcell [m] for the NR Rel-15 PRACH formats for different subcarrier spacing assuming τ=0.87 µs maximum channel delay.

| Format | Δf=15 kHz | Δf=30 kHz | Δf=60 kHz | Δf=120 kHz |
| --- | --- | --- | --- | --- |
| A1 | 1276 | 573 | 221 | 45 |
| A2 | 2682 | 1276 | 573 | 221 |
| A3 | 4088 | 1979 | 924 | 397 |
| B1 | 924 | 397 | 133 | 1 |
| B2 | 1627 | 748 | 309 | 89 |
| B3 | 2330 | 1100 | 485 | 177 |
| B4 | 4440 | 2155 | 1012 | 441 |
| C0 | 5924 | 2897 | 1383 | 626 |
| C2 | 9870 | 4870 | 2370 | 1120 |

Table A-6 lists the cyclic shifts NCS for different ZC sequence lengths NZC and corresponding maximum cell radii for 15 kHz or 30 kHz subcarrier spacing, using NSPR0=64 and NSPR14=2 assuming τ=0.87 µs maximum channel delay.

Table A-6. Cyclic shifts NCS for different ZC sequences lengths NZC and corresponding maximum cell radii Rcell for 15 kHz or 30 kHz SCS, using NSPR0=64 and NSPR14=2 assuming τ=0.87 µs maximum channel delay.

| zeroCorrelation ZoneConfig | NZC=571, Δf=15 kHz |  | NZC=1151, Δf=15 kHz |  | NZC=283, Δf=30 kHz |  | NZC=571, Δf=30 kHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NCS | Rcell [m] | NCS | Rcell [m] | NCS | Rcell [m] | NCS | Rcell [m] |
| 0 | 0 | max | 0 | max | 0 | max | 0 | max |
| 1 | 8 | <1 | 17 | <1 | 4 | <1 | 8 | <1 |
| 2 | 10 | 9 | 21 | 34 | 5 | <1 | 10 | <1 |
| 3 | 12 | 44 | 25 | 69 | 6 | <1 | 12 | <1 |
| 4 | 15 | 97 | 30 | 112 | 7 | <1 | 15 | <1 |
| 5 | 17 | 132 | 35 | 156 | 8 | <1 | 17 | <1 |
| 6 | 21 | 202 | 44 | 234 | 10 | 10 | 21 | 35 |
| 7 | 25 | 272 | 52 | 303 | 12 | 46 | 25 | 70 |
| 8 | 31 | 377 | 63 | 399 | 15 | 99 | 31 | 123 |
| 9 | 40 | 534 | 82 | 564 | 20 | 187 | 40 | 202 |
| 10 | 51 | 727 | 104 | 755 | 25 | 275 | 51 | 298 |
| 11 | 63 | 937 | 127 | 955 | 31 | 381 | 63 | 403 |
| 12 | 81 | 1253 | 164 | 1276 | 40 | 540 | 81 | 561 |
| 13 | 114 | 1830 | 230 | 1850 | 56 | 823 | 114 | 850 |
| 14 | 190 | 3161 | 383 | 3179 | 94 | 1494 | 190 | 1515 |
| 15 | 285 | 4825 | 575 | 4847 | 141 | 2325 | 285 | 2347 |