3GPP TSG RAN WG1 Meeting #98	R1-1908107

Prague, Czech Republic, August 26-30, 2019

Agenda Item:	7.2.2.1.2

Source:	Huawei, HiSilicon

Title:	DL channels and signals in NR unlicensed band

Document for:	Discussion and Decision

Introduction

In RAN1#97, the following agreement was achieved [1]

- Additional durations from 2 to 13 (other than 2, 4 and 7 which are already supported in Rel-15) for PDSCH mapping type B are supported.

  - FFS: A duration of 14

  - FFS: DMRS aspects for the additional durations (Reuse Rel-15 patterns or define new patterns for these additional durations)

- When GC-PDCCH is configured, explicit indication via GC-PDCCH is supported as a mechanism to inform the UE that one or more carriers and/or LBT bandwidths are not available or available for DL reception, at least for slot(s) that are not at the beginning of DL transmission burst.

  - FFS: Signalling details of the indication, including e.g., the time domain validity of the indication

  - FFS: Whether and how to support the mechanism at the beginning of DL transmission burst

  - FFS: Whether and how to handle the case when GC-PDCCH is not configured or not received by the UE

The following agreement was agreed in RAN1#96b meeting [2]

- Only further discuss/consider Option 3 (PDSCH mapping type B with durations other than 2/4/7 symbols) as potential enhancements to PDSCH transmissions.

It was agreed in AdHoc#1901 meeting [3] that

- The UE may assume the presence of a signal, such as the DMRS in any [PDCCH or GC-PDCCH] transmission, to detect transmission bursts by the serving gNB, to enable power saving by not necessitating performing blind decodes to detect the transmission burst (Note: The power saving possibility by not necessitating blind decodes assumes performance relaxation for PDCCH decoding is not needed. Also, this does not mandate a two-step PDCCH decoding process for the UE with respect to DMRS detection).

  - If a preamble transmitted at the start of a burst is agreed to be specified (this does not preclude usage of preambles by implementation), it may be used in addition to the DMRS of PDCCH/GC-PDCCH or any other signals in the gNB transmission to detect the start of transmission bursts by the serving gNB and potentially for power saving

    - Note: Whether a preamble, if defined, can be used for power saving in all cases depends on the details of the design.

  - Note: Other signals present in the transmission burst may also be used for the purpose of detection of the transmission burst

  - FFS: Potential enhancements to DMRS design to address issues with detection probability

- The payload of a PDCCH and/or GC-PDCCH transmission can contain information regarding COT structure that may be used by the UE for power saving

In this contribution, we mainly discuss enhancement to PDSCH mapping type B, PDCCH transmission and monitoring in time domain, CORESET configuration in wideband operation, and CSI-RS transmission enhancement in unlicensed band. Considerations on the initial signal design and indication of COT structure are also presented.

Enhancement of PDSCH/PDCCH DMRS

DMRS pattern of PDSCH mapping type B

According to last meeting’s agreement, additional durations from 2 to 13 (other than 2, 4 and 7 which are already supported in Rel-15) for PDSCH mapping type B shall be supported in NR-U. As defined in NR Rel-15, front-loaded DMRS for PDSCH is always occurs at the 1st symbol of scheduled PDSCH resources. Alternatively, if one or multiple symbol(s) at the beginning of scheduled PDSCH resources is/are configured for CORESET, the first DMRS symbol for PDSCH occurs immediately after the CORESET. In NR Rel-15, only DMRS configuration for PDSCH mapping type B with length of 2, 4 and 7 symbols are specified. However, as illustrated in Figure 1, DMRS patterns are specified for PUSCH mapping type B with duration from 3 to 14 symbols. From our perspective, these patterns should be reused for DMRS in PDSCH mapping type B with duration of 3, 5, 6, 8, 11 and 12 symbols.

Figure 1 Time-domain location of DMRS for type B PUSCH in NR Rel-15

Proposal 1: DMRS pattern defined for PUSCH mapping type B could be the baseline as DMRS pattern for PDSCH mapping type B with duration of 3, 5, 6, 8, 11 and 12 symbols.

For 15kHz SCS, it is also discussed in our companion paper [6]. The following factors are considered:

- Actual symbol number of front-loaded (FL) DM-RS

- Maximum number of additional DM-RS

- PDSCH duration in symbols

- Different PDSCH Type B starting symbols

- LTE-CRS positions with different numbers of antenna ports

- Channel estimation accuracy

Therefore, the criteria for determining DM-RS positions for PDSCH mapping Type B of length 9 and 10 OFDM symbols are as follows:

- The lowest probability of collision between LTE-CRS and NR DM-RS, equivalent to highest collision-free probability

- Common design to all possible starting symbols and to all number of LTE-CRS ports

- Common design to both length 9 and 10 PDSCH Type-B, if possible

- DM-RS distribution as uniform as possible

Taking single additional single-symbol DM-RS as an example, Figure 2 and  Figure 3 show two possible DM-RS position (i.e. 2 and 7) with all possible PDSCH Type B starting symbols without collision with LTE-CRS for 2 antenna ports and 4 antenna ports, respectively, where  k is the index in the frequency domain and l refers to the symbol position in the time domain. Within total 6 possible starting symbols for length 9 PDSCH Type B mapping, the starting symbol with l=0 and l=4 in  Figure 2 are precluded due to collision with LTE-CRS of 2 antenna ports while the PDSCH Type B with starting symbol l=0,1, 4 are precluded due to collision with LTE-CRS of 4 antenna ports. Therefore, regarding length 9 PDSCH Type B mapping, only 4 and 3 PDSCH Type B starting symbols are possible for 2 antenna ports and 4 antenna ports LTE-CRS, respectively. Similarly, regarding length 10 PDSCH Type B mapping, only 3 and 2 starting symbols are possible for 2 antenna ports and 4 antenna ports LTE-CRS, respectively. In summary, Table 1 shows the ratio of the number of PDSCH Type B DM-RS position without LTE-CRS collision for all possible PDSCH Type B starting symbols. Taking the first value highlighted in Table 1 as an example, only 2 out of 4 possible PDSCH Type B starting symbols are LTE-CRS collision-free, as also shown in Figure 2. The ratio of available number of DM-RS position to all possible PDSCH Type B starting symbols (i.e. 4) symbols is 2/4=50% for additional DM-RS position l=2. Then the ratio of available number of DM-RS position to all possible PDSCH Type B starting symbols (i.e. 4) symbols is 4/4=100% for additional DM-RS position l=7.

Figure 2 2 Ant ports LTE-CRS and NR PDSCH Type B of length 9 and 10 with one single-symbol additional DM-RS  l=2 and l=7

Figure 3 4 Ant ports LTE-CRS and NR PDSCH Type B of length 9 and 10 with one single-symbol additional DM-RS   l=2 and l=7

Table 1 Ratio of LTE-CRS collision-free with respect to different DM-RS position of PDSCH mapping type B, single additional DM-RS case

| DM-RS position | PDSCH Type B of length 9 symbols |  | PDSCH Type B of length 10 symbols |  |
| --- | --- | --- | --- | --- |
|  | 2 antenna ports LTE-CRS | 4 antenna ports LTE-CRS | 2 antenna ports LTE-CRS | 4 antenna ports LTE-CRS |
| 2 | 2/4=50% | 1/3=33% | 2/3=67% | 1/2=50% |
| 3 | 3/4=75% | 2/3=67% | 2/3=67% | 2/2=100% |
| 4 | 3/4=75% | 2/3=67% | 2/3=67% | 1/2=50% |
| 5 | 3/4=75% | 1/3=33% | 2/3=67% | 0/2=0% |
| 6 | 2/4=50% | 1/3=33% | 2/3=67% | 1/2=50% |
| 7 | 4/4=100% | 3/3=100% | 3/3=100% | 2/2=100% |
| 8 | 3/4=75% | 2/3=67% | 2/3=67% | 1/2=50% |
| 9 | - | - | 2/3=67% | 1/2=50% |

Based on above Table 1, the additional DM-RS position l=7 is the best because it can be applied for all possible starting symbols of length 9 and 10 PDSCH Type B .

Observation 1: The position l=7 is the best for the additional single-symbol DM-RS of PDSCH mapping Type B because it can be applied for all possible starting symbols of PDSCH mapping Type B with both length 9 and 10.

In the cases of two additional DM-RSs, it is preferable that one of two additional DM-RSs reuses the position l=7  that has been applied for single additional DM-RS case. All possible second additional DMR-RS positions have been checked and summarized in Table 2. Because the first 5 symbols in a NR slot with 15 kHz SCS may have been occupied by LTE CRS, LTE and NR PDCCH, PDSCH mapping Type B of length 9 is more attractive and useful than that of length 10. If only the first 4 symbols in a NR slot have been occupied, then PDSCH mapping Type A is more useful than Type B of length 10. Therefore, the pair of DM-RS position l=4,7, as illustrated in Figure 4, is the best which has the highest LTE-CRS collision-free probability for PDSCH mapping Type B of length 9.

| 2 Ant ports LTE-CRS | 4 Ant ports LTE-CRS |
| --- | --- |

Figure 4 LTE-CRS and NR PDSCH Type B of length 9 and 10 with two single-symbol additional DM-RS   l=4,7

Table 2 Ratio of LTE-CRS collision-free with respect to different DM-RS position of PDSCH mapping type B, two additional DM-RS case

| DM-RS position | PDSCH Type B of length 9 OFDM symbols |  | PDSCH Type B of length 10 OFDM symbols |  |
| --- | --- | --- | --- | --- |
|  | 2 antenna ports LTE-CRS | 4 antenna ports LTE-CRS | 2 antenna ports LTE-CRS | 4 antenna ports LTE-CRS |
| 2,7 | 2/4=50% | 1/3=33% | 2/3=67% | 1/2=50% |
| 3,7 | 3/4=75% | 2/3=67% | 2/3=67% | 2/2=100% |
| 4,7 | 3/4=75% | 2/3=67% | 2/3=67% | 1/2=50% |
| 5,7 | 3/4=75% | 1/3=33% | 2/3=67% | 0/2=0% |
| 6,7 | 2/4=50% | 1/3=33% | 2/3=67% | 1/2=50% |
| 7,8 | 3/4=75% | 2/3=67% | 2/3=67% | 1/2=50% |
| 7,9 | - | - | 2/3=67% | 1/2=50% |

Observation 2: In the case of two additional single-symbol DM-RS position, the pair  l=4, 7  has the highest LTE-CRS collision-free probability for all possible PDSCH Type B starting symbols and for length 9 OFDM symbols.

For the efficiency of data transmission, three additional single-symbol DM-RSs and double-symbol DM-RS cases are not considered for PDSCH mapping type B of length 9 and 10 OFDM symbols.

In summary, for PDSCH in 9-symbol or 10-symbol length, one front-loaded DM-RS and one or two additional single-symbol DM-RS can be supported considering DSS with LTE.

Proposal 2: In case that dmrs-AdditionalPosition equal to 'pos1', PDSCH duration ld=9 and ld=10, the PDSCH DM-RS positions l=7 is adopted for additional single-symbol DM-RS for PDSCH mapping type B.

Proposal 3: In case that dmrs-AdditionalPosition equal to 'pos2', PDSCH duration ld=9 and ld=10, the PDSCH DM-RS positions l=4, 7 are adopted for additional single-symbol DM-RS for PDSCH mapping type B.

Proposal 4: The case that dmrs-AdditionalPosition equal to 'pos3' with single-symbol DM-RS and the case with double-symbol DM-RS are not supported in Rel-16 TEI for PDSCH mapping type B with duration ld=9 and ld=10.

Consideration on sequence generation of PDSCH/PDCCH DMRS within a COT

The current sequence generation of PDSCH and PDCCH DMRS in Rel-15 NR depends on the index of the slot and the symbol where the RS is located. As PDCCH DMRS will be used to identify DL burst, the gNB needs to prepare multiple versions or regenerate it in a short time repeatedly after it is contending for channel access, as it does not know when channel access succeeds. For simplicity, it is also possible to generate PDSCH and PDCCH DMRS independent of symbol and slot number for the whole DL burst. For example, we can always set symbol index  and slot index  in the following equation to 0. In such case, the performance of interference whitening might be compromised.

(1)

Alternatively, the DMRS sequence generation can be enhanced by following mechanism

- For the DMRS of PDCCH(s) and PDSCH(s) in the first slot of a DL burst, always set slot and symbol number in sequence generation equation (1) to 0.

- For the PDCCH and PDSCH DMRS(s) in the rest of slot of DL burst, set slot number relative to the start of DL burst.

Once UE correctly decodes the GC-PDCCH and its DMRS, UE can acquire the relative slot number which is used to generate DMRS sequence of other PDCCH and PDSCH in the DL burst. It is also beneficial to transmit additional GC-PDCCHs within a DL burst, in case UE fail to detect at the beginning of DL burst. Considering these UE cannot recover relative slot numbers, the DMRS sequence(s) of GC-PDCCH(s) within the DL burst should also be generated by setting slot and symbol number in equation (1) to 0. The actual slot offset from detected GC-PDCCH to the start of DL burst could be carried in GC-PDCCH, by which UE can recover the relative slot number used to generate DMRS sequences of the following PDCCH(s) and PDSCH(s).

Proposal 5: The slot and symbol number used to generate DMRS sequence(s) of PDSCH(s) and PDCCH(s) in the 1st slot of a DL burst should be set to 0.  The slot numbers used to generate DMRS sequences of PDSCHs and PDCCHs in the following slots of the DL burst are relative to the slot where DL burst starts.

FFS: slot number configuration where additional GC-PDCCH(s) are transmitted within the DL burst.

PDCCH transmission and monitoring in time domain

DL burst identification

According to the agreement in RAN1 Adhoc#1901 meeting, at least DMRS of any [PDCCH or GC-PDCCH] can be used by the UE to detect transmission bursts by the serving gNB. DMRS of GC-PDCCH is more preferred than DMRS of UE specific PDCCH because the initial signal is required by all the UE within the serving cell.  In order to reduce false detection rate (correspondingly improve miss detection performance), GC-PDCCH at beginning of COT could be detected together with its DMRS if the UE is configured to monitoring GC-PDCCH. The flexibility of GC-PDCCH in Type3-PDCCH common search space can be restricted in order to avoid blind decoding. For example, the GC-PDCCH can use fixed aggregation level for a predefined PDCCH candidate in the Type3-PDCCH common search space. The GC-PDCCH and its DMRS at beginning of COT could also provide extra functionalities, such as fine synchronization, phase tracking and spatial reuse and COT structure indication.

The detection performance is also evaluated, as captured in Appendix, showing that GC-PDCCH with the corresponding DMRS provides good detection performance.

Proposal 6: The group common PDCCH together with its DMRS could be used by UE to identify the DL burst from serving cell.

- FFS: mechanism to reduce blind decoding of GC-PDCCH

- FFS: wideband DMRS for GC-PDCCH

GC-PDCCH and its DMRS could be transmitted at the beginning of a gNB’s acquired COT. Considering the UE will miss all resources allocated to it within the COT when missing GC-PDCCH, it is beneficial to transmit the GC-PDCCH multiple times within a COT, e.g. similar mechanism as DCI format 2_0 with CRC scrambled by SFI-RNTI. Besides, DCI content indicating COT structure could be updated since it may indicate the remaining COT information.

Proposal 7: NR-U should support to transmit GC-PDCCH and its DMRS multiple times within a gNB’s acquired COT.

Dynamic adjustment of PDCCH monitoring in NR-U

In NR Rel-15, a UE can be semi-statically configured to monitoring PDCCH within the first 3 OS of a slot or any OS of a slot or both. Different from the use case of URLLC, the UE is only required to monitor PDCCH more frequently at the beginning of the COT. It allows NR-U to start transmission as soon as possible once the gNB passes LBT. Increasing the monitoring period in the remaining COT helps to reduce UE’s energy consumption on PDCCH blind detection. Thus, NR-U UE should support a PDCCH search space configuration which allows the UE dynamically adjust monitoring periodicity depending on the start of DL burst, according to following steps:

- When outside of gNB’s COT, the UE will only perform GC-PDCCH detection according to limited type-3 PDCCH common searching space configuration at every possible starting position periodically in order to save power. The UE identifies start of DL burst from serving cell once the GC-PDCCH is detected.

- After the UE detects start of DL burst in a slot duration until the first full slot boundary, the UE could continue monitoring PDCCH at each possible starting positions in the remaining partial slot. If the UE detects start of DL burst right at the slot boundary, go to step 3.

- From the first full slot until the end of the DL COT, the UE could only detect PDCCH at the first 3 symbols of each slot.

An example is illustrated in Figure 5.

Figure 5 dynamic adjustment of PDCCH monitoring

Proposal 8: NR-U should support a PDCCH search space configuration allowing different PDCCH monitoring occasion configurations before and after the first slot boundary of a COT in terms of UE power consumption.

Indication of COT structure

In TR 38.889, it is identified beneficial to indicate COT structure in addition to the functionalities provided by DCI format 2_0. It can be carried in the GC-PDCCH transmitted at the beginning of COT and within the duration of COT as described in section 3.1. The transmission of COT indication in NR-U should be more flexible than that of DCI format 2_0 in NR Rel-15 due to impact of LBT.

Support flexible starting position

In NR Rel-15, the monitoring occasions of DCI format 2_0 are configured by higher layer signaling. It is restricted within the first 3 OFDM Symbols in a slot with a periodicity in the unit of slot. If a COT starts at the 4th OFDM symbol of a slot (i.e. a partial slot), the UE cannot obtain slot format of the first partial slot. In NR-U, the restriction should be relaxed which allowing UE monitoring DCI Format 2_0 carrying COT structure indication at each possible start point in a slot before the UE identifies the DL burst from serving cell. Then the UE can scale up the monitoring periodicity within the COT in case there is an update on the COT structure broadcasted by the gNB.

Proposal 9: UE could monitor DCI Format 2_0 carrying COT structure indication at each possible starting point in a slot before UE identifies the DL burst from serving cell.

The COT structure indication should be prepared in advance before LBT if it is expected to be transmitted at the beginning of COT considering the processing delay. The COT structure indication reflects slot format from the slot carrying DCI format 2_0 if UE receives the indication at the slot boundary.  The UE could obtain slot format from the 2nd slot when COT structure is received in the middle of a slot. A default slot format could be assumed in the partial starting slot prior to 2nd slot, such as all DL. An example is illustrated in Figure 6.

Figure 6 COT structure indication at beginning of COT

Proposal 10: The COT structure indication reflects slot format from the slot carrying DCI format 2_0 carrying COT structure indication when it is received at slot boundary. UE could obtain slot format from 2nd slot in the COT when it is received in the middle of COT.

Support flexible ending position

SFI carries slot format with a minimum granularity of one slot. The last slot within a COT may also be a partial slot. The SFI corresponds to the last partial slot that across the COT boundary should reflect symbols inside and outside of COT as well as ‘Downlink’, ‘Uplink’ and ‘Flexible’. The following two options could be considered:

Option 1: Indicating a reference symbol S in the last slot of the COT. The SFI corresponding to the last slot is only applicable to the symbols not later than the reference symbol S. The remaining symbols in the slot are outside of COT.

Option 2: Defining a new ‘Out of COT’ state for slot format indication. New entries could be introduced into NR Rel-15 SFI table to indicate state of each symbol in the last slot as ‘downlink’, ‘uplink’, and ‘flexible’ or ‘out of COT’.

Proposal 11: The following two method could be considered for slot format indication for the last partial slot within a COT.

Option 1: Introducing a reference symbols for the ending slot

Option 2: Introducing new entries in the SFI table including a new state indicating ‘out of COT’

Support Pause functionality

In unlicensed band, ‘pause’ state has been introduced in LTE to facilitate the indication of a duration for UL grant. However, no ‘pause’ state can be indicated by DCI format 2_0. Currently, only 3 states for OFDM symbols are included, i.e. ‘D’ for DL reception for UE, ‘U’ for UL transmission for UE, ‘F’ for flexible. However, all of 3 states indicated by SFI has to follow pre-defined UE behavior to solve the conflict configuration between semi-static configured resource and dynamic indicated resource by SFI, and none of them can well support the UE behavior of ‘pause’.  For example, the UE behavior is given by following Table 3.

Table 3: UE behavior for semi-static configuration and dynamic configuration.

(√: follows dynamic configuration by SFI, X: follows semi-static configuration)

|  |  | Semi-static configured |  |  |
| --- | --- | --- | --- | --- |
|  |  | D | U | F |
| Dynamically configured by DCI format 2_0 | D | √ | X | √ |
|  | U | X | √ | √ |
|  | F | X | X | √ |
|  | P | √ | √ | √ |

According to Table 1, if a resource is semi-statically configured as Downlink or Uplink, all existing symbol state (i.e. ‘D’, ‘U’, and ‘F’) indicating by DCI format 2_0 cannot override the semi-static configured resource. As a results, UEs have to follows semi-static configurations, once symbol states are configured by ‘D’ or ‘U’. Therefore, it is preferred to introduce the ‘pause’ state for COT indication. The UE does not expect to receive nor transmit in the set of symbols of the slot indicated as ‘pause’ by SFI-index field value in DCI format 2_0, even if the set of symbols have been configured as ‘D’ or ‘U’ by RRC signaling, as summarized in Table 1.

Proposal 12: NR-U should support indicating “Pause” state within the COT.

COT structure indication in frequency domain

When bandwidth of configured BWP is larger than 20MHz, it is possible that gNB may dynamically transmit on part of BWP either due to LBT failure on one or multiple LBT subband(s) or traffic fluctuation. gNB could also use a specific field in a GC-DCI to indicate which LBT bandwidth(s) in an active wideband BWP are available to receive (called “LBW indication” for short) together with other COT structure indication in time domain. If transmission on non-contiguous LBT bandwidth is supported, a bitmap with each bit corresponding to a LBT bandwidth could be used. If only transmission on contiguous LBT bandwidth(s) is allowed, a compact indication could be used, i.e. RIV-like or an index to a table consisting of all allowed LBT bandwidth combinations. gNB can transmit GC-PDCCH at beginning of a COT (pre-indication) and/or in pre-defined GC-PDCCH monitoring occasion(s) within a COT (post-indication). If gNB cannot update GC-DCI at beginning of a COT immediately after LBT, a predefined value (such as “0”) can be configured in advance which informs UE the LBW indication will be provided later and UE is expected to receive the current DL burst(s) in all LBT bandwidths. gNB could update LBW indication within the COT. gNB will also indicate how long the current LBW indication is valid. Thus, UE is going to detect another LBW indication. . If UE do not receive GC-PDCCH or is not configured to receive GC-PDCCH, UE is expected to receive DL burst(s) in all LBT bandwidths.

Figure 7 time domain validity of LBW indication

Proposal 13: The LBT bandwidth available for reception (LBW indication) can be indicated through a bitmap field in GC-DCI, each bit corresponding to a LBT bandwidth in an active BWP. UE is notified to obtain LBW indication later (or the current value is invalid) when all bit in the bitmap is “zero”.

Proposal 14: The valid time period of the LBT bandwidth available for reception (LBW indication) will be given in GC-PDCCH. UE is expected to detect another GC-PDCCH after that time.

Proposal 15: If GC-PDCCH is transmitted less often than the LBW changes, multiple LBW indications can be included in one GC-PDCCH transmission wherein the LBW indication contains a relative index of the slot at which the LBW update ends.

Proposal 16: UE is expected to receive on all LBT bandwidths if it does not receive GC-PDCCH carrying valid LBW indication.

Support switching from CAT4 to CAT2 UL LBT

In FeLAA, if the ‘UL duration and offset’ field in CPDCCH configures an 'UL offset'  and an 'UL duration'  for subframe , then the scheduled UE may use CAT2 LBT for the transmissions in subframes where i=0,1,…,d-1, irrespective of the channel access Type signalled in the UL grant for those subframes, if the end of UE transmission occurs in or before subframe . Similarly, for a UE configured for AUL transmission, and thus use CAT4 LBT by default, the UE may use CAT2 LBT for transmitting the AUL transmissions within the 'UL duration' subframes given that a flag is set in the CPDCCH to enable sharing of the DL COT to AUL.

Such an ability to switch the UL LBT type based on a group common indication facilitates DL-to-UL COT sharing and thus leads to improved UL channel access opportunities, latency  and resource utilization. It can be observed that a similar behaviour can be achieved in NR-U using the COT structure indication in GC-PDCCH wherein a number of DL slots resembles an ‘UL offset’ and a number of subsequent contiguous UL slots resembles a ‘UL duration’. The mechanism can be applied to both scheduled and CG UL transmissions.

Proposal 17: Switching the UL LBT type from CAT4 to CAT2 based on COT structure indication should be supported for both scheduled and CG UL transmissions in NR-U.

COT structure indication for FBE

According to the WID for NR-U [4], Frame Based Equipment (FBE) specifications in Rel-16 NR-U are intended for environments where the absence of Wi-Fi is guaranteed (e.g., by level of regulations, private premises policies, etc.). The targeted scenario is further limited to a single NR-U network (i.e. single operator) in the operating band and geographic area.

As per the ETSI regulations [9], the FBE transmit/receive structure has a periodic timing with a periodicity equal to a Fixed Frame Period (FFP) between 1 and 10 ms. An FBE-based transmission can start only at the beginning of the FFP immediately following a successful CAT2 LBT of at least 9μs, while the COT cannot be greater than 95% of the FFP and has to be followed by an Idle Period. Therefore, as discussed in further details in our companion contribution [10], gNB acquires the channel and initiates the DL CO as an initiating device whereas the UEs are scheduled/configured to transmit their UL transmissions within the gNB-initiated CO as response device. UE do not require to monitor PDCCH for the whole fixed frame period, if COT indication at the frame boundary is not detected. It can be observed that no FBE-specific information or modification to the indication mechanism is required for either FBE-based transmission schemes or a combination thereof.

Proposal 18: For indicating the COT structure in NR-U, no FBE-specific information is required.

NR-U CORESET configuration and monitoring in wideband BWP

In this section, consideration on NR-U CORESET configuration in frequency domain is discussed.

As per the previous agreement, LBT can be performed in each subband with channel bandwidth of 20MHz. Since the actual transmission bandwidth is dynamic due to LBT outcome, the CORESET is preferred to be configured within a 20MHz subband. Otherwise, if CORESET is configured with bandwidth larger than 20MHz and REG bundle interleaving is enabled, NR-PDCCH REG bundle(s) will be uniformly distributed in the frequency domain that assigned to the CORESET, and part of REG bundle(s) may become void if LBT of the corresponding subband LBT fails, which will lead to decoding failure of the whole PDCCH. When REG bundle interleaving is disabled, the actual allocated REG bundle for PDCCH could be constrained within one 20MHz subband even if the bandwidth of configured CORESET is larger than 20MHz. However, decoding performance of PDCCH may be degraded since the frequency diversity gain cannot be obtained via REG bundle interleaving. Therefore, NR-U CORESET is preferred to be configured within a 20MHz subband.

As discussed above, when the CORESET is configured within a 20MHz channel, the following two alternatives, which is shown in Figure 8, could be considered for the CORESET configuration in NR-U:

Alternative 1: The CORESET configuration is duplicated in each subband. When LBT for at least one subband succeeds, the corresponding CORESET is valid and the CORESET configuration will not be impacted by LBT. However, the signaling overhead is comparatively larger in case there are a large number of available subbands.

Alternative 2: The CORESET is only configured in one fixed subband. However, the CORESET configuration becomes void when LBT fails. In this case, the gNB loses the transmission opportunity and the spectral utilization efficiency decreases.

Figure 8 Possible NR-U CORESET configuration schemes

In order to mitigate LBT impact on the CORESET configuration while keeps the signaling overhead comparatively small, enhancement on CORESET configuration shall be investigated in NR-U. For example, if LBT for the configured subband for CORESET fails, the gNB may attempt to transmit CORESET in another subband that passes LBT. This subband switching pattern for CORESET shall be known at both gNB and UE side, which could be configured to the UE via RRC signaling. Then the UE can get the corresponding control information, if any, accordingly. If multiple CORESETs are configured simultaneously, and only partial subbands are available to transmit, then additional rules shall be defined to determine which CORESET(s) will be transmitted and the corresponding subband used for transmission.

Proposal 19: CORESET shall be configured within the bandwidth of subband LBT in NR-U, e.g. 20MHz.

Periodic CSI-RS transmission

In NR, CSI-RS is used for CSI acquisition and beam management, and both periodic and aperiodic configurations are supported. In NR-U, since LBT procedure is required for all DL/UL transmissions and hence periodic CSI-RS transmission cannot be guaranteed. However, based on our understanding, it shall also be supported in NR-U. Two main reasons are elaborated as follows.

- Signaling overhead reduction: For periodic CSI-RS transmission, the corresponding periodicity/offset, and transmission time/frequency allocation configurations are indicated by RRC signaling. Hence, 1 or limited bits are required to be carried by DCI to trigger such transmission. While for aperiodic CSI-RS transmission, more signaling bits are required for transmission resource indication.

- Power consumption reduction: For a UE configured with periodic CSI-RS, it may not need to periodically monitoring COT indication. Instead, it only measures CSI-RS at the pre-configuration time/frequency location. On the other hand, for a UE that configured with aperiodic CSI-RS, it shall continuing monitoring trigger DCI at every one/multiple slot(s) within current COT to ensure that the aperiodic CSI-RS is successfully detected.

When period CSI-RS is configured, it could be transmitted even if configured transmission occasion falls outside of COT. More precisely, gNB shall also perform LBT prior to the pre-configured CSI-RS occasion in case there is no DL data to transmit. Since the actual transmission state of periodic CSI-RS depends on dynamic LBT results, gNB will send notification after CSI-RS transmission occasion(s) with regard to transmission state(s) of one or multiple periodic CSI-RS, and measurement results at UE side could be updated accordingly.

To mitigate LBT impact on CSI-RS transmission, introducing more CSI-RS transmission opportunities and/or CSI-RS transmission state indication could be considered as potential enhancement to unlicensed operation. As illustrated in Figure 9, in case A, all three configured CSI-RS is not allowed to be transmitted due to LBT failure. In case B, the gNB may transmit a CSI-RS replacement triggered by PDCCH until the next pre-configured CSI-RS TO when the transmission fails at the current TO. Once the UE receives the CSI-RS replacement, it knows that the previous CSI-RS is missing due to LBT. In addition, the gNB could only indicate the transmission state of previous CSI-RS, as shown in case C, to the UE via PDCCH.

Figure 9 NR-U CSI-RS transmission schemes

Observation 3: Periodic CSI-RS transmission shall be supported in NR-U.

Proposal 20: gNB shall also attempt to transmit periodic CSI-RS that fall outside of COT.

Proposal 21: gNB could transmit a CSI-RS replacement triggered by PDCCH or indicate transmission state of CSI-RS to UE, which falls outside of configured CSI-RS transmission occasion.

Conclusions

In this contribution, we mainly discuss enhancement to PDSCH mapping type B, PDCCH transmission and monitoring in time domain, CORESET configuration in wideband operation, and CSI-RS transmission enhancement in unlicensed band. Considerations on the initial signal design and indication of COT structure are also presented. Based on the discussion, we have made the following observations and proposals:

Observation 1: The position l=7 is the best for the additional single-symbol DM-RS of PDSCH mapping Type B because it can be applied for all possible starting symbols of PDSCH mapping Type B with both length 9 and 10.

Observation 2: In the case of two additional single-symbol DM-RS position, the pair  l=4, 7  has the highest LTE-CRS collision-free probability for all possible PDSCH Type B starting symbols and for length 9 OFDM symbols.

Observation 3: Periodic CSI-RS transmission shall be supported in NR-U.

Proposal 1: DMRS pattern defined for PUSCH mapping type B could be the baseline as DMRS pattern for PDSCH mapping type B with duration of 3, 5, 6, 8, 11 and 12 symbols.

Proposal 2: In case that dmrs-AdditionalPosition equal to 'pos1', PDSCH duration ld=9 and ld=10, the PDSCH DM-RS positions l=7 is adopted for additional single-symbol DM-RS for PDSCH mapping type B.

Proposal 3: In case that dmrs-AdditionalPosition equal to 'pos2', PDSCH duration ld=9 and ld=10, the PDSCH DM-RS positions l=4, 7 are adopted for additional single-symbol DM-RS for PDSCH mapping type B.

Proposal 4: The case that dmrs-AdditionalPosition equal to 'pos3' with single-symbol DM-RS and the case with double-symbol DM-RS are not supported in Rel-16 TEI for PDSCH mapping type B with duration ld=9 and ld=10.

Proposal 5: The slot and symbol number used to generate DMRS sequence(s) of PDSCH(s) and PDCCH(s) in the 1st slot of a DL burst should be set to 0.  The slot numbers used to generate DMRS sequences of PDSCHs and PDCCHs in the following slots of the DL burst are relative to the slot where DL burst starts.

FFS: slot number configuration where additional GC-PDCCH(s) are transmitted within the DL burst.

Proposal 6: The group common PDCCH together with its DMRS could be used by UE to identify the DL burst from serving cell.

- FFS: mechanism to reduce blind decoding of GC-PDCCH

- FFS: wideband DMRS for GC-PDCCH

Proposal 7: NR-U should support to transmit GC-PDCCH and its DMRS multiple times within a gNB’s acquired COT.

Proposal 8: NR-U should support a PDCCH search space configuration allowing different PDCCH monitoring occasion configurations before and after the first slot boundary of a COT in terms of UE power consumption.

Proposal 9: UE could monitor DCI Format 2_0 carrying COT structure indication at each possible starting point in a slot before UE identifies the DL burst from serving cell.

Proposal 10: The COT structure indication reflects slot format from the slot carrying DCI format 2_0 carrying COT structure indication when it is received at slot boundary. UE could obtain slot format from 2nd slot in the COT when it is received in the middle of COT.

Proposal 11: The following two method could be considered for slot format indication for the last partial slot within a COT.

Option 1: Introducing a reference symbols for the ending slot

Option 2: Introducing new entries in the SFI table including a new state indicating ‘out of COT’

Proposal 12: NR-U should support indicating “Pause” state within the COT.

Proposal 13: The LBT bandwidth available for reception (LBW indication) can be indicated through a bitmap field in GC-DCI, each bit corresponding to a LBT bandwidth in an active BWP. UE is notified to obtain LBW indication later (or the current value is invalid) when all bit in the bitmap is “zero”.

Proposal 14: The valid time period of the LBT bandwidth available for reception (LBW indication) will be given in GC-PDCCH. UE is expected to detect another GC-PDCCH after that time.

Proposal 15: If GC-PDCCH is transmitted less often than the LBW changes, multiple LBW indications can be included in one GC-PDCCH transmission wherein the LBW indication contains a relative index of the slot at which the LBW update ends.

Proposal 16: UE is expected to receive on all LBT bandwidths if it does not receive GC-PDCCH carrying valid LBW indication.

Proposal 17: Switching the UL LBT type from CAT4 to CAT2 based on COT structure indication should be supported for both scheduled and CG UL transmissions in NR-U.

Proposal 18: For indicating the COT structure in NR-U, no FBE-specific information is required.

Proposal 19: CORESET shall be configured within the bandwidth of subband LBT in NR-U, e.g. 20MHz.

Proposal 20: gNB shall also attempt to transmit periodic CSI-RS that fall outside of COT.

Proposal 21: gNB could transmit a CSI-RS replacement triggered by PDCCH or indicate transmission state of CSI-RS to UE, which falls outside of configured CSI-RS transmission occasion.

References

Chairman’s Note RAN1#97

Chairman’s Note RAN1#96b

Chairman’s Note RAN1 AdHoc 1901

RP-182806, New WID on NR-based Access to Unlicensed Spectrum

TR 38.889 V16.0.0, “Study on NR-based access to unlicensed spectrum”

R1-1909433, TEI for PDSCH rate matching and views on DSS, Huawei, HiSilicon, Prague, Czech, August, 2019

- Appendix: Performance evaluation of initial signal candidates

NR-U is expected to have multiple mini-slot/symbol-level starting points to allow prompt access into channels. One issue arising along with the flexible starting point of DL/UL transmission is how to identify the start of DL/UL burst. In DL, unnecessary PDCCH blind-decoding could be avoided by introducing an ‘initial signal’ since blind-decoding imposes high requirement of UE capability and significantly decreases battery life of the device.

Wideband PDCCH DMRS could be a potential candidate as initial signal. An alternative approach is through detection of group-common PDCCH. 802.11a preamble is also identified as a candidate for initial signal which also facilitates co-existence between NR-U and WiFi devices.

In the following, details of each candidate for evaluation will be provided. As initial signal is only detected by the UE that already connected the serving cell, synchronization is assumed before the initial signal detection.

- A-I NR PDCCH DMRS

As shown in Figure A-1, RE density of PDCCH DMRS is 3 RE/RB and it spreads across configured CORESET CCE(s). The configured symbols of PDCCH DMRS within each slot could be 1-3 symbols.

The current sequence generation of DMRS in Rel-15 NR depends on the index of the slot and the symbol where the RS is located. gNB needs to prepare multiple versions of the identification signal or regenerate it in a short time repeatedly after it is contending for channel access, as it doesn’t know when channel access succeeds. In our evaluation, we assume sequence generation of PDCCH DMRS is time independent.

- A-II NR 802.11a preamble

802.11a preamble can be considered as a legacy preamble that is appended to all PHY frames for the purpose of backward compatibility. In our investigation, both Non-HT Short Training Field (L-STF) and Non-HT Long Training Field (L-LTF) are employed for DL burst identification, which are shown as follows. Since both sequences will be repeated twice in time domain, then L-STF is mapped to the 1st and 2nd symbol while L-LFT is mapped to the 3rd and 4th symbol within each slot. All sequences are mapped to the center frequency of BWP, as illustrated in Figure A-2, which occupy 52 sub-carriers with 312.5 kHz SCS.

Figure A-9 Illustration of resource mapping of NR PDCCH DMRS

Figure A-10 Illustration of 802.11 L-STF/L-LTF

- A-III Requirement of detection performance

Reliable identification of serving cell DL burst is very important for NR-U UE to determine whether to start PDCCH monitoring and receiving PDSCH in the following COT. In Figure A-3, we provide system level evaluation in indoor scenario to show the impact on the DL mean UPT assuming different detection performance of initial signal. We observed that 1% miss detection rate can achieve almost all the UPT. Further decreasing miss detection rate of initial signal has marginal gain. Moreover, if the detection sensitivity of the initial signal increases above -76 dBm, UPT will drop dramatically because UE cannot receive initial signal with high probability and will stop decoding the PDCCH and PDSCH.

Figure A-11 impact of initial signal detection on the UPT indoor scenario

- A-IV Performance evaluation of candidates for initial signal

As illustrated in Figure A-4, although 802.11a preamble occupies 4 symbols, detection performance of 802.11a preamble is the worst. This is because L-LTF and L-STF should work together with L-SIG field that modulated with BPSK ½ BCC, which is the bottle neck of the preamble detection [3]. Moreover, L-STF and L-LTF requires different numerology (with 312.5 kHz SCS), which may impose additional requirement on hardware implementation.

Detection performance of PDCCH DMRS is better than that of 802.11a preamble. According to agreements of UE power saving AI, the false alarm rate of 1% is agreed as baseline for evaluation of power saving signal/channel for wake-up purpose. In our investigation, detection performance of PDCCH DMRS with 0.1% false alarm rate is also provided. It is observed that miss detection performance with higher false alarm rate outperforms that with lower false alarm rate. However, higher false alarm rate leads to more power consumption at UE side. Due to limited sequence length, there is a considerable gap between the performance of PDCCH DMRS and that of group-common DCI with lower payload size (e.g. 20 bits) when SNR is above -5 dB for both 30 kHz SCS and 60 kHz SCS cases. The performance gap becomes larger with higher SNR. Furthermore, additional SFI could be carried by group-common DCI, which could also facilitates UE’s further DL reception.

Detection performance of group common DCI mainly depends on DCI payload size and SCS. With the same aggregation level, smaller DCI payload size corresponds to lower coding rate and hence results in better miss detection performance. While with the same DCI configuration, detection performance of group common DCI with 30 kHz SCS is better than that with 60 kHz SCS due to larger DMRS sequence length and better channel estimation performance. To summarize, detection performance of group-common DCI depends on its numerology, aggregation level and DCI payload size.

Figure A-12. Detection performance of group common DCI, PDCCH DMRS and 802.11a preamble

- A-V: system level simulation assumptions

| Parameters | Indoor Sub-7GHz |
| --- | --- |
| Layout for nodes | Layout dimensions: 120mx80m a=20 meters, b=40 meters, c=20 meters, and d=40 meters |
| Carrier Frequency | 5GHz |
| Carrier Channel Bandwidth | 20MHz baseline |
| Number of carriers | 1 |
| Number of users per operator | Exactly 5 per gNB per 20MHz |
| SCS | 30KHz and 60KHz |
| Channel Model | NR InH Mixed Office model |
| BS/AP Tx Power | 23dBm (total across all TX antennas) |
| UE/STA Tx Power | 18dBm (total across all TX antennas) |
| BS/AP Antenna gain | 0 dBi |
| UE/STA Antenna gain | 0 dBi |
| BS/AP Noise Figure | 5dB |
| UE/STA Receiver Noise Figure | 9dB |
| Minimum received power from serving cell for UE dropping | -82dBm |
| CCA-ED | -72dBm for NRU |
| UE receiver | MMSE-IRC |
| TXOP | 8ms for NRU |
| MCS | NR MCS with 256QAM (LDPC) |
| MIMO | TM9 with one layer |
| UE Processing Time Capability | #2 |
| Link adaptation | CQI feedback + OLLA |
| Scheduling | Proportional Fairness |
| BS/AP antenna Array configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| UE/STA antenna Array configuration | Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| Traffic model | DL only |
| UE/STA to UE/STA link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |
| gNB to gNB link pathloss model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |

- A-VI: link level simulation assumptions

| System bandwidth | 51 RBs (30 kHz SCS) 24 RBs (60 kHz SCS) |
| --- | --- |
| Numerology | 30/60 kHz SCS |
| Channel | TDL-C |
| Channel delay | 100 ns |
| Number of Antennas | Tx@ gNB =1 Rx@ UE =2 |
| NID(1),NID(2) | 0 |
| Probability of false alarm | 0.1% , 1% |
| UE moving speed | 3 km/h |
| Channel estimation method | MMSE |