3GPP TSG RAN WG1 Meeting #98	　	                                 R1-1908118

Prague, Czech Republic, 26th Aug. – 30th Aug. 2019

Source:	Mitsubishi Electric

Title:	Views on DL reference signal designs for NR positioning

Agenda Item:	7.2.10.1 DL Reference Signals for NR Positioning

Document for: 		Discussion/Decision

Introduction

In RAN1 #97 meeting, parameters and measurements needed DL and UL only positioning techniques were discussed. In this contribution, we make proposals related to the PRS (positioning reference signals). In Section 2, proposals related to DL PRS are described. The contribution is summarized in Section 3.

DL positioning OTDOA

General procedure

For 30GHz, robust design of PRS needs to be discussed, especially considering factory environments where positioning process can be interrupted by mobile equipment. For DL positioning, OTDOA (Observed Time Difference of Arrivals) is assumed here. OTDOA is characterized by multiple TRPs transmitting reference signals to UE. At the UE side, RSTD (reference signal time difference measurement) is calculated and based on coordinates of TRPs, location of UE can be determined. In NR, multiple TRPs or panel operation is envisioned,

The following formula is used to approximate the location of the UE:

RSTDi,1=1cxt-xi2+yt-yi2+1cxt-x12+yt-y12+Ti-T1+(ni-n1)

where RSTDi,1 is the time difference between reference TRP and target, Ti-T1 is the time offset between TRPi and TRP1. Measurement error in arrival time at UE is indicated by ni. The speed of light is denoted by c. The Cartesian coordinate of UE and ith TPR is indicated by (xt,yt) and  (x1,y1), respectively. In the following subsections, possible reference signal designs are introduced.

RS designs for OTDOA : PRS patterns

In LTE, positioning reference signals (PRS) are defined and used for positioning. As in LTE, reference signals dedicated for positioning can be used to meet various accuracy requirements. Although CSI-RS or SSB can be used as positioning RS, RS with higher density and configuration optimized for positioning as agreed in AH#1. In this subsection, possible designs for PRS are introduced.

In the example shown in Figure 1a, NR PRS are placed in a resource block so that multiplexing up to 6 PRS patterns are possible, and the pattern is spread across the time and frequency domain. One of the appealing characteristics in LTE PR is that PRS was dedicated for positioning, spread across time and frequency domain. In NR RS, although DMRS and CSI-RS are both designed for maximum coverage across frequency and time domain, they do not have flexibility compared to PRS. Thus, in this contribution, we propose to investigate performance of PRS to observe if the proposed designs meet the target. One difference between PRS design for LTE and NR is that cell reference signal (CRS) is not defined in NR. Thus, PRS can be placed without consideration for CRS. One of the possible configurations for PRS is shown in Figure 1a, in which similar to PRS in LTE, up to six patterns can be multiplexed. Thus, PRS transmitted from gNB in non-serving cells can be multiplexed in the frequency domain orthogonally. In addition, since there are no CRS in NR, PRS can be placed evenly in a resource blocks.

Figure 1 Placement of PRS, various densities with comb-3 and comb-12

Configurable PRS frequency and time allocation was agreed. However, granularity of time allocation should be discussed, i.e., whether slot or mini-slot level granularity is adopted. As in LTE, PRS should be placed so that PRRs do not overlap with PDCCH. The PRS layouts shown in Figure 1a and 1b avoid the first 3 or 2 symbols, respectively, in a slot to avoid PDCCH. In addition, to avoid interference with PDSCH, at least for FR1, there should be slots dedicated for PRS, i.e., UE is not expected to receive PDSCH in REs where PRS are not scheduled. Depending on the slot structure, placement of PRS should be truncated. For example, the PRS symbols are dropped from the pattern if some symbols are reserved for uplink.

Regarding CP length, both normal CP and extended CP should be used for positioning. Extended CP may be useful when large delay spread is expected in the channels. Furthermore, it is preferable that PRSs sent from gNB in both serving and non-serving cells have the same CP length for synchronization and ease of computation at UE.

Proposal 1: For RSTD calculation, use the same CP length, i.e., either NCP or ECP

In addition, considering higher requirement for positioning accuracy, density of PRS should be configurable. In NR, wideband positioning is possible, and density of PRS can be adjusted to allow multiplexing. In addition, since at least dedicated PRS resources are agreed in AH#1, PRS and PDCCH are not allowed to be multiplexed. Thus, configurations shown in Figure 1a and 1b allow at most three symbols for PDCCH One of the configurations for higher and lower density of PRS, compared to the density in Figure 1a, is shown in Figure 1b and Figure 1c, respectively. Finally, beam based operation in FR2, PRS may be suitable for FR1 considering wider beamwidth implemented in FR1.

Proposal 2: Variable densities for PRS for OTDOA should be supported

In RAN1#97, comb values were discussed and comb {2, 4, 6} were selected. Variable densities are supported through different comb values. From PRS multiplexing point of view, comb 12 for PRS should be supported as illustrated in Figure 1b. In addition, 12 symbols should be chosen as one of the configurable number of symbols.

Proposal 3: In addition to {2, 4, 6}, support comb 12

Proposal 4: In addition to {2, 4, 6}, support 12 as the number of configurable symbols for PRS

In addition, relationship between the number of symbols and comb pattern must be considered. It is clear from Figure 1a and 1b that N symbols are needed to represent the full comb-N pattern. Thus, the at least a mapping pattern which can represent full comb-N pattern should be supported.

Proposal 5: Mapping of at least comb-N PRS pattern for N symbols should be supported.

In addition, whether staggered patterns, such as the pattern shown in Figure 1a should be supported as a single PRS or as a collection of PRS resources where one PRS resource consists of one symbol with vertical shift in the frequency domain as a parameter. To reduce overhead for PRS configuration, it is recommended to support staggered patterns as a single resource, i.e., preconfigured sequence of symbols with different vertical shifts is defined as a resource.

Proposal 6: Support staggered patterns (a collection of PRS symbols from the same antenna port with different vertical frequency offsets for at least some symbols) in a single PRS resource

RS designs for OTDOA : RB-level density for PRS

The following definition of a PRS resource can be found in TR 38.855 [2].

NR DL PRS resource is defined as a set of resource elements used for NR DL PRS transmission that can span multiple PRBs within N (1 or more) consecutive symbol(s) within a slot, where N is left to future specification work. - In any OFDM symbol, PRS resource occupies consecutive PRBs - Whether multiple symbols can be non-consecutive is left to future specification work.

Furthermore, to increase flexibility and multiplexing capability in PRS, we propose RB-level comb patterns. An example of configuration is shown in Figure 2. In the example, PRS with comb-4 configuration is inserted in every other RB while RBs which do not carry PRS are left as an empty RB. The configuration shown in Figure 2 is an example where RB density is 1/2, where red tiles indicate PRS. The RB level combing increases flexibility in PRS assignment. PRS sent from neighbouring cells can be placed in vacant RBs to send its PRS. A similar configuration is implemented for CSI-RS in Rel. 15. Thus, we make the following proposal.

The RB-level combing can be defined as one of the configuration of an RS resource since PRS spans over multiple RBs with the predefined RB density. The RB level density can be defined as one of the parameters. The RB level comb can be applied to any PRS patterns.

Another view of defining RB-level comb is RB-level muting in the frequency domain. Muting is currently discussed for the time domain to decrease interference for other UEs. Similar approach can be applied to the frequency domain to decrease interference and improve spectrum efficiency.

Proposal 7: Support RB-level comb pattern for PRS, e.g., PRS pattern shown in Figure 2

The parameter to indicate RB-level density should be configured semi-statistically, similar to the comb density descried earlier.

Figure 2 RB-level comb for PRS

Furthermore, whether PRS designs for non-slot are needed should be considered. In low-latency applications, burst PRS may be required to locate UEs. The aforementioned PRS designs for various sizes of non-slots should be considered. One of the possible placement of PRS in a non-slot is shown in Figure 1d.

Proposal 8: PRS designs for nonslots should be supported

Generation of PRS sequences and characteristics of PRS sequence

In AH#1, it was agreed to generate sequence using PRS sequence ID. Furthermore, in RAN1#96b, it was agreed to use the god sequence generator, as defined in TS 38.211, to generate the sequences. Details related to the generation need to be discussed.

Finally, configuration of PRS should be panel/TRP specific, following RS designs in NR. Such flexibility will allow configuration of unique PRS to different panels or TRPs. PRS sequences should be generated with the unique ID, following the sequence designs for NR-DMRS, NR CSI-RS or NR-SRS. The ID should be configurable semi-statistically. Using configurable IDs, it should be possible to provide PRS to large number of devices in a scenario like IIoT. Since PRS will occupy multiple symbols, the initialization seed should depend both on symbol and slot index.

Proposal 9: C_init should depend on the slot and symbol index

Similar to NR CSI-RS or NR DMRS, resource specific PRS should be supported. The advantage of resource specific RS is that there’s less overhead to generate RS. In addition, it is easier for UE to hear RSs transmitted from neighbouring cells, since UE only needs to know cell ID (or other unique IDs) used to generate sequences from neighbouring cells.

Proposal 10: Support resource specific PRS

There could be a need to perform burst positioning to find approximate location of the equipment. In such a case, a burst of slots containing PRS can be sent for UE to perform quick measurement. As described above, density of PRS can be configured and a burst of high density PRS can be sufficient to locate equipment. A trigger from UE to initiate positioning process should be supported, i.e., UE can directly request gNB for PRS transmission.

Proposal 11: Support on-demand resource allocation for PRS

Multiple BWPs for PRS

The measurement gap before switching to different BWPs can be used for positioning. When wideband positioning is implemented, multiple BWPs can be used. In this case, PRS can be allocated for each BWP, allowing positioning for each BWP. In such a case, measurement gap during which no transmission, but SSB, can be allowed may be suitable region for transmission of PRS. As explained in the previous section, PRS should be transmitted when no other signals or channels are transmitted to minimize interference experienced by PRS.

Muting configuration

In RAN1 #96b, muting pattern in which PRS transmission is turned off at preset periodicity was agreed to be supported. Muting of PRS is implemented to minimize interference from transmission of PRS. Since a resource set is associated with its transmission source, muting pattern should be configured at resource set level. In addition, granularity of muting pattern should be at slot level.

Proposal 12: PRS Muting pattern should be configured at resource set level

Proposal 13: Slot level granularity for PRS muting patterns should be supported

Periodicity of PRS

In RAN1#97, periodicity of PRS was discussed and whether periodicity is configured at resource level or resource set level needs to be decided. Since a resource set is related to the source of transmission, to minimize the amount of configuration and simplicity, periodicity should be configured at the resource set level. If different periodicities were required for different resources, multiple resource sets with different periodicities can be configured.

Proposal 14: Support the following alternative, “Alt.1 - Periodicity of DL PRS Resource is configured at DL PRS Resource Set level”, i.e., common period is used for DL PRS resources within a DL PRS Resource Set.

Sub-symbol level beam sweeping

Finally, it should be noted that from the UE perspective, PRS does not share its resource with other RS or channels during its transmission. Thus, during beam sweeping, sub-symbol sweeping is feasible. Due to the intrinsic characteristic of combed signal to generate repeating patterns in the time domain, beams can be swept in one symbol. Similar method was proposed for CSI-RS during Rel. 15 and 16. However, for CSI-RS, additional RRC signalling is needed to dedicate resources for CSI-RS. Sharing a CSI-RS resource with other channels or RS will create random interference which will prevent inter-symbol beam sweeping. However, due to interference-free nature of PRS resource, sub-symbol level beam sweeping can be supported. Thus we make the following proposal:

Proposal 15: Sub symbol level beam sweeping is supported for DL PRS

PRS configuration

During the SI phase, there were discussions on whether PRS should be part of CSI-RS, similar to TRS which is a part of configuration for CSI-RS. As CSI-RS is optimized and tightly integrated in the Rel. 15 MIMO framework, our recommendation is to establish dedicated RS for positioning, since many of the adopted features such as PRS patterns are too complex to be realized using the CSI-RS framework. Thus we make the following proposal

Proposal 16: Support a new PRS which cannot be configured as a part of CSI-RS

Conclusion

In this contribution we made the following proposals related to PRS densities and configuration.

Proposal 1: For RSTD calculation, use the same CP length, i.e., either NCP or ECP

Proposal 2: Variable densities for PRS for OTDOA should be supported

Proposal 3: In addition to {2, 4, 6}, support comb 12

Proposal 4: In addition to {2, 4, 6}, support 12 as the number of configurable symbols for PRS

Proposal 5: Mapping of at least comb-N PRS pattern for N symbols should be supported.

Proposal 5: Mapping of at least comb-N PRS pattern for N symbols should be supported.

Proposal 6: Support staggered patterns (a collection of PRS symbols from the same antenna port with different vertical frequency offsets for at least some symbols) in a single PRS resource

Proposal 7: Support RB-level comb pattern for PRS, e.g., PRS pattern shown in Figure 2

Proposal 8: PRS designs for nonslots should be supported

Proposal 9: C_init should depend on the slot and symbol index

Proposal 10: Support resource specific PRS

Proposal 11: Support on-demand resource allocation for PRS

Proposal 12: PRS Muting pattern should be configured at resource set level

Proposal 13: Slot level granularity for PRS muting patterns should be supported

Proposal 14: Support the following alternative, “Alt.1 - Periodicity of DL PRS Resource is configured at DL PRS Resource Set level”, i.e., common period is used for DL PRS resources within a DL PRS Resource Set.

Proposal 15: Sub symbol level beam sweeping is supported for DL PRS

Proposal 16: Support a new PRS which cannot be configured as a part of CSI-RS

- References

- [1]	RAN1 Chairman’s Note, RAN1 #97, Reno, USA, May, 2019.

- [2]	3GPP TR 38.855, “Study on NR positioning support (Rel.16)”, ver. 1.0.0., March 2019.

Appendix:

Agreements from RAN1#96b Agreement: - A DL PRS Resource Set is defined as a set of DL PRS Resources, where each DL PRS Resource has a DL PRS Resource ID - The DL PRS Resources in a DL PRS Resource set are associated with the same TRP - A DL PRS Resource ID in a DL PRS Resource set is associated with a single beam transmitted from a single TRP (A TRP may transmit one or more beams) - Note: This does not have any implications on whether the TRPs and beams from which signals are transmitted are known to the UE. Agreement: - DL PRS sequence is generated using a Gold sequence generator as defined in TS 38.211 Section 5.2.1 - FFS: cinit value for DL PRS - QPSK modulation is used for the DL PRS signal transmitted using CP-OFDM - FFS: Whether a DL PRS sequence generated using a different mechanism is also specified Agreement: Comb-N resource element pattern per DL PRS Resource is supported to map DL PRS sequence to resource elements in frequency domain - Comb-N pattern can shift in frequency domain across symbols within DL PRS Resource - FFS values for N. The potential values for down-selection are provided in the set {1,2,3,4,6,8,12} - FFS relationship between N and number of symbols per DL PRS Resource - FFS support of staggered and non-staggered patterns and exact definition of staggered pattern Agreement: DL PRS configuration including DL PRS transmission schedule is to be indicated to the UE for DL PRS positioning measurements - The UE is not expected to perform any blind detection of DL PRS configurations Agreement: Support of numerologies (CP length and sub-carrier spacing) for the DL PRS is the same as for data transmissions in Rel-15. Agreement: DL PRS muting is supported. The UE is expected to be indicated when the DL PRS is muted. Agreements from RAN1#97 Agreement: DL PRS Resource is described by at least the following parameters - DL PRS Resource ID (previously agreed) - Sequence ID (previously agreed) - Comb Size-N - RE Offset in frequency domain - FFS whether this offset is a single value or multiple values - Starting slot and symbol of DL PRS Resource - FFS whether it can be represented by time offset with respect to some reference - Number of symbols per DL PRS Resource (Duration of DL PRS Resource) - Quasi-colocation information (QCL with other DL reference signals) - FFS QCL type and source reference signals - FFS: Number of Tx Ports - FFS: Power offset b/w DL PRS and SSB - FFS: Transmission bandwidth and starting PRB with respect to Point A - FFS: Numerology Note: RAN1 to discuss further whether some of the above parameters belong to Resource Set configuration and are applied to all Resources within a Resource Set or not. Agreement: - Number of symbols for DL PRS Resource is configurable from the following set {2, 4, 6} - FFS: Inclusion of other values in the set including values in {1, 3, 8, 12} - DL PRS Resource comb-N value is configurable from the set {2, 4, 6} - FFS: Inclusion of other values in the set including values in {1, 8, 12} - Note: The dependence between the number of symbols and the comb size should be considered when considering the inclusion of additional values in the sets for these parameters R1-1907895 Outcome of offline discussion #2 on AI 7.2.10.1 - DL Reference Signals for NR Positioning Intel Corporation Agreement: RAN1 to select one of the following alternatives for periodic DL PRS Resource Allocation at RAN1#98: - Alt.1 - Periodicity of DL PRS Resource is configured at DL PRS Resource Set level - Common period is used for DL PRS resources within a DL PRS Resource Set - Alt.2 - Periodicity of DL PRS Resource is configured at DL PRS Resource level - Different periods can be used for DL PRS resources within a DL PRS Resource Set Agreement: DL PRS occasion is one instance of periodically repeated time windows (consecutive slot(s)) where DL PRS is expected to be transmitted - FFS: If this definition is introduced for muting or DL PRS resource allocation or both