3GPP TSG RAN WG1 #98		R1- 1908138

Prague, CZ, August 26th – 30th, 2019

Source:	vivo

Title:	Discussion on physical DL channel design in unlicensed spectrum

Agenda Item:	7.2.2.1.2

Document for:	Discussion and Decision

- Introduction

According to WID on NR-based access to unlicensed spectrum [1], the following parts should be specified:

-	Physical layer aspects including [RAN1]:

-	For DL data channel, support of multiple PDSCH starting positions;

-	Mechanism to detect a gNB’s transmission burst in line with the TR 38.889, Section 7.2.1.2 related to UE power consumption;

-	DL control in line with the agreements during the study phase (TR 38.889, Section 7.2.1.2) including extensions allowing dynamic change of the time domain instances in which the UE is expected to receive PDCCH, modifications enabling DRS transmissions without gaps in the time-domain, and indication of time domain COT structure;

In RAN1 #97 meeting, the following agreement is made [2]:

Agreement:

- Additional durations from 2 to 13 (other than 2, 4 and 7 which are already supported in Rel-15) for PDSCH mapping type B are supported.

  - FFS: A duration of 14

  - FFS: DMRS aspects for the additional durations (Reuse Rel-15 patterns or define new patterns for these additional durations)

- Capability signalling will be defined for UEs to indicate which specific subset of durations are supported by the UE.

- FFS: Some duration(s) are mandatory for the UE to support for initial access

- FFS: PDSCH processing times for the new durations (reuse the values for durations of 2/4/7 or define new ones)

- Note: PDCCH monitoring occasions will be discussed separately as part of the NR-U WI

Note: This agreement does not by itself imply an increased requirement in PDSCH processing and decoding capabilities compared to Rel-15

In RAN #84 meeting, the following enhancements for dynamic spectrum sharing will be done as part of TEI16 in RAN1,

- Introduce PDSCH Type B mapping of length 9 and 10 OFDM symbols where the positions of DMRS are defined or can be configured so they do not collide with symbols containing LTE CRS

- Extend the higher layer configuration of LTE-CRS rate matching pattern to multiple rate matching patterns to support multiple LTE carriers within an NR carrier

  - RAN1 will decide on a reasonable limit for the number of rate matching patterns to be supported

  - The LTE-CRS rate matching patterns are applicable to 15 kHz SCS

In this paper, physical DL channel design including DM-RS design, COT structure indication and etc. are discussed in the following sections. Besides, DMRS enhancements for length 9&10 PDSCH Type B mapping in dynamic spectrum sharing (DSS) case are also discussed.

- Discussion

COT structure indicator

In LAA/eLAA/FeLAA, UEs are monitoring DCI format 1C in common PDCCH with CRC scrambling with CC-RNTI to obtain some COT information, i.e. ‘Subframe configuration for LAA’, ‘UL duration and offset’ and ‘COT sharing indication for AUL’. First, ‘Subframe configuration for LAA’ indicates the partial DL subframe for current subframe or next subframe, which conveys part of DL COT information. Second, ‘UL duration and offset’ indicates the subframes for UL transmission in eNB’s COT. Third, ‘COT sharing indication for AUL’ is used to enable/disable AUL transmission to share eNB’s COT.

In NR Rel15, SFI is used to convey the dynamic UL/DL information in addition to semi-static UL/DL configuration. It could indicate the state of each symbol for the succeeding slots. For NRU, it is not appropriate to have semi-static UL/DL configuration since the transmission state depends on LBT results, i.e. semi-static UL/DL configuration could indicate all symbols as flexible. In this case, SFI carried by GC-PDCCH is more feasible to be used for indication of NRU UL/DL transmission. Besides, SFI should be transmitted when or after gNB acquires the COT. So it is natural to extend current SFI for COT structure indicator with the following state:

- Indicated ‘D’ symbols inside COT: UE will assume the symbols are already scheduled for DL transmission,   which means it won’t proceed with any UL transmission and may skip CCA.

- Indicated ‘U’ symbols inside COT: UE will assume that the symbols are already scheduled for UL transmission, which means it won’t proceed with any DL reception and may skip CCA (optional).

- Indicated ‘F’ symbols inside COT:

  - ‘F’ symbols between ‘D’ and ‘U’ symbols inside COT: switching symbols with no DL and UL transmissions

  - Otherwise: symbols not decided as DL or UL, maybe overwritten by following COT indication

- Indicated COT ending symbol

  - New ‘E’ state symbol indicates the COT ending or

  - Additional field indicates COT ending symbol position

Figure 1	Examples for COT structure indication

For example, Figure 1(a) illustrates COT structure indication for DL data scheduling where red symbol represents SFI transmission. In the first SFI, the scheduling for later slots is not decided yet so they are indicated as flexible. Here GC-PDCCH is configured to be monitored per two slots. In later slots, the SFI could override the flexible symbol. Besides, the COT ending symbol is indicated as ‘e’ state. The indication outside the COT is not valid anymore. Similarly, Figure 1(b) shows one example for UL data scheduling. Note that the flexible symbols between ‘D’ and ‘U’ symbols corresponds to pause state without DL or UL transmission.

Proposal 1: SFI is updated to indicate COT structure with at least the following enhancement:

- COT ending indication

  - New ‘E’ state symbol indicates the COT ending or

  - Additional field indicates COT ending symbol position

- Indicated ‘F’ symbols inside COT:

  - ‘F’ symbols between ‘D’ and ‘U’ symbols inside COT: switching symbols with no DL and UL transmissions

  - Otherwise: symbols not decided as DL or UL, maybe overwritten by following COT indication

Dynamic PDCCH monitoring

In TR 38.889 [3], the following agreement is achieved in NRU SI:

Compared to NR Rel-15, it has been identified to be beneficial if the time domain instances in which the UE is expected to receive PDCCH can change dynamically, e.g. by implicit determination related to the gNB’s COT, or explicitly signaled by the gNB.

In this text, it mentions the following two options to change the PDCCH monitoring:

- Opt. 1: implicit determination at UE side in different phases;

- Opt. 2: UE follows gNB’s explicit signalling to change the PDCCH monitoring dynamically.

For Opt. 1, it is important that UE and gNB have the same understanding on the phase switching point, which is difficult to achieve in every case. Besides, it is already agreed that 2-13 symbol duration PDSCH are all supported so that gNB has more flexibility to schedule the DL data. For example, to achieve multiple starting point in one slot, gNB at least has the following two alternatives according to current agreement:

- For Alt. 1, gNB will prepare multiple small Type B PDSCHs in advance and transmit part of them once LBT succeeds while the other part of them is waiting for the next opportunity transmission, e.g. at the end of this transmission. For the PDSCH in later slots, large Type A PDSCHs are transmitted to reduce the overhead and number of HARQ processes. One example is provided in Figure 2, where 1 slot processing time including both L2 and L1 processing is assumed. As shown in Figure 2, gNB start L2 processing to prepare the data one slot before the transmission, i.e. the data transmitted in slot n+1 starts L2 processing at the front boundary of slot n. gNB starts transmission of TB3 at the 3rd mini-slot when LBT succeeds. In succeeding slot, it switches to slot based transmission and perform transmission of TB1 and TB2 at the end of the burst. In this case, there are two switching points between mini-slot based and slot based PDCCH monitoring.

Figure 2		Example for Alt .1 with front-prepared multiple 2-symbol type B PDSCHs

- For Alt. 2, gNB starts L2 processing of the next opportunity transmission for the type B PDSCH in next slot immediately when LBT fails, e.g. L2 processing of TB1 and TB2 starts immediately after LBT fails for their transmission. gNB starts L2 processing of new transmission for the type B PDSCH in the rest time of next slot (e.g. 10 symbols in Figure 3) when LBT results is known. One example is shown in Figure 3.

Figure 3		Example for Alt .2 with cyclic shift transmitted multiple 2-symbol type B PDSCHs

First, UE doesn’t know which alternative gNB adopts and each alternative corresponds to different switching point, e.g. at the 2nd slot boundary for Alt. 1 and the 3rd slot boundary for Alt. 2. Furthermore, even for one alternative, there may be multiple switching point, e.g. Alt. 1. Once there is mismatch on the switching point, UE’s scheduled data may be lost due to different monitoring occasion understanding. Thus, Opt. 1 is not a good and robust way to change UE PDCCH monitoring. For Opt. 2, UE follows gNB’s signalling to change the PDCCH monitoring occasions which is more robust compared to Opt. 1. Then the following proposal is made:

Proposal 2: NR-U supports dynamic PDCCH monitoring by explicit signalling.

There are two ways to convey this signalling, one is UE specific PDCCH and the other one is GC-PDCCH. If reusing UE specific PDCCH that schedules the data, the non-scheduled UEs couldn’t change the PDCCH monitoring for saving power. If introducing additional UE specific PDCCH to signal the information, the overhead is too much when signalling to multiple UEs. GC-PDCCH could solve the above problems very well, i.e. it conveys the information that which occasion needs to be monitored in certain slot within one duration. Then UE determines the monitoring occasions by combining both RRC configured one and GC-PDCCH’s signalled one. In this way, both scheduled and non-scheduled UEs could save power consumption with less monitoring occasions. Meanwhile, this indication could be integrated into COT structure indication, i.e. the symbol where PDCCH is transmitted is indicated as ‘P’ state inside the COT.

Proposal 3: GC-PDCCH could deliver information on PDCCH monitoring indicator as well as COT information.

CSI-RS related UE behavior

In NR Rel15, UE could be configured with periodic/semi-persistent CSI-RS associated with CSI reporting. For each CSI reporting, one CSI reference slot will be defined, i.e. nCSI_ref  slots before the CSI reporting slot. nCSI_ref depends on SCS setting and number of configured CSI-RS resource. For example, assuming SCS is 15KHz for both DL and UL, and only one CSI-RS resource is configured, nCSI_ref is smallest value greater than or equal to 4 such that it is a valid DL slot.

As shown in Figure 4(a), if a UE is not configured with time Restriction for Channel Measurements, the UE shall derive the channel measurements for computing CSI value reported in uplink slot n based on only the NZP CSI-RS, no later than the CSI reference resource associated with the CSI resource setting.

As shown in Figure 4(b), if a UE is configured with time Restriction for Channel Measurements, the UE shall derive the channel measurements for computing CSI reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of NZP CSI-RS associated with the CSI resource setting.

Figure 4   Periodic CSI-RS transmission and CSI reporting

However for NRU, configured periodic/semi-persistent CSI-RS may not be transmitted due to LBT failure, e.g. CSI-RS #2 is not transmitted due to LBT failure. In this case, UE should skip measurement on this non-transmitted CSI-RS. For case a, UE needs to omit the measurement occasion without CSI-RS transmission; for case b, UE should select the most recent transmitted CSI-RS no later than the CSI reference resource. The basic condition to achieve this is that UE needs to determine whether configured P/SP CSI-RS is transmitted or not.

Proposal 4: UE needs to determine whether configured P/SP CSI-RS is transmitted or not before performing measurement.

DL burst detection signal

In LTE LAA, together with transmitting PDSCH, the always-on CRS could be used for LAA UEs to detect whether there is DL transmission or not. In implementation, LAA UEs will monitor CRS first and then start blind PDCCH monitoring after detecting CRS, which will avoid unnecessary PDCCH blind decoding and thus save UE power. However, CRS has been removed in NR Rel15 and NR-U will inherit this feature. One simple way is to start PDCCH blind decoding in configured search space directly, which results in a waste of UE power due to the uncertainty of channel availability in NRU. Therefore, it is necessary to introduce low complexity signal to facilitate identification of DL transmission for NRU. The following options are identified

- Alt. 1: PDCCH DM-RS

  - Alt. 1-1: UE specific PDCCH DM-RS

    - Alt. 1-1-1: Narrow band DM-RS (e.g. 1, 2 or 4 CCEs)

    - Alt. 1-1-2: Wideband DM-RS (e.g. 8 CCEs)

  - Alt. 1-2: GC-PDCCH DM-RS

    - Alt. 1-2-1: Narrow band DM-RS (e.g. 4 CCEs)

    - Alt. 1-2-2: Wideband DM-RS (e.g. 8 CCEs)

- Alt. 2: CSI-RS

- Alt. 3: PSS/SSS like signal

The comparison of the above options is summarized in terms of performance, complexity, spec impact, flexibility and information capability in the following table:

Table 2		Pros and Cons comparison of 4 options

| Schemes | Performance | Complexity | Spec impact | Flexibility | Information |
| --- | --- | --- | --- | --- | --- |
| Alt. 1-1-1 | Bad | Bad | Good | Bad | Normal |
| Alt. 1-1-2 | Good | Good | Normal | Bad | Normal |
| Alt. 1-2-1 | Normal | Good | Good | Good | Good |
| Alt. 1-2-2 | Good | Good | Normal | Normal | Good |
| Alt. 2 | Good | Good | Good | Good | Normal |
| Alt. 3 | Normal | Good | Bad | Bad | Normal |

From performance aspect, the comparison is determined from evaluation of miss detection performance based on the simulation assumptions provided in Annex. As shown in Figure 5, wideband DM-RS (Alt. 1-1-2 and Alt. 1-2-2) and CSI-RS (Alt. 2) could achieve better performance, e.g. the performance of PDCCH DMRS in 8 CCEs has about 3dB gain compared with PDCCH DMRS in 4 CCEs. For Alt. 1-1-1, its performance is bad since there are less PRBs used for narrow band DM-RS with low aggregation level. Except Alt. 1-1-1, other Alts can achieve acceptable performance in terms of miss-detection performance assuming 1% target false alarm rate, i.e. <1% misdetection rate when SNR=-2dB, where almost 100% NRU UEs are operating above according to system level evaluation observation.

Figure 5	Performance of DL burst detection signal

From complexity aspect, it is determined by the number of correlation times for different alternatives. UE specific PDCCH is most probably configured with multiple aggregation levels and candidate positions to enhance scheduling flexibility, e.g. AL=1,2,4. For Alt. 1-1-1, UE needs to detect the narrow band DM-RS blindly in all candidate PDCCH positions which leads to higher complexity. Wide band DM-RS could alleviate this since DM-RS is spanning in the whole CORESET by Alt. 1-1-2. For GC-PDCCH, it is broadcast to a group of UEs and thus it is always with large ALs, e.g. AL=4,8. With large AL, the number of PDCCH candidates is limited, i.e. 1 or 2. Then the complexity for Alt. 1-2-1 could be kept low. Obviously, Alt. 1-2-2 will have even less complexity. For other Alts, the complexity will be low if small number of sequences (e.g. 1-4) is assumed for one UE.

From spec impact aspect, Alt. 1-1-1, Alt. 1-2-1 and Alt. 2 are mandatory existing NR signals in Release 15 so NRU could reuse them with little spec impact. Since wideband DM-RS is optional in NR Release 15, it needs to become mandatory if it is used for NRU as DL burst detection signal. For Alt. 3, although PSS/SSS is existing signal, it is only used in SSB as one integrated block. If PSS/SSS without PBCH is used for DL burst detection, this is one new kinds of signal which new configuration design. Besides, they should be carefully designed to meet some basic requirement such as OCB requirement, multiplexing with other channel (e.g. PDSCH) and etc. This should introduce more spec impact than others.

From flexibility aspect, different Alts are analyzed in time domain and frequency domain:

-	In time domain, for Alt. 1-1-1 and Alt. 1-1-2, the DM-RS monitoring position is coupled with search space of UE specific PDCCH scheduling data. Referring to discussions in Section 2.1, frequent UE specific PDCCH monitoring is the basis of multiple PDSCH starting positions, e.g. PDCCH monitoring every 2 symbols as shown in Figure 2. Thus, Alt. 1-1-1 and Alt. 1-1-2 may occur in multiple symbol positions so that UE needs to monitor multiple positions within one slot. Similarly, for Alt. 1-2-1 and Alt. 1-2-2, the DM-RS monitoring position is coupled with search space of GC-PDCCH which could be flexibly configured since it is not scheduling data. For Alt. 2, CSI-RS could also be configured flexibly by RRC signalling. For Alt. 3, in order to avoid misunderstanding with PSS/SSS in SSB, there will be some limitation, e.g. better not to be used in DRS window. In general, there will be some restrictions for Alt. 1-1-1, Alt. 1-1-2 and Alt. 3 in time domain;

-	In frequency domain, wideband DM-RS has obvious limitation since it needs to span the whole configured CORESET even when PDCCH is in part of it. As a result, PDCCH multiplexing between multiple UEs is limited in the CORESET, i.e. only PDCCH for the UEs with the same beamforming and configured scrambling ID could be multiplexed. This restriction will apply to both Alt. 1-1-2 and Alt. 1-2-2. For Alt. 3, PSS/SSS’s transmission bandwidth is 7.2MHz, which is not meeting OCB requirement in unlicensed band. Besides, PSS/SSS may mislead cell search UEs when they coincide with sync raster. Thus, there will some restrictions for Alt. 1-1-2, Alt. 1-2-2 and Alt. 3 in frequency domain.

From information aspect, although the DM-RS sequence is fixed according to PDCCH configuration and transmission time and can’t convey additional information, the coupled PDCCH could deliver some information to the UE(s). For Alt. 1-1-1 and Alt. 1-1-2, the coupled UE specific PDCCH could convey additional information to one scheduled UE only; For Alt. 1-2-1 and Alt. 1-2-2, the coupled GC-PDCCH could convey information to a group of UEs as also described in Section 2.3. CSI-RS and PSS/SSS could carry information bits by different sequences to all UEs, which will degrade the detection performance. From the evaluation result, carrying 2bits information along with CSI-RS and SSS is still acceptable in terms of performance. Thus, Alt. 2 and 3 is feasible to carry small information bits to the UEs.

Based on the above analysis and summary in Table 2, Alt. 1-2-1, Alt. 1-2-2 and Alt. 2 are better candidates for DL burst detection signal. Whether or which RS is used for DL burst detection is up to UE’s implementation depending on the configuration. For example, if one slot is configured with GC-PDCCH, the DM-RS in GC-PDCCH could be used by UE for DL burst detection. Then the following proposal is made:

Proposal 5: DL burst detection is up to UE’s implementation based on network configurations and there is no need to discuss this in RAN1 anymore.

DM-RS design Type B PDSCH in NRU case

In NR Rel-15, for PUSCH Type B mapping, when single-symbol is configured, all length of symbols within a slot are supported; when double-symbol is configured, length of symbols larger than 4 are supported. This brings UL scheduling flexibility and UL transmission efficiency. However, for PDSCH Type B mapping, when single-symbol is configured, only lengths of 2, 4, and 6(ECP)/7(NCP) symbols are supported; when double-symbol is configured, only lengths of 6(ECP)/7(NCP) symbols are supported. This will restrict gNB’s DL scheduling and thus lead to DL transmission inefficiency.

It was agreed to support NR PDSCH Type B mapping with the length of symbols from 2 to 13 for NRU case in RAN1 #97 meeting. For DMRS enhancements for NR PDSCH Type B mapping in NRU case, at least the following aspects should be taken into account for DMRS position design,

- Reuse NR Rel-15 PDSCH/PUSCH DMRS positions as much as possible.

- The last additional DMRS symbol(s) position should locate as near to the end as possible to make better DMRS demodulation performance.

- Reserve 1 or 2 OFDM symbols for UE processing to achieve early demodulation.

- Support up to 3 additional DMRS symbols for single-symbol DMRS case, and up to 1 additional DMRS for double-symbol DMRS case.

- Support both NCP and ECP cases.

Based on above principles, for PDSCH mapping Type B, DMRS positions for single-symbol DMRS case and double-symbol DMRS case are shown in Table1 and Table2 respectively.

Table1. DMRS positions for single-symbol DMRS

| Length of scheduling symbols | PDSCH mapping type B DMRS positions |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  | , 4 | , 4 | , 4 |
| 7 |  | , 4 | , 4 | , 4 |
| 8 |  | , 6 | , 3, 6 | , 3, 6 |
| 9 |  | , 6 | , 3, 6 | , 3, 6 |
| 10 |  | , 8 | , 4, 8 | , 4, 8 |
| 11 |  | , 8 | , 4, 8 | , 3, 6, 9 |
| 12 |  | ,10 | , 5, 10 | , 3, 6, 9 |
| 13 |  | ,10 | , 5, 10 | , 3, 6, 9 |
| 14 | - | - | - | - |

Table2. DMRS positions for double-symbol DMRS

| Length of scheduling symbols | PDSCH mapping type B DMRS positions |  |
| --- | --- | --- |
|  | 0 | 1 |
| <4 | - | - |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |
| 8 |  | ,5 |
| 9 |  | ,5 |
| 10 |  | ,7 |
| 11 |  | ,7 |
| 12 |  | ,9 |
| 13 |  | ,9 |
| 14 | - | - |

Proposal 6: For PDSCH Type B mapping, support DMRS positions Table1 and Table2 for single-symbol DMRS case and double-symbol DMRS case respectively.

DM-RS for Type B PDSCH in DSS case

In the case of LTE and NR dynamic spectrum sharing scenario, the scheduling restriction on PDSCH Type B mapping in NR Rel-15 will cause unutilized OFDM symbols within a slot. As the example shown in Figure1 with system configuration parameters listed in Table3, the last 2 OFDM symbols may not be utilized.

Figure 6  Example of LTE and NR dynamic spectrum sharing

Table 3. Example of LTE and NR dynamic spectrum sharing configurations

| Configurations | LTE | NR |
| --- | --- | --- |
| SCS | 15KHz | 15KHz |
| Control channel symbols | 2 | 2 |
| DL scheduling symbols | 14 | Type B mapping with length 7 |
| Demodulation RS | CRS with 4 ports | Single-symbol DMRS |

In LTE and NR dynamic spectrum sharing scenario, at least the following 3 cases in Table4 were identified to be handled. From the perspective of network deployment, collision between LTE CRS and DMRS for NR PDSCH Type B mapping should be avoided. However, a common DMRS position design is preferred from system design perspective. If we just reuse above Table1 and Table2 for LTE and NR dynamic spectrum sharing scenario, it shall lead to collision between DMRS for NR PDSCH Type B mapping and LTE CRS as also depicted in Table4.

Table 4. Use cases of LTE and NR dynamic spectrum sharing

| Use cases | System configurations | Collision configurations |
| --- | --- | --- |
| Case1 | LTE 15KHz, 4 ports CRS, 2 symbols control channel NR 15KHz, PDSCH Type B mapping with symbol length 9, 2 symbol PDCCH | - Single-symbol DMRS, 2 and 3 additional DMRS - Double-symbol DMRS, 1 additional DMRS |
| Case2 | LTE 15KHz, 4 ports CRS, 2 symbols control channel NR 30KHz, PDSCH Type B mapping with symbol length 9, 1 symbol PDCCH | - Single-symbol DMRS, 3 additional DMRS |
| Case3 | LTE 15KHz, 4 ports CRS, 2 symbols control channel NR 30KHz, PDSCH Type B mapping with symbol length 10 | - Single-symbol DMRS, 3 additional DMRS |

As the scheduling of NR PDSCH Type B mapping is very flexible, if we just rate match the collided DMRS symbols, it shall degrade demodulation performance. If we just shift the collided DMRS symbols, it might increase processing complexity for both network and UE sides. Probably the proper way is to comprise both of them as depicted in Table5.

Table5. DMRS positions for dynamic spectrum sharing

| Use cases | DMRS positions figure | DMRS position |
| --- | --- | --- |
| Case1 |  | Single-symbol DMRS - 1 additional DMRS, {0,7} - 2 additional DMRS, {0,4,7} - 3 additional DMRS, {0,4,7} |
|  |  | Double-symbol DMRS, - 1 additional DMRS, {0, 5}, the additional DMRS will be dropped due to collision |
| Case2 |  | Single-symbol DMRS - 1 additional DMRS, {0,7} - 2 additional DMRS, {0,4,7}, - 3 additional DMRS, {0,4,7} For 2 and 3 additional DMRS, the 1st additional DMRS will be dropped due to collision. |
|  |  | Double-symbol DMRS, - 1 additional DMRS, {0, 5} |
| Case3 |  | Single-symbol DMRS - 1 additional DMRS, {0,8} - 2 additional DMRS, {0,4,8} - 3 additional DMRS, {0,4,8} For 2 and 3 additional DMRS, the 1st additional DMRS will be dropped due to collision. |
|  |  | Double-symbol DMRS, - 1 additional DMRS, {0, 7} |

Proposal 7: For NR PDSCH Type B mapping in dynamic spectrum sharing scenario, support DMRS positions Table6 and Table7 for single-symbol DMRS case and double-symbol DMRS case respectively.

Table6. DMRS positions for single-symbol DMRS in dynamic spectrum sharing scenario

| Length of scheduling symbols | PDSCH mapping type B DMRS positions |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 9 |  | , 7 | , 4, 7 | , 4, 7 |
| 10 |  | , 8 | , 4, 8 | , 4, 8 |

Table7. DMRS positions for double-symbol DMRS in dynamic spectrum sharing scenario

| Length of scheduling symbols | PDSCH mapping type B DMRS positions |  |
| --- | --- | --- |
|  | 0 | 1 |
| 9 |  | ,5 |
| 10 |  | ,7 |

- Conclusion

In this contribution, we focus on the design of DL signals and channels for NRU, and have the following proposals:

Proposal 1: SFI is updated to indicate COT structure with at least the following enhancement:

- COT ending indication

  - New ‘E’ state symbol indicates the COT ending or

  - Additional field indicates COT ending symbol position

- Indicated ‘F’ symbols inside COT:

  - ‘F’ symbols between ‘D’ and ‘U’ symbols inside COT: switching symbols with no DL and UL transmissions

  - Otherwise: symbols not decided as DL or UL, maybe overwritten by following COT indication

Proposal 2: NR-U supports dynamic PDCCH monitoring by explicit signalling.

Proposal 3: GC-PDCCH could deliver information on PDCCH monitoring indicator as well as COT information.

Proposal 4: UE needs to determine whether configured P/SP CSI-RS is transmitted or not before performing measurement.

Proposal 5: DL burst detection is up to UE’s implementation based on network configurations and there is no need to discuss this in RAN1 anymore.

Proposal 6: For PDSCH Type B mapping, support DMRS positions Table1 and Table2 for single-symbol DMRS case and double-symbol DMRS case respectively.

Proposal 7: For NR PDSCH Type B mapping in dynamic spectrum sharing scenario, support DMRS positions Table6 and Table7 for single-symbol DMRS case and double-symbol DMRS case respectively.

- References

- RP-182878, New WID on NR-based Access to Unlicensed Spectrum, Dec. 2018.

- Chairman’s notes, RAN1 #97, Reno, USA, May 13th – 17th, 2019

- TR 38.889, Study on NR-based Access to Unlicensed Spectrum, Dec. 2018.

- Annex-Simulation parameters

Table 2 	Simulation parameters for miss-detection performance of DL burst detection signal

| Simulation parameters | values |
| --- | --- |
| Bandwidth | 20 MHz |
| Channel model | TDL-C Low 100ns 10Hz 1X2 |
| Subcarrier spacing | 30 kHz |
| Target false alarm rate for CSI-RS/SSS | 1% |
| Number of RS sequence candidates for CSI-RS/SSS | 1 and 4 candidate sequences for 1bits and 2bits carried by RS sequence, respectively |
| CORESET BW | 48 RB |
| PDCCH Aggregation Level | 2/4/8 |