3GPP TSG RAN WG1 Meeting #98							R1-1908154

Prague, CZ, August 26th – 30th, 2019

Source:	vivo

Title:	Physical layer procedure for NR sidelink

Agenda Item:	7.2.4.5

Document for:	Discussion and Decision

- Introduction

The V2X WI has been approved in RAN#83 meeting [1]. The sidelink physical layer procedures, including the HARQ procedures, CSI acquisition for unicast, and Power control, should be defined in Rel-16.

- Sidelink physical layer procedures as per the study outcome - HARQ procedures [RAN1, RAN2] - CSI acquisition for unicast [RAN1] - CQI/RI reporting is supported and they are always reported together. No PMI reporting is supported in this work. Multi-rank PSSCH transmission is supported up to two antenna ports. - In sidelink, CSI is delivered using PSSCH (including PSSCH containing CSI only) using the resource allocation procedure for data transmission. - Power control [RAN1, RAN2]

In this contribution, we provide our view on the discussion points for the design of sidelink physical layer procedures.

- Physical layer ID

It has been agreed during the study phase that L1 source and destination IDs are included in SCI. The remaining issue is how to derive these IDs. According to the SA2 agreement, for unicast and groupcast communications, the source and destination L2 IDs are provided by V2X layer to AS layer. Therefore, it is straightforward to derive the L1 IDs from the L2 IDs. The actual number of bits for these L1 IDs can be determined during WI phase, together with the SCI design.

Proposal 1: The L1 source and destination IDs are derived from the L2 IDs.

- HARQ procedure

Some remaining aspects for HARQ procedure are discussed in this section.

  - HARQ feedback operation

HARQ feedback is enabled or disabled by (pre-) configuration according to the QoS requirement of the associated traffic and service, as the following agreement in the RAN1 AH1901 meeting [2].

Agreements: - (Pre-)configuration indicates whether SL HARQ feedback is enabled or disabled in unicast and/or groupcast. - When (pre-)configuration enables SL HARQ feedback, FFS whether SL HARQ feedback is always used or there is additional condition of actually using SL HARQ feedback

There is some discussion on whether additional condition is needed to enable the HARQ feedback. Some potential conditions include the changes in QoS control and congestion level. However, in our view, these conditions do not vary in short-term, therefore can be handled by higher layer configuration. Moreover, dynamically disabling HARQ may affect the achievable QoS of the service, which should be not done autonomously in physical layer.

Proposal 2: Sidelink HARQ feedback is always used if enabled by (pre-)configuration. No additional condition is defined in physical layer to enable/disable sidelink HARQ feedback.

  - HARQ feedback for groupcast

Both HARQ ACK/NACK and HARQ NACK only are supported for groupcast in NR V2X in the previous RAN1 meeting [3].

Agreements: - Confirm the following working assumption: - Working assumption: - When HARQ feedback is enabled for groupcast, support (options as identified in RAN1#95): - Option 1: Receiver UE transmits only HARQ NACK - Option 2: Receiver UE transmits HARQ ACK/NACK - Note: RAN1 has not concluded the respective applicability of option 1 vs. option 2 yet Agreements: - In HARQ feedback for groupcast, - When Option 1 is used for a groupcast transmission, it is supported - all the receiver UEs share a PSFCH - FFS: a subset of the receiver UEs share a PSFCH - FFS: all or a subset of receiver UEs share a pool of PSFCH. - When Option 2 is used for a groupcast transmission, it is supported - each receiver UE uses a separate PSFCH for HARQ ACK/NACK. - FFS: all or a subset of receiver UEs share a PSFCH for ACK transmission and another PSFCH for NACK transmission - FFS on which entity and how to allocate PSFCH resource to the receiver UE(s) - FFS whether or not to additionally support a mixture of option 1 and option 2 for a groupcast transmission - Note: Each PSFCH is mapped to a time, frequency, and code resource.

There are some remaining open issues to be resolved. For Option 1, it is proposed that, in the case of the transmitter UE has established unicast connection to some of the receiver UEs in the group, it is possible that dedicated PSFCH can be used for them to send HARQ feedback for groupcast, while all the other UEs share the same PSFCH for NACK feedback. It may be beneficial to avoid DTX for these UEs; however, it increases the complexity of the transmitter UE due to higher decoding burden of PSFCH. Moreover, as pointed out in our simulation results [4], the DTX is not a critical issue in this case.

Another issue is whether all or a subset of receiver UEs share a pool of PSFCH. It may require multiple PSFCH resources to be configured and used for receiver UEs, which not only increases the system overhead but also degrades the resource utilization in the case of only a few receiver UEs with low traffic load. Further, the benefit is not clear for this proposal.

Proposal 3: In HARQ feedback for groupcast, when Option 1 is used for a groupcast transmission, all the receiver UEs share a PSFCH. It is not necessary to support a pool of PSFCH, or only a subset of the receiver UEs sharing a PSFCH.

For Option 2, each receiver UE sends HARQ ACK or NACK respectively for success or failure of each reception. A variation of Option 2 is that all or a subset of the receiver UEs share a PSFCH for ACK and another PSFCH for NACK. The benefit of this scheme is not clear, as the DTX issue may still occur in this case. On the other hand, given that the PSFCH is configured by transmitter for each receiver UE, it may be supported by implementation without specification change.

Proposal 4: In HARQ feedback for groupcast, when Option 2 is used for a groupcast transmission, the variation that all or a subset of the receiver UEs share a PSFCH for ACK and another PSFCH for NACK can only be supported if no specification change is required.

  - Distance based HARQ feedback

The working assumption and agreement reached in previous meeting [3] are at least supporting the Tx-Rx geographical distance, and FFS the support of L1-RSRP.

Working assumption: - Regarding the use of TX-RX geographical distance and/or RSRP in determining whether to send HARQ feedback for groupcast - Support at least the use of TX-RX geographical distance - FFS whether or not to additionally use L1-RSRP - Companies are encouraged to perform additional evaluations/analysis Agreements: - For at least option 1 based TX-RX distance-based HARQ feedback for groupcast, - A UE transmits HARQ feedback for the PSSCH if TX-RX distance is smaller or equal to the communication range requirement. Otherwise, the UE does not transmit HARQ feedback for the PSSCH - TX UE’s location is indicated by SCI associated with the PSSCH. - Details FFS - The TX-RX distance is estimated by RX UE based on its own location and TX UE location. - The used communication range requirement for a PSSCH is known after decoding SCI associated with the PSSCH - FFS implicit or explicit - FFS how to define location

- RSRP measurement based HARQ

Compared to the geographical distance, RSRP based HARQ feedback is more straightforward. RSRP is already widely used in physical layer, for example, in the sensing procedure. Therefore, no additional complexity is required.

Moreover, due to the coverage hole, the geographical position information may not always be available. On the contrary, the RSRP is more robust as it is always available once the UE receives the transmission. In this case, the receiver can decide whether to send feedback via the RSRP measurement result without any valid position information about the transmitter.

Observation 1: The RSRP based scheme is necessary for distance based HARQ feedback in the coverage hole.

The only concern to the RSRP based solution is that due to the transmitter side power control, the receiver UE may not have an accurate estimation of pathloss because it does not know the Tx power of the transmitter UE. However, it may not be an issue, as the pathloss estimation can be done simultaneously in the transmitter UE. Moreover, it can also be resolved by signaling the Tx power or pathloss from the transmitter UE to the receiver UE.

Observation 2: For the RSRP based scheme, the pathloss estimation can be either measured at the receiver UE, or signaled from the transmitter UE.

System level simulations are executed to evaluate the performance of TX-RX distance based and RSRP based solution for HARQ feedback. The simulation results for highway scenario and urban scenario are shown from Figure 1 to Figure 4, respectively. Simulation assumptions are provided in Annex A. The distance for feedback in simulation is 320m and 150m for highway and urban scenarios respectively.

Figure 1 Average PRR for distance based and RSRP based schemes in Highway scenario

Figure 2 Average packet loss rate due to half-duplex issue in Highway scenario

Figure 3 Average PRR for distance based and RSRP based schemes in Urban scenario

Figure 4 Average packet loss rate due to half-duplex issue in Urban scenario

In the highway scenario, it can be observed that in the nearby region (i.e. around 0 ~ 300 meters), a lower RSRP threshold results in a higher packet loss rate.

In contrast, for the far-end UEs, the lower the RSRP threshold, the higher the PRR performance can achieve. Therefore, it shows a clear tradeoff of performance between the nearby and far-end UE. The reason for this phenomenon is obvious. A lower RSRP threshold results in a larger HARQ feedback coverage where the receiver should send NACK. The increasing number of far-end receiver will inevitably increase the probability of retransmission, which in turn improves the PRR performance of the far-end receiver by HARQ combining gain. However, the average PRR degrades for the nearby UEs, because the packet loss rate due to half-duplex issue is increased as the number of transmission increases, which can be observed in Figure 2. This principle is true also for the TX-RX distance based scheme. The similar phenomenon can also be observed in the urban scenario from Figure 3 and Figure 4.

On the other hand, a turning point can be observed for the TX-RX distance based scheme at the point of around 320m and 150m for both scenarios due to the fixed feedback coverage. Nevertheless, this turning point has no significant contribution to the PRR performance.

According to the evaluation results, the RSRP based schemes with RSRP thresholds of -75dBm and -85dBm have slightly better performance than that of TX-RX distance based scheme in both highway and urban scenarios. Nevertheless, the gain is not significant. The largest performance gain of PRR between the TX-RX distance based scheme and the RSRP based schemes with RSRP thresholds is around 2% to 3%.

Observation 3: Comparable performance between RSRP based scheme and TX-RX distance based scheme can be observed in both Urban and Highway scenarios.

Therefore, the RSRP based HARQ feedback should be supported for the groupcast.

Proposal 5: The RSRP based HARQ feedback is supported for groupcast.

- TX-RX geographical distance based HARQ

From a signalling perspective, it is not desirable to apply the geographical position information (e.g., GNSS position information) directly in physical layer due to large signalling overhead. Some companies propose to use “quantized” position information, e.g. similar to the geographical zone concept used in LTE V2X.

As a solution, the resolution of the geographical zone can be (pre-)configured. The geographical zone id of TX UE and the communication range requirement can be carried in SCI, and then RX UE can calculate the distance to the TX UE according to the information in SCI and its own geographical zone id. The communication range requirement can be indicated in different methods like quantized distance value or the geometrical relationship of the geographical zones between TX UE and RX UE.

This scheme is straightforward. Nevertheless, the increased system overhead is a concern. There is a tradeoff between the signaling overhead and the accuracy of positioning at determining the resolution of the zone. A large number of zones increases the positioning accuracy at the cost of larger signaling overhead, while a small number of zones may increase the number of unnecessary NACK feedback, which in turn results in increasing of unnecessary retransmission and higher resource collision probability. Moreover, the optimal resolution of zone may vary in different scenarios (highway, urban, etc.), or under different QoS requirements (e.g., communication range).

As a result, it seems challenging to design a proper zone resolution applicable to all the scenarios. An alternative way is to define a quantized zone size as small as possible that can be used in every scenario.  However, it brings larger signaling overhead.

Proposal 6: Tradeoff between signaling overhead and positioning accuracy should be considered for the design of TX-RX geographical distance based scheme.

Consequently, there are two issues in quantized position scheme according to the analysis above:

- Issue 1: It is difficult to design a proper zone resolution adapted to all the scenarios.

- Issue 2: Signaling overhead is large if a high resolution is used for better positioning accuracy.

The issues mentioned above can be mitigated when the size of the geographical zone varies with the communication range requirement. In this case, the receiver UE would feedback only if it is in the same zone as the transmitter UE or the adjacent zones from the transmitter UE. For issue 1, the optimal resolution would be determined dynamically for each transmission, which is no longer constrained by the scenario and QoS parameters. An example is shown in Figure 5 and Figure 6, where the zone size equals to half of the range requirement can lead to a higher ranging accuracy, which can further eliminate unnecessary HARQ NACK feedback. As a result, the transmitter UE can select preferable resolution to reach a higher accuracy, which is more flexible for different scenarios and service requirements.

| Figure 5 The length of a zone equals to the communication range | Figure 6 The length of a zone equals to half of the communication range |
| --- | --- |

Issue 2 can also be mitigated via the dynamic zone size scheme. The largest and smallest minimum required communication range defined in 22.186 are 1000m and 50m, respectively. When the resolution is pre-configured, the zone size cannot be larger than the smallest range value (i.e. 50ms), in order to guarantee the accuracy for all the scenarios and QoS requirements. Based on this assumption, the total SCI overhead of zone ID and communication range requirement is about 15bits.

Figure 7 The pathloss curves in different scenario

However, such overhead is too large for SCI and can be reduced by further investigation. Firstly, the scenario requiring a very large communication range (e.g., up to 1000m) has high probability under the excellent channel condition like highway, where the pathloss retains at a relatively small level and increases slightly with the distance as shown in Figure 7. In this case, a small value of zone size (e.g., 50m) is not required at all, and is undesirable due to the large SCI overhead. A larger zone size is more desirable (e.g., 250m) to reduce the zone ID costs (e.g., around 5 bits assuming 250m zone size).

On the contrary, the scenario requiring smaller communication range (e.g., 50m or 100m) has high probability in a high-loss channel condition. Figure 7 also shows the curve of pathloss in the urban scenario. The pathloss exceeds 130dB when the distance is larger than 400m, where the receiver has difficulties in decoding the signal. Unnecessary retransmission due to the “warp-around” zone ID (issue discussed in our contribution in [5]) can be avoided in this case using less number of zone ID. For example, the cost of zone ID can be reduced to about 6bits when 400m is selected as the critical point.

Based on the analysis above, the signaling overhead for zone ID can be reduced (e.g., down to about 5~6 bits) if the zone size varies according to the scenarios and requirements. The transmitter UE can select preferable resolution according to the communication range requirement.

For the indication of communication range in SCI, 4bits is sufficient according to the number of range values defined in 22.186. Therefore, the total overhead can be reduced compared to the pre-configured scheme.

Observation 4: It is more flexible to adapt to different scenario and service requirement when the zone size varies according to the communication range requirement.

Observation 5: The signaling overhead is reduced when the zone size varies according to the communication range requirement.

The dynamic zone size determination associated with the communication range requirement is more flexible and robust. It not only brings much benefit to the diversity transmission, which is no longer constrained by scenario and QoS parameter, but also saves the overhead that position information cost. Hence, the dynamic zone size determination associated with the communication range requirement should be supported.

Proposal 7: The dynamic zone size determination associated with communication range requirement is supported.

  - HARQ-ACK resource determination

Following agreements were reached related to HARQ-ACK resource determination.

Agreements: - (Pre-)configuration indicates the time gap between PSFCH and the associated PSSCH for Mode 1 and Mode 2. Agreements: - It is supported, in a resource pool, that within the slots associated with the resource pool, PSFCH resources can be (pre)configured periodically with a period of N slot(s) - N is configurable, with the following values - 1 - At least one more value >1 - FFS details - The configuration should also include the possibility of no resource for PSFCH. In this case, HARQ feedback for all transmissions in the resource pool is disabled - HARQ feedback for transmissions in a resource pool can only be sent on PSFCH in the same resource pool Agreement: - For the period of N slot(s) of PSFCH resource, N=2 and N=4 are additionally supported. Agreements: - At least for the case when the PSFCH in a slot is in response to a single PSSCH: - Implicit mechanism is used to determine at least frequency and/or code domain resource of PSFCH, within a configured resource pool. At least the following parameters are used in the implicit mechanism: - Slot index (FFS details) associated with PSCCH/PSSCH/PSFCH - Sub-channel(s) (FFS details) associated with PSCCH/PSSCH - Identifier (FFS details) to distinguish each RX UE in a group for Option 2 groupcast HARQ feedback - FFS detailed applicability of the above parameters - FFS: Other parameters (e.g. SL-RSRP/SINR, Layer-1 source ID, location information, etc.)

Based on the agreement, it can be assumed that a set of PSSCH resource(s) (i.e., one sub-channel and one slot) can be associated with a PSFCH occasion (containing a set of PSFCH resource(s)), it is noted that the set of PSSCH resource(s) is comprised of PSSCH resource(s) from N slot(s), where N is the PSFCH occasion period. As illustrated in Figure 8, PSSCHs in slot#(n-1), #(n-2), #(n-3), #(n-4) are associated with PSFCH occasion in slot#n.

Figure 8 A set of PSSCH associated with a PSFCH occasion

At the RAN1#97 meeting, it was agreed to use implicit mechanism to determine at least frequency and/or code domain resource of PSFCH corresponding to a single PSSCH resource [5]. To achieve the implicit mechanism, two aspects can be further considered: 1) association between PSSCH resource and PSFCH resource; 2) Based on the association relation, RX UE derives corresponding PSFCH resource in response to a certain PSSCH transmission. In the following discussion, the two aspects will be discussed in details.

- Association between PSSCH resource and PSFCH resource

Firstly, the association rule between a PSSCH resource and a PSFCH resource(s) in the corresponding PSFCH occasion can be defined or (pre-)configured. For the simplicity of association, a PSSCH resource in a certain sub-channel is always associated with a PSFCH resource(s) confined in the same sub-channel.

Moreover, a PSSCH resource should correspond to multiple PSFCH resources. This is necessary at least to support option-2 based groupcast, where each receiver UE feedbacks in a separate PSFCH resource. Regarding the detailed mapping relation between a PSSCH resource and PSFCH resource(s), it can be defined after confirming the principle of the mapping rule.

Proposal 8: A PSSCH is associated with multiple PSFCH resource(s) in the same set of sub-channel.

- PSFCH resource derivation by RX UE

RX UE determines PSFCH resource in response to a PSSCH transmission based on the association relation. For different transmission types, the method for RX UE to derive its PSFCH resource can be different.

If RX UE receives a unicast transmission on a PSSCH resource, it performs HARQ-ACK feedback on one of the multiple corresponding PSFCH resource(s). More specifically, it can derive the exact PSFCH resource based on layer-1 source ID. The benefit to involve layer-1 source ID is to randomize interference on PSFCH and to avoid TX UE misunderstanding if more than one RX UEs in proximity feedback HARQ-ACK in response to different data transmissions using the same PSSCH resource.

Proposal 9: For unicast transmission, RX UE derives PSFCH resource among a set of PSFCH resource(s) based on the layer-1 source ID for the corresponding PSSCH transmission.

If RX UE receives an option-1 based groupcast transmission, RX UE performs HARQ-ACK feedback on one of the multiple corresponding PSFCH resource(s). Given that only one single NACK feedback may be sent for each TB, a similar approach for unicast can be reused, e.g., RX UE derives the PSFCH resource based on layer-1 source ID. However, it is necessary to avoid the case that a PSFCH resource for unicast is CDM’ed with a PSFCH resource for option-1 based groupcast. Otherwise, the accumulated power on the PSFCH resource from a large number of group members may degrade the decoding performance of the CDM’ed unicast PSFCH. Therefore, the set of PSFCH resources for option-1 based groupcast transmission should be separated from that for unicast transmission.

Proposal 10: For option-1 based groupcast transmission, RX UE derives PSFCH resource among a set of PSFCH resource(s) based on the layer-1 source ID, similarly as the unicast case. However, the PSFCH resources for unicast and option-1 based groupcast transmissions should not be multiplexed in the same PRB(s).

If RX UE receives an option-2 based groupcast transmission, it needs to selects HARQ-ACK feedback resource based on its in-group ID, where dedicated in-group ID is assigned per UE in a groupcast group by high layer. Assuming that number of PSFCH resource in the PSFCH resource set (denoted as M) corresponds to a PSSCH transmission is more than the number of UEs in the group, a UE can derive the HARQ-ACK feedback resource as m-th resource in a PSFCH resource set corresponding to a given PSSCH transmission, where m = in-group ID mod M.

Proposal 11: For option-2 based groupcast transmission, RX UE derives PSFCH resource among a set of PSFCH resource(s) based on its in-group ID configured by high layer.

  - PSFCH transmission/reception overlap

At the RAN1#97 meeting, it was concluded to further study the following cases for PSFCH transmission and reception overlap.

Conclusion: - Study further whether/how to handle/avoid the following cases for PSFCH transmission and reception: - Case 1 (PSFCH TX/RX overlap): A UE transmitted a PSSCH and received SCI scheduling another PSSCH where PSFCH resources corresponding the two PSSCHs appear in the same slot. - Case 2 (PSFCH TX to multiple UEs): A UE received SCI from different UEs and the associated PSFCHs appear in the same slot. - Case 3 (PSFCH TX with multiple HARQ feedback to the same UE): A UE received multiple SCI from the same UE and the associated PSFCHs appear in the same slot.

For case 1, due to half-duplex constraint, a UE cannot transmit and receive PSFCHs simultaneously. There are generally two directions to address such case: 1) UE resource selection to avoid PSFCH transmission/reception collision; 2) when PSFCH TX/RX overlap occurs, the UE either drops PSFCH transmission or stops PSFCH reception. For 1), UE cannot always predict the need of PSFCH transmission, because PSSCH reception may occur at any time. Hence, it is challenging to optimize resource selection to solve PSFCH half-duplex issue. Therefore, direction 2) is preferred, i.e., UE can drop some PSFCH transmission/reception when overlap occurs.

For case 2, when UE needs to feedback multiple PSFCHs to different UEs, due to UE capability constrain or power limitation (e.g., limited total TX power or high PAPR for simultaneous PSFCH transmissions) it may not support simultaneous PSFCH transmissions. In such case, UE can drop some of the PSFCH transmission(s).

For case 3, although it is similar to case 2, whether to use PSFCH dropping needs to be carefully studied. In case 3, destination of multiple PSFCH transmission is the same UE, so HARQ-ACK bundling can be used to reflect a more complete PSSCH decoding results, instead of merely dropping the HARQ feedback.

Proposal 12: For PSFCH transmission and reception overlapping case 1 and 2, the UE is allowed to drop some of the PSFCH transmission(s)/reception(s). On the other hand, for case 3 HARQ-ACK bundling is applied.

  - HARQ process

The TX/RX process number should be restricted for a reasonable UE complexity. Therefore, the maximum HARQ process number needs to be determined. In NR Uu, the maximum TX/RX HARQ process number is 16, which can be a baseline for determination of SL TX/RX HARQ process number.

In the Uu interface, each UE has only a single connected link, i.e., connection between base station and UE. However, in the SL interface, each UE may have multiple connected links with different UEs for unicast transmission. Besides, there are groupcast and broadcast as well. Assuming different unicast links, different groupcast UE groups and broadcast transmissions share the total SL HARQ processes, the HARQ process number needs to be limited per unicast or groupcast link. This is because, when a UE is connected to multiple other UEs, if each link corresponding to a UE pair (including this UE) assumes maximum HARQ process number, simultaneous transmission of these multiple UE pairs will incur HARQ process overload. The maximum HARQ process number of each link can be determined during PC5-RRC link establishment, as also discussed in section 6.1.

Proposal 13: The maximum HARQ process number of a UE should be determined for sidelink. The maximum HARQ process number per link can be determined during PC5-RRC link establishment.

- CSI acquisition

Agreements: - Support at least Sidelink CSI-RS for CQI/RI measurement - Sidelink CSI-RS is confined within the PSSCH transmission

At the RAN1#96bis meeting, the above was agreed, which means that no stand-alone CSI-RS transmission is allowed in Rel-16 NR SL. Consequently, aperiodic CSI-RS transmission is a straightforward assumption. To indicate the absence/presence of the aperiodic CSI-RS, 1 bit in SCI can be used.

Proposal 14: 1 bit in SCI is used to indicate the presence of CSI-RS on the associated PSSCH.

In response to the aperiodic CSI-RS transmission, aperiodic CSI report can be supported as well. Regarding the resource for CSI report, it is preferred that RX UE selects resource for CSI feedback. Compared with the other alternative where CSI feedback resource is selected by TX UE, less spec impact on resource selection and reservation procedure is expected, and less SCI overhead can be expected because resource reservation for the CSI transmission needs to be indicated when TX UE performs the resource selection.

Moreover, for RX UE to select CSI transmission resource, the selected resource should be constrained within a time window after receiving CSI-RS to guarantee the CSI accuracy.

Proposal 15: Support RX UE to select PSSCH resource for CSI report.
-    FFS: resource selection window for CSI transmission resource selection considering CSI report accuracy.

Based on the SL CSI-RS transmission and CSI report discussion, the CSI (including CQI and RI) report would be appropriate for short-term link adaptation for the following unicast transmission(s) between the UE pairs. Nevertheless, how the TX UE can perform link adaptation needs to be further studied and concluded, e.g., whether UE implementation can determine the related transmission parameters based on CSI report.

Proposal 16: Whether UE implementation can adapt transmission parameter based on CSI report needs to be concluded in RAN1.

- Power control

Open-loop power control scheme is supported in NR sidelink. The Rel-14 LTE downlink pathloss based power control is reused for NR unicast, groupcast and broadcast transmissions in the network coverage. Additionally, the open-loop power control based on the pathloss between Tx UE and Rx UE in sidelink is also supported at least for unicast [2].

  - Power control for unicast

In this section, some remaining aspects of supporting open-loop power control for unicast are discussed.

- RS for RSRP measurement

For open-loop power control, the transmitting UE derives the SL pathloss estimation from an SL RSRP reported by the receiving UE. One open issue is that which reference signal is used to obtain SL RSRP. In the NR downlink, the Uu pathloss can be derived from the RS of SSB or periodic CSI-RS configured by the serving gNB. However, these reference signals are not suitable for RSRP measurement in NR sidelink.

Firstly, unlike the downlink SSB where the PCI can be used to identify the gNB, the SLSS ID in the S-SSB is used to determine the synchronization source. Therefore, it is not possible to identify the UE from the S-SSB, nor perform SL pathloss estimation for a UE. Actually, the RSRP of the S-SSB may be inaccurate at all if more than one UEs transmitting the same S-SSB in the same synchronization resource. Furthermore, if a pair of UEs having the same synchronization source, due to the half-duplex issue they cannot monitor the S-SSB of each other because they always sending the S-SSB at the same time. Consequently, it is not possible to perform SL pathloss estimation based on the S-SSB.

In RAN1 #96, it has been agreed that no standalone RS transmission dedicated to CSI reporting is supported in Rel-16, and CSI reporting is supported based on non-subband-based aperiodic CSI reporting mechanism. Consequently, there is no periodic CSI-RS agreed for NR sidelink, nor perform SL pathloss estimation based on the periodic CSI-RS. In order to support pathloss measurement, reference signal for SL pathloss estimation needs to be studied.

Observation 6: It is not possible to perform SL pathloss estimation based on the SL synchronization RS or periodic CSI-RS.

In [7], it is proposed to use the PSCCH DMRS for sidelink RLM measurement, which can also be considered for sidelink RSRP measurement. Another option is the aperiodic CSI-RS used for CSI acquisition.

Proposal 17: PSCCH DMRS or aperiodic CSI-RS can be used for SL pathloss estimation.

- Physical channels and signals for OLPC

The candidates of physical layer channels and signals for OLPC include at least the PSCCH, PSSCH, PSFCH, S-SSB, and CSI-RS.

At least the OLPC based on the sidelink pathloss between Tx UE and Rx UE is applicable to PSSCH and PSFCH for unicast. On the other hand, the S-SSB is obviously a broadcast transmission, which is used for time and frequency synchronization not only for unicast, but also for groupcast and broadcast. Thus, the sidelink pathloss component is not applicable to the S-SSB for OLPC, while the downlink pathloss should still be used in order to suppress the interference to the Uu interface in the licensed band.

Observation 7: The OLPC based on the sidelink pathloss is applicable to PSSCH and PSFCH.

Observation 8: The OLPC based on the sidelink pathloss is not applicable to S-SSB.

For PSCCH, in order to mitigate the hidden UE problem, the PSCCH should be decoded by all the UE in the proximity for sensing. Therefore, it should be sent in a broadcast manner regardless of whether it is scheduling unicast, groupcast or broadcast transmission. In other words, the sidelink pathloss component may not be applicable for OLPC, either. An example is illustrated in Figure 9. Although OPLC based on the sidelink pathloss between the white car (the Rx UE) and the yellow car (the Tx UE) is used for PSSCH, it should not be used for the PSCCH transmission, so that the black car can detect the transmission and avoid potential collision at the Rx UE side.

Figure 9 Different Tx power between PSCCH and PSSCH for mitigating hidden UE problem

Proposal 18: Sidelink pathloss based OLPC is not applicable to PSCCH regardless of unicast, groupcast or broadcast transmission, nor the CSI-RS if agreed for sidelink measurement.

In RAN1 #97, it has been agreed that total sidelink transmit power is the same in the symbols used for PSCCH/PSSCH transmissions in a slot. Besides, sidelink CSI-RS and/or CSI reporting is also confined within the PSSCH transmission. One remaining issue is how to execute power control when PSSCH multiplex with other physical layer channels and/or signals (i.e. CSI-RS, CSI reporting, etc.). It seems reasonable that the same transmit power should be used for PSSCH symbols with or without other channels/signals.

On the other hand, given that the CSI-RS is agreed for sidelink measurement (CSI acquisition, pathloss estimation, etc.), its transmission power should not be retuned dynamically according to the sidelink pathloss. Otherwise, the measurement result can hardly be stable and accurate.

Proposal 19: In the case when PSSCH is multiplexed with other channels/signals (CSI-RS, CSI/RSRP reporting, etc.), the total sidelink transmit power is the same in the PSSCH symbols with or without other channels/signals.

- SL pathloss coexist with DL pathloss

According to the agreement, both the downlink pathloss and the sidelink pathloss should be considered for compensation in unicast OLPC. The downlink pathloss component is beneficial for mitigating the interference to UL reception at gNB, while the sidelink pathloss component is useful for interference control in sidelink. If both of them are enabled, it is necessary to determine the final pathloss compensation for OLPC.

A straightforward approach is that, the downlink pathloss component is considered as the upper bound for pathloss compensation, while the actual transmission power can be further reduced based on the sidelink pathloss component. However, the problem may occur if the pathloss between the Tx and Rx UEs are significantly larger than the pathloss between the Tx UE and the gNB, as illustrated in Figure 10. In this case, the UE has difficulty to receive and decode the packet due to very low SNR.

Figure 10 The issue of sidelink unicast transmission with OLPC based on DL pathloss

Furthermore, similar issue may also occur for broadcast/groupcast transmission. A UE near the gNB would have a limited coverage compared with that of another UE far from the gNB, which may not be able to meet the minimum required communication range.

Observation 9: For sidelink transmission within the network coverage, using both SL and DL pathloss for OLPC may cause coverage issue when the Tx power given by OLPC is not enough for compensation of SL pathloss.

A TDM based solution can be applied to resolve this issue. UE can perform different OLPC schemes at different time domain resource sets. For example, as illustrated in Figure 11, some unicast sidelink transmissions may be performed with OPLC based on sidelink pathloss only in resource set A, while other sidelink transmissions may be sent with OPLC concerning the downlink pathloss in another resource set B. By this way, the power of transmission in resource set A is not limited by the downlink pathloss, so that the coverage and performance can be satisfied. The network can avoid scheduling uplink transmission in slots of resource set A, in order to mitigate the interference from sidelink to Uu.

Figure 11 Example of the TDM based solution

- Preliminary evaluation results

System level simulation is applied to verify the performance of the TDM based solution. The corresponding evaluation assumption is shown in Table 2 in the Annex A. In this simulation, sidelink pathloss and downlink pathloss based OLPC is modeled. Both highway and urban scenarios are evaluated.

- Baseline scheme:

  - OLPC based on the minimum value of sidelink pathloss and downlink pathloss.

  - Mode-1 resource allocation mechanism.

- TDM based scheme:

  - OLPC based on sidelink pathloss only.

  - Mode-1 resource allocation mechanism. A TDM pattern including resource set A and B as illustrated in Figure 11 is used for OLPC. For the UE whose sidelink pathloss is greater than downlink pathloss, resource set A is used. On the other hand, for the UE whose downlink pathloss is greater than sidelink pathloss, all resources in selection window (including both set A and B) could be used.

System evaluation results of average PRR and average PIR for aperiodic and periodic traffic of above two schemes are as follows.

Highway case:

| Figure 12 Average PRR of periodic traffic | Figure 13 Average PIR of periodic traffic |
| --- | --- |

| Figure 14 Average PRR of aperiodic traffic | Figure 15 Average PIR of aperiodic traffic |
| --- | --- |

Urban case:

| Figure 16 Average PRR of periodic traffic | Figure 17 Average PIR of periodic traffic |
| --- | --- |

| Figure 18 Average PRR of aperiodic traffic | Figure 19 Average PIR of aperiodic traffic |
| --- | --- |

As shown in Figure 12, Figure 14, Figure 16 and Figure 18, the average PRR of TDM based scheme is much better than that of the baseline scheme. Besides, as shown in Figure 13, Figure 15, Figure 17 and Figure 19, the average PIR of TDM based scheme also outperform the baseline scheme. There are about 48% UE in highway case and 26% UE in urban case whose sidelink pathloss is greater than downlink pathloss. By employing the TDM based scheme, these UEs can have a more suitable transmission power to compensate the sidelink pathloss.

Observation 10: The performance of average PRR and average PIR of TDM based scheme is significantly better than that of baseline scheme.

Proposal 20: TDM based scheme is supported for OLPC, where UE can perform individual OLPC schemes in different time domain resource sets based on different pathloss compensation for different sidelink transmission (e.g., using SL pathloss only in one set, while using DL and SL pathloss in another set).

  - Power control for groupcast

It has not been decided whether the Open-loop power control scheme based on the sidelink pathloss is supported for groupcast.

According to the scenario defined by SA1, the typical deployment scenarios for groupcast transmission are platooning and cooperative driving, where the vehicle UEs in the group are in proximity from one to another. Moreover, the radio channel in the group is relatively stable. In this case, the open-loop power control for groupcast can still base on the sidelink pathloss, i.e. between the Tx UE and the farthest Rx UE. In an interference-limited channel, it is beneficial via power control to reduce the interference among different groups of UEs, and achieving higher spatial reuse gain.

Proposal 21: Sidelink pathloss based OLPC is supported for groupcast transmission.

- Link management

  - UE capability

RAN2 has agreed that there will be RRC message defined for UE capability exchange, and possibly communication establishment for unicast and groupcast. From physical layer perspective, the UE capability negotiation should take into account the link management. Given the limited processing capability (as discussed in section 3.6) and hardware resource (e.g., soft buffer), it is obviously not possible for a UE to set up a large number of unicast or groupcast connections, while still maintains the QoS requirements. During link establishment, the UE may negotiate the available hardware resources to be assigned for the link, and simply reject the link establishment in the case of out of hardware resource.

Proposal 22: UE capability negotiation should take into account the limitation of UE processing capability and hardware resource.

  - Multiplexing of unicast, groupcast and broadcast transmissions

Multiple types of transmissions, e.g. unicast, groupcast and broadcast traffics, likely coexist in the system. The most straightforward solution is to configure separate resource pools for unicast, groupcast and broadcast transmission, respectively. However, such a semi-static allocation scheme may lead to resource underutilization in the case of imbalanced traffic load among different transmission types. The resources assigned to one transmission type cannot be dynamically shared with another type. Furthermore, it may lead to configuration of a large number of resource pools according to variable services, which unnecessarily increases the implementation complexity. On the other hand, a unified design for resource allocation of unicast, groupcast and broadcast transmissions can simplify both the specification and implementation. A single SCI format can be achieved to reduce the blind decoding burden for PSCCH, where the L1 destination ID can be used to distinguish the transmission type. For example, one or more specific destination ID(s) may be defined for broadcast messages, similar to the broadcast RNTIs defined in the NR downlink. Therefore, it is preferable that unicast, groupcast and broadcast transmissions share the same resource pool.

Proposal 23: Unicast, groupcast and broadcast transmissions can be multiplexed in the same resource pool.

- Conclusion

In the contribution, we provide our considerations on the detailed procedure for the design of NR sidelink physical layer procedure with the following observations.

Observation 1: The RSRP based scheme is necessary for distance based HARQ feedback in the coverage hole.

Observation 2: For the RSRP based scheme, the pathloss estimation can be either measured at the receiver UE, or signaled from the transmitter UE.

Observation 3: Comparable performance between RSRP based scheme and TX-RX distance based scheme can be observed in both Urban and Highway scenarios.

Observation 4: It is more flexible to adapt to different scenario and service requirement when the zone size varies according to the communication range requirement.

Observation 5: The signaling overhead is reduced when the zone size varies according to the communication range requirement.

Observation 6: It is not possible to perform SL pathloss estimation based on the SL synchronization RS or periodic CSI-RS.

Observation 7: The OLPC based on the sidelink pathloss is applicable to PSSCH and PSFCH.

Observation 8: The OLPC based on the sidelink pathloss is not applicable to S-SSB.

Observation 9: For sidelink transmission within the network coverage, using both SL and DL pathloss for OLPC may cause coverage issue when the Tx power given by OLPC is not enough for compensation of SL pathloss.

Observation 10: The performance of average PRR and average PIR of TDM based scheme is significantly better than that of baseline scheme.

Based on these observations, we have the following proposals.

Proposal 1: The L1 source and destination IDs are derived from the L2 IDs.

Proposal 2: Sidelink HARQ feedback is always used if enabled by (pre-)configuration. No additional condition is defined in physical layer to enable/disable sidelink HARQ feedback.

Proposal 3: In HARQ feedback for groupcast, when Option 1 is used for a groupcast transmission, all the receiver UEs share a PSFCH. It is not necessary to support a pool of PSFCH, or only a subset of the receiver UEs sharing a PSFCH.

Proposal 4: In HARQ feedback for groupcast, when Option 2 is used for a groupcast transmission, the variation that all or a subset of the receiver UEs share a PSFCH for ACK and another PSFCH for NACK can only be supported if no specification change is required.

Proposal 5: The RSRP based HARQ feedback is supported for groupcast.

Proposal 6: Tradeoff between signaling overhead and positioning accuracy should be considered for the design of TX-RX geographical distance based scheme.

Proposal 7: The dynamic zone size determination associated with communication range requirement is supported.

Proposal 8: A PSSCH is associated with multiple PSFCH resource(s) in the same set of sub-channel.

Proposal 9: For unicast transmission, RX UE derives PSFCH resource among a set of PSFCH resource(s) based on the layer-1 source ID for the corresponding PSSCH transmission.

Proposal 10: For option-1 based groupcast transmission, RX UE derives PSFCH resource among a set of PSFCH resource(s) based on the layer-1 source ID, similarly as the unicast case. However, the PSFCH resources for unicast and option-1 based groupcast transmissions should not be multiplexed in the same PRB(s).

Proposal 11: For option-2 based groupcast transmission, RX UE derives PSFCH resource among a set of PSFCH resource(s) based on its in-group ID configured by high layer.

Proposal 12: For PSFCH transmission and reception overlapping case 1 and 2, the UE is allowed to drop some of the PSFCH transmission(s)/reception(s). On the other hand, for case 3 HARQ-ACK bundling is applied.

Proposal 13: The maximum HARQ process number of a UE should be determined for sidelink. The maximum HARQ process number per link can be determined during PC5-RRC link establishment.

Proposal 14: 1 bit in SCI is used to indicate the presence of CSI-RS on the associated PSSCH.

Proposal 15: Support RX UE to select PSSCH resource for CSI report.
-    FFS: resource selection window for CSI transmission resource selection considering CSI report accuracy.

Proposal 16: Whether UE implementation can adapt transmission parameter based on CSI report needs to be concluded in RAN1.

Proposal 17: PSCCH DMRS or aperiodic CSI-RS can be used for SL pathloss estimation.

Proposal 18: Sidelink pathloss based OLPC is not applicable to PSCCH regardless of unicast, groupcast or broadcast transmission, nor the CSI-RS if agreed for sidelink measurement.

Proposal 19: In the case when PSSCH is multiplexed with other channels/signals (CSI-RS, CSI/RSRP reporting, etc.), the total sidelink transmit power is the same in the PSSCH symbols with or without other channels/signals.

Proposal 20: TDM based scheme is supported for OLPC, where UE can perform individual OLPC schemes in different time domain resource sets based on different pathloss compensation for different sidelink transmission (e.g., using SL pathloss only in one set, while using DL and SL pathloss in another set).

Proposal 21: Sidelink pathloss based OLPC is supported for groupcast transmission.

Proposal 22: UE capability negotiation should take into account the limitation of UE processing capability and hardware resource.

Proposal 23: Unicast, groupcast and broadcast transmissions can be multiplexed in the same resource pool.

- Reference

- RP-190766, “New WID on 5G V2X with NR sidelink”, RAN#83, Shenzhen, March 2019.

- Chairman Notes, 3GPP TSG RAN WG1 AH1901 meeting, Taipei, February 2019.

- Chairman Notes, 3GPP TSG RAN WG1 #96bis meeting, Xi’an, April 2019.

- R1-1904077, “Physical layer procedure for NR sidelink”, vivo, Xi’an, April 2019.

- R1-1906142, “Physical layer procedure for NR sidelink”, vivo, Reno, May 2019.

- Chairman Notes, 3GPP TSG RAN WG1 #97 meeting, Reno, May 2019.

- R1-1908157, “Discussion on RLM in NR sidelink”, vivo, Prague, Aug 2019.

- TR37.885, “Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR”, V15.2.0, 2018-12.

Annex A

Table 1  System level simulation assumption for HARQ procedure

| Parameter | value |
| --- | --- |
| Deployment | Highway and Urban scenario |
| UE drop | Option A (140km/h and 60km/h for Highway and Urban scenario respectively) |
| Communication type | Groupcast |
| Carrier frequency | 6GHz |
| Bandwidth | 20MHz |
| Subcarrier spacing | 30KHz |
| TTI structure | 10 symbols for data |
| Traffic parameter | Traffic type: Aperiodic traffic Traffic load: Medium Intensity Packet arrival interval: 50ms+an exponential random variable with the mean of 50ms Packet latency requirement: 50ms Packet size: 200-2000byte |
| HARQ max transmission time | Four time |
| HARQ combination method | IR |
| Feedback distance | Urban: 150m Highway: 320m |
| RSRP threshold | (-65/-75/-85/-95) dBm |
| Resource for retransmission | Reserved via resource selection at first transmission time |
| Channel model | NR highway channel model defined in 37.885 [8] |

Table 2 System level simulation assumption for power control

| Parameter | value |
| --- | --- |
| Deployment | Highway, Urban |
| UE drop | Option A (140km/h for highway, 60km/h for urban) |
| Carrier frequency | 6GHz |
| Bandwidth | 20MHz |
| Subcarrier spacing | 15KHz |
| OLPC parameter | P0 = -80 dBm, α = 1 |
| Transmission type | Unicast |
| Traffic model | Periodic traffic: Medium Intensity - Inter-packet arrival time: 10ms - Packet size: 800 bytes or 1200 bytes - Latency requirement: 10 ms Aperiodic traffic: Medium Intensity - Inter-packet arrival time: 50ms + an exponential random variable with the mean of 50 ms - Packet size: Uniformly random in the range between 200 bytes and 2000 bytes with the quantization step of 200 bytes - Latency requirement: 50 ms |
| Resource selection | Mode 1 resource allocation mechanism |
| Pattern of TDM based scheme | Highway case: For periodic traffic： - Slot number of resource set A: 6 - Slot number of resource set B: 4 For aperiodic traffic: - Slot number of resource set A: 26 - Slot number of resource set B: 24 Urban case: For periodic traffic： - Slot number of resource set A: 4 - Slot number of resource set B: 6 For aperiodic traffic: - Slot number of resource set A: 15 - Slot number of resource set B: 35 |
| TTI structure | 10 symbols for data |
| Channel model | NR V2X channel model defined in 37.885 [8] |
| MCS for periodic traffic of different packet sizes | 800bytes: QPSK, CR = 0.44 1200bytes: QPSK, CR = 0.59 |
| MCS for aperiodic traffic of different packet sizes | 200bytes: QPSK, CR = 0.12 400bytes: QPSK, CR = 0.19 600bytes: QPSK, CR = 0.3 800bytes, 1000bytes: QPSK, CR = 0.44 1200bytes, 1400bytes: QPSK, CR = 0.59 1600bytes: 16QAM, CR = 0.37 1800bytes, 2000bytes: 16QAM, CR = 0.48 |