3GPP TSG RAN WG1 Meeting #98							R1-1908157

Prague, CZ, August 26th – 30th, 2019

Source:	vivo

Title:	Discussion on RLM in NR sidelink

Agenda Item:	7.2.4.8

Document for:	Discussion and Decision

- Introduction

The V2X WI has been approved in RAN#83 meeting [1]. One of the objectives is to introduce AS level link management for unicast:

- Sidelink L2/L3 protocols and signalling - Support of sidelink transmission and reception in RRC, MAC, RLC, PDCP, and SDAP [RAN2] - AS level link management for unicast [RAN2, RAN1] - Define the criteria of PC5 availability/unavailability for unicast based on this functionality.

In the RAN1 #96bis meeting, the design of RLM was discussed, and the following agreements were achieved [2].

Agreements: - No new reference signal dedicated to SL RLM is introduced. - Existing SL RS is reused for SL RLM/RLF - Note: CSI-RS is not precluded - RAN1 has no intention to introduce RS transmitted in a periodic manner only for SL RLM purposes - FFS: - Whether SL RS is transmitted in a stand-alone manner for SL RLM/RLF Agreements: - Regarding metric for SL RLM/RLF declaration, RAN1 discussed the following (to be further studied): - Reuse IS/OOS metric in Uu RLM as much as possible but considering the condition that RAN1 has no intention to introduce RS transmitted in a periodic manner only for SL RLM purposes - Other metrics, e.g., congestion control metric (similar to CBR in LTE), consecutive HARQ-NACKs, etc. - Note: RAN1 expects further input from RAN2 to further progress on this topic

In the previous RAN1 meeting, it was further achieved the following agreement for RLM/RLF [3].

Agreements: - No standalone RS dedicated to SL RLM/RLF in Rel-16

In this contribution, we provide our view on the remaining issues for RLM in NR sidelink.

- Discussion

It has been agreed that no new reference signal is introduced for SL RLM/RLF. RS dedicated for SL RLM purpose is not transmitted in a period manner. Further, it has been agreed that no standalone RS is introduced for RLM/RLF. As a result, the remaining candidates for SL RLM include:

- Alt.1: SL-SSB

- Alt.2: PTRS

- Alt.3: Aperiodic CSI-RS

- Alt.4: PSSCH DMRS

- Alt.5: PSCCH DMRS

Alt.1 has the merit to reuse the NR downlink framework for sidelink. However, unlike the downlink SSB where the PCI can be used to identify the gNB, the SLSS ID in the S-SSB is used to determine the synchronization source. Therefore, it is not possible to identify the transmitter UE from the S-SSB, nor perform link monitoring for a particular UE.

Furthermore, the UEs having the same synchronization source, implying that they are in the proximity, may transmit S-SSB in the same synchronization resource. In this case, for a pair of UEs having the same synchronization source, due to the half-duplex issue they cannot monitor the S-SSB of each other because they are always sending the S-SSB at the same time. Consequently, it is not possible to perform RLM based on S-SSB.

Alt.2 may not be a good choice, as PTRS is only agreed for FR2. Moreover, the performance of RLM based on PTRS may be a concern due to the low density in frequency domain.

Proposal 1: S-SSB and PTRS are not used for SL RLM/RLF.

Alt.3 seems to be a straightforward solution if periodic CSI-RS is not supported. However, according to the RAN2 LS [4], a periodic indication of in-sync / out-of-sync to the upper layer as in Uu RLM mechanism is preferred by RAN2. It is then questionable whether the aperiodic CSI-RS can meet the requirement. For example, if the transmitter UE has no unicast data to send during the in-sync / out-of-sync indication period, there is no aperiodic CSI-RS for RLM measurement. Another issue is that a misdetection of SCI means that no AP-CSI-RS scheduled by the SCI can be used for RLM measurement, which further reduces the available number of samples for RLM measurement. Similar issues also happen for Alt.4 since both DMRS and AP-CSI-RS are multiplexed and transmitted with PSSCH together.

Observation 1: If the transmitter UE has no unicast data to send during the in-sync / out-of-sync indication period, there is no aperiodic CSI-RS nor PSSCH DMRS for RLM measurement.

On the other hand, these issues can be mitigated by Alt.5. It is worth noting that a UE anyway needs to send periodic broadcast or groupcast transmissions, beside the unicast data. Thus, not only the unicast transmission between the UE pair but also the groupcast and broadcast transmissions can be used by the receiver UE for RLM measurement, which means that the number of measurement samples should be sufficient. It has been agreed that the L1 source ID is included in the SCI. Thus, the receiver UE can always figure out the DMRS associated to the SCI sent from the peer UE for RLM measurement. This is feasible even if the peer UE sends unicast data to other UEs, because the SCI (regardless of unicast, groupcast or broadcast) should be transmitted in a broadcast manner, in order to facilitate all the nearby UEs to perform the sensing procedure in the resource pool, for the sake of mitigating hidden UE problem as discussed in [5]. Noted that in this case, the DMRS for PSSCH may not be useful, because UE-specific precoding, as well as power control, may be applied to the unicast or even groupcast transmission.

Observation 2: For a receiver UE to perform SL RLM of a transmitter UE in a unicast pair, the PSCCH DMRS associated to the following transmissions from the transmitter UE can be used for RLM measurement: 
-- 	Unicast transmission from the transmitter UE to the receiver UE, 
-- 	Broadcast transmission from the transmitter UE, 
-- 	Groupcast transmission from the transmitter UE, 
-- 	Unicast transmission from the transmitter UE to the other receiver UEs.

Figure 1 Different coverages between PSCCH and PSSCH to mitigate hidden UE problem

Some concerns were raised on using the PSCCH DMRS for RLM measurement, saying that due to UE-specific beamforming may be applied to the single-layer PSCCH DMRS, Alt.5 is not adequate for sidelink RLM. Actually, in the sidelink case, this is not a problem. Unlike the downlink case where the PDCCH is only targeted to the receiver UE, in the sidelink case, the PSCCH should be decoded not only by the receiver UE but also by all the nearby UEs, so that the nearby UEs are able to perform sensing and avoid collision for resource allocation, as illustrated in Figure 1. Consequently, the PSCCH should be transmitted in a broadcast manner, and UE-specific beamforming should not be used for the PSCCH even for unicast/groupcast transmission.

Based on the discussion, it seems that alt.5 based on PSCCH DMRS is the simplest yet efficient solution. The SL RLM may be performed even if no unicast data is sent from the transmitter UE.

Proposal 2: The PSCCH DMRS is used for the unicast UE pair to perform RLM/RLF.

Proposal 3: For a receiver UE to generate an in-sync / out-of-sync indication of a unicast link, the PSCCH DMRS associated to the following transmissions can be used for measurement within a periodicity: 
-- 	Unicast transmission from the transmitter UE to the receiver UE, 
-- 	Broadcast transmission from the transmitter UE, 
-- 	Groupcast transmission from the transmitter UE, 
-- 	Unicast transmission from the transmitter UE to the other receiver UEs.

If the PSCCH DMRS is used for RLM/RLF, the Uu-based RLM / RLF declaration model considered by RAN2, described in the LS [4], can be reused for SL as well, and the in-sync / out-of-sync indication can also be reused based on the hypothetical PSCCH BLER. Both the transmitter and the receiver UEs should send PSCCH; there is no need to distinguish the RLM/RLF procedures or behaviors between the transmitter and the receiver UEs. This is also aligned with RAN2’s preference that both peer UEs involved in unicast transmission perform RLM/RLF detection.

Proposal 4: The in-sync / out-of-sync indication is determined based on hypothetical PSCCH BLER.

Other metrics, e.g., congestion control metric or consecutive HARQ-NACKs, are proposed, in addition to the IS/OOS. However, the benefit is not clear. The RLM/RLF is defined per link (UE pair), while the congestion control metric (e.g., CBR) is related to QoS requirement, i.e., it is a per-service metric. More specifically, according to the congestion control mechanism, even with the same CBR in a radio link, one packet with a higher service priority can be sent while another packet with lower priority cannot be sent. Thus, it is not suitable to represent the link state. Moreover, the CBR (if defined by S-RSSI) represents the interference of the receiver UE, without considering the channel quality between the transmitter and the receiver UEs. Even if the CBR is high, the link quality may still be good enough if both UEs are near each other.

On the other hand, the metric of consecutive HARQ-NACKs can be used only if HARQ feedback is enabled. Moreover, a similar mechanism (i.e., the maximum number of RLC retransmissions) may be reused in sidelink by RAN2. Thus additional consecutive HARQ-NACKs may not be needed.

Observation 3: Congestion control metric (e.g., CBR) is not suitable to represent the link-state because it is a per-service metric.

Observation 4: The metric of consecutive HARQ-NACKs can be used only if HARQ feedback is enabled.

- Conclusion

In the contribution, we provide our view on the support of AS level link management in NR sidelink, and observe that,

Observation 1: If the transmitter UE has no unicast data to send during the in-sync / out-of-sync indication period, there is no aperiodic CSI-RS nor PSSCH DMRS for RLM measurement.

Observation 2: For a receiver UE to perform SL RLM of a transmitter UE in a unicast pair, the PSCCH DMRS associated to the following transmissions from the transmitter UE can be used for RLM measurement: 
-- 	Unicast transmission from the transmitter UE to the receiver UE, 
-- 	Broadcast transmission from the transmitter UE, 
-- 	Groupcast transmission from the transmitter UE, 
-- 	Unicast transmission from the transmitter UE to the other receiver UEs.

Observation 3: Congestion control metric (e.g., CBR) is not suitable to represent the link-state because it is a per-service metric.

Observation 4: The metric of consecutive HARQ-NACKs can be used only if HARQ feedback is enabled.

Based on these observations, we propose that,

Proposal 1: S-SSB and PTRS are not used for SL RLM/RLF.

Proposal 2: The PSCCH DMRS is used for the unicast UE pair to perform RLM/RLF.

Proposal 3: For a receiver UE to generate an in-sync / out-of-sync indication of a unicast link, the PSCCH DMRS associated to the following transmissions can be used for measurement within a periodicity: 
-- 	Unicast transmission from the transmitter UE to the receiver UE, 
-- 	Broadcast transmission from the transmitter UE, 
-- 	Groupcast transmission from the transmitter UE, 
-- 	Unicast transmission from the transmitter UE to the other receiver UEs.

Proposal 4: The in-sync / out-of-sync indication is determined based on hypothetical PSCCH BLER.

- Reference

- RP-190766, “New WID on 5G V2X with NR sidelink”, RAN#83, Shenzhen, March 2019.

- Chairman Notes, 3GPP TSG RAN WG1 #96bis meeting, Xi’an, April 2019.

- Chairman Notes, 3GPP TSG RAN WG1 #97, Reno, May 2019.

- R1-1908000, “Response LS on SL RLM / RLF in NR V2X for unicast”, RAN2, Reno, May 2019.

- R1-1908154, “Physical layer procedure for NR sidelink”, Prague, August, 2019.