3GPP TSG RAN WG1 #98	 	R1-1908158

Prague, CZ, August 26th – 30th, 2019

Source:	vivo

Title:	PDCCH enhancements for URLLC

Agenda Item:	7.2.6.1

Document for:	Discussion and Decision

- Introduction

In RAN1#97 meeting, DCI format for URLLC are discussed. Some agreements are achieved as following [1]:

Support configurable TDRA table as in Rel-15 DCI format 1_1 (i.e. 0, 1, 2, 3 or 4 bits for time domain resource assignment) for the DL DCI format scheduling Rel-16 URLLC

Support at least resource allocation type 1 for frequency domain resource assignment for the DCI format scheduling Rel-16 DL URLLC with one of the following modifications compared to Rel-15:

- Option 1: a single configurable scheduling granularity applicable for both the starting point and length indication

  - Alt.1: The scheduling granularity reuses the RBG sizes for RA 0 and can be configured between configuration 1 and 2 as in Rel-15

  - Alt. 2: A new RRC parameter to configure the scheduling granularity

- Option 2: Separate configurable starting point granularity and length indication granularity

In this contribution, we share our view on PDCCH enhancements for URLLC.

- DCI format for URLLC

According to the previous agreements, the minimum DCI size with a reduction of 10~16 bits relative to Rel-15 DCI format 0_0/1_0 is supported, which targets to meet reliability requirement. On the other hand, DCI format(s) with configurable sizes can be used for URLLC scheduling. The DCI with configurable size can includes MIMO related field and URLLC specific field to achieve the appropriate scheduling, e.g. antenna ports indication, priority indication, repetition factor and so on. Both two DCI format is beneficial for URLLC service. In the following sections, DCI format with minimum size and configurable size are discussed.

  - DCI format with minimum size for URLLC

According to SI conclusion, DCI size with reduction of 10~16 bits, so called compact DCI, is beneficial for meeting 10^-6 reliability requirement. The compact DCI design can base on fallback DCI format with payload reduction in some field sizes.

Proposal 1: Define a new DCI format for URLLC with the minimum DCI size targeting a reduction of 10~16 bits relative to Rel-15 DCI format 0_0/1_0.

In following section, the compact DCI design are proposed. Assuming 50 RBs bandwidth, the fallback DCI payload size is 63 bits. The corresponding payload size of compact DCI would be 47~53 bits.

DL related DCI field with the potential payload reduction

- Frequency domain resource assignment

For URLLC, the wider frequency-domain bandwidth is beneficial for low code rate transmission and can achieve the high reliability. Compared to resource allocation type 0, resource allocation type 1 can achieve the less overhead. Furthermore, resource allocation type 1 with the larger granularity is beneficial the further reduction of payload size in frequency domain RA field. Rel-15 RBG size can be reused as the basic scheduling unit. In this case, both the starting point and scheduling granularity can base on RBG. In the case of multiplexing eMBB and URLLC service, if some RBs of a RBG are occupied by eMBB service. URLLC UE can not be scheduled on the remaining RBs of the RBG due to the coarser starting point granularity and a larger RBG size. This is the potential resource waste issue and the separate starting point indication and RBG size are suggested. Actually, this issue mainly is relative to the RBG size. When the resources of eMBB service occur in multiple fragments in frequency domain, the finer starting point granularity with separate indication method cannot well solve resource waste issue. gNB implementation is still required. Moreover, for separate indication of starting point and RBG size indication, the more specification effort and overhead are required. The main target of compact DCI is the reduction of DCI payload size to satisfy reliability. Non-fallback DCI can be used to handle the resource utilization issues when mix scheduling of eMBB and URLLC service.

In Rel-15, the maximum RBG size with 16 RB are supported. It is sufficient for both scheduling flexibility and overhead aspects. Therefore, reusing Rel-15 RBG configuration 1 and 2 is slightly preferred.

Proposal 2: For Resource allocation type 1 with a configurable RBG size, a single configurable scheduling granularity applicable for both the starting point and length indication. (Option 1)

  - The scheduling granularity reuses the RBG sizes for RA 0 and can be configured between configuration 1 and 2 as in Rel-15.(Alt 1)

- Time domain resource assignment

It has been agreed that configurable TDRA table as in Rel-15 DCI format 1_1 is supported for the DL DCI format scheduling Rel-16 URLLC. In Rel-15, time domain resource assignment indicates the set of OFDM symbols used for PDSCH transmission, the start slot, and the PDSCH mapping type. This field is comprised of 4 bits, which indicates the partial combinations due to the limited number of bits. Generally, PDSCH mapping type B can be as the default configuration and needn’t be indicated. Besides, PDCCH and PDSCH can be allocated based on the adjacent or overlapped manner considering the shorter latency requirement. Thus, K0 value range can be refined.

In the current mechanism of time domain RA, the time reference is relative to slot boundary. Table 1 lists the starting symbol index relative to the start of the slot. Five bits are required for all the combinations of SLIV. Actually, only partial combinations can be indicates in scheduling DCI due to the limited bits of TDRA, which leads to DL grants on the different symbols are restricted to schedule PDSCHs with the same starting symbol as shown in Figure 1. This would influence the scheduling flexibility and increase latency.

Table 1 the starting symbol relative to the start of the slot

| # Length | # Starting symbols | # The number of combination |
| --- | --- | --- |
| # 2 | # 0~11 | # 29 |
| # 4 | # 0~9 |  |
| # 7 | # 0~6 |  |

Figure 1. Since the limited TDRA value configured, DL grant on the different symbol is restricted to schedule PDSCH with the same starting symbol

Considering the latency requirement of URLLC, indication of the relative location of starting symbols is beneficial, for example, the offset of PDSCH starting symbol relative to the end of the monitoring CORESET can be applied, as shown in Table 2. Compared to Table 1, this indication is more suitable to URLLC service character and signaling overhead can be reduced. In one extreme case, the TDRA indication field can be removed by UE assuming RRC configured PDSCH length and DMRS type, and assuming PDSCH starts immediately after the ending symbol of PDCCH CORESET. On the other hand, 0bits TDRA field has been agreed. If zero bit TDRA field is configured. UE would use the TDRA value configured by RRC signaling. If using the starting symbol with an absolute value, only one scheduling opportunity is available within one slot, which is not feasible for URLLC service.

Table 2 the starting symbol relative to the end of the CORESET

| # Length | # Offset |
| --- | --- |
| # 2 | # 0,1 |
| # 4 | # 0,1 |
| # 7 | # 0,1 |

Notes: Offset value 0 expresses CORESET is embedded in PDSCH transmission. Offset 1 expresses the starting symbol of PDSCH is next to the end of the monitoring CORESET in time domain. Other values can be FFS.

Proposal 3: Time domain resource assignment indicates the starting symbol relative to the end of the CORESET where the PDCCH is detected.

- VRB-to-PRB mapping

VRB-to-PRB mapping can achieve the distributed resource allocation, which can obtain frequency diversity gain. If this field is triggered, the localized or distributed mapping can be applied. The localized mapping with frequency hopping is beneficial for obtaining scheduling gain, while distributed mapping can obtain frequency diversity gain. One bit is used for triggering VRB-to-PRB mapping. If the enabling VRB-to-PRB mapping is configured by RRC signaling, this field can be absent in DCI.

- MCS and RV

For URLLC service with the high reliability requirement, the lower code-rate entries are introduced to guarantee BLER target requirement in Rel-15. For compact DCI, the size of MCS filed can be reduced, for example, some high modulation order entries may be not configured for saving the payload size. MCS table with the fewer entries can be configured by gNB.

Considering the latency requirement, the only self-decodable RV can be configured for URLLC use cases, e.g. {0, 0, 0, 0} and {0, 3, 0, 3}. In this case, 1 bit RV indication can be used. Furthermore, if a certain RV is configured by RRC signaling, this field can be absent.

- HARQ process number

For URLLC service, non-slot scheduling is suitable to reduce HARQ round trip time. The less HARQ process numbers can achieve the pipeline scheduling due to the fast feedback. Thus, the payload size of the number of HARQ processes can be decreased. The payload size reduction on HARQ process number field can be considered.

- Downlink Assignment Index

In Rel-16, multiple PUCCHs for HARQ-ACK within a slot are supported. For FDD case, the sufficient PUCCH resources can be used. Thus, this field can be omitted for FDD case. Otherwise, 2 bits are needed.

- PDSCH-to-HARQ feedback timing indicator

In Rel-15 DCI, this field indicates HARQ-ACK timing relative to the slot boundary of PDSCH reception. For the service with the low latency, PUCCH feedback based on sub-slot has been agreed. The payload size reduction can be considered.

The examples of compact DCI for payload size with 16bits reduction and 10bits reduction for DL are shown in table 3. The payload size of each field can be configured within a range as described by the comments in the table. The details can be further discussed.

Table 3 URLLC DCI field for DL:

| URLLC DCI for DL assignment | Rel -15 fallback DCI | Rel -16 compact DCI | Comment |
| --- | --- | --- | --- |
| Identifier for DCI formats | 1 | 1 | Agreed |
| Frequency domain resource assignment | 11 | 7 | The payload size is 11 bits assuming 50 RBs for Rel-15. Rel-15 RBG configuration can be used. Starting point and scheduling unit based on RBG. |
| Time domain resource assignment | 4 | 0-4 | The PDSCH starting symbol relative to the end of the CORESET should be supported. 0-4 bits can be configured for this field. |
| VRB-to-PRB mapping | 1 | 0-1 | This field can achieve the distributed resource allocation. If VRB-to-PRB mapping is enabled by RRC configuration, this field can be absent. 0-1 bits can be configured for this field. |
| Modulation and coding scheme | 5 | 2-5 | Some high modulation order and code rate entries can be disabled by RRC configuration. 2-5 bits can be considered for this field. |
| New data indicator | 1 | 1 | Agreed. |
| Redundancy version | 2 | 0-2 | If RV sequence is configured by RRC signaling, this field can be absent. Otherwise, 2bits are used. 0-2 bits can be configured for this field. |
| HARQ process number | 4 | 2-4 | The reduced number of process is applied. 2-4 bits can be configured for this field. |
| Downlink Assignment Index | 2 | 0-2 | For FDD scenario, this field can be configured to zero. 0-2 bits can be configured for this field. |
| TPC command for PUCCH | 2 | 0-2 | If power control bases on open loop adjustment, this field can be absent. 0-2 bits can be configured for this field. |
| PUCCH resource indicator | 3 | 2-3 | The reduced number of bits can be considered. 2-3 bits can be configured for this field. |
| PDSCH-to-HARQ feedback timing indicator | 3 | 0-3 | The reduced number of bits can be considered. 0-3 bits can be configured for this field. |
| RNTI and CRC | 24 | 24 |  |
| Total | 63 | 47-53 | Assuming 50 RBs bandwidth, the fallback DCI payload size is 63 bits. The corresponding compact DCI with payload size reduction would be 47~53 bits. |

UL related DCI field

For UL related DCI field, more combinations are observed in time domain resource assignment due to flexible PUSCH duration (length) from 1 symbol to 14 symbols, which leads to the maximum 105 combinations. gNB configures a table for a UE with the limited entries. Similar to DL assignment, the available starting symbol and duration for PUSCH scheduled by UL grants are still restricted due to this limitation. Thus, the starting symbol relative to the CORESET is suggested, where the CORESET is the one on which UL grant is detected. Similarly as in DL, in one extreme case, the TDRA in UL grant can be removed if the PUSCH length and DMRS type is RRC configured and UE assumes the scheduled PUSCH starts from the ending symbol of CORESET where UL grant is detected.

For UL-SCH indicator, configuration 0-1 bits is preferred. If 0 bit is configured, two alternatives are observed.

- Alt 1: aperiodic CSI is always multiplexed with UL-SCH if CSI request is triggered.

- Alt 2: aperiodic CSI is not allowed to be transmitted together with UL-SCH.

We are fine for these alternatives. For URLLC services with the higher reliability requirement, using lowSE MCS is a typical case. The effect of aperiodic CSI seems limited.

Proposal 4: For PUSCH, the TDRA indicates the time-domain offset of starting symbol relative to the CORESET where UL grant is detected.

A compact DCI design for UL is shown in table 4. The payload size of each field can be further discussed.

Table 4 URLLC DCI field for UL:

| URLLC DCI for UL assignment | Rel-15 fallback DCI | Rel -16 compact DCI | Comment |
| --- | --- | --- | --- |
| Identifier for DCI formats | 1 | 1 | Agreed. |
| Frequency domain resource assignment | 11 | 7 | The payload size is 11 bits assuming 50 RBs for Rel-15. Rel-15 RBG configuration can be used. Starting point and scheduling unit based on RBG. |  |
| Time domain resource assignment | 4 | 0-4 | The starting symbol relative to the end of the CORESET can be considered. 0-4 bits can be configured for this field. |
| Frequency hopping flag | 1 | 0-1 | If frequency hopping is always enabled by RRC signaling, this field can be absent. 0-1 bits can be configured for this field. |
| Modulation and coding scheme | 5 | 2-5 | Some high modulation order and code rate entries can be disabled by RRC configuration. 2-5 bits can be configured for this field. |
| New data indicator | 1 | 1 | Agreed. |
| Redundancy version | 2 | 0-2 | If RV sequence is configured by RRC signaling, this field can be absent. Otherwise, 2bits are used. 0-2 bits can be configured for this field. |
| HARQ process number | 4 | 2-4 | The reduced number of bits can be considered. 2-4 bits can be configured for this field. |
| TPC command for PUSCH | 2 | 0-2 | If power control bases on open loop adjustment, this field can be absent. 0-2 bits can be configured for this field. |
| UL-SCH | 0 | 0-1 | 0-1bits can be configured for this field. If 0 bit is configured, the meaning of this field should be clarified. |
| UL/SUL indicator | 1 | 0-1 | 0-1bits can be configured for this field. |
| RNTI and CRC | 24 | 24 |  |
| Total | 63 with 7 padding bits | 47-53 | Assuming 50 RBs bandwidth, the fallback DCI payload size is 63 bits. The corresponding compact DCI with payload size reduction would be 47~53 bits. |

Proposal 5: Table 3 and table 4 are suggested for compact DCI format.

  - DCI format with additional fields for URLLC

For scheduling DCI for URLLC service, the maximum DCI size can be larger than Rel-15 DCI format 0_0/1_0. In our opinion, Rel-15 DCI format 0-1/1-1 can also be used for URLLC scheduling, because some URLLC service only requires the control and data transmission with the less reliability requirement. Rel-15 DCI format 0-1/1-1 can provide the sufficient scheduling flexibility. There is no obvious motivation to support DCI payload size larger than Rel-15 DCI format 0_1/1_1.

For URLLC transmission, the MIMO-related field can be configured, e.g. antenna ports, transmission configuration indication and so on. The size of the corresponding fields can be reduced compared to the existing payload size. Some existing fields can be removed, which can be discussed further. Some new fields can be introduced, for example, repetition factor, priority indication etc..

- Repetition factor

Repetition transmissions are an efficient manner to improve the PDSCH and PUSCH transmission reliability. Repetition factor indication in DCI is beneficial for UE supporting multiple services with the different reliability requirement. For a service with the lower reliability requirement, a small number of repetitions can be indicated to a UE. While a service with the higher reliability requirement arrives at UE side, more repetition transmissions can be used. Thus, repetition factor can be included in DCI format for URLLC. For this field, 0-2 bits can be configured.

- Priority indication

Priority indication can be used for intra-UE multiplexing/prioritization of different services. For resources conflict between dynamic grant scheduling and configured grant, the conflict handling can base on priority indication in DG. For example, for resources conflict between eMBB and URLLC service, if priority indication field indicates the higher priority, DG PUSCH is prioritized over CG PUSCH regardless of the priority of CG PUSCH. On the contrary, CG PUSCH with higher priority can prioritize over DG PUSCH with the lower priority indication. Some priority differentiation method in PHY can be found in our companion contribution [2].

On the other hand, the priority indication can be used for power control loop determination. For example, URLLC service uses power control loop 1 and eMBB service uses power control loop 2 based on the priority indication. For this field, 0-2 bits can be configured.

For URLLC service, if multiple kinds of priority classes are supported, priority differentiation for different URLLC service can be considered. The priority indication field can be included in DCI.

Multiple TRP transmission are discussed on MIMO section and the corresponding indication also can be introduced in URLLC DCI.

Proposal 6: In addition to the newly introduced compact DCI format, existing non-fallback DCI with necessary modifications can be used for URLLC scheduling (potentially including new fields, reducing size of some existing fields or removing of some existing fields).

Proposal 7: For both compact DCI format and the modified existing non-fallback DCI format, repetition factor and priority indication can be introduced as the configurable field.

- Increased PDCCH monitoring capability

The number of BDs and CEs are defined per slot in Rel-15. The number of BDs/CCEs for different SCSs are defined as 44/36/22/20 and 56/56/48/32, respectively.

The number of non-overlapping CCEs for channel estimation is relative to UE capability and impacts the PDCCH decoding complexity. Current UE capability supports the maximum seven PDCCH monitoring span within a slot with 2-symbol mini-slot granularity. According to evaluation results from SI stage, PDCCH with 16 CCEs are required to satisfy reliability requirement. Considering URLLC service with the sporadic arrival character and the lower latency requirement, seven monitoring spans and at least 16 CCEs per span should be supported by UE capability with a slot.

For PDCCH monitoring span with a minimum time separation of X OFDM symbols and length up to Y consecutive OFDM symbols. Each combination of [X,Y]i is corresponding to M i CCEs, the parameters {[X,Y]i, Mi} are reported as UE capability. How to define UE monitoring capability should be discussed.

A simple method is assuming the same monitoring capability across different spans within a slot. In this case, a single value M is defined as the maximum number of CCEs per span. The maximum number of CCEs monitored within a slot can be derived by M* the number of spans within a slot.

Generally, the number of CCEs monitored within a slot can be non-uniformly distributed across different monitoring spans. At the beginning of a slot, UE may be configured with common search space and UE-specific search space for UE supporting both eMBB and URLLC services. Thus the larger CCE numbers for monitoring are required at least in the 1st span of a slot. Besides, UE may need to monitor PDCCH with UE-specific DCI (mainly for URLLC grants) in the other spans in a slot. In these spans, the requirement for monitoring CCE numbers may be lower. For this case, the non-uniform number of CEs per span is beneficial from UE capability point of view. For instance, UE can report the maximum number of CCEs in a span with two values (M1 and M2, M1>M2), where M1 corresponds to UE monitoring capability in the first span of each slot and M2 corresponds to UE monitoring capability in the remaining spans of each slot.

On the other hand, for UEs having URLLC services only, the number of CEs for each span can be similar and uniform number of CEs per span seems proper. Thus, the UE capability definition can consider this condition. For example, in case of uniform monitoring capability per span, UE supports the maximum number of CCEs per span with M3, where M3= (M1-M2)/the number of spans+M2. The same monitoring capability is assumed across different spans within a slot.

According to above analysis, for UE capability for the combinations of [X,Y]i, three alternatives can be considered for UE monitoring capability.

- Alt 1: UE reports the maximum number of CCEs per span with a single value M. The same monitoring capability is assumed across different spans within a slot.

- Alt 2: UE reports the maximum number of CCEs in a span with two values (M1 and M2, M1>M2), where M1 corresponds to UE monitoring capability in the first span of each slot and M2 corresponds to UE monitoring capability in the remaining spans of each slot.

- Alt 3: UE report two sets of capabilities corresponding to the configurations of uniform and non-uniform number of non-overlapping CEs across spans,

- For non-uniform case, UE report the maximum number of CCEs in a span with two values (M1 and M2, M1>M2), where M1 corresponds to UE monitoring capability in the first span of each slot and M2 corresponds to UE monitoring capability in the remaining span of each slot.

- For uniform case, UE supports the maximum number of CCEs per span with M3, where M3= (M1-M2)/the number of spans+M2. The same monitoring capability is assumed across different spans within a slot.

Taken span (2,2) as an instance, it should be provided at least 16CCE within a monitoring occasion for URLLC DCI monitoring considering reliability requirement. The maximum number of different start symbol indices of PDCCH monitoring occasions per slot including PDCCH monitoring occasions equals to 7. Thus, the number of CCEs monitored within a slot is at least 112. For UE capability with the different span combination, we assume the same monitoring number of non-overlapping CCEs within a slot, which is the same to Rel-15. Thus, C equals to 16 corresponding to combination (2,2). PDCCH monitoring capability is 36 and 56 for combination (4,3) and (7,3), respectively.

The instance for some combinations of (X,Y) are listed in table 5.

Table 5 PDCCH monitoring capability for different span

|  | X | Y | C |  |
| --- | --- | --- | --- | --- |
|  |  |  | =0 | =1 |
| Case 3(2,2) | 2 | 2 | 16 | 16 |
| Case 6(4,3) | 4 | 3 | 36 | 36 |
| Case 9(7,3) | 7 | 3 | 56 | 56 |

Proposal 8: Table 5 can be considered for further discussion for UE PDCCH monitoring capability.

Proposal 9: For UE capability for the combinations of [X,Y]i, three alternatives can be further discussed for UE monitoring capability:

- Alt 1: UE reports the maximum number of CCEs per span with a single value M. The same monitoring capability is assumed across different spans within a slot.

- Alt 2: UE reports the maximum number of CCEs in a span with two values (M1 and M2, M1>M2), where M1 corresponds to UE monitoring capability in the first span of each slot and M2 corresponds to UE monitoring capability in the remaining spans of each slot.

- Alt 3: UE report two sets of capabilities corresponding to the configurations of uniform and non-uniform number of non-overlapping CEs across spans,

- For non-uniform case, UE report the maximum number of CCEs in a span with two values (M1 and M2, M1>M2), where M1 corresponds to UE monitoring capability in the first span of each slot and M2 corresponds to UE monitoring capability in the remaining span of each slot.

- For uniform case, UE supports the maximum number of CCEs per span with M3, where M3= (M1-M2)/the number of spans+M2. The same monitoring capability is assumed across different spans within a slot.

- Conclusion

In the contribution, we have some investigations on URLLC PDCCH enhancement, and propose that,

Proposal 1: Define a new DCI format for URLLC with the minimum DCI size targeting a reduction of 10~16 bits relative to Rel-15 DCI format 0_0/1_0.

Proposal 2: Resource allocation type 1 with a configurable RBG size is supported for DL frequency domain RA.

Proposal 3: Time domain resource assignment indicates the starting symbol relative to the end of the CORESET where the PDCCH is detected.

Proposal 4: For PUSCH, the TDRA indicates the time-domain offset of starting symbol relative to the CORESET where UL grant is detected.

Proposal 5: Table 3 and table 4 are suggested  for DCI format with minimum DCI size for URLLC.

Proposal 6: In addition to the newly introduced compact DCI format, existing non-fallback DCI with necessary modifications can be used for URLLC scheduling (potentially including new fields, reducing size of some existing fields or removing of some existing fields).

Proposal 7: For both compact DCI format and the modified existing non-fallback DCI format, repetition factor and priority indication can be introduced as the configurable field.

Proposal 8: Table 5 can be considered for further discussion for UE PDCCH monitoring capability.

Proposal 9: For UE capability for the combinations of [X,Y]i, three alternatives can be further discussed for UE monitoring capability:

- Alt 1: UE reports the maximum number of CCEs per span with a single value M. The same monitoring capability is assumed across different spans within a slot.

- Alt 2: UE reports the maximum number of CCEs in a span with two values (M1 and M2, M1>M2), where M1 corresponds to UE monitoring capability in the first span of each slot and M2 corresponds to UE monitoring capability in the remaining spans of each slot.

- Alt 3: UE report two sets of capabilities corresponding to the configurations of uniform and non-uniform number of non-overlapping CEs across spans,

- For non-uniform case, UE report the maximum number of CCEs in a span with two values (M1 and M2, M1>M2), where M1 corresponds to UE monitoring capability in the first span of each slot and M2 corresponds to UE monitoring capability in the remaining span of each slot.

- For uniform case, UE supports the maximum number of CCEs per span with M3, where M3= (M1-M2)/the number of spans+M2. The same monitoring capability is assumed across different spans within a slot.

4. References

[1] Chairman notes RAN1 #97

[2] R1-1908159, UCI enhancements for URLLC, 3GPP TSG RAN WG1 #97, vivo