3GPP TSG RAN WG1 #98		R1-1908163

Prague, CZ, August 26th – 30th, 2019

Source:	vivo

Title:	Enhanced UL configured grant transmissions for URLLC

Agenda Item:	7.2.6.6

Document for:	Discussion and Decision

- Introduction

In RAN1 #96bis and RAN1 #97 meetings, following agreements were made on enhanced UL configured grant for URLLC [1][2]. In this contribution, we discuss the activation/deactivation of multiple resource configuration, configuration method, and construction of repetitions aiming at ensuring K repetitions.

Agreements: - Support separate activation for different configured grant Type 2 configurations for a given BWP of a serving cell. - FFS whether or not to support joint activation in a DCI for two or more configured grant Type 2 configurations Agreements: - Support joint release in a DCI for two or more configured grant Type 2 configurations for a given BWP of a serving cell if the bit-length for indication which configurations released is no more than 4 bits and DCI size is not impacted by adopting joint release. - FFS details.

- Discussion

  - Activation/deactivation for multiple resource configurations

    - Activation

- Joint activation for multiple type 2 UL configured grant configurations

Regarding type 2 UL configured grant, it was agreed to support separate activation for different configured grant Type 2 configurations. In such case, the activation DCI needs to indicate which resource configuration is to be activated. E.g. an activation field is included in DCI by using new field or the existing fields. The codepoint of the activation field in the activation DCI indicates a given resource configuration.

In case of multiple resource configuration for enhancing reliability and reducing latency, it is beneficial to support joint activation in a DCI for two or more configured grant Type 2 configurations. The joint activation can reduce the latency of activation and the signaling overhead. It should also be pointed out that some parameters among different resource configurations can be common in such case. Therefore, a DCI activating multiple type 2 resource configurations can be supported.

To accommodate the cases of activation by separate DCI or joint DCI, the size of activation field in the activation DCI can be preconfigured by RRC. Besides, each codepoint of the activation field is associated with one or multiple resource configurations configured by RRC. To be specific, the mapping between each codepoint for activation field and the corresponding resource configuration set is configured by RRC. Note that a single codepoint can correspond to a set of resource configurations. An example is shown in Table 1. Each type 2 UL configured grant configurations are configured with a ID by RRC.

Table 1: Mapping between activation/deactivation field in DCI and corresponding configurations

| Codepoint | Activation/deactivation configurations |
| --- | --- |
| 0 | UL configured grant configuration ID=0 |
| 1 | UL configured grant configuration ID=1 |
| 2 | UL configured grant configuration ID=2 |
| 3 | UL configured grant configuration ID=3 |
| 4 | Set of UL configured grant configuration ID={0,1} |
| 5 | Set of UL configured grant configuration ID={2,3} |
| 6 | Set of UL configured grant configuration ID={0,1,2,3} |
| … | … |

Proposal 1: Support joint activation in a DCI for two or more configured grant Type 2 configurations

- A set of resource configuration (including one or multiple configurations) is indicated by the activation state field in the activation DCI

- The mapping between activation state field and the corresponding configuration is configured by RRC

If a DCI activating multiple type 2 configured grant configuration can be supported, one question to be solved is that how to indicate the scheduling information, e.g. time/frequency resource or MCS, for different resource configuration. Note that one of the use case of multiple resource configuration is to reduce latency and ensure reliability. In such case, different resource configurations can share the same parameters except the starting position of UL configured grant transmission occasion.

A time offset for a given resource configuration can be configured by RRC. In Rel.15, the starting position for UL configured grant is determined by the receiving timing of activation DCI and the indication of TDRA in the activation DCI. To activate multiple resource configurations by a joint activation DCI, the time offset configured by RRC is used to determine the starting position for the corresponding resource configuration. For example, as shown in Figure, 4 resource configurations are configured and a single activation DCI is used to activate the 4 configurations. Separate offset is configured for each resource configuration. The starting position of resource configuration ID=0 is indicated by the activation DCI. The starting position of other resource configurations are determined based on the timing indicated by the activation DCI and the corresponding time offset.

Figure 1: Example of indication of starting position by a single activation DCI for multiple resource configurations

- DCI format for activation DCI for multiple type 2 UL configured grant configurations

In Rel-15, DCI format 0_0 or DCI format 0_1 can be used for activation of type 2 UL configured grant. In case of activation by fallback DCI, i.e. DCI format 0_0, single resource configuration is activated due that there may not be new field for activation field. Therefore, fallback DCI may not be applicable in case of multiple resource configuration. If new field is needed for activation in case of multiple configurations, multiple resource configurations activated by joint DCI should be based on non-fallback DCI, e.g. Rel-15 DCI format 0_1 or DCI format for eURLLC. In case of single resource configuration, either fallback DCI or non-fallback DCI can be used for activation of type 2 UL configured grant.

Proposal 2: In case of multiple resource configurations, one or multiple configurations are activated by DCI based on Rel-15 non-fallback DCI or DCI for eURLLC.

    - Deactivation

- Joint deactivation for multiple type 2 UL configured grant configurations

Similar to the case of activation for multiple resource configuration, the deactivation DCI needs to indicate which resource configuration is to be deactivated. E.g. a deactivation field is included in DCI by using new field or the existing fields. Note that the deactivation field and the activation field can be common.

It is beneficial to support joint deactivation in a DCI for two or more configured grant Type 2 configurations to reduce the latency of deactivation and the signaling overhead. The deactivation field in the deactivation DCI can be associated with a set of resource configurations. To be specific, the mapping between each codepoint for deactivation field and the corresponding resource configuration set is configured by RRC. Note that a single codepoint can correspond to a set of resource configurations. An example is shown in Table 1.

Proposal 3: Support joint deactivation in a DCI for two or more configured grant Type 2 configurations

- A set of resource configuration (including one or multiple configurations) is indicated by the deactivation state field in the activation DCI.

- The mapping between deactivation state field and the corresponding configuration is configured by RRC.

- DCI format for deactivation DCI for multiple type 2 UL configured grant configurations

In Rel-15, only DCI format 0_0 can be used for deactivation of type 2 UL configured grant. If new field is needed for deactivation in case of multiple configurations, non-fallback DCI is needed for deactivation of type 2 UL configured grant in case of multiple configurations. In such case, a single DCI based on non-fallback DCI, e.g. Rel-15 DCI format 0_1 or DCI format for eURLLC is used for deactivation of one or multiple resource configurations. For deactivation by fallback DCI, either single resource configuration or all resource configurations deactivated by the DCI can be considered.

Proposal 4: In case of multiple resource configurations, one or multiple configurations are deactivated by DCI based on Rel-15 non-fallback DCI or DCI for eURLLC.

  - Configuration of multiple resource configuration

In RAN1 #96bis meeting, it was agreed to support separate RRC parameters for different configured grant configurations for a given BWP of a serving cell. It should be noted that multiple resource configurations are used to address two use cases identified during the SI, i.e. supporting different services or enhancing reliability and reducing latency.

In case of different services, separate parameters are needed for a given service, e.g. following parameters for a resource configuration can be configured separately.

  - Time resource allocation

  - Frequency resource allocation

  - MCS table and/or MCS level

  - Periodicity

  - DMRS configuration/sequence

  - RV sequence

  - Power control parameters

  - Others if any

In case of enhancing reliability and reducing latency, there could be common parameters among different resource configurations, e.g. periodicity, MCS, etc. The only parameter that is different among configurations may be the starting offset. Given that separate configuration for RRC parameters is supported, it is also applicable for the case of enhancing reliability and reducing latency.

Observation 1: gNB can configure common or different RRC parameters for multiple resource configurations depending on the use case.

To differentiate the resource configuration, an ID or index is introduced for UL configured grant. It is also useful to facilitate the activation and deactivation in case of multiple resource configurations. The configuration ID is globally configured for a given UL configured grant, regardless of type of UL configured grant. To be specific, the configuration IDs of type 1 UL configured grant and type 2 UL configured grant are sharing the same ID pool.

Proposal 5: The configuration ID for a given UL configured grant is globally configured by RRC.

  - Differentiation of multiple resource configurations

Another issue is that gNB needs to detect the resource configuration on which UE is using to transmit the data if multiple resource configurations are enabled for a UE. Following methods can be adopted.

- Option 1: DMRS based method

- Option 2: UCI based method

For option 1, Different DMRS parameters/sequences are configured for different resource configurations, i.e. each configuration is associated with an individual DMRS sequence or parameter.

For option 2, each transmission includes a UCI carrying information of actual transmitted resource configuration. Note that UCI transmission for UL configured grant is supported for NR-U. Similar to NR-U, HARQ ID can be determined and selected by UE and be transmitted via control information, e.g. UCI, which is separately encoded from data. Therefore, gNB can decode the UCI and acquire HARQ ID before decode the data part.

Due to flexible starting occasion, it may be difficult for gNB to identify the initial transmission of K repetitions. Especially when part of the K repetitions could be missed at gNB side, gNB may not be aware that which repetitions need to be combined. Therefore, indication of initial transmission and repetitions could be useful. NDI is used to differentiate initial transmission and retransmission for dynamic grant. Similarly, NDI can be used for grant-free transmission and indicated via UCI such that gNB can identify the initial transmission of K repetitions from UCI.

UE can be configured with RV sequences for K repetitions. In Rel. 15, RV is associated with the transmission occasions within period. Thus, gNB can identify RV from the transmission occasion where UL data are transmitted. As discussed above, flexible starting occasion is needed for URLLC. Therefore, it is not necessary to associate RV sequences with transmission occasion any more. There may be issue that gNB may need to blindly detect the UL data if RV information is not acquired at gNB. To reduce the detection complexity, RV can be indicated by UCI.

For UCI on grant-free transmission, either puncturing or rate-matching can be used for UCI mapping on PUSCH. Considering the payload of UCI, rate-matching with PUSCH is preferred. When K repetitions are adopted, UCI needs to be mapped to each repetition. Since gNB needs to combine the K repetitions, the code rate of K repetitions should be kept the same.

Proposal 6: Regarding multiple resource configurations, following options can be adopted.

- Option 1: Different DMRS parameters/sequences are configured for different resource configurations

- Option 2: UCI carrying information of UE selected resource configuration is transmitted with data.

  - UCI including HARQ ID is transmitted together with data.

  - NDI is included in grant-free UCI to differentiate initial transmission and repetitions.

  - RV sequence is not associated with the transmission occasion, and RV is included in grant-free UCI.

  - UCI is multiplexed onto each repetition.

  - Multiple resource configurations for ensuring K repetitions

In Rel.15 configured grant, a UE transmits a TB with repetitions with the resources of a resource configuration. It is simple while the starting position or ending position of repetitions need to be restricted to differentiate initial transmission and repetitions at gNB side. This leads to either large latency or lower reliability for repetitions of a TB.

For URLLC traffic, both low latency and high reliability need to be satisfied. Therefore, it is important for a UE to start the transmission for a TB at any transmission occasion and ensure K repetitions regardless of the starting transmission occasion. This means flexible starting occasion for a TB should be supported. Based on the framework of multiple resource configuration, following method is proposed to ensure K repetitions regardless of starting timing of transmission occasion.

With multiple resource configurations, it is possible to differentiate initial transmission and repetitions without sacrificing latency or reliability. More specifically, K repetitions are ensured by using multiple resource configurations, where different resource configurations are configured with different starting offsets, as described in Figure 2. For example, UE starts an initial transmission at the first transmission occasion of a resource configuration and K repetitions are ensured within a period P, where the resource configuration with shortest latency is selected, according to the packet arrival timing. For repetitions with multiple resource configurations, repetitions of a TB are transmitted with a resource configuration. Besides, HARQ ID determination based on Rel.15 configured grant can be reused.

Figure 2: Example of repetitions with multiple resource configurations

Proposal 7: For repetitions with multiple resource configurations,

- Multiple resource configurations with different starting offsets can be configured to enable flexible starting time and ensure K repetitions.

- Conclusion

In this contribution, we provide our views on enhancements of UL grant-free transmission to meet the latency and reliability requirements of URLLC. The observations and proposals are summarized below.

Observation 1: gNB can configure common or different RRC parameters for multiple resource configurations depending on the use case.

Proposal 1: Support joint activation in a DCI for two or more configured grant Type 2 configurations

- A set of resource configuration (including one or multiple configurations) is indicated by the activation state field in the activation DCI

- The mapping between activation state field and the corresponding configuration is configured by RRC

Proposal 2: In case of multiple resource configurations, one or multiple configurations are activated by DCI based on Rel-15 non-fallback DCI or DCI for eURLLC.

Proposal 3: Support joint deactivation in a DCI for two or more configured grant Type 2 configurations

- A set of resource configuration (including one or multiple configurations) is indicated by the deactivation state field in the activation DCI

- The mapping between deactivation state field and the corresponding configuration is configured by RRC

Proposal 4: In case of multiple resource configurations, one or multiple configurations are deactivated by DCI based on Rel-15 non-fallback DCI or DCI for eURLLC.

Proposal 5: The configuration ID for a given UL configured grant is globally configured by RRC.

Proposal 6: Regarding multiple resource configurations, following options can be adopted.

- Option 1: Different DMRS parameters/sequences are configured for different resource configurations

- Option 2: UCI carrying information of UE selected resource configuration is transmitted with data.

  - UCI including HARQ ID is transmitted together with data.

  - NDI is included in grant-free UCI to differentiate initial transmission and repetitions.

  - RV sequence is not associated with the transmission occasion, and RV is included in grant-free UCI.

  - UCI is multiplexed onto each repetition.

Proposal 7: For repetitions with multiple resource configurations,

- Multiple resource configurations with different starting offsets can be configured to enable flexible starting time and ensure K repetitions.

- References

- Chairman’s Notes RAN1 #96bis.

- Chairman’s Notes RAN1 #97.