3GPP TSG RAN WG1 #98							R1-1908172

Prague, Czech, Aug 26th – 30th, 2019

Source:	vivo

Title:	Discussion on UE adaptation to maximum number of MIMO layers

Agenda Item:	7.2.9.3

Document for:	Discussion and Decision

- Introduction

For UE adaptation to the maximum number MIMO layers, the following is agreed in updated power saving WID in RAN#84,

Specify the power saving techniques of UE adaptation to the maximum number of MIMO layers [RAN1, RAN2, RAN4] - Specify configuration of a different MIMO layer configuration of the initial/default BWP compared with other BWPs of a Serving Cell. [RAN2, RAN4] - Discuss whether to also extend this to define per-BWP MIMO layer configuration [RAN1, RAN2] - Evaluate if switching and interruption times for UE dynamic adaptation to the maximum number of MIMO layers are needed and which case assuming a relationship between the number of RF ports and the MIMO layer configuration [RAN4] NOTE: Switching on/off the RF is part of the evaluation

This contribution addresses the issues for introducing the adaptation of maximum number MIMO layers.

- Discussion

In Rel-15, for NR Bands n7, n38, n41, n77, n78, and n79, the UE shall be equipped with 4Rx ports as a baseline. In some conditions, e.g., in RRC idle/inactive mode, or in low DL data rate services for UE with high SINR (e.g., UEs at cell center), fallback to 2Rx antenna operation is also allowed to save UE power.  In [1], simulation results show that power saving gain of adaptation to number of DL antenna is up to 30%, with tolerable packet delay loss.

In addition, for UE equipped with 2Tx, it is also power efficient for UE to do UL MIMO layer adaptation. For example, in low UL data rate services for UE with high UL SINR, fallback to 1Tx antenna operation is beneficial to save UE power.

Observation 1: Power saving gain of adaptation to number of DL antenna is up to 30%, with tolerable packet delay loss.

In Rel-15 spec, maxMIMO-Layers is included in IE PDSCH-ServingCellConfig and in IE PUSCH-ServingCellConfig, indicating the maximum MIMO layer to be used for PDSCH and for PUSCH in all BWPs of this serving cell respectively. This means per-serving cell indication of maximum number of MIMO layers is supported by RRC signaling. Compared with layer 1 signalling (e.g., BWP switch), RRC-based configuration for MIMO layers leads to much higher latency. In addition, it is already agreed in WID to specify configuration of a different MIMO layer configuration of the initial/default BWP compared with other BWPs of a Serving Cell. So it is beneficial and natural to support Layer 1 per BWP indication of MIMO layers in Rel-16.

Observation 2: Compared with layer 1 signalling (e.g., BWP switch to configure MIMO layers), Rel-15 RRC-based configuration for MIMO layers leads to much higher latency.

  - Power saving signal/channel triggering MIMO layer adaptation

A power saving signal/channel configured outside Active Time was agreed. The content of the power saving/channel is still pending for further discussion, e.g.,

Further study in the new DCI how to potentially indicate at least the following techniques (subject to further WID update): - Indicating UE to use the aperiodic RS - Aperiodic CSI-RS - Aperiodic SRS - TRS - Triggering aperiodic CSI report - Cross-slot scheduling - Rel-15 DCI-based BWP switching The power saving techniques can be explicitly included in the DCI contents or implicitly indicated by other techniques (e.g., BWP switching).

As discussed in [1], since power saving PDCCH is monitored before DRX On duration, the power saving PDCCH can be used to trigger BWP switching to take advantage of the time gap between PS-PDCCH and DRX On. Considering MIMO layer adaptation is based on BWP switching diagram, it is natural to support power saving signal/channel implicitly adapts MIMO layer by BWP switching.

Proposal 1: Support power saving signal/channel implicitly adapts MIMO layer by BWP switching.

  - Extension per-BWP MIMO layer configuration to more than initial/default-BWP

Regarding BWP other than initial/default BWP, we think it is still useful for the following cases, e.g.,

Enables multiple levels of power consumption by combining bandwidth and antenna configuration. For example, the following power consumptions can be derived according to the current TR38.840 power model for FR1.

|  | BWP configuration | Power consumption |  |
| --- | --- | --- | --- |
|  |  | PDCCH-only | PDCCH+PDSCH |
| Configuration 0 | BWP 0 (initial/default BWP, e.g., 20MHz), 2Rx | 50[Note1] | 84 |
| Configuration 1 | BWP 1 (100MHz), 2Rx | 70 (+40%) | 210 (+150%) |
| Configuration 2 | BWP 2 (100MHz), 4Rx | 100 (+100%) | 300 (+257%) |

Note 1: According to TR38.840, Scaling of X MHz = 0.4 + 0.6 * (X - 20) / 80. Linear interpolation for intermediate bandwidths. Valid only for X = 10, 20, 40, 80, and 100. However if the power after scaling is smaller than the BWP transition power, assume the BWP transition power as the output of scaling unless otherwise justified. And for evaluation, when BWP transition duration is one slot or longer, the slot-average power level is 50 power unit. Hence, for 20MHz, we assume PDCCH-only is 50 power unit.

BWP switching to larger BW but without switching to 4-layer MIMO (configuration 1) has much less power consumption compared to switching 4-layer (configuration 2). Almost 30% power saving is expected for that slot. Considering power consumption caused by other DL receiver process, e.g. RRM measurement, CSI calculation, periodic activities and etc., when switching from 2-layer to 4-layer. Configuration 1 may have much more power saving gain.

Therefore, an inter-medium power consumption level  (e.g., configuration 1 in the above table) is beneficial. It is proposed that besides initial/default BWP, other BWPs can be configured with maximum MIMO layer independently.

Proposal 2: Besides initial/default BWP, other BWPs can be configured with maximum DL MIMO layer independently.

  - Decouple of DL MIMO layer adaptation and UL BWP switching in TDD

TDD-specific issue is important for NR since currently most of the NR deployment is for TDD.

If DL MIMO layer adaptation is triggered by DL BWP switching, it may be occurred when receiving BWP switching DCI including UL grant. Since for TDD, DL BWP is switched accompanied with UL BWP switching and vice versa.

However, the DL traffic may not be necessary enough to switch from 2Rx to 4Rx when receiving an UL grant. In some case, there is not so much downlink traffic when uplink traffic is heavy. Moreover, system information, broadcast information may not be necessary to be switched to 4Rx when there is only uplink traffic.

Therefore, decoupling DL MIMO layer adaptation and UL BWP switching in TDD is very necessary. One natural way of switching 4 DL maximum MIMO layer (which means  4 Rxantenna) after BWP switching based on the downlink grant (Figure 1). UE use 2 Rx antennas for receiving broadcast channels. Once the DL grant is detected, UE switches to 4 Rx antennas to adapt with 4 DL maximum MIMO layer.

Figure 1. exact DL Rx switching and BWP switching at different time.

Proposal 3: For UL grant triggered BWP switching in TDD, maximum DL MIMO layer (e.g., switch from 2 to 4) is switched until reception of DL grant after UL BWP switching in TDD.

  - UE assistance information

In updated power saving WID, it is proposed to

-Study and select among the following UE assistance information for RAN2 by RAN#85: power preferred information (baseline LTE PPI in a well-defined manner) and UE's preference on C-DRX, BWP and SCell configuration [RAN2]. NOTE: additional UE assistance information for RAN1-specific power saving techniques to indicate what setting of parameters related to this Work Item will lead to power savings for the UE can be included if agreed in RAN1.

Besides the RAN2 suggestions on UE assistance information, from RAN1 perspective,	UE preferred antenna configuration including MIMO layers, should also be considered in WI phase. It is noted that Rel-15 spec already supports UE reporting reduced number of MIMO layers in the overheating framework. In Rel-16, it can be further extended so that UE can also report increased number of MIMO layers, according to the traffic characteristic and/or battery information.

Proposal 4: UE can report its preferred MIMO layers including both reduced and increased MIMO layers to gNB.

  - UL MIMO layer adaptation

So far for evaluation purpose, [2] assumes 2Tx power is 1.4x 1Tx power at 0dBm and 1.2x.at 23dBm. For non power-limited case (e.g., 0dBm),  using lower maximum UL MIMO layer can bring additional power saving gain compared to higher maximum UL MIMO layer. We think it can be open for discussion.

Proposal 5: FFS for UL MIMO layer adaptation for power saving purpose.

- Conclusion

This contribution address the issues for introducing the adaptation of maximum number MIMO layers. The followings observations and proposals are given.

Observation 1: Power saving gain of adaptation to number of DL antenna is up to 30%, with tolerable packet delay loss.

Observation 2: Compared with layer 1 signalling (e.g., BWP switch to configure MIMO layers), Rel-15 RRC-based configuration for MIMO layers leads to much higher latency.

Proposal 1: Support power saving signal/channel implicitly adapts MIMO layer by BWP switching.

Proposal 2: Besides initial/default BWP, other BWPs can be configured DL maximum MIMO layer independently.

Proposal 3: for UL grant triggered BWP switching in TDD, maximum MIMO layer (e.g., switch from 2 to 4) is switched until reception of DL grant after BWP switching in TDD.

Proposal 4: UE can report its preferred MIMO layers including both reduced and increased MIMO layers to gNB.

Proposal 5: FFS for UL MIMO layer adaptation for power saving purpose.

Reference

- 3GPP TR 38.840 Study on UE Power Saving

- R1-1908170, PDCCH-based power saving signal/channel design, vivo, Prague, Czech, RAN1#98