3GPP TSG RAN WG1 #98                                                                             R1-1908183

Prague, CZ, August 26th – 30th, 2019

Source:               ZTE, Sanechips

Title:                    Evaluation results for 2-step RACH

Agenda item:      7.2.1.3

Document for:    Discussion and Decision

# Introduction

Agreement in R1-1903834:

- Adopt the link-level simulation assumptions in the following table for the initial evaluation of feasible payload size, and for the potential down-selection of schemes, e.g.

  - whether to have shared ROs and/or preambles between 2-step RACH and 4-step RACH, and if yes the percentage for partitioning

  - whether to have the guard time between PRACH and PUSCH and/or guard period within PUSCH, and if yes the length of GT/GP

  - the mapping scheme between preamble and PUSCH+DMRS, .e.g 1-to-1, multiple-to-1, or 1-to-multiple

  - appropriate power offset(s) between preamble and PUSCH, and whether to support repetition of MsgA PUSCH

  - whether to have UCI in msgA PUSCH, if a PUCCH transmission overlaps the PUSCH part of MsgA

  - whether to dynamically adapt the payload size and indicate by UCI in msgA PUSCH, and if yes the content and structure of UCI

  - FFS other schemes, e.g. whether guard band is included

- Additional system-level simulations or analytical evaluations can be considered for the following analysis:

  - Latency

  - Signalling overhead

  - Resource reservation overhead

  - PUSCH collision, with definition FFS, e.g. overlapped PUSCH occasion, with shared or separate DMRS port, and with same or different scrambling ID

  - Rx detection complexity

- Note 1: the supported/recommended payload size from RAN1 perspective may also need to take other factors into account, e.g. use cases, resource utilization.

- Note 2: the WID scope should be strictly followed when using the evaluation results for the comparison of schemes.

Agreement in R1-1903834:

- Adopt the link-level simulation assumptions in the following table for the initial evaluation of feasible payload size, and for the potential down-selection of schemes, e.g.

  - whether to have shared ROs and/or preambles between 2-step RACH and 4-step RACH, and if yes the percentage for partitioning

  - whether to have the guard time between PRACH and PUSCH and/or guard period within PUSCH, and if yes the length of GT/GP

  - the mapping scheme between preamble and PUSCH+DMRS, .e.g 1-to-1, multiple-to-1, or 1-to-multiple

  - appropriate power offset(s) between preamble and PUSCH, and whether to support repetition of MsgA PUSCH

  - whether to have UCI in msgA PUSCH, if a PUCCH transmission overlaps the PUSCH part of MsgA

  - whether to dynamically adapt the payload size and indicate by UCI in msgA PUSCH, and if yes the content and structure of UCI

  - FFS other schemes, e.g. whether guard band is included

- Additional system-level simulations or analytical evaluations can be considered for the following analysis:

  - Latency

  - Signalling overhead

  - Resource reservation overhead

  - PUSCH collision, with definition FFS, e.g. overlapped PUSCH occasion, with shared or separate DMRS port, and with same or different scrambling ID

  - Rx detection complexity

- Note 1: the supported/recommended payload size from RAN1 perspective may also need to take other factors into account, e.g. use cases, resource utilization.

- Note 2: the WID scope should be strictly followed when using the evaluation results for the comparison of schemes.

The motivation of evaluation and the corresponding LLS assumptions are agreed in [7] during the email discussion.

In this contribution, mainly five parts of preliminary evaluation results for 2-step RACH are provided, the first part shows the link performance with different payload sizes in various scenarios. Both long preamble Format 0 and short preamble Format A1 are considered in payload size evaluation. The second part shows the preamble performance for Format 0, Format A1 and Format A2. Besides, the MCL of preamble and payload are compared. The 3rd part studies different mapping schemes between PRACH preamble and PUSCH-DMRS, to have better understanding on the performance impacted by the potential collision. Besides, the effects of receiving antenna numbers and preamble formats are also considered. The 4th part and 5th part study the effect of time domain overlap and frequency domain gap respectively.

# Payload size evaluations

The methodology of the payload size evaluation is as follows:

Step 1: For each scenario with a specific assumption for timing offset (based on cell size), for various payload sizes, derive the required SNRs at the target BLER (e.g. 10%) by LLS, for the given resource size (bandwidth).

Step 2: Calculate the supported MCL based on the required SNR (corresponding to SINR in SLS) for the given occupied bandwidth, Tx power and noise figure.

Step 3:  Draw the CDF of coupling loss for each scenario and find the target percentage of CL, e.g. 95%.

Step 4: Draw the MCL vs payload size curve for each scenario with different PRB numbers and receiving antennas. Find the maximum supported payload size at the target percentage of CL.

## Link level simulation results

3 scenarios are considered:  200m, UMi, 4 GHz; 500m, UMa, 4 GHz and 1732m, RMa, 700 MHz.

The following 2 scenarios 200m, UMi, 4 GHz and 500m, UMa, 4 GHz are evaluated with Preamble format A1, the subcarrier spacing at 4GHz is 30kHz. The PUSCH resource size is [1 3 6] PRBs + 1slot, with 1/7 DMRS overhead.

The scenario 1732m, RMa, 700 MHz is relatively large cell and simulated with Preamble Format 0, the subcarrier spacing at 700MHz is 15kHz. The PUSCH resource size is [1 6 12] PRBs + 1slot, with 1/7 DMRS overhead.

Both 2 and 4 receiving antennas are considered. More detailed simulation assumptions following the agreements can be found in Table A1 in the Appendix.

- 200m, UMi, 4 GHz

Assume the total PUSCH frequency resource is 1, 3, 6 PRB, single UE, TDL-A channel. The BLER performance for the considered scenarios can be found in Figure 1. The timing offset is estimated by preamble and the channel estimation is done by DMRS with compensated TO. The frequency offfset is 0.15ppm, i.e. 600 Hz, without estimation/compensation. It can be seen that in general larger allocated bandwidth can provide better link-level performance, and the performance of 4 receiving antennas is much better than 2 receiving antennas.

(a) 1 PRB

(b) 3 PRBs

(c) 6 PRBs

Figure 1 BLER vs. SNR for 200m, UMi, 4 GHz

- 500m, UMa, 4 GHz

Assume the total PUSCH frequency resource is 1, 3, 6 PRB, single UE, TDL-A channel. Similar observations as those of UMi scenario can be found.

(a) 1 PRB

(b) 3 PRBs

(c) 6 PRBs

Figure 2 BLER vs. SNR for 500m, UMa, 4 GHz

- 1732m, RMa, 700 MHz

Assume the total PUSCH frequency resource is 1, 6, 12 PRB, single UE, TDL-A channel. Although the cell radius is larger than UMi/UMa, the performance is even better mainly due to less imposed frequency offset. The trends of BLER vs. bandwidth/antenna number are similar to that of other scenarios.

(a) 1 PRB

(b) 6 PRBs

(c) 12 PRBs

Figure 3 BLER vs. SNR for 1732m, RMa, 700 MHz

## Link budget calculation

Since the link budget calculation process of different scenarios is similar, here we consider the 4GHz + 200m results in Figure 1(c), 4Rx as an example. The elements (1)(2)(3)(4)(9) are the same for all cases, and element(5) is the channel bandwidth. The element (7) is the required SNR at 0.1 working point BLER derived from the figures in Section 2.1.

Table 1 Link budget calculation with 4G+200m Format A1

|  | 10bytes | 40bytes | 80bytes | 120bytes | 160bytes |
| --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |
| (1) Tx Power (dBm) | 23 | 23 | 23 | 23 | 23 |
| Receiver |  |  |  |  |  |
| (2) Thermal noise density (dBm/Hz) | -174 | -174 | -174 | -174 | -174 |
| (3) eNB receiver noise figure (dB) | 5 | 5 | 5 | 5 | 5 |
| (4) Interference margin (dB) | 0 | 0 | 0 | 0 | 0 |
| (5) Occupied channel bandwidth (Hz) | 2160000 | 2160000 | 2160000 | 2160000 | 2160000 |
| (6) Effective noise power = (2)+(3)+(4)+10log((5)) (dBm) | -105.7 | -105.7 | -105.7 | -105.7 | -105.7 |
| (7) Required SINR (dB) (Target BLER=0.1) | -7.2 | -4.3 | -0.8 | 4.1 | 10.6 |
| (8) Receiver sensitivity = (6)+(7) (dBm) | -112.9 | -110 | -106.5 | -101.6 | -95.1 |
| (9) Receiver processing gain | 0 | 0 | 0 | 0 | 0 |
| (10) MCL = (1)-(8)+(9) (dB) | 135.9 | 133 | 129.5 | 124.7 | 118.1 |

## Coupling loss distribution from SLS

System level simulation can be adopted to assist the payload size evaluation by drawing the coupling loss distribution for various scenarios. The simulation assumptions can be found in appendix, where most of the parameters are taken from the calibration campaign in the previous study of NOMA [8].

Figure 4 Coupling loss distribution in different scenarios

## Maximum supported payload size

The MCL vs payload size figures are drawn from the BLER vs SNR curves and link budget calculation. The 95% percentage of MCL is obtained from the coupling loss distribution from SLS. Then we plot a line of dashes on the MCL vs payload size figure, and the maximum supported payload size can be observed clearly in the Figure 5, 6, 7.

It can be found that in general 10 bytes payload size can be satisfied with more than 95% of the coverage for the evaluated scenarios, i.e. ISD with 200/500/1732m with appropriate settings on the PRB number and received antenna.  Allocating more bandwidth or adding more receiving antennas can enhance the performance for large payload size, e.g. for 200m ISD, up to 160bytes can be supported with 4Rx and 6PRB allocated bandwidth.

The percentage is decreasing when the cell size is enlarged due to higher path loss, and when the payload size is increased due to the loss of decoding performance.

- 4GHz + 200m

Figure 5 MCL vs TBsize of  scenario 4GHz + 200m

It can be found that more than 160 bytes can be supported in this scenario.

- 4GHz + 500m

Figure 6 MCL vs TBsize of  scenario 4GHz + 500m

It can be found that more than 11 bytes can be supported in this scenario.

- 700MHz + 1732m

Figure 7 MCL vs TBsize of  scenario 700MHz+1732m

It can be found that more than 29 bytes can be supported in this scenario.

Observation 1: With appropriate settings of resource size and antenna number, the minimum payload size same as msg3 (i.e. 56/72 bits) can be supported in different scenarios (cell size with 200, 500, 1732m) with 95% coverage. Larger payload sizes or cells sizes can be supported by allocating more resources or adding receiving antennas, e.g. for 200m ISD, up to 160 bytes can be supported with 4Rx and 6PRB allocated bandwidth.

# Preamble performance evaluations

In 2-step Rach, preamble performance can be evaluated by miss detection rate. Besides, false alarm rate, which is defined as the ratio between the number of detected but not transmitted preambles and total transmitted frames for 1 UE evaluation, should be kept below 0.1%. The frequency offset is considered with 0.15ppm, i.e. 600Hz for 4GHz carrier frequency, 105Hz for 700MHz carrier frequency, without compensation. Both 2 and 4 receiving antennas are considered, the scenario is 700M+1732m with Format0, 4GHz+200m, 4G+500m with Format A1 and A2. The subcarrier spacing of preamble with Format A1 and A2 is 30kHz.

## Format 0

- 2Rx & 4Rx

Figure.8 700M 1732m Format 0 2Rx & 4Rx

TOFO has little influence on Format 0, for both 2Rx and 4Rx antennas.

## Format A1

- 2Rx

Figure.9 4G 200m & 4G 500m Format A1 2Rx

- 4Rx

Figure.10 4G 200m & 4G 500m Format A1 4Rx

## Format A2

- 2Rx

Figure.11 4G 200m & 4G 500m Format A2 2Rx

- 4Rx

Figure.12 4G 200m & 4G 500m Format A2 Rx

## Preamble MCL vs Payload MCL

Based on the miss detection rate of different preamble formats, the MCL of each scenarios are shown as below:

Table 2 Preamble MCL in different scenarios

| MCL(dB) | 700M+1732m format 0 | 4G+200m format A1 | 4G+500m format A1 | 4G+200m format A2 | 4G+500m format A2 |
| --- | --- | --- | --- | --- | --- |
| 2Rx | 145.7 | 135.6 | 135.6 | 136.6 | 136.6 |
| 4Rx | 150.7 | 139.6 | 139.6 | 142.1 | 142.1 |

To compare the coverage of preamble and payload, the MCL of 10 bytes payload is calculated according to chapter 2.4 shown as below.

Table 3 Payload MCL of 10 bytes in different scenarios

| MCL(dB) | 700M+1732m | 4G+200m | 4G+500m |
| --- | --- | --- | --- |
| 2Rx | 138 | 134 | 133.7 |
| 4Rx | 141 | 138 | 138 |

MCL of long preamble format is more than 7 dB better than MCL of 10 bytes payload, MCL of short preamble format A1 is less than 2 dB better than MCL of 10 bytes payload.

Observation 2: For long preamble format (format 0, 700M 1732m), the coverage of preamble is much better than that of 10bytes payload for both 2 and 4 receiving antennas. For short preamble format (format A1 A2, 4G 200m, 4G 500m), the coverage difference between preamble and 10bytes payload is smaller.

# Preamble and PUSCH occasions mapping methods

For contention-based RA, when more than one UEs transmit preambles in the same RO, they may or may not transmit payload in the same PUSCH occasion, depending on the mapping schemes between PRACH preamble and PUSCH occasion. One way to minimize the PUSCH collision (which means sharing the same PUSCH occasion) and DMRS collision (which means different UEs select the same PUSCH occasion and the same DMRS port) is to allocate multiple PUSCH occasions (orthogonal) to be associated with the preambles in one PRACH occasion. While on the other hand, the code rate is enlarged under if we do not allocate more total reserved resources associated with a RO. Therefore, it is necessary to have some evaluations on the link performance regarding the number of PUSCH occasions associated to one RO.

## Evaluation results with 1 DMRS sequence

With the same evaluation assumptions listed in Table A1, and under the same total resource overhead (6 or 12 PRBs), 6 mapping types between Preamble and PUSCH occasions are considered for the evaluation. Two UEs with no preamble collision but possible collision of PUSCH transmission are assumed. This is mainly because the probability of more than two UEs shared on the same PUSCH resource is extremely small if the preamble collision probability is 1%, according to the analysis in our previous contribution [6]. In this case, only one DMRS sequence with 12 DMRS ports is considered.

The timing offset is estimated by preamble and the channel estimation is done by DMRS with compensated TO. The frequency offset is considered with 0.15ppm, i.e. 600Hz, without compensation. Both 2 and 4 receiving antennas are considered, the scenario is 4GHz + 200m with Format 0 and A1.

The transmitting power of the following schemes is normalized as the same:

As shown in Figure.13, each group is denoted using different colors. For total 6 PRBs, PUSCH resource can be equally divided into 2, 3 and 6 groups, each UE can use 3, 2 and 1 PRBs respectively, denoted as scheme 1, 2, 3;

For total 12 PRBs, PUSCH resource can be equally divided into 4, 6 and 12 groups, each UE can use 3, 2 and 1 PRBs respectively, denoted as scheme 4, 5, 6.

Figure 13 Different mapping schemes

For the 6 mapping types above, scheme 1 and 2 with total 6 PRBs has a DMRS collision probability of 1/24 and 1/36 respectively. Scheme 4 with total 12 PRBs has a DMRS collision probability of 1/48. The other 3 mapping types have no DMRS collision probability.

Note: X label is normalized SNR in order to ensure that each scheme has the same transmit power.

(a). 1T2R

(b). 1T4R

Figure 14 BLER vs. SNR for 2UEs with Preamble Format 0

(a). 1T2R

(b). 1T4R

Figure 15 BLER vs. SNR for 2UEs with Preamble Format A1

Based on the evaluation results shown in figure 14, it can be found that with normalized SNR, different mapping schemes show similar performance at the 0.1 BLER operation point for 2Rx. With more equipped antennas for example 4, the mapping types scheme 1 and 2 have slightly better performance at 0.1 BLER. But these two schemes possessing an unsolved error floor with lower operation point such as 0.01 BLER especially for 2 receiving antennas, which are mainly caused by higher collision probability on DMRS.

Performance of different formats does not show significant difference, which means the accuracy of TO estimation by short preamble format A1 should be good enough for 4G+200m scenario.

Schemes with total 12 PRBs have less than 1dB performance better than total that of total 6 PRBs.

Although scheme 1 has the highest DMRS collision probability, the overall impact of performance loss can be covered by the benefit from low code rate.

Observation 3: With the aligned transmit power and total reserved resource overhead, the mapping schemes with multiple preambles to one DMRS mapping have slightly better performance than one-to-one mapping (single DMRS sequence) at 10% BLER, but at the meantime suffer from a higher error floor for lower operation points, e.g. 1% BLER.

## Comparison between different DMRS sequences

There is an opinion to introduce more DMRS sequences to avoid DMRS collision without increasing the total resource allocation. The following simulation results show the comparison between the performance of 1 DMRS sequence with collision and multiple DMRS sequences without collision. The scheme 1, 2 and 4 are considered in this case.

It can be found from Figure 16 and 17 that the performance of 1-to-1 mapping with multiple DMRS sequences (i.e. no DMRS collision) is similar with that of multiple-to-1 mapping with 1 DMRS sequence  in most of the evaluated cases. While in particular cases (e.g. with 6 total PRBs, 3 PRBs per UE, and 4 Rx), the error floor could be lower by introducing multiple DMRS sequences.

(a)1T2R

(b) 1T4R

Figure 16 Comparison between different DMRS sequences with Preamble Format 0

(a). 1T2R

- . 1T4R

Figure 17 Comparison between different DMRS sequences with Preamble Format A1

Observation 4: Performance of multiple DMRS sequences without collision is similar with that of 1 DMRS sequence with potential collision in most of the evaluated cases. While in particular cases (e.g. with 6 total PRBs, 3 PRBs per UE, and 4 Rx), the error floor could be lower by introducing multiple DMRS sequences.

# Time domain PO overlap

To evaluate the performance loss due to the potential overlapping between the POs associated to different RO, the following simulations are conducted. Suppose there are 4 POs, which are totally overlapped in frequency domain. 4 POs are divided into 2 PO groups, PO1 and PO2 belong to the first PO group, PO3 and PO4 belong to the second PO group, each PO occupies a slot. In each PO group, 2 UEs randomly select a PO, there are 3 cases in this scenario. In this part of evaluation, only 1 DMRS sequence is considered and there is no DMRS collision.

- 1 slot overlap

- 2 slots overlap

- No overlap

Figure.18 Different cases on time domain overlap

Figure.19 Comparison between different time domain overlap types

Compared with no overlap case, performance of 1 slot overlap case has about 0.5dB degradation, performance of 2 slots overlap case is more than 1 dB worse.

Figure.20 Comparison between different time domain overlap types

For 1 slot overlap case, performance at 0.1 BLER is similar with 2 UEs per slot. Considering more than 2 UEs may be multiplexed on the overlapped slot, which causes the error floor at 0.01BLER with MMSE-IRC receiver. As no DMRS collision is considered in this chapter, the result of overlapped POs will be even worse if DMRS are randomly selected.

Observation 5: Time domain PO overlap will impose more UEs multiplexed on the same resources, and thus cause certain performance loss, e.g. less than 1 dB performance degradation at 0.1 BLER for 1 PO overlapping. Larger performance degradation is expected with the increasing of the number of overlapped POs.

# Frequency gap

In frequency domain, the performance of a PO will be affected by adjacent POs because of the time/frequency offset. In this chapter, the necessity of frequency gap and influence of adjacent POs will be evaluated. Suppose there are 3 PRBs in frequency domain, fully overlapped in time domain with 1 slot.  Each PO is occupied by 1 UE. There are 3 cases for this scenario:

Case 1: 2 UEs with gap        Case 2: 2 UEs without gap                Case 3: 3 UEs without gap

Figure.21 Different cases on frequency gap

Figure.22 Comparison between different frequency gap cases

- Compare Case 1 and Case 2, with the same frequency resource, performance of 2 UEs with gap is about 1dB worse than 2 UEs without gap, which means lower code rate has greater influence than frequency domain interference.

- Compare Case 1 and Case 3, with the same frequency resource, performance of 2 UEs with gap is about 0.5dB better than 3 UEs without gap, which means the frequency gap can give 0.5 dB gain in case of the same code rate.

Figure.23 Performance of different UEs in Case 3

For Case 3, the performance of UE1 and UE3 on the sides, and UE2 in the middle are similar.

Figure.24 Impact of frequency offset on adjacent UEs

FO is the main reason for performance degradation of Case 3 compared with Case 1.

Observation 6: 1PRB frequency gap between POs can avoid the performance loss caused by time/frequency offset, at the cost of reduced resource utilization. When the total frequency resource is limited, making full use of resources can achieve better performance than leaving 1PRBs guard band within each PO.

# Conclusion

Observation 1: With appropriate settings of resource size and antenna number, the minimum payload size same as msg3 (i.e. 56/72 bits) can be supported in different scenarios (cell size with 200, 500, 1732m) with 95% coverage. Larger payload sizes or cells sizes can be supported by allocating more resources or adding receiving antennas, e.g. for 200m ISD, up to 160 bytes can be supported with 4Rx and 6PRB allocated bandwidth.

Observation 2: For long preamble format (format 0, 700M 1732m), the coverage of preamble is much better than that of 10bytes payload for both 2 and 4 receiving antennas. For short preamble format (format A1 A2, 4G 200m, 4G 500m), the coverage difference between preamble and 10bytes payload is smaller.

Observation 3: With the aligned transmit power and total reserved resource overhead, the mapping schemes with multiple preambles to one DMRS mapping have slightly better performance than one-to-one mapping (single DMRS sequence) at 10% BLER, but at the meantime suffer from a higher error floor for lower operation points, e.g. 1% BLER.

Observation 4: Performance of multiple DMRS sequences without collision is similar with that of 1 DMRS sequence with potential collision in most of the evaluated cases. While in particular cases (e.g. with 6 total PRBs, 3 PRBs per UE, and 4 Rx), the error floor could be lower by introducing multiple DMRS sequences.

Observation 5: Time domain PO overlap will impose more UEs multiplexed on the same resources, and thus cause certain performance loss, e.g. less than 1 dB performance degradation at 0.1 BLER for 1 PO overlapping. Larger performance degradation is expected with the increasing of the number of overlapped POs.

Observation 6: 1PRB frequency gap between POs can avoid the performance loss caused by time/frequency offset, at the cost of reduced resource utilization. When the total frequency resource is limited, making full use of resources can achieve better performance than leaving 1PRBs guard band within each PO.

# References

- RP-182894, “New work item: 2-step RACH for NR”, ZTE Corporation, Sanechips, 3GPP RAN#89, Sorrento, Italy, December 2018.

- “Chairman’s notes”. 3GPP RAN1#96, Athens, Greece, February 2019.

- “Chairman’s notes”. 3GPP RAN1#96bis, Xi’an, China, Apr. 2019.

- “Chairman’s notes”. 3GPP RAN1#97, Reno, USA, May 2019.

- R1-1905989, “Further discussions on the channel structure of msgA”, ZTE Corporation, Sanechips, 3GPP RAN1#97, Reno, USA, May 2019.

- R1-1908181, “On the remaining issues of msgA channel structure”, ZTE, Sanechips, 3GPP RAN1#98, Prague, Czech Republic, Aug. 2019.

- R1-1903834, “Email discussion on potential link-level simulation assumption for 2-step RACH”, ZTE Corporation, 3GPP RAN1#96, Athens, Greece, Feb. 2019.

- 3GPP TR38.812, “Study on Non-Orthogonal Multiple Access (NOMA) for NR”, Release 15, v1.0.0

# Appendix

## A.1 Link Level simulation assumptions

Table A1: Link-level evaluation assumptions

| Parameters | Values |
| --- | --- |
| Scenario | 1) For evaluation of schemes: 200m, UMi, 4 GHz. 2) For evaluation of payload size: 200m, UMi, 4 GHz; or 500m, UMa, 4 GHz; or 1732m, RMa, 700 MHz. |
| Preamble format | long Format 0 short Format A1 A2 |
| Waveform (data part) | CP-OFDM |
| Subcarrier spacing for PUSCH | 15kHz at 700MHz, 30kHz at 4 GHz |
| TBS | As described in section 2. |
| MCS and Resource size | QPSK or 16QAM, [1 3 6 12] PRBs |
| Number of UEs | 1 or 2 |
| Traffic model | Full buffer |
| UE antenna configuration | 1Tx |
| gNB antenna configuration | 2Rx or 4Rx |
| Propagation channel & UE velocity | TDL-A 30ns, 3km/h |
| Timing offset | Uniform [0, RTT]. |
| Frequency offset | 105 Hz for 700MHz, 600 Hz for 4 GHz |
| Max number of HARQ transmission | 1 |
| Receiver | MMSE-IRC |
| Channel estimation | Realistic for both channel estimation and TO estimation. |
| Target BLER | 10% |
| Performance metrics | 1) Missed detection probability vs. SNR for a given false alarm rate, e.g. 0.1%; 2) BLER vs. SNR; MCL can be reported using link budget calculations. 3) Optional: False alarm probability vs. SNR |

## A.2 System Level simulation assumptions

Table A2 System level simulation assumptions

| Parameters | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |  |  |
| Inter-BS distance | 1732m | 500m | 200m |
| Minimum distance between UE and BS | 35m | 10m | 10m |
| Carrier frequency | 700MHz | 4GHz | 4GHz |
| Bandwidth | 10MHz |  |  |
| Channel model | UMa in TR 38.901 |  |  |
| Building penetration loss | The building penetration model defined in Table 7.4.3-3 in TR 38.901 is used for SLS with frequencies below 6 GHz |  |  |
| BS Tx power | 46 dBm | 46 dBm | 41 dBm |
| BS antenna configurations | 2 ports: (M, N, P, Mg, Ng) = (10, 1, 2, 1, 1), +-45 Polarization; dH = dV = 0.8λ; |  |  |
| TXRU mapping to antenna elements on BS side | One TXRU per vertical dimension per polarization |  |  |
| TXRU mapping weights on BS side | TXRU virtualization only in the vertical dimension, i.e., sub-array partition model with 1D virtualization, refer to TR36.897 |  |  |
| Polarized antenna modeling | Model-2 in TR36.873 |  |  |
| BS antenna downtilt | 92 | 98 | 102 |
| BS antenna height | 25m |  |  |
| BS antenna element gain + connector loss | 8 dBi, 0dB cable loss |  |  |
| BS receiver noise figure | 5dB |  |  |
| UE Tx power | Max 23 dBm |  |  |
| UE antenna configuration | 1 (vertical polarization) |  |  |
| UT array orientation | uniformly distributed on [0,360] degree |  |  |
| UE antenna gain | 0dBi |  |  |
| UE receiver noise figure | 9dB |  |  |
| UE distribution and antenna height | 20% of users are outdoors (3km/h), 80% of users are indoor (3km/h); Users dropped uniformly in entire cell. |  |  |
| UE power control | Open loop PC, P0 = [-90] dBm, alpha = 1. |  |  |
| HARQ/repetition | 1 |  |  |
| Handover margin | 0dB |  |  |
| Wrapping method | Geographical distance based wrapping |  |  |
| UE attachment | Based on RSRP |  |  |