3GPP TSG RAN WG1 Meeting #98                             	    R1-1908208

Prague, CZ, 26th - 30th August, 2019

Title:          	Discussion on configured grant for NR-U

Source: 	ZTE, Sanechips

Agenda Item:	7.2.2.2.4

Document for:	Discussion and Decision

Introduction

In RAN #82 meeting, the new WID on NR-U (NR-based Access to Unlicensed Spectrum) [1] aims to identify solutions and techniques for next generation wireless systems operating on unlicensed bands. In the WID, one of the objectives is to support configured grant operation. In RAN1 #97 meeting, the following agreements were reached on configured grant for NR-U.

Agreement:

For configured grant time domain resource allocation, the mechanisms in Rel-15 (both Type 1 and Type 2) are extended so that the number of allocated slots following the time instance corresponding to the indicated offset can be configured

- FFS: How to indicate multiple PUSCHs within a slot.

Agreement:

UE can only start transmissions accessing transmission opportunities provided by a configured grant at the configured/indicated starting position.

In RAN #84 meeting, the following conclusion were reached on configured grant for NR-U prioritization:

Essential

- DFI design, including content and CG PUSCH to DFI timing

- UCI design, including content and multiplexing

Optimizations

- Ending symbol flexibility

- CBG based transmission with CG resource

In this contribution, we will discuss configured grant aspects for NR-U, including HARQ related enhancement, UCI design, Resource configuration for configured grant and UE multiplexing and collision handling.

HARQ related enhancement

CBG-based retransmission

Rel-15 NR supports CBG based transmission and HARQ-ACK feedback. gNB or UE can retransmit the failed CBGs instead of the entire TB. This mechanism is quite useful when some CBGs suffer from bursty interference in unlicensed band. In RAN1 #96 meeting, CBG-based retransmission by using dedicated scheduled resource has been approved as shown in the agreement below.

Agreement:

For PUSCH transmitted using CG, CBG-based retransmission is supported at least by using dedicated scheduled resource allocated by an UL grant.

- FFS: CBG-based retransmission using a configured grant

- Note: Include this agreement in an LS to RAN2 informing them of relevant RAN1 agreements

CBG-based retransmission by using configured resource has some advantages. For example, UE can transmit the CBG-based retransmission autonomously based on the NACK information included in the DFI. Meanwhile, UE can save resource overhead and signaling overhead for the reason that UE does not need to use dedicated scheduled resource by an UL grant. Therefore, CBG-based retransmission by using configured grant resource should also be approved.

Besides, how to design CBG related control information included in the DFI or UCI should also be considered. Here two options can be considered for configured grant CBG operation. And these two options can be combined used.

- Option 1: UE selects the configured grant UL CBG transmission and the transmitted CBGs information, such as the CBGTI can be carried in the UCI.

- Option 2: the DFI can provide CBG level feedback indication while UE can retransmit the failed CBGs based on the DFI feedback information. Or gNB uses the grant based scheduling to schedule the CBG retransmission and uses the CBGTI in the DCI to indicate which CBG is scheduled.

Proposal 1:

- CBG-based retransmission with CG resource should be supported.

- CBG level ACK/NACK feedback information can be included in the DFI.

HARQ related signaling and procedures

In RAN1 #93 meeting, it was agreed to introduce UCI on PUSCH to carry HARQ process ID, NDI, RVID. Therefore, gNB can obtain HARQ ID and NDI value from a successfully detected PUSCH and can use the above information to decide the transmission is initial transmission or retransmission. Specifically, for the same HARQ ID, toggled NDI means the transmission is initial transmission and non-toggled NDI means the transmission is retransmission.

In FeLAA AUL, DFI is introduced to indicate HARQ-ACK for the PUSCH. In the DFI, eNB needs to feedback one HARQ bit for every HARQ process. Assuming 16 HARQ processes configured for CG transmission, and each TB has 8 CBGs configured, the DFI size should be very large. In order to reduce DFI overhead and the number of blind detection, a scheme that limits DFI size should be introduced. Here two options can be considered.

- Option 1: Similar to DAI, UAI can be introduced in UCI and gNB only need to feed back TB-level ACK/NACK for actually PUSCH transmission according to UAI. For the HARQ process in which the TB-level HARQ feedback is NACK, gNB should also feed back CBG-level ACK/NACK for the HARQ process.

- Option 2: For the HARQ process configured to configured grant transmission, gNB feed back a bitmap which can be TB-level ACK/NACK and CBG-level ACK/NACK. The bitmap is used to indicate which PUSCH transmission of the HARQ processes are received by the gNB. For the HARQ process of PUSCH transmission, gNB feeds back TB-level ACK/NACK according to the PUSCH decoding result. Similar to option 1, for the HARQ process in which the TB-level HARQ feedback is NACK, gNB should also feed back CBG-level ACK/NACK for the HARQ process.

For example, gNB configures HARQ process 1-8 for configured grant transmission and UE transmits PUSCH by HARQ process 2, 5, 7 and 8. The number of CBG per TB is configured as 4. gNB receives the PUSCH transmission of 2, 7 and 8 and correctly decodes PUSCH transmission of HARQ process 2 and 8. For PUSCH transmission of HARQ process 7, CBG1 and CBG4 is not correctly decoded and the PUSCH transmission of HARQ process 5 is missed. Figures 1 and 2 are examples of HARQ-ACK feedback for Option 1 and Option 2, respectively.

Figure 1  HARQ-ACK feedback for Option1

Figure 2  HARQ-ACK feedback for Option2

Furthermore, to reduce the UE blind detection complexity, the DFI size should be aligned with an existing DCI size. And the transmission time location of DFI can be indicated by COT sharing in UCI or the same as other DCI configured by search space.

In AUL, UE expects HARQ-ACK feedback in the AUL-DFI at earliest in subframe n+4 for an uplink transmission in subframe n. However, NR-U does not have a similar regulation and NR-U CG UE transmits a PUSCH transmission and receives a DFI feedback immediately. The UE may misunderstand the DFI feedback ‘0’ as NACK and perform a wrong CWS adjustment and initiate a HARQ retransmission. Therefore, a minimum CG PUSCH to DFI timing should be introduced. Reference timing of the PDSCH to HARQ-ACK feedback is N1 symbols, the minimum timing between CG PUSCH and DFI can be N1+X symbols, otherwise DFI feedback is invalid for the CG PUSCH.

For DFI content, besides the HARQ-ACK, additional information bits can also be introduced in DFI. For example, a bit flag is used to distinguish the DFI and UL grant for active/deactive CG resource and 2 bits TPC is used to adjust PUSCH transmission power. Meanwhile, the CIF and the BWP index should be included to tell UE which carrier and BWP configured grant transmission it feedback. In addition, precoding information should also be included for the reason gNB can notify the UE to do channel change adaptation by updating the precoding information.

Proposal 2: Transmission is initial or retransmission, which can be judged by NDI.

Proposal 3: DFI size should be similar to the UL grant DCI size and the solution to minimize DFI size should be introduced if CBG-level HARQ-ACK feedback is included in DFI.

Proposal 4: The minimum timing between CG PUSCH and DFI should be introduced and it can be N1+X symbols.

UCI design

UCI content

For NR configured grant, since HARQ ID is implicit from selected time resource, only synchronous HARQ is supported for configured grant. However, if the UE fails LBT on the predetermined HARQ locations, it needs to wait till next HARQ instances for uplink transmission, which will cause large delay. Considering the uncertainty of medium access, asynchronous HARQ should be supported for NR-U configured grant. In RAN1 #93 meeting, it was agreed to introduce UCI on PUSCH to carry HARQ process ID, NDI, RVID. Therefore, asynchronous HARQ can be achieved.

In RAN1 AH_1901 meeting，it was agreed to introduce COT sharing information in CG UCI. In NR-U study phase, besides HARQ process ID, NDI, RVID, COT sharing information, additional information fields were proposed, e.g. UE-ID, PUSCH duration. However, considering the PUSCH starting position and duration can be configured in RRC or indicated by DCI, the above mentioned information is not necessary to be carried in UCI. And as the gNB can identify the UE ID through the DMRS, UE ID is also no longer necessary to be included in UCI.

As discussed in section 2.1, CBGTI should also be included in the UCI. Meanwhile, according to the discussion of section 2.2, UAI may also be included in the UCI.

Proposal 5:

- UE ID is not need to be included in the UCI.

- CBGTI should be included in the UCI.

- UAI may also be included in the UCI.

UCI multiplexing

Due to flexible candidate transmission occasion within a period, PUSCH starting position can be anywhere in the slot and gNB needs to blindly detect specific PUSCH transmission occasion. Similar to Rel-15 configured grant mapping Type B transmission, gNB can detect specific PUSCH transmission occasion via front loaded DMRS and then decode UCI to achieve PUSCH-controlled information and decoding PUSCH. Therefore, Rel-15 mapping rules in NR can be reused for UCI multiplexing on PUSCH. Namely, configured grant NR-U UCI can be mapped from the first symbol after DMRS in PUSCH on the allocation subband.

Proposal 6: The UCI can be mapped using the Rel-15 UCI multiplexing rules.

COT sharing

In FeLAA, UE supporting AUL can initiate a COT and eNB can share the COT to transmit DL control information to the UE. But UL-DL-UL sharing is not supported which is constrained by frame structure 3. However, in NR, as flexible frame structure is allowed, and multiple UL to DL and DL to UL switching within a shared COT obtained by UE can be supported. In the NR-U SI stage, it was identified that sharing resources with gNB within COT(s) that is acquired by UE(s) as part of configured grant based transmissions should be supported. Besides, the resource sharing mechanisms and rules should be determined. For example, in the CG UCI, the COT sharing information can be the remaining slot number and the symbol number of the MCOT. If the BWP transmitted for gNB is the same as the UE configured grant UL transmission, then gNB can perform a Cat-2 LBT or no LBT if the gap between the end of the configured grant UL transmission and the beginning of the DL burst is not more than 16 sec. Besides, gNB can schedule this UE to transmit UL in the remaining of the COT according to the rules set by EN 301 893. For this problem, the rules should be the same as gNB initiated COT sharing.

It was also agreed to introduce Downlink Feedback Information (DFI) including HARQ feedback for configured grant transmission, and UE may perform non-adaptive retransmission for configured grant according to DFI including NACK feedback. However, improving the DFI and the CS-RNTI scrambled DCI transmission opportunity may also be a good choice especially for the NACK case. The scheme of the COT sharing as illustrated in Figure 3 may be used as a tool to solve this problem. After PUSCH is received, gNB can transmit DFI or the CS-RNTI scrambled DCI within the same COT. At UL-DL switching point, no LBT or one shot LBT (Cat-2 LBT) will be performed for gNB, which can decrease the LBT impact on DFI and the CS-RNTI scrambled DCI transmission.

Figure 3 An example of UE COT sharing scheme

Proposal 7: The COT sharing information can be the remaining slot number and the symbol number of the MCOT.

Proposal 8: The resource sharing mechanisms and rules for UE initiated COT sharing is the same as the gNB initiated COT sharing.

Proposal 9: A method for decreasing the LBT impact on DFI and the CS-RNTI scrambled DCI transmission can be considered, such as COT sharing scheme with configured grant transmission UE.

Resource configuration for configured grant

Autonomous uplink access (AUL) with frame structure type 3 has been specified in Rel-15 FeLAA. Meanwhile, configured grant Type 1 and Type 2 were also specified in Rel-15 for NR UL data transmission. Compared with the grant-based access, both AUL and configured grant scheme have the following advantages: energy efficiency on UE side, very low cost/complexity with UE design, reduced signalling overhead and transmission latency. These advantages are also the goals of NR-U. And in the RAN1 92bis meeting, it was agreed to support configured grant scheme for NR-U UL transmission, with necessary enhancements or changes taking into account of unique characteristic of unlicensed band operation.

In configured grant Type 1 and Type 2, gNB can configure a UE with periodicity and repetition times by RRC and some enhancement should be considered. For instance, the number of repetitions can be re-interpreted as candidate transmission occasions within the periodicity as illustrated in Figure 4. UE can transmit PUSCH after a successful LBT before any of the candidate transmission opportunities scheduled by the configured grant, and gNB can perform blind detection on uplink data according to the configuration.

Figure 4 An example of multiple CCA or transmission occasion candidates within a period for NR-U CG-PUSCH

In AUL, ending symbol of CG PUSCH can be configured as symbol #12 or #13. In NR-U, a configurable ending symbol should also be introduced. Different from AUL, NR-U supports multiple numerology and determining the ending symbol of NR-U CG PUSCH should consider SCS and LBT gap. For example, LBT gap only needs 1 symbol for 15/30kHz SCS&25 us Cat2 LBT. However, it needs at less 2 symbols for 60kHz SCS&25 us Cat2 LBT.

In order to increase UL transmission opportunities, multiple frequency domain opportunities can also be considered for NR-U. For example, multiple subbands can be configured to UE. When UL data arrives, UE can attempt to perform LBT in multiple subbands according to the resource configuration for configured grant. If any LBT on these subbands succeeds, UE can transmit on either multiple subbands or a selected one. More details may be further studied on multiple subbands transmission opportunities.

Proposal 10: Determining the ending symbol of NR-U CG PUSCH should consider subcarrier spacing and LBT gap.

Proposal 11: Multiple candidate transmission opportunities in time domain and/or frequency domain can be configured for configured grant.

UE multiplexing and collision handling

For NR configured grant, as gNB may configure multiple UEs to share the same resource, UL transmission collisions might occur. There are two cases to be considered.

- Case1: configured grant UE is allocated to occupy parts of the channel bandwidth

In this case, different UE allocate different interlaces, but LBT is performed on the whole BWP or bandwidth. If one UE successfully performs LBT earlier and transmits data on the configured BWP, then other UEs may not be able to access channel due to LBT failure on the same BWP. Therefore multiple channel access or UL multiple user multiplexing cannot be achieved. Some mechanisms should be considered to resolve these collision problems among multiple UEs attempting to access the same PUSCH resource. For example, blank pattern method as discussed in [2] or same UL data transmission starting point mechanism can be used to reduce UL collisions.

- Case2: configured grant UE is allocated to occupy whole of the channel bandwidth

In this case, a configured grant UE is allocated to occupy the full channel bandwidth. The same resource would be used by other UEs to avoid the resource waste if this UE has LBT failure or with no data to transmit. To alleviate the collision problem when multiple UEs attempting to access the same PUSCH resource, FeLAA AUL approach (i.e., UE-selected starting offset and RRC configured starting offset set) can be the baseline. Further enhancement can be considered for NR flexible and more starting point support.

Proposal 12: To support multiple user access for NR-U considering LBT influence, the methods below can be considered:

- same UL data transmission starting point mechanism

- blank pattern method

- FeLAA AUL approach based

Conclusion

In this contribution, we discuss the configured grant aspects for NR operation in unlicensed spectrum, and have the following proposals:

Proposal 1:

- CBG-based retransmission with CG resource should be supported.

- CBG level ACK/NACK feedback information can be included in the DFI.

Proposal 2: Transmission is initial or retransmission, which can be judged by NDI.

Proposal 3: DFI size should be similar to the UL grant DCI size and the solution to minimize DFI size should be introduced if CBG-level HARQ-ACK feedback is included in DFI.

Proposal 4: The minimum timing between CG PUSCH and DFI should be introduced and it can be N1+X symbol.

Proposal 5:

- UE ID is not need to be included in the UCI.

- CBGTI should be included in the UCI.

- UAI may also be included in the UCI.

Proposal 6: The UCI can be mapped using the Rel-15 UCI multiplexing rules.

Proposal 7: The COT sharing information can be the remaining slot number and the symbol number of the MCOT.

Proposal 8: The resource sharing mechanisms and rules for UE initiated COT sharing is the same as the gNB initiated COT sharing.

Proposal 9: A method for decreasing the LBT impact on DFI and the CS-RNTI scrambled DCI transmission can be considered, such as COT sharing scheme with configured grant transmission UE.

Proposal 10: Determining the ending symbol of NR-U CG PUSCH should consider subcarrier spacing and LBT gap.

Proposal 11: Multiple candidate transmission opportunities in time domain and/or frequency domain can be configured for configured grant.

Proposal 12: To support multiple user access for NR-U considering LBT influence, the methods below can be considered:

- same UL data transmission starting point mechanism

- blank pattern method

- FeLAA AUL approach based

References

- 3GPP, RP-182878, New WID on NR-based access to unlicensed spectrum, Qualcomm, December, 2018, RAN#82

- R1-1908205, “Discussion on channel access procedure for NR-U”, ZTE, Sanechips, August, 2019, RAN1 #98