3GPP TSG RAN WG1 Meeting #98 	R1-1908228

Prague, CZ, 26th August – 30th August 2019

Title:	Link budget analysis

Source: 	Thales

Type:	Discussion

Document for:	Approval

Agenda Item:	7.2.5.5

Study Item: 	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Introduction

In June 2018, the study item “Study on NR to support non-terrestrial networks” was completed. Potential impacts have been identified (see [1] for the full list) and solutions are currently investigated in the study item “Solutions on NR to support non-terrestrial networks”[2].

During the last 3GPP RAN WG1 meeting in Reno, the following agreements [3] have been achieved :

Agreement:

R1-1907836	TP for Link Budget Analysis in TR 38.821	Nokia, Nokia Shanghai Bell, Thales, MediaTek, Ericsson, ZTE

The text proposal in Section 2 of R1-1907836 is endorsed

This document aims to present the link budgets obtained based on the methodology agreed during last meeting and captured in section 6.1.3 of [2]. The link budgets analysis is performed so the lowest CINR values achievable in NTN can be evaluated. The analysis results should help adjusting the CINR range that should be considered when running link level simulations.

# Link budget analysis

### GEO in Ka-Band with VSAT terminals

#### Satellite characteristics

We have considered the Set 1 satellite parameters for GEO satellites operated in Ka-band coming from Table 6.1.1-1 of [2]. For both uplink and downlink transmissions, we applied a loss of 3 dB on the satellite antenna gain as it would be the case if the terminal was located at the edges of the HPBW coverage.

#### Terminal characteristics

We have considered VSAT terminals. The VSAT characteristics used here are coming from Table 6.1.1-3 of [2].

#### Frequency reuse factor and polarization reuse

We assumed a frequency reuse factor of 2. Furthermore, polarization reuse is enabled since VSAT antennas are circularly polarized. As a consequence, the beam layout described on Figure 1 is adopted. From our perspective, it is probably one of the most representative configurations among existing GEO based SatCom systems.

Figure 1 : Adopted beam layout for GEO in Ka-band configuration

#### Elevation angle

We assumed an elevation angle of 10° on the transmission link as instructed in section 6.1.3 of [2].

#### Atmospheric loss

Concerning the atmospheric loss, we assumed the Azenith values proposed in Figure 6 of [4] at 20 GHz for the downlink transmission and at 30 GHz for the uplink transmission.

#### Scintillation loss

Concerning the scintillations loss, only the tropospheric scintillation loss are considered. We used the values proposed in Table 6.6.6.2.1-1 of [2] corresponding to an elevation angle of 10°.

#### Shadowing margin

Concerning the shadowing margin, we assumed a 0 dB margin as instructed in section 6.1.3 of [2] when VSAT terminals are considered.

#### Additional margins

Finally, no additional margins were assumed.

#### Carrier to Interference Ratio (CIR)

The CIR values used in the link budgets are coming from calibration results. The calibration framework for single satellite simulation described in [2] has been respected. The bore sight direction of the central beam is chosen such that the central beam center elevation angle is fixed at 10°.

The calibration results are performed inside the central beam. The 18 additional neighbour beams are used as wrap-around beams so realistic interference levels can be computed.

Furthermore, the shadow fading has been neglected during the calibration procedure since only VSATs are considered.

The CIR CDF values at 1%, 50% and 99% of time are reported in Table 1. We propose to consider the 1% of time value in the link budget computation in order to evaluate the worst case CINR (assuming clear sky conditions).

Table 1 : CIR CDF values on DL and UL transmissions for GEO in Ka-Band with VSATs

| % of time CIR < X | X [dB] for DL | X [dB] for UL |
| --- | --- | --- |
| 1% | 10.47 | 8.96 |
| 50 % | 10.79 | 12.10 |
| 99 % | 11.65 | 16.40 |

#### Link budget analysis

The results captured in Table 2 and Table 3 should be taken in to account by RAN WG1 to carry out the link budget analysis in GEO based NTN.

##### Down Link

Table 2 : Down Link budget analysis for GEO in Ka-Band with VSATs

| Satellite parameters | - | Set 1 |
| --- | --- | --- |
| Satellite orbit | - | GEO |
| Frequency band | - | Ka |
| Frequency | GHz | 20,00 |
| Elevation | deg | 10,00 |
| Polarization reuse | - | YES |
| Frequency reuse factor | - | 2 |
| System bandwidth | MHz | 400,00 |
| Bandwidth per beam | MHz | 200,00 |
| Max bandwidth per UE | MHz | 200,00 |
| EIRP |  |  |
| Max EIRP density | dBW/MHz | 40,00 |
| Antenna gain loss | dBi | 3,00 |
| EIRP | dBW | 60,01 |
| G/T |  |  |
| N_f | dB | 1,20 |
| Ta | K | 150,00 |
| T0 | K | 290,00 |
| G_R | dBi | 39,70 |
| L_p | dB | 0,00 |
| G/T | dB/K | 15,86 |
| Boltzmann constant |  |  |
| k | dBW/K/Hz | -228,60 |
| Free Space Loss |  |  |
| PL_FS | dB | 210,64 |
| Atmospheric Loss |  |  |
| A_zenith | dB | 0,28 |
| PL_A | dB | 1,58 |
| Scintillation Loss |  |  |
| Tropospheric attenuation with 99% | dB | 1,08 |
| PL_S | dB | 1,08 |
| Shadowing Margin |  |  |
| PL_SM | dB | 0,00 |
| Additional Margin |  |  |
| PL_AD | dB | 0,00 |
| CNR |  |  |
| CNR | dB | 8,16 |
| CIR |  |  |
| CIR (from calibration results) | dB | 10,47 |
| CINR |  |  |
| CINR | dB | 6,15 |

##### Up Link

Table 3 : Up Link Budget analysis for GEO in Ka-Band with VSATs

| Satellite parameters | - | Set 1 |
| --- | --- | --- |
| Satellite | - | GEO |
| Frequency band | - | Ka |
| Frequency | GHz | 30,00 |
| Elevation | deg | 10,00 |
| Polarization reuse | - | YES |
| Frequency reuse factor | - | 2 |
| System bandwidth | MHz | 400,00 |
| Bandwidth per beam | MHz | 200,00 |
| Max bandwidth per UE | MHz | 200,00 |
| EIRP |  |  |
| P | dBW | 3,01 |
| G_T | dBi | 43,2 |
| L_C | dB | 0,50 |
| EIRP | dBW | 45,71 |
| G/T |  |  |
| Max G/T | dB/K | 28 |
| Antenna gain loss | dBi | 3 |
| L_P | dB | 0 |
| G/T | dB/K | 25 |
| Boltzmann constant |  |  |
| k | dBW/K/Hz | -228,60 |
| Free Space Loss |  |  |
| PL_FS | dB | 214,16 |
| Atmospheric Loss |  |  |
| A_zenith | dB | 0,22 |
| PL_A | dB | 1,27 |
| Scintillation Loss |  |  |
| Tropospheric attenuation with 99% prob | dB | 1,08 |
| PL_S | dB | 1,08 |
| Shadowing Margin |  |  |
| PL_SM | dB | 0,00 |
| Additional Margin |  |  |
| PL_AD | dB | 0,00 |
| CNR |  |  |
| CNR | dB | -0,21 |
| CIR |  |  |
| CIR (from calibration results) | dB | 8,96 |
| CINR |  |  |
| CINR | dB | -0,70 |

### LEO in S-Band with Handheld

#### Satellite characteristics

We have considered the Set 1 satellite parameters for LEO satellites located at 600 km altitude and operated in S-band coming from Table 6.1.1-1 of [2]. We focused the link budget analysis on the 600 km LEO configuration since it seems to be the best option to provide direct access services to handheld devices. For both uplink and downlink transmissions, we applied a loss of 3 dB on the satellite antenna gain as it would be the case if the terminal was located at the edges of the HPBW coverage.

#### Terminal characteristics

We have considered handheld terminals. The handheld terminal characteristics used here are coming from Table 6.1.1-3 of [2].

#### Frequency reuse factor and polarization reuse

We assumed a frequency reuse factor of 3. Furthermore, polarization reuse is disabled since handheld terminal antennas are linearly polarized. As a consequence, the beam layout described on Figure 1 is adopted. From our perspective, it is probably the most promising configuration when linear polarized terminals should be considered in the system.

Figure 2 : Adopted beam layout for LEO in S-band configuration

#### Elevation angle

We assumed an elevation angle of 30° on the transmission link as instructed in section 6.1.3 of [2].

#### Atmospheric loss

Concerning the atmospheric loss, we assumed the Azenith values proposed in Figure 6 of [4] at 2 GHz for both downlink and uplink transmissions.

#### Scintillation loss

Concerning the scintillations loss, only the ionospheric scintillation loss are considered. If we assume having a LEO constellation based on inclined orbits then no service access is provided in the high latitudes regions. As a consequence, the most severe scintillation loss experienced on ionospheric paths will be observed in the equatorial areas.

Based on the procedure defined in section 6.6.6.1.4 of [1], we derived a Pfluc value based on the statistic curves displayed on Figure 3. The I5 curve is representative of one of the most intensive scintillation periods and corresponds to the right elevation angle. We considered the Pfluc value given at 1% of time :

Pfluc@4GHz=3 dB

Then, we proceeded to the frequency scaling.

Pfluc@2GHz=3×24-1.5=8.48 dB

Finally, we derived the following scintillation loss.

PLS=AIS=Pfluc2=6 dB

Figure 3 : Annual statistics of peak-to-peak fluctuations observed at Hong Kong earth station (Curves I1, P1, I3-I6, P3-P6) and Taipei earth station (Curves P2 and I2). Extracted from [1]

#### Shadowing margin

Concerning the shadowing margin, we assumed a 3 dB margin as instructed in section 6.1.3 of [2] when VSAT terminals are considered.

#### Additional margins

Finally, no additional margins were assumed.

#### Carrier to Interference Ratio (CIR)

The CIR values considered here are also coming from calibration results. The bore sight direction of the central beam is chosen such that the central beam center elevation angle is fixed at 30°.

This time the shadow fading model has been considered during the calibration procedure (based on the instructions described in section 6.6.2 of [1]) since handheld terminals are considered.

The CIR CDF values at 1%, 50% and 99% are reported in Table 4. Once again, we have considered the 1% of time value for the link budget computations.

Table 4 : CIR CDF values on DL and UL transmissions for LEO-600 in S-Band with handheld terminals

| % of time CIR < X | X [dB] for DL | X [dB] for UL |
| --- | --- | --- |
| 1% | 9.38 | 6.27 |
| 50 % | 10.59 | 10.81 |
| 99 % | 11.07 | 15.58 |

#### Link budget analysis

The results captured in Table 5 and Table 6 should be taken in to account by RAN WG1 to carry out the link budget analysis in LEO based NTN.

##### Down Link

Table 5 : Down Link budget analysis for LEO in S-Band with handheld terminals

| Satellite parameters | - | Set 1 |
| --- | --- | --- |
| Satellite | - | LEO |
| Altitude | km | 600,00 |
| Frequency band | - | S |
| Frequency | GHz | 2,00 |
| Elevation | deg | 30,00 |
| Polarization reuse | - | NO |
| Frequency reuse factor | - | 3 |
| System bandwidth | MHz | 30,00 |
| Bandwidth per beam | MHz | 10,00 |
| Max bandwidth per UE | MHz | 10,00 |
| EIRP |  |  |
| EIRP density | dBW/MHz | 34,00 |
| Antenna gain loss | dBi | 3,00 |
| EIRP | dBW | 41,00 |
| G/T |  |  |
| N_f | dB | 7,00 |
| Ta | K | 290,00 |
| T0 | K | 290,00 |
| G_R | dBi | 3,00 |
| L_p | dB | 3,00 |
| G/T | dB/K | -31,62 |
| Boltzmann constant |  |  |
| k | dBW/K/Hz | -228,60 |
| Free Space Loss |  |  |
| PL_FS | dB | 159,10 |
| Atmospheric Loss |  |  |
| A_zenith | dB | 0,04 |
| PL_A | dB | 0,07 |
| Scintillation Loss |  |  |
| P_fluc | dB | 8,48 |
| PL_S | dB | 6,00 |
| Shadowing Margin |  |  |
| PL_SM | dB | 3,00 |
| Additional Margin |  |  |
| PL_AD | dB | 0,00 |
| CNR |  |  |
| CNR | dB | -0,19 |
| CIR |  |  |
| CIR (from calibration results) | dB | 9,38 |
| CINR |  |  |
| CINR | dB | -0,65 |

##### Up Link

Table 6 : Up Link budget analysis for LEO-600 in S-Band with handheld terminals

| Satellite parameters | - | set 1 |
| --- | --- | --- |
| Satellite | - | LEO |
| Altitude | km | 600,00 |
| Frequency band | - | S |
| Frequency | GHz | 2,00 |
| Elevation | deg | 30,00 |
| Polarization reuse | - | NO |
| Frequency reuse factor | - | 3 |
| System bandwidth | MHz | 30,00 |
| Bandwidth per beam | MHz | 10,00 |
| Max bandwidth per UE | MHz | 1,00 |
| EIRP |  |  |
| P | dBW | -6,9897 |
| G_T | dBi | 3 |
| L_C | dB | 0,00 |
| EIRP | dBW | -3,99 |
| G/T |  |  |
| Max G/T | dB/K | 1,1 |
| Antenna gain loss | dBi | 3 |
| L_P | dB | 3 |
| G/T | dB/K | -4,9 |
| Boltzmann constant |  |  |
| k | dBW/K/Hz | -228,60 |
| Free Space Loss |  |  |
| PL_FS | dB | 159,10 |
| Atmospheric Loss |  |  |
| A_zenith | dB | 0,04 |
| PL_A | dB | 0,07 |
| Scintillation Loss |  |  |
| P_fluc | dB | 8,48 |
| PL_S | dB | 6,00 |
| Shadowing Margin |  |  |
| PL_SM | dB | 3,00 |
| Additional Margin |  |  |
| PL_AD | dB | 0,00 |
| CNR |  |  |
| CNR | dB | -8,46 |
| CIR |  |  |
| CIR (from calibration results) | dB | 6,27 |
| CINR |  |  |
| CINR | dB | -8,60 |

One can observe that the CNR and CINR reach very low values in this worst case scenario. One should recall that the values presented here are representative of a small portion of time.

Moreover, there are alternatives to increase the CNR :

- Increase the satellite G/T

- Increase the UE’s power and/or UE’s antenna gain

- Increase the number of satellites composing the constellation such that the minimal elevation angle is increased.

The UE bandwidth cannot be reduced to a lower value since the PRACH must be spread over no less than 1080 kHz if the lowest numerology is considered.

# Conclusion

In this paper, the following proposals have been made :

- The results captured in Table 2 and Table 3 should be taken in to account by RAN WG1 to carry out the link budget analysis in GEO based NTN.

The results captured in  and  should be taken in to account by RAN WG1 to carry out the link budget analysis in LEO based NTN.

# Reference

| [1] | "TR38.811 Study on NR to support non terrestrial network" |
| --- | --- |
| [2] | “TR38.821 Solutions for NR to support non-terrestrial networks (NTN)” |
| [3] | “RAN1 Chairman’s Notes”, 3GPP TSG RAN WG1 Meeting #97 |
| [4] | ITU-R P.676 |