3GPP TSG RAN WG1 #98		                         R1-1908233

Prague, CZ, August 26th – 30th, 2019

Agenda Item:	7.2.8.3

Source:	InterDigital, Inc.

Title:	Views on Panel Activation and Deactivation

Document for:	Discussion and Decision

# Introduction

In RAN1 meeting #97 [1], the discussion on enhancements for beam operation continued, and several agreements on various aspects of beam operation were reached. Among other topics, RAN1 began discussion on panel activation/deactivation (PAD). In this contribution, we provide our perspective on general operation of PAD.

Agreement The supported feature of MAC CE based spatial relation update for aperiodic SRS per resource level is applicable to at least 3 supported usages as codebook-based UL, non-codebook-based UL, beam management. Working Assumption The supported feature of MAC CE based spatial relation update for aperiodic SRS is applicable to the usage of antenna switching per SRS resource level Working Assumption For the supported feature of simultaneous update/indication of a single spatial relation per group of PUCCH by using one MAC CE, the following configuration options for the group are supported: - At least up to two groups per BWP - FFS: Details on configuring the groups including whether to use implicit method or explicit method - For example, each corresponding to different TRP/panel, at least for multi-TRP/panel case - Another example, each corresponding to different active spatial relation at least for single TRP case - If there is no consensus to support more than two groups, only up to two groups will be supported in Rel-16 For further discussion Study the spatial relation for the PUCCH/SRS to follow a TCI-state/QCL of PDCCH/CSI-RS/SSB if spatial relation info of PUCCH/SRS is not configured in FR2 Agreement During a BFRQ procedure, UE reports only 1 beam with corresponding beam index only per SCell Agreement On BFRQ procedure for SCell - Step 1 can be carried by at least a dedicated SR-like PUCCH resource for BFR over PCell or PSCell - FFS: Details including whether or not it is precluded that MAC CE in step 2 is multiplexed in a PUSCH not triggered by step 1 - (Working Assumption) Step 2 is carried by MAC CE Above applies at least for SCell with downlink only Send an LS to RAN2 to ask their input with reference to this agreement from their specification work point of view considering their workload. The draft LS in R1-1907850 is modified and endorsed in R1-1907870. Agreement When SCell BFD RS is configured in an implicit manner, BFD RS can be transmitted in active BWP of either current CC or another CC. Agreement A UE can be configured to perform BFR for any configured SCells - The maximum number of SCells for which the UE performs BFR is a UE capability Agreement Select one of the following alternatives in RAN1#98. Companies should take into account the maturity, forward compatibility to future releases, efficient use of SRS resource usage, and extension to simultaneous transmission across multiple panels of each alternatives for completion within the intended Rel-16 schedule. If there is no consensus in RAN1#98, UL multi-panel enhancement will not be specified in Rel-16. gNB can configure/indicate panel-specific transmission for UL transmission, via - Alt.2: Introduce a UL-TCI framework in Rel-16 and support UL-TCI based signaling analogous to DL beam indication supported in Rel-15, e.g., as illustrated below. - A new panel ID may or may not be introduced. - A panel specific signaling is performed using UL-TCI state - Alt.3: a new panel-ID is introduced, which can be implicitly/explicitly applied to the transmission for a target RS resource or resource set, for PUCCH resource, for SRS resource, FFS for PRACH - A panel specific signaling is performed using the new panel-ID implicitly (e.g., by DL beam reporting enhancement) or explicitly. - If explicitly signaled, the ID can be configured in the target RS/channel or reference RS(e.g., in the DL RS resource configuration or in spatial relation info). - No new MAC CE is specified for the purpose of introducing the ID. (For example) Alt.2 UL-TCI states Valid UL-TCI state Configuration; Source (reference) RS; (target) UL RS; [qcl-Type ]. 1; SRS resource (for BM) + [panel ID]; DM-RS for PUCCH or SRS or PRACH; Spatial-relation. 2; DL RS(a CSI-RS resource or a SSB) + [panel ID]; DM-RS for PUCCH or SRS or PRACH; Spatial-relation. 3; DL RS(a CSI-RS resource or a SSB) + [panel ID]; DM-RS for PUSCH; Spatial-relation + [port(s)-indication]. 4; DL RS(a CSI-RS resource or a SSB) and SRS resource + [panel ID]; DM-RS for PUSCH; Spatial-relation + [port(s)-indication]. 5; SRS resource + [panel ID]; DM-RS for PUSCH; Spatial-relation + [port(s)-indication]. 6; UL RS(a SRS for BM) and SRS resource + [panel ID]; DM-RS for PUSCH; Spatial-relation + [port(s)-indication]. Agreement Down-select in RAN1#98 from the following options for beam management enhancements: - Alt1. Support UE to report CRI/SSBRI where the CRI/SSBRI refers to a preferred spatial relation RS for UL transmission - FFS: Whether to support SRI in addition to CRI/SSBRI - FFS on details of the reporting configuration (e.g. separate or joint reporting with existing DL beam reporting, introduction of new information from UE such as MPR) - Alt2. Support SRI field in the DCI can be used to indicate multiple SRS resources and UE’s autonomous selection of one SRS resource for PUSCH beam determination out of the multiple - Alt3: Reuse Rel-15 beam specific PHR reporting to determine beam-specific MPE impact transparently, i.e., by difference value between Pc,max (which is calculated based on P-MPR) and the required transmission power. - FFS: Enhancement on UL beam configuration for virtual PHR. - Alt4: No enhancements considering MPE issues in Rel-16 RAN1 specifications. It is up to UE implementation in conjunction to RAN4 specicfiation support. If no consensus in RAN1#98, no further discussion in RAN1. Agreement Decide in RAN1#98 whether to support updating path loss reference RSs for power control for PUSCH and SRS via MAC-CE. - FFS: Condition that the RS for PL will follows the downlink RS in spatial relation. - FFS: When the spatial relation of AP-SRS for CB/NCB UL is activated by MAC-CE, UL power control parameters for PUSCH can be activated via the MAC-CE. For further discussion Study beam indication/activation for a group of CCs Agreement - When SCell BFR is configured and RS for new beam identification is configured, the threshold for new beam identification should be always configured - If a SCell has failed, when there is no new beam with L1-RSRP higher than configured threshold for SCell BFR, for new beam information reporting, UE reports that there is no new beam identified for the SCell Agreement - When dedicated IMR is not configured, - If CMR is based on CSI-RS, when L1-SINR is configured, and interference measurement is performed using CMR with CSI-RS only with density 3 REs/RB for 1-port CSI-RS is used - Spec does not require UE to use SSB for interference measurement - Note: CSI-RS above is CSI-RS for BM - When dedicated IMR is configured, - NW can configure interference measurement for L1-SINR with either of the following options - ZP-IMR only - NZP-IMR only - (WA) ZP-IMR and NZP IMR (interference measurement is taken on both) - Maximum Number of ZP IMR is 1 - If IMR is configured based on NZP IMR only, when L1-SINR is configured, interference measurement is performed only with density 3 REs/RB CSI-RS - If IMR is configured based on ZP IMR only, when L1-SINR is configured, interference measurement is performed using ZP IMR - FFS: interference measurement is performed using CMR additionally - Support of L1-SINR is optional - FFS: Support of NZP IMR and ZP IMR are separate UE capabilities - Note: CSI-RS above is CSI-RS for BM

# Background

A UE can be equipped with several sets of antenna panels where each set may have multiple antenna elements. As shown in  Figure 1, each panel may be placed on a different side of a UE allowing better reception and transmission by increasing diversity and avoiding potential blockage. While enhanced transmission through employing multiple panels is beneficial, it is not always necessary to have all panels activated.

In general, there are a couple of motivations for panel activation/deactivation (PAD). The main expected benefits of PAD are fast selection of the best beam/panel for uplink transmission, and power saving. Moreover, PAD is also a relevant solution for the issues related to Maximum Permissible Emission (MPE). In brief, P-MPR is introduced to address MPE requirements. However, in a multi-panel UE, when the power assigned to a panel is impacted by P-MPR, the initially selected panel may become a less appropriate choice for uplink transmission. As such, fast panel switching through PAD can offer a better transmission through a panel that is not impacted by P-MPR.

Figure 1 Multi-panel UE

Multi-panel operation for uplink transmission has been discussed in NR, and three categories of multi-panel UE (MPUE) are identified [2],

- MPUE-Assumption1: Multiple panels are implemented on a UE and only one panel can be activated at a time, with panel switching/activation delay of [X] ms

- MPUE-Assumption2: Multiple panels are implemented on a UE and multiple panels can be activated at a time and one or more panels can be used for transmission

- MPUE-Assumption3: Multiple panels are implemented on a UE and multiple panels can be activated at a time but only one panel can be used for transmission

Based on [3], the focus of the work in Rel-16 is on MPUE A3, however design decisions should be forward compatible with MPUE A2 requirements. MPUE A2 that deals with simultaneous uplink transmission is to be studied in Rel-17.

a					b

Figure 2 Multi-panel UE, a) All panels are active b) Only 4 panels are active and only 1 panel transmits

# Overall Operation of PAD

According to the definition, under MPUE A3 assumption, multiple panels are implemented on a UE and multiple panels can be activated at a time but only one panel can be used for transmission. However, this does not mean that a UE must always activate multi-panels simultaneously. Figure 2 shows a multi-panel UE architecture. In configuration (a), all panels are assumed activated and can be used for transmission and reception. However, in configuration (b), according to MPUE A3, only 4 panels out of the 16 panels are activated, and only one panel is selected for transmission. For the brevity, the panel selected for transmission can be referred as the hot panel.

Figure 2b shows the basic panel structure for PAD. In operation of PAD three different groups of panels may be considered, namely: deactivated panels, activated panels and hot panels.

  - Deactivated panels are the panels that are turned off,

  - Activated panels are the panels that are turned on and may be used for reception,

  - Hot panels are the activated panels that may be used for transmission.

From the switching decision perspective, UE panel activation/deactivation can be implemented in different ways, where in some cases, PAD operation may have some impacts on gNB transmission/reception as well. More specifically, two main category of solutions may be considered for panel selection, namely: gNB-driven and UE-driven.

Figure 3 gNB-driven PAD

gNB-Driven

Figure 3 shows an example of a gNB-driven panel selection process. In a gNB-driven PAD process, gNB manages the operation, i.e., activation/deactivation of UE panels. The main benefit of the approach is that there will never be a mismatch in terms of transmission/reception characteristics between gNB and a UE, however the speed of beam/panel selection reduces significantly.

In such operation, a gNB wakes panels up periodically for beam or CSI measurement to select the best subset of panels. Also, a UE could request a panel selection process based on an event or a measurement. Then, the UE reports the set of desired panels from the set of activated panels. The CSI-RS configuration may also include CSI resources for deactivated panels. These CSI-RS resources can be considered dormant when a panel is deactivated. However, based on a measurement performed on the activated panels, the UE can begin monitoring the CSI-RS resources by temporarily activating all or a subset of previously deactivated panels for measurement.

For panel selection and proper linkage of QCL properties, in operation of PAD, CSI-RS process may need to be tied through an ID to each panel. However, it is not clear if the existing CSI-RS framework can support such distinction, and some enhancement may be needed to include panel-ID into CSI-RS framework.

Proposal 1: RAN1 considers CSI-RS enhancement for panel activation/deactivation.

Figure 4 UE-driven PAD

UE-Driven

UE-driven panel switching can be conceived as a purely event-driven (UE rotation, MPE) or condition-driven (power saving) mechanism where UE selects a new panel/beam entirely on its own. Figure 4 shows an example of UE-driven PAD where a UE autonomously selects the best panel. In the simplest way, in a UE-driven PAD operation, the UE decides a specific panel to be used for uplink transmission.

A UE can quickly and independently decide to turn on/off a panel for transmission to save power or select the best beam. Such mechanism expects some flexibility in the spatial relation where a UE can update its TX beam by itself without gNB updating the spatial relations. As such, a UE sends its panel status as a report to gNB. The indication of a panel switch to the gNB can be done prior to the actual switch. For example, the UE can signal to gNB that it is going to activate/deactivate a panel. Indication of panel status to gNB can be done through SRS transmission where in the configured SRS resource set, each SRS resource is assigned to specific subset of panels. Therefore, by selecting a new hot panel, the UE can transmit an SRS to the network.

Proposal 2: RAN1 considers SRS transmission for indication of a selected hot panel.

# Conclusion

In this contribution, we share our views on the operation of panel activation/deactivation. Based on the presented discussion, following observations and proposals are made;

Proposal 1: RAN1 considers CSI-RS enhancement for panel activation/deactivation.

Proposal 2: RAN1 considers SRS transmission for indication of a selected hot panel.

# References

Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #97, Reno, Nevada, May 2019

Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #96, Athens, Greece, February 2019

Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #96b, Xi’an, China, April 2019