3GPP TSG RAN WG1 Meeting #98                                                                        R1-1908283

Prague, CZ, August 26th – 30th, 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion of Resource Allocation for Sidelink - Mode 1

Agenda item:	7.2.4.2.1

Document for:	Discussion and Decision

# Introduction

In the description of the work item [1] for 5G V2X with NR sidelink, the following objectives were specified for Mode 1 resource allocation for NR sidelink:

1. NR sidelink: Specify NR sidelink solutions necessary to support sidelink unicast, sidelink groupcast, and sidelink broadcast for V2X services, considering in-network coverage, out-of-network coverage, and partial network coverage.

- Support of sidelink signals, channels, bandwidth part, and resource pools [RAN1, RAN2]

- Resource allocation [RAN1, RAN2]

  - Mode 1

    - NR sidelink scheduling by NR Uu and LTE Uu as per the study outcome

  - Mode 2

    - Sensing and resource selection procedures based on sidelink pre-configuration and configuration by NR Uu and LTE Uu as per the study outcome

  - Support for simultaneous configuration of Mode 1 and Mode 2 for a UE

    - Transmitter UE operation in this configuration is to be discussed after the design of mode 1 only and mode 2 only.

    - Receiver UE can receive the transmissions without knowing the resource allocation mode used by the transmitter UE.

  - UE relaying resource pool configuration or resource configuration is not supported in this work in Rel-16.

At RAN1#96b, the following agreements were reached on mode 1 resource allocation [2]:

Agreements:

	A dynamic grant provides resources for one or multiple sidelink transmissions of a single TB.

	A configured grant (type-1, type-2) provides a set of resources in a periodic manner for multiple sidelink transmissions.

o	UE decides which TB to transmit in each of the occasions indicated by a given configured grant.

o	FFS: whether different transmissions of a TB can take place across multiple configured grants.

o	Other restrictions on what can be transmitted in a given configured grant (e.g., based on QoS, destination UE, etc.) are up to RAN2.

At RAN1#97, the following agreements were reached on mode 1 resource allocation [3]:

Agreements:

- Sidelink HARQ ACK/NACK report from transmitter UE to gNB is supported with details FFS.

Note: this reverts the following agreement from RAN1#96:

  - Sidelink HARQ ACK/NACK report from UE to gNB is not supported in Rel-16.

- SR/BSR report to gNB for the purpose of requesting resources for HARQ retransmission is not supported.

Agreements:

- NR sidelink does not support performing different transmissions of a TB using different configured grants.

Agreements:

- For mode 1:

  - A dynamic grant by the gNB provides resources for transmission of PSCCH and PSSCH.

In this contribution, we provide our views on mode 1 of sidelink resource allocation for NR V2X.

# Resource allocation for mode 1

In TR 37.885 “Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR” [4], multiple traffic models were specified for evaluation.  For NR V2X, diverse traffic needs to be supported, including: periodic/aperiodic traffic, different magnitudes of packet sizes: from hundreds of bytes up to tens of thousands of bytes, and fixed/varying packet sizes. Also diverse communication types need to be supported, including broadcast, groupcast and unicast.

Hence resource allocation modes specified for LTE V2X should be enhanced to adapt to much more diverse traffic requirements encountered in NR V2X.

NR V2X mode 1 (base station schedules sidelink resource) proposed in RAN1#94 should use LTE V2X mode 3 as basis and inherit all functionalities of LTE V2X mode 3. As agreed at the study item stage, both dynamic and SPS (configured grant) resource allocation will be supported for NR V2X mode 1. For configured grant, both type 1 and type 2 configured grants are supported for NR SL. While dynamic resource allocation is suitable for aperiodic data traffic, configured grant is suitable for periodic data traffic. These options provide flexibility for gNB scheduled resource allocation to suit diverse data traffic and use cases.

## UE assistance information

For NR V2X mode 1, sidelink resources are scheduled at base station, which is more or less an implementation issue and doesn’t require standardization. However, how well mode 1 resource allocation can provide depends on relevant information the base station gathers. Considering diverse data traffic to be supported and much more stringent performance requirements in terms of reliability and latency, more information is needed to be transmitted from vehicular UEs to the base station to guarantee performance and further improve resource efficiency. On the other hand, signaling overhead cost is associated at Uu interface. Hence this part requires careful standardization to balance performance and signaling overhead.

As agreed at the study item stage, NR supports that UE reports assistance information to the gNB, consisting of at least: UE-related geographic information (e.g., position), and V2X traffic-related information (at least for periodic traffic) including traffic periodicity, timing offset, and message size.

One open question with this existing agreement is what exactly is included in UE-related geographic information. UE position is listed as one example, but the exact content is still open. In our view it is beneficial to report UE speed and direction in addition to UE position; this will assist the gNB in predicting the future trajectory of the UE and allow better resource scheduling.

Proposal 1: UE-related geographical information in UE assistance information includes position, speed and direction.

For NR V2X, diverse transmission types need to be supported explicitly at physical layer, including broadcast, groupcast and unicast. Different transmission types are of different properties. The knowledge of transmission type at physical layer can assist gNB in scheduling sidelink resources. Hence the information of transmission type should be included in UE assistance information sent from the transmitter UE.

For NR V2X, even for periodic data traffic, there are different magnitudes of packet sizes for different traffic models [4]. For example, for periodic traffic Model 2 (medium traffic intensity), packet size is 1200 bytes with probability of 0.2 and 800 bytes with probability of 0.8. For periodic traffic Model 3 (high traffic intensity), packet size is uniformly random in the range between 30000 bytes and 60000 bytes with the quantization step of 10000 bytes. Hence the information of packet size range instead of only one packet size value should be included in UE assistance information.

Proposal 2: UE assistance information sent to gNB to assist mode 1 resource allocation additionally includes the following: transmission type (i.e. broadcast/groupcast/unicast), packet size range (for periodic traffic).

## UE-autonomous control of TX parameters

One fundamental question in gNB-scheduled mode (mode 1) is if the values of TX parameters such as MCS, precoder matrix etc are selected by the gNB or if the UE has some autonomy in selecting these parameter values from value sets configured by the network.

Since reporting sidelink CSI to the gNB is not within the scope of this WI it seems clear that the gNB does not have the required information for optimal selection of TX parameter values. Hence the UE must be given autonomy in selecting TX parameter values; however, the network shall be able to configure the set of values from which the UE can select.

Proposal 3: In mode 1, the UE autonomously selects the values of the TX parameters (such as MCS) from a set of allowed values configured by the gNB.

## Configured grants for sidelink

For configured grant, both type 1 and type 2 configured grants are supported for NR SL. Configured grant is particularly suitable for periodic data traffic. But NR V2X is also required to support periodic data traffic with large varying packet sizes, which calls for further enhancements. For this type of periodic data traffic with stringent QoS requirement (e.g. very low latency), the gNB can assign periodic sidelink resource (configured grant) to accommodate maximal packet size.  Then for each arrived packet, the UE can directly release unused sidelink resource at sidelink interface, i.e. the UE transmits a PSCCH (SL SCI) to release unused sidelink resource to other UEs (in mode 2 or in both modes 1/2) promptly. It provides best performance in terms of latency for sidelink transmission, and at the same time increases efficiency of sidelink resource usage.

Alternatively, the gNB can configure periodic SR (scheduling request) resources at Uu interface. The gNB configures a mapping between PUCCH information bits and the amount of released SL resource. To save signaling, the mapping can also be implicitly derived from the PUCCH information bit number and the packet size range. For an arrived packet with smaller size, the UE sends a PUCCH at configured SR resource to inform the gNB the SL resource to be released. Then the gNB can reschedule the released SL resource for other use.

Proposal 4: Consider allowing a UE releasing unused sidelink resource (periodically configured) directly at sidelink interface or at Uu interface by using periodic SR resources.

# Conclusions

In this contribution, we provide our views on mode 1 sidelink resource allocation for NR V2X, summarized in the following proposals:

Proposal 1: UE-related geographical information in UE assistance information includes position, speed and direction.

Proposal 2: UE assistance information sent to gNB to assist mode 1 resource allocation additionally includes the following: transmission type (i.e. broadcast/groupcast/unicast), packet size range (for periodic traffic).

Proposal 3: In mode 1, the UE autonomously selects the values of the TX parameters (such as MCS) from a set of allowed values configured by the gNB.

Proposal 4: Consider allowing a UE releasing unused sidelink resource (periodically configured) directly at sidelink interface or at Uu interface by using periodic SR resources.

# References

- RP-190766, “New WID on 5G V2X with NR sidelink”, 3GPP RAN #83.

- RAN1#96bis chairman notes

- RAN1#97 chairman notes

- 3GPP TR 37.885: “Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR”.