3GPP TSG RAN WG1 Meeting #98                                                                         R1-1908315

Prague, CZ, August 26th – 30th, 2019

Source:				ZTE, Sanechips

Title:						Discussion on configurations for SSB Tx and Rx

Agenda item:		7.2.3.1

Document for:  Discussion and Decision

# Introduction

RAN1 #96bis[1], RAN1 #97[2] and RAN1 #AH1901 made the following agreements on SSB for inter-IAB-node discovery and measurements:

Agreements (#96bis):

- For IAB node discovery and measurement, the maximum number of SMTC windows that can be configured for an IAB node is 4.

Agreements (#97):

For IAB node discovery and measurement, the maximum number of STCs that can be configured for an IAB node DU per cell at one frequency location is 4.

- Note: this number does not include the cell-defining SSBs for initial access

Agreements(#97):

The configurable values of the parameters in the SSB reception configurations including SMTC for IAB node discovery and measurement are provided in the following

- SMTC window periodicity:

  - 5ms, 10ms, 20ms, 40ms, 80ms, 160ms, 320ms, 640ms, 1280ms

- SMTC window timing offset:

  - [0, ..., (number of subframes within SMTC window periodicity) – 1]

- SMTC window duration:

  - [1, …, 5] (subframes)

    - FFS larger than 5

- List of physical cell IDs to be measured

  - [cell ID 0, …, cell ID M-1 ]

    - FFS value of M

- SSB-ToMeasure

  - Same as Rel-15

Agreements (#AH1901):

- SSB transmission configuration information (i.e. the necessary information for the DU to determine when to transmit SSB) and SSB reception configuration information (i.e. the necessary information for the MT to determine when to search for SSB) for IAB inter-node measurements are determined in a centralized manner, i.e. not locally by the IAB node.

In this contribution, we discuss some remaining issues for SMTC and STC.

# SMTC

For SSB measurement configuration, there are still two remaining issues to be solved:

- FFS the value of M: list of physical cell IDs (PCIs) to be measured [cell ID 0, …, cell ID M-1] (i.e. the maximum number of PCIs per SMTC)

- Whether SMTC duration can be larger than 5 ms or not.

In Rel-15, maximum number of PCIs per SMTC is 64. In IAB network, there seems no strong motivation to increase the maximum number of PCIs per SMTC. In addition, measuring too many cells in one SMTC occasion will degrade measurement performance due to inter-cell interference. Hence, Rel-15 maximum number of PCIs per SMTC can be reused for SMTCs of IAB nodes.

Proposal 1: Rel-15 maximum number of PCIs per SMTC can be reused for SMTCs of IAB nodes, i.e. M=64.

In the last meeting, it was agreed that the SMTC duration can take values from 1ms to 5ms with step size of 1ms, and whether the maximum value of SMTC duration can be larger than 5ms should be for further study. Larger SMTC duration may reduce the number of SMTC windows configured for an IAB node. However, it may also bring some problems, e.g. longer duration of measurements may impact on flexibility of resource scheduling and data transmission because an IAB node cannot transmit other data within SMTC duration due to half-duplex constraint. Hence, there is no need to use a larger SMTC duration.

Proposal 2:  The maximum SMTC duration is 5 ms.

# Collision between SMTC and STC

For an IAB node, a SMTC window on parent backhaul link may overlap with SSB transmission window on child link.

For the case where SSB transmission is for UE cell search and measurement, according to RAN1 agreement, the IAB node should not mute this SSB transmission when doing inter-IAB cell search in stage 2 in order to maintain UE’s backward compatibility. In other words, the SSB transmission takes priority in this case.

For the case where STC is for other IAB’s stage-2 measurement but not for UE to receive, SSB transmission in the STC can be muted according to previous agreements. Therefore, we recommend muting SSB Tx when a SMTC and a STC overlap in time domain for an IAB node, i.e. SMTC takes higher priority.

As long as SMTC can take higher priority over STC, there could be a risk that SSBs in STC may never get chance to be transmitted. This risk can happen because both SMTC and STC are configured in a periodic way and the collision between the two, if any, is also periodic. Then if STC has longer periodicity than SMTC and overlaps with SMTC in time, STC could be always at lower priority. This is certainly not desirable.

Proposal 3: When a SSB transmission and a SMTC overlap in time,

- If the SSB transmission is for access UE, the SSB transmission takes higher priority.

- If the SSB transmission belongs to STC for other IAB’s stage-2 measurement but not for UE to receive, the SMTC takes higher priority.

  - If SMTC of higher priority blocks all SSB transmission opportunities in a STC for an IAB node, the corresponding SSB Tx/Rx configuration is regarded as incorrect network configuration.

# Conclusions

This paper concludes that:

Proposal 1: Rel-15 maximum number of PCIs per SMTC can be reused for SMTCs of IAB nodes, i.e. M=64.

Proposal 2:  The maximum SMTC duration is 5 ms.

Proposal 3: When a SSB transmission and a SMTC overlap in time,

- If the SSB transmission is for access UE, the SSB transmission takes higher priority.

- If the SSB transmission belongs to STC for other IAB’s stage-2 measurement but not for UE to receive, the SMTC takes higher priority.

  - If SMTC of higher priority blocks all SSB transmission opportunities in a STC for an IAB node, the corresponding SSB Tx/Rx configuration is regarded as incorrect network configuration.

# References

- Chairman's Notes RAN1 #96bis, April, 2019.

Chairman's Notes RAN1 #97, May, 2019.

Chairman's Notes RAN1 AdHoc 1901, January, 2019.