3GPP TSG RAN WG1 #98		R1-1908316

Prague, CZ, August 26th - 30th, 2019

Title: 	Resource multiplexing between backhaul and access links

Source: 	ZTE, Sanechips

Agenda item:	7.2.3.3

Document for:	Discussion and decision

# Introduction

This contribution provides our views on resource multiplexing between backhaul and access links encompassing the agreements reached before.

# DU resource configuration

Several alternatives for DU resource configuration of H/S/NA were recognized in RAN1 #97 meeting [1]:

Agreements:

For the semi-static DU resource configuration, the following is supported:

- Indication of H/S/NA for the DU resource configuration is based on one of the following alternatives:

  - Alt. 1: H/S/NA is additionally explicitly indicated per-resource type (D/U/F) in each slot

    - To handle potential misalignment in time of the configured DU and MT resources when determining the validity of H/S/NA at the DU one of the following sub-alternatives need to be supported:

      - 1a: H/S/NA is applied relative to the DU resource configuration (D/U/F) slot timing without considering the MT resource configuration or timing.

        - FFS: definition of additional restrictions on the usage of the semi-static configuration (e.g. guard symbols) based on deployment scenario or DL/UL switching times within an IAB node, etc.

        - FFS: How the CU can get information about the required guard symbols for a given DU configuration if needed

      - 1b: H/S/NA is applied relative to the MT resource configuration (D/U/F) slot timing.

        - FFS: Whether and/or how the CU will know the actual H/S/NA resources at the child DU

        - FFS whether S is explicitly indicated or not

      - 1c: H is applied relative to the DU resource configuration (D/U/F) slot timing. S is not explicitly indicated, but implicitly determined by the DU based on whether the corresponding MT configuration indicates the MT resources is F (DU-S). The remaining resources are assumed to be NA at the child DU.

        - FFS: Whether and/or how the CU will know the actual S/NA resources at the child DU

  - Alt. 2: NA is explicitly indicated as a resource type in each slot for both the DU and MT configuration. H/S is not explicitly indicated, but implicitly determined by the DU based on the corresponding MT configuration

    - FFS: how to handle the case where there is not a 1-1 mapping of DUs and MTs in the child IAB node

- Details of the F1-AP signaling design is up to RAN3

Within the case-1 timing, the DU DL TX and MT DL RX are not timing aligned due to propagation delay. H/S/NA for DU resource can be configured with MT timing or DU timing. Figure 1 depicts these two timing of H/S configuration.

Figure 1 H/S for DU resource configured with different timing

If DU resource of H/S/NA are timing aligned with MT, due to timing gap between DU and MT, some symbols may overlap with two different types as shown in Figure 2, the attributes of these symbols are ambiguous.

Figure 2 Symbol ambiguity on H/S/NA attributes

If H/S/NA of DU resource type is determined with DU timing, the above issues are avoided.

- Proposal 1：H/S/NA is configured with DU timing.

It should be also noted that the alternatives 1b, 1c and 2 have following issues:

- It was agreed in RAN1 AH1901 meeting [2] that

IAB-node/IAB-donor DU resources are provided by a semi-static configuration which is provided separately from the MT resource indication.

But Alt 1c and Alt 2 have the DU configuration partially based on MT configuration, which is against above agreement and also means the CU partially loses control upon DU configuration.

- Alt 1c and Alt 2 make the DU resource types partially depending on MT resources, and therefore are not applicable to IAB-donor nodes which have no MT component. If Alt 1c or Alt 2 were adopted, RAN1 needs to find other solutions for IAB-donor DU anyway.

- Alt 2 adds NA resource attribute to MT resource type, which against numerous long-time lasting RAN1 agreements and design logic that assumes the MT resource types reuse the Rel-15 resource types of D/F/U only.

- Proposal 2：Alt 1a is preferred for indication of H/S/NA for the DU resource.

# Cell-specific signals/channels

Some signals/channels were identified as cell-specific signals/channels, when such cell-specific signals/channels are configured on DU NA or not Indicated Available Soft resource, the resource is treated as if it were a Hard DU resource.

Agreements[1]:

If a DU NA or Soft resource is configured with cell-specific signals/channels, the resource is treated as if it were a Hard DU resource (Alt. 2 from RAN1#96bis).

  - The list of cell-specific signals/channels includes:

- resources for SSB transmission at DU, including both CD-SSB and non-CD-SSB;

- configured RACH occasions for receiving at the DU

- periodic CSI-RS transmission at the DU

- scheduled resource for receiving SR at DU

- The parent does not need to be aware of the cell-specific signals/channel configurations of the child DU

RMSI transmission should be guaranteed since it is needed for UE/MT access on the child link.

From Table 7-1[3] and Table 7-2[4] in Annex, we can find that SSB/CORESET0/RMSI are in the same slot for SSB/CORESET 0 multiplexing pattern 3. When SSB is transferred on NA resource, it is impossible to avoid NA or Soft not indicated Available resources for RMSI. For other SSB/CORESET0 multiplexing patterns, the scheduling flexibility for RMSI is not sufficient to guarantee RMSI to be transmitted always on Hard or Soft Indicated Available resources. Therefore RMSI should be included in the cell-specific signals/channels list.

- Proposal 3：RMSI should be included in the cell-specific signals/channels list, for which the configured resource is treated as if it were a Hard DU resource even when the resource is configured as DU NA or Soft INA.

At RAN1 #96b meeting, two types of resource were involved in cell-specific signals/channels:

- DU Hard or Soft Indicated Available resource which collides with cell-specific signals/channels at MT

- DU NA or Soft not Indicated Available resources with cell-specific signals/channels configured at DU

The cell-specific signals/channels at RAN1 #96b meeting were treated as the same, while RAN1 #97 meeting only focused on the second issue.

- Proposal 4：When the following cell-specific signals/channels at MT collide with DU Hard or Soft Indicated Available resource, the decision on whether to give priority to the DU or to the MT for the use of the resource is left to the IAB node implementation.

- resources for SSB reception at MT, including both CD-SSB and non-CD-SSB;

- resources for RMSI reception at the MT;

- configured RACH occasions for transmission at the MT;

- periodic CSI-RS reception at the MT;

- scheduled resource for transmission SR at the MT.

Note: The proposal 5 does not apply to the case where the SSB transmission at DU collides with SSB reception at MT, which is handled in a separate agenda item.

# Soft resource indication

Some indication schemes for soft resource were identified as below.

Conclusion:

The following alternatives are considered (to be down-selected in RAN1#98) for the explicit indication of the availability of soft resources:

- Alt 1) Indicate which MT resources are “IA” for the child DU (DU-IA)

- Alt 2a) Indicate DU-IA and MT resource type (MT-D/MT-U/MT-F)

- Alt 2b) Indicate DU-IA and DU resource type (DU-D/DU-U/DU-F)

- Alt 3) Jointly or separately indicate DU-IA, the DU resource type, and/or MT resource type

- FFS: monitoring occasions for the explicit indication at the MT

- FFS: whether the processing time for applying an explicit indication at the child DU is defined or left to implementation.

MT resource type configuration can be implemented by (extended) Rel-15 semi-static or dynamic signaling schemes and there is no strong motivation to indicate MT resource type again through explicit indication of DU soft resources.

Whether DU H/S/NA type is defined based on DU timing or MT timing is still under discussion. We could first wait for the conclusion of which timing is used for DU H/S/NA type determination, then determine whether MT resources or DU resources are "IA" for the child DU.

Before receiving explicit indication, child DU already knows the 7 types of DU resource. Whether explicit indication of availability can further indicate or override DU resource type should be studied.

- Proposal 5：Explicit indication of the availability of soft resources should be decoupled from MT configuration. Whether explicit indication of availability can further indicate or override DU resource type should be studied.

# Conclusion

Based on the discussion, we have the following observation and proposals:

- Proposal 1：H/S/NA is configured with DU timing.

- Proposal 2：Alt 1a is preferred for indication of H/S/NA for the DU resource.

- Proposal 3：RMSI should be included in the cell-specific signals/channels list, for which the configured resource is treated as if it were a Hard DU resource even when the resource is configured as DU NA or Soft INA.

- Proposal 4：When the following cell-specific signals/channels at MT collide with DU Hard or Soft Indicated Available resource, the decision on whether to give priority to the DU or to the MT for the use of the resource is left to the IAB node implementation.

- resources for SSB reception at MT, including both CD-SSB and non-CD-SSB;

- resources for RMSI reception at the MT;

- configured RACH occasions for transmission at the MT;

- periodic CSI-RS reception at the MT;

- scheduled resource for transmission SR at the MT.

- Proposal 5：Explicit indication of the availability of soft resources should be decoupled from MT configuration. Whether explicit indication of availability can further indicate or override DU resource type should be studied.

# References

Chairman's Notes RAN1 97 final

Chairman's Notes RAN1 AdHoc 1901 final

38213-f60

38214-f60

# Annex

Table 7-1[3] PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 3 and {SS/PBCH block, PDCCH} SCS {120, 120} kHz

| Index | PDCCH monitoring occasions (SFN and slot number) | First symbol index (k = 0, 1, … 15) |
| --- | --- | --- |
| 0 |  | 4, 8, 2, 6 in , , , |
| 1 | Reserved |  |
| 2 | Reserved |  |
| 3 | Reserved |  |
| 4 | Reserved |  |
| 5 | Reserved |  |
| 6 | Reserved |  |
| 7 | Reserved |  |
| 8 | Reserved |  |
| 9 | Reserved |  |
| 10 | Reserved |  |
| 11 | Reserved |  |
| 12 | Reserved |  |
| 13 | Reserved |  |
| 14 | Reserved |  |
| 15 | Reserved |  |

Table 7-2[4] : Default PDSCH time domain resource allocation C

| Row index | dmrs-TypeA-Position | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- | --- |
| 1 (Note 1) | 2,3 | Type B | 0 | 2 | 2 |
| 2 | 2,3 | Type B | 0 | 4 | 2 |
| 3 | 2,3 | Type B | 0 | 6 | 2 |
| 4 | 2,3 | Type B | 0 | 8 | 2 |
| 5 | 2,3 | Type B | 0 | 10 | 2 |
| 6 | Reserved |  |  |  |  |
| 7 | Reserved |  |  |  |  |
| 8 | 2,3 | Type B | 0 | 2 | 4 |
| 9 | 2,3 | Type B | 0 | 4 | 4 |
| 10 | 2,3 | Type B | 0 | 6 | 4 |
| 11 | 2,3 | Type B | 0 | 8 | 4 |
| 12 | 2,3 | Type B | 0 | 10 | 4 |
| 13 (Note 1) | 2,3 | Type B | 0 | 2 | 7 |
| 14 (Note 1) | 2 | Type A | 0 | 2 | 12 |
|  | 3 | Type A | 0 | 3 | 11 |
| 15 (Note 1) | 2,3 | Type A | 0 | 0 | 6 |
| 16 (Note 1) | 2,3 | Type A | 0 | 2 | 6 |
| Note 1: The UE may assume that this PDSCH resource allocation is not used, if the PDSCH was scheduled with SI-RNTI in PDCCH Type0 common search space |  |  |  |  |  |