3GPP TSG-RAN WG1 Meeting #98	 R1-1908343

Prague, CZ, 26th – 30th August 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	2-step RACH evaluation results

Agenda Item:	7.1.2.3

Document for:	Discussion

# Introduction

In RAN#82 a new work item on “2-step RACH for NR” was agreed [1]. In this contribution, we present preliminary evaluation results based on a single radio link focusing on the simulation parameters agreed in [2] during the email discussion. At first, we present simulation results for preamble detection of MsgA and then, BLER performance results for PUSCH of MsgA are analyzed for different payload sizes. We compared the performance results using either different DMRS ports or different DMRS sequences to handle potential PUSCH resource unit (PRU) collision. Then, we focus our study on whether to have separate preamble power control parameters for 2-step and 4-step RACH or have shared power control parameters based on the results presented.

# Link level evaluation results

## Preamble detection

Simulation results for miss-detection probability as a function of received SNR are provided in Figure 1 for the preamble format A1 and format 0 in 200m ISD scenario. For the evaluations, the target false-alarm rate is fixed (Pf=1%) as illustrated in Figure 2 as example. Based on the simulation assumptions provided in Table A.1, the time offset is within 0 and round-trip time. In that case, the timing estimation accuracy is within +/-0.5 us for all cases and Figure 3 illustrates the evaluation results of the time offset estimation used in the coming evaluations for PUSCH of MsgA.

(a) Format A1, 30 kHz 		 (b) Format 0, 1.25KHz

Figure 1. Miss-detection rate of preamble detection for false alarm rate 1%

Figure 2. False alarm rate.                          Format A1, 30kHz

Figure 3. Standard deviation of TA.           Format A1, 30kHz

Observation 1. Increasing the number of transmitting UEs for a fixed false alarm rate leads to an increase of the miss- detection rate of preamble detection.

The link budget analysis for the preamble formats 0 and A1 is given in Table 1 as example for single UE case, where the SNR value is given for a target BLER of 0.1.

Table 1. Link budget calculation of preamble formats A1 and 0

|  | Preamble A1 | Preamble 0 |
| --- | --- | --- |
| (1) Tx Power (dBm) | 23 | 23 |
| (2) Thermal noise density (dBm/Hz) | -174 | -174 |
| (3) eNB receiver noise figure (dB) | 5 | 5 |
| (4) Interference margin (dB) | 0 | 0 |
| (5) Occupied channel bandwidth (Hz) | 4.32e6 | 1.08e6 |
| (6) Effective noise power = (2)+(3)+(4)+10log((5)) (dBm) | -102.79 | -108.66 |
| (7) Required SINR (dB) | -5.5 | -7.1 |
| (8) Receiver sensitivity = (6)+(7) (dBm) | -108.29 | -115.79 |
| (9) Receiver processing gain | 0 | 0 |
| (10) MCL = (1)-(8)+(9) (dB) | 131.29 | 138.79 |

## Payload size evaluations using different DMRS ports and different DMRS sequences

The structure of MsgA consists of a PRACH preamble and PUSCH resources for data transmission. Related to the PUSCH resource unit (PRU) of MsgA, we presented in our previous contribution [3] an analysis of the trade-off between MsgA PUSCH resource reservation and the probability of collision and latency of the 2-step RACH procedure.  In this case, multiple transmissions from different UEs may collide in the same PRU.  In order to decrease the collision probability between UEs, different DMRS ports or DMRS sequences (scramblings IDs) can be used. Hence, we compare in the coming evaluations the performance obtained using both approaches to define the maximum coupling loss (MCL) based on [1] as shown in Table 2.

Table 2. Link budget calculation of MsgA for different payload sizes

| (1) Tx Power (dBm) | 23 |
| --- | --- |
| (2) Thermal noise density (dBm/Hz) | -174 |
| (3) eNB receiver noise figure (dB) | 5 |
| (4) Interference margin (dB) | 0 |
| (5) Occupied channel bandwidth (Hz) | 30kHz * 12 * #PRBs |
| (6) Effective noise power = (2)+(3)+(4)+10log((5)) (dBm) |  |
| (7) Required SINR (dB) |  |
| (8) Receiver sensitivity = (6)+(7) (dBm) |  |
| (9) Receiver processing gain | 0 |
| (10) MCL = (1)-(8)+(9) (dB) |  |

Different payload sizes (72, 198, 408 and 1032) are used in the evaluations according to [1], where we assume 14 OFDM symbols for PUSCH with 30 kHz subcarrier spacing.  A TDL-C-300ns channel model is used with 4 Rx antennas and the number of PRBs is assumed to be 1,2,3, 6 or 12. The rest of the simulation assumptions are defined in Appendix A.

BLER performance results are shown in Figure 4 in the case of single UE and 2 or 3 colliding UEs using different DMRS ports/sequences for all payload sizes evaluated, respectively. As increasing the payload size, the number of PRBs should be increased to achieve the target BLER from 1, 2 or 3 PRBs for payload size of 72 bits up to 12 PRBs for payload size of 1032 bits. The MCL calculation based on the required SNR at a target value of 1% BLER is shown in Table 3 for each payload size. Comparing the MCL obtained when different DMRS ports or DMRS sequences are used, we can observe a difference of 0.5 up to 1 dB for the benefit of using different DMRS ports in the case of payload size 72.

- Payload size of 72

- Payload size of 198

- Payload size of 408

- Payload size of 1032

Figure 4.BLER performance for 1, 2 and 3 colliding UEs using different DMRS ports or DMRS sequences and payload sizes of 72, 198, 408 and 1032

Table 3. Link budget calculation of MsgA for 1, 2 and 3 colliding UEs using different DMRS antenna ports or DMRS sequences and payload sizes of 72, 198, 408 and 1032

- Payload size of 72

| 1 UEs | 1 PRB |  | 2 PRB |  | 3 PRB |  |
| --- | --- | --- | --- | --- | --- | --- |
| Required SINR (dB) | 2.6 |  | -1.4 |  | -3.0 |  |
| MCL (dB) | 133.8 |  | 134.8 |  | 134.6 |  |
| 2 UEs | 1 PRB |  | 2 PRB |  | 3 PRB |  |
|  | DMRS Ports | DMRS Sequences | DMRS Ports | DMRS Sequences | DMRS Ports | DMRS Sequences |
| Required SINR (dB) | 5.85 | 6.95 | 0.1 | 0.6 | -2.06 | -1.4 |
| MCL (dB) | 130.58 | 129,48 | 133,32 | 132,82 | 133,72 | 133,06 |
| 3 UEs | 1 PRB |  | 2 PRB |  | 3 PRB |  |
|  | DMRS Ports | DMRS Sequences | DMRS Ports | DMRS Sequences | DMRS Ports | DMRS Sequences |
| Required SINR (dB) | 8.8 | - | 1.5 | 3.6 | -0.3 | 1.5 |
| MCL (dB) | 127.63 | - | 131.92 | 129.82 | 131.96 | 130.16 |

- Payload size of 198

| 1 UEs | 2 PRB |  | 3 PRB |  | 6 PRB |  |
| --- | --- | --- | --- | --- | --- | --- |
| Required SINR (dB) | 2.3 |  | 0.85 |  | -3.15 |  |
| MCL (dB) | 131.12 |  | 130.82 |  | 131.80 |  |
| 2 UEs | 2 PRB |  | 3 PRB |  | 6 PRB |  |
|  | DMRS Ports | DMRS Sequences | DMRS Ports | DMRS Sequences | DMRS Ports | DMRS Sequences |
| Required SINR (dB) | 5.25 | - | 2.7 | 3.3 | -2.75 | -2.45 |
| MCL (dB) | 128.17 | - | 128.96 | 128.36 | 131.40 | 131.10 |
| 3 UEs | 2 PRB |  | 3 PRB |  | 6 PRB |  |
|  | DMRS Ports | DMRS Sequences | DMRS Ports | DMRS Sequences | DMRS Ports | DMRS Sequences |
| Required SINR (dB) | - | - | 5.75 | - | -1.05 | -0.25 |
| MCL (dB) | - | - | 125.91 | - | 129.70 | 128.90 |

- Payload size of 408

| 1 UEs | 3 PRB |  | 6 PRB |  | 12 PRB |  |
| --- | --- | --- | --- | --- | --- | --- |
| Required SINR (dB) | 6.3 |  | 0.3 |  | -4.6 |  |
| MCL (dB) | 125.36 |  | 128.35 |  | 130,24 |  |
| 2 UEs | 3 PRB |  | 6 PRB |  | 12 PRB |  |
|  | DMRS Ports | DMRS Sequences | DMRS Ports | DMRS Sequences | DMRS Ports | DMRS Sequences |
| Required SINR (dB) | - | - | 0.75 | 1.08 | -4 | -3.9 |
| MCL (dB) | - | - | 127,90 | 127,57 | 129,64 | 129,54 |
| 3 UEs | 3 PRB |  | 6 PRB |  | 12 PRB |  |
|  | DMRS Ports | DMRS Sequences | DMRS Ports | DMRS Sequences | DMRS Ports | DMRS Sequences |
| Required SINR (dB) | - | - | 3 | 4.1 | -2.7 | -2 |
| MCL (dB) | - | - | 125.65 | 124.55 | 128.34 | 127.64 |

- Payload size of 1032

| 1 UEs | 12 PRB |  |
| --- | --- | --- |
| Required SINR (dB) | -0.9 |  |
| MCL (dB) | 126.54 |  |
| 2 UEs | 12 PRB |  |
|  | DMRS Ports | DMRS Sequences |
| Required SINR (dB) | 0.14 | 0.5 |
| MCL (dB) | 125,50 | 125,14 |
| 3 UEs | 12 PRB |  |
|  | DMRS Ports | DMRS Sequences |
| Required SINR (dB) | 2.6 | 4.1 |
| MCL (dB) | 123.04 | 121.54 |

Observation 2. Lower MCL differences between MsgA PRACH and PUSCH are observed for short preamble format A1 than for preamble format 0.

Observation 3. Larger MCL differences between MsgA PRACH and PUSCH are observed using a larger payload size of 1032 bits compared to payload of 72 bits.

As depicted in Figure 4 increasing the number of users multiplexed in a PUSCH occasion degrades the BLER performance. In addition, when the payload size increases from 72 to 1032 bits, the performance degradation is more significant and the number of PRBs should be increased to achieve higher MCL. Different DMRS ports/sequences can be used to decrease the collision probability of PRUs, although the number of UEs trying to perform the two-step RACH procedure simultaneously on the same PRACH occasion should be rather small to avoid high collision probabilities.

Observation 4.  Using different DMRS ports or DMRS sequences can decrease the collision probability within a PRU, where using different DMRS ports achieves better MCL performance.

Observation 5. As the number of users multiplexed in a PUSCH occasion increases, the BLER performance degrades significantly.

Observation 6. Increasing the number of DMRS ports/sequences has diminishing returns as the BLER performance with multiple simultaneously transmitted UEs degrades.

# Power Control Analysis

In RAN1#97, there was a discussion on whether to have separate preamble power control parameters for 2-step and 4-step RACH or have shared power control parameters. The benefit of having separate preamble power control parameters for 2-step and 4-step RACH is that it provides more flexibility to the network when setting these parameters. In this section, we show the benefit of this flexibility.

As mentioned in [5], if 2-step RACH and 4-step RACH share the same PRACH Occasions, the preamble power control parameters should be common to avoid performance degradation. The analysis presented in this section is for the case when 2-step RACH and 4-step RACH have separate PRACH Occasions.

The cost of a misdetection for 2-step RACH is higher than that of 4-step RACH, as a 2-step RACH retransmission involves retransmitting the MsgA PRACH and MsgA PUSCH, this leads to higher energy from the UE, and consequently more interference.

In this analysis, we consider two scenarios for 4-step RACH and 2-step RACH as shown in 4. For the preamble we assume format A1 with a duration of 2 symbols, for the PUSCH transmission (Msg3 or MsgA PUSCH) we assume a transmission of length 14 symbols and with 2 PRBs. For comparison of the various schemes with the various RACH types, we define a quantity called normalized energy en which is the product of the duration of the transmission in symbols Tsym and the absolute value of the SNR.

en=Tsym∙10SNRdB/10

Table 4: Scenarios for power control analysis of 4-step RACH and 2-step RACH.

| RACH Type | Scenario | Parameters |
| --- | --- | --- |
| 4-step RACH | Scenario 1 | Pmd (PRACH) = 10% BLER (Msg3) = 1% |
|  | Scenario 2 | Pmd (PRACH) = 1% BLER (Msg3) = 1% |
| 2-step RACH | Scenario 1 | Pmd (PRACH) = 10% BLER (PUSCH) = 1% |
|  | Scenario 2 | Pmd (PRACH) = 1% BLER (PUSCH) = 1% |

Figure 5 shows the state diagram of the 4-step RACH procedure. The average number of times in the preamble state is

Npreamble=11-pmd

In the preamble state, the UE transmits or retransmits the preamble. The average number of times in Msg3 state is

Nmsg3=11-BLER

In the Msg3 state, the UE transmits or retransmits Msg3.

Figure 5: 4-step RACH state diagram

Figure 6 shows the state diagram of the 2-step RACH procedure. The average number of times in the MsgA state is

NmsgA=11-pmd

In the MsgA state, the UE transmits or retransmits MsgA including MsgA PRACH and MsgA PUSCH. The average number of times in the MsgA PUSCH state is

NmsgAPUSCH=BLER1-BLER

In the MsgA PUSCH state, the UE only retransmits MsgA PUSCH. The UE reaches MsgA PUSCH state if the MsgA transmission has a successfully received PRACH part, but not the PUSCH part.

Figure 6: 2-step RACH state diagram

Table 5 shows the normalized energy analysis for 4-step RACH and 2-step RACH scenario 1 and scenario 2. Based on this analysis, for 4-step RACH, scenario 1 with pmd = 10% has a lower normalized energy than scenario 2. However, for 2-step RACH, scenario 2 with pmd = 1% has a lower normalized energy than scenario 1.

Table 5: Normalized energy for 4-step RACH and 2-step RACH scenario 1 and scenario 2.

|  | 4-Step RACH Scenario 1 | 4-Step RACH Scenario 2 | 2-Step RACH Scenario 1 | 2-Step RACH Scenario 2 |
| --- | --- | --- | --- | --- |
| Preamble pmd | 10% | 1% | 10% | 1% |
| Preamble SINR (Figure 1) | -10 dB | -5.5 dB | -10 dB | -5.5 dB |
| Normalized energy for 1 preamble enpre=Tsym∙10SNRdB/10 | 0.2 | 0.56 | 0.2 | 0.56 |
| Preamble (Msg1) normalized energy, en1=enpre1-pmd | 0.22 | 0.56 |  |  |
| Msg3 or MsgA PUSCH BLER (Figure 4) | 1% | 1% | 1% | 1% |
| Msg3 or MsgA PUSCH SINR | -1.5 dB | -1.5 dB | -1.5 dB | -1.5 dB |
| Normalized energy for PUSCH enpus=Tsym∙10SNRdB/10 | 9.91 | 9.91 | 9.91 | 9.91 |
| MsgA (PRACH + PUSCH) normalized energy enA=enpre+ enpus1-pmd |  |  | 11.23 | 10.57 |
| Msg3 normalized energy, en1=enpus1-BLER | 10.01 | 10.01 |  |  |
| MsgA PUSCH fallback normalize energy, enAFB=enpus∙BLER1-BLER |  |  | 0.1 | 0.1 |
| Total RACH procedure normalized energy: 4-step: en1+en3 2-step: enA+enAFB | 10.23 | 10.57 | 11.33 | 10.67 |

Proposal 1: To optimize the performance of 2-step RACH and 4-step RACH, the preamble power control parameters can be set differently.

# Conclusions

In this contribution, we presented first link level performance evaluations of MsgA for 2-step RACH. Based on the evaluation results, we have the following observations and proposal:

Observation 1. Increasing the number of transmitting UEs for a fixed false alarm rate leads to an increase of the miss- detection rate of preamble detection.

Observation 2. Lower MCL differences between MsgA PRACH and PUSCH are observed for short preamble format A1 than for preamble format 0.

Observation 3. Larger MCL differences between MsgA PRACH and PUSCH are observed using a larger payload size of 1032 bits compared to payload of 72 bits.

Observation 4.  Using different DMRS ports or DMRS sequences can decrease the collision probability within a PRU, where using different DMRS ports achieves better MCL performance.

Observation 5. As the number of users multiplexed in a PUSCH occasion increases, the BLER performance degrades significantly.

Observation 6. Increasing the number of DMRS ports/sequences has diminishing returns as the BLER performance with multiple simultaneously transmitted UEs degrades.

Proposal 1: To optimize the performance of 2-step RACH and 4-step RACH, the preamble power control parameters can be set differently.

# References

- RP-182894, “New work item: 2-step RACH for NR”, ZTE Corporation, Sanechips, 3GPP RAN#89, Sorrento, Italy, December 2018.

[2] R1-1903834. Email discussion on potential link-level simulation assumption for 2-step RACH. Meeting #96, 3GPP

[3] R1-1906748, “2-step RACH latency and resource reservation”, Nokia, Nokia Shanghai Bell, 3GPP RAN#96bis, Reno, USA, May 2019.

[4] R1-1908341, “Considerations On 2-step RACH Channel Structure”, Nokia, Nokia Shanghai Bell, 3GPP RAN#96bis, Prague, Czech Republic, August 2019.

[5] R1-1908342, “On 2-step RACH Procedure”, Nokia, Nokia Shanghai Bell, 3GPP RAN#96bis, Prague, Czech Republic, August 2019.

# Appendix A. Simulation assumptions

Table A.1 General simulation assumptions

| Parameters | Values |
| --- | --- |
| Subcarrier spacing | 30 kHz |
| Frequency band | 4 GHz |
| PRACH preamble | Format A1 and 0 |
| Propagation channel model | TDL-C DS=300ns |
| UE velocity | 3 kph |
| Waveform | CP-OFDM |
| Antenna configuration | 1Tx4Rx |
| DMRS configuration | Type 1 DMRS, 2 OFDM symbols |
| Payload | 72, 408, 1032 bits |
| Number of UEs | 1, 2 and 3 |
| Receiver | linear MMSE-IRC |
| Channel estimation | Realistic Ideal could be used for calibration purposes |
| Timing offset | 200m, UMi: 0.66us |
| Frequency offset | 0.05ppm (fixed) at TRP, and 0.1 ppm (fixed) at UE |