3GPP TSG RAN WG1 Meeting #98																R1-1908345

Prague, Czech Republic, August 26 – 30, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | DRAFT | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Payload for CSI reports configured for PUCCH and transmitted on PUSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When a CSI report is configured to be transmitted on PUCCH, but due to overlap of the PUCCH with a PUSCH, the CSI report is transmitted on PUSCH, in 38.214, it states that: “the encoding scheme follows that of PUCCH”. The intention of this statement is for the payload generation (CSI part 1 and CSI part2) and not the channel coding. For example, a wideband CSI report when configured for PUCCH has single CSI part, but when configured for PUSCH has CSI part 1 and CSI part 2. A CSI report configured for PUCCH but transmitted on PUSCH follows the payload generation of PUCCH, i.e. single CSI part. This CR clarifies this intended behavior. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that when a CSI report that has been configured for PUCCH is transmitted on PUSCH, the determination of the payload for CSI part 1 and CSI part 2 follows that of PUCCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity in the specs as it could be interpreted that the channel coding of CSI configured for PUCCH and piggy-backed on PUSCH follows that of PUCCH, which is not the intended behavior and conflicts with other specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: - This CR clarifies the intended behavior when CSI reports configured for PUCCH are piggy-backed on PUSCH. - If either the UE or the gNB doesn’t implement the CR, and interprets the “encoding scheme” as the channel encoding scheme rather than for payload determination, the reception of the UCI payload fails at the gNB when piggy-backed on PUSCH. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.2.3	CSI reporting using PUSCH

<Omitted Parts>

For both Type I and Type II reports configured for PUCCH but transmitted on PUSCH, the determination of the payload for CSI part 1 and CSI part 2 follows that of PUCCH as described in Subclause 5.2.4.

<Omitted Parts>