3GPP TSG-RAN WG1 98                                                    	 R1-1908350

Prague, CZ, August 26th – 30th, 2019

Source:	OPPO

Title:	Enhancements on type II CSI feedback

Agenda Item:	7.2.8.1

Document for:	Discussion and Decision

Introduction

The new WID [1] for NR MIMO was agreed in RAN #80 meeting. The enhancement of type II codebook can be considered in Rel-16 from the following aspects:

- Extend specification support in the following areas [RAN1]

  - Enhancements on MU-MIMO support:

    - Specify overhead reduction, based on Type II CSI feedback, taking into account the tradeoff between performance and overhead

    - Perform study and, if needed, specify extension of Type II CSI feedback to rank >2

In RAN1 #97 meeting, the compression scheme for the above enhancements was discussed with following agreements:

Agreement

Alt3C (illustrated in the table below) is supported where the parameter p=v0 for RI=3-4 is higher-layer configured in conjunction with the parameter p=y0 for RI=1-2.

- The parameters (y0, v0)(y0,v0) take value from  12,14,14,14,14,18

| RI | Layer | L | p |
| --- | --- | --- | --- |
| 1 | 0 | x0 | y0 |
| 2 | 0 |  |  |
|  | 1 |  |  |
| 3 | 0 |  | v0 |
|  | 1 |  |  |
|  | 2 |  |  |
| 4 | 0 |  |  |
|  | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |

- FFS: Possible down-selection on the FD combination parameters in RAN1#98

Agreement

For further details on the agreed UCI parameters in Table 1 of R1-1905629:

- RI ({1,…, RIMAX}∈1,…,RIMAX) and KNZ,TOTKNZ,TOT (the total number of non-zero coefficients summed across all the layers, where KNZ,TOTKNZ,TOT {1,2,…, 2K0}KNZ,TOT∈1,2,…,2K0 are reported in UCI part 1

  - FFS: If the total number of non-zero coefficients are jointly encoded with M’ (if supported) or independently encoded

- For RI=3-4, bitmaps, each with size-2LMi (i=0,1,…, RI-1, where i denotes the i-th layer) are reported in UCI part 2

  - FFS: If alt 3-4 is supported, size-2LMi2LMi-1 (i=0,1,…, RI-1i=0,1,…,RI-1, where i i denotes the ii-th layer) are reported in UCI part 2

- The following FD basis subset selection scheme is supported:

  - For N3≤19N3≤19, one-step free selection (cf. Alt5.1 in RAN1#96bis) is used

  - For N3>19, IntS is window-based and fully parameterized with MinitialMinitial, indicating that the intermediate set consists of FD bases mod(Minitial + n, N3), n=0,1, …,  modMinitial+n,N3, n=0,1,⋯N3'-1

    - The value N3'=αM where  is higher-layer configured from two possible values

      - FFS (to be finalized in RAN1#98 Prague): the supported parameter combinations for (L, p, β, )L,p,β,α

  - The 2nd step subset selection is indicated by an X2-bit combinatorial indicator (for each layer) in UCI part 2

Agreement

In RAN1#98, finalize the values of  based on the following aspects

- Candidate values for  to be down selected/evaluated: at least {1.5, 2, 2.5}

  - The set of values is to be finalized via offline email discussion prior to RAN1#98

- Configuration of :

  - Whether it is independent of other FD compression parameters, or dependent on at least one of the other FD compression parameters, i.e. p (=y0, and/or v0 for RI=3-4), L, β, and/or R

  - Whether  is rank-specific or rank-common

  - Note: This is to be discussed along with the supported parameter combinations for (L, p, β, ) L,p,β,α

Agreement

In RAN1#98, decide if the specification will restrict the UE from reporting all “zero” in the bitmap for a polarization for each layer

Agreement

On SCI (RI>1) and FD basis subset selection indicator, support Alt B described in the following table.

- FFS: details on bitwidth and possible values for Minitial Minitial reporting in UCI part 2

- FFS: whether the possible value(s) for Minitial Minitial can depend on configured FD compression parameters

- Up to the editor to capture this agreement

|  | Alt B |
| --- | --- |
| SCI for RI>1 | Alt3.4: Per-layer SCI, where SCIi SCIiis a –bit (i=0,1,…(RI - 1)). The location (index) of the strongest LC coefficient for layer i before index remapping is (li*,mi*), SCIi=li*, and mi* is not reported |
| Index remapping | For layer i, the index mimi of each nonzero LC coefficient cli,mi is remapped with respect to mi* to mi such that mi*=0. The FD basis index kmiassociated to each nonzero LC coefficient cli,mi is remapped with respect to kmi* to kmi such that kmi*=0. The sets cli,mi≠cli*,0 and kmi≠0 are reported. Informative note (for the purpose of reference procedure): The index li,mi of nonzero LC coefficients is remapped as (li,mi)→(li,mi-mi*modMi). The codebook index associated with nonzero LC coefficient index li,mi is remapped as kmi→kmi-kmi*modN3. |
| Combinatorial indicator for N3 ≤ 19N3≤19 | log2N3-1Mi-1bits |
| Combinatorial indicator for N3>19N3 > 19 | log2N3'-1Mi-1 bits |
| Minitial | Reported in UCI part 2, details on bitwidth and possible values are FFS |

Discussion

Value of alpha

Window-based FD basis subset selection is supported for N3 > 19. It was agreed that two possible values of higher configured parameter alpha shall be selected from the set {1.5, 2, 2.5, 3}, where alpha is used to determine the intermediate window size  N3'=αM . Configuring a large window size reduce the possibility of FD basis being out of window, but it is inefficient for overhead saving. Overhead of FD basis indicator selection and the window staring position Minitial are relevant to the value of alpha, while the later would be less impacted. For example, consider N3 = 26 and p = 1/2, overhead of FD basis indicator is {8, 11, 14, 16} bits per layer, and {4, 4, 5, 5} bits could be used to indicate  Minitial , corresponding to alpha {1.5, 2, 2.5, 3} respectively.

SLS results evaluated the performance-overhead trade-off achieved by each of the candidate alpha value. 10MHz bandwidth with R=2 is assumed in the SLS. Rank1-2 and Rank3-4 are evaluated separately and compared with reference to Rel-15 Type I codebook. Detail simulation conditions are listed in appendix.

Figure 1. Comparison of alpha value for rank 2

Figure 2. Comparison of alpha value for rank 4

Overall, similar performance-overhead trade-off is observed for different values of alpha from the set {1.5, 2, 2.5, 3}. The performance gap of these values are within 1% for both MU and SU-MIMO scenarios. Selection of alpha =2.5 and 3 provide marginal gain over alpha=1.5 and 2. We can also observe the pairs of {1.5, 2} and {2.5, 3} have very close results on performance-overhead trade-off. The SLS results seem to show that the choice of alpha does not have impact on performance when K0 is small. Hence, it is proposed to use a small alpha to minimize the overhead.

Observation 1: For the value of alpha, {1.5, 2, 2.5, 3} have similar performance-overhead trade-off.

Based on the observation, it is preferred to support the minimum alpha value to minimize overhead since increasing alpha does not provide significant gain. Therefor we have the proposal:

Proposal 1: Choose the values of {1.5, 2.5} for alpha in Rel-16 codebook.

Minitial reporting

For window-based FD basis subset selection, Minitial is reported in UCI part 2 to indicate the starting position of window. Since the intermediate window should always cover FD basis with index 0, the maximum number of candidate values for Minitial is N3'. Then m’th code-point to indicate Minitial could be mapped to Minitial=mod-m,N3.  Considering the maximum size of N3' is 30 (assuming alpha=3 and M=10), thus at most 5 bits is required to report Minitial. The candidate values of  Minitial could be further restricted to reduce overhead. For instance, only Minitial=mod(-N3'+1,N3) or Minitial=mod(-N3'/2,N3) are reported with 1 bit overhead.

Different bit-widths of  Minitial are evaluated for MU-MIMO with the assumption of alpha = 2.5. The SLS results are summarized in Table 1. It’s observed that only 4-bits bit-width of Minitial can provide nearly 0.5% Tput gain over all other values.

Table 1. Comparison of different bit-width for  Minitial

| (L, p, beta) | Bit-width | Gain |
| --- | --- | --- |
|  | Type I | 0% |
| (4,1/2,1/4) | 1 | 28.96% |
|  | 2 | 28.83% |
|  | 3 | 28.78% |
|  | 4 | 29.47% |
| (4,1/2,1/2) | 1 | 34.54% |
|  | 2 | 34.38% |
|  | 3 | 34.67% |
|  | 4 | 34.87% |

Considering 5 bits overhead is acceptable, and the gain of optimizing Minitial selection is marginal, we have the following proposal:

Proposal 2: Support free  Minitial reporting with bit-width of log2N3'  for Rel-16 codebook.

{L, p, beta} combination

There are totally 27 possible higher layer configured {L, p, beta} combinations determining #SD basis, #FD basis, #NZC respectively. It is agreed to remove redundant combinations to simplify implementation following the agreed criteria in email discussion:

- Use the following criteria for down-selecting the supported parameter combinations: - Avoid overly complex down selection by reducing the number of combinations for (L,p,beta) only - Alpha has not been decided and making it N3 dependent is unnecessarily convoluted - For #1 - Remove combinations with: - the total overhead exceeding the max of Rel.15 Type II - the UPT lower than Rel.15 Type II for the same overhead - When several combinations appear redundant in terms of overhead (sharing similar overhead – or vice versa), choose the one with the best UPT, unless the best combo is shown to be scenario-dependent. Then it is justified to support several combos with more or less the same overhead

| Higher layer configured | Possible value |
| --- | --- |
| L | 2, 4, 6 (32 ports, R=1, Rank 1/2) |
| p | {y0, v0} = {1/2,1/4}, {1/4,1/4}, {1/4, 1/8} |
| β | 1/4, 1/2, 3/4 |

SLS is conducted to evaluate several {L, p, beta} combinations for down-selecting the combinations and the SLS results are illustrated in Figures 3 and 4 for Rank 2 and 4, respectively.

Figure 3 {L, p, beta} combination, Rank 2

Figure 4 {L, p, beta} combination, Rank 4

We can make the following observations from the SLS results:

- {L, p}= {2, 1/2} and {4, 1/4} have same overhead,  but performance of {4,1/4} outperforms that of {2, 1/2}.

- Beta=3/4 does not provide good performance–overhead trade-off compared to other combinations.

- In comparison of {y0, v0} = {1/4, 1/4}  with {1/4, 1/8}, only {L, y0, v0, beta} = {4, 1/4, 1/4, 3/4} provides some gain and the gain is marginal.

To select parameter combinations with better performance-overhead trade-off , wepropose that:

Proposal 3: Consider to support following {L, p, beta} combinations for Rel-16 codebook:

| L | p=y0 (rank 1-2) | p=v0 (rank 3-4) | β | Restriction (if any) |
| --- | --- | --- | --- | --- |
| 2 | 12 | 14 | 14 |  |
| 2 | 12 | 14 | 12 |  |
| 4 | 14 | 18 | 14 |  |
| 4 | 14 | 18 | 12 |  |
| 4 | 12 | 14 | 12 |  |

CSI omission

The payload size of Part 2 UCI highly depends on the value of RI. The overhead of RI>1 is two times or even more of that with RI = 1. The gNB may allocate improper PUSCH resource without perfect knowledge of RI. If gNB allocate resource for a lower rank but UE want to report a higher rank, part 2 UCI cannot be fully transmitted and CSI omission may happen. In that case, the UE has to reduce the size of part 2 UCI. There are two alternatives to ensure part 2 UCI is smaller the size of PUSCH

- Alt1: UE drops non-zero coefficients through implementation to meet code rate restriction.

- Alt2: Define CSI omission and priority rule.

For Alt.1, UE can ensure the dropped NZC to impose minimum impact on CSI report performance. For example, UE drops weaker NZC and controls the reported NZC distribution across layers. If dropping NZC cannot fit PUSCH resource, UE may reduce the rank. This method does not change UCI part 2 format, thus it’s transparent to spec and to the gNB.

For Alt.2, the UE can omit a portion of NZC according to pre-defined priority order. One example of priority order is the NZC with stronger power and with FD index being close to FD index 0 shall have higher priority for reporting. Similar to rank 3/4 {L, p} issue, reducing p first is preferred. Therefore, SD component should have higher priority. Similar to per layer K0 restriction issue, it’s desired to have uniform distribution of NZC across layers. Furthermore, considering bitmap as prefix of LC, bitmap of the dropped NZC could be omitted.

However, it seems the above solutions are not flexible since the size of higher rank UCI cannot always be smaller than the size of allocated resource by reducing the number of NZC only. Especially for {y0, v0}={1/4, 1/4} and small beta, reporting 1 NZC per layer cannot solve the PUSCH resource limitation for large bitmap.

Alt.1 is simpler and it does not have spec impact. The benefit of supporting CSI omission is unclear. Therefore, we slightly prefer Alt.1 and the following proposal is made:

Proposal 4: Consider not supporting CSI omission for Rel-16 codebook.

CBSR

It was agreed codebook subset restriction is supported in Rel-16. CBSR reduces interference strength for other cells by restricting power of spatial beams in Rel-15. Joint SD/FD component power restriction was discussed. The feasibility of FD component restriction is unclear. Restricting FD component cannot reduce interference strength but increase frequency correlation. Direction restriction is a quite straightforward solution, thus we prefer to only support spatial beam restriction. Another issue is that Rel-15 CBSR restricts WB amplitude but Rel-16 does not defines WB amplitude. So, we support maximum amplitude restriction on each LC correspond to spatial basis. In summary, we made the following proposal for CBSR:

Proposal 5: Reuse Rel-15 Type II CBSR for Rel-16 codebook and only restrict the spatial basis amplitude.

Conclusion

In this contribution, we analyze the overhead reduction of Type II CSI feedback with some system level evaluation results. Based on the analysis and evaluation, we have following observations and proposals:

Observation 1: For the value of alpha, {1.5, 2, 2.5, 3} have similar performance-overhead trade-off.

Proposal 1: Choose the values of {1.5, 2.5} for alpha in Rel-16 codebook.

Proposal 2: Support free  Minitial reporting with bit-width of log2N3'  for Rel-16 codebook.

Proposal 3: Consider to support following {L, p, beta} combinations for Rel-16 codebook:

| L | p=y0 (rank 1-2) | p=v0 (rank 3-4) | β | Restriction (if any) |
| --- | --- | --- | --- | --- |
| 2 | 12 | 14 | 14 |  |
| 2 | 12 | 14 | 12 |  |
| 4 | 14 | 18 | 14 |  |
| 4 | 14 | 18 | 12 |  |
| 4 | 12 | 14 | 12 |  |

Proposal 4: Consider not supporting CSI omission for Rel-16 codebook.

Proposal 5: Reuse Rel-15 Type II CBSR for Rel-16 codebook and only restrict the spatial basis amplitude.

References

- RP-181453, WI Proposal on NR MIMO Enhancements, Samsung, RAN#80, June 2018

Appendix

| Parameter |  | Value |
| --- | --- | --- |
| Duplex |  | FDD |
| Waveform |  | OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban |
| Frequency Range |  | 4GHz. |
| Inter-BS distance |  | 200m |
| Layout |  | Hexagonal grid, 3 sectors per site, 19 macro sites, 570 UEs |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ for overhead reduction |
| Antenna setup and port layouts at UE |  | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for overhead reduction 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank extension |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS |
| Simulation bandwidth |  | 10 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO for rank4, MU-MIMO for rank2 |
| maximum MU layers |  | 12 |
| CSI feedback |  | CSI feedback periodicity (full CSI feedback) : 5 ms, Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms Codebook coeff. quantization (Amplitude, phase )= (3bits,4bits) |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 20% for SU-MIMO, 70% for MU-MIMO |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Baseline for performance evaluation |  | Rel-15 Type II Codebook for overhead reduction. |
| Overhead |  | 2 PDCCH symbols DMRS overhead: up to actually scheduled total layers 1 SSB per 20ms CSI-RS: 32ports, 5ms period, 1RE/port/RB CSI-IM: 4 REs/PRB, 5ms period TRS: 12 REs/PRB, 20ms period, maximal bandwidth with 52 PRB Total overhead: 24.24% |