3GPP TSG-RAN WG1 Meeting #98	R1-1908368

Prague, Czech Republic, August 26 – 30, 2019

Agenda item:	7.2.4.4

Title:	Discussion on in-device coexistence

Source:	OPPO

Document for:	Discussion and Decision

1. Introduction

In the RAN#83 meeting, it was approved a new Release 16 WI on 5G V2X with NR sidelink [1] and later updated to [2] during the RAN#84 meeting, which includes an objective to specify:

- Solutions for ‘not co-channel’ in-device coexistence between LTE and NR sidelinks

  - TDM-based solutions as per the study outcome [RAN1, RAN2, RAN4]

  - FDM-based solutions with static power allocation as per the study outcome [RAN4]

    - This will not consider the case where LTE and NR sidelinks are in the same frequency band.

  - No impact to LTE specifications at least from RAN1 and RAN2 perspective.

During the last RAN1#96bis and RAN1#97 meetings, further progress was made on TDM-based long-time scale and short-time scale solutions [3] [4]. Since it has been concluded that the support of TDM-based long-time scale solution will not have any specification impact and FDM-based solutions with static power allocation is under the responsibility of RAN4 work, we will focus only on discussing TDM-based short-time scale solution for in-device NR and LTE sidelink coexistence in this contribution.

In the followings, we have listed all relevant agreements reached during both the SI and WI phases so far for convenience.

Agreements: - For TDM solutions for in-device coexistence between LTE and NR V2X: - Time Alignment - Subframe boundary alignment is required between LTE and NR V2X sidelinks - Both LTE and NR V2X sidelinks are aware of the time resource index (e.g., DFN for LTE) in both carriers Agreements: Assuming SPS scheduling (mode -3 or mode-4) for LTE V2X, for short time scale TDM solutions for in-device coexistence for V2X, - For each occurrence of Tx/Tx overlap, one RAT is prioritized over another - This requires some information exchange between LTE and NR sidelinks within the UE - FFS: whether the information exchange between LTE and NR sidelinks can support this requirement - FFS: if there is impact to RAN1 LTE specification with this agreement - FFS: whether this solution can be up to UE implementation - For each occurrence of Tx/Rx overlap, one RAT is prioritized over another - This requires some information exchange between LTE and NR sidelinks within the UE - FFS: if there is impact to RAN1 LTE specification with this agreement - FFS: whether this solution can be up to UE implementation - FFS: If determination of priority for Rx operation is feasible and whether the information exchange between LTE and NR sidelinks can support this requirement Agreements: - From RAN1 point of view, short term TDM solutions for NR and LTE V2X in-device coexistence is considered to be feasible for a UE when the load for the UE from LTE side and from NR side is at or below an acceptable level - For each occurrence of Tx/Tx overlap and of Tx/Rx overlap, one RAT is prioritized over another - High-level principles of prioritization (e.g., BSM is deemed to have a higher priority, etc.) of LTE/NR can be discussed during the WI phase, while it is expected that detailed solutions may be left for implementation Conclusion: - RAN1 does not see any specification impact for support of Long Term Time-Scale TDM for coexistence of NR and LTE sidelinks Working assumption: - For Tx/Tx overlap, - If packet priorities of both LTE and NR sidelink transmissions are known to both RATs prior to time of transmission subject to processing time restriction, then the packet with a higher relative priority is transmitted - In case the priorities of LTE and NR SL transmissions are the same, then it is up to UE implementation as to which transmission is chosen (e.g., taking into account congestion, etc.) - If packet priorities of both LTE and NR sidelink transmissions are not known to both RATs prior to time of transmission subject to processing time restriction, then it is up to UE implementation to manage Tx/Tx overlaps (e.g., LTE transmissions are always prioritized, etc.) - RAN1 does not assume any impact to LTE physical layer specifications Agreements: - For Tx/Tx overlap, - Confirm the working assumption made in RAN1#96bis - UE capability is defined for short-term time-scale TDM for in-device coexistence Agreements: - For Rx/Rx overlap, - Up to UE implementation to manage receptions of LTE and NR sidelinks.

2. Discussions

For short time scale TDM-based approach, to resolve the Tx/Tx coexistence issue, the working assumption confirmed in the last meeting that if packet priority information of both LTE and NR SL transmissions are known to both RATs prior to time of transmission, the packet with a higher relative priority is transmitted. If packet priorities are the same, then it is up to UE implementation to choose which packet to transmit and which one to drop. This implies that some sort of packet priority comparison between LTE and NR should be done at the UE. To our understanding, the definition of packet priority levels in LTE and NR are different due to their different QoS models. Therefore, in order to for the UE to perform packet priority comparison between the two RATs, some sort of mapping of relative priority levels between the two RATs would be necessary.

Proposal 1: Based on the confirmed working assumption on short-time scale TDM solution for Tx/Tx overlap, RAN1 should send an LS to RAN2 asking to them to define mapping of relative priority levels between the two RATs.

For the case of Tx/Rx overlap in short-time scale TDM solution, this would require exchange of sensing information between LTE and NR sidelinks within the UE. However, even if sensing information and priority information can be exchange fast enough, this would account for periodic type of traffic and the retransmissions of aperiodic traffic at the most. From one RAT’s perspective, aperiodic transmissions could happen at any time in the other RAT and it is not possible to know in advance. Therefore, it is not always possible to perform packet priority comparison between the two RATs and be 100% certain that there are no other aperiodic traffic transmissions in the other RAT. This is especially true for the case when LTE RAT is performing SL transmission in a subframe and NR RAT is receiving SL packets from other UEs at the same time. Furthermore, if the LTE RAT is always prioritised for reception and LTE traffic is congested, it is not reasonable for the NR RAT to always drop its packets in favour to always receive LTE packets.

Proposal 2: For the case of Tx/Rx overlap in short-time scale TDM solution, it is up to UE implementation to manage in-device coexistence between LTE and NR.

Based on the above discussion for the Tx/Rx overlap in short-time scale TDM solution, it is also our understanding that the agreed UE capability to defined for short-term time-scale TDM for in-device coexistence is only applicable for the case of Tx/Tx overlap. As such, if it is up to UE implementation to manage in-device coexistence between LTE and NR for the cases of Tx/Rx and Rx/Tx overlap, there is no need to define / introduce UE capability for these cases.

Observation: For the cases of Tx/Rx and Rx/Tx overlap in short-time scale TDM solution, if it is up to UE implementation to manage in-device coexistence between LTE and NR, then there is no need to define / introduce UE capability for these cases.

3. Conclusion

In this contribution, we discussed about short-time scale TDM-based solution to resolve in-device coexistence issue between LTE and NR-V2X operations. In summary, we have the following observations and proposals:

Proposal 1: To confirm the working assumption on short-time scale TDM solution for Tx/Tx overlap, RAN1 should send an LS to RAN2 asking to them to define mapping of relative priority levels between the two RATs.

Proposal 2: For the case of Tx/Rx overlap in short-time scale TDM solution, it is up to UE implementation to manage in-device coexistence between LTE and NR.

Observation: For the cases of Tx/Rx and Rx/Tx overlap in short-time scale TDM solution, if it is up to UE implementation to manage in-device coexistence between LTE and NR, then there is no need to define / introduce UE capability for these cases.

4. References

[1]	RP-190766 “New WID on 5G V2X with NR sidelink”, RAN#83, LG Electronics, Huawei

[2]	RP-190984 “Revised WID on 5G V2X with NR sidelink”, RAN#84, LG Electronics

[3]	RAN1#96bis Chairman’s notes (final version)

[4]	RAN1#97 Chairman’s notes (final version)