3GPP TSG RAN WG1 #98			R1-1908377

Prague, CZ, August 26th – 30th, 2019

Agenda Item: 7.1.6

Source: MediaTek Inc.

Title: Remaining issues on UE features

Document for: Discussion

# Introduction

In this contribution, we provide proposals for the modification of FG 3-5b and FG 6-25 in the latest R-15 NR UE feature list.

# FG 3-5b: PDCCH monitoring occasions for Case 2 with a span gap

The current description of FG 3-5b is difficult to achieve in some situations. For example, consider Table 13-15 of TS 38.213, i.e., the PDCCH monitoring occasions for Type0-PDCCH CSS set where the SS/PBCH block and CORESET multiplexing pattern 3 and {SS/PBCH block, PDCCH} SCS {120, 120} kHz. The corresponding monitoring occasions are depicted in Figure 1.

Figure 1. The PDCCH monitoring occasions in Table 13-15 of TS 38.213

It is difficult to have the same span pattern repeats in every slot. Specifically, we assume, in Figure 1, the CORESET duration is 2 OFDM symbols. The favourable SSB of a UE generally varies with time as the UE moves and can be any of the SSBs. So, the corresponding PDCCH monitoring occasions for PDCCH Type 0, 0A, and 2 CSS configured by MIB can happen at OFDM symbols 2 to 9 as shown in Figure 2. For a UE reports (X,Y)=(7,3), there is no room for monitoring occasions of PDCCH Case 2. For a UE reports (X,Y)=(4,3) and (7,3), Case 2 of PDCCH can only be assigned in OFDM symbols 12 and 13.

Figure 2. Pattern of PDCCH monitoring occasions for PDCCH Type 0, 0A, and 2 CSS configured by MIB

To get rid of this issue, it is proposed the monitoring occasions for PDCCH Type 0, 0A, and 2 CSS are not taken into account when defining the span gap in FG 3-5b. Our proposal is given as follows.

Proposal #1: The following modification is made on FG 3-5b.

| # | Feature group | Feature component | Note | RAN WG recommendations |
| --- | --- | --- | --- | --- |
| 3-5b | All PDCCH monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 with a span gap | PDCCH monitoring occasions of FG-3-1, plus additional PDCCH monitoring occasion(s) can be any OFDM symbol(s) of a slot for Case 2, and for any two PDCCH monitoring occasions other than the monitoring occasions for PDCCH Type 0, 0A, and 2 CSS belonging to different spans, where at least one of them is not the monitoring occasions of FG-3-1, in same or different search spaces, there is a minimum time separation of X OFDM symbols (including the cross-slot boundary case) between the start of two spans, where each span is of length up to Y consecutive OFDM symbols of a slot. Spans do not overlap. Every span is contained in a single slot. The same span pattern repeats in every slot. The separation between consecutive spans within and across slots may be unequal but the same (X, Y) limit must be satisfied by all spans. Every monitoring occasion is fully contained in one span. In order to determine a suitable span pattern, first a bitmap b(l), 0<=l<=13 is generated, where b(l)=1 if symbol l of any slot is part of a monitoring occasion, b(l)=0 otherwise. The first span in the span pattern begins at the smallest l for which b(l)=1. The next span in the span pattern begins at the smallest l not included in the previous span(s) for which b(l)=1. The span duration is max{maximum value of all CORESET durations, minimum value of Y in the UE reported candidate value} except possibly the last span in a slot which can be of shorter duration. A particular PDCCH monitoring configuration meets the UE capability limitation if the span arrangement satisfies the gap separation for at least one (X, Y) in the UE reported candidate value set in every slot, including cross slot boundary. For the set of monitoring occasions which are within the same span: - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for FDD - Processing one unicast DCI scheduling DL and two unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD - Processing two unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD The number of different start symbol indices of spans for all PDCCH monitoring occasions per slot, including PDCCH monitoring occasions of FG-3-1, is no more than floor(14/X) (X is minimum among values reported by UE). The number of different start symbol indices of PDCCH monitoring occasions per slot including PDCCH monitoring occasions of FG-3-1, is no more than 7. The number of different start symbol indices of PDCCH monitoring occasions per half-slot including PDCCH monitoring occasions of FG-3-1 is no more than 4 in SCell. | This capability is necessary for each SCS. | Optional with capability signaling Candidate value set for (X, Y): {(7, 3), (4, 3) and (7, 3), (2, 2) and (4, 3) and (7, 3)} |

# FG 6-25: Support of synchronous NR-NR DC operation

In RP-181473 “Rel-15 Work Item Exception for New Radio Access Technology – Core”, task(s) within WI which are not complete are identified. In RAN#80, 3 months exception was granted for the tasks.

(from RP-181473) Further details for each WG are shown in below - RAN1 - Option 4: - Evaluate whether new design on power control, multiplexing, etc. for both LTE & NR specs - Strive for minimum RAN1 specification impact - Some (limited) RAN1 meeting time is expected - NR-NR Dual Connectivity - Synchronous operation - Minimum RAN1 impact and no HW impact - No PUCCH-SCell

It is noted in RP-181473, for NR-NR Dual Connectivity, there is no PUCCH-SCell (yellow part above). However, this restriction is not stated in FG 6-25. Our proposal is given in the following.

Proposal #2: The following modification is made on FG 6-25.

| # | Feature group | Feature component | Note | RAN WG recommendations |
| --- | --- | --- | --- | --- |
| 6-25 | Support of synchronous NR-NR DC operation only wherein MCG is only in FR1 and SCG is only in FR2, and no PUCCH in SCell |  | This is conditioned on the support of DC band combination(s). UE reports a set of supported band partitionings corresponding to MCG in FR1 and to SCG in FR2. | Optional with capability signaling |