3GPP TSG RAN WG1 #98	                                                       R1-1908381

Prague, CZ, August 26th – 30th, 2019

Source: MediaTek Inc.

Title: Full Tx power UL transmission

Agenda Item: 7.2.8.4

Document for:  Discussion

# Introduction

A number of agreements were reached in previous meetings:

From RAN1 #96

Agreement

Note: UE capability 1, 2, 3 agreed in RAN1#AH1901 mean the PA architectures.

At least for PC3, UE capability 1, 3 can support full power transmission.

Working assumption: For PC3, UE capability 2 can support full power transmission.

- Companies to check for any implementation issues and/or performance of Rel-16 full power transmission compared to Rel-15 non-coherent codebook subset uplink transmission)

From RAN1 #96b

Agreement

Regardless of UE capability 1, 2, or 3, signalling of “UL full power tx capability” is supported for UEs with full power uplink transmission capability

- FFS: For UE capability 1, if any other information is necessary

- For UE capability 2 and UE capability 3, in addition to signalling “UL full power tx capability”, further information on UE capability are signalled if needed

  - FFS: Details such as support of UE capability signaling of supported one or group of TPMI precoder(s) for full power transmission, support different number of SRS ports for resources for codebook, and other UE capability signaling can be introduced

  - FFS: Whether full uplink TX power capability can be explicitly/implicitly derived from the TPMI/TPMI group precoders for full power transmission

UEs with full power uplink transmissions are those Rel-16 UEs which can transmit at full power at least for rank1

The signalling of above information does not imply any specific UE PA architecture implementation.

Agreement

The working assumption made in RAN1#96 is confirmed

For UL beam management latency and overhead reduction, support MAC CE based spatial relation update for aperiodic SRS per resource level

- FFS: Whether this is a UE optional feature

FFS: Whether above is applicable regardless of the aperiodic SRS target use

From RAN1 #97

Working Assumption

Support following scheme for UL full power Tx for UE capability 2 and 3:

- A UE can be configured for one of two modes of full power operation to support ‘Capability 2’ and ‘Capability 3’ subject to UE capability

- A UE can be configured by the network to support full power transmission

- Mode 1: The UE can be configured with one or more SRS resources with same number of SRS ports (according to Rel-15) within an SRS resource set which usage is set to ‘codebook’

  - gNB can configure the UE to use a subset of TPMIs that combine ports in a layer to produce full power transmission.

  - A new codebookSubset is introduced only for the rank value(s) where full power transmission in UL is not achievable includes the TPMI precoders in fullyAndPartialAndNonCoherent defined in Rel-15

    - FFS: At least a subset of the non-antenna selection TPMI precoder(s) is(are) supported

    - FFS: Additional support of antenna selection TPMI precoders

  - Note: as non-coherent UE, it is not capable of maintaining relative phase of antenna ports according to TPMI

- Mode 2: The UE can be configured with one SRS resource or multiple SRS resources with different number of SRS ports within a SRS resource set which usage is set to ‘codebook’

  - UE transmits SRS and PUSCH in same manner, whether antenna virtualization is used or not

  - Rel-15 codebooks and codebook subsets are used

    - Note: Antenna selection precoder can be used to enable full power related PA(s) to produce full power transmission for Capability-3 UE.

  - UL full power Tx is achieved for PUSCH transmission according to indicated SRI and/or TPMI

    - A set of TPMIs that deliver full power can be signalled by the UE in order to support at least  UEcap3, for SRS resource with more than 1 ports,

    - e.g. For SRI indicating SRS resource with 1 port then single layer PUSCH is transmitted with full power in same manner as single port SRS, if SRI indicating SRS resource with multiple ports is signalled based on Rel-15 MIMO behaviour (transmission rank indicator, TPMI indicator, etc) except the power scaling aspects

    - The following cases are not precluded

      - For example, for 4TX on UE side (with 20+20+17+17dBm) virtualized as 2 SRS ports, full uplink power transmission can be enabled by precoder [1 0] or [0 1]

    - FFS: number of SRS resources supported

      - 2

      - 3

    - FFS: for 4 Tx, how many different TPMIs/TPMI groups support full power

  - FFS: any rules for spatial filter update for the SRS resources with different number ports

Note: How to capture the behaviour for ‘Mode 1’ and ‘Mode 2’ in specifications is TBD

Note: For single port, there is no SRI and TPMI

Note: Support of Mode 1, Mode 2 have separate UE capability

In this contribution, we provide our view for this topic.

# UL full power Tx for UE capability 2 and 3

In RAN1#97 meeting, two modes were proposed to support UL full power for UE capability 2 and 3. It is UE capability to indicate which mode (or both) is supported by UE.

The basic concept of 2 modes are similar in terms of using power from multiple physical antenna ports to achieve target Tx power. However, the signalling and RRC configuration for the two approaches are different. Consider implementation complexity, both modes can be supported by specification. In this contribution, we provide details of the design for mode 1 and mode 2.

Proposal 1: Confirm the working assumption made in RAN1#97 for UE capability 2 and 3.

## Mode 1

gNB can configure the UE to use a new subset of TPMIs for rank values that cannot achieve full Tx power in Rel-15. The number of codewords in the subset should not exceed the number in Rel-15, as shown in Table 1.

In 2Tx UL for UE capability 2, the TPMI to support full power can be any non-antenna selection TPMI (note that for non-coherent UE, it is not capable of maintaining relative phase of antenna ports).

However, for UE capability 3, one may wish to use such TPMI to leverage the full rated PA so the appropriate TPMIs in general depends on UE architecture. For example, in Case 2 and Case 3 in Table 2, different codebook subset are preferred for different PA configurations. In that case, the association of port index to a PA can be arranged to reduce the number of combinations. For example, for 2Tx, the PA output power can be assumed to be non-increasing with port index, e.g. PA output power at port 0 is always larger than that at port 1 and preclude Case 3.

Similarly in 4Tx UL, the preferred TMPIs could vary according to UE’s PA configuration if all possible combinations are considered. As two possible configurations in Case 4, [17, 17, 17, 17] dBm requires a coherent precoder to get full Tx power while [20,20,20,20] dBm can have two options by either a coherent precoder or a partial-coherent precoder (e.g., [1 0 1 0]). To reduce possible choices, both configurations can be treated the same and use the same new codebook subset for full Tx power.

Another example in Case 5 considering 4Tx partial-coherent UE. When UE belongs to capability 3, we can assign the the most capable PA to port0 (coherent group 0) so a fixed precoder (TPMI=0) can be used for full Tx power without ambiguity. In other words, in order to reduce the number of combinations, we may ask the PA output power within a coherence group can be arranged in a non-increasing order and coherent group 0 has more (or equal) capable PA than coherent group 1.

After this arrangement, the new codebook subset is determined by number of Tx ports, UE PA’s capability (capability 2/ capability 3), UE’s coherence capability, and regardless of UE’s detailed PA configuration.

Table 1. Allowed number of codewords according to UE capability and rank

| Rank | Full Coherence | Partially Coherent | Non Coherent |
| --- | --- | --- | --- |
| 1 | 28 | 12 | 4 |
| 2 | 22 | 14 | 6 |
| 3 | 7 | 3 | 1 |
| 4 | 5 | 3 | 1 |

Table 2. Examples of Mode 1 for rank=1
(Ωr r=1,..,maxRank-1 denote codebook subset for rank r transmission)

| Case | codebookSubset (codeword indices) | UE’s PA configuration | UE’s capability | Note |
| --- | --- | --- | --- | --- |
|  | 2Tx non-coherent UE |  |  |  |
| 1 | Ω1 = [2,3] | PA = [20, 20] dBm | Capability 2 |  |
| 2 | Ω1 = [0,3] | PA = [23, 20] dBm | Capability 3 | Port0 can achieve 23dBm |
| 3 | Ω1 = [1,3] | PA = [20, 23] dBm | Capability 3 | Port1 can achieve 23dBm. (this configuration can be excluded by requiring port-to-PA mapping following non-increasing capable power order |
|  | 4Tx non-coherent UE |  |  |  |
| 4 | Ω1 =[12,14,20,22] | PA = [17, 17, 17, 17] dBm or PA = [20, 20, 20, 20] dBm | Capability 2 | For rank=1, 4-port combination is needed to achieve 23dBm |
|  | 4Tx partial-coherent UE |  |  |  |
| 5 | Ω1 =[0] | PA = [23*, 17**, 17*, 17**] dBm *: coherent group 0 **: coherent group 1 | Capability 3 | For rank=1, single port0 can achieve 23dBm |

Proposal 2: In mode 1, to facilitate full Tx power transmission, arrange antenna port such that

- the PA output power is non-increasing with the port index for non-coherent UE

- the PA output power within a coherence group is non-increasing with the port index for partial-coherent UE and coherent group 0 contains a more capable PA.

- the new codebook subset for full Tx power is determined by by number of Tx ports, UE PA’s capability (capability 2/ capability 3), UE’s coherence capability, and regardless of UE’s detailed PA configuration.

### Discussion of signalling message

In order to work with various UE’s hardware/PA configuration, one possible way to report UE supporting codewords for full power Tx is to use bitmap, indicating codewords for which full power transmission is supported. To reduce overhead, one can also consider to pre-define a few combinations of codebook subset and UE only can report one of those combination.

For non-coherent/partial-coherent UE, the reported codebook subset should include codewords that outside non-coherent/partial-coherent codewords. There are some options on how gNB/UE interpret the codebook subset and signal TPMI in DCI,

- Option 1: use DCI size is set according to codebookSubset= fullyAndPartialAndNonCoherent. In this case, all the codewords, irrespective whether they are for non-coherent, partially-coherent and fully coherenet are supported from signaling’s point of view. The TPMI has no ambiguity but require more DCI bits.

- Option 2: use the same DCI size as in Rel-15, and replace the codeword that TPMI points to within the restricted subset as the numbers are limited according to Table 1. Note that the reported codebook subset are intended for full power Tx while the normal non-coherent/partial-coherent precoders (depend on UE’s capability) should be still valid when UE transmits power below Pcmax. Therefore, 1-bit indication for switching to new set might be required which facilitates switching between the Rel-15 behavior and Rel-16 behavior at the UE if it is desirable to support both of them at a UE without RRC reconfiguration. However, Rel-15 behavior or Rel-16 behavior can be also selected and prescribed by the network for a Rel-16 UE, if one behaviour is good enough, then we can forgo the introduction of the additional “switching” bit.

## Mode 2

In operation of mode 2, UL full power feature is achieved by means of multiple SRS resources with different nrofSRS-Ports. In particular, UE is configured multiple SRS resources, one of them has fewer antenna ports that are virtualized from full antennas set. The virtualized antenna ports can support higher power than each individual PA, therefore fulfill full power transmission. As in mode 1, small delay CDD could be implemented by UE in virtualized SRS resource, which could be transparent to receiver.

There are several issues need to be taken care to make mode 2 work:

- Number of SRS resources, common parameters sharing and update.
If additional SRS resources are added for full Tx power purpose, most of the parameters (other than nrofSRS-Ports) should be shared between the SRS resources. Furthermore, in Rel-15 codebook based transmission, two SRS resources were already used for selecting between analog beams (via spatialRelationInfo) and we note that it is expected more dynamic behaviour of SRS resource in Rel-16 (note that it is agreed in RAN1 #96b to support MAC CE based spatial relation update for aperiodic SRS per resource level). That is said, it requires maximum X (X>2) SRS resources in codebook based transmission to fulfill analog beams switching and full power transmission. In that regard, considering structured SRS resource can help to reduce overhead.

Observation 1: In mode 2, max X (X>2) SRS resources are required for codebook based transmission. Structured SRS resource IE can help to reduce overhead.

- Requirement of virtualized SRS sounding
If the virtualized SRS has nrofSRS-Ports=1, according to mode 2 it should have same behaviour as single antenna case and therefore no precoding. One may think there is no need to actual SRS sounding. However, According to TS38.214 “The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI.” which means actual transmission of a SRS resource may still be required, otherwise the UE behaviour concerning Tx analog beamforming for PUSCH un-defined. Moreover, as in Case 8 in Table 3, determining TPMI to align between two coherent groups may still need a real sounding.

Observation 2: In mode 2, virtualized resources may still need to sound.

To deal with aforementioned issues, one solution is to introduce structured SRS resource. To explain the concept, we can first define ‘primary’ (or ‘master’) SRS resource and ‘secondary’ (or ‘slave’) SRS resource where the primary SRS works same as the original SRS resource in Rel-15. A secondary SRS resource is linked to one primary SRS resource and share most of the common parameters as well as the update (e.g., spatialRelationInfo from MAC CE). Secondary SRS resources are configured to capability 2 and capability 3 UEs who support mode 2 operation. In more general case, one may consider a primary SRS resource can potentially linked to more than one secondary SRS resources to leverage different levels of virtualization. The real implementation can be based on the example shown in the bottom of this section by a LinkTag field and illustrated in Figure 1.

We can interpret SRI in DCI as in Rel-15 for primary SRS resources and additional 1-bit to indicate if it is to use virtualized SRS or non-virtualized SRS. In that way, we can clarify in TS38.214 6.1.1.1 “The indicated SRI in slot n is associated with the later one between the most recent transmission of SRS resource identified by the SRI and the most recent transmission of SRS resource linked to the SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI.”

To summary mode 2, the general procedure can be as described as follows:

- UE report its capability: number of Tx, UL coherence capability, full power Tx capability (for Capability 2 or 3) and {mode1, mode2} capability.

- SRS-Resource configuration in RRC includes one field to link between associated SRS resources, as LinkTag shown below.

- gNB configures up to N primary SRS resources to an UE for maximum N spatial filters configured for SRS-ResourceSet::usage=codebook. For each n=1…N primary SRS resource, it can be configured at most M-1 secondary SRS resources, so up to M SRS resources are linked together (i.e., with the same LinkTag value). All SRS resources with the same LinkTag are configured with different nrofSRS-Ports for different antenna ports virtualization. The relationship between SRS resources is illustrated in Figure 1. Any dynamic behaviour for the SRS resource update is applied to all SRS resources with the same LinkTag.

- UE transmits SRS resources upon request.

- In DCI, gNB indicates one among M*N SRS resources for PUSCH transmission. If transparent S-CDD is applied during virtualization, same S-CDD are applied to both SRS and PUSCH.

Figure 1. Illustration of SRS resources relationship

SRS-Resource ::=                        SEQUENCE {

srs-ResourceId                          SRS-ResourceId,

nrofSRS-Ports                           ENUMERATED {port1, ports2, ports4},

ptrs-PortIndex                          ENUMERATED {n0, n1 }                                                OPTIONAL,   -- Need R

transmissionComb                        CHOICE {

n2                                      SEQUENCE {

combOffset-n2                           INTEGER (0..1),

cyclicShift-n2                          INTEGER (0..7)

},

n4                                      SEQUENCE {

combOffset-n4                           INTEGER (0..3),

cyclicShift-n4                          INTEGER (0..11)

}

},

resourceMapping                         SEQUENCE {

startPosition                           INTEGER (0..5),

nrofSymbols                             ENUMERATED {n1, n2, n4},

repetitionFactor                        ENUMERATED {n1, n2, n4}

},

freqDomainPosition                      INTEGER (0..67),

freqDomainShift                         INTEGER (0..268),

freqHopping                             SEQUENCE {

c-SRS                                   INTEGER (0..63),

b-SRS                                   INTEGER (0..3),

b-hop                                   INTEGER (0..3)

},

groupOrSequenceHopping                  ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

...

},

semi-persistent                         SEQUENCE {

periodicityAndOffset-sp                     SRS-PeriodicityAndOffset,

...

},

periodic                                SEQUENCE {

periodicityAndOffset-p                      SRS-PeriodicityAndOffset,

...

}

},

sequenceId                              INTEGER (0..1023),

spatialRelationInfo                     SRS-SpatialRelationInfo                                         OPTIONAL,   -- Need R

LinkTag   INTEGER (0..N-1)   % the linked SRS resources have the same LinkTag, so they are on equal footing, i.e. no slave or master. Where N can be the maximum number of spatial filters can be configured at the same time for codebook transmission

...

}

Proposal 3: In mode 2, consider structured SRS resource to facilitate mode 2 operation to minimize overhead.

# Conclusion

Observation 1: In mode 2, max X (X>2) SRS resources are required for codebook based transmission. Structured SRS resource IE can help to reduce overhead.

Observation 2: In mode 2, virtualized resources may still need to sound.

Proposal 1: Confirm the working assumption made in RAN1#97 for UE capability 2 and 3.

Proposal 2: In mode 1, to facilitate full Tx power transmission, arrange antenna port such that

- the PA output power is non-increasing with the port index for non-coherent UE

- the PA output power within a coherence group is non-increasing with the port index for partial-coherent UE and coherent group 0 contains a more capable PA.

- the new codebook subset for full Tx power is determined by by number of Tx ports, UE PA’s capability (capability 2/ capability 3), UE’s coherence capability, and regardless of UE’s detailed PA configuration

Proposal 3: In mode 2, consider structured SRS resource to facilitate mode 2 operation to minimize overhead.