3GPP TSG RAN WG1 #98			          R1-198387

Prague, Czech, August 26th – 30th, 2019

Agenda Item:	7.2.2.2.3

Source: 	MediaTek Inc.

Title:	Enhancements to HARQ for NR-U operation

Document for: 	Discussion and Decision

Introduction

In this contribution, we provide our views on the following issues for NR-U:

- Cross-COT HARQ-ACK feedback and multiple HARQ-ACK opportunities

- Multi-TTI PUSCH scheduling

Discussion

Cross-COT HARQ-ACK feedback and multiple HARQ-ACK opportunities

Group-based HARQ-ACK feedback for operation with dynamic HARQ-ACK codebook

In RAN1#97 meeting [1], it was agreed to introduce group-based HARQ-ACK feedback to enable both cross-COT HARQ-ACK feedback and multiple HARQ-ACK opportunities for operation with dynamic HARQ-ACK codebook.

Agreement from RAN1#97: For operation with dynamic HARQ codebook (type-2 codebook): - PDSCH grouping by explicitly signalling a group index in DCI scheduling the PDSCH - For any PDSCH scheduled with numerical or non-numerical value of K1 - The number of HARQ-ACK bits for one PDSCH group can change between successive requests for HARQ-ACK feedback for the same PDSCH group - HARQ-ACK feedback for all PDSCHs in the same group is carried in the same PUCCH - One DCI can request HARQ-ACK feedback for one or more PDSCH groups in the same PUCCH - C-DAI/T-DAI is accumulated only within each PDSCH group - FFS: Choose between the following options: - T-DAI is included only for the scheduled group - T-DAI is included for each group - New ACK-Feedback Group Indicator for each PDSCH Group operates as a toggle bit - Maximum number of PDSCH groups: 2 (FFS: maximum number of groups 4 and maximum number of groups for which feedback is requested in the same PUCCH) - A UE can signal support of this feature as part of capability signalling

For group-based HARQ-ACK feedback, PDSCHs scheduled to a UE would be grouped by a network. When UE receives a DCI scheduling a PDSCH belonging to a PDSCH group, all PDSCHs in the same PDSCH group are requested to be acknowledged in the same PUCCH indicated by the DCI (through PDSCH-to-HARQ-timing-indicator and PUCCH resource indicator). To make UE has a correct understanding of the number of scheduled PDSCHs in a PDSCH group, C-DAI/T-DAI are always accumulated over all PDSCHs per PDSCH group.

In DCIs scheduling PDSCHs, Group Index (GI) and New-ACK-Feedback-Group-Indicator (NGI) are conveyed in these DCIs for a UE to determine the grouping for the scheduled PDSCHs. For PDSCHs in the same PDSCH group, their corresponding GI values shall be the same, i.e., if PDSCHs are indicated with different GI values, UE can assume that they are in different PDSCH groups. Once a HARQ-ACK feedback of a PDSCH group is successfully received by the network, the network can initiate a new PDSCH group having the same GI value, which is indicated to UE by toggling corresponding NGI bit. From UE’s perspective, if an NGI bit indicated in a current DCI scheduling a PDSCH is toggled compared to an NGI bit indicated in a previously received DCI indicating the same GI value as the one indicated in the current DCI, UE can assume that the PDSCH scheduled by the current DCI belongs to a new PDSCH group, and the HARQ-ACK information of the old PDSCH group having the same GI value may be flushed out by the UE. Otherwise, the PDSCH scheduled by the current DCI still belongs to the PDSCH group including the PDSCH scheduled by the previously received DCI, and the HARQ-ACK information of the PDSCH group shall be reported using the PUCCH provided by the current DCI.

Since a PDSCH group can be dynamically enlarged to include a later scheduled PDSCH(s) and acknowledged together, the PUCCH(s) provided by the next DCI(s) with downlink assignment could be always taken as a supplemental HARQ-ACK opportunity until the HARQ-ACK feedback of the PDSCH group is received and decoded by the network, as the example shown in Figure 1. In other words, group-based HARQ-ACK feedback can provide unlimited number of HARQ-ACK opportunities to correct possible error cases for each scheduled PDSCH.

Figure 1. Example of group-based HARQ-ACK feedback

In some scenarios, the network may need to initiate more than one PDSCH groups to allow flexible scheduling behaviour, as the example shown in Figure 2, and up to two PDSCH groups should be sufficient. Meanwhile, it is also beneficial to allow that two PDSCH groups can be requested to be acknowledged in the same PUCCH. In order to indicate UE whether to piggyback the HARQ-ACK feedback of the other PDSCH group in the same PUCCH, a request (RQ) can be conveyed in a DCI with PDSCH scheduling.

For example, when the RQ bit is set to “0” in a DCI scheduling a PDSCH in a PDSCH group, only the HARQ-ACK feedback of the PDSCH group is transmitted in the PUCCH provided by the DCI. When the RQ bit is set to “1” in the DCI, the HARQ-ACK feedbacks for both two PDSCH groups are transmitted in the same PUCCH provided by the DCI. If two PDSCH groups are requested to be acknowledged in the same PUCCH, UE concatenates the two HARQ-ACK codebooks of the two PDSCH groups having different GI values by increasing/decreasing order of the GI values, where each HARQ-ACK codebook is individually generated according to C-DAI/T-DAI accumulated within each PDSCH group.

Figure 2. Example of group-based HARQ-ACK feedback

Proposal 1: Group-based HARQ-ACK feedback for operation with dynamic HARQ-ACK codebook:

- Maximum # of PDSCH groups: 2 (1 bit for group index)

- Maximum # of PDSCH groups for which HARQ-ACK feedbacks can be requested in the same PUCCH: 2

- A request bit is indicated in a DCI scheduling a PDSCH belonging to a scheduled  PDSCH group:

  - Set to “0”: only the HARQ-ACK feedback of the scheduled PDSCH group is requested in the PUCCH provided by the DCI.

  - Set to “1”: the HARQ-ACK feedbacks for both PDSCH groups having different GI values are requested in the PUCCH provided by the DCI.

  - If more than one PDSCH groups are requested to be acknowledged in the same PUCCH, UE concatenates the multiple codebooks individually generated for the multiple PDSCH groups by increasing/decreasing order of corresponding group index values.

Since C-DAI/T-DAI is only accumulated per PDSCH group, misdetection of the last DCI(s) scheduling PDSCH(s) in a PDSCH group can lead to misalignment of codebook size if HARQ-ACK information bits of multiple PDSCH groups are concatenated in the same PUCCH. To deal with possible error cases, in some proposals, a DCI scheduling a PDSCH belonging to a PDSCH group not only indicates a T-DAI value accumulated for the PDSCH group, but also indicates an additional T-DAI value accumulated for a PDSCH group having the other GI value. However, we don't see clear benefit of introducing the additional T-DAI. First, group-based HARQ-ACK feedback itself provides multiple opportunities to correct the possible error cases. For example, as shown in Figure 3, due to misdetections of PDCCHs in slots #n+1 and #n+2, the gNB may not correctly decoded/interpret the HARQ-ACK information bits transmitted in UCI#2. Then, in the later PDSCH scheduling, the gNB can include new PDSCH(s) in the two PDSCH groups, and request a HARQ-ACK feedback in UCI#3 for the two PDSCH groups. If UE can correctly decode the DCIs scheduling PDSCHs #10 and #11, it can aware that there are two misdetected PDCCHs according to the C-DAI values indicated in the DCIs. Second, since the error cases only happen when more than one PDSCH groups are requested to be acknowledged in the same PUCCH, the gNB can avoid such error cases by requesting a HARQ-ACK feedback in each PUCCH for only one PDSCH group. Even latency for obtaining feedback for multiple PDSCHs groups may be larger, constant DCI overhead of the additional T-DAI value is not needed and the reliability can be guaranteed.

Proposal 2: Group-based HARQ-ACK feedback for operation with dynamic HARQ-ACK codebook:

- T-DAI is included in a scheduling DCI only for the scheduled PDSCH group

Figure 3. Example of group-based HARQ-ACK feedback

In some proposals, in a DCI scheduling a PDSCH belonging to a PDSCH group having a GI value, an additional NGI bit for a PDSCH group having the other GI value is also indicated. There are two possible intentions to indicate the additional NGI bit:

- Request a HARQ-ACK feedback for a new or old PDSCH group having the other GI value. As the example shown in Figure 4, when the DCIs in slots #p+1 and #p+2 set the RQ bits to “1”, not only the PDSCH group having GI = 1, but also the PDSCH group having GI = 0 are requested to be acknowledged in the same PUCCH. In Figure 3(a), since the additional NGI bit (NGI-2) is set to 1, UE acknowledges reception of the new PDSCH group including PDSCHs #3, #4, and #5 indicated with GI = 0 and NGI-1 = 1 in UCI #2. In Figure 3(b), different from Figure 3(a), since the additional NGI bit (NGI-2) is set to 0, UE acknowledges reception of the old PDSCH group including PDSCHs #1, #2, and #3 indicated with GI = 0 and NGI-1 = 0 in UCI #2. However, in general, a new PDSCH group is initiated only if the HARQ-ACK feedback of the old one has been received and decoded by the network. Therefore, it is not necessary to request feedback for the old PDSCH group. Instead, without the additional NGI bit, if the PDSCH group having the other GI value is requested to be acknowledged in the same PUCCH, UE should always transmit the HARQ-ACK feedback for the “newest” PDSCH group having the other GI in the same PUCCH.

Figure 4. Example of indicating the additional NGI bit (NGI-2) for a PDSCH group having the other GI value

- The additional NGI bit indicates the same status as the one indicated for the newest PDSCH group have the other GI value. Based on the additional NGI bit, UE can aware that whether the newest PDSCH group having the other GI value miss due to e.g., PDCCH misdetections, as the example shown in Figure 5. If the newest PDSCH group having the other GI value misses, UE can determine a number of PDSCHs in the missing PDSCH group according to an additional T-DAI value (NGI-2) accumulated for the missing PDSCH group (however, it works only if the number of PDSCHs in the missing PDSCH group is less or equal than 4), and feedback NACKs for them. However, missing of all PDSCHs in a PDSCH group rarely happens. Even the worst case happens, as we mentioned before, group-based HARQ-ACK feedback can provide multiple opportunities to correct the worst case. Thus, we don't see clear benefit to introduce the additional NGI bit and T-DAI value.

By taking account the above aspects, we see that it is feasible to indicate an NGI bit only for the scheduled PDSCH group.

Proposal 3: Group-based HARQ-ACK feedback for operation with dynamic HARQ-ACK codebook:

- New ACK-Feedback Group Indicator is indicated in a scheduling DCI only for the scheduled PDSCH group

- If a non-scheduled PDSCH group (a PDSCH group having the other GI value) is requested to be acknowledged in the same PUCCH, UE should always transmit HARQ-ACK feedback for the “newest” non-scheduled PDSCH group in the PUCCH.

Figure 5. Example of indicating the additional NGI bit (NGI-2) for a PDSCH group having the other GI value

Triggering one-shot HARQ-ACK feedback for all configured HARQ processes

In RAN1#96bis meeting [2], it was agreed to discuss whether to introduce one-shot HARQ-ACK feedback for all configured HARQ processes for a UE configured with dynamic or semi-static HARQ-ACK codebook:

Agreement from RAN1#96bis: Restrict further discussion on HARQ codebook to the following: - If request/trigger for one-shot group HARQ ACK feedback for all configured HARQ processes is introduced (at least for non-CBG HARQ), select one or more of the following candidate schemes: - The request is carried in a UE-specific DCI carrying a PUSCH grant - The request is carried in a UE-specific DCI carrying a PDSCH assignment - The request is carried in a UE-specific DCI not scheduling PDSCH nor PUSCH - The request is carried in a UE-common DCI - The request can be used for UE configured with dynamic or semi-static HARQ codebook

Triggering one-shot HARQ-ACK feedback for all configured HARQ processes is a simple and robust scheme for providing additional transmission opportunities for pended/unreported/missed HARQ-ACK feedbacks. However, corresponding HARQ-ACK codebook results in large UCI payload, especially when more than CBGs per TB are taken into account.

When UE operates with semi-static HARQ-ACK codebook, one-shot HARQ-ACK feedback can be used since robustness is more crucial in this mode rather than resource efficiency. Once UE receives a positive request in a DCI scheduling PDSCH, it shall report HARQ-ACKs for all configured HARQ processes instead of a semi-static HARQ-ACK codebook. Also, the request/trigger for one-shot HARQ-ACK feedback can be supported in a PUSCH grant to carry the large payload of HARQ-ACKs for all configured HARQ processes.

In NR-U, CBG-based retransmission becomes more important since much more variations in the time domain interference are expected. We see that CBG-based HARQ-ACK feedback should not be excluded in one-shot HARQ-ACK feedback. However, to reduce payload size, a compression scheme should be introduced.

Proposal 4: For operation with semi-static HARQ-ACK codebook, the gNB can request/trigger one-shot HARQ-ACK feedback for all configured HARQ processes.

- The request/trigger can be carried in DCI carrying PUSCH grant or PDSCH assignment

- CBG-based HARQ-ACK feedback should be supported in one-shot HARQ-ACK feedback

  - FFS: compression scheme to reduce payload size due to CBG-based HARQ-ACK feedback

When UE operates with dynamic HARQ-ACK codebook, we see that group-based HARQ-ACK feedback already provides unlimited number of HARQ-ACK transmission opportunities in a reliable and efficient way. Furthermore, DCI overhead caused by dynamic PDSCH grouping is not that high (e.g., 3 bits for group index, new group indicator, and request feedback for all groups, repetitively), one-shot HARQ-ACK feedback doesn’t show much advantage of signalling overhead. Thus, we see one-shot HARQ-ACK feedback for all configured HARQ processes is not necessary when a UE is configured in dynamic HARQ-ACK codebook.

Proposal 5: For operation with dynamic HARQ-ACK codebook, triggering one-shot HARQ-ACK feedback for all configured HARQ processes is NOT supported.

Multiple frequency domain transmission opportunities

To increase the probability for reporting a HARQ-ACK feedback as it will be subject to LBT before transmission, it is desirable to provide multiple transmission opportunities in advance. Since UE does not need to await a new PUCCH/PUSCH resource assigned by e.g., a separate DCI triggering group-based HARQ-ACK feedback, the additional latency can be avoided to facilitate more efficient HARQ procedure. Thus, at least for a carrier (or a BWP) comprising multiple 20 MHz LBT subbands, we see that providing a set of frequency domain candidate resources distributed in different 20 MHz LBT subbands for a PUCCH transmission would be beneficial. UE can perform LBT for each frequency domain candidate resource in each 20 MHz LBT subband, and selects one available candidate resource to transmit a HARQ-ACK feedback on the PUCCH.

Proposal 6: Allocation of more than one frequency domain candidate resources distributed in different 20MHz LBT subbands for a PUCCH transmission should be supported in NR-U.

In the case of a UE configured with multiple carriers, providing a set of PUCCH resources distributed in different carriers for a UCI report also provides LBT diversity. However, configuring PUCCH resources on multiple carriers would essentially establish new PUCCH cells, which leads to specification and implementation impacts, as changes are needed on resource configurations, resource allocation mechanisms, power control, UL prioritization rules, UCI multiplexing rules, etc. On the other hand, if a wideband carrier already spans multiple 20MHz LBT subbands, it can also provide LBT diversity for a UCI reportse ly establish aestion 2anowledged together H . Consider the required specification and implementation efforts, significant gains should be identified before allowing PUCCH resource configurations on SCells other than PCell/PSCell/PUCCH-SCell.

Proposal 7: Due to larger specification and implementation efforts, no need to consider PUCCH resource configurations on SCells other than PCell/PSCell/PUCCH-SCell if no significant gain is identified.

Multi-TTI PUSCH scheduling

In RAN1#97 meeting [1], the following agreement were made for multi-TTI PUSCH scheduling:

Agreement from RAN1#97: For multiple PUSCHs scheduled by a single DCI - The following fields in the DCI are signalled per PUSCH - NDI - RV (FFS: compression scheme for RV) - FFS if CBG-based re-transmission is supported for multi-TTI scheduling, at least the following options are considered for signalling the CBGTI field in the DCI - Option 1: per re-transmitted PUSCH - FFS: limitations on number of re-transmitted PUSCH for which CBGTI field is signalled - Option 2: per PUSCH - Option 3: only for a fixed number of PUSCHs - HARQ process ID signalled in the DCI applies to the first scheduled PUSCH. HARQ process ID is then incremented by 1 for subsequent PUSCHs in the scheduled order (with modulo operation as needed) - Time domain resource assignment mechanism is enhanced for indicating the number of scheduled mapping Type A and Type B PUSCHs, and their starting and ending symbols - At least continuous time domain resource assignment is supported - FFS: whether multiple mapping Type B PUSCHs is allowed within the first slot - FFS: whether multiple starting positions is allowed for UE-initiated COT (discussed as part of agenda 7.2.2.1.3) - FFS: configuration/signalling details - CSI request field in the DCI applies to a single PUSCH - FFS: relation between the timing of the triggered CSI-RS and the PUSCH carrying the CSI feedback, and how to determine which PUSCH carries the CSI feedback - FFS: Whether existing DCI formats can be extended or new formats are necessary and associated details for supporting scheduling multiple PUSCHs - FFS: Potential impact on MCS signalling for re-transmission - Note: Indication of the LBT type and priority class should be possible with the DCI - Note: SRS request field in the DCI applies to a single slot with the applicable slot signalled as in Rel-15 - Note: The number of DAI fields is not increased as compared to the single PUSCH scheduling DCI in Rel-16. Applicability of the DAI field is the same as multi-slot PUSCH scheduling in Rel-15.

In the followings, we provide our views on the enhancements to time domain resource allocation, CBGTI, and triggering aperiodic CSI report on PUSCH for multi-TTI PUSCH scheduling:

Time domain resource allocation

In NR Rel-15, time domain resource assignment field in an UL grant indicates a row of an RRC configured list for PUSCH time domain resource allocation (pusch-AllocationList). The indicated row defines a slot offset (K2), a starting symbol (S), and an allocation length (L), and a PUSCH mapping type to be applied in the PUSCH transmission. To indicate a set of continuous time domain resources in a multi-TTI grant, the existing time domain resource allocation mechanism can be reused but with different interpretations as follows:

- Slot offset (K2): The slot offset between the multi-TTI grant and the first scheduled PUSCH

- Starting symbol (S): The starting symbol of the first scheduled PUSCH.

  - If the number of PUSCHs scheduled by the multi-TTI grant is larger than one (M > 1), the ending symbol of the first scheduled PUSCH is #13.

  - If the number of PUSCHs scheduled by the multi-TTI grant is equal to one (M = 1), the ending symbol of the first scheduled PUSCH is determined according to the allocation length (L).

- Allocation length (L): The allocation length of the last scheduled PUSCH.

  - If the number of PUSCHs scheduled by the multi-TTI grant is larger than one (M > 1), the starting symbol of the last scheduled PUSCH is #0.

  - If the number of PUSCHs scheduled by the multi-TTI grant is equal to one (M = 1), the starting symbol of the last scheduled PUSCH is determined according to the starting symbol (S).

- PUSCH mapping type: The PUSCH mapping type is applied to the first scheduled PUSCH. If the number of PUSCHs scheduled by the multi-TTI grant is larger than one (M > 1), PUSCH mapping type A is always used for all the scheduled PUSCHs except the first one.

Besides, the number of scheduled PUSCHs (M) can be indicated using a separate field in a multi-TTI grant. We can see that the design described above provides flexibility to schedule one single PUSCH or multiple PUSCHs.

Figure 5. Time domain resource allocation in a multi-TTI grant

Proposal 8: When UE is scheduled to transmit one or more TBs and no CSI report, or UE is scheduled to transmit one or more TBs and a CSI report on one or more PUSCHs by a multi-TTI grant, TDFA field value m of the multi-TTI grant provides a row index m +1 to an allocated table. The indexed row defines the parameters K2, S, and L as in NR Rel-15 but with different interpretations.

Proposal 9: Introduce a separate field to indicate the number of scheduled PUSCHs in a multi-TTI grant.

In some proposals, multiple mini-slot PUSCHs can be scheduled by a multi-TTI grant at least in the beginning of an UL burst to increase channel access opportunities. Thus, additional signaling would be necessary for time domain resource assignment. However, if multiple starting positions within a PUSCH transmission is supported in NR-U, i.e., transmission is not restricted at slot boundary, we see that scheduling multiple mini-slot PUSCHs using a multi-TTI grant is not necessary, which may lead to larger DMRS overhead and number of required HARQ processes. Whether to allow multiple starting positions within a scheduled PUSCH transmission is still under discussion in agenda 7.2.2.1.3.

Proposal 10: If multiple starting positions within a scheduled PUSCH transmission is allowed in NR-U, multiple mini-slot PUSCHs scheduling using a multi-TTI grant is not necessary.

CBGTI

In NR-U, CBG-based retransmission becomes more important since much more variations in the time domain interference are expected. Moreover, if multiple starting positions within a scheduled PUSCH transmission are supported in NR-U, since some symbols may be dropped depending on LBT outcome, it is favourable to retransmit the not-transmitted or partially transmitted CBG(s) instead of the entire TB.

In the agreement for multi-TTI scheduling made in RAN1#97 meeting, at least the following three options are considered for signalling the CBGTI field in a multi-TTI grant:

- Option 1. Per re-transmitted PUSCH: Including CBGTI only for the re-transmitted PUSCH(s) avoids unnecessary overhead from CBGTI indication(s) for new transmission(s) on PUSCH(s). In order to avoid the worst case, i.e., all the PUSCHs are associated with HARQ processes that have to be retransmitted, the maximum number of re-transmitted PUSCH(s) in a multi-TTI grant should be limited. However, limiting the maximum number of re-transmitted PUSCH(s) in a multi-TTI grant may be difficult since the multi-TTI grant cannot schedule PUSCHs for arbitrary HARQ processes. Instead, as the agreement made in previous meeting, a multi-TTI grant can only schedule PUSCHs for HARQ processes with consecutive IDs.

- Option 2. Per PUSCH: This seems not feasible since DCI overhead is significantly increased if CBGTI is indicated per PUSCH. For example, a multi-TTI grant schedules up to 8 PUSCHs, and 8 CBGs per TB is configured. Then, 64 bits are required for CBGTI field in the multi-TTI grant.

- Option 3. Only for a fixed number of PUSCHs: Since CBG-based retransmission is not necessary for some cases, e.g., HARQ process without (or with a few) correctly decoded CBGs, and HARQ process with initial transmission, we can limit the maximum number of CBG-based retransmissions that can be scheduled by a multi-TTI grant (e.g., through an RRC parameter) to constraint the size of CBGTI field. In a possible enhancement, as shown in Figure 6, a separate field (e.g., CBGReTx) is included in a multi-TTI grant to inform UE which PUSCHs are scheduled with CBG-based retransmissions. In the example shown in Figure 6, the maximum number of CBG-based retransmissions in a multi-TTI grant is limited to 4, a multi-TTI grant can schedule up to 8 PUSCHs, and 8 CBGs per TB is configured. Then, up to 40 bits payload are required for CBGReTx and CBGTI fields. Note that without this enhancement, 64 bits are required for CBGTI field in option 2.

Proposal 11: Introduce following schemes to reduce CBGTI field size in a multi-TTI grant

  - CBGTI is only indicated for a fixed number of PUSCHs, where the maximum number is limited

  - Introduce a separate field in a multi-TTI grant to inform UE which PUSCHs are scheduled with CBG-based retransmissions

Figure 6. A possible enhancement to retransmission indication for NR-U multi-TTI scheduling

Aperiodic CSI report on PUSCH

In NR Rel-15, when UE receives an UL grant that requests an aperiodic CSI report (by a CSI request filed in the UL grant) on a PUSCH without data (by indicating UL-SCH field = 0), a slot offset (K2) between the UL grant and the PUSCH is determined from a list of slot offsets (reportSlotOffsetList) in the RRC configuration of the aperiodic CSI report. The stating symbol (S) and allocation length (L) for the PUSCH is provided by a row of resource allocation table (pusch-AllocationList) indicated by TDFA field in the UL grant. In NR-U, if a multi-TTI grant requests an aperiodic CSI report without data, the time domain resource allocation in NR Rel-15 (including K2, S, and L) can be directly reused, and only one PUSCH is scheduled to transmit the aperiodic CSI report.

Proposal 12: If UE receives a multi-TTI grant that requests a CSI report without data, only one PUSCH is determined as in NR Rel-15 to transmit the CSI report.

In NR Rel-15, when UE receives an UL grant that requests an aperiodic CSI report (by a CSI request filed in the UL grant) on a PUSCH with data (by indicating UL-SCH field = 1), a UCI and a TB are multiplexed together in a scheduled PUSCH. In NR-U, if a multi-TTI grant requests an aperiodic CSI report with data, a set of PUSCHs could be scheduled over multiple slots. Then, the aperiodic CSI report should be transmitted on which PUSCH? At least three options can be considered as follows:

  - Option 1. Transmit on the first scheduled PUSCH: Generally, in unlicensed band, the first scheduled PUSCH has a relatively higher probability to be blocked due to LBT failure. Therefore, it is not prefer to transmit the aperiodic CSI report on the first scheduled PUSCH.

  - Option 2. Transmit on the first available PUSCH: After LBT succeeds, UE can start to transmit the aperiodic CSI report on the first available PUSCH as soon as possible to reduce latency. However, it leads to additional burden at UE since UE has to prepare two different PUSCHs (w/ and w/o UCI) or re-generate a new PUSCH depending on LBT outcome.

  - Option 3. Transmit on the last scheduled PUSCH: The last scheduled PUSCH has a relatively higher probability to be transmitted since there are more LBT attempts before the transmission occasion. Moreover, UE doesn't have to transmit different PUSCHs depending on LBT outcome. We see that it is more feasible to transmit the aperiodic CSI report on the last scheduled PUSCH.

Proposal 13: If UE receives a multi-TTI grant that requests a CSI report with data, the CSI report should be transmitted on the last scheduled PUSCH.

Conclusion

In this contribution, HARQ operation and multi-TTI scheduling for NR-U operation were discussed. Based on the discussion in the previous sections, we made the following observations and proposals:

Cross-COT HARQ-ACK feedback and multiple HARQ-ACK opportunities

Proposals 1, 2, and 3: Group-based HARQ-ACK feedback for operation with dynamic HARQ-ACK codebook:

- Maximum # of PDSCH groups: 2 (1 bit for group index)

- Maximum # of PDSCH groups for which HARQ-ACK feedbacks can be requested in the same PUCCH: 2

- A request bit is indicated in a DCI scheduling a PDSCH belonging to a scheduled  PDSCH group:

  - Set to “0”: only the HARQ-ACK feedback of the scheduled PDSCH group is requested in the PUCCH provided by the DCI.

  - Set to “1”: the HARQ-ACK feedbacks for both PDSCH groups having different GI values are requested in the PUCCH provided by the DCI.

  - If more than one PDSCH groups are requested to be acknowledged in the same PUCCH, UE concatenates the multiple codebooks individually generated for the multiple PDSCH groups by increasing/decreasing order of corresponding group index values.

- T-DAI is included in a scheduling DCI only for the scheduled PDSCH group

- New ACK-Feedback Group Indicator is indicated in a scheduling DCI only for the scheduled PDSCH group

- If a non-scheduled PDSCH group (a PDSCH group having the other GI value) is requested to be acknowledged in the same PUCCH, UE should always transmit HARQ-ACK feedback for the “newest” non-scheduled PDSCH group in the PUCCH.

Proposal 4: For operation with semi-static HARQ-ACK codebook, the gNB can request/trigger one-shot HARQ-ACK feedback for all configured HARQ processes.

- The request/trigger can be carried in DCI carrying PUSCH grant or PDSCH assignment

- CBG-based HARQ-ACK feedback should be supported in one-shot HARQ-ACK feedback

  - FFS: compression scheme to reduce payload size due to CBG-based HARQ-ACK feedback

Proposal 5: For operation with dynamic HARQ-ACK codebook, triggering one-shot HARQ-ACK feedback for all configured HARQ processes is NOT supported.

Proposal 6: Allocation of more than one frequency domain candidate resources distributed in different 20MHz LBT subbands for a PUCCH transmission should be supported in NR-U.

Proposal 7: Due to larger specification and implementation efforts, no need to consider PUCCH resource configurations on SCells other than PCell/PSCell/PUCCH-SCell if no significant gain is identified.

Multi-TTI PUSCH scheduling

Proposal 8: When UE is scheduled to transmit one or more TBs and no CSI report, or UE is scheduled to transmit one or more TBs and a CSI report on one or more PUSCHs by a multi-TTI grant, TDFA field value m of the multi-TTI grant provides a row index m +1 to an allocated table. The indexed row defines the parameters K2, S, and L as in NR Rel-15 but with different interpretations.

Proposal 9: Introduce a separate field to indicate the number of scheduled PUSCHs in a multi-TTI grant.

Proposal 10: If multiple starting positions within a scheduled PUSCH transmission is allowed in NR-U, multiple mini-slot PUSCHs scheduling using a multi-TTI grant is not necessary.

Proposal 11: Introduce following schemes to reduce CBGTI field size in a multi-TTI grant

  - CBGTI is only indicated for a fixed number of PUSCHs, where the maximum number is limited

  - Introduce a separate field in a multi-TTI grant to inform UE which PUSCHs are scheduled with CBG-based retransmissions

Proposal 12: If UE receives a multi-TTI grant that requests a CSI report without data, only one PUSCH is determined as in NR Rel-15 to transmit the CSI report.

Proposal 13: If UE receives a multi-TTI grant that requests a CSI report with data, the CSI report should be transmitted on the last scheduled PUSCH.

- References

- 3GPP RAN1 #97, “Chairman’s Notes RAN1 #97”, Reno, USA, May 2019.

- 3GPP RAN1 #96bis, “Chairman’s Notes RAN1 #96bis”, Xi’an, China, April 2019.