3GPP TSG RAN WG1 Meeting RAN1 #98                                                                         R1-1908392

Prague, CZ, August 25th – 30th, 2019

Agenda item: 7.2.12.1

Source: MediaTek Inc.

Title: Physical layer feasibility considerations for mobility enhancement

Document for: Discussion and Decision

1	Introduction

In RAN1 #96 [1] for mobility enhancement, it has been agreed that RAN1 focus on the potential physical layer aspects of RACH-less HO, dual connectivity (DC) based HO, Make-before-break (MBB) based HO, and possible lower-layer mobility enhancements.

In RAN1 #96-bis [2] for mobility enhancement, RAN1 agreed a reply LS (R1-1905780) [3] to RAN2 on NR mobility enhancements about feasibility of simultaneous transmission and receptions.

In RAN1 #97 [4] for mobility enhancement, the following conclusions are drawn:

Conclusion:

- RACH-less HO in NR can at least support TA scenarios where target cell TA is zero or same as one of serving cell TAs in FR1

  - FFS: RACH-less HO in FR2 including feasibility

  - FFS: whether to indicate a different TA reference for a different SS/PBCH block or CSI-RS resource

- For FR1 intra-frequency HO, further study whether any enhancement on determining TA compared to LTE RACH-less HO is needed and feasible. The following options can be used as a starting point:

  - Option 1: Network indicates a timing refinement factor to UE so that UE can use at least the indicated timing refinement factor to adjust the target cell TA from the source cell TA.

  - Option 2: UE performs an autonomous TA adjustment

  - Option 3: Network indicates the target cell TA that is estimated by the target cell based on SRS transmission to the source cell

  - FFS the case of FR1 inter-frequency HO & FR2

- Note: RACH-less alone is not able to achieve 0ms handover interruption time. RAN2 is expected to make the decision whether RACH-less HO is supported in NR.

Conclusion:

- On UL grant for PUSCH transmission in RACH-less HO, further study whether any enhancement for UL grant indication compared to LTE RACH-less HO is needed.

Conclusion:

- RAN1 expects similar physical layer specification impact for make-before-break (MBB) based HO enhancements and DC-based HO enhancements.

- For both DC-based and MBB based HO that is feasible in scenarios identified in R1-1905780,

  - it is expected that UE can receive:

    - PDCCH from both source and target cells.

    - PDSCH from both source and target cells.

    - FFS: BWP and CORESET configurations, semi-static PDCCH configuration for source and target cells, PDCCH blind decoding budgets, etc.

  - it is expected that UE can transmit:

    - Multiple PUCCH for HARQ-ACK to both source and target cells.

    - FFS: whether the transmission is TDM or other manners.

    - FFS: multiple PUSCH to both source and target cells.

    - FFS: Power Control related aspects.

  - Further study dependence of simultaneous transmission and/or reception on UE capability, if any.

- Continue to discuss the feasibility of DC/MBB-based HO for scenarios listed in R1-1905780 where RAN1 has not concluded on feasibility.

In this contribution, we further elaborate mobility enhancement issues and solutions.

2	Interruption Reduction Analysis

In LTE/NR, The latency during handover (HO) execution is defined as the interruption from reception of RRCReconfiguration (HO command) to the transmission of RRCReconfigurationComplete as shown in Fig. 1. To reduce the interruption time, RACH-less handover is mentioned in RAN1 #96 [1] while another method “2-step RACH for NR” was agreed as a new work item in RAN#82 [5]. In RAN1 #96 [1] for mobility enhancement, dual connectivity (DC) based and make-before-break (MBB) based handover which requires simultaneous Tx/Rx operation with the source cell and target NR cell during HO/SCG change are also discussed.

Figure 1: Service interruption time in NR HO.

Table 1: Handover interruption time components

| Component/ Step | Description | Time (ms) |
| --- | --- | --- |
| 7 | RRC Reconfiguration Incl. ReconfigurationwithSync | 10 |
| 8 | SN Status Transfer | 0 |
| 9.1 | Target cell search | 0 |
| 9.2 | UE processing time for RF/baseband re-tuning, security update | 20/40 |
| 9.3 | Delay to acquire first available PRACH in target eNB | 10 (10+10*x) |
| 9.4 | PRACH preamble transmission | 1 |
| 9.5 | Fine time tracking and acquiring full timing information | 5/10(SMTC periodicity) |
| 10 | UL Allocation + TA for UE | 3/5 |
| 11 | UE sends RRC Connection Reconfiguration Complete | 6 |
|  | Minimum/Typical Total delay [ms] | 55ms/82ms |

The interruption reduction is analyzed as in Table I [6] where contention free RACH is assumed for HO. It can be seen that 14 out of 55 ms interruption can be saved for RACH-less handover and 6 out of 55 ms can be saved for 2-step RACH based HO.  Using 2-step RACH as a baseline, RACH-less HO can save 8 out of 49ms interruption.

Observation 1: In NR, without considering feasibility, RACH-less HO saves about 16% of interruption time using 2-step RACH as a baseline, while DC-based and enhanced MBB based handover can achieve 0ms interruption time.

3	Feasibility of RACH-less Handover

For RACH-less HO, the RAN1 impacts are:

Observation 2: RAN1 impacts on RACH-less HO is how can UE obtain UL grant, power control, timing advance, and UL beam pair selection for initial PUSCH transmission.

For UL grant, in the RAN4 reply LS [7], it is preliminarily observed in RAN4 that “if the source cell can configure multiple UL grant resources associated with different DL RS (SSB or CSI-RS) in target cell, handover successful rate can be increased. Note this also increase resource overhead in the target cell. However, the gain in handover successful rate over the resource overhead may need further study. RAN2 can decide if to support this”. This observation matches previous agreement in RAN1 #97 that further study whether any enhancement for UL grant indication compared to LTE RACH-less HO is needed. Since using multiple UL grants with different DL RS (SSB or CSI-RS) on multiple beams requires much resource overhead, it should be discussed after some companies perform simulation to justify the handover successful rate gain in various channel scenarios.

Proposal 1: Configuring multiple UL grant resources associated with different DL RS (SSB or CSI-RS) in target cell during RACH-less handover should be discussed after some companies perform simulation to justify the handover successful rate gain.

For power control, RAN1 had an agreement in [8] for LTE RACH-less handover that PUSCH (re)transmissions follow the same power control procedures (power ramping) as in RACH process. The parameters which do not exist for RACH-less HO are set to be zero ( ΔPrampuprequested,c=δmsg2,c=0).

Proposal 2: For NR RACH-less HO, similar approach as [8] where PUSCH (re)transmissions follow the same power control procedures (power ramping) as in RACH process with parameters not exist for RACH-less HO set to zero can be discussed.

For TA, LTE RACH-less HO supports only synchronous deployments with the limitation target cell TA = 0 or source cell TA = target cell TA. To generalize the use of RACH-less HO in more deployment scenarios, UE would be required to perform TA measurement at some time, ex., during SMTC window. The SMTC window duration may need to be adjusted if TA measurements for neighbor cells in various frequency layers are to be performed in SMTC window.

Observation 3: The requirement of TA measurement in SMTC window by UE may have RAN4 impact since there is no requirement for TA measurement in current RAN4 spec.

For UE based TA measurement in LTE network for RACH-less HO, RAN4 replied in R4-166817 [9] and mentioned that the legacy accuracy requirements for uplink timing alignment, i.e. the accuracy of TA for initial uplink transmission, cannot be met by UE calculated TA scheme in either synchronous or asynchronous network. For UE based TA measurement in NR network for RACH-less HO, RAN4 replied in R4-1904826 [10] that RACH-less handover for NR with zero or equal TA on FR1 is feasible for intra and inter frequency in synchronous and asynchronous scenarios, while the feasibility is FFS for different TA or FR2. In the RAN4 LS reply [7], it is mentioned that RACH-less handover with 0 or equal TA is feasible in FR2 if the source and target cells are well synchronized. But it is up to RAN2 to decide if the handover can be supported in this work item.

Observation 4: According to current RAN4 reply, RACH-less HO is feasible with target cell TA = 0 or source cell TA = target cell TA in FR1 and FR2. It is not clear why FR2 is required to be well synchronized (synchronous) to support RACH-less HO while FR1 is not.

For different TA, the mechanism for UE to obtain timing advance of the target cell can be studied. For synchronous network, the TA needs to be calculated by UE with measurements of the reference signals timing transmitted by source cell and target cell as shown in Fig. 2. Target cell TA can be derived based on source cell TA and the difference between TRX_S and TRX_T for (perfectly) synchronous network.

Figure 2: Propagation delay of source cell and target cell.

In 38.133 [11] 7.4.2, it is specified that the cell phase synchronization accuracy measured at BS antenna connectors shall be within 3μs. There could be up to 3μs uncertainty between cell timings for synchronous network. This also means 3μs uncertainty for UE calculated TA value. For NR, 3μs TA uncertainty can exceed CP length for subcarrier spacing above 30kHz as shown in Table II.

Table II: OFDM CP length for different subcarrier spacing

| Parameter / Numerology (µ) | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| Subcarrier Spacing (kHz) | 15 | 30 | 60 | 120 | 240 |
| OFDM Symbol with CP (us) | 66.67 | 33.33 | 16.67 | 8.33 | 4.17 |
| OFDM Symbol Duration (us) | 71.43 | 35.71 | 17.86 | 8.93 | 4.46 |
| Cyclic Prefix Duration (us) | 4.76 | 2.38 | 1.19 | 0.60 | 0.29 |

Observation 5: For synchronous network, the cell phase offset between source cell and target cell may be required for UE to do TA estimation for RACH-less HO. The behavior to acquire the offset between source and target cells is up to RAN3 to determine.

For RACH-less handover in asynchronous network, the UE based TA calculation would also require the timing offset between source and target gNBs expressed as “D” in Fig. 2. The behavior to acquire the timing offset between source and target gNBs is not defined in current spec and is up to RAN3 to determine, as stated in [9].

Observation 6: The feasibility of TA estimation for RACH-less HO in asynchronous network requires RAN3’s input.

In RAN1 #97 [4], it is drawn in the conclusion that: For FR1 intra-frequency HO, further study whether any enhancement on determining TA compared to LTE RACH-less HO is needed and feasible. The following options can be used as a starting point:

- Option 1: Network indicates a timing refinement factor to UE so that UE can use at least the indicated timing refinement factor to adjust the target cell TA from the source cell TA.

- Option 2: UE performs an autonomous TA adjustment

- Option 3: Network indicates the target cell TA that is estimated by the target cell based on SRS transmission to the source cell

With observations 5 and 6, even if UE wants to perform an autonomous TA adjustment as in Option 2, it still requires NW’s help to provide assistant timing information. In this case, it would make more sense to consider Option 1 or Option 3.

Observation 7: Among the 3 options to determining TA raised in RAN1 #97

- Option 1: Network indicates a timing refinement factor to UE so that UE can use at least the indicated timing refinement factor to adjust the target cell TA from the source cell TA.

- Option 2: UE performs an autonomous TA adjustment

- Option 3: Network indicates the target cell TA that is estimated by the target cell based on SRS transmission to the source cell

Option 1 and Option 3 makes more sense since in Option 2 UE still requires Network’s help as stated in observations 5 and 6.

For UL beam pair selection, there is no chance for UE to perform UL beam pair identification without RACH process and may have to reuse the beam sweeping results from RRM measurement. Fig. 3 shows the RRM measurement period for one measurement target.

Figure 3: Cell search (TCS), measurement (TCM), and SSB index search (TSBI) period define in 38.133 [11], unit in ms.

Observation 8: Due to the long measurement period in FR2 for each measurement target, the beam sweeping results from RRM can be outdated for UL beam selection in RACH-less handover. Further enhancement for this issue may be required.

4	Feasibility of DC-based or EMBB-based Handover

For DC-based and enhanced MBB based HO, according to [12] (R2-1900619), for split or non-split bearer operations, the two methods both keep the source link until a UE can receive control/data from the target cell and requires simultaneous DL/UL.

Observation 9: From RAN1’s perspective, DC-based HO and enhanced Make-Before-Break HO have the same level of specification impact.

Previously RAN2 inquired the feasibility of DC-based HO for NR UE to RAN1 and RAN4. RAN1 replied in R1-1905780 [3] and RAN4 replied in R4-1904826 [10]. Based on the replies, we can classify three categories based on the capability to perform simultaneous DL/UL and determine whether DC-based HO should be supported.

Proposal 3: Classify the scenarios to perform simultaneous DL/UL into three different capability categories:

- Category 1: feasible to support simultaneous Tx/Rx with the source and target cells during HO;

- Category 2: feasible to support only simultaneous Rx with the source and target cells during HO;

- Category 3: no support of simultaneous Tx/Rx with the source and target cell during HO.

Proposal 4: DC-based HO is supported for the category 1 scenarios, e.g., inter-frequency synchronous inter-band/intra-band, inter-frequency asynchronous inter-band and intra-frequency synchronous.

Proposal 5: DC-based HO is supported for the category 2 scenarios, e.g. intra-frequency asynchronous or single UL transmission. TDM pattern for UL transmission is coordinated between the source and target cells.

Proposal 6: DC-based HO doesn't need to be supported for the category 3 scenarios.

For the procedure of DC-based HO, as discussed in [12], it is based on LTE Rel-12 DC procedure to add/release SeNB as shown Fig. 4 [12]. In Step 7 of Fig. 4, UE needs to perform random access to the target cell (SeNB) and still maintain link to the source cell (MeNB). This requires UE to perform simultaneous UL. According to the power control mechanism in Rel15 for EN-DC (38.213 7.6.1 [13])

the target cell is treated as SCG and the priority of power sharing is lower than the source cell (MCG). However, since it is more important to ensure a robust handover to the target cell, the target cell should be prioritized in this scenario.

Proposal 7: For UE capable of dynamic power-sharing for simultaneous UL, SCG (target cell) UL power is prioritized over MCG (source cell) for the case of DC-based handover. If UE transmissions of the source cell and target cell overlap in time and the transmitted power exceeds total power constraint, UE reduces transmission power of source cell until the total UE power constraint is met. If the reduced power amount of transmission for source cell exceeds a configured threshold, UE drops the transmission for source cell.

For some scenario, UE may be configured to perform single uplink operation (SUO) between source cell and target cell (Ex. For UEs which support only simultaneous Rx with the source and target cells) to do DC-based HO. This would require a UL TDM pattern for both source cell and target cell to coordinate UE’s transmission to both source and target gNBs. How to configure these TDM patterns can be FFS. To maximize the UL scheduling flexibility among MCG (source cell) and SCG (target cell), some UL slots can be scheduled by both CGs (Ex. For UEs capable of dynamic power sharing which can robustly handle the situation where network unexpectedly schedules simultaneous UL transmissions on CGs). In this case, if there is a UL collision, UE should drop the UL for source cell to prioritize the target cell UL transmission.

Proposal 8: For UE performing single uplink operation (SUO) during DC-based HO, if there is a UL collision for source cell and target cell due to network scheduling, UE drops the UL for source cell to prioritize the target cell UL transmission.

Figure 4: An example message flow of DC-based HO [10].

5	Feasibility of Lower-Layer Mobility Enhancements

In [14], three lower-layer mobility enhancements mechanisms are mentioned:

(1). L1-RSRP reporting on SSB and CSI-RS for neighbor cell

(2). MAC CE based updates of QCL source for neighbor cell

(3). Beam failure recovery on neighbor cell

For (1), (2), (3), they are originally used for beam management and only defined for serving cell.

For (1), it’s doable from configuration point of view. However, there is some concern for UE measurement to enable beam management cross different cells. Those RSs configured for beam management may be different from the RS configured for RRM. Thus, L1-RSRP measurement for beam management may have different measurement requirement for the L1-filtered measurement results, which has no requirement in RAN4 and left to UE implementation.

Observation 9: L1-RSRP reporting on SSB and CSI-RS for neighbor cell may have RAN4 impact since beam management measurement for neighbor cell has no requirement in current RAN4 spec.

For (3), it is claimed in [14] that there is no RAN2 impact. However, this procedure may have impact to higher layer, since UE context needs to be relocated and security needs to be updated. Furthermore, the benefit of reduction on HO interruption time needs further investigation. Because UE context relocation from the source cell to the target cell, security update etc., for HO will involve both higher layer procedures and the interaction between source cell and target cell over Xn interface.

Observation 10: Beam failure recovery on neighbor cell should be evaluated by RAN2 due to higher layer procedures and the interaction between source cell and target cell over Xn interface.

Besides, to enable beam failure recovery on neighbor cell, UE also needs to obtain UL grant, power control, and timing advance of target cell. This part of discussion follows the one for RACH-less handover.

Observation 11: For beam failure recovery on neighbor cell, UE needs to obtain UL grant, power control, and timing advance of target cell like RACH-less handover.

6	Summary

In this invention, we investigate NR mobility enhancement with various considerations. In particular, we have:

Observation 1: In NR, without considering feasibility, RACH-less HO saves about 16% of interruption time using 2-step RACH as a baseline, while DC-based and enhanced MBB based handover can achieve 0ms interruption time.

Observation 2: RAN1 impacts on RACH-less HO is how can UE obtain UL grant, power control, timing advance, and UL beam pair selection for initial PUSCH transmission.

Proposal 1: Configuring multiple UL grant resources associated with different DL RS (SSB or CSI-RS) in target cell during RACH-less handover should be discussed after some companies perform simulation to justify the handover successful rate gain.

Proposal 2: For NR RACH-less HO, similar approach as [8] where PUSCH (re)transmissions follow the same power control procedures (power ramping) as in RACH process with parameters not exist for RACH-less HO set to zero can be discussed.

Observation 3: The requirement of TA measurement in SMTC window by UE may have RAN4 impact since there is no requirement for TA measurement in current RAN4 spec.

Observation 4: According to current RAN4 reply, RACH-less HO is feasible with target cell TA = 0 or source cell TA = target cell TA in FR1 and FR2. It is not clear why FR2 is required to be well synchronized (synchronous) to support RACH-less HO while FR1 is not.

Observation 5: For synchronous network, the cell phase offset between source cell and target cell may be required for UE to do TA estimation for RACH-less HO. The behavior to acquire the offset between source and target cells is up to RAN3 to determine.

Observation 6: The feasibility of TA estimation for RACH-less HO in asynchronous network requires RAN3’s input.

Observation 7: Among the 3 options to determining TA raised in RAN1 #97

- Option 1: Network indicates a timing refinement factor to UE so that UE can use at least the indicated timing refinement factor to adjust the target cell TA from the source cell TA.

- Option 2: UE performs an autonomous TA adjustment

- Option 3: Network indicates the target cell TA that is estimated by the target cell based on SRS transmission to the source cell

Option 1 and Option 3 makes more sense since in Option 2 UE still requires Network’s help as stated in observations 5 and 6.

Observation 8: Due to the long measurement period in FR2 for each measurement target, the beam sweeping results from RRM can be outdated for UL beam selection in RACH-less handover. Further enhancement for this issue may be required.

Observation 9: From RAN1’s perspective, DC-based HO and enhanced Make-Before-Break HO have the same level of specification impact.

Proposal 3: Classify the scenarios to perform simultaneous DL/UL into three different capability categories:

- Category 1: feasible to support simultaneous Tx/Rx with the source and target cells during HO;

- Category 2: feasible to support only simultaneous Rx with the source and target cells during HO;

- Category 3: no support of simultaneous Tx/Rx with the source and target cell during HO.

Proposal 4: DC-based HO is supported for the category 1 scenarios, e.g., inter-frequency synchronous inter-band/intra-band, inter-frequency asynchronous inter-band and intra-frequency synchronous.

Proposal 5: DC-based HO is supported for the category 2 scenarios, e.g. intra-frequency asynchronous or single UL transmission. TDM pattern for UL transmission is coordinated between the source and target cells.

Proposal 6: DC-based HO doesn't need to be supported for the category 3 scenarios.

Proposal 7: For UE capable of dynamic power-sharing for simultaneous UL, SCG (target cell) UL power is prioritized over MCG (source cell) for the case of DC-based handover. If UE transmissions of the source cell and target cell overlap in time and the transmitted power exceeds total power constraint, UE reduces transmission power of source cell until the total UE power constraint is met. If the reduced power amount of transmission for source cell exceeds a configured threshold, UE drops the transmission for source cell.

Proposal 8: For UE performing single uplink operation (SUO) during DC-based HO, if there is a UL collision for source cell and target cell due to network scheduling, UE drops the UL for source cell to prioritize the target cell UL transmission.

Observation 9: L1-RSRP reporting on SSB and CSI-RS for neighbor cell may have RAN4 impact since beam management measurement for neighbor cell has no requirement in current RAN4 spec.

Observation 10: Beam failure recovery on neighbor cell should be evaluated by RAN2 due to higher layer procedures and the interaction between source cell and target cell over Xn interface.

Observation 11: For beam failure recovery on neighbor cell, UE needs to obtain UL grant, power control, and timing advance of target cell like RACH-less handover.

7	Reference

Chairman’s Notes, RAN1 #96

Chairman’s Notes, RAN1 #96-bis

R1-1905780, Reply LS on NR mobility enhancements, RAN1 #96-bis

Chairman’s Notes, RAN1 #97

RP-182894, New WID: 2-step RACH for NR, 3GPP TSG RAN#82

R2-1906805, General Consideration on HO/SCG Interruption in NR, MediaTek, RAN2 #106

R4-1907730, LS on NR mobility enhancement, RAN4 #91

R1-1903313, Power control for RACH-less HO, Qualcomm Incorporated, RAN1 #96

R4-166817, Reply LS on the feasibility of mobility enhancement solutions, RAN4 #80

R4-1904826, Reply LS on NR mobility enhancement, RAN4 #90bis

3GPP TS 38.133, Requirements for support of radio resource management V15.4.0

R2-1900619, Report from [104#61][LTE/feMOB]  Solution directions for minimizing user data interruption for UL/DL, Nokia, RAN2 #105

3GPP TS 38.213, NR; Physical layer procedures for control V15.5.0

R1-1905162, Ericsson, Details on lower-layer mobility enhancements