3GPP TSG RAN1 WG1 Meeting #98		R1-1908402

Prague, CZ, August 26th – 30th, 2019

Agenda item:	7.2.4.7

Title:	Support of NR-Uu controlling LTE sidelink

Source:	MediaTek Inc.

Document for:	Discussion and Decision

# Introduction

In RAN1#97, the following agreements were made on NR-Uu controlling LTE sidelink [1].

Agreements: - RRC-based activation/deactivation is not supported - DCI-based activation/deactivation is supported - Support of LTE PC5 scheduling by NR Uu (mode 3-like ) is based on UE capability - NR DCI provides the fields of DCI 5A in LTE-V that are related to SPS scheduling - The size of DCI for activation/deactivation is one of the DCI size(s) that will be defined for NR Uu scheduling NR V2V - FFS whether the DCI format is the same as one of the DCI formats that will be defined for NR Uu scheduling NR V2V - Activation/deactivation applies to the first LTE subframe after Z+X ms after receiving the DCI - Z is the same timing offset in current LTE V2X specs - X>0. FFS value(s) of X, and if one or multiple values of X are possible

In this contribution we discuss remaining details on DCI-based activation/deactivation in LTE sidelink when it is controlled by NR Uu.

# NR DCI format for LTE SL-SPS activation/deactivation

One of the open issues is whether the NR DCI that activates/deactivates for LTE sidelink Mode-3 SPS is based on a new DCI format or based on the same DCI format that will be defined for NR sidelink Mode-1 RA.

In our view, it is preferable to reuse the same DCI format. Introducing another DCI format for LTE sidelink control would increase UE blind detection complexity. Instead, a new RNTI can be used to CRC scramble the same DCI format. UEs that has the capability to support DCI-based LTE sidelink SPS activation/deactivation by NR-Uu control can be configured with this new RNTI. Also, the payload size of the DCI can be aligned to one of the DCI sizes by zero padding that will be defined for NR-Uu scheduling NR sidelink Mode-1.

We have the following proposal:

Proposal 1: Reuse the same DCI format for NR-Uu scheduling NR-SL Mode-1 with a different RNTI. Use zero padding to align DCI payload size to existing DCI format 0_0/1_0.

# Inter-RAT information exchange delay

DCI-based activation/deactivation for LTE sidelink Mode-3 SPS has been agreed as UE capability. The information signaled in DCI needs to be exchanged from the NR module to LTE module via some proprietary connection (see Figure 1). Since inter-module exchange highly depends on  UE implementation, in our view it is difficult to define a universal minimum timing value (i.e., X ms) for all UE.

In addition, V2X services that are supported in LTE sidelink are not subject to low latency requirement. The most strict latency requirement in LTE V2X is around 10ms-20ms. Therefore, the inter-RAT information exchange delay is not likely to impact LTE V2X service.

Figure 1:  NR-Uu activating/releasing LTE sidelink Mode-3 SPS.

Since DCI-based activation is defined as UE capability, it is reasonable to allow each UE report its inter-module information exchange delay in its capability report.

We have the following proposal:

Proposal 2: UE reports its inter-module information exchange delay timing (i.e., value of X) to network.

# Conclusions

We have the following proposals:

Proposal 1: Reuse the same DCI format for NR-Uu scheduling NR-SL Mode-1 with a different RNTI. Use zero padding to align DCI payload size to existing DCI format 0_0/1_0.

Proposal 2: UE reports its inter-module information exchange delay timing (i.e., value of X) to network.

# References

- Chairman’s notes, RAN1#97, Reno, Nevada, May 2019.