3GPP TSG-RAN WG1 #98 Meeting	R1-1908407

Prague, Czech Republic, 26th August – 30th August, 2019

Agenda item:		7.2.10.4

Source:	MediaTek Inc.

Title:	Procedure design for NR positioning

Document for:		Discussion

# Introduction

The procedure design for positioning has reached the following agreements in previous meeting,

Agreement: For intra-frequency measurements, when measurement gaps are configured, the UE is expected to measure the DL PRS resource outside the configured (active and inactive) BWPs (Option 2 in the related Agreement from RAN1 #96-Bis). - Note: The UE has flexibility to measure only on a part of the DL PRS configured bandwidth as long as measurement requirements are met. Agreement: For positioning purposes, to assist UE to perform Rx beamforming, the following options are supported (options are from previous related agreement in RAN1#96bis): - Option 1: The DL PRS can be configured to be QCLed Type D with a DL Reference Signal from a serving or neighboring cell. - Discuss further which DL reference signal can be used (e.g., SSB, CSI-RS, DL-PRS). - Option 2: The UE may perform a Rx beam sweeping on DL PRS resources which are transmitted with the same downlink spatial domain transmission filter. - Option 3: The UE may use a fixed Rx beam to receive DL PRS resources which are transmitted with different downlink spatial domain transmission filter. - FFS: Whether this option can be achieved by Option 1 - FFS on specification impacts. Agreement: For positioning purposes, with regards to the UL SRS transmission power, support at least the following in addition to the existing Rel-15 behaviour: - Support configuring a DL reference signal of a neighbouring cell to be used as DL path loss reference for the purpose of SRS power control. - FFS: Which reference signals can be used (e.g. a CSI-RS, or a SSB, or DL PRS). - Study further the UE configuration signalling and procedure. - FFS on a fallback procedure if the UE is not able to obtain the pathloss reference. - FFS on number of measurements for pathloss. Agreement: For positioning purposes, for UL Beam management/alignment towards serving and neighbouring cells, the following is supported (in addition to UE TX beam sweeping): - Configuration of a spatial relation between a reference DL RS from serving or neighbouring cells and the target SRS. - Reference DL RS that can be used include at least SSB. FFS on CSI-RS, DL-PRS. - A fixed Tx beam for UL SRS transmissions across multiple UL SRS Resources, for both FR1 and FR2. - FFS how the fixed beam is selected. - FFS on specification impacts. - Note: In Rel-16, UE is not expected to transmit multiple SRS resources with different spatial relations in the same OFDM symbol (Rel-15 behaviour)

In this contribution, we express our view for the remaining issues.

# UE reception procedure

## 2a Intra-frequency measurement without gaps

According to current agreement, the UE only measures PRS within the active BWP when there is no measurement gap. The UE may measure PRS outside the configured BWP within the measurement gap.

We have raised the concern since in previous meeting that,

- The BWP switching for SSB based intra-frequency measurement could be different from that for PRS based intra-frequency measurement

- If the transmission occasion of PRS from a cell is not perfectly aligned with the measurement gap, which is UE specific, the measurement and reporting delay needs to be further extended

For SSB based intra-frequency measurement, as long as SSB is not completely within the active BWP, the measurement is performed within the measurement gaps. It is due to the need of BWP switching of the UE. When SSB is completely within the active BWP, the measurement can be performed without the gaps.

BWP switching includes, the adjustment of center frequency and/or the bandwidth adaptation. The bandwidth adaptation and without the adjustment of center frequency may require a shorter time period. In R4-1702029, there is the following observation,

– Transition time (RF aspects) • For intra-band operation, at least for sub6, the transition time can be up to 20 µs if the center frequency is the same before and after the bandwidth adaptation, regardless other conditions listed in the LS

PRS would be configured with wider transmission bandwidth than SSB. For PRS based intra-frequency measurement, it is very possible that the active BWP may not fully cover all the PRS transmission bandwidth, and the BWP switching in this case actually doesn’t need to adjust the center frequency, instead only the RF filter bandwidth adjustment (bandwidth adaption) is needed. The new BWP for PRS measurement also includes the original (active) BWP BW.

There is a case that when only one BWP is configured to the UE, and when the BWP switching is through the RRC re-configuration. Then, it is possible that the DL active BWP is small before the PRS measurement.

When only a small time period for transition is needed for bandwidth adaptation without the center frequency change, we don’t see the particular need that a larger PRS bandwidth measurement may only be performed within the measurement gap.

Assigning more measurement objects to be measured within the measurement gaps may lead to the increase of measurement gap occasions by shortening the periodicity and/or the increase of the reporting delay of the measurement. Note that the network may only provide a single per-UE measurement gap pattern for concurrent monitoring of all frequency layers.

So we would like to consider PRS based intra-frequency measurement without gaps by incorporating the short transition time for bandwidth adaption without center frequency change.

An example is given in Fig. 2a-1, showing that when performing the neighbouring cell PRS measurement in slot N, the PDSCH duration in slot N-1 can be shortened so that the UE may have one or two symbol time for bandwidth adaption. If there is PDSCH scheduling in slot N+1, the type-B PDSCH transmission can be applied so that the UE may have one or two symbol time to switch back to the original active DL BWP.

Figure 2a-1,

Observation 2a-1: BWP adjustment includes, the adjustment of center frequency and/or the bandwidth adaptation

Observation 2a-2: When only a small time period for transition is needed for bandwidth adaptation without the center frequency change, we don’t see the particular need that a larger PRS bandwidth measurement may only be performed within the measurement gap

Observation 2a-3: According to 38.133, the network must provide a single per-UE measurement gap pattern for concurrent monitoring of all frequency layers if the UE does not support independent measurement gap patterns for different frequency ranges

Observation 2a-4: Assigning more measurement objects to be measured in the measurement gaps may lead to the increase of measurement gap by shortening the periodicity and/or the longer reporting delay of the measurement

Proposal 2a-1: For measuring wider PRS transmission bandwidth than active DL BWP, consider the mechanism to facilitate PRS based intra-frequency measurement without gaps by incorporating a short transition time for bandwidth adaption without center frequency change

## 2b Beam management aspect

In previous meeting, there is the following agreement related to the DL beam management,

Agreement: For positioning purposes, to assist UE to perform Rx beamforming, the following options are supported (options are from previous related agreement in RAN1#96bis): - Option 1: The DL PRS can be configured to be QCLed Type D with a DL Reference Signal from a serving or neighboring cell. - Discuss further which DL reference signal can be used (e.g., SSB, CSI-RS, DL-PRS). - Option 2: The UE may perform a Rx beam sweeping on DL PRS resources which are transmitted with the same downlink spatial domain transmission filter. - Option 3: The UE may use a fixed Rx beam to receive DL PRS resources which are transmitted with different downlink spatial domain transmission filter. - FFS: Whether this option can be achieved by Option 1 - FFS on specification impacts

The three options are all supported. For option 1, we have expressed our strong concern that, to configure type-D QCL for PRS, at least the coarse timing should have been identified. Therefore, if a DL RS can be type-D source RS for PRS, the same DL RS should be able to simultaneously serve as type-C source RS for PRS as well.

This is the reason we think that the description of option 1 is not complete. We suggest to re-word option 1 by the following,

Proposal 2b-1: Modify option 1 as “The DL PRS can be configured to be QCLed Type C and Type D with a DL Reference Signal from a serving or neighboring cell”

Option 3 is the UE behaviour to realize the DL angle based measurement. Fig. 2b-1 shows the example, where each UE uses the same RX beam to receive multiple TX beams for RSRP measurement, and the measured number of RSRPs provides the signature to identify the direction between the TP to the UE.

The behaviour of “use the same RX beam to receive multiple TX beams for RSRP measurement” needs to be described in the specification. For the UE to be able to perform RSRP measurement, a coarse wide RX beam has been formed for reception. The basic timing search and energy detection have also been done. So the UE has known which TX beam can be received with better power.

For the following multiple-beam RSRP measurement, the RX beam needs to at least receive the best TX beam which is identified in previous stage. Different UE may see different best TX beam, and it may not be possible that all the UEs can detect all the TX beams.

We consider the following wordings to describe the behaviour,

- For DL-AoD measurement, the UE shall not assume that the PRS resources for measurement are transmitted with the same downlink spatial domain transmission filter. The UE may also assume that the PRS resources for measurement are quasi co-located with 'QCL-TypeD' for each other

Fig. 2b-1,

Proposal 2b-2: To address the behaviour of “use the same RX beam to receive multiple TX beams for RSRP measurement”, the following wordings are proposed: “For DL-AoD measurement, the UE shall not assume that the PRS resources for measurement are transmitted with the same downlink spatial domain transmission filter. The UE may also assume that the PRS resources for measurement are quasi co-located with 'QCL-TypeD' for each other”

# Conclusion

Based on the above, we have,

Observation 2a-1: BWP adjustment includes, the adjustment of center frequency and/or the bandwidth adaptation

Observation 2a-2: When only a small time period for transition is needed for bandwidth adaptation without the center frequency change, we don’t see the particular need that a larger PRS bandwidth measurement may only be performed within the measurement gap

Observation 2a-3: According to 38.133, the network must provide a single per-UE measurement gap pattern for concurrent monitoring of all frequency layers if the UE does not support independent measurement gap patterns for different frequency ranges

Observation 2a-4: Assigning more measurement objects to be measured in the measurement gaps may lead to the increase of measurement gap by shortening the periodicity and/or the longer reporting delay of the measurement

Proposal 2a-1: For measuring wider PRS transmission bandwidth than active DL BWP, consider the mechanism to facilitate PRS based intra-frequency measurement without gaps by incorporating a short transition time for bandwidth adaption without center frequency change

Proposal 2b-1: Modify option 1 as “The DL PRS can be configured to be QCLed Type C and Type D with a DL Reference Signal from a serving or neighboring cell”

Proposal 2b-2: To address the behaviour of “use the same RX beam to receive multiple TX beams for RSRP measurement”, the following wordings are proposed: “For DL-AoD measurement, the UE shall not assume that the PRS resources for measurement are transmitted with the same downlink spatial domain transmission filter. The UE may also assume that the PRS resources for measurement are quasi co-located with 'QCL-TypeD' for each other”