3GPP TSG RAN WG1 Meeting #98 	R1-1908409

Prague, Czech, 26th – 30th August, 2019

Agenda Item: 7.2.6.2

Source: MediaTek Inc.

Title: Multiple HARQ procedures and intra-UE UCI prioritization

Document for: Discussion and Decision

# Introduction

The first part of the contribution:

- addresses the complementary, simultaneous use of HARQ codebooks and codebook-less HARQ feedback. We argue that this would allow keeping the sub-slot size as large as possible (slot or half-slot) for flexible HARQ multiplexing, while meeting the latency requirements with corner case traffic patterns as well. The solution optimizes downlink and uplink spectral efficiency and potentially simplifies traffic prioritization, too.

- compares solution options for the dynamic indication of HARQ-ACK codebook.

- Addresses pending issues (agreed FFS) on dynamic signalling and PUCCH allocation

The second part of the contribution addresses intra-UE traffic prioritization, presenting:

- the solutions summarized in Table 1, along with a comparison to alternative options

- an analysis of available scheduler actions in specific scenarios

- a HARQ-ACK recovery method that uses the skeleton of A-CSI report scheduling as template for simplicity.

Table 1: Collision resolution scenarios (with agreed labels indicated in brackets)

|  | PUSCH | URLLC SR | HARQ-ACK (“fast” / “slow” codebooks) | P-CSI |
| --- | --- | --- | --- | --- |
| HARQ-ACK | - | Rel-15# [01,11] | Prioritize last scheduled codebook [12] | - |
| P-CSI | - | SR prioritized [02] | Prioritize “fast” HARQ [03,13] | - |
| PUSCH | [08] [04,15] L2* [10] [07] |  | Multiplex or prioritize based on timing and codebook [05,14,16] | Rel-15* [06,17] |
| eMBB SR |  |  | Rel-15 [08] | Rel-15 [09] |
| - L2*: L2 only passes prioritized PDU/BSR/SR to L1, conflicting on-going transmissions are dropped by L1. Note: for eMBB SR vs. PUSCH. L1 behaviour does not change w.r.t. Rel-15. - Rel-15#: Rel-15 as baseline (albeit sub-optimal). Enhancement: delayed eMBB HARQ sending mechanism. - Rel-15*: Rel-15 multiplexing rules are in use with small extensions (e.g. extended beta range, etc.) - Prioritize “fast” HARQ: Rel-15 multiplexing rules used unless the “fast” HARQ procedure is selected, in which case HARQ codebook is transmitted only and CSI is dropped. - PUSCH vs. HARQ: if multiplexing timeline not met, or “fast” HARQ scheduled last then prioritize the transmission associated with the last DCI. Otherwise, apply Rel-15* |  |  |  |  |

Table 2: Labels used with collision resolution scenarios

|  | URLLC SR | URLLC HARQ-ACK | CSI | URLLC PUSCH |
| --- | --- | --- | --- | --- |
| URLLC HARQ-ACK | Scenario-01 |  |  |  |
| CSI | Scenario-02 | Scenario-03 |  |  |
| URLLC PUSCH | Scenario-04 | Scenario-05 | Scenario-06 |  |
| eMBB SR | Scenario-07 | Scenario-08 | Scenario-09 | Scenario-10 |
| eMBB HARQ-ACK | Scenario-11 | Scenario-12 | Scenario-13 | Scenario-14 |
| eMBB PUSCH | Scenario-15 | Scenario-16 | Scenario-17 | Scenario-18 |

# Complementary and simultaneous use of codebook-based and codebook-less HARQ-ACK feedback

This section addresses the following pending issues marked as FFS in the text highlighted by yellow:

R1-1905716 Summary of RAN1#96bis on UCI enhancements for URLLC OPPO Agreements: For supporting multiple PUCCHs for HARQ-ACK within a slot for constructing HARQ-ACK codebook, support sub-slot-based HARQ-ACK feedback procedure. - A UL slot consists of a number of sub-slots. No more than one transmitted PUCCH carrying HARQ-ACKs [i.e., HARQ-ACK codebook, as was clarified in on-line session] starts in a sub-slot. - PDSCH transmission is not subject to sub-slot restrictions (if any) - FFS: PDSCH-to-sub-slot association. - FFS: Allowing PUCCH across sub-slot boundary or not. - R15 HARQ-codebook construction is applied in unit of sub-slot at least for Type II HARQ-ACK codebook. - FFS for Type I HARQ-ACK codebook. - R15 PUCCH resource overriding procedures is applied in unit of sub-slot. - Number or length of UL sub-slots in a slot is UE-specifically semi-statically configured. - FFS: Limit of number of PUCCH transmissions carrying HARQ-ACKs in a slot. - FFS: K1 definition. - FFS: Details of PUCCH resource configuration and determination. FFS: Use “Codebook-less HARQ” as a complementary or not. FFS: If HARQ-ACK can be omitted in case latency requirement cannot be met. FFS: PDSCH groupings and PHY identification for separate HARQ-ACK constructions for different service types.

## Problem: slot partitioning dilemma

For low latency, the smaller the sub-slot size is the better. On the other hand, HARQ-ACK multiplexing favours codebook segmentation based on slots, or a minimal number of sub-slots per slot. To illustrate this by an extreme case, let the sub-slot size be 1 OS. Now, K1 fully defines the PUCCH timing, and cannot be overridden by later DCI’s. I.e., the exact timing is decided before any HARQ multiplexing opportunity could arise, reducing the flexibility of HARQ multiplexing. Similarly, from HARQ multiplexing aspect, slot-based HARQ segmentation is better than half-slot based and half-slot based is better than 3 or 4 sub-slots per slot. This point is illustrated by Figure 1.

Figure 1: PUCCH overriding – essential for HARQ multiplexing – may get over-constrained as sub-slot size is reduced. On the other hand, sub-slot boundaries are instrumental in breaking a chain of PUCCH over-riding when the latency would exceed the requirements. Trade-offs for sub-slot size configuration can be subtle in some cases.

We elaborate further on the question why PUCCH timing needs to be shifted with multiplexing.

When the latency budget is restricted for a re-transmission then K1 and PRI should appoint the earliest resource that meets the N1 UE timelines and does not conflict with other uplink transmissions, rather than the latest resource that still meets the delay targets. Why? On one hand, the allocation size, hence the delay, following a potential multiplexing with other HARQ-ACK is unknown. On the other hand, the probability of other scheduling constraints (resource conflicts) for HARQ-ACK sending or retransmission increases as HARQ-ACK gets delayed. Therefore, URLLC HARQ-ACK multiplexing should in general rely on PUCCH overriding for the timing.

The optimal sub-slot size is the one that allows HARQ multiplexing by PUCCH resource overriding while also enables HARQ codebook segmentation that meets the latency requirements. However, a good trade-off is not guaranteed when the HARQ latency budget and the average packet inter-arrival time are both short and comparable in duration.

Observation 1: HARQ multiplexing favours larger sub-slots or no partitioning at all, which may involve losing the retransmission occasion in certain corner cases. A good trade-off for sub-slot partition size is not always guaranteed.

Observation 2: Only codebook-less HARQ feedback sending can achieve zero PUCCH alignment delay.

## Rationale: handle deviations separately from average regime

Paradoxically, to reduce the number of trivial HARQ codebooks, we consider allowing codebook-less HARQ feedback as a simultaneous, complementary option for the gNB, i.e. as a component of a hybrid scheme.

The goal is to relax the constraints on the sub-slot partitioning for the codebook-based procedure, and to arrive at configurations of larger sub-slots, so as to allow more HARQ multiplexing.

As illustrated by Figure 2, in the hybrid scheme, the partitioning is decided based on the frequent patterns (e.g. 80%-percentile) rather than the infrequent ones (e.g. 99%-percentile). In turn, codebook-less HARQ is available as a last resort to avoid single-shot downlink allocations. Without this scheme the sub-slot size can be “over-fitted” to the latency requirements of rare patterns. As a result the HARQ multiplexing can become more rigid since PUCCH resource overriding cannot operate across sub-slot boundaries.

Figure 2: Flow-charts presenting sub-slot size selection; 
comparison between sub-slot based and hybrid HARQ procedures.

An example FDD scenario is also depicted in Figure 3, in which codebook-less HARQ allows configuring a larger sub-slot size which allows transmitting more PUCCH’s. By its flexibility and small size codebook-less HARQ can also be useful in scheduling around or puncturing other transmissions if necessary for intra/inter-UE prioritization.

Observation 3: Complementary codebook-less HARQ procedure can allow choosing coarser or no partitioning for the codebook-based procedure. This benefits scheduling and HARQ multiplexing.

Proposal 1: Use “Codebook-less HARQ” as a complementary procedure, simultaneously to the codebook-based procedure. The number of PUCCH’s carrying codebook-less HARQ in a (sub-)slot needs not be restricted.

Proposal 2: Limit the configurable number of sub-slots to maximum two sub-slots per slot.

Figure 3: Example FDD scenario where codebook-less HARQ allows having longer sub-slots and less PUCCH transmissions. Constraints to scheduling: restricted HARQ latency budget, uplink resource conflicts, max. 1 HARQ codebook per sub-slot.

## Potential benefits

The hybrid scheme outlined in the previous section

- ensures that the deadline for HARQ feedback is always met as long as resources are available

  - codebook-less HARQ can even support 0-delay by enhanced time density of PUCCH allocation and the lack of over-riding

  - “scheduling around” other allocations is more flexible

- simplifies scheduling and HARQ multiplexing by minimizing sub-slot boundary effects in PUCCH overriding; in fact, slot-based codebook segmentation procedure might well fit most application scenarios

- provides an efficient solution to support multiple URLLC traffic with heterogeneous requirements, since codebook-less mode supports separate, out of order HARQ-ACK feed-back

- comes for free in terms of UE signalling, UE complexity, standardization complexity, as follows from the details below

## Operation details

Codebook-less HARQ mode differs from Type-2 HARQ codebook determination procedure in that:

- DAI counters are not used for HARQ codebook determination

- PUCCH resource over-riding is not applied

- the number of PUCCH’s need not be limited per (sub-)slot, (at least, not for segmentation)

- PUCCH timing and PUCCH resource assignment can be simplified (as detailed further below)

Self-recovery

A lost DCI indicating codebook-less HARQ must not cause any harm to the operation of the other HARQ procedure(s). Therefore, codebook-less HARQ mode would operate as a separate procedure, decoupled from the codebook segmentation, (as opposed to just enabling/disabling the over-riding rule). This guarantees that the DAI counter and PUCCH resource overriding mechanism of the codebook-based procedure(s) cannot be affected by a DCI reception failure.

Slot-partitioning

The codebook-based procedure could either use slots or half-slots for the codebook segmentation (as the unit of K1) and the PUCCH resource allocation. The codebook-less procedure does not use sub-slots for segmentation, the only configuration issue is the PUCCH allocation.

PUCCH allocation and assignment

By default, codebook-based procedure and codebook-less HARQ procedure may use the same PUCCH allocation. For both procedure(s) the PUCCH assignment and timing can follow the Rel-15 procedures adapted to sub-slots.

PUCCH timing

For the codebook-less procedure the Rel-15 mechanism can be used to select the sub-slot where the PUCCH resource is transmitted. Alternatively, the sub-slot can also be inferred using the N1 UE processing timeline.

Signalling

One of the signalling options used for codebook selection can be used with codebook-less HARQ as well. However, notice that K1 indication is not needed for the timing. Hence, an economic way to select codebook-less sending is to define a special K1 index that selects codebook-less sending when high-priority HARQ transmission has been indicated. This is illustrated in Figure 4. If K1 is not inferred, then multiple special K1 indexes need to be selected.

Figure 4: Special K1 index to select between codebook-based and codebook-less HARQ

Proposal 3: For HARQ codebook sending the PUCCH resource selection should use the Rel-15 mechanism adapted to sub-slots.

Proposal 4: For codebook-less procedure, the PUCCH resource assigned by the PRI should be sent in the earliest sub-slot that abides by the N1 UE timeline.

Proposal 5: To avoid signalling overheads, a configurable special K1 (index) value could select codebook-less HARQ_ACK sending.

# Support of HARQ-ACK feedback for mixed URLLC and eMBB traffic in a UE

This section reflects onto some of the open issues on signaling agreed in RAN1#96bis and RAN1#97.

Agreements from RAN1#96bis: When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, for both Type I (if supported) and Type II HARQ-ACK codebooks (if supported), and for dynamically-scheduled PDSCH, down-select from below for the PHY identification for identifying a HARQ-ACK codebook: - Opt.1: By DCI format - Opt.2: By RNTI - Opt.3: By explicit indication in DCI (FFS: new field or reuse existing field) - Opt.4: By CORESET/search space - FFS additional option(s) for Type I HARQ-ACK codebook FFS: For SPS PDSCH (including SPS release PDCCH)

Agreements RAN1#97: For sub-slot-based HARQ-ACK feedback procedure, K1 is the number of sub-slots from the sub-slot containing the end of PDSCH to the sub-slot containing the start of PUCCH. - Use UL numerology to define the sub-slot grid for PDSCH-to-sub-slot association. - FFS: The configurable value range of K1 needs to be extended, and impact to related DCI field bitwidth. - Note: It has been agreed that K1 is defined following R15 approach but in unit of sub-slot.

## HARQ codebook indication

The so-far considered signalling options are listed below

- Opt.1: By DCI format

- Opt.2: By RNTI

- Opt.3: By explicit indication in DCI (FFS: new field or reuse existing field)

- Opt.4: By CORESET/search space

Introducing a new DCI field, increases the size, hence slightly impairs the reliability, which should be handled with scrutiny. All the other options avoid that, but each solution comes with its own drawback, as we discuss below.

Opt.1 by DCI format: causes either inflexibility to the scheduling or requires additional CCE’s/BD’s for PDCCH monitoring by the UE:

- Both codebook-based and codebook-less HARQ feedback should be able to use the conventional DCI format to minimize the number of formats. E.g. compact DCI might cause some restriction why URLLC traffic needs to use DCI 1_1, or e.g. it would be advantageous to use compact DCI with eMBB traffic when PDCCH scheduling is a bottle neck.

- Otherwise, further DCI formats need to be defined, which imply increased complexity for PDCCH monitoring by the UE.

Opt.2 by RNTI:

- If new RNTI’s are introduced that increases the false alarm probability

- Else, to avoid that, MCS-C-RNTI need to be tied in with low latency HARQ feedback. This has several issues, which represent an unacceptable restriction for the scheduling flexibility:

  - If URLLC needs to use MCS-C-RNTI for the MCS table, it must use the low latency feedback even when no more retransmission occasions are available. Normally, in such a case URLLC would use the same codebook as eMBB, and HARQ-ACK would only be used for gathering statistics by gNB.

  - A different indication would have to be introduced for SPS activation

Opt.3 by explicit indication in DCI (new DCI field or reuse an existing DCI field):

3a) new DCI field: this would increase the DCI size by an extra 1 bit, which affects reliability. Adopting this solution without further examining the possibility of reusing existing DCI fields would go against the philosophy of the compact DCI, where each bit needs to be justified.

3b) reuse HARQ process ID field:

This solution consists of mapping HARQ processes ID’s to ‘slow’ or ‘fast’ codebooks within a sub-slot. Thus the HARQ process ID field is used for codebook indication as well.

At first sight the drawback can be that the number of HARQ processes used by eMBB and URLL cannot fluctuate freely. A conservative HARQ process ID allowance for URLLC would impede the eMBB throughput. However, this effect is negligible for the following reasons:

- Currently 16 HARQ processes are supported: this is twice the number of maximum HARQ processes in LTE! So, even if 8 HARQ processes were reserved to URLLC this would still leave plenty of processes for eMBB. However, this is far from being the case. The number of required HARQ processes is correlated to the HARQ Round Trip Time. Since RTT is short in URLLC, it only requires a few HARQ processes (e.g. 1-4). Notice that the eMBB downlink throughput is already restricted by the URLLC downlink traffic, and TCP-IP behaviour also favours lower RTT for eMBB HARQ feedback.

- Without this signalling method, URLLC will take the HARQ processes it requires. The only difference is that fluctuations in their potential HARQ process utilization ratios are not allowed by fixing the allowances between the ‘slow’ and the ‘fast’ HARQ codebooks. However, these fluctuations are small anyway because the round trip times are very different for URLLC and eMBB, so ‘borrowing’ cannot really work. Without safe, fixed allowances it could also occur that all the HARQ processes are already in use when a new URLLC downlink transmission needs to be scheduled. In this case an eMBB HARQ process needs to be terminated by toggling the NDI, which, in turn, causes RLC overhead.

Hence, fixed allowances between ‘slow’ and the ‘fast’ HARQ codebooks can have but negligible impact on eMBB throughput, and would probably be applied anyways by the gNB scheduler.

3c) reuse of K1 field:

This solution consist of having a common K1 table between the HARQ codebooks, but each item is tied in with an indication for either the ‘slow’ or the ‘fast’ HARQ codebook. This does not preclude that ‘slow’ and ‘fast’ codebooks have identical entries, or that the sub-slot sizes used as the unit of K1 are configured separately for the two codebooks.

The only possible drawback is that it depends case-by-case whether a saving in the DCI length is achieved or not. E.g. if the table has five entries: three for the ‘slow’ and two for the ‘fast’ codebooks then the K1 field requires three bits, which is equivalent to having a new DCI field of one bit, and a 2-bit K1 field to index two separate K1 tables. However, if ‘fast’ and ‘slow’ codebooks use e.g. two and five entries, respectively, then the explicit indication field is saved.

3d) reusing K1 and HARQ ID fields:

This solution is the combination of 3b) and 3c). It consists of partitioning the HARQ process ID’s into three groups, the third being the common pool. (The two other are tied in with ‘slow’ and ‘fast’ HARQ codebooks, respectively.) Each pool has its own K1 table. When the common pool is selected, the K1 table selects the ‘slow’ or ‘fast’ HARQ codebook by making use of the method discussed in option 3c).

This solution guarantees that one bit is always saved with respect to option 3a), on the other hand, the HARQ process allowances are not fixed as opposed to 3b).

Opt.4: By CORESET/search space: also introduces unnecessary scheduling constraint, and it can potentially increase the number of CCEs/BDs that a UE needs to monitor.

From the above list, option 3) has the flexibility to down-select the optimal trade-off. Options 3b), 3c) and 3d) are the most efficient solutions in particular.

Proposal 6: Use explicit indication in DCI for the PHY identification for identifying a HARQ-ACK codebook when at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE. 
FFS: introduce new field or reuse existing field.

## Dynamic vs. static format codebooks

The rationale behind each codebook determination method is explained by the way they ensure predictable codebook format and size despite occurrences of DCI reception failures; codebook size also plays a role in decoding the PUCCH assignment, hence the stakes to get it right. Type-1 (semi-static) guarantees the format by including padding bits to cover all PDSCH reception opportunities regardless of the successfully detected DCI’s. Padding bits make this method inefficient. Type-2 (dynamic) format applies modulo-4 sequence indexing (DAI counters) to DCI’s, which allows detecting failing DCI’s as long as a burst of four or more failures does not occur. While such burst can be produced in eMBB over time, in scenarios having URLLC traffic alone they can virtually be precluded. Hence, no need to look further to solutions based on Type-1 method.

Observation 4: DAI counter mechanism in Type-2 HARQ codebook is very reliable with URLLC traffic alone.

The semi-static HARQ codebook format has the advantage of fixed size and format, hence robustness. It also allows reusing the DAI field for the purpose of indicating codebook-less HARQ feedback (without allowing another codebook in the same slot). However, these advantages vanish in the URLLC scenario, where dynamic HARQ codebook format can be just as robust and codebook-less HARQ feedback can be triggered easily by other means.

Due to the padding bits, Type-1 format is impractical for URLLC transmissions, and should be avoided. Therefore, it requires no enhancements. Meanwhile, in mixed traffic scenarios, Type-1 might be considered to be a safer configuration to use with eMBB traffic.

Proposal 7: Semi-static HARQ codebook does not need enhancement.

# Intra-UE multiplexing and prioritization of UCI

Table 1 and 2 in the Introduction provides a short overview of our views expressed in the email discussion preceding the current RAN1 meeting (“Summary of email discussion [97-NR-05] on handling intra-UE”, Oppo). This section elaborates on the rationales, and formulates the concrete proposals for the enhancements.

In the sequel we assume that frequency domain multiplexing of PUCCH/PUSCH with PUCCH/PUSCH is either not viable or the last option to consider for enhancement.

Enhancements based on prioritization involve dropping or delaying one transmission, which may involve the early termination of an already started transmission on and after the conflicting symbol. The latter capability we believe is necessary, but its possible impact on UE timeline and inter-UE multiplexing is not discussed here further. Puncturing is not considered at this point due to various complexity issues of assessment, implementation and standardization.

## HARQ-ACK

When URLLC HARQ conflicts with another scheduled resource, and Rel-15 timelines and multiplexing does not allow meeting the reliability and/or latency targets set for a transmission, the scheduler could have the following options to choose from to resolve the prioritization conflict:

Opt-Harq-1: Schedule a DL resource that allows single-shot transmission – assuming the hit to downlink resource utilization–, to relax the latency and reliability requirements on the HARQ feedback, which can thus be sent later (possibly using the “slow” HARQ codebook even: alone, or along with eMBB HARQ codebook).

Opt-Harq-2: Prioritize URLLC HARQ – requiring further enhancement to the current standard, whereby either dynamic signalling or/and static rule tells the UE to prioritize HARQ over the other transmission rather than apply multiplexing.

Opt-Harq-3: Repeat DL transmission without waiting for HARQ-ACK – requiring further enhancement to the current standard. Similarly to option 1, this method, too, would allow deprioritizing the URLLC HARQ transmission without jeopardizing the DL reliability itself. An example is given in the sequel for the collision resolution between URLLC PUSCH and URLLC HARQ.

We will refer back to these resolution options in the case of different scenarios.

Figure 5:      a) Prioritize HARQ (Opt-Harq-2)     b) Single-shot (Opt-Harq-1)    c) Repeat DL (Opt-Harq-3)

### URLLC HARQ vs eMBB HARQ (scenario-12)

In previous RAN1 meetings, agreements have been formed to support two simultaneously constructed HARQ-ACK codebooks and to study further their identification based on some PHY indications/properties (Section 3.1). Since the two codebooks are motivated by different latency requirements, we shall label them “slow” and “fast” respectively. Also, the collision between the respective PUCCH resources only occurs when “slow” HARQ codebook is scheduled before DL-DCI of URLLC traffic. The only way to avoid the collision is by deprioritizing URLLC HARQ according to Opt-Harq-1, i.e., by single-shot DL transmission.

If UE timeline and latency permitted the scheduler could actually schedule URLCC HARQ on a common codebook with eMBB. Although the potentially stricter reliability targets of URLLC HARQ and the limited PUCCH resource set might lead to an inefficient encoding, this is still more affordable then deprioritizing eMBB HARQ, which triggers multiple retransmissions.

Multiplexing PUCCH resources could bring some minor improvements to the above (e.g. by separate encoding), but due to difficulties to meet the latency and to the PUCCH selection, it is cumbersome in our view, and due to latency issues would not eliminate the case for prioritization anyways.

For the aforementioned reasons, the collision scenario should be used for prioritization of the later HARQ (URLLC, or ”fast” codebook). Alternatively, the rule could also use the yet to agreed identification method as a means to select priority for this scenario.

Proposal 4-1: In collision scenario-12 (URLLC HARQ – eMBB HARQ), prioritize the PUCCH resource that was scheduled by the later DCI, dropping the transmission of the other PUCCH, possibly already started. (Scenario-12)

Dropping eMBB HARQ codebook definitely results in multiple retransmissions in downlink. However, recovery of HARQ-information could occur by sending the codebook in full in a later slot. A possible solution for the delayed sending is that HARQ codebook gets queried by gNB similarly to a CSI report over UL-DCI/PUSCH. The special grant or RRC configuration required for this could reuse the existing UL-DCI format used with A-CSI. An alternative solution would be to use PUCCH for the delayed sending, which for small codebook sizes could result in a more efficient encoding, but the insignificant gains would be outweighed by the standardization difficulties of an appropriate solution. We only need to refer to the fact that PUCCH resource set selection is based on size. Any simplifying solution could mean that the efficiency advantage is lost. A further advantage of UL-DCI is that the uplink retransmission mechanism can handle further de-prioritization or unsuccessful decoding by regular PUSCH retransmissions.

It is important to note that delayed eMBB HARQ sending could be used in other collision scenarios as well, as opposed to PUCCH multiplexing, which only addresses the HARQ vs. HARQ collision. Even if multiplexing is supported, the increased eMBB HARQ codebook decoding failure rate could call for delayed re-sending.

Proposal 4-2: The UE should store any HARQ codebook transmitted on PUCCH that has been deprioritized, along with the slot/subslot, for later re-sending in full. FFS: memory depth. (Scenario-12, …)

Proposal 4-3: For querying the UE’s history of deprioritized HARQ codebooks (Scenario-12, …), reuse A-CSI measurement and reporting, by reserving a special RRC configuration for selecting this report type instead of CSI measurement. FFS: precise report format

### HARQ vs P/SP-CSI (scenarios 03 and 13)

Multiplexing URLLC HARQ-ACK information with P/CSI can result in late HARQ feedback and the collision can occur way too often to overcome by Opt-Harq-1. Therefore, URLLC HARQ should be prioritized over P-CSI.

Assuming one URLLC HARQ codebook on average per slot (i.e. less than 0.5 ms average packet inter-arrival time at 30 kHz, as an extreme worst case), the chance that P/SP-CSI gets dropped can be in the order of 30% in certain cases. In the case of long CSI period, this could be compensated for by reduced CSI periodicity. On the other hand, with frequent CSI the dropping can be tolerated because RI, PMI vary more slowly and the occasional loss of CQI do not have dramatic affect. In typical scenarios the occurrences have lower probability and the losses are affordable.

Meanwhile the rule should specify multiplexing for the “eMBB”/”slow” HARQ codebook, based on the PHY indication signal that will support the codebook selection. As a side benefit, when multiplexing can meet URLLC HARQ latency and reliability, and a “slow” codebook is not yet scheduled in the current slot, the URLLC DL-DCI could select HARQ/CSI multiplexing by indicating “slow” codebook. This could further reduce the probability that CSI is dropped.

Proposal 4-4: When HARQ-ACK collides with P/SP-CSI (scenarios 03 and 13), then based on a distinction using the codebook selection PHY indication method, “slow” codebook should be multiplexed with CSI, whereas a “fast” codebook should be prioritized.

## SR colliding with HARQ/CSI (scenarios 01, 02, 07, 08, 09, 11)

URLLC SR is not simply a grant request but implies a timestamp for a request on the gNB side, too. The gNB has no means to distinguish between an SR that was received within a fraction of a period, 1 or more periods, or a cycle of the prohibition timer following a grant request from UE’s L2. One exception would be if the surplus delay was introduced by de-prioritization that the gNB is aware of (or can detect with sufficient reliability). In conclusion, URLLC SR should have the same reliability as the overall reliability target (including retransmissions).

Observation 4-1: URLLC SR transmission also serves as a timestamp for the request, and should have the same reliability as the overall reliability target of the uplink traffic.

As a consequence, it is of primary importance not to drop URLLC SR when it collides with another resource. If timelines are met then multiplexing should be allowed, provided that the gNB makes sure that false detection of negative SR occurs with negligible probability. If decoding of the multiplexed resource fails, positive SR needs to be assumed with the worst case timing hypothesis. This way both the timestamp and the reliability are preserved, and the only concern can be latency.

Observation 4-2: Probability of false detection of negative URLLC SR must not increase by multiplexing. Then timestamp associated with SR event is also preserved.

Unlike in the case of HARQ vs. CSI collision where the gNB knows the delay budget, and has the option to avoid collision or not, when SR collides with CSI or HARQ, such arbitration is not available to the UE. In fact, the SR occasions can be much more frequent then the actual request, thus scheduling around may be impossible or inefficient.

Alt-1.	Transmit URLLC positive SR, otherwise transmit CSI/HARQ.

Alt-2. 	Multiplex if timelines allow, otherwise prioritize URLLC positive SR.	

The scheduler needs to make sure that multiplexing never introduces more latency than what would be tolerable by the delay budget. 	Although in some cases an excessive delay could be compensated for by larger UL allocations (e.g. used to skip BSR-transmission-to-data delay, or to avoid the need for data retransmission), however the probability of such occurrences should stay very low. In some cases the delay budget for SR transmission can be increased by shorter SR periodicity, or by taking the fact of the multiplexing delay into account in the scheduling of the following UL data grants. Such solutions are more affordable than e.g. dropping a URLLC HARQ as in Opt-Harq-1. Ultimately, configured data grant is also available to support strict uplink delay budgets.

Alt-3. 	Multiplexing is allowed if it does not delay the decoding of URLLC SR. Otherwise multiplexing is disabled and URLLC positive SR takes priority.

While the above condition is a sensible improvement over blindly dropping CSI/HARQ whenever it collides with URLLC positive SR (Alt-1), the probability that HARQ/CSI is dropped has not decreased considerably.

When Alt-1 is inefficient, then Alt-2 is preferred for its simplicity, whereas Alt-3 is insufficient.

Observation 4-3: Some headroom in the latency budget of URLLC positive SR should be assumed.

Finally, we note that prioritization of URLLC positive SR has clear advantages when positive SR occurs rarely, but the latency requirement is extremely tight. Such cases could be identified by short SR periodicity and long prohibition timer duration.

### URLLC SR colliding with HARQ

Table 3: Rel-15 multiplexing rules between SR and HARQ

| Collision case | SR with PF0 | SR with PF1 |
| --- | --- | --- |
| HARQ-ACK with PF0 | Transmit both HARQ-ACK and SR on PF0 for HARQ-ACK | Transmit both HARQ-ACK and SR on PF0 |
| HARQ-ACK with PF1 | Transmit HARQ-ACK on PF1, drop SR | Transmit HARQ-ACK on PF1 for SR when SR is positive; transmit HARQ-ACK on PF1 for HARQ-ACK when SR is negative. |
| HARQ-ACK with PF2 | Transmit both HARQ-ACK and SR on PUCCH resource for HARQ-ACK based on UCI payload and PRI |  |
| HARQ-ACK with PF3 |  |  |
| HARQ-ACK with PF4 |  |  |

We first consider collision with HARQ transmitted in PF 0/1. Here, Rel-15 rules are not optimal but can be maintained:

- Rel-15 rule for multiplexing PF1 SR with PF1 HARQ preserves both reliability and latency of SR.

- Rel-15 rule for multiplexing PF0 SR with PF0 HARQ, preserves both reliability and – if properly used – latency of SR.

- Rel-15 rule for dropping PF0 SR when it collides with PF1 HARQ is harmful but could be circumvented. This rare scenario can only be produced when a downlink URLLC flow has much higher reliability target than uplink URLLC flow. Either a PF1 is configured for SR to avoid it, or the SR period is shortened. In the latter case, the prohibit timer will only start after SR gets sent and the gNB will assume that SR collided with HARQ and reduce the delay budget by one SR period.

- Rel-15 rule to multiplex PF1 SR with PF0 HARQ on HARQ resource may compromise SR reliability. In such cases it should not be used.

  - If SR is frequent then HARQ reliability might need to be adjusted by using PF1 instead of PF0, else if PF1 SR is infrequent then changing the rule from multiplexing to SR prioritization could enhance efficiency.

  - Encoding of positive/negative SR on two symbol PF0 can be as reliable as the on/off encoding of positive SR on PF1 on four symbols.

Observation 4-4: URLLC SR colliding with 1 or 2 bit HARQ codebook should be multiplexed if the SR PUCCH format is preserved, otherwise URLLC SR should preferably be prioritized.

In the case of more than 3 HARQ bits, scheduling PF2 HARQ around SR can mitigate situations when multiplexing would fail to meet reliability. When multiplexing, the scheduler has the choice to opt for a more robust PF3 or PF4 resource. Although URLLC SR reliability target might be stricter than that of URLLC HARQ codebook, however, multiplexing over a more robust resource is still the most efficient solution unless the chance of collision is negligible. Note that if PDCCH fails, SR gets transmitted; else SR+HARQ is transmitted with the same positive-to-SR probability as NACK-to-ACK probability by default.

In the case of Reed-Muller encoding it is also possible to bias SR detection towards positive SR. Hence, e.g., if we assume a 10-6 overall reliability target in both uplink and downlink, NACK-to-ACK probability may target e.g. 10-5, with 0.5% retransmission per packet, whereas SR decoding could favour a 5*10-7 positive-to-negative error rate at the expense of a 2-5% negative-to-positive SR rate.

Remark: It may occur that the HARQ reliability target is much lower than the SR target: e.g. when multiple retransmission occasions are available. In such situations scheduling PF2 around SR occasions can be a solution.

Observation 4-5: Multiplexing URLLC SR onto URLLC HARQ PUCCH Format 2, 3, 4 should be allowed.

Prioritization can be affordable when the SR occurrence rate is very low while the SR periodicity may still be short. Such a case could be classified by the large ratio between the SR prohibition timer and SR periodicity: e.g. the SR periodicity is half a slot whereas the prohibition timer expires after tens of slots.

Distinction between “slow” and “fast” HARQ codebooks can be beneficial. Dropping “slow” HARQ codebook instead of multiplexing URLLC SR with it can be justified by considering the lower reliability targets of eMBB and the opportunity to recover the loss of information by delayed sending.

Proposal 4-5: Rel-15 multiplexing rules (although sub-optimal) should be applied if URLLC SR collides with URLLC HARQ as baseline. (Scenario-01).

Proposal 4-6: eMBB HARQ should be dropped on and after the symbol(s) colliding with URLLC positive SR, unless multiplexing results in the same PUCCH format as SR and the UE timelines are met. (scenario-11).

### URLLC SR colliding with CSI

CSI parameters can vary considerably in measurement content and periodicity as well as report size. In general, multiplexing URLLC SR with CSI risks a considerable loss in reliability and/or latency. Therefore, dropping CSI should be considered when colliding URLLC positive SR. The collision between CSI and positive SR is less frequent than the collision between CSI and SR. Dropping CSI can be tolerated because RI, PMI vary more slowly and the occasional loss of CQI do not have dramatic affect. The periodicity of CSI should account for variability in the interference, hence CSI measurement and reporting can be more frequent than what would be justified by the rate of channel variation.

Proposal 4-7: Drop CSI colliding with URLLC positive SR. Otherwise transmit CSI. (scenario-02)

### eMBB SR colliding HARQ/CSI (scenarios-08, -09)

Meanwhile, eMBB SR can always be safely multiplexed over other resources, it would be wasteful to prioritize eMBB SR in situations where URLLC SR is. Therefore, introduction a priority flag associated with each SR configuration is proposed, and low-priority SR should be multiplexed with HARQ/CSI.

URLLC HARQ reliability is either unaffected by eMBB SR, as in the case of PF0/PF1. When URLLC HARQ contains more than two bits, and the PUCCH resource overlaps an SR occasion, it needs to be made more robust. However, eMBB SR occasion periodicity is relatively long, because it is not sensitive to latency, and thus the probability of eMBB SR – URRLC HARQ collision is low.

Proposal 4-8: Use Rel-15 multiplexing when eMBB SR colliding with HARQ/CSI (scenarios-08, 09).

### URLLC SR colliding eMBB SR

This scenario is not realistic. If L2 passes a URLLC positive SR while eMBB SR is already being transmitted then eMBB SR should be dropped on and after the colliding symbol and URLLC SR transmitted.

## PUSCH vs. SR priority (scenarios 04, 10, 15)

Resolution methods and according priority rules are needed for this scenario.

Method 1: Dropping lower priority transmission on and after the colliding symbol

- MAC is configured with priorities to arbitrate a received uplink grant between logical channels (LCH). Whether a grant is suitable for a particular LCH is determined by respective LCP rules. When SR is triggered in MAC, and an uplink grant is overlapping the next SR occasion(s) then L2 is expected to pass the higher priority transmission to L1: whether it be SR or PDU/BSR.

- L1’s only role in the above behaviour is to drop an on-going transmission, such as an eMBB PUSCH, if it receives a conflicting new request from L2, e.g. to send an SR on a colliding resource.

- Advantage: URLLC latency and reliability can be maintained for competing logical channels

- Disadvantage: PUSCH may get dropped and PDU retransmitted

Method 2: Puncturing by URLLC SR, dropping PUSCH on the colliding symbol(s) only

- L2 passes both PDU and SR to L1 in this case.

- Advantage: SR latency and robustness guaranteed

- Disadvantages: implementation difficulty; lower code rate required for grant whenever the SR occasion overlaps with PUSCH (impact of puncturing depends on distance to DMRS, number of symbols, RE mapping, etc.);
SR puncturing not to hit PUSCH DMRS.

Method 3: Multiplexing URLLC SR, dropping PUSCH on the colliding symbol(s) only

- L2 passes both PDU and SR in this case.

- Advantage: PUSCH never needs be dropped when UE timelines are met

- Disadvantage: multiplexing timeline constraint; SR latency constrains PUSCH resource allocation (on all collision with SR occasion); additional resource requirements of multiplexing

Since multiplexing does not guarantee latency, puncturing or dropping are better solutions. For its simplicity, dropping is considered as a favourable choice.

Proposal 4-9: For the case when URLLC PUSCH is colliding with URLLC SR (scenarios 04), RAN1 should consider which resolution methods should be supported or eliminated: dropping, puncturing, multiplexing. Priority rules and mechanisms are subject to RAN2 discussion outcomes.

Scenarios 10 and 15 should be analysed after the method is selected for scenario 4. Priority rules and mechanisms are subject to RAN2 discussion outcomes.

## PUSCH vs. PUSCH priority (scenarios 18)

Similarly to PUSCH vs. SR collision, L1 should only be passed any PDU if the PDU needs to be sent. If so, any conflicting transmission needs to be dropped, and stored in HARQ buffer for resending. RAN1 should wait for RAN2 conclusion on the behaviour.

## PUSCH vs HARQ (scenarios 05, 14, 16)

We consider Rel-15 multiplexing rules as starting point to handle the collision between PUSCH and HARQ. The problem with multiplexing URLLC HARQ with URLLC PUSCH is that the latency for one or the other will not necessarily be met.

Scenario A) If the scheduler is already aware of HARQ at the time of scheduling PUSCH then it can attempt to adjust the PUSCH allocation so as to meet the requirements of both allocations. If this is not possible than it can choose between prioritizing PUSCH or HARQ in the following way:

- schedule a single-transmission PUSCH after the HARQ

- multiplex HARQ on a PUSCH resource that fails to meet the HARQ delay budget. To mitigate DL transmission, it should be able to schedule a DL repetition without waiting for HARQ feedback if necessary.

Scenario B) If the scheduler is not aware of HARQ at the time of scheduling PUSCH and a collision still happens then meeting the UE timelines for multiplexing has very little chance. Therefore, the choice is more between prioritizing either HARQ or PUSCH. To prioritize PUSCH, HARQ needs simply be delayed beyond its budget, possibly involving a single-shot DL transmission (Harq-Opt-1). To prioritize HARQ, a rule is required to drop PUSCH. A slight chance for multiplexing in this scenario could be if the rule distinguished between “slow” and “fast” codebooks to select the behaviour. If no other “slow” codebook is scheduled for the same slot then URLLC HARQ-ACK could be scheduled onto it. This is illustrated by Table 3.

Proposal 4-10: When PUSCH collides with HARQ-ACK (scenarios 05,14,16) then if multiplexing timeline is not met, or DL-DCI indicates “fast” HARQ codebook and arrived later than the UL-DCI then prioritize the transmission associated with the later DCI. Otherwise, apply Rel-15, possibly with extended beta range.

Table 3: HARQ vs. PUSCH scenarios according to proposal

|  | “fast” HARQ first DCI (URLLC) | “slow” HARQ first DCI (eMBB) | “fast” HARQ later DCI (URLLC) | “slow” HARQ later DCI (URLLC) |
| --- | --- | --- | --- | --- |
| PUSCH (URLLC or eMBB) | Multiplex if timelines met, otherwise drop HARQ (earlier DCI). DL repetition without waiting for HARQ-ACK. | Multiplex if timelines met, otherwise drop HARQ (earlier DCI). Delayed eMBB HARQ sending. | Drop PUSCH. | Multiplex if timelines met. Otherwise drop PUSCH (earlier DCI). |

## PUSCH vs CSI (scenarios 06 and 17)

The scheduler is aware of the collision when configuring the grant for PUSCH. Beta-offset and alpha-factor allow to set the appropriate code rate for CSI. Although the fact of multiplexing can increase the duration of the transmission, this is just another one of the constraints that the scheduler and the system design needs to account for.

Proposal 4-11: When PUSCH colliding with CSI (Scenarios-06, 17), reuse Rel-15 handling as baseline. FFS: extended beta-offset range, prioritization of PUSCH when UE timeline is not met.

## Summary

In conclusion, the following points and Table 4 summarize the features and solutions options we consider the most important.

- PUSCH/SR vs. PUSCH/SR: L2 only passes prioritized data/SR to L1, conflicting on-going transmissions are dropped. If this such a solution is unsatisfactory, then multiplexing and puncturing can be further considered. Prioritization rules and mechanisms are also subject to RAN2 discussion outcome.

- When eMBB HARQ transmission is dropped, a delayed sending is possible, similarly to the scheduling mechanism of an A-CSI report.

- PUSCH vs. HARQ: If multiplexing timeline is not met, or DL-DCI indicates “fast” HARQ codebook and arrived later than the UL-DCI then prioritize the transmission associated with the later DCI. Otherwise, apply Rel-15, possibly with extended beta range.

  - DL transmissions can be repeated without waiting for HARQ-ACK feedback, as a means for optimal recovery from de-prioritization

  - See both Table 3 and Table 4 for a summary

- It is paramount to guarantee URLLC SR reliability (especially against positive-to-negative errors), whereas some headroom for multiplexing delay should be assumed, (since the total delay budget has several components). It is also paramount that the first positive SR occasion be identified by the gNB (not precluding delay in the decoding, or pessimistic hypothesis on decoding failure)

Table 4: Collision resolution proposals by scenario

|  | URLLC SR | URLLC HARQ-ACK | CSI | URLLC PUSCH |
| --- | --- | --- | --- | --- |
| URLLC SR |  |  |  |  |
| URLLC HARQ-ACK | Reuse Rel-15 as a baseline |  |  |  |
| CSI | Drop CSI. | Drop CSI. |  |  |
| URLLC PUSCH | If L2 passes SR, drop PUSCH on and after colliding symbol. | Prioritize transmission scheduled by later DCI if either it was URLLC HARQ or if multiplexing timelines would fail. Multiplex otherwise. | Reuse Rel-15. Beta-offset and alpha cap should control UCI code rate. |  |
| eMBB SR | Drop eMBB SR on and after colliding symbol. | Reuse Rel-15 as a baseline | Reuse Rel-15. | Drop eMBB SR on and after colliding symbol. |
| eMBB HARQ-ACK | Alt 1. Delayed sending of eMBB HARQ-ACK when latency / reliability not met for SR. Alt 2. Reuse Rel-15 as a baseline | Drop eMBB HARQ on and after colliding symbol. Delayed sending of eMBB HARQ-ACK. | Reuse Rel-15. | Multiplex if timelines met. Otherwise, prioritize transmission scheduled by later DCI. Delayed eMBB HARQ-ACK. |
| eMBB PUSCH | If L2 passes SR, drop PUSCH on and after colliding symbol. | Prioritize transmission scheduled by later DCI if either it was URLLC HARQ or if multiplexing timelines would fail. Multiplex otherwise. | Reuse Rel-15. | If L2 passes data/BSR, then drop other transmission on and after colliding symbol. |

# Conclusion

In the first part of the contribution we had the following observations and proposals on multiple HARQ procedures:

Observation 1: HARQ multiplexing favours larger sub-slots or no partitioning at all, which may involve losing the retransmission occasion in certain corner cases. A good trade-off for sub-slot partition size is not always guaranteed.

Observation 2: Only codebook-less HARQ feedback sending can achieve zero PUCCH alignment delay.

Observation 3: Complementary codebook-less HARQ procedure can allow choosing coarser or no partitioning for the codebook-based procedure. This benefits scheduling and HARQ multiplexing.

Proposal 1: Use “Codebook-less HARQ” as a complementary procedure, simultaneously to the codebook-based procedure. The number of PUCCH’s carrying codebook-less HARQ in a (sub-)slot needs not be restricted.

Proposal 2: Limit the configurable number of sub-slots to maximum two sub-slots per slot.

Proposal 3: For HARQ codebook sending the PUCCH resource selection should use the Rel-15 mechanism adapted to sub-slots.

Proposal 4: For codebook-less procedure, the PUCCH resource assigned by the PRI should be sent in the earliest sub-slot that abides by the N1 UE timeline.

Proposal 5: To avoid signalling overheads, a configurable special K1 (index) value could select codebook-less HARQ_ACK sending.

Proposal 6: Use explicit indication in DCI for the PHY identification for identifying a HARQ-ACK codebook when at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE.
FFS: introduce new field or reuse existing field.

Observation 4: DAI counter mechanism in Type-2 HARQ codebook is very reliable with URLLC traffic alone.

Proposal 7: Semi-static HARQ codebook does not need enhancement.

In the second part of the contribution we had the following observations and proposals on intra-UE prioritization and multiplexing of UCI:

Proposal 4-1: In collision scenario-12 (URLLC HARQ – eMBB HARQ), prioritize the PUCCH resource that was scheduled by the later DCI, dropping the transmission of the other PUCCH, possibly already started. (Scenario-12)

Proposal 4-2: The UE should store any HARQ codebook transmitted on PUCCH that has been deprioritized, along with the slot/subslot, for later re-sending in full. FFS: memory depth. (Scenario-12, …)

Proposal 4-3: For querying the UE’s history of deprioritized HARQ codebooks (Scenario-12, …), reuse A-CSI measurement and reporting, by reserving a special RRC configuration for selecting this report type instead of CSI measurement. FFS: precise report format

Proposal 4-4: When HARQ-ACK collides with P/SP-CSI (scenarios 03 and 13), then based on a distinction using the codebook selection PHY indication method, “slow” codebook should be multiplexed with CSI, whereas a “fast” codebook should be prioritized.

Observation 4-1: URLLC SR transmission also serves as a timestamp for the request, and should have the same reliability as the overall reliability target of the uplink traffic.

Observation 4-2: Probability of false detection of negative URLLC SR must not increase by multiplexing. Then timestamp associated with SR event is also preserved.

Observation 4-3: Some headroom in the latency budget of URLLC positive SR should be assumed.

Observation 4-4: URLLC SR colliding with 1 or 2 bit HARQ codebook should be multiplexed if the SR PUCCH format is preserved, otherwise URLLC SR should preferably be prioritized.

Observation 4-5: Multiplexing URLLC SR onto URLLC HARQ PUCCH Format 2, 3, 4 should be allowed.

Proposal 4-5: Rel-15 multiplexing rules (although sub-optimal) should be applied if URLLC SR collides with URLLC HARQ as baseline. (Scenario-01).

Proposal 4-6: eMBB HARQ should be dropped on and after the symbol(s) colliding with URLLC positive SR, unless multiplexing results in the same PUCCH format as SR and the UE timelines are met. (Scenario-11).

Proposal 4-7: Drop CSI colliding with URLLC positive SR. Otherwise transmit CSI. (Scenario-02)

Proposal 4-8: Use Rel-15 multiplexing when eMBB SR colliding with HARQ/CSI (scenarios-08, 09).

Proposal 4-9: For the case when URLLC PUSCH is colliding with URLLC SR (scenarios 04), RAN1 should consider which resolution methods should be supported or eliminated: dropping, puncturing, multiplexing. Priority rules and mechanisms are subject to RAN2 discussion outcomes.

Proposal 4-10: When PUSCH collides with HARQ-ACK (scenarios 05,14,16) then if multiplexing timeline is not met, or DL-DCI indicates “fast” HARQ codebook and arrived later than the UL-DCI then prioritize the transmission associated with the later DCI. Otherwise, apply Rel-15, possibly with extended beta range.

Proposal 4-11: When PUSCH colliding with CSI (Scenarios-06, 17), reuse Rel-15 handling as baseline. FFS: extended beta-offset range, prioritization of PUSCH when UE timeline is not met.

# References

- R1-1814342 (R2-1818795) “LS on Intra-UE Prioritization/Multiplexing”, Nokia, RAN1#95, Spokane, USA, 12 - 16 November 2018

- R2-1817579 “E-mail discussion summary [103bis#41] [NR/IIoT] Intra-UE Prioritization”, Nokia, RAN2#104, Spokane, USA, 12 - 16 November 2018

- R1-1901825 “Study and evaluation of scheduling, HARQ and CSI processing timeline”, MediaTek, RAN1#96, Athens, Greece, 25 Feb. - 1 Mar. 2019