3GPP TSG RAN WG1 #98                                        R1-1908416

Prague, CZ, August 26th - 30th, 2019

Source:	OPPO

Title:	UL signals and channels for NR-U

Agenda Item:	7.2.2.1.3

Document for:	Discussion and Decision

Introduction

In RAN1 Meeting #97, the following agreements were achieved for UL signals and channels for NR-U:

Agreement:

Support enhancement of Rel-15 PUCCH formats PF0 and PF1 as follows:

- Mapping to physical resources of one full interlace in 20 MHz.

  - FFS: Sequence type and mapping considering the following alternatives:

    - Alt-1: Repetition of the length-12 Rel-15 PF0 and PF1 sequence in each PRB of an interlace with mechanism to control PAPR/CM considering the following alternatives

      - Alt-1a: Cycling of cyclic shifts across PRBs

      - Alt-1b: Phase rotation across PRBs of an interlace where the phase rotation is can be per RE or per PRB

    - Alt-2: Mapping of different length-12 Rel-15 PF0 and PF1 sequences to the PRBs of an interlace based on different group number u (range is 0 .. 29)

    - Alt-3: Mapping of a single long sequence to the PRBs of an interlace

  - FFS: Impact due to guardbands

- Note: Decisions on the above should be based on at least performance using the agreed MCL metric and specification impact

- Note: Interlaced PF2 and 3 are not enhanced to support 1-2 bit payloads

Agreement:

For enhanced Rel-15 PF3 supporting interlaced mapping, do not replace DFT-s-OFDM with CP-OFDM

This contribution mainly discusses potential enhancements to UL signals and channels for NR-U, including PUCCH design, PUSCH design, flexible starting point for PUSCH transmission due to LBT, SRS enhancement.

PUCCH design

For PUCCH transmission, from frequency domain perspective, the interlace structure of each PUCCH resource should be defined within a subband of 20MHz, considering that LBT can be performed in unit of subband and multiple PUCCH transmission opportunities can be provided if the BWP for PUCCH transmission consists of multiple subbands. In addition, the power efficiency can be largely used within 20MHz.

From time domain perspective, multiple PUCCH resources are identified to be beneficial for HARQ-ACK feedback on unlicensed carrier to increase the PUCCH transmission opportunities. On the other hand, it was agreed that no-LBT can be used for gap of no more than 16us within a shared gNB COT. If no-LBT is used for one PUCCH transmission, there is no need to configure multiple PUCCH resources for this PUCCH. Therefore, multiple PUCCH resources should be configured based on the corresponding LBT type for PUCCH transmission.

PUCCH resources for NR PUCCH formats 2 and 3 with configurable durations should be enhanced as discussed above. In addition, if multiple PUCCH resources are configured in time domain, those resources should be located as closely as possible to reduce the HARQ feedback delay, as shown in Figure 1.

Proposal 1: Multiple PUCCH resources should be configured based on the corresponding LBT type.

- One PUCCH resource is configured for no-LBT case.

- Multiple PUCCH resources are configured closely in time domain.

Figure 1 Multiple PUCCH resources in time domain

PUSCH transmission

For the case of 20MHz bandwidth, it was agreed that M = 10 interlaces with N = 10 or 11 PRBs / interlace for 15kHz SCS and M = 5 interlaces with N = 10 or 11 PRBs / interlace for 30kHz SCS. For 60kHz SCS, M = 2 interlaces with N = 12 PRBs / interlace is preferred to meet OCB requirement.

For the detailed design of interlace structure within 20MHz, the available PRB number and the minimum guardband requirements for different SCSs for 20MHz transmission bandwidth defined in [2] should be considered, which are summarized in Table 1.

Table 1 Available NRB and minimum guardband for 20MHz

| SCS [kHz] | NRB | Minimum guardband [kHz] |
| --- | --- | --- |
| 15 | 106 | 452.5 |
| 30 | 51 | 805 |
| 60 | 24 | 1330 |

Although the reserved guardbands for different SCSs are different according to Table 1, it is beneficial to have a common starting point as the starting point of interlace structure for different SCSs. Figure 2 gives examples of interlace structures for 15kHz, 30kHz and 60kHz SCSs. In Figure 2, the starting point of interlace #0 in frequency domain for different SCSs are aligned.

Figure 2 Interlace structure design for PUCCH

For PUSCH resource allocation, the common starting point associated with SCS for each UL BWP should be taken as the starting point of the BWP and be indicated to UE. The PRBs allocated for PUSCH transmission should be determined according the common starting point.

Proposal 2: The common starting point of the BWP should be indicated to UE for resource allocation.

In addition, since different UL BWPs can be configured with different bandwidth sizes, determining basic interlace structure for each configured BWP before PUSCH resource allocation is important. Figure 3 gives an example. As shown in Figure 3, the bandwidth of BWP 0 is 40MHz, which can be divided into BWP 1 and BWP 2 with each BWP has a bandwidth of 20MHz. The basic interlace structures for different BWPs are different.

Figure 3 Basic interlace structure for one BWP

Proposal 3: The basic interlace structure of the configured BWP should be indicated to UE for resource allocation.

As discussed above, it is preferred to define a PUCCH resource within a subband, i.e., 20MHz. Since one interlace can across multiple subbands, multiple PUCCH resources and each resource for one subband can be configured within one PUSCH interlace to achieve better multiplexing between PUSCH and PUCCH transmission.

Take 40MHz carrier, 30kHz SCS, and total 5 interlaces as example. Figure 4 give the examples of PUSCH and PUCCH multiplexing for carrier with bandwidth larger than 20MHz. How to make use of the PRBs for potential guardband should be considered if the partial wideband transmission based on LBT result for uplink is supported.

Figure 4 PUCCH and PUSCH multiplexing for wideband

Proposal 4: Multiplexing between PUSCH and PUCCH for a bandwidth larger than 20MHz should be considered.

Flexible PUSCH transmission

In LAA-LTE, partial PUSCH transmission based on the result of the channel access procedure is supported. Although NR supports mini-slot based PUSCH transmission, the finer PUSCH scheduling requires more scheduling overheads especially for continuously scheduled PUSCH. Therefore, it is beneficial to support partial PUSCH transmission based on the LBT result for NR-U with multiple candidate starting positions within one PUSCH, as illustrated in Figure 5. How to determine the candidate PUSCH starting positions can be further discussed.

In addition, CBG-based transmission/retransmission can be considered for flexible PUSCH transmission since it was already supported in NR and it is beneficial to improve transmission efficiency.

Figure 5 Partial PUSCH transmission based on LBT result

Proposal 5: Support partial PUSCH transmission based on the result of the channel access procedure.

- CBG-based transmission/retransmission can be considered for partial PUSCH.

SRS transmission

Unlike LAA-LTE, block-interlaced SRS waveform could be supported in NR-U so that SRS transmission may be multiplexed with SR/PUCCH. Both periodical SRS and triggered SRS can be configured. If the periodical SRS occasion overlaps with the DL transmission in one COT, this occasion can be delayed after gNB finished the COT.

Within a gNB-initiated COT, PUCCH can be transmitted immediately after a gap at the end of a DL burst to reduce HARQ feedback delay. If the gap is no more than 16us, no-LBT is needed. However, to support self-COT feedback, UE needs some processing time to demodulate the PDSCH received at the end of a DL burst, some padding signals are needed before PUCCH transmission. Triggered SRS can be one of these padding signals. Figure 6 illustrates this case.

Figure 6 SRS transmission at the front of PUCCH

Proposal 6: SRS should be enhanced for NR-U.

- Support block-interlaced SRS waveform.

- Support periodical and triggered SRS transmission.

Conclusions

This contribution mainly discusses potential enhancements to UL signals and channels for NR deployment on unlicensed spectrum, including PUCCH design, PUSCH design, flexible starting point for PUSCH transmission due to LBT, SRS enhancement. The following proposals are made.

Proposal 1: Multiple PUCCH resources should be configured based on the corresponding LBT type.

- One PUCCH resource is configured for no-LBT case.

- Multiple PUCCH resources are configured closely in time domain.

Proposal 2: The common starting point of the BWP should be indicated to UE for resource allocation.

Proposal 3: The basic interlace structure of the configured BWP should be indicated to UE for resource allocation.

Proposal 4: Multiplexing between PUSCH and PUCCH for a bandwidth larger than 20MHz should be considered.

Proposal 5: Support partial PUSCH transmission based on the result of the channel access procedure.

- CBG-based transmission/retransmission can be considered for partial PUSCH.

Proposal 6: SRS should be enhanced for NR-U.

- Support block-interlaced SRS waveform.

- Support periodical and triggered SRS transmission.

References

- R1-1906485, “UL signals and channels for NR-U”, OPPO.

- 3GPP TS 38.104.