3GPP TSG RAN WG1 Meeting #98																			R1-1908450

Prague, CZ, August 26th – 30th, 2019

Agenda item:	6.2.2.5

Source:	Samsung

Title:	Discussion on Quality report in Msg3 for NB-IoT

Document for:	Discussion/Decision

# 1. Introduction

Msg3 report on anchor carrier was introduced for Rel-14 NB-IoT UE in order to improve decoding performance during random access procedure. Since the presence of NRS on non-anchor carrier is different compared with anchor carrier, the mechanism of Msg3 report needs to be enhanced to support non-anchor access. In addition, DL quality report in connected mode was introduced in RAN1#83 meeting for both anchor and non-anchor access.

Design of Msg3 report for non-anchor access and in connected mode was discussed in RAN1#97 meeting with the following:

Agreement

For channel quality measurement in Msg3 for non-anchor access, the maximum number of carriers reported in Msg3 is 1.

Agreement

For channel quality report in Msg3 on non-anchor access in idle mode, channel quality measurement on the carrier UE monitors to receive Msg2 (i.e. RAR) is supported.

- FFS: Support of network indication of which carrier to use to report Msg3 (e.g. explicit signaling in SI)

Agreement

For channel quality report in connected mode other than Msg3, RAN1 assumes the following aspects are decided by RAN2 and RAN4.

- number of candidate values for channel quality reports

- mapping value of reported channel quality

- report in RRC message or MAC CE

In this contribution, the essential enhancements of DL quality report for non-anchor access are discussed.

# 2. Discussion

For Rel-14 NB-IoT UEs, DL channel quality reporting was agreed as an optional feature in RAN1#92 and RAN1#92b meeting. However, this feature is only supported for anchor carrier on which UE received Msg2. If the DL carrier used for random access is configured as a non-anchor carrier, the DL quality measured on anchor carrier might be inaccurate. Therefore, support of Msg3 quality reporting for non-anchor access is introduced in Rel-16 NB-IoT.

For enhancement of Msg3 quality reporting of non-anchor in Rel-16, try to reuse the Msg3 reporting mechanism on anchor carrier with essential modification will be a reasonable solution. Similarly, Msg3 quality reporting for non-anchor carrier could carry a hypothetical NPDCCH repetition number (R), which represents the interference from other cells, corresponding to target BLER of 1%. R is derived based on averaging the DL quality during a period of time. The value set of R for anchor carrier can be reused in non-anchor reporting.

The difference of Msg3 reporting for anchor and non-anchor cases is the mechanism of DL quality measurement. If the hypothetical repetition for non-anchor access is based on NRS measurement on anchor carrier, and is derived with the power offset between anchor and non-anchor carriers, the status of interferences from other cells on non-anchor carrier, which might be totally different with interference on anchor carrier, can hardly be reflected. Therefore, DL quality measurement on anchor carrier should be used for non-anchor access.

Observation 1: DL quality of non-anchor carrier derived by NRS measurement on anchor carrier can hardly reflect actual interference condition on non-anchor carrier.

NRS is used as measure resource to derive the required NPDCCH repetition number, but different with on anchor carrier, NRS transmission on non-anchor carrier is not always-on transmission. On non-anchor carrier, UE can only assume NRS transmissions occurred around NPDCCH candidate of paging and Type-2 CSS for RAR, including the subframes occupied by NPDCCH candidate of paging and Type-2 CSS for RAR, and 10 subframes before and 4 subframes after the NPDCCH candidate and CSS.

For UE with capability of generating RRC message after transmission of PRACH, the UE is able to support report measurement of hypothetical repetition number based on NRS transmitted around Type-2 CSS for RAR. Hence Msg3 quality reporting can be always transmitted in every random access procedure, and UE is expected to benefit from support of Msg3 reporting.

On the contrary, for UE which need to generate Msg3 before transmission of PRACH, DL quality measurement based on NRS transmission around CSS for RAR cannot be supported. Therefore, some other candidate resources for NRS measurement need to be defined to allow this type of UE supporting the feature. The following cases for NRS measurement for non-anchor access can be considered:

- Report of R corresponding to measurement on non-anchor carrier on which paging is monitored. NRS will be transmitted before, during and after the NPDCCH candidate of paging, and UE can perform DL quality measurement periodically. In random access procedure, UE report the latest measurement result in Msg3. It is preferred that the non-anchor carrier is the paging carrier for the UE which performs NRS measurement, since no additional measurement is introduced.

- Report of R corresponding to measurement on non-anchor carrier on which the location of NRS is configured by eNodeB. UE could measure NRS based on eNodeB configuration and report the latest measurement result in Msg3. For example, NRS can be configured in every DRX cycle and the location of NRS is configured based on PO or WUS.

Therefore, the following proposal is provided:

Proposal 1: The following carriers are used for DL quality measurement to support Msg3 report for non-anchor access:

- If UE is capable of generating Msg3 after transmission of PRACH:

  - Non-anchor carrier on which RAR is monitored, UE measure subframes containing NRS corresponding to Type-2 CSS for RAR

- Otherwise

  - Non-anchor carrier on which paging is monitored, UE measure subframes containing NRS corresponding to NPDCCH candidate of paging

  - Non-anchor carrier on which the position of NRS is configured by eNodeB

For connected mode, when DL quality is reported needs further discussion. Unlike Msg3 report, obviously it is impossible to support always-on report in every UL transmission. Both periodical DL quality report and triggered DL quality report could be further discussed. For triggered DL quality report, similarly as in MTC, both PHY layer signaling and higher layer signaling can be discussed, e.g. DCI, RAR and MAC-CE based triggering.

For triggered DL quality report in connected mode, the time duration of DL quality measurement needs to be further studied. For PHY signaling based triggering, one possible solution is to introduce a set of larger scheduling value as measurement gap. For higher layer signaling based triggering, a report window could be defined by RAN2.

In addition, the NRS presence during DL quality measurement for non-anchor report in connected mode needs further study. If NRS presence cannot be guaranteed for DL quality measurement corresponding to connected mode report, it might be necessary to introduce a periodical measurement, and the power consumption needs further evaluation.

Proposal 2: For DL quality report in connected mode, the triggering signal is down-selected between:

- DCI

- MAC CE

- RAR

Proposal 3: NRS presence for DL quality measurement should be studied to support DL quality measurement on non-anchor in connected mode.

# 3. Conclusion

Based analysis above, the following proposals are provided:

Observation 1: DL quality of non-anchor carrier derived by NRS measurement on anchor carrier can hardly reflect actual interference condition on non-anchor carrier.

Proposal 1: The following carriers are used for DL quality measurement to support Msg3 report for non-anchor access:

- If UE is capable of generating Msg3 after transmission of PRACH:

  - Non-anchor carrier on which RAR is monitored, UE measure subframes containing NRS corresponding to Type-2 CSS for RAR

- Otherwise

  - Non-anchor carrier on which paging is monitored, UE measure subframes containing NRS corresponding to NPDCCH candidate of paging

  - Non-anchor carrier on which the position of NRS is configured by eNodeB

Proposal 2: For DL quality report in connected mode, the triggering signal is down-selected between:

- DCI

- MAC CE

- RAR

Proposal 3: NRS presence for DL quality measurement should be studied to support DL quality measurement on non-anchor in connected mode.

# Reference

[1] RP-191576, Revised WID of Additional enhancements for NB-IoT

[2] Chairman’s note of RAN1#97

[3] R1-1906899, Discussion on quality report in Msg3 for NB-IoT, Samsung