3GPP TSG RAN WG1 #98                                             R1-1908458

Prague, CZ, August 26th – 30th, 2019

Agenda item:	7.1.6

Source:	Samsung

Title:	Clarification on simultaneous signaling of FG 3-5a and FG 3-5b

Document for:	Discussion and Decision

# Introduction

In this contribution, we provide our views on simultaneous signaling of FG 3-5a and FG 3-5b as UE capabilities.

# Clarification on FGs 3-5a and 3-5b

In RAN1 96bis meeting [1], FG 3-5b is updated as follows:

Agreements: Update “Feature component” of FG 3-5b as below: PDCCH monitoring occasions of FG-3-1, plus additional PDCCH monitoring occasion(s) can be any OFDM symbol(s) of a slot for Case 2, and for any two PDCCH monitoring occasions belonging to different spans, where at least one of them is not the monitoring occasions of FG-3-1, in same or different search spaces, there is a minimum time separation of X OFDM symbols (including the cross-slot boundary case) between the start of two spans, where each span is of length up to Y consecutive OFDM symbols of a slot. Spans do not overlap. Every span is contained in a single slot. The same span pattern repeats in every slot. The separation between consecutive spans within and across slots may be unequal but the same (X, Y) limit must be satisfied by all spans. Every monitoring occasion is fully contained in one span. In order to determine a suitable span pattern, first a bitmap b(l), 0<=l<=13 is generated, where b(l)=1 if symbol l of any slot is part of a monitoring occasion, b(l)=0 otherwise. The first span in the span pattern begins at the smallest l for which b(l)=1. The next span in the span pattern begins at the smallest l not included in the previous span(s) for which b(l)=1. The span duration is max{maximum value of all CORESET durations, minimum value of Y in the UE reported candidate value} except possibly the last span in a slot which can be of shorter duration. A particular PDCCH monitoring configuration meets the UE capability limitation if the span arrangement satisfies the gap separation for at least one (X, Y) in the UE reported candidate value set in every slot, including cross slot boundary. For the set of monitoring occasions which are within the same span:  Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for FDD  Processing one unicast DCI scheduling DL and two unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD  Processing two unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD The number of different start symbol indices of spans for all PDCCH monitoring occasions per slot, including PDCCH monitoring occasions of FG-3-1, is no more than floor(14/X) (X is minimum among values reported by UE). The number of different start symbol indices of PDCCH monitoring occasions per slot including PDCCH monitoring occasions of FG-3-1, is no more than 7. The number of different start symbol indices of PDCCH monitoring occasions per half-slot including PDCCH monitoring occasions of FG-3-1 is no more than 4 in SCell.

Based on the component description of the updated FG 3-5b, UE can report one of {(7,3)}, {(4,3), (7,3)}, {(2,2), (4,3), (7,3)} as candidate value set {(X,Y)} and the PDCCH monitoring configuration should comply with the reported span gap limitation.

There is another PDCCH monitoring capability described in FG 3-5a which constrains a symbol-based gap between two DL unicast DCIs, between two UL unicast DCIs, or between a DL and an UL unicast DCI in different monitoring occasions. This essentially provides alternative trade-off to FG 3-b in terms of network flexibility and UE complexity. Under the current capability signalling, a UE can report both FG 3-5a and FG 3-5b as the PDCCH monitoring capabilities. However, it is not clear how gNB should interpret if a UE reports FG 3-5a and FG 3-5b simultaneously. One possible interpretation for gNB is to have a PDCCH monitoring configuration which complies with both the reported FG 3-5a and FG 3-5b capabilities at the same time, i.e., DCI gap and span gap are satisfied in the PDCCH monitoring configuration. Such stringent requirement severely limits the configuration flexibility of gNB and also deviates from the point of having 2 different trade-offs. Therefore, we propose the following clarification when UE reports FG 3-5a and FG 3-5b simultaneously.

Proposal 1: Clarify the interpretation of simultaneous signaling of capabilities FG 3-5a and FG 3-5b from a UE as the UE is capable of monitoring PDCCH with the reported DCI gap restriction defined in FG 3-5a or the span gap defined in FG 3-5b.

Given the fact that signalling both 3-5a and 3-5b should be reasonably understood as a UE being capable of supporting these two functionalities and given the fact that they represent different trade-offs, we think the clarification proposed above is quite a natural understanding. In that sense, we think that the spec update would not be necessary, and confirming RAN1’s common understanding would be enough.

Based on the proposal, when a UE reports FG 3-5a and FG 3-5b as the PDCCH monitoring capabilities, the expected PDCCH monitoring configuration to UE should satisfy either the DCI gap restriction or the span gap restriction. Consequently, UE is also able to determine which restriction is applied to the current PDCCH monitoring configuration, which is likely necessary for a UE to prepare its operation for different trade-offs. For example, if a UE reports FG 3-5a and FG 3-5b simultaneously and the PDCCH monitoring configuration satisfies the reported span gap limitation, then the UE will assume no DCI gap restriction is applied between two unicast DCIs. On the other hand, if the PDCCH monitoring configuration doesn’t fulfill any of the reported span gap limitation, then the UE will assume the DCI gap restriction is applied between two unicast DCIs.

# Conclusions

To resolve the potential ambiguity to gNB on the interpretation of simultaneous signaling of FG 3-5a and FG 3-5b, we made a proposal described below.

Proposal 1: Clarify the interpretation of simultaneous signaling of capabilities FG 3-5a and FG 3-5b from a UE as the UE is capable of monitoring PDCCH with the reported DCI gap restriction defined in FG 3-5a or the span gap defined in FG 3-5b.

# References

[1] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting 96bis.