3GPP TSG RAN WG1 #98                                             R1-1908459

# Prague, CZ, August 26th – 30th, 2019

Agenda item:	7.2.1.1

Source:	Samsung

Title:	Channel Structure for Two-Step RACH

Document for:	Discussion and Decision

# Introduction

A new work item on “2-step RACH for NR” was approved in RAN#82 [1] and the objectives of this work item for physical layer are identified as follows:

- 2-step RACH [RAN1, RAN2]

- 2-step RACH shall be able operate regardless of whether the UE has valid TA or not.

- 2-step RACH is applicable to any cell size supported in Rel-15 NR;

- 2-step RACH is applied for RRC_INACTIVE , RRC_CONNECTED and RRC_IDLE state

- Specify contention-based 2-step RACH procedure (RAN2)

- Channel structure of msgA is Preamble and PUSCH carrying payload (RAN1)

  - Only reuse the Rel-15 NR PRACH Preambles design.

  - Only reuse the Rel-15 NR PUSCH including Rel-15 DMRS for transmission of payload of msgA)

    - No new CP length and no sub-PRB guard subcarrier(s)

Note 1: The above sub-bullet is to ensure that signal structure optimizations for any specific cell size (e.g. cells with RTT larger than Rel-15 PUSCH CP duration) are not pursued.

  - Specify the mapping between the PRACH preamble and the time-frequency resource of PUSCH in msgA+ DMRS

    - PRACH Preamble and PUSCH in a msgA is TDMed

  - Specify the supported MCS(s) and time-frequency resource size(s) of PUSCH in msgA

    - Consider the msgA payload contents determined by RAN2

  - Specify power control of PUSCH of msgA

- Specify msgA’s content: to include the equivalent contents of msg3 of 4-step RACH (RAN2/RAN1)

  - Inclusion of UCI in msgA is not precluded

- Specify msgB’s content: to include the equivalent contents of msg2 and msg4 of 4-step RACH (RAN1/RAN2)

- Contention resolution for 2-step RACH (RAN2)

- Design of RNTI for msgB of 2-step RACH (RAN2)

- Specify the fall back procedure from 2-step RACH to 4-step RACH (RAN2/RAN1)

- All triggers for Rel-15 NR 4-step RACH are applied for 2-step RACH except for SI Request and BFR which are up to RAN2 discussion

  - No new triggers for 2 step RACH

In RAN1#96bis meeting [2] & 97 meeting [3], some agreements are made for channel structure of 2-step RACH which will be discussed detailed in the paper. This contribution discusses the channel structure design for 2step RACH, including the resource configuration for both RACH part and PUSCH part, and the association between RO and PUSCH.

# Resource configuration for two-step RACH

For two-step RACH, two aspects of resource configuration needs to be considered for MSG.A transmission, one is the RACH resource including the PRACH occasion (denoted as 2step RO) and the preamble; the other is the PUSCH resource including the time-frequency resource and the DMRS resources. In addition, how to associate the selected RACH resource and the available PUSCH resource is also an important issue to tackle.

## RACH resource configuration

As aforementioned, UE needs to determine the available 2step RO and the corresponding preambles. First of all, since the WID has restricted that “Only reuse the Rel-15 NR PRACH Preambles design” and “All triggers for Rel-15 NR 4-step RACH are applied for 2-step RACH except for SI Request and BFR which are up to RAN2 discussion”, there is no motivation to introduce new PRACH configuration table/entries.

Observation 1: it’s enough to reuse the current PRACH configuration table for 2step RO configuration.

Agreements:

- For the relation of PRACH resources between 2-step and 4-step RACH, the network has the flexibility to configure the following options:

  - Option 1: Separate ROs are configured for 2-step and 4-step RACH

  - Option 2: Shared RO but separate preambles for 2-step and 4-step RACH

In last meeting, the following agreement on the PRACH resource for 2step RACH has been agreed. Generally, PRACH resources for 2-step RACH could have separate ROs or separate preambles within the shared ROs to these for 4-step RACH.

Regardless of the option 1 or option 2, 2-step RACH PRACH configuration index could share that for 4-step RACH, i.e., both use the same preamble format. The different part is that, the additional indication of the ROs used for 2-step RACH should be given. For option 1, gNB needs to indicate the frequency domain starting position and the # of FDMed ROs for the 2step RACH. For the starting position, it could use either directly a msgA preamble F-domain starting position in RB index or using an F-domain offet towards the end of last 4step RACH FDMed RO as shown in Fig. 1(a). For option 2, when gNB configures the 4step RACH ROs, it also indicates which parts are also shared to 2step RACH in time domain and/or frequency domain. One example is given in Fig. 1(b), the gNB indicates that last 2 FDMed ROs are shared to 2step RACH.

Fig.1 (a) example of separate 2/4step RACH ROs          Fig.1 (b) example of shared 2/4step RACH ROs

Proposal 1: 2-step RACH and 4-step RACH use the same PRACH configuration index indicated by gNB.

## PUSCH resource configuration

PUSCH waveform and numerology

Agreements from RAN1# 96:

- Both DFT-s-OFDM and CP-OFDM are supported for the payload transmission in msgA

  - FFS how to indicate/configure the waveform

- Consider the following numerology for msgA PUSCH (for possible down-selection)

  - Alt 1: ​follow the numerology configured for the UL BWP

    - FFS initial vs. active UL BWP

  - Alt 2: same as msgA preamble numerology at least for some cases

    - E.g., when short preamble is used (L=139)

Agreements from RAN1#96bis:

- Support the PRACH and PUSCH for msgA transmission in different slots. In this case, the numerology for msgA PUSCH follow the numerology configured for the UL BWP for msgA transmission.

  - FFS whether to support PRACH and PUSCH in the same slot for msgA transmission. If supported, down-select from the following option

    - Opt 1: the numerology for msgA PUSCH follows that of msgA preamble

    - Opt 2: gNB configure whether the numerology for msgA PUSCH follows that of msgA preamble or UL BWP

    - Opt 3: a UE is not expected to be configured with different numerology among PRACH preamble, msgA PUSCH and UL BWP for msgA transmission

    - Note: in Rel.15 the PRACH and PUSCH transmitted in the same slot for a UE are not supported

For determining the waveform and numerology of the PUSCH in 2step RACH msgA, several FFS points are left as listed below.

One is “how to indicate/configure the waveform”, since initial access is at least happened in initial active UL BWP, the waveform indication could follow the msg.3 waveform indication (which is the same as the waveform for initial active UL BWP).

Proposal 2: waveform indication for msgA PUSCH should follow msg.3 waveform indication.

The PRACH and PUSCH for msgA transmission in different slots were agreed to support in last meeting. However, since the preamble SCS might be different from the msgA PUSCH (which equals the UL BWP SCS), so the slot definition should be clarified. As followed by the rules used in many places in Rel-15, such slot could be defined by the min{preamble SCS, UL BWP SCS} to give enough budget for implementation.

Proposal 3: Further clarify the previous agreements “Support the PRACH and PUSCH for msgA transmission in different slots” by adding “where the slot is defined by the min{preamble SCS, UL BWP SCS}.”

One leftover issue is that whether to support the PRACH and PUSCH in the same slot. In Rel-15, the transmission of PRACH and other UL signals in the same slot or within a defined gap are not supported; UE will determine which one to transmit. In addition, for 2step RACH, gNB usually needs to detect the preamble(s) thus decides to decode which PUSCH occasion, if the PRACH and PUSCH are too close to each other, gNB may need buffer all the data due to the insufficient time to process. This will not only require more costs on the hardware or operation overhead, but also potentially degrade the accuracy of the reception of the PUSCH, e.g., gNB cannot know which Rx beam is good enough to receive the PUSCH since the preamble detection has not yet been successfully completed. Thus, the PRACH and PUSCH for msgA transmission in the same slot are not supported, where the slot is also defined by the min{preamble SCS, UL BWP SCS}.

Proposal 4: PRACH and PUSCH for msgA transmission in the same slot are not supported, where the slot is defined by the min{preamble SCS, UL BWP SCS}.

PUSCH resource unit definition

Agreements from RAN1#96bis:

- One or more PUSCH occasion(s) within an msgA PUSCH configuration period are configured.

  - FFS msgA PUSCH configuration period, e.g.

    - For opt. 1 with separate PUSCH configuration, msgA PUSCH configuration period may or may not be the same as PRACH configuration period

    - For opt. 2 PUSCH configuration with relative location, msgA PUSCH configuration period is the PRACH configuration period

The msgA PUSCH configuration period was introduced in last meeting. A msgA PUSCH configuration period is a time period where the configured PUSCH resource for 2step RACH are located in, the configured PUSCH resource for 2step RACH will be repeated from one msgA PUSCH configuration period to another. Such definition is similar as the PRACH configuration period. Thus, for opt. 1 with separate PUSCH configuration, msgA PUSCH configuration period will be configured by gNB, and it may or may not be the same as PRACH configuration period. However, msgA PUSCH configuration period could use same value range of that for PRACH configuration period, which is 10,20,40,80,160ms. While for opt. 2 PUSCH configuration with relative location the PRACH, msgA PUSCH configuration period is same as the PRACH configuration period

Proposal 5: Value range of msgA PUSCH configuration period is also {10, 20, 40, 80, 160} ms.

Agreements from RAN1#96bis:

- PUSCH resource unit for 2-step RACH is defined as

  - The PUSCH occasion and DMRS port / DMRS sequence used for an msgA payload transmission.

    - FFS support only one or both of DMRS port / DMRS sequence

    - The DMRS sequence generation mechanism should follow Rel.15.

Proposals from RAN1#97:

- For the definition of PRU, support both DMRS ports and (working assumption) DMRS sequences at least for CP-OFDM

  - FFS how to support multiple sequences for DFT-s-OFDM

  - FFS the maximum number of sequences

For the definition of PUSCH resource unit, one FFS point is that whether Port only or both DMRS port/DMRS sequences will be supported. Different DMRS port, as defined in current release, could have different cyclic shifts, OCC and/or Comb configuration; while the different DMRS sequence will require gNB to reserve a group of scrambling ID, which currently is using Cell ID for scrambling ID before RRC connected. The so claimed benefit of using scrambling ID is that it might save time/frequency resource by having larger capacity of DMRS resource than that of using the DMRS port.

However, we think only the DMRS port to be supported is enough for the following reasons:

- Benefits to support different DMRS sequences are not convincing.

- Drawbacks and burdens to support DMRS sequences are clear.

From the discussions of previous meetings, the logic of supporting DMRS sequence is that if a relatively large number of preambles (e.g., 64 preambles) are assigned to 2step RACH and if the 1-1 mapping is needed to achieve a relatively low collision probability, a large number of PRUs should be reserved, e.g., 64 PRUs. Then in order to save the T/F resource, the number of DMRS resource per T/F resource needs to be increased. However, such logic cannot apply to the scenario which the 2step RACH targets to. According to our simulation assumption, the simultaneous active UE # is one or two, let’s take 2 as example since 1 will have no collision at all. Besides, let’s assume maximum preamble number, i.e., 64, is assigned to 2step RACH, since if the # of preambles is small, it’s also not motivated for having much larger # of PRUs even considering 1-1 mapping. Then the potential combination of the resource (DMRS and PRU) and the mapping ratio are given in below table as well as the expected BLER performance,

Table 1 – comparison between DMRS Port only and DMRS Port + DMRS Sequence

| Solution | Preamble # | Mapping ratio | PRU # | DMRS # (port*sequence) | Expected BLER |
| --- | --- | --- | --- | --- | --- |
| Port+Sequence | 64 | 1-1 | 1 | 12*6 | [1-(1/64)2]* Pdetection*BLERPS ~0.999*… |
|  |  |  | 2 | 12*3 |  |
|  |  |  | 4 | 12*2 |  |
| Port-only | 64 | 1-1 | 6 | 12*1 | [1-(1/16)2]* Pdetection*BLERSP |
|  |  | 2-1 | 3 |  |  |
|  |  | 4-1 | 2 |  | ~0.996*… |
|  |  | 6-1 | 1 |  | ~0.984*… |

where the BLERPS and BLERSP are represented by the BLER performance of “same DMRS port but different DMRS sequence” and “different DMRS port but same DMRS sequence”, respectively.

First, for the fair comparison, we choose the one case that same # of PRU is used by both solutions, i.e., PRU # = 2. In this case, the Port+Sequence solution still uses 1-1 mapping while using 3 different DMRS sequences and the Port-only solution uses 1 DMRS sequence but to apply 4-1mapping between preambles and PRU. Then we compare the expected BLER performance with assumption that if two UEs are collided in PRU, both of them are not decoded:

BLERexpected=Pnon-collided*Pdetected*BLER.

- Pnon-collided , the non-collision probability. For Port+Sequence solution, the Pnon-collided is 1-(1/64)2≈0.999 while such value for Port-only solution is 1-(1/16)2≈0.996. It is the clearly that such probability is almost identical to each other. Thus, it can conclude that the preamble collision probability is not the bottleneck of the expected BLER performance of 2step RACH. Even for the case of 6-1 mapping ratio in Port only solution, the non-collision probability could reach 0.984.

- Pdetected, the detection probability of the preamble. We assume this is same for both solutions.

- BLER, PUSCH decoding performance of each solution. In the following, we compared 2 cases: 1 DMRS port with 2 DMRS sequences and 2 DMRS port with 1 DMRS sequences. The BLER performance is shown in below figure, more results could find in [4]. The observed BLER performance loss could be 0.7~1.5 dB.  Clearly, the BLER performance has larger (negative) impact compared to the preamble collision probability.

Fig.2 comparison of BLER performance of 2 solutions.

Note that in the simulation the 1 DMRS port with 2 DMRS sequences might be considered as the worst case of the configuration using DMRS port + sequence, since it assumes the two UEs will always use different sequence, while in practice, there is also some possibility they could select the different port and same sequence. But the best performance they can achieve as same as the port only solution. Thus, the port + sequence solution at least has no benefits compared to port only solution.

Observation 2: the BLER performance of each DMRS solution is the bottleneck of the expected BLER performance of 2step RACH rather than the preamble collision probability.

Observation 3: the benefits to support different DMRS sequences in PRU definition are not convincing.

In addition, from speciation work point of view, in order to support DMRS sequence in the PRU definition, we have to study the selection of the particular scrambling ID to be reserved in each cell by not only considering the performance within its own cell but also the impact on the inter-cell interference management. What’s more, the separate consideration of two different waveforms, i.e., CP-OFDM and DFT-s-OFDM, has to be pursued as already be indicated by the proposal from last meeting.

Observation 4: the drawbacks and the burdens to support DMRS sequences are clear.

Proposal 6: For the definition of a PUSCH resource unit, only DMRS port is supported.

As commonly understood, a preamble (in a given 2step RO) should be associated with at least one PUSCH resource unit (including a PUSCH T/F resource unit and a DMRS port). The same PUSCH T/F resource unit with different DMRS ports is considered as different PUSCH resource units. Thus by selecting a preamble, UE could find the available PUSCH resources. For example, a PUSCH resource unit for 2step RACH could be introduced as shown in following figure, which could include two potential parts: guard period and guard band. The guard period is used for handling the potential inter-symbol interference (ISI) since the normal CP of the OFDM symbols may not cover the RTT if it’s a relatively large cell, thus a guard period similar to preamble format might be needed. The guard band is introduced due to the similar reason to handle the potential inter-subcarrier interference (ICI) between UEs. Since the 2step RACH needs to support the UEs in RRC idle or inactive mode, the configuration of PUSCH will be broadcasted in the system information, so that it leaves limited room for very flexible configuration of PUSCH.

Fig. 3 an example of PUSCH resource unit for 2step RACH

Proposal 7: a PUSCH resource unit should support including a GP and/or a GB, which can be configured by gNB on a need basis.

Agreeents from RAN1#96bis:

- PUSCH occasion for 2-step RACH is defined as

  - the time-frequency resource for payload transmission associated with a PRACH preamble in msgA

- Consider the following methods for PUSCH occasion of msgA transmission:

  - Opt 1: PUSCH occasions are separately configured from PRACH occasions

    - For one PUSCH occasion, it is derived based on:

      - Alt 1: reuse the resource allocation for NR configured grant in principle

      - Alt 2: other potential configurations (e.g., reuse semi-static SFI + BWP, reuse PRACH RO, etc.)

    - FFS detailed association rule between the PRACH and PUSCH for msgA transmission

  - Opt 2: Specify/configure the relative location (in time and/or frequency) of the PUSCH occasion with respect to the associated PRACH occasion

    - Alt 1: Time/frequency relation between PRACH preambles in PRACH occasion(s) and PUSCH occasions are single specification fixed value.

    - Alt 2: Time/frequency relation between each PRACH preamble in PRACH occasion(s) to the PUSCH occasion is single specification fixed value. Different preambles in different PRACH occasions can have different values.

    - Alt 3: Time/frequency relation between PRACH preambles in PRACH occasion(s) and PUSCH occasions are single semi-statically configured value.

    - Alt 4: Time/frequency relation between each PRACH preamble in PRACH occasion(s) to the PUSCH occasion is semi-statically configured value. Different preambles in different PRACH occasions can have different values.

    - Note: The time and frequency relation is not required to be the same alternative.

  - FFS detailed mapping between preamble and PUSCH resource + DMRS

Agreements from RAN1#97:

- The following parameters are defined per msgA PUSCH configuration:

  - Common parameters for both option 1 (separate configuration) and option 2 (relative location), at least include:

    - MCS and/or TBS (to be further decided)

    - Number of FDMed POs

      - POs (including guard band or guard period, if exist) under the same msgA PUSCH configurations are consecutive in frequency domain

    - Number of PRBs per PO

    - Number of DMRS symbols/ports/sequences (if support) per PO

    - FFS whether or not support repetitions for msgA PUSCH

    - FFS bandwidth of PRB-level guard band or duration of guard time

    - FFS PUSCH mapping type

As discussed in the RAN1#96, two options are considered for configuring the PUSCH resource in 2-step RACH. One is separately configuring from PRACH resource. By the indicated two alternatives for this option, the outcome of the PUSCH configuration is that such PUSCH resource will have its own T/F configuration information for indicating the time/frequency position and periodicity. In addition, the time/frequency position and periodicity are independent from the RACH configuration, e.g., its periodicity is not related to RACH configuration periodicity. In sum, by this configuration information, a UE could have idea of what the PUSCH resource pattern for 2-step RACH looks like. This is similar to the RACH resource configuration, by which UE will have the image of what the RO patterns for 2step RACH looks like. Then this option will request to additionally define the mapping between PUSCH resource and RACH resources. The other option is trying to link the PUSCH resource to the 2step RO, which could do the PUSCH resource configuration/determination and association between PUSCH and RACH resource at the same time.

Association between SSB and PUSCH

Agreements:

- The beam association rule between SSB and RACH occasion of 4-step RACH is to be used for 2-step RACH

  - FFS beam association for PUSCH

For option 1, since the PUSCH is independently configured from RACH resource, it might be the cases that the periodicity of the PUSCH doesn’t equal that of PRACH, so for example it could be in 40ms, there are 4 PRACH configuration periodicities but only one PUSCH configuration periodicity or vice versa. Thus, it might be very difficult for associate the PRACH to PUSCH due to such flexibility. However, the RACH resources are already associated with SSB, thus if the association between SSB and PUSCH is also created, the association between RACH resource and PUSCH resource could be defined for both resources associated to the same SSB index, which will be discussed more detailed in the next section. To create such association between SSB and PUSCH, the similar association rule as that for SSB and RACH could be reused. For example, the PUSCH T/F resource unit and the DMRS ports on it could be regarded as RACH occasion and the preambles on it. Thus, by defining “SSBperPUSCH” as similar to “ssb-perRACH-Occasion” and also “DMRS-PortsPerSSB” as similar to “CB-PreamblesPerSSB”, we can determine the mapping pattern between SSB and PUSCH.

Proposal 8: Association between SSB and PUSCH could use similar association rule between SSB and RACH.

Agreements from RAN1#97:

- The following parameters are defined per msgA PUSCH configuration:

  - Parameters specific to option 1, at least include:

    - Periodicity (msgA PUSCH configuration period)

      - FFS value range

    - Offset(s) (e.g., symbol, slot, subframe, etc.)

    - Time domain resource allocation, details FFS, e.g., in a slot for msgA PUSCH: starting symbol, number of symbols per PO, number of time-domain POs, etc.

    - Frequency starting point

  - Parameters specific to option 2, at least include:

    - Single time offset (combination of slot-level and symbol-level indication) with respect to a reference point

      - FFS, e.g., each PRACH slot (e.g., start or end of the PRACH slot), etc.

    - Number of symbols per PO

      - FFS explicit or implicit indication

    - Single frequency offset with respect to FFS (the start of the first RO in frequency or the end of the last RO in frequency)

  - FFS: Number of TDMed POs

Some FFS points are left from last meeting.

- For the value range of the msgA PUSCH configuration period in option 1, we think it could have same value range of the PRACH configuration period. However, during configuration, the PUSCH configuration period is not smaller than the PRACH configuration period, otherwise, it might have 2 ROs from different PRACH configuration period mapped to the same PUSCH then the gNB may have ambigulity to identify which estimated time offset to apply to the PUSCH detection.

- For the reference point applied to the offset in option 2. We have more detailed discussion in the next section. However, one point to emphasize is that the motivation to support the start of the RACH slot in order to support preamble and PUSCH in the same slot for NR-U case is not solid. First, for NR-U case, the preamble and pusch doesn’t have to be consecutive to have some benefits on LBT, e.g., the PUSCH could be in the COT initiated by the preamble LBT thus a CAT2 (shot LBT) could be applied. Thus, the preamble in NRU is consecutive in F-domain while the PUSCH in NRU is interlaced structure, anyway putting two channels are not so attractive.

Agreements from RAN1#97:

- Support multiple msgA PUSCH configurations for a UE

  - FFS the maximum number of configurations

  - FFS which parameters, if any, are common for all configurations

  - FFS indication of different msgA PUSCH configurations, e.g. by different ROs, by different preamble groups, or by UCI

  - FFS whether or not resources for different msgA PUSCHs can be overlapped in time-frequency, and if so, any spec impact

For Number of TDMed POs. we understand that for option 1, it might need to indicate separate offset for each PO, thus it might be an issue of the indication overhead. While for the option 2, it might be easy, the offset could indicate the starting position of the PO, and UE could determine the rest of the PO by predefined order.

Last meeting, it is agreed that the multiple msgA PUSCH configuration for a UE is supported. Since the Rel-16 2step RACH targets for non-UP data transmission at least for idle/inactive state, so that too much flexibility of the “multiple” is not necessary. Following the rules in the LTE and Rel-15, which have preamble grouping for different msg3 size and pathloss condition, i.e., group A and group B, we can use group A and group B to map to different TBS/MCS configuration. By doing so, it could provide a certain level of flexibility to gNB to configure the PUSCH resource size to accommodate different data size level, while not increase the blind detection burden too much.

Proposal 9: preamble grouping based solution similar to Rel-15 NR, i.e., group A and group B, can be used to map to indicate different msgA PUSCH configurations, e.g., different TBS/MCS configuration.

## Mapping between RACH resource and PUSCH resource

One important issue to solve is the mapping between RACH resource and PUSCH resource. From the selected 2step RO, UE should be able to find the corresponding position of the PUSCH transmission. First, there should be a gap between 2step RO and the corresponding PUSCH for two reasons. One is that it usually takes some time for gNB to detect the possible transmitted preamble in the 2step RO, and it’s beneficial for gNB to know this information (which preambles are detected) so that it could know in which corresponding PUSCH there might be a real transmission. Thus gNB doesn’t need to try all the PUSCH detection and decoding. The other reason is that in case of multi-beam operation, there is a case that gNB is performing Rx beam sweeping during preamble detection, once it detects one preamble with one suitable beam, it could use the same Rx beam for the PUSCH reception.

Observation 5: it’s beneficial for gNB to detect the preamble first before reception of the PUSCH.

Working assumption:

- At least support one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit.

  - Configurable number of preambles (including one or multiple) mapped to one PUSCH resource unit

  - FFS one-to-multiple mapping

- Companies are strongly encouraged to perform additional evaluations/analysis

One-to-multiple mapping is left for FFS based on last meeting’s working assumption, which supports the one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit. There could be two understanding for one-to multiple mapping:

- One preamble in a RO maps to multiple PUSCH occasions;

- One preamble in a RO maps to one PUSCH occasion but multiple DMRS ports.

Considering usually the bottleneck for the 2-step RACH is the PUSCH resources, both understandings of the one-to-multiple mapping will require more PUSCH resource (either T/F resource or DMRS port) than the corresponding preambles, which we think is not necessary.

Proposal 10: one-to-multiple mapping between preambles in each RO and associated PUSCH resource unit are not supported.

Discussion on option 1

As aforementioned, for option 1, if the association between SSB and PUSCH are created, a UE could have the mapping pattern between SSB and RACH and mapping pattern between SSB and PUSCH, e.g., the mapping circle definition between SSB and RO could also apply to SSB and PUSCH. Because the number of RACH resources (ROs and preambles) associated to one SSB in one SSB-RO mapping circle and the number of PUSCH resource units (PUSCH T/F resource units and DMRS ports) associated to one SSB in one SSB-PUSCH mapping circle are the same across all SSBs for a given configuration, respectively. Thus, it will be very convenient to define the association between RACH resource and PUSCH resource these are associated to the same SSB index.

Fig. 4 an example of defining association between RACH and PUSCH associated to the same SSB index

Observation 6: it is beneficial to define the association between RACH resource and PUSCH resource these are associated to the same SSB index.

Proposal 11: the association rule should be defined between RACH resource and PUSCH resource associated to the same SSB index.

By doing so, the flexibility of configuring the RACH resource and PUSCH resource are still keeping for gNB, while UE only needs to care about the SSB index it selected, and then the available RACH resource and PUSCH resource in every mapping circle could be determined. Then by determining the preamble-per-PUSCH-resource-unit, a UE could easily find the PUSCH resource corresponding to the preamble/RO it selected. For example, in Fig. 5, N_preamble(perSSB)=32, N_pusch(perSSB)=8 and the DMRS_id ∈{0~11} (i.e., 12 DMRS ports are available for one PUSCH T/F resource unit), if one preamble maps to one PUSCH resource unit, then totally 64 preambles (in 2 ROs) will map to the 64 PUSCH resource units. In this example, there will be 8*12=96 PUSCH resource units available. Thus 32 PUSCH resource units will be left over without any preamble mapping to it. This is similar handling for these left over ROs cannot map to the SSBs in Rel-15 NR. Then by selecting one preamble in one RO (i.e., one P_id), UE could determine the corresponding PUSCH T/F resource unit (i.e., TF_id) and DMRS port (i.e., DMRS_id).

Fig. 5 an example of association rule between RACH and PUSCH associated to the same SSB index

Discussion on option 2

If using option 2, it’s better to allocate the PUSCH resource following the PRACH configuration periodicity to avoid complicated time/frequency relation between PRACH and PUSCH. It’s also inefficient to indicate the PUSCH resource for each 2step RO individually. Since a PUSCH resource unit for 2step RACH should have same time/frequency resource, it’s enough to indicate the starting position on time-domain and frequency domain of the first PUSCH corresponding to a reference point. There are several options for the reference point:

- The selected RO,

- The last RO of the RACH slot where the selected RO located,

- The last RO of the RACH frame where the selected RO located.

The pros and cons of the above 3 options for the reference point are actually the balance among the access delay for 2-step RACH, scheduling limits and signaling overhead. The option 1 will could have relative lowest access delay the most scheduling limits (use single gap value for all RO) or signaling overhead (use separate gap value for each RO); while the option 3 may have the largest access delay, in some case the first RO (located in the front of the RACH frame) will have to wait almost 10ms for its PUSCH transmission. Thus the option 2 is a more balanced choice.

Proposal 12: the gap value is configured between the last RO of the RACH slot where the selected RO located and PUSCH.

Such starting position information could be determined by alt.3 for option 2, e.g., a time-domain gap between the last symbol of the 2step RO and the start of the first PUSCH resource unit and similar for the frequency domain gap. The rest PUSCH unit corresponding to the 2step RO(s) could be determined by UE via some pre-defined rules, like frequency domain consecutive mapping. Thus, for a simply one-2-one mapping, the 2step RO could be mapped to the PUSCH resource.

Proposal 13 At least using a single semi-statically configured value for time/frequency relation between PRACH preambles in PRACH occasion(s) and PUSCH occasion(s) (alt.3 for option 2) should be supported if option 2 is adopted.

- If MU-MIMO is not enabled in this case, the UE should map the preamble index to the PUSCH for 2step RACH, e.g., preamble 0 for this 2step RACH maps to the first PUSCH, preamble 1 maps to the second PUSCH, and so on. As shown in the following figure.

Fig. 6 - illustration of mapping between preamble and PUSCH

- If the MU-MIMO is enabled, the DMRS port index could be used to further group the preambles in the 2step RO. For example, if 12 DMRS ports are allowed in one PUSCH, we can group a 12-preamble group map to the same PUSCH but different DMRS ports (which is with different cyclic shift, OCC or Comb) as shown in the following figure.

Fig. 7 - Illustration of mapping between preamble and PUSCH and DMRS

Scrambling ID for PUSCH transmission

Agreements:

- The c_init for msgA PUSCH scrambling is at least derived based on a RNTI, preamble index, and/or n_ID (which can be  cell ID or configurable, to be FFS).

  - FFS details of the RNTI

  - FFS the inclusion of DMRS index.

For PUSCH transmission, it needs a scrambling ID in the bit level processing, and such ID is usually the C-RNTI or TC-RNTI in Rel-15 NR. For the idle or inactive UE, it may not have a valid C-RNTI. So which RNTI should be used as the scrambling ID is an issue to be solved. One possible solution could be using a modified version of RA-RNTI incorporate with preamble index or DMRS port index (only useful in case 1-N mapping between preamble and PUSCH). In this case, both gNB and UE could have clearly understanding on the Tx and Rx processing, different UEs selecting different ROs/preambles will not collide on this scrambling ID.

Proposal 14: the scrambling ID for PUSCH transmission in 2step RACH Msg A could be derived based on legacy RA-RNTI and preamble index.

# Conclusion

The proposals made in this contribution are summarized below:

Observation 1: it’s enough to reuse the current PRACH configuration table for 2step RO configuration.

Observation 2: it’s beneficial to separate the 2step RACH and 4step RACH.

Observation 2: the BLER performance of each DMRS solution is the bottleneck of the expected BLER performance of 2step RACH rather than the preamble collision probability.

Observation 3: the benefits to support different DMRS sequences in PRU definition are not convincing.

Observation 4: the drawbacks and the burdens to support DMRS sequences are clear.

Observation 5: it’s beneficial for gNB to detect the preamble first before reception of the PUSCH.

Observation 6: it is beneficial to define the association between RACH resource and PUSCH resource these are associated to the same SSB index.

Proposal 1: 2-step RACH and 4-step RACH use the same PRACH configuration index indicated by gNB.

Proposal 2: waveform indication for msgA PUSCH should follow msg.3 waveform indication.

Proposal 3: Further clarify the previous agreements “Support the PRACH and PUSCH for msgA transmission in different slots” by adding “where the slot is defined by the min{preamble SCS, UL BWP SCS}.”

Proposal 4: PRACH and PUSCH for msgA transmission in the same slot are not supported, where the slot is defined by the min{preamble SCS, UL BWP SCS}.

Proposal 5: Value range of msgA PUSCH configuration period is also {10, 20, 40, 80, 160} ms.

Proposal 3: a PUSCH resource unit is defined as consisting of a PUSCH T/F resource unit and a DMRS port, which is used to associate with a PRACH preamble in Msg. A.

Proposal 4: a PUSCH resource unit could potentially include a GP and/or a GB.

Proposal 5: Association between SSB and PUSCH could use similar association rule between SSB and RACH.

Proposal 6: For the definition of a PUSCH resource unit, only DMRS port is supported.

Proposal 7: a PUSCH resource unit should support including a GP and/or a GB, which can be configured by gNB on a need basis.

Proposal 8: the association rule should be defined between RACH resource and PUSCH resource associated to the same SSB index.

Proposal 9: preamble grouping based solution similar to NR Rel-15, i.e., group A and group B, can be used to map to different TBS/MCS configuration.

Proposal 10: one-to-multiple mapping between preambles in each RO and associated PUSCH resource unit are not supported.

Proposal 11: the association rule should be defined between RACH resource and PUSCH resource associated to the same SSB index.

Proposal 12: the gap value is configured between the last RO of the RACH slot where the selected RO located and PUSCH.

Proposal 13: At least using a single semi-statically configured value for time/frequency relation between PRACH preambles in PRACH occasion(s) and PUSCH occasion(s) (alt.3 for option 2) should be supported if option 2 is adopted.

Proposal 14: the scrambling ID for PUSCH transmission in 2step RACH Msg A could be derived based on legacy RA-RNTI and preamble index.

# References

- RP-182894, New work item: 2-step RACH for NR, ZTE Corporation, Sanechips.

- RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #96bis, Xi’an, China, April, 2019.

- RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #97, Reno, USA, May 2019.

- R1-1908461, “Discussion on PUSCH resource unit”, Samsung, 3GPP RAN1 #98, Prague, CZ, Aug. 2019.