3GPP TSG-RAN WG1 Meeting #98	        		            R1-1908465

Prague, CZ, August 26th – August 30th, 2019

Agenda item:	7.2.2.2.1

Source: 	Samsung

Title: 	Channel access procedures for NR-U

Document for:	Discussion and Decision

# Introduction

In RAN1 #97 [1], the following agreements regarding channel access for NR-U were made:

Agreement:

For LBT by a UE prior to transmission of a UL burst within a gNB-initiated channel occupancy as an LBE device, for gap durations shorter than 25 microseconds, Cat 2 LBT can be indicated (FFS: explicit and/or implicit) to the UE if the gap is 16 microseconds (allowing for implementation tolerances)

Note: this is the Alt 1 identified in RAN1#96bis

Agreement:

Select one of the following alternatives for Cat2 LBT in a 16 us gap.

- Alt 1: The 16us measurement period is split into two slots with the first slot having a duration 7us and second slot having a duration of 9us.

  - Energy measurement is done in the 9us slot with the measurement including averaging for at least 4 us in any portion of the slot. LBT is said to be successful if the measured energy is lower than the ED threshold.

- Alt 2: The 16us measurement period is split into two slots with the first slot having a duration 7us and second slot having a duration of 9us.

  - Energy measurement is done in both the 7us and 9us slot with the measurement including averaging for at least 4 us in any portion of each slot. LBT is said to be successful if the measured energy is lower than the ED threshold in both slots.

- Alt 3: Energy measurement is done in any portion of the 16 us duration including averaging for at least 4 us. LBT is said to be successful if the measured energy is lower than the ED threshold.

Agreement:

Multi-carrier channel access schemes that are applicable to multiple carriers with a single LBT sub-band per carrier that is the same as the carrier bandwidth are also applicable to multiple LBT sub-bands within a carrier when such carriers are used either as a single wideband carrier or as part of carrier aggregation of a set of carriers.

This contribution discusses the design considerations for FR1 NR-U channel access, including the details for 16 µs CAT-2 LBT, LBT type/priority class indication method, contention window size adaptation rule for CAT-4 LBT of NR-U data transmissions and transmissions without HARQ/ACK feedbacks, LBT for wide-band operation design, support for directional LBT, LBT for FBE-based NR-U, LBT with handshake mechanism, and UE to gNB COT sharing.

# Detailed design for 16 µs CAT-2 LBT

In RAN1 97 [1], it has been agreed that to transmit an UL burst within a gNB-initiated channel occupancy time (COT), CAT-2 LBT can be indicated to the UE if the gap duration is 16 µs. In addition, there are three alternatives for the detailed design of the 16 µs CAT-2 LBT.

The specific definition of the 16µs CAT-2 LBT can be directly modified from that of the 25µs CAT-2 LBT for fair coexistence with other unlicensed systems. Specifically, the 16µs CAT-2 LBT can consist of a slot of 7µs and another slot of 9µs, and the CAT-2 LBT is successful if the NR-U initiating device observes the energy measured for at least 4µs during the first slot and the energy measured for at least 4µs during the second slot respectively are both below the energy detection threshold. The position of the 4µs duration within each slot can be up to implementation.

Proposal 1: 16µs CAT-2 LBT can consist of a slot of 7µs and another slot of 9µs, and energy measurement is performed for at least 4µs during both slots.

# Signaling support for LBT type/priority indication

Since the LBT type by a UE prior to transmission of the UL burst can include CAT-1 LBT, 16µs CAT-2 LBT, 25 µs CAT-2 LBT, and CAT-4 LBT, a corresponding signaling method of the LBT type needs to be indicated to the UE.

Either an explicit signaling method or an implicit signaling method for the LBT type indication can be considered. With an explicit signaling method, the LBT type for UL burst can be indicated dynamically through the UL grant scheduling the UL transmission. For example, an LBT type field of 2 bits can be included in the UL grant to indicate one of the LBT type of CAT-1 LBT, 16µs CAT-2 LBT, 25 µs CAT-2 LBT, and CAT-4 LBT.

With an implicit signaling method, the UE can determine its LBT type implicitly without an explicit LBT type indication through the UL grant. However, the implicit indication method could have ambiguity when the end of previous DL transmission and start of the scheduled UL burst has a gap duration of 16µs, in which case either CAT-1 LBT or 16µs CAT-2 LBT can be applied by the UE. Compared to the implicit signaling method, the explicit signaling method can provide better flexibility in configuring the UE LBT type as well as less ambiguity in indicating the LBT type for the UL burst. As a result, an explicit indication method of the LBT type for UL transmission can be utilized.

When CAT-4 LBT is configured for the transmission of UL transmission, the CAT-4 LBT priority class value corresponding to the scheduled UL burst can also be explicitly indicated similar to the LBT type indication. If the UE-initiated COT includes multiple UL signals/channels with different CAT-4 LBT priority classes, the highest channel access priority class value can be applied by the UE.

Proposal 2: NR-U can use an explicit indication method of the LBT type for UL transmission.

# Contention Window Size Adaptation for NR-U Data Transmissions

To guarantee fair coexistence with the incumbent Wi-Fi system in the 5 GHz unlicensed spectrum, LTE Rel-13 licensed-assisted access (LAA), Rel-14 enhanced LAA (eLAA), and Rel-15 further enhanced LAA (FeLAA) introduced energy detection (ED) based omni-directional LBT protocols (e.g., CAT-4 LBT and single-shot LBT), which are performed at the potential transmitter side for both downlink and uplink operations. Such LBT procedures are the baseline channel access framework for FR1 NR-U.

In addition to the basic channel access procedure, a contention window size (CWS) adaptation mechanism has also been defined in LTE-LAA for alleviating transmission collisions. Specifically, the CWS for CAT-4 LBT increases to next available value if at least 80% of HARQ-ACK values corresponding to PDSCH transmission(s) in a reference subframe are determined as NACK, wherein the reference subframe is the starting subframe of the most recent transmission of the eNB, for which at least some HARQ-ACK feedback is expected to be available.

NR-U DL CWS adaptation can also be based on HARQ-ACK feedbacks from the UE(s). However, different from LTE wherein the HARQ-ACK feedback timing is fixed by specification (e.g., 4 ms for LTE-LAA), NR supports a flexible timing relation from PDSCH transmission to its corresponding HARQ-ACK feedback, which ranges from 0 to 15 slots. NR also supports code-block group (CBG)-based HARQ-ACK feedback compared to only the transport block (TB)-based HARQ-ACK feedback for LTE-LAA. In addition, NR-U can also enhance HARQ-ACK feedbacks with cross-COT HARQ-ACK feedback and/or multiple opportunities for HARQ-ACK feedback. As a result, the CWS adaptation rule for NR-U can be more flexible compared to that of LTE-LAA.

By the timing that NR-U gNB starts to perform a CAT-4 LBT procedure, gNB needs to identify a set of HARQ-ACK values corresponding to a set of reference slot(s) from a reference DL transmission burst for the CWS adjust decision. Specifically, since first PDSCH transmission(s) within a transmission burst is more likely to be the victims of collisions, the reference slot(s) of a DL transmission burst can be chosen as the first slot of the DL transmission burst on the carrier of the gNB wherein PDSCH transmission(s) is scheduled. Furthermore, the time-domain duration of each NR-U PDSCH can be shorter than one slot (e.g., TDM’ed mini-slots for different UEs), in which case the CWS adaptation can be only based the HARQ-ACK values of the PDSCH(s) that starts from the beginning of the reference slot. The reference transmission burst can be defined for NR-U as the most recent transmission burst wherein at least some HARQ-ACK feedbacks for the reference slot (or the first PDSCH(s) of the reference slot) within this transmission burst are expected to be available by the time gNB performs CWS adaptation.

An illustration of the timing relation is provided in Figure 1, wherein the gNB attempts a new LBT process and updates the CWS on slot n-3. A corresponding reference downlink transmission burst consists of slot n-5 and n-4, wherein three UEs are scheduled with the PDSCH to HARQ delay of K1 = 1, 5, 4 slots respectively. The reference slot in this example is slot n-5, and the HARQ-ACK value corresponding to the reference slot from UE1 can be expected in a HARQ-ACK slot n-4, while the HARQ-ACK values corresponding to UE2 and UE3 can be expected in a HARQ-ACK slot n. Then, gNB updates CWS for a DL burst starting from slot n-2 based on HARQ-ACK of UE1.

Figure 1. An example of the reference slot.

Proposal 3: A reference DL transmission burst can be the most recent burst wherein at least some HARQ-ACK feedbacks corresponding to the reference slot of this burst is expected to be available by the time gNB performs CWS adaptation, wherein the reference slot of this burst is the first slot with PDSCH transmission(s).

In LTE-LAA, each eNB shall adjust the contention window based on a given reference subframe only once. For NR-U, due to its flexible and configurable timing from PDSCH transmission to corresponding HARQ-ACK feedback (i.e., K1), potential LBT failure for HARQ-ACK and potential HARQ-ACK enhancements, the limit on how gNB utilizes a given reference slot for CW adjustment can be relaxed from LTE-LAA. Specifically, it is possible for different transmission bursts to be corresponding to different PDSCHs in the same reference slot for CWS adaptation, and thus gNB may adjust the CWS based on a given reference slot more than once but based on a given PDSCH only once. An example is provided in Figure 2, wherein DL burst 0, 1, and 2 spans from slot n-5 to n-3, slot n to slot n+2, and slot n+4 to slot n+6 respectively, and K1 represents the PDSCH to HARQ delay in terms of number of NR-U slots. The CW adjust for DL burst 1 can be based on HARQ-ACK feedback of PDSCH transmission(s) to UE0 on DL burst 0; and the CW adjust for DL burst 2 can be based on HARQ-ACK feedback of PDSCH transmission(s) to UE1 on DL burst 0 (HARQ-ACK feedbacks for DL burst 1 is not expected to be received by slot n+3). As a result, gNB adjusts the contention window based on the reference slot of DL burst 0 twice.

Figure 2. An example of the reference slot and reference transmission burst.

Proposal 4: NR-U gNB can adjust CWS based on a reference slot more than once.

In the set of UL resources that are expected to contain the HARQ-ACK feedback(s) corresponding to the reference slot for CW adjustment, HARQ-ACK feedbacks in addition to those corresponding to the reference slot can be included due to flexible PDSCH to HARQ delay. For example, in Figure 2, UE0 is expected to report HARQ-ACK feedbacks at slot n-2, which corresponds to the PDSCH transmissions from slot n-5 to n-3. Under such a scenario, only the HARQ-ACK feedback(s) corresponding to the reference slot received by the timing that gNB performs CW adjustment can be utilized for CW adjustment. In the example of Figure 2, only the HARQ-ACK feedback(s) corresponding to PDSCH transmission to UE0 on slot n-5 will be utilized for CW adjustment decision on slot n-1.

Proposal 5: NR-U gNB can adjust CWS only based on the HARQ-ACK feedbacks corresponding to the reference slot.

In addition to defining the reference slot, the CWS adaptation rule for NR-U also needs to be enhanced from LTE-LAA, given that CBG-based HARQ-ACK feedback is supported in addition to TB-based feedback. Specifically, the gNB can increase CWS to next available value if at least 80% of HARQ-ACK values corresponding to the TBs transmitted in the reference slot are determined as NACK. In particular, when CBG-based HARQ-ACK feedbacks are used for a TB, this TB is determined as NACK if the HARQ-ACK value for the first transmitted (or re-transmitted) CBG within this TB is NACK, considering that the first transmitted CBG is more likely to be subject to collision than remaining CBGs of a TB. Alternatively, the TB is determined as NACK if the HARQ-ACK value for all transmitted (or re-transmitted) CBGs within this TB is NACK. In addition, by the timing that gNB performs the CWS adaptation, the ACK/NACK corresponding to the reference slot that is missing (e.g., due to failed LBT) or has not become available yet (e.g., due to large K1) can be treated as DTX and not counted towards the current CWS adaptation decision.

Proposal 6: CWS is increased to next available value if at least 80% of the TBs transmitted in the reference slot are determined as NACK; and a TB with CBG-based feedback is determined as NACK if the HARQ-ACK value for the first (re)transmitted CBG is NACK.

Proposal 7: By the timing that gNB performs the CWS adaptation, missed/unavailable HARQ-ACK of reference slot can be not counted towards the current CWS adaptation decision.

NR-U UL CWS adaptation can also be based on HARQ-ACK feedbacks from gNB, e.g., by DFI-like DCI, or by NDI in UL grant. In LTE-LAA, the minimum latency between the slot in which UE receives UL grant or DFI and the reference slot is defined as 4ms. Similarly, a minimum latency should be defined for NR-U. In NR phase-1, several minimum latencies are specified, such as the latency between PDSCH and its HARQ-ACK, or the latency between UL grant and PUSCH, but there is no latency defined between PUSCH and its HARQ-ACK. A proper value of such latency should be further studied in NR-U.

Proposal 8: The minimum latency between the slot in which UE receives UL grant or DFI and the reference slot should be defined.

# Contention Window Size for Wideband Operation of NR-U

Compared to LTE-LAA where the channel bandwidth per carrier is 20 MHz, FR1 NR-U supports wideband operation with up to 100 MHz channel bandwidth per carrier. In addition, in order to support UEs with different capabilities, NR has introduced the support for bandwidth part (BWP), such that the network can configure up to four DL/UL BWPs for a UE. LBT designs to support such wideband operations shall be considered for FR1 NR-U.

For NR-U wideband operation, operation on a single active BWP for an unlicensed carrier is supported for both DL and UL. Furthermore, for DL, gNB may transmit PDSCH on parts or whole of LBT bandwidths within the active BWP where CCA is successful at gNB. As discussed in our companion discussion [2], for UL, it is preferred that UE transmits on a single BWP if the LBT is successful on the whole BWP for the UL wideband operation.

It has been agreed in RAN1 #97 [1] that multi-carrier channel access schemes of LTE-LAA are applicable for the sub-band LBT of NR-U wideband operations. One remaining issue is how the contention window size (CWS) for each LBT bandwidth is maintained and updated, wherein either each LBT bandwidth can maintain a CWS independently, or a single CWS can be maintained across all the LBT bandwidths.

Since it was concluded in [1] that a UE can receive a PDSCH scheduled within an LBT bandwidth or over multiple LBT bandwidths, thus when UE receives a PDSCH scheduled within multiple LBT bandwidths, a mechanism to calculate HARQ-ACK for each LBT bandwidth that overlaps with the scheduled PDSCH needs to be designed if each LBT bandwidth maintains a CWS independently. For example, such mechanism could depend on the PDSCH mapping scheme over multiple LBT bandwidths.

As a result, a baseline method to maintain CWS for NR-U wideband operation is to maintain a single CWS across all LBT bandwidths similar to type B1 multi-carrier LBT scheme of LTE-LAA. Specifically, a single CWS can be maintained for all LBT bandwidths, and CWS is updated according to HARQ-ACK corresponding to the PDSCH transmissions on reference slots across all LBT bandwidths. Compared to maintaining a CWS for each LBT bandwidth independently, such baseline method works irrespective of the PDSCH mapping scheme, and does not require defining a mechanism to calculate HARQ-ACK for each LBT bandwidth that overlaps with the scheduled PDSCH.

Proposal 9: A single CWS can be maintained across LBT bandwidths for NR-U wideband operation similar to type B1 of LTE-LAA multi-carrier LBT.

Despite better spectrum utilization can be achieved with sub-band LBT, it can have several potential enhancements from existing Rel-15 NR. For example, to achieve efficient DCI transmission under the sub-band LBT, gNB can configure CORESET on a per LBT bandwidth basis, such that gNB can transmit DCI on the LBT bandwidth(s) that have passed the LBT [2].

Proposal 10: CORESET configuration can be enhanced to support sub-band LBT for NR-U.

# Contention Window Size for Transmissions without HARQ-ACK

In addition to the regular data transmission, NR-U can also use CAT-4 LBT to grant transmissions of non-unicast signals/channels without HARQ-ACK feedbacks, such as the DRS, OSI, paging, Msg2, PRACH, PUCCH, etc. The maintenance and adjustment of CAT-4 LBT CWS for transmissions without HARQ-ACK feedback needs to be enhanced from that of the regular NR-U data transmissions.

## A baseline CWS maintenance rule for signals/channels without HARQ-ACK

When CAT-4 LBT is used for signals/channels without HARQ-ACK feedback, the CWS corresponding to the CAT-4 LBT priority class of such signals/channels can be updated according to the procedure defined in Section 4 when they are multiplexed with other unicast data within a same transmission burst. Specifically, the CWS is increased to next available value or reset to the minimum according to the HARQ-ACK feedbacks of the unicast data on the reference slot. Otherwise, if the signals/channels without HARQ-ACK are transmitted alone or multiplexed only with other non-unicast data within a same transmission burst, the CWS corresponding to the CAT-4 LBT priority class of such signals/channels can remain unchanged.

Proposal 11: If the signals/channels without HARQ-ACK are transmitted alone or multiplexed only with other non-unicast data within a same transmission burst, the CWS corresponding to the CAT-4 LBT priority class of such signals/channels can remain unchanged.

## CWS Adjust for Random Access

The random access procedure is based on a 4-step procedure, wherein PRACH/Msg1, Msg2, Msg3 and Msg4 need to be exchanged between UE and gNB in a sequential order. Therefore, the CWS adjust for the random access procedure can be further enhanced from the baseline CWS maintenance rule for signals/channels without HARQ-ACK.

It has been agreed that CAT-4 is applied for UL transmissions that are part of a RACH procedure that initiate a channel occupancy. Therefore, CAT-4 LBT is used for standalone transmission of NR-U PRACH. A CWS adjust rule corresponding to the CAT-4 LBT of PRACH needs to be designed, which can be based on the reception status of RAR. Specifically, the CWS for PRACH LBT can be increased to the next available value if RAR corresponding to the transmitted PRACH has not been received within the RAR monitoring window; otherwise, the CWS for PRACH LBT can reset to the minimum value.

Proposal 12: CWS for PRACH LBT can increase to the next available value if RAR is not received within the RAR monitoring window; otherwise, the CWS for PRACH LBT can reset to the minimum value.

The LBT for Msg2 can be CAT-2 LBT if the Msg2 can be transmitted within the UE-initiated MCOT obtained by CAT-4 LBT of PRACH; otherwise, LBT for Msg2 can be CAT-4 based LBT potentially with higher LBT priority class for a faster Msg2 transmission. If Msg2 is transmitted alone and is subject to CAT-4 LBT, the CWS adaptation for Msg2 can be based on the reception of Msg3. For example, if Msg3 that corresponds to the Msg2 is received, the Msg2 transmission is considered as successful and CWS for Msg2 LBT can reset to minimum. By contrast, if Msg3 that corresponds to this Msg2 is not received (e.g., after certain duration following Msg2 transmission), this Msg2 transmission can be considered as subject to collision and CWS can be increased to the next available value.

Proposal 13: CWS for Msg2 with CAT-4 LBT can reset to the minimum value if correct Msg3 corresponding to the transmitted Msg2 is received; otherwise CWS for Msg2 increases to the next available value.

When RAR is subject to CAT-4 LBT, Msg3 can be scheduled to be within the MCOT of RAR through UL grant, such that the Msg3 can be transmitted subject to a CAT-2 LBT. When RAR is transmitted following a CAT-2 LBT, or if Msg3 is scheduled to be outside the MCOT of RAR, a CAT-4 LBT potentially with the lowest channel access priority class value can be used for Msg3 transmission.

If CAT-4 LBT is used for Msg3, the CWS adaptation for Msg3 can be based on the reception status of Msg4. For example, if correct Msg4 that corresponds to this Msg3 is received (e.g., containing the correct UE contention resolution identity), the Msg3 transmission is considered as successful and CWS for Msg3 LBT can reset to minimum. On the other hand, if the correct Msg4 that corresponds to this Msg3 is not received (e.g., after certain duration following Msg3 transmission, or a wrong UE contention resolution identity is detected), this Msg3 transmission can be considered as subject to collision and CWS can be increased to the next available value.

Proposal 14: CWS for Msg3 with CAT-4 LBT can reset to the minimum value if correct Msg4 corresponding to the transmitted Msg3 is received; otherwise CWS for Msg3 increases to the next available value.

# Channel Access for FBE-based NR-U

Besides the load-based equipment (LBE) operation mode, NR-U can also support the FBE operation mode as discussed in [3]. The FBE operations as in the ETSI BRAN specifications can be the baseline for channel access of FBE-based NR-U, wherein the gNB performs a single-shot LBT immediately before starting transmission on the operating channel at the start of a fixed frame period.

For the baseline FBE channel access scheme, when one or multiple synchronized FBE-based NR-U operators coexist in the operating channel, the channel access can always be successful since the gNBs perform LBT at the same time. As a result, despite the spec impacts for synchronized FBE NR-U are minimal, strong interference may exist between neighboring gNBs during the COT.

For asynchronous FBE NR-U, LBT performed by a gNB may always be blocked by the transmission from a nearby asynchronous gNB. The above scenarios can be alleviated by supporting directional LBT and directional transmissions by the gNB. For example, LBT by a gNB is less likely to be blocked by neighboring asynchronous gNB when their beam directions are not aligned. As a result, spatial reuse can be improved when directional LBT is supported for FBE-based NR-U.

Another enhancement for asynchronous FBE NR-U with potential spec impacts is to introduce a window of L (L>1) observation slots consecutively allocated in time-domain. As shown in Figure 3, gNB 1 and gNB 2 can randomly select one observation slot within the observation slot window, wherein the observation slot window is within the idle period and the device can reserve the channel if LBT passes on its observation slot. The enhancement as shown in Figure 5 is subject to regulation allowance that transmission can start within the idle period; otherwise, the observation slot window can be shifted to the start of the fixed frame period.

Figure 3.  Random CCA slots within idle period

Alternatively, each gNB chooses a random number M before accessing the unlicensed channel. Then, the maximum number of the fixed frame periods that gNB can grab before entering “muting” periods is set to M. If gNB grabs the channel for M fixed frame periods, it chooses a random number N and will not access the channel for N fixed frame periods. During such muting periods, neighbour gNBs that have failed to grab the channel due to the biased CCA timing can get channel access opportunities so that fairness among multiple gNBs can be improved as shown in Figure 4.

Figure 4. One example of modified FBE for resolving the biased channel access

Proposal 15: Support directional LBT and/or introduce random observation slot within idle period and/or random muting period for asynchronous FBE NR-U to improve spatial reuse.

# UE to gNB COT sharing

Another remaining issue for NR-U LBT design is on the applicability of COT sharing from UE to gNB. Specifically, a UE-initiated COT obtained through CAT-4 LBT can be shared with its serving gNB, at least when the downlink transmission that shares the UE-initiated COT includes PDCCH and/or PDSCH intended to the UE. Such UE to gNB COT sharing can be allowed for both scheduled PUSCH and configured grant (CG)-PUSCH, as long as the UE-initiated COT is obtained through CAT-4 LBT. In addition, a CAT-1 LBT can be used at the UL to DL switching point when the gap is smaller or equal to 16 µs; otherwise, a 25µs CAT-2 LBT can be used.

Proposal 16: UE-initiated COT through CAT-4 LBT can be shared with gNB when the downlink transmission includes PDCCH and/or PDSCH intended to the UE.

# Directional LBT for NR-U

To support the multi-beam operation for NR-U FR1, the directionality is also an important design consideration for the LBT operation. The baseline option to perform LBT omni-directionally by the potential transmitter can provide good coexistence with the incumbent systems, such as when NR-U operates in 5 GHz unlicensed bands or when NR-U coexists with another Wi-Fi network.

Another option to cope with directional transmissions of NR-U and improve spatial reuse is for the potential transmitter to perform a directional LBT over its intended transmit beam direction. This option can be at least used for NR-U and NR-U coexistence, and for the 6 GHz unlicensed bands as well as unlicensed bands with carrier frequency higher than 52.6 GHz.

As shown in our companion contribution [4], compared to the baseline omni-directional LBT, directional LBT can lead to better channel access probability and correspondingly better spatial reuse for NR-U/NR-U coexistence under the same ED threshold. The channel access gain for directional LBT will further outweigh its potential SINR performance loss compared to omni-directional LBT, which correspondingly will lead to a better throughput performance than the omni-directional LBT. In particular, the throughput gain under directional LBT compared to omni-directional LBT is observed to increase as traffic arrival rate increases for NR-U/NR-U coexistence. More details for the performance evaluation of directional LBT are provided in [4].

Proposal 17: Directional LBT can be supported at least for NR-U/NR-U coexistence.

# LBT with Handshake Mechanism for NR-U

In general, a potential transmission in the unlicensed band can be granted channel access if the following two conditions are met: (1) the receiver(s) of the potential transmission is ready to receive; and (2) the potential transmission will not cause strong interference to the receivers of ongoing transmissions. Condition (1) ensures that the potential transmission can have enough SINR and can avoid the hidden node issue, while condition (2) ensures that the potential transmission can provide fair coexistence with ongoing unlicensed transmissions. LBT schemes that are performed at the potential transmitter side can only approximately satisfy both conditions.

A general approach for NR-U transmissions to satisfy both condition (1) and condition (2) is through supporting a configurable handshake mechanism between gNB and UE, on top of the LBT schemes discussed in Section 2 for data transmissions. It has been discussed in [3] that handshaking between transmitter and receiver is a possible means to reduce the impact of hidden nodes for NR-U, and that detailed solutions for the handshaking can be considered.

The handshake between NR-U gNB and UE can be including an exchange of a channel access request (CARQ) and a channel access response (CARP). Specifically, CARQ and CARP can be message-based, wherein the CARQ can be transmitted by the potential transmitter upon successful LBT for data transmission; and the CARP can be transmitted by the receiver upon receiving CARQ and after successful LBT (e.g., single-shot LBT). The CARQ (CARP) message can include the following information: the indicator for CARQ (CARP), the transmitter (receiver) identity, transmitter (receiver) beam direction information, the maximum channel occupancy time (MCOT), etc. Figure 5 illustrates an example of the LBT with handshake mechanism for NR-U downlink transmission.

Figure 5. Example of LBT with CARQ/CARP exchange for NR-U downlink transmission.

For a potential NR-U transmitter and receiver pair with the handshake mechanism, the transmission can be granted if (1) the LBT for data transmission at the transmitter side is successful; (2) the CARQ/CARP message exchange between the transmitter and receiver is successful; and optionally that (3) through monitoring the CARQ/CARP messages from other NR-U transmitter/receiver pairs, the potential NR-U transmitter can determine that its transmission will not cause strong interference to existing transmissions. Figure 6 illustrates an example where gNB1 is transmitting to UE1; and gNB2 intends to transmit to UE2. Despite gNB2 can succeed in the directional LBT scheme, gNB2 can decide to not transmit to UE2, since UE2 will fail to respond the CARP addressed to gNB2 and/or gNB2 can receive the CARP message from UE1 that is addressed to gNB1. Therefore, the handshake mechanism can be combined with the LBT scheme for NR-U to achieve fair and efficient channel access with the coexisting radio access technologies (RATs), which can be beneficial for both omni-directional and directional LBT schemes.

Figure 6. Example of addressing hidden node issue with handshake mechanism.

Proposal 18: NR-U can support LBT with handshake mechanism through the exchange of a CARQ message transmitted by the potential transmitter upon successful LBT for data transmission; and a CARP message transmitted by the receiver upon receiving CARQ and after a successful LBT.

# Remaining Issues on NR-U channel access

## Max number of CAT-2 LBT attempts in a 5ms DRS transmission window

In LTE-LAA, Cat2 LBT can be applied to the transmission of DRS, with a granularity of starting location as 1 ms. For example, every subframe in the configured DRS transmission window can be utilized for DRS transmission, and Cat2 LBT is performed right before the subframe boundary for DRS.

In NR-U, the granularity of starting position for DRS transmission should be at least the same as the one in LTE-LAA for fair coexistence. If NR-U uses a smaller granularity than LTE-LAA, there are scenarios that DRS in LTE-LAA cannot be transmitted at any time, and the more aggressive channel access mechanism in NR-U does not fair coexist with LTE-LAA on the 5 GHz unlicensed spectrum.

One example for this issue is shown in Figure 7. If both NR-U and LTE-LAA implemented on the same band, NR-U has much higher channel access opportunity for DRS transmission comparing to LTE-LAA, if using a granularity of starting location smaller 1 ms.

Figure 7. Example of unfair coexistence with LTE-LAA when using small granularity for starting location of DRS transmission.

Proposal 19: The granularity of the starting locations for DRS transmission in a DRS transmission window should be 1 ms.

## Max number of CAT-2 LBT attempts for UL burst

One remaining NR-U LBT design consideration is the number of LBT attempts allowed to grant the transmission of an UL burst within a gNB-initiated COT [3]. Specifically, the UL burst transmission can start after a CAT-2 LBT is successful at the UE, otherwise the UE can perform a new CAT-2 LBT attempt. In order to increase the opportunity to transmit the UL burst, such CAT-2 LBT attempt can always be allowed, as long as the UL transmission can start within the COT and the total number of CAT-2 LBT attempts does not exceed a pre-configured number M >= 1. In this case, the maximum number of such CAT-2 LBT attempts can be determined as min(ceil(T/τ), M), wherein T can be the duration of the UL burst, and τ refers to the NR-U OFDM symbol duration.

Proposal 20: CAT-2 LBT can be used for an UL burst within gNB-initiated COT as long as UL burst can start within the COT and the total number of LBT attempts does not exceed a pre-configured number.

## NR-U LBT and transmission rule for Japan

Another remaining NR-U LBT design consideration is LBT and transmission rule for Japan. For LAA operation in Japan, it was specified that if the eNB has transmitted a transmission after Cat 4 LBT, the eNB may transmit the next continuous transmission, for duration of maximum 4ms, immediately after sensing the channel to be idle for at least a 34us and if the total sensing and transmission time is not more than 8ms + 34us, which is specified in TS37.213. It was introduced because that the maximum channel occupancy of Japan is limited to 4ms. Therefore, it is desirable for NR-U to support such LBT and transmission rule for Japan at least for 5GHz unlicensed spectrum.

Proposal 21: NR-U should support the same LBT and transmission rule defined for LTE-LAA operation in Japan, at least for 5GHz unlicensed spectrum.

# Conclusions

This contribution considered the overall design considerations for channel access mechanisms in NR unlicensed spectrum and proposes the following.

Proposal 1: 16µs CAT-2 LBT can consist of a slot of 7µs and another slot of 9µs, and energy measurement is performed for at least 4µs during both slots.

Proposal 2: NR-U can use an explicit indication method of the LBT type for UL transmission.

Proposal 3: A reference DL transmission burst can be the most recent burst wherein at least some HARQ-ACK feedbacks corresponding to the reference slot of this burst is expected to be available by the time gNB performs CWS adaptation, wherein the reference slot of this burst is the first slot with PDSCH transmission(s).

Proposal 4: NR-U gNB can adjust CWS based on a reference slot more than once.

Proposal 5: NR-U gNB can adjust CWS only based on the HARQ-ACK feedbacks corresponding to the reference slot.

Proposal 6: CWS is increased to next available value if at least 80% of the TBs transmitted in the reference slot are determined as NACK; and a TB with CBG-based feedback is determined as NACK if the HARQ-ACK value for the first (re)transmitted CBG is NACK.

Proposal 7: By the timing that gNB performs the CWS adaptation, missed/unavailable HARQ-ACK of reference slot can be not counted towards the current CWS adaptation decision.

Proposal 8: The minimum latency between the slot in which UE receives UL grant or DFI and the reference slot should be defined.

Proposal 9: A single CWS can be maintained across LBT bandwidths for NR-U wideband operation similar to type B1 of LTE-LAA multi-carrier LBT.

Proposal 10: CORESET configuration can be enhanced to support sub-band LBT for NR-U.

Proposal 11: If the signals/channels without HARQ-ACK are transmitted alone or multiplexed only with other non-unicast data within a same transmission burst, the CWS corresponding to the CAT-4 LBT priority class of such signals/channels can remain unchanged.

Proposal 12: CWS for PRACH LBT can increase to the next available value if RAR is not received within the RAR monitoring window; otherwise, the CWS for PRACH LBT can reset to the minimum value.

Proposal 13: CWS for Msg2 with CAT-4 LBT can reset to the minimum value if correct Msg3 corresponding to the transmitted Msg2 is received; otherwise CWS for Msg2 increases to the next available value.

Proposal 14: CWS for Msg3 with CAT-4 LBT can reset to the minimum value if correct Msg4 corresponding to the transmitted Msg3 is received; otherwise CWS for Msg3 increases to the next available value.

Proposal 15: Support directional LBT and/or introduce random observation slot within idle period and/or random muting period for asynchronous FBE NR-U to improve spatial reuse.

Proposal 16: UE-initiated COT through CAT-4 LBT can be shared with gNB when the downlink transmission includes PDCCH and/or PDSCH intended to the UE.

Proposal 17: Directional LBT can be supported at least for NR-U/NR-U coexistence.

Proposal 18: NR-U can support LBT with handshake mechanism through the exchange of a CARQ message transmitted by the potential transmitter upon successful LBT for data transmission; and a CARP message transmitted by the receiver upon receiving CARQ and after a successful LBT.

Proposal 19: The granularity of the starting locations for DRS transmission in a DRS transmission window should be 1 ms.

Proposal 20: CAT-2 LBT can be used for an UL burst within gNB-initiated COT as long as UL burst can start within the COT and the total number of LBT attempts does not exceed a pre-configured number.

Proposal 21: NR-U should support the same LBT and transmission rule defined for LTE-LAA operation in Japan, at least for 5GHz unlicensed spectrum.

# References

[1] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #97, Reno, Nevada, May 13th – May 17th, 2019.

[2] R1-1908469, “Wideband operation for NR-U”, Samsung

[3] TR 38.889, Study on NR-based Access to Unlicensed Spectrum (Release 16), Dec. 2018.

[4] R1-1908470, “Remaining issues for NR-U”, Samsung