3GPP TSG RAN WG1 meeting #98	R1-1908497

Prague, CZ, August 26th – 30th, 2019

Agenda item:	7.2.8.1

Source: 	Samsung

Title: 	CSI enhancement for MU-MIMO

Document for:	Discussion and Decision

# Introduction

Based on the agreements made in RAN1#97 [1] and the offline email discussions after the meeting, the following remaining issues about the DFT-compression based Type II overhead reduction are discussed in this contribution.

- UCI parameters

  - FD basis subset selection indicator: α and Minit

  - FFS on bitmap for rank 3-4

  - FFS on joint encoding of M’ and KNZ

  - FFS on polarization with all zero coefficients

- Supported parameter combinations

- UCI omission

- Codebook subset restriction

- N3 for NSB×R>13

- Extension to Type II port selection codebook

The relevant simulation results are provided in the appendix.

# UCI parameters

The following agreements were made in RAN1#97 [1] and via offline email discussion regarding the remaining details about the UCI parameters.

RAN1#97 Agreement In RAN1#98, finalize the values of based on the following aspects - Candidate values for to be down selected/evaluated: at least {1.5, 2, 2.5} - The set of values is to be finalized via offline email discussion prior to RAN1#98 - Configuration of : - Whether it is independent of other FD compression parameters, or dependent on at least one of the other FD compression parameters, i.e. p (=y0, and/or v0 for RI=3-4), L, β, and/or R - Whether is rank-specific or rank-common - Note: This is to be discussed along with the supported parameter combinations for (L, p, β, ) Agreement In RAN1#98, decide if the specification will restrict the UE from reporting all “zero” in the bitmap for a polarization for each layer Agreement For further details on the agreed UCI parameters in Table 1 of R1-1905629: - RI ({1,…, RIMAX}∈1,…,RIMAX) and KNZ,TOTKNZ,TOT (the total number of non-zero coefficients summed across all the layers, where KNZ,TOTKNZ,TOT {1,2,…, 2K0}KNZ,TOT∈1,2,…,2K0 are reported in UCI part 1 - FFS: If the total number of non-zero coefficients are jointly encoded with M’ (if supported) or independently encoded - For RI=3-4, bitmaps, each with size-2LMi (i=0,1,…, RI-1, where i denotes the i-th layer) are reported in UCI part 2 - FFS: If alt 3-4 is supported, size-2LMi2LMi-1 (i=0,1,…, RI-1i=0,1,…,RI-1, where i i denotes the ii-th layer) are reported in UCI part 2 Agreement On SCI (RI>1) and FD basis subset selection indicator, support Alt B described in the following table. - FFS: details on bitwidth and possible values for Minitial Minitial reporting in UCI part 2 - FFS: whether the possible value(s) for Minitial Minitial can depend on configured FD compression parameters Conclusion On the candidate UCI parameters listed in Table 2 of R1-1905629: - The following parameters are not supported as a consequence of the previous agreements in RAN1#96bis: - Indication of zero polarization reference amplitude values - FD oversampling (rotation) Q3, - (N1’ N2’) - The following parameters are not supported due to lack of consensus: - M’ as an independent parameter - Does not preclude the support of M’ for the purpose of CSI omission (if supported) - Basis sufficiency indicator (BSI)L,p,β,α

Offline agreement: In RAN1#98, finalize the values of  via down selection from {1.5, 2, 2.5} - FFS: =3 as an additional candidate

In this section, we discuss the highlighted remaining issues about of the agreed UCI parameters.

The first issue is the values of α for the two-step FD basis subset selection indicator. The simulation results comparing the three candidate values {1.5, 2, 2.5} is provided in Evaluation 1 in appendix A. Based on these results, we can observe the following.

Observation 1: There is no clear benefit of supporting multiple values of α, and making α configurable

- the overhead difference for different alpha values is small

- α=1.5 has slightly wosre performance than α=2 and 2.5 in high overhead regime

The value of α can therefore be fixed to a suitable value e.g. 2 or 2.5.

Proposal 1: α is fixed, e.g. to 2 or 2.5.

The second issue is the bit-width and candidate values of Minit for the two-step FD basis subset selection indicator. Let B be the bit-width for Minit reporting, where Minit is parameterized as Minit= N3-N3'2+Δ. The values of Δ for B=1,2,3 are as follows.

- B=1: Δ∈{-1,0}

- B=2: Δ∈{-1,0,1,2}

- B=3: Δ∈{-3,-2,-1,0,1,2,3,4}

The simulation results comparing B=1,2,3 is provided in Evaluation 2 in appendix A. Based on these results, we can observe the following.

Observation 2: Reducing the bit-width of Minit incurs some loss (~0.2% per 1 bit reduction)

When compared with the unrestricted Minit reporting, the max overhead saving is small (5 bits), and the performance loss is about 1%. The Minit can therefore be unrestricted, i.e. the bit-width of Minit is log2N3.

Proposal 2: Support an unrestricted value set for Minit reporting, i.e., the bit-width of Minit is log2N3.

The third issue is the FFS on the size-2LMi-1 bitmap for rank 3-4. Since size-2LM bitmap is already agreed for rank 1-2, and 1-bit overhead saving is too small, for simpler specification, we prefer the same solution for rank 3-4, i.e., size-2LMi bitmap for rank 3-4.

Proposal 3: Support size-2LMi bitmap for rank 3-4.

The fourth issue is the FFS on joint encoding of M’ and KNZ. The arguments in support of M’ reporting are (a) dynamic FD basis selection at the UE, and (b) potential overhead saving due to reduction in the bitmap size if M’ < M is reported by the UE. But, there are at least the following issues with reporting M’ jointly with KNZ.

- The gNB cannot be sure about the quality of CSI. It may receive CSI than is lower resolution (due to poor UE implementation) than the actual CSI that it configured the UE for.

- Assuming M’ reporting is layer-common, the benefits of M’ reporting is unclear for high rank (i.e., rank 3-4) since the UE needs to be determine M’ considering all layers.

- There is additional UE complexity due to dynamic FD basis subset selection and joint processing across layers.

- If M’ is reported jointly with KNZ without any additional overhead, the remaining X=log2KNZ-KNZ states are used for M’ reporting. However, X can be small (even zero), and hence the gain of M’ reporting is not always guaranteed. Perhaps, a better solution is when log2KNZ states are divided equally into two for the two values of M’ (assuming 1-bit M’). This, however, reduces the range of values for KNZ.

Observation 3: Reporting M’ might have some benefits, but there are several issues that need to be considered, e.g. CSI resolution, solution for high rank, additional UE complexity/processing, and chance of such occurrences.

Proposal 4: Joint encoding of M’ and KNZ is not supported.

The fifth issue is the FFS on polarization with all zero coefficients. The chance of UE reporting all zero coefficients for a polarization is in all likelihood too small in general. For some parameter combinations, however, the UE may indeed report all zero coefficients quite often. This could have some adverse effect on gNB implementations. To avoid this, a specification supported can be provided. But, the solution should be simple, e.g. for each layer, UE is expected to report at least 1 NZ coefficient cl,m with index l∈{0,1,..,L-1} and at least 1 NZ coefficient cl,m with index l∈{L,..,2L-1}.

Proposal 5: Support a simple solution to avoid a polarization with all zero coefficients, i.e., for each layer, UE is expected to report at least 1 NZ coefficient cl,m with index l∈{0,1,..,L-1} and at least 1 NZ coefficient cl,m with index l∈{L,..,2L-1}.

# Supported parameter combinations

The following agreements were made in RAN1#97 [1] and via offline email discussion regarding the supported parameter combinations.

RAN1#97 Agreement - …. - FFS: Possible down-selection on the FD combination parameters in RAN1#98 In RAN1#98, finalize the values of based on the following aspects - Candidate values for to be down selected/evaluated: at least {1.5, 2, 2.5} - The set of values is to be finalized via offline email discussion prior to RAN1#98 - Configuration of : - Whether it is independent of other FD compression parameters, or dependent on at least one of the other FD compression parameters, i.e. p (=y0, and/or v0 for RI=3-4), L, β, and/or R - Whether is rank-specific or rank-common - Note: This is to be discussed along with the supported parameter combinations for (L, p, β, ) L,p,β,α

Offline agreement: - Use the following criteria for down-selecting the supported parameter combinations: - Avoid overly complex down selection by reducing the number of combinations for (L,p,beta) only - Alpha has not been decided and making it N3 dependent is unnecessarily convoluted - For #1 - Remove combinations with: - the total overhead exceeding the max of Rel.15 Type II - the UPT lower than Rel.15 Type II for the same overhead - When several combinations appear redundant in terms of overhead (sharing similar overhead – or vice versa), choose the one with the best UPT, unless the best combo is shown to be scenario-dependent. Then it is justified to support several combos with more or less the same overhead - Use the following format for a proposed supported parameter combination: L; p=y0 (rank 1-2); p=v0 (rank 3-4); β; Restriction (if any).

The simulation results for the following parameter combinations (Table 1 and Table 2) is provided in Evaluation 3 in appendix A.

Table 1: parameter combinations for rank 1 and dynamic rank 1-2 evaluations

| Evaluation | Rank | L | p | β | Phase | R | #ports |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.1 | 1 | 2,4 | 14,12 | 14,12,34 | 8PSK,16PSK | 1 | 16 |
| 3.2 | Dynamic 1-2 | 2,4 |  |  |  |  | 16 |
| 3.3 | 1 | 6 |  |  |  |  | 32 |
| 3.4 | Dynamic 1-2 | 6 |  |  |  |  | 32 |

Table 2: parameter combinations for dynamic rank 1-4 evaluations

| Evaluation | Rank | L | (y0,v0) | β | Phase | R | α | #ports |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.5 | Dynamic 1-4, N3≤19 | 2,4 | 14,18,14,14,12,14 | 14,12,34 | 8PSK,16PSK | 1 | - | 16 |
| 3.6 | Dynamic 1-4, N3>19 |  |  |  | 16PSK | 2 | 2 |  |

Based on the simulations results, we observe the following.

Observation 4: According to the agreed criteria

- For rank 1-2, the following parameter combinations achieve the best performance-overhead trade-offs

| L | p | β |
| --- | --- | --- |
| 2 | 0.25 | 0.75 |
| 4 | 0.25 | 0.50 |
| 4 | 0.25 | 0.75 |
| 4 | 0.50 | 0.50 |
| 6 | 0.25 | 0.25 |
| 6 | 0.25 | 0.50 |
| 6 | 0.50 | 0.50 |

- For rank 3-4, the following parameter combinations achieve the best performance-overhead trade-offs

| L | (y0,v0) | β |
| --- | --- | --- |
| 2 | (1/4,1/8) | 0.25 |
| 4 | (1/4,1/8) | 0.25 |
| 4 | (1/4,1/8) | 0.50 |
| 4 | (1/4,1/4) | 0.75 |

The supported set of parameter combinations can be a union of the two parameter combinations tables.

Proposal 6: Support the following parameter combinations

| L | p=y0 (rank 1-2) | p=v0 (rank 3-4) | β | Restriction (if any) |
| --- | --- | --- | --- | --- |
| 2 | 1/4 | 1/8 | 1/4 |  |
| 2 | 1/4 | 1/8 | 3/4 |  |
| 4 | 1/4 | 1/8 | 1/4 |  |
| 4 | 1/4 | 1/8 | 1/2 |  |
| 4 | 1/4 | 1/4 | 3/4 |  |
| 4 | 1/2 | 1/4 | 1/2 |  |
| 6 | 1/4 | - | 1/4 | 32 ports, rank 1-2, R=1 |
| 6 | 1/4 | - | 1/2 |  |
| 6 | 1/4 | - | 3/4 |  |

# UCI omission

Figure 1: UCI part 2

In this section, UCI omission mechanism for the DFT-compression based Type II CSI is discussed. Similar to Rel. 15 UCI omission, UCI part 2 can be segmented into the following segments (Figure 1).

- UCI part 2 wideband: including the first PMI i1; and

- UCI part 2 subband: including the second PMI i2, where the components of the second PMI are grouped into two groups

  - G1 (retained and reported): first group of second PMI components (for example, amplitude and phase of the first group of NZ coefficients)

  - G2 (omitted): second group of second PMI components (for example, amplitude and phase of the second group of NZ coefficients).

The two groups, G1 and G2, can be determined based on the following alternatives.

- Alt0: The SD beam indices i∈{0,1,..,2L-1} is used for grouping. For example, two polarizations can form two groups.

- Alt1: The FD beam indices m∈{0,1,..,M-1} is used for grouping.

- Alt2: The layer indices l∈{0,1,..,ν-1} is used for grouping.

- Alt3: The NZ coefficients KNZ reported in UCI part 1 is used for grouping. For example, the KNZ NZ coefficients can be divided into two group.

Out of these alternatives, Alt0 is not preferred since the gNB can’t reconstruct half of the precoding matrix if a group is omitted. Likewise, Alt2 is not preferred since the gNB will not receive some of the layers if a group is omitted. The issue with Alt1 is the ambiguity in the payload of the two groups since the gNB can’t determine the payload of two groups after decoding UCI part 1. We therefore prefer Alt3 since it has no payload ambiguity.

Also, the strongest coefficient (indicated via SCI) is included in the first group G1 since the gNB needs to know the strongest coefficient even if G2 is omitted.

Proposal 7: For UCI omission,

- Reuse Rel.15 UCI omission mechanism comprising the following segments

  - CSI part 2 wideband including the first PMI i1

  - CSI part 2 subband including the second PMI i2 segmented into two groups

    - G1 (retained and reported): first group of second PMI components

    - G2 (omitted): second group of second PMI components

- The NZ coefficients KNZ reported in UCI part 1 is used for grouping.

- The strongest coefficient (indicated via SCI) is included in G1.

Regarding PMI components, similar to Rel. 15, the PMI comprises a first PMI (i1) and a second PMI (i2). The first PMI (i1) comprises the following components.

- Oversampling (rotation) factor (q1,q2): 2 bits for each qi

  - (q1,q2) to indicate SD orthogonal basis comprising N1N2 DFT beams

- SD basis indicator (layer-common): indicated using log2N1N2L bits

- FD basis indicator (layer-specific): indicated using X bits, e.g., X=i=0RI-1log2N3Mi if N3≤19 and X=log2Z+i=0RI-1log2N3'Mi if N3>19, where Z= number of supported values for Minit

- Strongest coefficient indicator (layer-specific): Y bits indicating the strongest coefficient, Y=log2KNZ for rank 1 and log22L per layer for rank > 1.

  - The strongest coefficient = 1, hence its amplitude and phase are not reported.

- Size-KNZ subset selection (layer-specific): 2LMi bits per layer indicating reported NZ coefficients

The second PMI (i2) comprises indication about the SB components, i.e., amplitude and phase of the remaining KNZ-1 NZ coefficients. The SB components are layer-specific. For each layer, the payload of these components are as follows.

- Amplitude: A1+A2(KNZ-1) bits, where

  - A1=4 is number of bits for the reference amplitude

  - A2=3 is number of bits for the differential amplitude

- Phase: P(KNZ-1) bits, where P∈{3,4} is number of bits configured for the phase.

Proposal 8: The PMI comprises a first PMI (i1) and a second PMI (i2).

- The PMI (i1) comprises the following components.

  - Oversampling (rotation) factor (q1,q2) for SD basis

  - SD basis indicator (layer-common)

  - FD basis indicator (layer-specific)

  - Strongest coefficient indicator (layer-specific)

  - Size-KNZ subset selection bitmap (layer-specific)

- The second PMI (i2) comprises the following layer-specific components.

  - Amplitude:

    - reference amplitude for the weaker polarization

    - differential amplitude for each of (KNZ-1) NZ coefficients

  - Phase for each of (KNZ-1) NZ coefficients

# Codebook subset restriction

In RAN1#95 [2], the following agreement was made about codebook subset restriction (CBSR).

Agreement For Rel-16 NR, agree on Alt1 (DFT-based compression) in Table 1 of R1-1813002 as the adopted Type II rank 1-2 overhead reduction (compression) scheme as formulated in Alt1.1 of R1-1813002 - Note: The same DFT-based compression scheme is extended for Type II port selection codebook - Codebook subset restriction (CBSR) is supported when DFT-based compression is utilized for Type II codebooks with overhead reduction (compression) scheme - FFS: detailed signaling mechanism - Note: Additional compression scheme(s) are not precluded

In Rel. 15, CBSR is supported to restrict SD beams. The same mechanism can be used to restrict SD beams in Type II CSI overhead reduction. The need for CBSR to restrict FD beams (components) is unclear,. While the technical motivation to restrict SD beams (which is to minimize interference in certain spatial directions) is well-understood, there is no clear motivation to restrict FD beams (components). Also, restricting FD beams may result in large performance loss if a few strong FD beams/components are prohibited/restricted (hence can’t be used) for FD compression. Unless some use cases or/and technical motivation is justified, CBSR to restrict FD beams needs more study/discussion in RAN1.

Now, according to agreed quantization scheme, the coefficient amplitude pl,m is a product pref,l,m×pdiff,l.m where pref,l,m is a reference amplitude and pdiff,l,m is a differential amplitude. In Rel. 15, a 2-bit amplitude restriction is used to restrict the maximum allowed value of the WB amplitude (which upper bounds the overall amplitude). The same 2-bit amplitude restriction mechanism can be used in Type II CSI overhead reduction. In particular, the 2-bit amplitude restriction can be according to one of the following alternatives.

- Alt0: restrict only reference amplitude

- Alt1: restrict only differential amplitude

- Alt2: restrict both differential and reference amplitudes

- Alt3: restrict overall amplitude pl,m=pref,l,m×pdiff,l.m

Comparing these alternatives, Alt0 is not valid since it can only restrict amplitudes corresponding to one (“weaker polarization”) of the two antenna polarizations (since reference amplitude = 1 for the polarization of the strongest coefficient). Restricting both reference and differential amplitudes (Alt2) is unnecessary and over-design since the purpose of amplitude restriction is to restrict the maximum amplitude values, which can be done by restricting only differential amplitude (Alt1) or the overall amplitude (Alt3). Between Alt1 and Alt3, Alt3 is preferable in our view since it restricts the overall amplitude, not only one of its components (e.g. only differential in Alt1). Also, there is another issue with Alt1 which is that the differential amplitude restriction has to consider the worst case, which is when the reference amplitude equals 1. When the reference amplitude < 1, it is obvious that the overall amplitude will always be less than the maximum allowed value due to the multiplication with the reference amplitude. This will result in some performance loss.

Proposal 9: Support the following for CBSR for Type II CSI overhead reduction in Rel. 16.

- For SD beams, reuse Rel. 15 CBSR mechanism for Type II CSI reporting.

- For FD beams/components, study whether CBSR is necessary.

- For amplitude restriction, reuse Rel. 15 2-bit amplitude restriction to restrict the overall amplitude pl,m=pref,l,m×pdiff,l.m

# N3 for NSB×R>13

The following agreements were made about N3 value for NSB×R>13 in RAN1#97 [1].

Agreement On the value of N3 N3for (N3=NSB×R) > 13: - For Alt1 (padding), consider only extrapolation-based scheme and decide on the final specific design alternatives in RAN1#97 (Reno) for down selection in RAN1#98 (Prague) - For Alt2 (two segments), the following alternatives will be considered for down selection in RAN1#98 (Prague): - Alt2.1: S1: 1, …, Y; S2: NSB×R -Y+1, …, NSB×R - Alt2.2: S1: 1, …, N3N3; S2: NS - N3+1, …, NS Agreement On Alt1 (padding, as described in R1-1907783) for N3, for evaluation purposes, select one of Alt1.1, 1.2, 1.3, and 1.4 in RAN1#98 (Prague) as described in the table of R1-1907783. - Alt1.1, 1.2, 1.3, 1.4 are described in R1-1907783

There are at least the following issues with these two alternatives (Alt1 and Alt2 highlighted above).

- Two different solutions for NSB×R≤13 and NSB×R>13 adds to UE implementation complexity. A single solution for both cases should be preferred.

- The threshold value (13) is arbitrary. The rationale behind this value is unclear. Due to this threshold value, a UE needs two different implementations (one for <= 13 case and another for >13 case) even for R=1.

Also, based on simulation results in appendix A, we observe the following.

Observation: When NSB×R>13,

- Alt0 (N3=NSB×R) achieves the best performance-overhead trade-off

- Alt1 (N3 is a multiple of 2, 3, or 5) incurs large performance loss due to possible misalignment/mismatch between an assumed precoder by the gNB and an actual precoder used by the UE while calculating CQI

- Alt2 (two segments) results in high overhead, and performs worse than Alt0 in terms of performance-overhead trade-off

Based on these results and the abovementioned issues, we prefer Alt0 for both NSB×R>13 and NSB×R≤13.

Proposal 10: Support N3=NSB×R for NSB×R>13 (i.e. Alt0).

# Extension to Type II port selection codebook

The following agreement was made in RAN1#95 [2].

Agreement For Rel-16 NR, agree on Alt1 (DFT-based compression) in Table 1 of R1-1813002 as the adopted Type II rank 1-2 overhead reduction (compression) scheme as formulated in Alt1.1 of R1-1813002 - Note: The same DFT-based compression scheme is extended for Type II port selection codebook - Codebook subset restriction (CBSR) is supported when DFT-based compression is utilized for Type II codebooks with overhead reduction (compression) scheme - FFS: detailed signaling mechanism - Note: Additional compression scheme(s) are not precluded

Another open issue (highlighted above) is the extension of DFT-based compression to Type II port selection codebook. A simple extension in our view is to reuse W1 in Rel.15 Type II port selection codebook, i.e., W1 performs L port selection (common for two polarizations).

Proposal 11: Reuse Rel. 15 W1 for extension of DFT-based compression to Type II port selection codebook, where W1 performs L port selection (common for two polarizations); and details follows Rel. 15.

# Conclusions

In this contribution, remaining issues about DFT-compression based Type II CSI overhead reduction. The proposals and observations made are summarized as follows.

Observation 1: There is no clear benefit of supporting multiple values of α, and making α configurable

- the overhead difference for different alpha values is small

- α=1.5 has slightly wosre performance than α=2 and 2.5 in high overhead regime

Observation 2: Reducing the bit-width of Minit incurs some loss (~0.2% per 1 bit reduction)

Observation 3: Reporting M’ might have some benefits, but there are several issues that need to be considered, e.g. CSI resolution, solution for high rank, additional UE complexity/processing, and chance of such occurrences.

Observation 4: According to the agreed criteria

- For rank 1-2, the following parameter combinations achieve the best performance-overhead trade-offs

| L | p | β |
| --- | --- | --- |
| 2 | 0.25 | 0.75 |
| 4 | 0.25 | 0.50 |
| 4 | 0.25 | 0.75 |
| 4 | 0.50 | 0.50 |
| 6 | 0.25 | 0.25 |
| 6 | 0.25 | 0.50 |
| 6 | 0.50 | 0.50 |

- For rank 3-4, the following parameter combinations achieve the best performance-overhead trade-offs

| L | (y0,v0) | β |
| --- | --- | --- |
| 2 | (1/4,1/8) | 0.25 |
| 4 | (1/4,1/8) | 0.25 |
| 4 | (1/4,1/8) | 0.50 |
| 4 | (1/4,1/4) | 0.75 |

Proposal 1: α is fixed, e.g. to 2 or 2.5.

Proposal 2: Support an unrestricted value set for Minit reporting, i.e., the bit-width of Minit is log2N3.

Proposal 3: Support size-2LMi bitmap for rank 3-4.

Proposal 4: Joint encoding of M’ and KNZ is not supported.

Proposal 5: Support a simple solution to avoid a polarization with all zero coefficients, i.e., for each layer, UE is expected to report at least 1 NZ coefficient cl,m with index l∈{0,1,..,L-1} and at least 1 NZ coefficient cl,m with index l∈{L,..,2L-1}.

Proposal 6: Support the following parameter combinations

| L | p=y0 (rank 1-2) | p=v0 (rank 3-4) | β | Resections (if any) |
| --- | --- | --- | --- | --- |
| 2 | 1/4 | 1/8 | 1/4 |  |
| 2 | 1/4 | 1/8 | 3/4 |  |
| 4 | 1/4 | 1/8 | 1/4 |  |
| 4 | 1/4 | 1/8 | 1/2 |  |
| 4 | 1/4 | 1/4 | 3/4 |  |
| 4 | 1/2 | 1/4 | 1/2 |  |
| 6 | 1/4 | - | 1/4 | 32 ports, rank 1-2, R=1 |
| 6 | 1/4 | - | 1/2 |  |
| 6 | 1/4 | - | 3/4 |  |

Proposal 7: For UCI omission,

- Reuse Rel.15 UCI omission mechanism comprising the following segments

  - CSI part 2 wideband including the first PMI i1

  - CSI part 2 subband including the second PMI i2 segmented into two groups

    - G1 (retained and reported): first group of second PMI components

    - G2 (omitted): second group of second PMI components

- The NZ coefficients KNZ reported in UCI part 1 is used for grouping.

- The strongest coefficient (indicated via SCI) is included in G1.

Proposal 8: The PMI comprises a first PMI (i1) and a second PMI (i2).

- The PMI (i1) comprises the following components.

  - Oversampling (rotation) factor (q1,q2) for SD basis

  - SD basis indicator (layer-common)

  - FD basis indicator (layer-specific)

  - Strongest coefficient indicator (layer-specific)

  - Size-KNZ subset selection bitmap (layer-specific)

- The second PMI (i2) comprises the following layer-specific components.

  - Amplitude:

    - reference amplitude for the weaker polarization

    - differential amplitude for each of (KNZ-1) NZ coefficients

  - Phase for each of (KNZ-1) NZ coefficients

Proposal 9: Support the following for CBSR for Type II CSI overhead reduction in Rel. 16.

- For SD beams, reuse Rel. 15 CBSR mechanism for Type II CSI reporting.

- For FD beams/components, study whether CBSR is necessary.

- For amplitude restriction, reuse Rel. 15 2-bit amplitude restriction to restrict the overall amplitude pl,m=pref,l,m×pdiff,l.m

Proposal 10: Support N3=NSB×R for NSB×R>13 (i.e. Alt0)

Proposal 11: Reuse Rel. 15 W1 for extension of DFT-based compression to Type II port selection codebook, where W1 performs L port selection (common for two polarizations); and details follows Rel. 15.

# References

- Chairman’s notes, RAN1#97

- Chairman’s notes, RAN1#95

# Appendix A

For performance evaluation, the non-full-buffer system-level evaluation is carried out for Dense Urban (Macro only) channel model. The relevant simulation assumptions and parameters are according to the agreed assumptions in RAN1#94bis, and are enlisted in Table 3 in Appendix B.

- For SU, as reference scheme, Rel. 15 Type II for rank 1-2 and Rel. 15 Type I for rank 3-4 is considered in this evaluation. For comparison, simple extension of Rel. 15 Type II codebook up to rank 4 is also considered.

- For MU, as reference, Rel. 15 Type II with L = 2, WB+SB amplitude, and 8-PSK phase is considered.

Evaluation 1: FD basis indicator, α value

The performance-overhead trade-offs of the three candidate values {1.5, 2, 2,5} of α are compared. The results are provided in Figure 2 for the following parameters.

- β=14,12,34, R=2, 16PSK phase, and (y0,v0)=(1/4,1/8)

Figure 2: performance-overhead trade-offs for values of α

# Evaluation 2: FD basis indicator, Minit

The performance of B=1,2,3 for B-bit Minit, as explained in Section 2, is evaluated. The results are shown in Figure 3 for the following parameters.

- β=14, R=2, 16PSK phase, L=4, and (y0,v0)=(1/4,1/4)

Figure 3: Minit

# Evaluation 3: supported parameter combinations

The performance-overhead trade-offs of different parameter combinations according to Evaluation 3.1 through 3.6, as explained in Section 2, are compared. The results are shown in Figure 4 through Figure 9 for Evaluation 3.1 through 3.6, respectively.

Figure 4: performance-overhead trade-offs for Evaluation 3.1 (rank 1, L=2,4)

Figure 5: performance-overhead trade-offs for Evaluation 3.2 (dynamic rank 1-2, L=2,4)

Figure 6: performance-overhead trade-offs for Evaluation 3.3 (rank 1, L=6)

Figure 7: performance-overhead trade-offs for Evaluation 3.2 (dynamic rank 1-2, L=6)

Figure 8: performance-overhead trade-offs for Evaluation 3.5 (dynamic rank 1-4, N3≤19)

Figure 9: performance-overhead trade-offs for Evaluation 3.6 (dynamic rank 1-4, N3>19)

# Evaluation 4: N3

We compare the two alternatives (Alt1 and Alt2 in Section 1) for N3. For comparison, (Alt0) N3=NSB×R is also considered in this evaluation. The results are provided in Figure 10 (NSB=21, R=1) and Figure 11 (NSB=11, R=2) for the following parameters and assumptions.

- Spatial compression: L = 4

- Frequency compression: M = 6

- Rank 1 only

- N3=24 for Alt1 and Alt2

- Padding in Alt1 is random between the following three padding options.

  - Option 0: N3-NSB×R FD units are padded before the NSB×R FD units corresponding to the CQI SBs

  - Option 1: N3-NSB×R FD units are padded after the NSB×R FD units corresponding to the CQI SBs

  - Option 2: N3-NSB×R/2 FD units are padded before the NSB×R FD units corresponding to the CQI SBs, and the remaining N3-NSB×R-N3-NSB×R/2 FD units are padded after.

- For Alt2, N3/2 FD units are allocated to the first segment, and the remaining N3-N3/2 FD units are allocated to the second segment. In order to align the number of FD units in the CQI SBs and the total number of FD units in two segments, some of FD units in the two segments can overlap.

Figure 10: Performance-overhead trade-off for NSB=21 and R=1

Figure 11: Performance-overhead trade-off for NSB=11 and R=2

# Appendix B

Table 3: Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | Evaluation 1,2,3: FR1, 4GHz with 13 SBs, 10 MHz BW Evaluation 4: FR1, 4GHz, 20 MHz BW |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | L=2,4: 16 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ L=6: 32 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | Evaluation 1, 2, 3.5, 3.6: 4RX: (M, N, P, Mg, Ng, Mp, Np) = (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ Evaluation 3.1, 3.2, 3.3, 3.4, 4: 2RX: (M, N, P, Mg, Ng, Mp, Np) = (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Evaluation 1,2,3: 15kHz Evaluation 4: 30kHz |
| Number of RBs |  | Evaluation 1,2,3: 52 for 15 kHz SCS SB size = 4 and #SBs = 13 Evaluation: #SBs = 11, 21 |
| Simulation bandwidth |  | Evaluation 1,2,3: 10 MHz,15kHz SCS Evaluation 4: 20 MHz, 30kHz SCS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | Evaluation 1,2, 3.5, 3.6: SU Evaluation 3.1, 3.2, 3.3, 3.4, 4: SU/MU-MIMO with rank adaptation |
| MIMO layers |  | Up to 4 layers |
| CSI feedback |  | Feedback assumption - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | DMRS, CSI-RS, PDCCH |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | Evaluation 3.1,3.2,3.3,3.4: 50% Evaluation 1,2,3.5,3.6: 20% |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput vs CSI feedback overhead (bits) |
| Baseline for performance evaluation |  | Rel-15 Type II Codebook |