3GPP TSG RAN WG1 98			R1-1908499

Prague, Czech Republic, 26th – 30th August, 2019

Agenda item:	7.2.8.1

Source: 	Samsung

Title: 	Feature lead summary for offline email discussion

Document for:	Discussion and Decision

# Introduction

In this contribution, the outcome of offline email discussions on the following issues is summarized:

- Values of 

- Supported parameter combinations

Summary

## Values of 

The following was agreed in RAN1#97 [1]:

“In RAN1#98, finalize the values of  based on the following aspects

- Candidate values for  to be down selected/evaluated: at least {1.5, 2, 2.5}

  - The set of values is to be finalized via offline email discussion prior to RAN1#98

- Configuration of :

  - Whether it is independent of other FD compression parameters, or dependent on at least one of the other FD compression parameters, i.e. p (=y0, and/or v0 for RI=3-4), L, β, and/or R

  - Whether  is rank-specific or rank-common

Note: This is to be discussed along with the supported parameter combinations for (L, p, β, ).”

The views expressed over the email thread can be summarized below.

Table 1 Values of : summary of companies’ views

| Candidate value(s) | Company | Reasoning |
| --- | --- | --- |
| Add α=3 | MotM/Lenovo | We simulated the throughput-overhead with BW =40MHz, SCS =30kHz and NSB =13, in which we compared the cases R=1 (N3=13, with free FD basis selection) to the case R=2 (N3=26, with a window FD basis selection of size ceil(α.M)). Performance of window approach improves at larger α (especially at cell edge) without notable overhead increase under some scenarios. For instance, at (y0, v0)=(1/4,1/4), cell-edge throughput improves by 0.4% when α increases from 2.5 to 3, at the expense of 2.5 extra overhead bits on average. |
| Keep only as {1.5, 2, 2.5} candidates for down selection | Samsung | In response to MotM/Lenovo: - The benefit of two-step approach is overhead saving (when compared with one-step). However, if the alpha value is large (such as 3), there may not be much overhead saving. So, I am not sure alpha = 3 is reasonable for performance evaluation. - The 0.4% gain in cell-edge UPT is rather small, which could just be due to simulation noise, hence can be ignored - What is the performance gain you observe in average UPT? Do you [have] results for other (y0,v0) values? |

After another round of discussion, it seems that Mot/M and Samsung observe that the potential gain of adding =3 (over the existing 3 candidate values) is marginal. In addition, it is unclear that =3 can result in noticeable overhead reduction (in fact, it can result in a small overhead increase compared to =2.5).

Based on the discussion, the following offline agreement was reached.

Offline agreement: In RAN1#98, finalize the values of  via down selection from {1.5, 2, 2.5} - FFS: =3 as an additional candidate

## Supported parameter combinations

The following offline agreement was made after a short offline email discussion.

Table 2 Supported parameter combinations: criteria for down-selection

Offline agreement: Use the following criteria for down-selecting the supported parameter combinations: - Avoid overly complex down selection by reducing the number of combinations for (L,p,beta) only - Alpha has not been decided and making it N3 dependent is unnecessarily convoluted - For #1 - Remove combinations with: - the total overhead exceeding the max of Rel.15 Type II - the UPT lower than Rel.15 Type II for the same overhead - When several combinations appear redundant in terms of overhead (sharing similar overhead – or vice versa), choose the one with the best UPT, unless the best combo is shown to be scenario-dependent. Then it is justified to support several combos with more or less the same overhead Use the following format for a proposed supported parameter combination: L; p=y0 (rank 1-2); p=v0 (rank 3-4); β; Restriction (if any).

# References

- 3GPP RAN1, RAN1#97, Chairman Notes