3GPP TSG RAN WG1 #98	R1-1908506

# Prague, CZ, August 26th – 30th, 2019

Agenda Item:		7.2.9.2

Source:				Samsung

Title:					Cross-slot scheduling power saving techniques

Document for:	Discussion/Decision

# Introduction

In RAN1#97 [1], the following were agreed:

Agreements:

For an active DL and an active UL BWP, a UE can be indicated via L1-based signalling(s) from gNB to adapt the minimum applicable value(s) of K0, K2 and/or aperiodic CSI-RS triggering offset (with/without QCL_typeD configured).

Agreements:

To adapt the minimum applicable value of K0 (K2) for an active DL (UL) BWP, indication of the minimum applicable value is supported.

- FFS: Direct assignment of the minimum application value, indication of one value from one or multiple preconfigured or predetermined value(s), and/or implicit indication.

- FFS: How the indicated minimum applicable value is applied to the selection of a DL (UL) TDRA entry. Example directions include at least the following:

  - Excluding the invalid TDRA entries

  - Re-interpret the selected K0 (K2) value

Agreements:

When UE is indicated of the minimum applicable value of K0 (K2) for an active DL (UL) BWP, the application method to the selection of a DL (UL) TDRA entry is to be decided from:

- An entry in the active DL (UL) TDRA table with K0 (K2) value smaller than the indicated minimum is not expected by or not valid for the UE for the TDRA indication(s)

Agreements:

At least for the L1-based adaptation on the minimum applicable value of K2, it does not apply to PUSCH scheduled by MAC RAR for at least contention-based RACH procedure.

- FFS: Exclude contention-free RACH

- FFS: Exclude PUSCH scheduled with TC-RNTI

Agreements:

- To adapt the minimum applicable value of the aperiodic CSI-RS triggering offset for an active DL BWP, implicit indication by defining the minimum applicable value the same as the minimum applicable K0 value when indicated is supported.

Agreements:

At least for the L1-based adaptation on the minimum applicable value of K2, it does not apply to:

- PUSCH scheduled by RAR UL grants for contention-free RACH procedure

- PUSCH scheduled with TC-RNTI

Conclusion:

Companies are encouraged to check the following proposal for the application delay:

For an active DL and an active UL BWP, when UE is indicated by L1-based signalling(s) in slot n to change the minimum applicable value(s) of K0 and/or K2, UE is not expected to apply the new indicated minimum applicable value(s) before slot  for K0, or slot  for K2, where

- X = max(Y, Z)

- Y is the minimum applicable K0 value prior to the indicated change

- Z = [1]

  - Z is the smallest feasible non-zero application delay that may depend on DL SCS

  - FFS: Z > 1 for 60kHz/120kHz SCS or multi-TRP

- FFS: Cross-carrier scheduling with different numerologies

- FFS: interruption time, if any

This contribution considers remaining issues on cross-slot scheduling based power saving techniques.

# Cross-slot scheduling based power saving techniques

## Indication method

The minimum applicable value of K0 (K0min) and K2 (K2min) can be indicated commonly or separately. For common indication method, one unified minimum applicable value (Kmin) can be indicated to the UE, so that the UE can apply it to both DL and UL TDRA tables. Unified indication is simple in terms of signalling method as well as corresponding UE behaviour. Within the time duration indicated by Kmin, the UE can operate in power saving mode such as PDCCH processing time relaxation, micro sleep, and no OFDM symbol buffering (for DL). On the other hand, for separate indication method, K0min (or K2min) can be indicated by DL DCI(or UL DCI) to the UE, then the UE can apply it to the DL(or UL) TDRA table, separately. It looks like separate indication is better than common indication in terms of flexibility. However, the gain from this flexibility is very limited since different PDCCH monitoring occasions for DL and UL scheduling DCIs are not allowed in Rel-15. As long as the monitoring occasions are the same for the DL and UL DCIs, the unified indication is sufficient.

This unified minimum applicable value, Kmin, can be used to determine the minimum values for aperiodic CSI-RS triggering offset and SRS triggering offset as well.

Proposal 1: A unified value for minimum applicable value, Kmin, is indicated to the UE.

It was agreed that the minimum applicable value is indicated by using at least existing scheduling DCI with additional field and/or re-purposed existing field. Similar to other configurable fields in the DCI, a set of candidates of the minimum applicable values can be configured by higher layer signalling, then one of the configured values can be indicated by DCI. Alternatively, the set of candidates of the minimum applicable values can be determined based on UE assistance information of UE’s preferred values as proposed in companion contribution [3]. For the fallback purpose, the lowest entry of this field can be fixed as “default”. If a UE is indicated by the lowest entry, the UE can use the original configured TDRA table.

Proposal 2: A table including candidates of the minimum applicable values is predetermined by higher layer signalling or UE assistance information. The lowest entry is fixed as “default”.

Once Kmin is indicated, the entries in the DL/UL TDRA table with K0/K2 value smaller than Kmin is considered as the invalid entries. The UE should apply the indicated Kmin value to the TDRA tables for not only active BWP but also inactive BWP(s). Otherwise, the UE cannot operate in power saving mode properly. If the indicated Kmin value is larger than all of K0/K2 values in the configured TDRA tables, then there can be no valid entry. This should not be allowed at least for active BWP but can be allowed for inactive BWPs. If all entries of TDRA table are invalid for an inactive BWP, the UE does not expect to be switched to the inactive BWP.

Proposal 3: The minimum applicable value, Kmin, is applied to the TDRA tables for all configured BWPs.

Proposal 4: For active BWP, the UE does not expect to be indicated with Kmin value larger than all K0/K2 values. For an inactive BWP, if there is no valid entry after applying the Kmin value, the UE does not expect to be switched to the inactive BWP.

If UE misses a DCI indicating a new Kmin value, the understanding on Kmin value between gNB and UE can be different. For this case, the UE can receive a DCI indicating invalid entry in the TDRA table. At this time, the UE can be aware of something is wrong. As long as the Kmin value is indicated by the scheduling DCI, the impact due to miss detection is not that critical. Figure 1 shows an example when the UE misses DCI indicating new Kmin value. It is assumed that current Kmin=2 and application delay/PDCCH processing time/no buffering time are equal to Kmin=2. In slot 0, first DCI#1 indicating new Kmin=1 is transmitted but the UE misses it. The UE finished DCI#1 decoding at slot 1 but failed, so that the current understanding on Kmin of the UE is still 2. In slot 2, next DCI#2 scheduling PDSCH#2 with K0=1 is transmitted. Since the UE assumes Kmin=2, the UE finishes DCI#2 decoding in slot 3. The UE receives K0=1 which is smaller than current Kmin=2, so that the UE can be aware of DCI#1 is missed. The UE can update its Kmin value to 1 which is indicated by DCI#2. In addition, the UE can be aware of that PDSCH#2 is not received due to no symbol buffering in slot 3. Then the UE will transmit NACK on PUCCH in slot 5 as indicated by DCI#2. Then the gNB knows the UE receives DCI#2 well. Therefore, the understanding on Kmin between gNB and UE can be aligned again after slot 5. According to above example, although the UE misses DCI indicating new Kmin value, there is no critical issue as long as the new Kmin is indicated by applicable DCIs in the future. However, it’s possible that some scheduling DCI formats, such as DCI format 0_0 or  DCI format 1_0, does not carry Kmin, and may indicate invalid entry in the TDRA table when a new Kmin carried by a previous DCI is missed..

Figure 1. Example when UE misses DCI indicating new Kmin

Observation 1: If UE receives a DCI indicating with invalid entry in TDRA table, the UE detects misalignment of Kmin with gNB due to DCI miss detection, and may miss buffering PDSCH/PUSCH before receiving next new Kmin.

Proposal 5: UE falls back to lowest applicable value for Kmin when detects an invalid entry in TDRA table.

## Application delay

If a UE receives new Kmin value in slot n, the UE can apply it to determine valid entries in TDRA table after some delay (T), e.g., slot n + T. This application delay Tapp is basically needed to allow UE to relax its PDCCH processing time for power saving purpose. For this reason, in RAN1#97, it was concluded that application delay is at least a function of prior Kmin value. On top of this, at least following aspects can be further considered.

Minimum application delay (Tmin)

During RAN1#97 discussion, a minimum application delay Tmin (noted as Z=[1]) has been discussed for the same-slot scheduling case, i.e., Kmin=0. However, Tmin does not need to be larger than 0 for the same-slot scheduling. If the current Kmin is 0, the UE will operate in normal mode (i.e., no PDCCH processing time relaxation, symbol buffering). Then there is no issue to receive any PDSCH scheduling by the UE regardless of the newly indicated Kmin value since Kmin=0 is the smallest number. Therefore, Tmin = 0 is possible even for the same-slot scheduling case.

On the other hand, minimum scheduling offset can be taken into account to determine Tmin. The minimum scheduling offset is required at least for cross-carrier scheduling with different numerologies and cross-BWP scheduling. For cross-carrier scheduling with different numerologies, the minimum scheduling offsets are defined as in Table 1. For BWP switching, the switching delay is provided in Table 2. The UE is not expected to be scheduled with PDSCH with scheduling offset smaller than the BWP switching delay.

The minimum application delay Tmin can be determined by the minimum scheduling offset.

Table 1. Minimum scheduling offset for cross-carrier scheduling with different numerologies

| SCS of PDCCH | 15kHz | 30kHz | 60kHz | 120kHz |
| --- | --- | --- | --- | --- |
| low-to-high SCS (w/ slot quant.) | 4 | 4 | 8 | - |
| high-to-low SCS (w/o slot quant.) | - | 4 | 8 | [12] |

Table 2. Interruption length for BWP switching

| SCS | NR Slot length (ms) | Interruption length X (slots) |
| --- | --- | --- |
| 15kHz | 1 | 1 |
| 30kHz | 0.5 | 1 |
| 60kHz | 0.25 | 3 |
| 120kHz | 0.125 | 5 |

Maximum application delay (Tmax)

The UE is required to finish PDCCH decoding before the application delay to apply the new indicated Kmin value after the application delay. This implies that the application delay is a kind of maximal allowed delay that UE can relax the PDCCH processing time. If the application delay is defined as prior Kmin value and the prior Kmin is very large, it would take very long time to process PDCCH for some UE implementation. This very large PDCCH processing time relaxation may give some impact to other functionalities requiring DCI decoding (e.g., SFI, SPS release, CSI reporting triggering, etc.). Therefore, it would be better to consider upper bound for the time to relax PDCCH processing time and this value can be used to restrict application delay under certain level. The maximum application delay, Tmax, can be fixed in the spec or configured by higher layer signaling.

Additional application delay

The application delay is tightly related to PDCCH processing time. Unfortunately, PDCCH processing time is not explicitly captured in the spec. Anyway, it is trivial that the UE can start PDCCH decoding after receiving PDCCH symbols. Therefore, if the UE receives the PDCCH symbols in the end of a slot, one more slot can be required to finish the PDCCH decoding. Considering this aspect, an additional delay (d0) can be considered in addition to the application delay. For example, the additional delay can be defined as d0 = 1 (slot) if the ending symbol is later than 7-th symbol in a slot, otherwise d0=0.

Based on the above discussions, the application delay can be defined as follows:

T = min(max(Kmin,prior, Tmin) + d0, Tmax), where Kmin,prior is the minimum applicable value prior to change

Proposal 6: The application delay takes into account at least following aspects

- Minimum application delay(Tmin)

- Maximum application delay (Tmax)

- Additional application delay (d0)

Proposal 7: When a UE is indicated by L1-based signalling(s) in slot n to change the minimum applicable value(s) of K0/K2, the UE is not expected to apply the new indicated minimum applicable value(s) before slot n’ for K0/K2, where

Application delay at gNB side

Since there is no HARQ-ACK feedback with respect to PDCCH reception, gNB and UE may apply different applicable value for Kmi if UE fails on receiving a DCI indicating a new Kmin.. As illustrated in Figure 2(a), UE may miss buffering PDSCH when Kmi assumed by UE is larger than the Kmi assumed by gNB. In this case, UE ends up wasting unnecessary energy on retransmission PDSCH, and gNB may trigger unnecessary link adaptation to improve PDSCH reception while the channel condition is good.

To avoid unnecessary retransmission of PDSCH due to miss detection of DCI, gNB can have an applicable delay for new Kmin that is relatively larger than the one used by UE, such that T2 = N*T1, where T1 is the applicable delay at UE, and N is a positive integer, e.g. N = 2.  As illustrated in Figure 2 (b), the longer applicable delay for a new Kmin at gNB makes UE robust against miss detecting of DCI. The retransmission of PDSCH due to miss buffering can be reduced because of the extended application delay at gNB side.

For flexible configuration, the application extension factor used at gNB side, N >=1, can be determined based on assistance information transmitted from UE.

(a)

(b)

Figure 2. Examples of application delay for new Kmin: (a) UE = gNB (b) gNB > UE.

Proposal 8: Support application delay used by gNB to be N>=1 times of the application delay used by UE, and N is determined based on UE assistance information.

## Impacts on the existing functionalities

If a UE relaxes its PDCCH processing time, some UE behaviours which is performed after PDCCH decoding can be affected due to the relaxed timeline. At least following functionalities may have some impact from the PDCCH processing time relaxation.

BWP operation

To ensure that BWP switching delay (Tbwp), a UE does not expect to detect a DCI format 1_1/0_1 indicating BWP change with the corresponding TDRA field providing a slot offset value for a PDSCH/PUSCH that is smaller than a delay required by the UE for an active BWP change. If the UE relaxes PDCCH processing time for DCI format 1_1/0_1, the UE can be aware of BWP switching at the very late time. For example, in normal case (i.e., w/o PDCCH processing time relaxation), if Kmin=2 and Tbwp = 1, the UE can receive DCI format scheduling PDSCH with K0=2 in new BWP. On the other hand, in power saving mode (i.e., w/ PDCCH processing time relaxation), if the UE relaxes PDCCH processing time until 2 slots, then the UE cannot receives the PDSCH with K0=2 in the new BWP since Tbwp = 1slot is required but not ensured. Therefore, if the UE relaxes PDCCH processing time up to T slots, this additional delay should be taken into account for cross-BWP scheduling. For example, the UE can only expect to be scheduled with data with slot offsets larger than T + Tbwp.

Proposal 9: A UE does not expect to detect a DCI format 1_1/0_1 indicating BWP change with the corresponding TDRA field providing a slot offset value for a PDSCH/PUSCH that is smaller than T + Tbwp where T is an application delay for cross-slot scheduling and Tbwp is a delay required by the UE for an active BWP change.

Since Kmin is indicated by scheduling DCI, Kmin and BWP index are indicated together. Therefore, if BWP switching is triggered by a DCI, the UE can apply the Kmin value indicated by the DCI to the new BWP. However, if BWP switching is triggered due to bwp-InactivityTimer, the UE cannot know the Kmin value to be applied to the default BWP. For this case, the UE can assume a default Kmin value for the new BWP until a new Kmin value is indicated by DCI.

Proposal 10: If BWP switching is triggered due to bwp-InactivityTimer expiration, a default Kmin value is assumed for the default BWP.

SPS release

A UE provides HARQ-ACK information in response to a SPS PDSCH release after N symbols from the last symbol of a PDCCH providing the SPS PDSCH release. For UE processing capability 1 and for the SCS of the PDCCH reception, N=10 for 15 kHz, N=12 for 30 kHz, N=22 for 60 kHz, and N=25 for 120 kHz. For a UE with capability 2 in FR1 and for the SCS of the PDCCH reception, N=5 for 15 kHz, N=5.5 for 30 kHz, and N=11 for 60 kHz. If Kmin is provided to a UE, “N” should be increased considering the potential PDCCH processing time relaxation of the UE.

Proposal 11: A UE provides HARQ-ACK information in response to a SPS PDSCH release after N’ symbols from the last symbol of a PDCCH providing the SPS PDSCH release where N’ = max(Tsym, N), N is defined in the current spec, and Tsym corresponds to application delay in symbol unit if cross-slot scheduling power saving technique is enabled .

SFI determination

A SFI-index field value in a DCI format 2_0 indicates to a UE a slot format for each slot in a number of slots starting from a slot where the UE detects the DCI format 2_0. The number of slots is equal to or larger than a PDCCH monitoring periodicity for DCI format 2_0. If PDCCH processing time is relaxed, for the PDCCH conveying DCI format 2_0 received in slot n, its decoding can be finished in slot n+T. Therefore, the UE can obtain the slot formation information for a number of slots starting from slot n to slot n+T-1 at slot n + T. In other words, the UE gets slot format information for the past. To avoid, this kind of situation, it is desirable not to relax PDCCH processing time for DCI format 2_0.

Proposal 12: For decoding DCI format 2_0, UE does not relax PDCCH processing time.

CSI triggering

As aforementioned UE may miss buffering OFDM symbols during the application delay, which may include OFDM symbols related with CSI process. For instance, the missing OFDM symbols may contain a DCI that activates/deactivates a semi-persistent CSI report or triggers an aperiodic CSI report. In another example, some of CSI-RS OFDM symbols can be overlapped with the missing OFDM symbols. One simplest solution could be prioritizing CSI related processes or OFDM symbols over power saving operation for all cases but it may limit the potential power saving gain especially for the UE with short CSI-RS periodicity. Alternatively, UE may assume that CSI-RS transmission occasions on the non-buffered OFDM symbols are not valid for the associated CSI report.

Proposal 13: UE may assume that CSI-RS transmission occasions on the non-buffered OFDM symbols are not valid for the associated CSI report.

## Exceptional case

Handling default TDRA table

The adaptation on the minimum applicable value of K0/K2 should not affect to the any common DCI. In this regard, some of RNTIs in CSS are agreed not to apply the minimum applicable value. For the unicast DCI, e.g., C-RNTI/MCS-C-RNTI/CS-RNTI, we need to take into account which search space set is used for monitoring the unicast DCI. The unicast DCI can be monitored in CSS configured by either PDCCH-Config or PDCCH-ConfigCommon. If it is configured by PDCCH-Config, the DCI format 0_0/1_0 does not used for scheduling broadcast message. Therefore, it would be find to apply the minimum applicable value. On the other hand, if the search space set corresponds to one of the search space sets configured by PDCCH-ConfigCommon, then not only C/MCS-C/CS-RNTI but also SI/RA/P-RNTI are monitored together as described in below [2]:

If a UE is provided - one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, and - a C-RNTI, an MCS-C-RNTI, or a CS-RNTI, the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI in the one or more search space sets in a slot where the UE monitors PDCCH candidates for at least a DCI format 0_0 or a DCI format 1_0 with CRC scrambled by SI-RNTI, RA-RNTI or P-RNTI.

In this case, the minimum applicable value does not need to be applied.

Regarding PUSCH scheduled by RAR UL grant, it is more desirable not to apply the minimum applicable value since it may cause delay during random access.

Proposal 14: The adaptation on the minimum applicable value of K0 does not apply to at least the following cases:

| RNTI | PDCCH search space |
| --- | --- |
| C-RNTI, MCS-C-RNTI, CS-RNTI | one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace |  |
| PUSCH scheduled by MAC RAR described in subclause 8.2 in [2] |  |

Handling default value when K0/K2 is absent

In TS38.331, a default value for K0/K2 is defined for the case when the field of K0/K2 is absent in the configured TDRA tables, as K0/K2 in the configured TDRA tables are optional parameters.

PUSCH-TimeDomainResourceAllocationList field descriptions

k2 Corresponds to L1 parameter 'K2' (see TS 38.214 [19], clause 6.1.2.1) When the field is absent the UE applies the value 1 when PUSCH SCS is 15/30 kHz; the value 2 when PUSCH SCS is 60 kHz, and the value 3 when PUSCH SCS is 120KHz.

PDSCH-TimeDomainResourceAllocationList field descriptions

k0 Slot offset between DCI and its scheduled PDSCH (see TS 38.214 [19], clause 5.1.2.1) When the field is absent the UE applies the value 0.

For applying applicable value of minimum K0/K2 indicated by DCI, we need address the case when the field of K0/K2 is absent in configured TDRA table. To enable power saving gain, the explicit minimum K0/K2 indicated by DCI can be considered as default K0/K2 directly; otherwise UE doesn’t expect to relax timeline as the fixed default value for K0/K2 is very small.

Proposal 15: Indicated minimum applicable value of K0/K2 are used as default values when the field of K0/K2 is absent in the configured TDRA table.

URLLC

As URLLC traffic can be sporadic and require low latency, it makes sense to consider different minimum applicable value of K0/K2 for URLLC service than MBB service. To avoid negative impact on latency for URLLC service, the signaling of minimum applicable value of K0/K2 can be applicable only for SS sets associated with non-URLLC DCI formats. If the UE is indicated large minimum applicable value for K0/K2, the UE applies this only for SS sets associated with the MBB UE-specific DCI formats. The indication is not applicable to SS sets associated with a URLLC DCI.

Proposal 16: Indicated minimum applicable value of K0/K2 are not applicable for search space sets associated with URLLC DCI.

## PDCCH skipping

For adaptation on min K0/K2 with candidate values larger than one, a UE can achieve power saving gain by means of 1) no PDSCH buffering, 2) micro-sleep after receiving PDCCH, 3) relaxing PDCCH processing timeline, etc. However, if the PDCCH monitoring periodicity is less than the minimum applicable K0/K2, UE cannot fully achieve the power saving gain due to the following issues.

Firstly, without considering on the PDCCH monitoring periodicity, the PDCCH processing time can be relaxed to a duration that is less then minimum K0/K2 for power saving purpose. However, the PDCCH monitoring occasions between a time of PDCCH reception and a time of potential PDSCH reception will accumulate to be processed. As illustrated in Figure 2, due to the accumulated PDCCH monitoring occasions over time, a UE has to start buffering PDSCH samples. In addition, the micro-sleep duration after receiving PDCCH is limited by the next PDCCH monitoring occasions other than the minimum K0/K2 as illustrated in Figure 3.

Therefore, joint adaptation on PDCCH skipping should be considered when UE wants to relax the PDCCH processing timeline for power saving purpose.

Figure 3: illustration of issues on adaptation on min K0/K2 without considering on PDCCH monitoring periodicity

There are two options for joint adaptation on PDCCH skipping based on the dynamically indicated minimum applicable value for K0/K2, i.e. Kmin.

- Option 1) UE adapts PDCCH periodicity to be no smaller than Kmin.

- Option 2) UE skips remaining PDCCH monitoring within time period [n, n+minK0), where n is the first PDCCH monitoring occasion with a periodicity.

Proposal 17: Support joint adaptation on minimum K0/K2 and PDCCH skipping if the UE is operated with cross-slot scheduling based power saving.

# Conclusion

This contribution considered cross-slot scheduling based power saving techniques. Following observations and proposal were made:

Proposal 1: A unified value for minimum applicable value, Kmin, is indicated to the UE.

Proposal 2: A table including candidates of the minimum applicable values is predetermined by higher layer signalling or UE assistance information. The lowest entry is fixed as “default”.

Proposal 3: The minimum applicable value, Kmin, is applied to the TDRA tables for all configured BWPs.

Proposal 4: For active BWP, the UE does not expect to be indicated with Kmin value larger than all K0/K2 values. For an inactive BWP, if there is no valid entry after applying the Kmin value, the UE does not expect to be switched to the inactive BWP.

Proposal 5: UE falls back to lowest applicable value for Kmin when detects an invalid entry in TDRA table

Proposal 6: The application delay takes into account at least following aspects

Proposal 7: When a UE is indicated by L1-based signalling(s) in slot n to change the minimum applicable value(s) of K0/K2, the UE is not expected to apply the new indicated minimum applicable value(s) before slot n’ for K0/K2, where

Proposal 8: Support application delay used by gNB to be N>=1 times of the application delay used by UE.

Proposal 9: A UE does not expect to detect a DCI format 1_1/0_1 indicating BWP change with the corresponding TDRA field providing a slot offset value for a PDSCH/PUSCH that is smaller than T + Tbwp where T is an application delay for cross-slot scheduling and Tbwp is a delay required by the UE for an active BWP change.

Proposal 10: If BWP switching is triggered due to bwp-InactivityTimer expiration, a default Kmin value is assumed for the default BWP.

Proposal 11: A UE provides HARQ-ACK information in response to a SPS PDSCH release after N’ symbols from the last symbol of a PDCCH providing the SPS PDSCH release where N’ = max(Tsym, N), N is defined in the current spec, and Tsym corresponds to application delay in symbol unit if cross-slot scheduling power saving technique is enabled .

Proposal 12: For decoding DCI format 2_0, UE does not relax PDCCH processing time.

Proposal 13: UE may assume that CSI-RS transmission occasions on the non-buffered OFDM symbols are not valid for the associated CSI report..

Proposal 14: The adaptation on the minimum applicable value of K0 does not apply to at least the following cases:

| RNTI | PDCCH search space |
| --- | --- |
| C-RNTI, MCS-C-RNTI, CS-RNTI | one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace |  |
| PUSCH scheduled by MAC RAR described in subclause 8.2 in [2] |  |

Proposal 15: Indicated minimum applicable value of K0/K2 are used as default values when the field of K0/K2 is absent in the configured TDRA table.

Proposal 16: Indicated minimum applicable value of K0/K2 are not applicable for search space sets associated with URLLC DCI.

Proposal 17: Support joint adaptation on minimum K0/K2 and PDCCH skipping if the UE is operated with cross-slot scheduling based power saving.

Observation 1: If UE receives a DCI indicating with invalid entry in TDRA table, the UE detects misalignment of Kmin with gNB due to DCI miss detection, and may miss buffering PDSCH/PUSCH before receiving next new Kmin.

# References

- RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #97

- 3GPP TS 38.213 v15.5.0 (2019-03), “Physical layer procedures for control (Release 15)”

- R1-1908508, “On UE assistance information” Samsung, RAN1#98, August, 2019