3GPP TSG RAN WG1 #98	R1-1908512

Prague, CZ, August 26th – 30th, 2019

Agenda item:	7.2.10.4

Source: 	Samsung

Title: 	Discussion on necessity and details for physical-layer procedures to support UE/gNB measurements

Document for:	Discussion and Decision

# Introduction

A new work item on “NR positioning support” was approved in RAN#83 and the following agreements were made in RAN1#97 [1].

Agreement:

For intra-frequency measurements, when measurement gaps are configured, the UE is expected to measure the DL PRS resource outside the configured (active and inactive) BWPs (Option 2 in the related Agreement from RAN1 #96-Bis).

- Note: The UE has flexibility to measure only on a part of the DL PRS configured bandwidth as long as measurement requirements are met.

Agreement:

For positioning purposes, to assist UE to perform Rx beamforming, the following options are supported (options are from previous related agreement in RAN1#96bis):

- Option 1: The DL PRS can be configured to be QCLed Type D with a DL Reference Signal from a serving or neighboring cell.

  - Discuss further which DL reference signal can be used (e.g., SSB, CSI-RS, DL-PRS).

- Option 2: The UE may perform a Rx beam sweeping on DL PRS resources which are transmitted with the same downlink spatial domain transmission filter.

- Option 3: The UE may use a fixed Rx beam to receive DL PRS resources which are transmitted with different downlink spatial domain transmission filter.

  - FFS: Whether this option can be achieved by Option 1

- FFS on specification impacts.

Agreement:

For positioning purposes, with regards to the UL SRS transmission power, support at least the following in addition to the existing Rel-15 behaviour:

- Support configuring a DL reference signal of a neighbouring cell to be used as DL path loss reference for the purpose of SRS power control.

  - FFS: Which reference signals can be used (e.g. a CSI-RS, or a SSB, or DL PRS).

- Study further the UE configuration signalling and procedure.

- FFS on a fallback procedure if the UE is not able to obtain the pathloss reference.

- FFS on number of measurements for pathloss.

Agreement:

For positioning purposes, for UL Beam management/alignment towards serving and neighbouring cells, the following is supported (in addition to UE TX beam sweeping):

- Configuration of a spatial relation between a reference DL RS from serving or neighbouring cells and the target SRS.

  - Reference DL RS that can be used include at least SSB. FFS on CSI-RS, DL-PRS.

- A fixed Tx beam for UL SRS transmissions across multiple UL SRS Resources, for both FR1 and FR2.

  - FFS how the fixed beam is selected.

- FFS on specification impacts.

- Note: In Rel-16, UE is not expected to transmit multiple SRS resources with different spatial relations in the same OFDM symbol (Rel-15 behaviour)

In this contribution, we discuss necessity and details for physical-layer procedures to support UE/gNB measurements.

# UE and gNB measurements

In our companion contribution [2], the following measurements are defined.

- UE measurement

  - DL RSTD

  - DL RSRP

  - UE Rx-Tx time difference

- gNB measurement

  - UL RTOA

  - UL Angle of Arrival

  - UL RSRP

  - gNB Rx-Tx time difference

The reference signals to be measured are also discussed in companion contribution [4] and it is proposed that CSI-RS/TRS and newly defined DL PRS can be used for UE measurement and SRS and PRACH can be used for gNB measurement. For UL measurement, UL PRS, e.g., SRS, is configured by the serving gNB but should be measured not only by the serving gNB but also the neighboring gNBs. The configuration of UL PRS should be exchanged between serving gNB and neighboring gNBs via X2 interface so that the neighboring gNBs know when and where exactly to measure UL PRS.

# Physical-layer procedures for NR positioning

## BWP operation

### UL BWP operation

A UE can be configured with up to 4 BWPs but only one BWP can be activated at one time. Even though UE might be capable of operating outside the BWP, it is not required to do so. As aforementioned, one of the main benefits for positioning using SRS is that SRS is an existing and well defined reference signal in NR thus the specification effort is minimal. Moreover, SRS can be used for positioning without disturbing the data/control information transmission by the UE. In this regard, the simplest solution is to configure the SRS in the active BWP for arrival time difference measurement. Another benefit of such solution is that different UEs may have different BWPs with different frequency location so that the interference from other UEs can be mitigated or randomized. However, similar to OTDOA, the time difference measurement accuracy may depend on many factors such as available bandwidth, subcarrier spacing and carrier frequency. The BWPs configured to a single UE may have different numerology, bandwidth, etc. With larger subcarrier spacing and shorter symbol duration, the timing measurement step gets smaller and it will generally lead to improved positioning accuracy. It might be necessary to configure a wider bandwidth or subcarrier spacing to meet the positioning accuracy target. In this regard, a BWP different from current active BWP, e.g., the BWP with the largest bandwidth or subcarrier spacing, can be configured to the UE for arrival timing measurement to achieve more accurate positioning. Such BWP can either be from the existing BWPs or a new dedicated BWP.

Proposal 1: For the BWP configuration of NR UL PRS, e.g. SRS, both solutions as follows should be supported

- Alt1: SRS is configured within the active UL BWP;

- Alt2: SRS is configured in a dedicated BWP with the largest bandwidth or subcarrier spacing.

## Power control

A major drawback for uplink based solutions is the poor hearability, especially in the UMa scenario where the distance between the UE and the gNB is large, because of the low transmission power of the UE. For UTDOA, another issue is the interference between UEs that could significantly limit the scalability.

Observation 1: Hearability significantly limits the applicability of UTDOA and due to interferences between UE the scalability of UTDOA is also limited.

UL PRS is expected to be received by multiple TRPs/gNBs but the current power control scheme is only based on the serving cell link quality. In order to improve the hearability of UL PRS, some enhancements for power control is needed. Power control can be categorized into open loop and closed loop depending on whether feedback is needed. For open loop power control, UE can measure received signal strength from multiple TRPs/gNBs and identify the weakest link. Then UE can transmit based on the weakest link as long as it is lower than the maximum allowed transmit power per carrier. However, open loop power control might keep a UE always transmitting with high power and thus causing strong interference to other UEs. Therefore, closed loop power control should also be considered. A UE can be allowed to receive multiple TPCs from multiple TRPs/gNBs and adjust its transmission power based on the received TPCs, e.g., maximum power, average power, etc. By doing this, a balance between hearability and interference can be well managed.

Proposal 2: In order to improve the hearability of UL PRS, both open loop and closed loop power control can be considered

- For open loop power control, UE can transmit based on the weakest link of surrounding TRPs/gNBs as long as it is lower than the maximum allowed transmit power per carrier;

- For closed loop power control, UE can receive multiple TPCs from multiple TRPs/gNBs and adjust its transmission power based on the received TPCs, e.g., maximum power, average power;

- A fall-back mechanism where UE always transmits with maximum power should also be supported, where the UE transmit with its maximum transmission power.

## Timing advance

For UL PRS, TA is configured by the serving cell and UE is expected to receive only one TA value. However, in UL positioning, UL PRS is supposed to be heard not only by the serving cell but also by as many neighboring cells as possible. For the neighboring cells far away from the UE, using TA values configured by the serving cell may cause an arrival time gap between UL PRS and signals from other UEs associated with the neighboring cells due to different propagation delay as shown in Fig. 1. Here UE A is associated with serving gNB A and UE B is associated with neighboring gNB B. UE A and B have different timing advance values configured by gNB A and B, respectively. However, there is a gap between UL PRS from UE A arrival timing and normal data/control from UE B arrival timing at neighboring gNB B, denoted as T4.

UE A Tx timing

UE A slot

UE A slot

T1

UE B Tx timing

UE B slot

UE B slot

T2

gNB A Rx timing

gNB B Rx timing

gNB A (serving gNB)

gNB B (neighbor gNB)

UE A slot

T3

T4=T3-T1

gNB B Rx timing for UL PRS from UE A

Fig. 1 TA difference

Considering the low transmission power of UE, UL PRS is not expected to be heard by neighboring cells located far away from the UE and the arrival timing gap T4 may be handled by CP. If T4 is larger than CP duration, there two different ways to adjust: 1) adjust the transmission timing of UE A; and 2) adjust the measurement window of gNB B. Compared with changing Tx timing, the spec impact of adjusting measurement window is less so that it is preferred to adjust the measurement window of gNB B to be aligned with UE A slot. If transmission timing adjustment is needed, a single TA should still be supported but its actual value can be based on multiple propagation delays including both the serving cell and neighboring cells. The configured TA can be adjusted for the purpose of transmitting UL SRS intended to a neighboring cell. However, the adjustment value can either be provided by the UE upon request or by the network to the UE. TA value can obtained via measurement of reference signals by both the serving cell and the neighboring cells. Adjusting TA may have impact on interference condition in both the serving cell and the neighboring cells, which is also closely relevant to the scheduling decisions. In such a case, only the network has the knowledge of such information and it is more reasonable for the network to adjust TA value rather than the UE.

Proposal 3:  The propagation delay difference between the serving cell and the neighbor cells can be handled in the following ways.

- Adjustment of measurement window;

- Adjustment of TA value based on multiple propagation delays and such adjustment should be indicated by the network.

## Measurement/reporting

The measurement metrics have been defined in study item stage and should be specified in this work item. However, the details of how UE should report its measurement results in a more efficient way has not been discussed.

UE report granularity

When UE reports back its measurement results, e.g., RSTD, the values should be quantized and the quantization granularity determines the quantization error as well as the positioning accuracy. Basically, the higher the quantization granularity the more accurate the positioning is. However, very high quantization granularity will also create large signaling overhead and may increase the positioning latency. In this regards, the quantization granularity should be determined by the required positioning accuracy. In this regard, multiple quantization granularities should be supported. For high accuracy requirement, finer quantization granularity can be configured and for low accuracy requirement, coarse granularity can be configured to reduce signaling overhead.

DL PRS quality

It has been agreed that NR should support RSRP measurement for NR DL PRS and such measurement can be used to determine DL PRS quality. The quality reflects how accurate the positioning measurement is expected. However, since UE needs to measure multiple gNBs and reporting all RSRP measurements may incur high signaling overhead. In such a case, UE can report only a subset of RSRP measurements based on gNB configuration to reduce reporting signaling overhead. For example, UE can report only the best N RSRP measurements. It should be noted that the RSRP measurement can be reported in a timing granularity different from RSTD measurement since the signal strength may change even without UE movement.

UE measurement window for low power consumption

PRS with wider bandwidth and short periodicity can be configured and shared by a group of UEs to guarantee that the most stringent positioning requirement can be met. However, in order to save UE power consumption, especially for IoT devices with stringent power consumption requirements, different measurement window in time and frequency domain can be configured to different UEs.

Proposal 4: Following aspects should be supported for UE measurement and report

- Multiple UE report granularities;

- Signalling overhead reduction for DL PRS quality report;

- Configurable UE measurement window for low complexity and low power consumption.

# Conclusions

In this contribution, we discuss necessity and details for physical-layer procedures to support UE/gNB measurements. Our observations and proposals are summarized below.

Observation 1: Hearability significantly limits the applicability of UTDOA and due to interferences between UE the scalability of UTDOA is also limited.

Proposal 1: For the BWP configuration of NR UL PRS, e.g. SRS, both solutions as follows should be supported

- Alt1: SRS is configured within the active UL BWP;

- Alt2: SRS is configured in a dedicated BWP with the largest bandwidth or subcarrier spacing.

Proposal 2: In order to improve the hearability of UL PRS, both open loop and closed loop power control can be considered

- For open loop power control, UE can transmit based on the weakest link of surrounding TRPs/gNBs as long as it is lower than the maximum allowed transmit power per carrier;

- For closed loop power control, UE can receive multiple TPCs from multiple TRPs/gNBs and adjust its transmission power based on the received TPCs, e.g., maximum power, average power.

- A fall-back mechanism where UE always transmits with maximum power should also be supported, where the UE transmit with its maximum transmission power.

Proposal 3:  The propagation delay difference between the serving cell and the neighbor cells can be handled in the following ways.

- Adjustment of measurement window;

- Adjustment of TA value based on multiple propagation delays and such adjustment should be indicated by the network.

Proposal 4: Following aspects should be supported for UE measurement and report

- Multiple UE report granularities;

- Signalling overhead reduction for DL PRS quality report;

- Configurable UE measurement window for low complexity and low power consumption.

# References

- RAN1#97 Chairman’s notes.

- R1-1908509, “DL reference signals design for NR positioning,” Samsung, RAN1#98.