3GPP TSG RAN WG1 #98						R1-1908543

Prague, CZ, August 26th – 30th, 2019

Agenda Item	:	7.2.6.3

Source	: 	LG Electronics

Title	: 	PUSCH enhancements for NR URLLC

Document for	:	Discussion and decision

- Introduction

In previous meetings [1], several agreements were made on PUSCH transmission for NR URLLC:

Agreements: Adopt option 4 with the following update: - The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates the resource for the first “nominal” repetition. - FFS the detailed interaction with the procedure of UL/DL direction determination

In this contribution, we provide our view on PUSCH enhancement for URLLC, including remaining issues for adopting option 4.

- Discussion

  - How to determine “nominal” repetition based on RA information

According to the agreement in RAN1#97, option 4 can be stated as below:

Option 4: One or more actual PUSCH repetitions in one slot, or two or more actual PUSCH repetitions across slot boundary in consecutive available slots, is supported using one UL grant for dynamic PUSCH, and one configured grant configuration for configured grant PUSCH. - The number of the repetitions signaled by gNB represents the “nominal” number of repetitions. The actual number of repetitions can be larger than the nominal number. - FFS dynamically or semi-statically signalled for dynamic PUSCH and type 2 configured grant PUSCH - The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates the resource for the first “nominal” repetition. - FFS the detailed interaction with the procedure of UL/DL direction determination - The time domain resources for the remaining repetitions are derived based at least on the resources for the first repetition and the UL/DL direction of the symbols. - FFS the detailed interaction with the procedure of UL/DL direction determination - If a “nominal” repetition goes across the slot boundary or DL/UL switching point, this “nominal” repetition is splitted into multiple PUSCH repetitions, with one PUSCH repetition in each UL period in a slot. - Handling of the repetitions under some conditions, e.g., when the duration is too small due to splitting, is to be further investigated in the WI phase. - Note: each UL period is the duration of a set of contiguous symbols within a slot for potential UL transmission as determined by the UE - No DMRS sharing across multiple PUSCH repetitions - The maximum TBS size is not increased compared to Rel-15. - FFS: L > 14 - S+L can be larger than 14 - FFS: The bitwidth for TDRA is up to 4 bits. - Note: different repetitions may have the same or different RV.

Still, there can be potentially multiple ways for interpretation on how to determine “nominal” repetition based on TDRA as exemplified in figure. The figure shows examples of each handling when S=2, L=4, K=4.

Fig. Examples of slot format handling for “nominal” repetition

For dropping, it would shorten PUSCH duration based on slot format so the ending of nominal repetition does not exceed L*K symbols from the indicated starting symbol. For postponing, a set of UL (and/or flexible) symbols is excluded for determining “nominal” repetition so it would guarantee the transmission of L*K symbols while it would have a drawback in terms of latency. For no handling, it just has a benefit of keeping SLIV shape in perspective of “nominal” repetition.

Considering configured grant, postponing would make some problem whatever we have as detail. Especially when the total transmission duration is lengthened, it would not only bring latency drawback but also transmission across periodicity boundary of configured grant, which could be a critical impact for configured grant PUSCH. Comparing dropping with no handing, the only difference is the shrunk “nominal” repetition, however, there would be no difference of “actual” repetition between them in the end after the segmentation procedure.

Considering that the latency would be dependent on when the total transmission is finished, postponing seems not a good solution. One can argue that no handling or dropping would not be able to guarantee the required amount of transmission resource, however, which can be managed by MCS adjustment/RB allocation/etc. In the sense, it would be more desirable to handle slot format interaction during segmentation for determining “actual” repetition.

Proposal 1: “nominal” repetition has no interaction with slot format and slot boundary.

  - How to determine “actual” repetition based on RA information

When an “actual” repetition is being defined, the latency is still an essential criterion to be taken into account. In case one “nominal” repetition is split into two segmentation due to UL/DL switching point and thus a set of symbols is not available for PUSCH transmission and thrown away, then one can consider that as compensation of these symbols, the two “actual” PUSCHs after segmentation should maintain the same duration as the original “nominal” repetition. This could be helpful in terms of reliability, however, as discussed above, it would incur latency. In our view, it is required to guarantee that a nominal repetition should not be changed when any other previous nominal repetition is being segmented. In other words, after segmentation, one or multiple “actual” repetition derived from a “nominal” repetition should be confined within the “nominal” repetition.

Proposal 2: After segmentation, one or multiple “actual” repetition derived from a “nominal” repetition should be confined within the “nominal” repetition.

  - How to determine UL/DL switching point based on slot format

For option 4, one “nominal” repetition can span UL-DL switching point. It hasn’t discussed yet since there was no contiguous resource allocation crossing slot boundary in Rel.15. Moreover, UL-DL switching point cannot be seen as deterministic “point” by UE. Perhaps, UE is only capable to assume few duration as switching point from a lot of flexible symbol between DL and UL. So, it is necessary for UE to assume a set of flexible symbols right after DL as invalid, which is implicitly used for UL-DL switching. And for successful UL transmission, gNB and UE should have same assumption on the set of symbols for UL-DL switching and thus this can be configured/indicated by gNB. Alternatively, a UE can expect no DL transmission in required switching period right before UL. In other words, a UE can regard a certain time duration as guard period itself which should be the same understanding as gNB.

Proposal 3: If a nominal repetition spans UL-DL switching point, following can be considered:

- Option 1: UE assume that X symbols after the preceding DL symbol are invalid for UL

- Option 2: UE assume that X symbols before beginning of given PUSCH are invalid for DL

- FFS: how to determine X

By doing so, UE and gNB could have same assumption on UL period for constructing PUSCH repetition. It means that UE can utilize flexible symbols for PUSCH repetition with proper guard period from DL transmission.

  - Remaining consideration points for enhanced RA

As shown from option 4, there are still some decision point. For examples, we need to specify how to determine TBS and RV. In addition to this, it is also necessary to handle wrong segmentation for Option 4. For instance, a segment having only 1 symbol or not sufficient for given TBS, should be handled. And, to indicate time-domain resource allocation S+L>14 or L>14, new SLIV design would be need. In below sections, we provide our view on those aspects.

DMRS mapping for enhanced PUSCH

For option 4, it has been discussed how to determine DMRS symbol for PUSCH. There are two of discussion points. One is DMRS symbol position of repeated PUSCH and the other is DMRS symbol position when one nominal PUSCH is split.

When Option 4 is used, UE may assume repeated PUSCH allocation in back-to-back manner from one SLIV indicated by DCI. So, some of PUSCH allocation would be different from original SLIV. If originally indicated SLIV instructs to use DMRS mapping type A with the PUSCH staring at beginning of slot, PUSCH allocation of following repetitions may be not able to apply DMRS mapping type A. So it is necessary to consider how to apply DMRS mapping type A for option 4.

There are several ways to solve this problem. Firstly, UE can expect that DMRS mapping type B is used when repetition is used. Alternatively, UE can apply DMRS mapping type B for repetitions arbitrarily. On the other hand, if we consider configured grant, UE may need to use same DMRS symbol position among different UE and different PUSCH allocation. In this sense, it may be beneficial to use same relative DMRS symbol position of first PUSCH for following repetitions.

Proposal 4: for PUSCH with DMRS mapping type A, following can be considered:

- Alternative 1: a UE doesn’t expect to receive DCI indicating DMRS mapping type A when repetition is used.

- Alternative 2: a UE applies DMRS mapping type B for following repetitions when PUSCH is scheduled with DMRS mapping type A

- Alternative 3: a UE assumes same relative DMRS symbol position of first PUSCH for following repetitions when repetition is used

If nominal repetition goes across the slot boundary or DL-UL switching point, nominal repetition is split into multiple of actual repetition. Otherwise, one nominal repetition becomes one actual repetition. If no DMRS sharing among repetitions is assumed, for UE to receive PUSCH of actual repetition, each actual repetition should have own DMRS symbol.

There are two ways to assign DMRS symbol for each actual repetition.

- Method 1: The first option is to apply DMRS symbol position which is determined by nominal repetition. For example, based on resource allocation of nominal repetition and given DMRS parameter, UE can determine virtual DMRS symbol position before splitting. After obtaining actual repetitions, UE applies the absolute position of the virtual DMRS symbols for all actual repetition. This approach may bring similar DMRS overhead between nominal and actual repetitions, however, in some cases special handling of actual repetition having no DMRS symbol might be needed.

- Method 2: Alternatively, UE can determine DMRS symbol from actual repetition (after segmentation) and given DMRS parameter. Regardless of where nominal repetitions are located, UE can apply given DMRS parameter for each actual repetition. By doing so, we can fully re-use existing DMRS mapping rule for option 4.

(a) Method 1                                  (b) Method 2
Figure 1. Examples of DMRS mapping methods for option 4

First of all, considering the case where nominal PUSCH contains only front-loaded DMRS, for method 2, by following DMRS parameter (i.e., front-loaded DMRS only), after segmentation, each actual repetition will have front-loaded DMRS. On the other hand, for method 1, as DMRS location is just inherited from nominal PUSCH, the second segment of actual repetition would have no DMRS. Thus, for method 1, as discussed in the last meeting, it seems reasonable that DMRS is transmitted at the beginning of each actual repetition, which results in the same outcome in terms of DMRS transmission.

In case additional DMRS is configured, method 1 still can suffer from absence of DMRS in some segment (e.g., if splitting happen after the last DMRS symbol), and thus the above special handling would be needed. On the other hand, method 2 can follow the existing DMRS parameter, in some cases, it may bring bit larger DMRS overhead up to configuration. For this case, UE can assume different DMRS parameter or use different table for DMRS position.

Proposal 5: DMRS determination should be based on each actual repetition after segmentation.

.

TBS determination

When we try to follow Rel.15 UE behavior, both scheme have some problems. For mini-slot based repetition, actual code rate is multiplied by number of repetitions if TBS is determine by one PUSCH instance. It would bring modulation order which is inappropriate to given code rate. For multi-segment transmission, it haven’t discussed how to determine TBS.

For RA scheme like “mini-slot based repetition”, considering that some repetition/symbols can be cancelled during time-domain resource determination, it can be beneficial to set MCS for one repetition. Especially, in the case of configured grant, available number of repetitions would be changed by dynamic SFI. So, at least for configured grant, it is preferred not to change current behavior. It is safer to maintain a reliability of one repetition regardless of SFI interaction.

If we use RA scheme like “multi-segment”, resource allocation would be separated multiple PUSCH instance every slot boundary. To serve one TB with multiple PUSCH, each PUSCH instance should have same TBS. So, it is necessary to share same TBS determination among PUSCH instances. As the simplest method, TBS can be determined by REs in resource allocations before separation. In this case, up to MCS and TBS value, some separated resource wouldn’t sufficient for UL transmission. For example, when one repetition have 3 symbols and original resource allocation is 14 symbol with IMCS=10, one repetition cannot convey even systematic bits only. It means that some systematic bits would be not transmitted up to scheduling and RV sequences. It would bring critical performance degradation or scheduling restriction.

Proposal 6: For TBS determination, it is necessary to consider shortened transmission duration.

- Option 1: TBS is determined by the resource of the one repetition

  - FFS: consider nominal repetition or actual repetition for option 4

  - FFS: Which repetition is considered (initial, largest, smallest)

- Option 2: TBS is determined by entire allocated resource for a TB

  - one repetition should be sufficient to convey X coded bits

  - FFS: size of X (e.g., TBS or TBS*(certain target code rate)-1 )

RV determination

As we considered in TBS section, RV 0 for small resource of segmented/repeated transmission may incur the problem that the sufficient number of systematic bits cannot be conveyed. To maintain maximum spectral efficiency in case of using enhanced PUSCH RA schemes, TBS could be determined by actual entire resource for a TB. In this case, it would be beneficial to allocate RV carefully in order to transmit systematic bits enough.

To allocate larger resource as much as possible for systematic bits, it can be considered to map RV sequence according to resource size. By doing so, largest repetition is mapped to RV0 so that it conveys sufficient systematic bits for a TB.

Alternatively, we can use circular buffer contiguously for same RV or for repetitions. To be specific, regardless of number of repetition, coded bits of TB can be mapped contiguously across all of repetitions as one PUSCH. Since there is no DCI missing ambiguity among repetitions unlike HARQ retransmissions, it is not necessary to stick conventional RV approach for enhance PUSCH allocation scheme.

Proposal 7: For PUSCH enhancement, how to mapping RV or coded bits among repetition should be specified.

- Option 1: The transmission with the n-th resource among all repetitions for a TB is associated with the (n+k)-th value among RV sequences, where the k is the index of the largest resource among all repetitions for a TB.

- Option 2: The transmission with the n-th largest resource among all repetitions for a TB is associated with the n-th value among RV sequences.

- Option 3: Once a RV is applied to a repetition, coded bits is mapped to remaining repetition contiguously

  - FFS: For repetitions mapped with same RV or for all of repetitions

Handling segments of insufficient length

As discussed in last meeting, segmentation, performed in option 4, could make a repetition with too short duration such as 1 or 2 symbols. Considering DMRS symbol occupies at least one symbol, one symbol segment need to be handled. Up to TBS determination method, some segments also need to be handled. For simplicity, we can drop those segment so that it won’t be transmitted. Otherwise, those repetition can be concatenated to adjacent repetition.

Figure 2 shows an example of simple resource dropping. In the figure, UE just drop PUSCH transmission where given repetition doesn’t meet some condition, such as minimum PUSCH duration. From this results, it is hard to use symbols around slot boundary. Needless to say, it is hard to allocate these resources to some other UE as well. In some cases, network would be able to avoid this kind of situations, however, it is not always possible considering multiple configured resource with different offset.

If we allow for UE to concatenate those resource to other resource rather than dropping, like figure 3, UE can utilize segmented symbol around slot boundary in most of cases. Moreover, it could reduce scheduling error case so that network has a lot of scheduling flexibility. Of course, it may bring higher resource utilization and reliability.

Figure 2. An example of dropping resource

Figure 3. An example of resource concatenation

Proposal 8: for a repetition of insufficient length, following options can be considered:

- Option 1: UE may drop a repetition of insufficient length.

- Option 2: UE assumes that a resource of insufficient length is concatenated to one of adjacent repetitions.

- FFS: how to determine insufficiency of a repetition

Extended SLIV design

To use Option 4, it is necessary to indicate resource allocation (RA) where start symbol + length is larger than 14. Moreover, it also can be considered to allow RA duration larger than 14. However, current SLIV design is optimized to current RA restrictions. So, we may need to design new/enhance SLIV indication.

| Length | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Start 0 | 0 | 14 | 28 | 42 | 56 | 70 | 84 | 98 | 97 | 83 | 69 | 55 | 41 | 27 |
| 1 | 1 | 15 | 29 | 43 | 57 | 71 | 85 | 99 | 96 | 82 | 68 | 54 | 40 | x |
| 2 | 2 | 16 | 30 | 44 | 58 | 72 | 86 | 100 | 95 | 81 | 67 | 53 | x | x |
| 3 | 3 | 17 | 31 | 45 | 59 | 73 | 87 | 101 | 94 | 80 | 66 | x | x | x |
| 4 | 4 | 18 | 32 | 46 | 60 | 74 | 88 | 102 | 93 | 79 | x | x | x | x |
| 5 | 5 | 19 | 33 | 47 | 61 | 75 | 89 | 103 | 92 | x | x | x | x | x |
| 6 | 6 | 20 | 34 | 48 | 62 | 76 | 90 | 104 | x | x | x | x | x | x |
| 7 | 7 | 21 | 35 | 49 | 63 | 77 | 91 | x | x | x | x | x | x | x |
| 8 | 8 | 22 | 36 | 50 | 64 | 78 | x | x | x | x | x | x | x | x |
| 9 | 9 | 23 | 37 | 51 | 65 | x | x | x | x | x | x | x | x | x |
| 10 | 10 | 24 | 38 | 52 | x | x | x | x | x | x | x | x | x | x |
| 11 | 11 | 25 | 39 | x | x | x | x | x | x | x | x | x | x | x |
| 12 | 12 | 26 | x | x | x | x | x | x | x | x | x | x | x | x |
| 13 | 13 | x | x | x | x | x | x | x | x | x | x | x | x | x |

Table 1. Current SLIV mapping to bit representation

Table 1 shows current (rel-15) SLIV design using 7 bits. As seen in the table, some bit representations are not used. Therefore, as simple method, we can consider additional table that can be configured by RRC signaling.

| SLIV | S | L |
| --- | --- | --- |
| 105 | 1 | 14 |
| 106 | 2 | 14 |
| … | … | … |
| 127 | 0 | 28 |

Table 2. An example of configured SLIV table

By adopting configurable table, like Table 2, UE can interpret unusable SLIV as additional combination of start and length. This approach extends the range of resource allocation without changing on current SLIV signaling design.

Alternatively, we can using additional 1bit in order to indicate 14*14 start and length combinations. For example, using 8bit SLIV (Option 2-1) or change SLIV interpretation by 1bit flag (Option 2-2). To be specific, it is possible to change SLIV equation like below:

SLIV = 14 * (L -1) + S

Or, if we use 1 bit flag, we can re-interpret S’ and L’ from current SLIV interpretation as below:

S = 13 – S’

L = 15 – L’

Proposal 9: To extend the range of PUSCH resource allocation, following option can be considered:

- Option 1: Adopt SLIV to RA table which overrides current SLIV interpretations.

- Option 2: Use 1 more bit to indicate extended resource Allocation

  - Option 2-1: 8bit SLIV

  - Option 2-2: SLIV re-interpretation triggered by 1bit flag

Difference between scheduled PUSCH and configured grant

In the last meeting, it has been decided to have unified solution for PUSCH as much as we can. For this, it is necessary to specify differences between scheduled PUSCH and configured grant in terms of resource allocation. All of difference between scheduled PUSCH and configured grant come from different signaling. It brings at least different slot format interaction. In current specification, only configured grant can be cancelled by dynamic SFI. Since configured grant is not scheduled dynamically, it is good to have robustness against slot format changing for configured grant.

Meanwhile, Configured grant is used continuously for certain services. It means that, once configured, sufficient TBS should be guaranteed without regarding situation. If TBS is determined by actual transmission duration, it would be hard to support service which requires certain packet size. In this point of view, TBS determination should be consistent in perspective of URLLC.

Those both are caused from the fact that configured grant is semi-static resource. So, in broad-sense, if we leave something as role of network scheduler, it could make a problem at least in configured grant case.

Proposal 10: For TBS determination, TBS should be determined without regarding of actual transmission duration at least for configured grant.

Power control on PUSCH transmission

When a UE is scheduled with multiple resources on multiple carriers, the UE will be also indicated transmit power on each carrier. In case of power limited case, it is currently assumed that a UE scales the power based on priorities and UE implementation. In terms of priority, not only UCI contents but also QoS required for each channel/UCI content needs to be considered. Furthermore, to guarantee the power allocated to URLLC packet, it should be possible to drop other overlapping transmissions. By dropping other transmissions, the computed PCmax can be changed (PCmax to transmit one UL can be different from PCmax to transmit two ULs with A-MPR, etc), and it is necessary to reapply recomputed PCmax after dropping.

Proposal 11: Power limited case should allow highest priority to URLLC traffic potentially including dropping other overlapping UL transmissions.

- Conclusion

We discussed PUSCH enhancement for NR URLLC, and proposed the followings.

Proposal 1: “nominal” repetition has no interaction with slot format and slot boundary.

Proposal 2: After segmentation, one or multiple “actual” repetition derived from a “nominal” repetition should be confined within the “nominal” repetition.

Proposal 3: If a nominal repetition spans UL-DL switching point, following can be considered:

- Option 1: UE assume that X symbols after the preceding DL symbol are invalid for UL

- Option 2: UE assume that X symbols before beginning of given PUSCH are invalid for DL

- FFS: how to determine X

Proposal 4: for PUSCH with DMRS mapping type A, following can be considered:

- Alternative 1: a UE doesn’t expect to receive DCI indicating DMRS mapping type A when repetition is used.

- Alternative 2: a UE applies DMRS mapping type B for following repetitions when PUSCH is scheduled with DMRS mapping type A

- Alternative 3: a UE assumes same relative DMRS symbol position of first PUSCH for following repetitions when repetition is used

Proposal 5: DMRS determination should be based on each actual repetition after segmentation.

.Proposal 6: For TBS determination, it is necessary to consider shortened transmission duration.

- Option 1: TBS is determined by the resource of the one repetition

  - FFS: consider nominal repetition or actual repetition for option 4

  - FFS: Which repetition is considered (initial, largest, smallest)

- Option 2: TBS is determined by entire allocated resource for a TB

  - one repetition should be sufficient to convey X coded bits

  - FFS: size of X (e.g., TBS or TBS*(certain target code rate)-1 )

Proposal 7: For PUSCH enhancement, how to mapping RV or coded bits among repetition should be specified.

- Option 1: The transmission with the n-th resource among all repetitions for a TB is associated with the (n+k)-th value among RV sequences, where the k is the index of the largest resource among all repetitions for a TB.

- Option 2: The transmission with the n-th largest resource among all repetitions for a TB is associated with the n-th value among RV sequences.

- Option 3: Once a RV is applied to a repetition, coded bits is mapped to remaining repetition contiguously

  - FFS: For repetitions mapped with same RV or for all of repetitions

Proposal 8: for a repetition of insufficient length, following options can be considered:

- Option 1: UE may drop a repetition of insufficient length.

- Option 2: UE assumes that a resource of insufficient length is concatenated to one of adjacent repetitions.

- FFS: how to determine insufficiency of a repetition

Proposal 9: To extend the range of PUSCH resource allocation, following option can be considered:

- Option 1: Adopt SLIV to RA table which overrides current SLIV interpretations.

- Option 2: Use 1 more bit to indicate extended resource Allocation

  - Option 2-1: 8bit SLIV

  - Option 2-2: SLIV re-interpretation triggered by 1bit flag

Proposal 10: For TBS determination, TBS should be determined without regarding of actual transmission duration at least for configured grant.

Proposal 11: Power limited case should allow highest priority to URLLC traffic potentially including dropping other overlapping UL transmissions.

- Reference

- [1]	RAN1 Chairman’s Note, RAN1#97 meeting.