3GPP TSG-RAN WG1 Meeting #98	R1-1908558

# Prague, CZ, August 26th – 30th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | DRAFT | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on PUSCH transmission with configured grant |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-07-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When higher layer parameter txConfig is not configured, transmission scheme of PUSCH configured by ConfiguredGrantConfig is unclear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify that if the higher layer parameter txConfig is not configured, the transmission of PUSCH configured by ConfiguredGrantConfig is based on a single antenna port, and the UE shall transmit PUSCH according to the spatial relation, if applicable, corresponding to the dedicated PUCCH resource with the lowest ID within the active UL BWP of the same cell, as described in sub-clause 9.2.1 of TS 38.213. For a UL BWP in frequency range 2 without configured PUCCH resource with PUCCH-SpatialRelationInfo in RRC connected mode, if it is not configured with txConfig, it is not expected to be configured with ConfiguredGrantConfig. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the higher layer parameter txConfig is not configured, UE’s behavior on PUSCH transmission of PUSCH configured by ConfiguredGrantConfig is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR has isolated impact on PUSCH transmission configured by ConfiguredGrantConfig and modifies the corresponding behavior as currently specified. Interoperability analysis: - If the network is implemented in accordance to the CR and the UE is not, or if the UE is implemented in accordance to the CR and the network is not, the network expects a different spatial relation and/or precoder and/or antenna ports of PUSCH than the UE actually uses. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.1.1	Transmission schemes

Two transmission schemes are supported for PUSCH: codebook based transmission and non-codebook based transmission. The UE is configured with codebook based transmission when the higher layer parameter txConfig in pusch-Config is set to 'codebook', the UE is configured non-codebook based transmission when the higher layer parameter txConfig is set to 'nonCodebook'. If the higher layer parameter txConfig is not configured, the UE is not expected to be scheduled by DCI format 0_1. If PUSCH is scheduled by DCI format 0_0, the PUSCH transmission is based on a single antenna port. The UE shall not expect PUSCH scheduled by DCI format 0_0 in a BWP without configured PUCCH resource with PUCCH-SpatialRelationInfo in frequency range 2 in RRC connected mode. If higher layer parameter txConfig is not configured, for PUSCH semi-statically configured to operate according to Subclause 6.1.2.3, PUSCH transmission is based on a single antenna port and according to the spatial relation, if applicable, corresponding to the dedicated PUCCH resource with the lowest ID within the active UL BWP of the same cell, as described in sub-clause 9.2.1 of [6, TS 38.213]. For a BWP without configured PUCCH resource with PUCCH-SpatialRelationInfo in frequency range 2 in RRC connected mode, if the higher layer parameter txConfig is not configured, the BWP is not expected to be configured with PUSCH transmission corresponding to a configured grant.