3GPP TSG RAN WG1 Meeting#98		R1-1908576													R1-190xxxx

Prague, CZ, August 26th – 30th, 2019

Source:	CATT

Title:	Network synchronization based on RAT-dependent signals

Agenda Item:	7.2.10.5

Document for:	Discussion and Decision

# Introduction

For supporting timing-based RAT-dependent positioning techniques, such as DL-TDOA and UL-TDOA techniques, it is critical to have precise time synchronization among the cells, which are involved in the position determination of a UE, especially for meeting 5G commercial requirements, where the positioning accuracy is required to be in meter/submeter-levels. In this contribution, we discuss the potential of using RAT-dependent reference signals for precise time synchronization among the cells (Note:This contribusion is a resubmittion of R1-1906309 [9]).

# Network synchronization techniques

There are many existing solutions for supporting network synchronization. One common approach to achieve the network synchronization in a telecommunication network is based on the global navigation satellite system (GNSS) signals, which can supportμs-level or even better network synchronization. The main limitation of the GNSS based approach is that it cannot be used at the places where the GNSS signals are blocked, such as the indoor environment. Another common approach is based on the synchronization protocol such as the Network Time Protocol (NTP) or Precision Time Protocol (PTP). With NTP, the clients request time synchronization from an NTP server that maintains a time standard. The clients perform time and phase synchronization in consideration of the time taken for a response to a request and the time between the server and the client. An NTP has usually the synchronization accuracy to the level of1 ms or better. PTP is able to support high-precision time and phase synchronization to 1 μs or even better with a hardware timestamp function and compensation for transmission path delay. Table 1 lists solutions that have been used to enable synchronization in the wireless network[1].

Table 1: Existing solutions [1]

| Sync Method | Freq Sync (Achievable on the radio interface) | Phase Sync (Achievable on the radio interface) | Comment |
| --- | --- | --- | --- |
| Synchronous PDH / SDH / Synchronous Ethernet (SyncE) | ±0.05 ppm or better | — | SyncE must be supported on all nodes in the sync chain. See applicable ITU-T recc. (G.803, G.813, G.8261, G.8262, etc.) |
| IEEE1588 PTP for freq only sync | ±0.05 ppm | — | It does not require on-path support between Master and Slave. Performance depends on the load of the network. See applicable ITU-T recc.(e.g. G.8261.1) |
| IEEE1588 PTP for freq/phase sync (partial timing support understudy) | ±0.05 ppm | 250ns – 1.5s | See applicable ITU-T recc.(e.g. G.8271.1) |
| Satellite time reference (e.g. GPS) | < ±0.01 ppm | < 100ns | Receiver antenna and cabling installation can be expensive in indoor installations. |
| NTP (Network Timing Protocol) | ±0.05 ppm | FFS | Depends on network characteristics (similar rules as for PTP apply in case of frequency sync; also, in this case, a specific clock recovery algorithm is required) |
| RIBS Radio Interface Based Synchronization (3GPP R12) | ±0.1 ppm | 2.5s | LTE-FDD: Requires capability to listen to sync source downlink frequency band |

For supporting 5G NR positioning accuracy of a few meters or higher, the network synchronization is required to be in the level of a few tens of nanoseconds or even a few nanoseconds considering that the radio signal travels about 3 meters in 10ns. The solution needs to work under all environments with low implementation complexity and cost. Thus, it is worthy to look for the solutions in addition to those shown in Table 1.

# LTE network synchronization based on RAT-dependent signals

3GPP has actually investigated the network synchronization based on RAT-dependent signals for LTE networks, in which various approaches were proposed[1]:

- Solution 1: Network-based solution using detection of UE transmission, where the eNBs determine their time offsets based on monitoring the signals from the same UEs;

- Solution 2: OTA Synchronization with Propagation Delay Compensation

- Solution 3: OTA Synchronization with Propagation Delay Compensation Based on Timing Advance

- Solution 4: Propagation Delay Compensation for RIBS Based on Location Information Exchange

- The eNBs determine their time offsets based on monitoring the signals from other eNBs (e.g., the slave eNB adjusts its synch to be aligned with the master eNB(s)), which can be in the distributed way, the centralized way, or the statistical way.

The investigation has reached the conclusion that it is feasible to obtain s-level network synchronization accuracy based on LTE signals[1].

# NR network synchronization based on RAT-dependent signals

For NR positioning, there were also proposals to support the gNBs to listen to each other’s PRS reference signals for supporting time synchronization and NR positioning during NR positioning SI (e.g., [2][3][4][5]). For network synchronization based on RAT-dependent signals, we may expect better time synchronization can be achieved than LTE due to more time and frequency radio resources are available for the purpose of the NR network synchronization.

In this section, we further discuss the use of the RAT-dependent signals for 5G NR network with the target of providing ns-level synchronization accuracy and perfect frequency synchronization.

Figure 1 illustrates the basis for the proposed network synchronization method, using both time and frequency synchronization between two cells as an example. Similar with the existing proposals of RAT-dependent time synchronization, one cell (Cell 1) transmits the reference signals for time synchronization, which can be the positioning reference signal (PRS) and the Cell 2 monitors the reference signals for time synchronization to achieve the time synchronization with Cell 1. Through the detection of the time of arrival of the PRS from Cell 1, and the known locations of the Cell 1 and Cell 2, the timing offset between the two cells can be estimated with certain accuracy. However, the time synchronization accuracy may be limited to μs-level due to various factors, especially the gNB clock drifts and the frequency offsets.

To improve the time synchronization accuracy, it is proposed that the Cell 1also transmits the carrier phase reference signal (C-PRS)) (See [5]for more details of the description of the C-PRS). Once Cell 2 has phase-locked the C-PRS from Cell 1, any time misalignments due to clock drifts and the frequency offsets can be counted precisely.

Therefore, by combining the periodical measurements of the TOA of the PRS and the continuous tracking the C-PRS signals, the time offset between the two cells can be estimated precisely with the estimation accuracy increasing with the monitoring time, since all changes of the clock offsets due to the clock drifts and frequency offset will be precisely estimated by the phase-lock loop, and the timing estimation error due to the TOA estimation error will be smoothed out with the increase of the synchronization time.

Figure 1.Time and frequency synchronization based on PRS and C-PRS signals

Compared with the time synchronization method between the base stations discussed so far by the 3GPP, adding the C-PRS reference signal for signal phase locking and frequency synchronization allows achieving complete frequency synchronization and eliminating the influence of the frequency offset of each base station on the time synchronization error between the base stations. It improves the accuracy of time synchronization between base stations by using periodically transmitted PRS signals without the worry of the clock drift caused due to the instability of the base station clocks.

The method shown in Figure 1 can be extended to different implementations. For example, we may use one cell to monitor the PRS and C-PRS from multiple cells to achieving the network synchronization among these cells as shown in Figure 2.

Figure 2.Time and frequency synchronization based on PRS and C-PRS signals with multiple cells

# Simulation Results

The proposed RAT-dependent network synchronization technique is evaluated with the indoor office scenario as defined in TR 38.855[6]. The layout of the BSs of the indoor office scenarios is shown in Figure 3.

Figure 3: Layout of indoor office scenarios.

Table 1 presents the parameters related to the evaluation. Most of the parameters are adopted from the indoor office scenario as defined in TR 38.855[6].

Table 1 Parameters for evaluations

| Parameter | Descriptions |
| --- | --- |
| Channel model (baseline, otherwise state any modifications) | According to 3GPP TR 38.901 (Indoor Open Office) |
| Carrier frequency | 4GHz |
| Subcarrier spacing | 15KHz |
| Reference Signal Transmission Bandwidth | 5MHz |
| Reference signal (type of sequence, number of ports, …) | Gold sequence 1 port |
| Number of sites | 12 |
| Number of symbols used per occasion | 13 |
| PRS occasion periodicity | 1s |
| Power-boosting level | 6dB |
| interference modelling (ideal muting, or other) | Ideal muting |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and Rx sides) | Wide Tx beam without beam sweeping is assumed. No Rx beam sweeping. |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx: 64 antenna elements Rx: 4 antenna elements codebook: 64*1 precoder used to generate a wide beam |
| Initial network synchronization error | 1 us |

Figure 4: Network synchronization performance

Figure 4 illustrates the network synchronization performance based on the proposed approach. As shown in the figure, with the proposed approach, the network synchronization accuracy improves with the time and is able to achieve very precise time synchronization.

# Conclusion

In this paper, we discussed the use of RAT-dependent reference signals for precise time synchronization among the cells for NR positioning. The proposal is evaluated with the indoor scenario defined in TR 38.855. The evaluation results show that it is feasible to use the proposed approach for precise network time/frequency synchronization. Based on the discussion, we propose:

Proposal 1: 3GPP should make further investigation on the RAT-dependent network synchronization techniques for NR positioning, where the precise network synchronization is achieved by the cells monitor the time and frequency synchronization signals transmitted from other cells.

# References

- 3GPP TR 36.898, “Network Assistance for Network Synchronization”

- R1-1902344, “Discussion on measurement reporting for network synchronization error compensation scheme”, CMCC

- R1-1901849, “DL and UL based NR Positioning”, Nokia, Nokia Shanghai Bell

- R1-1903141, “On the use of RTT for positioning”, Ericsson

- R1-1901980, “Further discussion of NR RAT-dependent DL Positioning”, CATT

- 3GPP TR 38.855, “Study on NR positioning support”

- Chairman’s Note 3GPP TSG RAN WG1#96bis

- 3GPP TS 38.133, “NR: Requirements for support of radio resource management”.

- R1-1906309, “Network synchronization based on RAT-dependent signals”, CATT