3GPP TSG RAN WG1 Meeting #98		                                                      R1-1908580

Prague, CZ, August 26th – 30th, 2019

Source:	CATT

Title:	Discussion on Mode 1 resource allocation in NR V2X

Agenda Item:	7.2.4.2.1

Document for:	Discussion and Decision

Introduction

In RAN1#97 [1], the following agreements have been achieved for NR Uu to control NR sidelink:

Agreements: - Sidelink HARQ ACK/NACK report from transmitter UE to gNB is supported with details FFS. Note: this reverts the following agreement from RAN1#96: - Sidelink HARQ ACK/NACK report from UE to gNB is not supported in Rel-16. - SR/BSR report to gNB for the purpose of requesting resources for HARQ retransmission is not supported. Send an LS to RAN2 with the agreement – R1-1907889, which is approved with final LS in R1-1907905 Agreements: - NR sidelink does not support performing different transmissions of a TB using different configured grants. Agreements: - For mode 1: - A dynamic grant by the gNB provides resources for transmission of PSCCH and PSSCH.

In RAN1#96bis [2], the following agreements have been achieved for NR Uu to control NR sidelink:

Agreements: - A dynamic grant provides resources for one or multiple sidelink transmissions of a single TB. - A configured grant (type-1, type-2) provides a set of resources in a periodic manner for multiple sidelink transmissions. - UE decides which TB to transmit in each of the occasions indicated by a given configured grant. - FFS: whether different transmissions of a TB can take place across multiple configured grants. - Other restrictions on what can be transmitted in a given configured grant (e.g., based on QoS, destination UE, etc.) are up to RAN2.

In RAN#86 meeting [3], the WID was approved and has the following objectives on NR Uu to control NR sidelink:

1. NR sidelink: Specify NR sidelink solutions necessary to support sidelink unicast, sidelink groupcast, and sidelink broadcast for V2X services, considering in-network coverage, out-of-network coverage, and partial network coverage. - Resource allocation [RAN1, RAN2] - Mode 1 - NR sidelink scheduling by NR Uu and LTE Uu as per the study outcome

In RAN1#96 meeting [4], the following agreements have been achieved on sidelink HARQ report to BS:

Agreements: - In mode 1 for unicast and groupcast, it is supported for the transmitter UE via Uu link to report an indication to gNB to indicate the need for retransmission of a TB transmitted by the transmitter UE. - FFS the format of the indication, e.g., in the form of HARQ ACK/NACK, or in the form of SR/BSR, etc. - RAN1 continues discussion on whether to support report from the receiver UE - No inter-BS communication will be considered. To discuss aspects related to 1st sub-bullet & 2nd bullet during this week -revisit later Agreements: - Sidelink HARQ ACK/NACK report from UE to gNB is not supported in Rel-16.

This contribution discusses NR sidelink mode 1 resource allocation mechanism controlled by NR Uu and LTE Uu.

Discussion

NR Uu to control NR sidelink mode 1

DCI design

According to the discussion and agreements for NR sidelink mode 1 resource allocation, configured grant type-1 and type-2 are supported. For aperiodic transmission, dynamic resource allocation can be applied. For periodic services, configured grant type-1 and type-2 can be used for resource allocation.

In order to control sidelink mode 1 transmission, a new DCI format should be designed. The DCI format 5A used for LTE V2X can be used as the reference of NR DCI format design. Furthermore, DCI should also include sidelink information that is carried in SCI. The new DCI format (e.g. DCI format 0_3) should align its size with that of the existing DCI format, such as DCI format 0_0 or 0_1, to fit to the DCI format budget for each UE.

Proposal 1: New DCI format should designed by considering new features in NR V2X.

Proposal 2: The size of the new DCI format for scheduling NR sidelink mode 1 should align with one of the existing NR DCI format.

Timing design

In LTE V2X, the timing between Uu and its scheduled sidelink subframe is fixed. In NR Uu, flexible timing is used due to different subcarrier spacing/slot lengths. By considering about UE capability and processing time at UE side, flexible scheduling timing between NR Uu and scheduled sidelink transmission can be used. Furthermore, the latency requirement should also be considered when determining the timing. The interval should also be indicated in DCI.

Proposal 3: For dynamic scheduling, flexible interval between DCI transmission and its scheduled first sidelink transmission can be supported.

Proposal 4: The interval should be indicated in DCI.

Support of sidelink resource allocation on shared carrier

When the frequency resources are shared by Uu-based and PC5-based sidelink transmission, TDD mode can be used to mitigate the transmission collision. For in-coverage scenario, UE would only need to decode the DL common channels that include SSBs, CORESETS (PDCCH) and group-common PDCCH. Other resources used by UE would be scheduled by gNB through DCI or by higher layer through semi-static allocation. Therefore, the V2X UEs should use any resources (e.g. UL, DL, X or any symbol of them) allocated by the gNB without concerning if it is DL/UL/GP in the slot format configuration in the shared carrier. UEs should not even consider any potential interference management scheme to other cells, since UEs would be controlled by the network when any sidelink resource is allocated either through mode-1 or mode-2 resource allocation.

Observation 1: For TDD and in-coverage scenario, V2X UEs should use any resources scheduled by gNB or semi-statically allocated by higher layer without concerning if it is DL/UL/GP in the shared carrier.

Proposal 5: At least, UL symbols can be used for sidelink transmission on shared carrier.

Support of high data rate services

Another issue of mode-1 resource allocation is to support the high data rate e.g. 25Mbps. Assuming packets are transmitted every 10ms, the average packet is 31250byte. While in 3GPP V2X Phase 2, the maximum transmission capability can be up to 9186 byte within a TTI, for a SL-C-TX Category 5 UE working in 30 MHz band combination and adopting 64 QAM. If supporting the packet size of 31250 byte within a slot, the much wider bandwidth should be supported, which is around 100MHz. While in FR 1, it can hardly be achieved. Therefore, slot aggregation should be studied in NR V2X, in order to scheduling more time domain resource for a big size packet. For example, resources of multiple continuous slots can be allocatedby dynamic scheduling.

Proposal 6: For resource allocation mode-1, the scheduling solution for slot aggregation should be further studied, in order to support high data rate services.

HARQ feedback in Resource allocation Mode 1

It was agreed in RAN1#97 that sidelink HARQ ACK/NACK report from transmitter UE to gNB is supported. In mode 1 when source UE reports HARQ feedback to the gNB, the UL resource, such as PUCCH or PUSCH, for the sidelink HARQ feedback should be allocated by the gNB. Flexible PSSCH-to-HARQ feedback timing should  be used to provide higher resource efficiency. Currently, the transmission time of HARQ feedback for PDSCH or PUSCH transmission is included in the DCI.

Proposal 7: For mode 1, the UL resource for sidelink HARQ feedback to the gNB should be explicitly indicated by the gNB.

When source UE transmits HARQ information to gNB, the UL resources for sidelink HARQ-ACK information can be allocated and indicated by DCI.

Proposal 8:  The UL resources of sidelink HARQ-ACK to the gNB can be indicated by DCI.

LTE Uu controlling NR sidelink mode 1

When NR sidelink is controlled by LTE network (e.g. eNB), configured grant type-1 is used for time/frequency domain resource allocation to NR sidelink mode 1 UEs. Without support configured grant type-2 and DCI based scheduling mechanism, the resource allocation can be down through RRC signaling. For periodic traffic, a set of time/frequency domain resources can be allocated to UE. For aperiodic traffic, UE can also use the allocated resources for periodic traffic, but it can lead to resource waste. Even for periodic traffic, configured grant type-1 resource allocation mechanism for NR sidelink mode 1 loses flexibility and efficiency on utilizing the resources.

Observation 3: configured grant type-1 can lead to low efficiency on resource utilization.

Proposal 9: RRC signaling from eNB can be used to control and schedule resources for NR sidelink mode 1 UE.

Conclusion

In this contribution, the NR Uu to control NR sidelink mode 1 transmission is discussed. We have the following observations and proposals:

Proposal 1: New DCI format should designed by considering new features in NR V2X.

Proposal 2: The size of the new DCI format for scheduling NR sidelink mode 1 should align with one of the existing NR DCI format.

Proposal 3: For dynamic scheduling, flexible interval between DCI transmission and its scheduled first sidelink transmission can be supported.

Proposal 4: The interval should be indicated in DCI.

Observation 1: For TDD and in-coverage scenario, V2X UEs should use any resources scheduled by gNB or semi-statically allocated by higher layer without concerning if it is DL/UL/GP in the shared carrier.

Proposal 5: At least, UL symbols can be used for sidelink transmission on shared carrier.

Proposal 6: For resource allocation mode-1, the scheduling solution for slot aggregation should be further studied, in order to support high data rate services.

Proposal 7: For mode 1, the UL resource for sidelink HARQ feedback to the gNB should be explicitly indicated by the gNB.

Proposal 8:  The UL resources of sidelink HARQ-ACK to the gNB can be indicated by DCI.

Observation 3: configured grant type-1 can lead to low efficiency on resource utilization.

Proposal 9: RRC signaling from eNB can be used to control and schedule resources for NR sidelink mode 1 UE.

References

  - 3GPP TSG RAN WG1 Meeting RAN1 #97 chairman notes, May 2019.

  - 3GPP TSG RAN WG1 Meeting RAN1 #96bis chairman notes, April 2019.

  - RP-190766, “New WID on 5G V2X with NR sidelink”, RAN #83 meeting.

  - 3GPP TSG RAN WG1 Meeting RAN1 #96 chairman notes, Feburary-March 2019.