3GPP TSG RAN WG1 Meeting #98                                                                       R1-1908588

Prague, CZ, August 26th – 30th, 2019

Source:	CATT

Title:	SCI design for sidelink

Agenda Item:	7.2.4.8

Document for:	Discussion and Decision

Introduction

In RAN1 Ad-Hoc 1901, RAN1#96bis and RAN1#97 [1][2][3], the following agreements on SCI were achieved:

Agreements: - Layer-1 destination ID can be explicitly included in SCI - FFS how to determine Layer-1 destination ID - FFS size of Layer-1 destination ID - The following additional information can be included in SCI - Layer-1 source ID - FFS how to determine Layer-1 source ID - FFS size of Layer-1 source ID - HARQ process ID - NDI - RV - FFS whether some of the above information may not be present etc. in some operations (e.g., depending on whether they are used for unicast, groupcast, broadcast) Agreements: - QPSK is used for PSCCH. Conclusion: - If two-stage SCI is supported, the following details are used. - Information related to channel sensing is carried on 1st-stage. - 2nd-stage is decoded by using PSSCH DMRS. - Polar coding used for PDCCH is applied to 2nd-stage - Payload size for 1st-stage in two-stage SCI case is the same for unicast, groupcast, and broadcast in a resource pool. - After decoding the 1st-stage, the receiver does not need to perform blind decoding of 2nd-stage. - FFS other details - Companies are encouraged to perform analysis (e.g., flexibility, complexity, forward compatibility, overhead, spec impact, latency, robustness, reliability, etc.)/evaluations with details of the SCI contents comparing single-stage vs. two-stage SCI. Aim to conclude in RAN1#98

In this contribution, we mainly discuss the issues on SCI contents design, and some evaluation results are provided.

Discussion

SCI contents

In contrast to LTE V2X, NR V2X supports different types of transmissions, such as unicast, groupcast and broadcast. The potential SCI contents for NR V2X can be categorized in Table 1.

Table 1: SCI contents for NR V2X

| SCI bit fields category | SCI bit fields | bit length | Comments |
| --- | --- | --- | --- |
| Cast type indicator | L1-detination ID | 8 | Use for all the cast type |
| HARQ combination related fields | L1-source ID | 8 | Use for all the cast type. According to [4], it is prefer to provide this field for all the cast type for flexibility. |
|  | HARQ process ID | 4 |  |
|  | RV | 2 |  |
|  | NDI | 1 |  |
| Sensing related bit fields | Resource reservation fields - Frequency resource location of initial transmission and retransmission - Time gap between initial transmission and retransmission - Resource reservation period indicator | X | Use for all the cast type. The details depend on the resource reservation mechanism design. |
|  | PSSCH DMRS pattern - Time domain pattern (2 bits) - Frequency domain pattern: type 1or type 2 (1 bit) - Antenna port/RI indicator (1 bit) | 4 | Used for all the cast type. |
|  | Priority indicator | 3 | Used for all the cast type |
| Modulation and coding scheme | MCS | 5 | Used for all the cast type |
| CSI measurement and feedback | CSI trigger and presence of CSI-RS | 1 | Used only for unicast |
|  | Presence of CSI report | 1 |  |
| Zone ID | Zone ID | [8] | Use for distance-based groupcast HARQ feedback Unicast can reuse this field for HARQ feedback trigger to avoid additional overhead. |

Further conlarification about the above bit fields as following:

- The length of L1-source/destination ID field needs to be unique for each V2X UE locally and not too long.   The length L1-source/destination ID should not be more than 8 bits and details should be further studied after the length of UE ID completed in RAN2 and SA2.

- For the SCI fields already appeared in LTE sidelink, we can simply update the size according to the actual situation of NR sidelink.

- Regarding the RS pattern indication field, the DMRS pattern should be decided first.

- As the CSI acquisition for unicast has been supported, SCI should also indicate whether a sidelink UE should request CSI reporting via PSSCH and whether CSI-RS is confined within PSSCH.

- The Antenna port(s) field is used to indicate the rank of PSSCH transmission and the DMRS port(s).

- Zone ID for measuring the distance between two UEs. The payload size needs to be further studied.

- Observation 1: There are only about 10bits difference between the SCI contents of broadcast and unicast/groupcast transmission

Proposal 1: The SCI contents in Table 1 should be taken into account.

Single SCI vs. 2-stage SCI

From the design principle of 2-stage SCI, the 1st SCI and 2nd SCI functionality are provide as following:

- 1st SCI shall include all the necessary information, at least include sensing related information bits, and introducing an extra bit field for resource indication of 2nd SCI,

- 2nd SCI shall include all the necessary information for data reception.

Furthermore, with the consideration of the cast type supported in NR sidelink, the minimum SCI bit fields set shall be transmitted in NR sidelink is the SCI used for broadcast.  Therefore, the design principle for whether introducing 2-stage SCI is as following:

- Principle 1: The SCI used for broadcast shall be at least included in 1st SCI if 2-stage SCI agreed

- Principle 2: 2nd SCI shall only include the bit fields which are beyond the SCI used for broadcast, and payload size without CRC bits shall be larger than 24 bits.

Base on the above discussion, the details of the SCI contents comparing between single SCI and 2-stage SCI are shown in Table 2.

Table 2: Comparison between single SCI and 2-stage SCI

| Bit field | Single SCI | 2-stage SCI |  |
| --- | --- | --- | --- |
|  |  | 1st SCI | 2nd SCI |
| L1-source ID | 8bits | 8bits |  |
| L1-destination ID | 8bits | 8bits |  |
| MCS | 5bits | 5bits |  |
| Priority | 3bits | 3bits |  |
| Resource reservation | 4bits | 4bits |  |
| Frequency resource location of initial transmission and retransmission | log2(NsubchannelSL(NsubchannelSL+1)/2) | log2(NsubchannelSL(NsubchannelSL+1)/2) |  |
| Time gap between initial transmission and retransmission | 4bits | 4bits |  |
| Retransmission index | 1bit | 1bit |  |
| 2nd-stage SCI aggregation level indicator |  | 4bits |  |
| HARQ process number | 4bits | 4bits |  |
| NDI | 1bit | 1bits |  |
| RV | 2bits | 2bits |  |
| PSSCH RS pattern indication | 3bits | 3bits |  |
| CSI trigger(only for unicast) | 1bits |  | 1bits |
| CSI report (only for unicast) | 1bits |  | 1bits |
| Antenna port(s)/RI indicator | 1bits | 1bits |  |
| Zone ID for Distance | 8bits |  | 8bits |
| CRC | 24bis | 24bits | 24bits |
| Total size | 78+ log2(NsubchannelSL(NsubchannelSL+1)/2) | 72+ log2(NsubchannelSL(NsubchannelSL+1)/2) | 34bits |

It can be observed that only 10 bits is moved to 2nd SCI if 2-stage SCI is supported, it is far less than the 24 CRC bits. It doesn’t seem to make sense to carry so few bits of information with an additional 24bits overhead.

Observation 2: Only 10 bits is moved to 2nd SCI if 2-stage SCI is supported, it is far less than the 24 CRC bits.

Observation 3: 2-stage SCI introduces about 28 extra bits than single-stage SCI.

Furthermore, the 2nd-stage SCI is decoded by using PSSCH DMRS, the performance of PSCCH will be affected when the PSSCH uses 2-ports DMRS pattern. In the case where PSCCH is rank-1 and the rank of PSSCH is 2, the mapping details of 2nd-stage SCI also need to be further studied.

Observation 4:  When the number of DMRS ports of PSCCH and PSSCH is different, the performance of 2-stage SCI may be affected.

The evaluation results about single SCI and 2-stage SCI are provided in Figure 1,  and the total size of single-stage SCI is about 78bits, and total size of 2-stage SCI is about 106bits. In simulation, the same size of resource is assigned to single-stage SCI and 2-stage SCI.

Figure 1: BLER performance for single-stage-SCI vs 2-stage-SCI

Observation 5: The impact of higher coding rate and error propagation deteriorates the BLER performance of the 2-stage SCI than single-stage SCI.

Proposal 2: Only single SCI shall be supported in NR sidelink, and single SCI format for unicast, groupcast, and broadcast.

Conclusion

In this contribution, we provide some proposals on the NR V2X SCI contents design as follow:

Observation 1: There are only about 10bits difference between the SCI contents of broadcast and unicast/groupcast transmission.

Observation 2: Only 10 bits is moved to 2nd SCI if 2-stage SCI is supported, it is far less than the 24 CRC bits.

Observation 3: 2-stage SCI introduces about 28 extra bits than single-stage SCI.

Observation 4: When the number of DMRS ports of PSCCH and PSSCH is different, the performance of 2-stage SCI may be affected.

Observation 5: The impact of higher coding rate and error propagation deteriorates the BLER performance of the 2-stage SCI than single-stage SCI.

Proposal 1: The SCI contents in Table 1 should be taken into account.

Proposal 2: Only single SCI shall be supported in NR sidelink, and single SCI format for unicast, groupcast, and broadcast.

References

  - 3GPP TSG RAN WG1 Meeting RAN1 Ad-Hoc Meeting 1901 chairman notes, January, 2019

  - 3GPP TSG RAN WG1 Meeting RAN1 #96bis  chairman notes, April, 2019

  - 3GPP TSG RAN WG1 Meeting RAN1 #97 chairman notes, May, 2019

  - R1-1908584, “Sidelink physical layer procedures in NR V2X”, CATT

- Appendix

In this section, we provide link level evaluation assumptions used for analysis of SCI contents.

Table 4. Link level evaluation assumption

| Parameters | Assumptions |
| --- | --- |
| Carrier frequency | 5.9 GHz |
| Waveform | CP-OFDM, |
| Subcarrier spacing | 30 kHz |
| DS | 100ns |
| DMRS pattern | PDCCH DMRS pattern |
| Channel estimation | No-Ideal |
| Channel model | Sidelink: Highway-LOS, Urban-NLOS |
| Channel coding | Polar |
| Antenna configuration | (Tx, Rx) = (2, 4) 1ports |
| UE speed (Relative ) | 280km/h |
| Modulation | QPSK |
| TBS | 83bits,111bits |
| PRB | 16RBs |