3GPP TSG RAN WG1 Meeting #98		                                                      R1-1908597

Prague, CZ, August 26th – 30th, 2019

Source:	CATT

Title:	Discussion on out-of-order scheduling/HARQ

Agenda Item:	7.2.6.4

Document for:	Discussion and Decision

Introduction

Rel-16 URLLC work item was approved in RAN#83 in [1]. One of the objectives is to specify enhancements to scheduling/HARQ including the support of out-of-order HARQ and out-of-order PUSCH for different HARQ processes and the support of scenario 1 (dynamic PDSCH and dynamic PDSCH conflict in time) and scenario 3 (dynamic PUSCH and dynamic PUSCH conflict in time) as defined in TR38.825[2].

- Specification of enhancements to scheduling/HARQ [RAN1]

  - Out-of-order HARQ-ACK associated with PDSCHs with different HARQ process IDs

  - Out-of-order PUSCH scheduling associated with different HARQ process IDs, including overlapping PUSCHs and non-overlapping PUSCHs in time-domain

  - Methods to handle DL data/data resource conflicts for overlapping PDSCHs in time-domain, scheduled by dynamic DL assignments

In RAN1#96bis meeting, the following agreements were made.

Agreements:

In case two unicast PDSCHs for a UE are overlapping, the following scenarios are identified:

- Scenario 1-1: Overlapping in the time domain and not in the frequency domain

- Scenario 1-2: Overlapping both in the time and frequency domains

Working assumption:

- When the two unicast PDSCHs for a UE are overlapping, the UE generates HARQ-ACK for both of the PDSCHs.

In RAN1#97, the following conclusion was reached regarding out-of-order HARQ.

Conclusion:

Study further whether/how to support the following scenarios for the handling of two unicast PDSCHs:

- When different DL processing times are associated with different PDSCHs on the same serving cell, and the two PDSCHs are non-overlapping.

  - Note: The PDSCH-to-PUCCHs can be out-of-order or in-order.

  - Note: The solution(s) should address the UE processing pipelining issue.

  - Two PDSCHs follow DL processing timing capability #1 and #2, respectively, on the same serving cell.

    - FFS if any different solutions are necessary to address different scenarios when the above condition occurs

- When the same DL processing time is configured on the same serving cell, and the two PDSCHs are non-overlapping, and the PDSCH-to-PUCCHs are out of order.

  - Note: There is no UE processing pipelining issue.

  - Note: the in-order PDSCH-to-PUCCHs are already handled in Rel-15.

- The two unicast PDSCHs are overlapping at least in the time domain, regardless of whether the same or different DL processing times is configured on the same serving cell.

  - Note: The solution(s) should address the UE processing pipelining issue in this case.

This contribution discusses out-of-order scheduling and out-of-order HARQ taking the above agreements into account.

Discussion

Out-of-order HARQ for non-overlapping PDSCHs in time

It was discussed in RAN1#97 that if Rel-15 NR configuration and scheduling limitations are maintained, out-of-order HARQ can be supported by just buffering the HARQ-ACK for the earlier PDSCH without impact on UE pipeline processing [3]. Otherwise, if Rel-15 NR configuration and scheduling limitations are relaxed, 3 cases were identified that UE pipeline processing may potentially be impacted regardless of in-order or out-of-order HARQ due to back-to-back PDSCHs require different processing timelines.

- Case-1: When the UE is configured with additional DMRS and DL processing capability #2 on a given serving cell, a PDSCH with additional DMRS follows the DL processing timing capability #1 and a PDSCH without additional DMRS follows the DL processing timing capability #2.

- Case-2: Under this scenario, DL processing timing capability #1 and #2 can be configured and be followed on the same serving cell.

- Case-3: For a UE indicating capability pdsch-Processing2Limited, when configured with DL processing capability #2 on a serving cell, PDSCH scheduled with larger than 136PRBs follows DL processing capability #1 and PDSCH scheduled with less than 136PRBs follows DL processing capability #2.

Among the three cases, we think case-1 is a valid case for a UE with both eMBB and URLLC traffic types and with certain mobility requiring additional DMRS. This case was not supported in Rel-15, i.e. additional DMRS cannot be configured if DL processing capability #2 is configured. In order to better support both eMBB and URLLC, it is useful to support configuring DL processing capability #2 and additional DMRS simultaneously on a given cell. For case-2, it is not clear why different DL processing capabilities need to be configured for the same serving cell and it is not clear how to associate each PDSCH with DL processing capability #1 or #2. Case-3 is already supported in Rel-15 by skipping the earlier PDSCH decoding if there is no sufficient processing time.

Proposal 1: Support configuring additional DMRS and DL processing capability #2 on a given serving cell.

Case-1 is similar as case-3 in terms that a PDSCH fallback to DL processing capability#1 is followed by a PDSCH with DL processing capability #2, so that similar handling as in Rel-15 can be reused, i.e. the UE may skip decoding the earlier PDSCH if the gap between the end of the earlier PDSCH and the start of the latter PDSCH is shorter than N symbols. Furthermore, N can be N1 value for the earlier PDSCH based on DL processing capability #1. Following this handling, UE pipeline processing is still maintained.

Figure 1: PDSCHs associated with different processing capabilities

Proposal 2: On a given cell, if a first PDSCH associated with DL processing capability #1 is followed by a second PDSCH associated with DL processing capability #2, UE may skip decoding the first PDSCH if the gap between the end of the first PDSCH and the start of the second PDSCH is shorter than PDSCH processing time N1 symbols based on the first PDSCH.

In addition, UE generates HARQ-ACK for both PDSCHs and gNB can schedule retransmission for the first PDSCH if needed.

Proposal 3: On a given cell, if a first PDSCH associated with DL processing capability #1 is followed by a second PDSCH associated with DL processing capability #2, UE generates HARQ-ACK for both PDSCHs regardless whether UE decodes the PDSCH or not. UE shall generate NACK if UE skips decoding the PDSCH.

Similar as case-3 in Rel-15, no additional processing time is required for the second PDSCH if UE skips decoding the first PDSCH.

Proposal 4: On a given cell, if a first PDSCH associated with DL processing capability #1 is followed by a second PDSCH associated with DL processing capability #2, the minimum PDSCH processing time of the second PDSCH is not increased when the UE drops the processing of the first PDSCH.

Out-of-order HARQ for overlapping PDSCHs in time

For overlapping PDSCHs, we think the following options can be considered.

- Option 1: UE may skip decoding the low priority PDSCH

  - Option 1-1: UE buffers the low priority PDSCH data if the PDSCH is not successfully decoded, i.e. legacy UE behavior

  - Option 1-2: UE does not buffer the low priority PDSCH data if the PDSCH is not successfully decoded

- Option 2: UE always skips decoding the low priority PDSCH if UE is not capable of processing two PDSCHs overlap in time

The advantage of option 1 is that the UE behavior is the same for overlapping PDSCHs and non-overlapping PDSCHs. However, due to the uncertainty that UE may or may not decode the first PDSCH, gNB cannot stop transmitting the low priority PDSCH and reallocate the resources to other UEs in order not to corrupt the soft bits in the HARQ buffer. In order to give gNB the scheduling flexibility, option 1-2 can be considered. In this option, UE does not buffer the low priority PDSCH data if the PDSCH is not successfully decoded so that UE will not buffer the soft bits targeted for other UEs and gNB can transmit the low priority PDSCH or not. Alternatively, option 2 can be considered. In this option, UE always skips decoding the low priority PDSCH if UE is not capable of processing two PDSCHs overlap in time. Accordingly, gNB always stop transmitting the low priority PDSCH.

Proposal 5: For overlapping PDSCHs, select one option from 1) UE may skip decoding the low priority PDSCH and UE does not buffer the low priority PDSCH data if the PDSCH is not successfully decoded and 2) UE always skips decoding the low priority PDSCH if UE is not capable of processing two PDSCHs overlap in time.

Out-of-order PUSCH

For out-of-order non-overlapping PUSCH, for a first PUSCH scheduled by an earlier DCI and a second PUSCH scheduled by a latter DCI, UE transmits the second PUSCH and omits transmitting the first PUSCH if the gap between the end of the second PUSCH and the start of the first PUSCH is shorter than N symbols. Furthermore, N can be PUSCH preparation time N2 based on the first PUSCH as shown in Figure 2.

Figure 2: Out-of-order non-overlapping PUSCH in time

Proposal 6: For out-of-order non-overlapping PUSCH, for a first PUSCH scheduled by an earlier DCI and a second PUSCH scheduled by a latter DCI, UE transmits the second PUSCH and omits transmitting the first PUSCH if the gap between the end of the second PUSCH and the start of the first PUSCH is shorter than PUSCH preparation time N2 symbols based on the first PUSCH.

For overlapping PUSCHs, similarly, UE transmits the second PUSCH and stops transmitting the first PUSCH from the start of the second PUSCH as shown in Figure 3.

Figure 3: overlapping PUSCHs in time

Proposal 7: For overlapping PUSCHs, UE transmits the second PUSCH and stops transmitting the first PUSCH from the start of the second PUSCH.

Similar as proposal 4, it is proposed that the minimum PUSCH preparation time of the second PUSCH is not increased when the UE drops the processing of the first PUSCH.

Proposal 8: For out-of-order PUSCH, the minimum PUSCH preparation time of the second PUSCH is not increased when the UE drops the processing of the first PUSCH.

Conclusion

This contribution discussed out-of-order scheduling and out-of-order HARQ. A few proposals are as follows:

Proposal 1: Support configuring additional DMRS and DL processing capability #2 on a given serving cell.

Proposal 2: On a given cell, if a first PDSCH associated with DL processing capability #1 is followed by a second PDSCH associated with DL processing capability #2, UE may skip decoding the first PDSCH if the gap between the end of the first PDSCH and the start of the second PDSCH is shorter than PDSCH processing time N1 symbols based on the first PDSCH.

Proposal 3: On a given cell, if a first PDSCH associated with DL processing capability #1 is followed by a second PDSCH associated with DL processing capability #2, UE generates HARQ-ACK for both PDSCHs regardless whether UE decodes the PDSCH or not. UE shall generate NACK if UE skips decoding the PDSCH.

Proposal 4: On a given cell, if a first PDSCH associated with DL processing capability #1 is followed by a second PDSCH associated with DL processing capability #2, the minimum PDSCH processing time of the second PDSCH is not increased when the UE drops the processing of the first PDSCH.

Proposal 5: For overlapping PDSCHs, select one option from 1) UE may skip decoding the low priority PDSCH and UE does not buffer the low priority PDSCH data if the PDSCH is not successfully decoded and 2) UE always skips decoding the low priority PDSCH if UE is not capable of processing two PDSCHs overlap in time.

Proposal 6: For out-of-order non-overlapping PUSCH, for a first PUSCH scheduled by an earlier DCI and a second PUSCH scheduled by a latter DCI, UE transmits the second PUSCH and omits transmitting the first PUSCH if the gap between the end of the second PUSCH and the start of the first PUSCH is shorter than PUSCH preparation time N2 symbols based on the first PUSCH.

Proposal 7: For overlapping PUSCHs, UE transmits the second PUSCH and stops transmitting the first PUSCH from the start of the second PUSCH.

Proposal 8: For out-of-order PUSCH, the minimum PUSCH preparation time of the second PUSCH is not increased when the UE drops the processing of the first PUSCH.

References

  - RP-190726, New WID: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC), Huawei, HiSilicon, RAN#83

  - TR38.825, Study on NR industrial Internet of Things (IoT)

  - R1-1907925, Summary #3 of Email discussion regarding the out-of-order HARQ issue, Qualcomm, RAN1#97