3GPP TSG RAN WG1 Meeting #98                                                   	              R1- 1908608

Prague, CZ, August 26th – 30th, 2019

# Source:             CATT

Title:                  PDCCH skipping

Agenda Item:     7.2.9.4

Document for:   Discussion and Decision

Introduction

According to TR38.840 Section 5.1.5 [1], the following schemes have shown promising gains in using power saving signal/channel for triggering adaptation of UE power consumption:

PDCCH monitoring periodicity switching:

PDCCH monitoring periodicity configurations can be dynamically adapted based on explicit/implicit signaling, and works with or without C-DRX. The power saving gain over the agreed baseline in the range of 5% ~ 63.8% is observed, with latency increase in the range of 1.25% ~ 38% and overhead 0% -1.3 %. One source shows 85% power saving gain with latency 118%. The high latency is due to the not-fully-flexible PDCCH monitoring. If non-adaptive PDCCH periodicity is applied, one source shows 32% - 50% power saving gain subject to 56% - 116% latency increase. Generally, the larger the PDCCH monitoring periodicity, the larger the power saving gain but the associated latency increase would be higher.

PDCCH monitoring skipping:

Reduction of PDCCH monitoring can also be achieved with explicit/implicit information for UE to skip monitoring PDCCH. A related approach, such as power saving signal/channel triggering/enabling PDCCH monitoring or "go-to-sleep" signaling where the gNB can signal the UE to go to the DRX OFF state or skip the PDCCH monitoring, is considered. Power saving gain in the range of 9%~83% is observed, with latency increase in the range of 0.1%~75% and overhead of 0% - 1% with one source showing 124% latency increase. Generally, the gain is higher for scenarios with CDRX configuration with larger DRX inactivity timer duration and for lower traffic load, and higher latency increase is associated with higher gain.

In this contribution, the procedures of power saving signal for PDCCH skipping and switching the PDCCH monitoring periodicity are proposed.

Description

In R15, for DL data transmission, UE needs to monitor/decode the PDCCH to determine whether there is PDSCH scheduled every slot. In most cases, UE decodes PDCCH without data transmission scheduled, which would wastes power consumption. In order to avoid unnecessary power consumption for UE, the PDCCH monitoring occasion can be skipped for several slots or be periodically performed, with a periodicity of K slots, where K is configured by higher layer. The power saving signal can be introduced to indicate the behavior of UE in order to reduce the PDCCH monitoring. Two power saving schemes are proposed in this contribution, i.e. the PDCCH skipping scheme and switching the PDCCH monitoring periodicity, the details are given in sections 2.1 and 2.2, respectively.

Procedure of PDCCH monitoring skipping

UE monitors the PDCCH candidates in the CORSETs according to corresponding search space sets in every K slot(s) (e.g., K=1), where monitoring implies decoding each PDCCH candidate according to the monitored DCI formats. If DRX is configured, UE only needs to monitor the PDCCH candidates during the whole On-duration period of DRX cycle, which can save some power consumption. However, if there is no data transmission scheduled, the frequent PDCCH monitoring would also be unnecessary for UE, which can result in much power consumption. From power saving point of view, an efficient power saving scheme is proposed in this section, in which the unnecessary PDCCH monitoring occasion can be skipped.

Firstly, an indication with 1 or 2 bits would be added in the DCI format 1_1 and DCI 1_0 (for DL assignments) to indicate the cases of PDCCH monitoring skipping, named “PM skipping indication”. However, in the case where there is no DL data scheduled, UE will not receive a DL assignment and could not skip any subsequent PDCCH occasions. Thus, the aperiodic PDCCH based power saving signal, in which the PM skipping indication is included, is also introduced to assist UE to skip the PDCCH monitoring behavior.

For PM skipping indication with 1 bit, the indication means whether PDCCH monitoring skipping is chosen. ‘0’ denotes normal case, i.e. UE monitors the PDCCH in one slot, and ‘1’ denotes PDCCH monitoring skipping. The number of PDCCH occasions to skip can be pre-configured by higher layer signaling. For this case, indication ‘0’ is the default mode.

For PM skipping indication with 2 bits, the indication means the periodicity of PDCCH monitoring.  An example of PM skipping is shown in the following cases:

- ‘00’ denotes PDCCH monitoring at next PDCCH occasion

- ‘01’ denotes Power saving signal monitoring in 2 PDCCH occasions

- ‘10’ denotes Power saving signal monitoring in 4 PDCCH occasions

- ‘11’ denotes Power saving signal monitoring in 8 PDCCH occasions

For this case, indication ‘00’ is the default mode.

The power saving signal is transmitted in the time-frequency resource pre-configured by higher layer signaling.

The scheme of PDCCH monitoring skipping is shown in Figure 1 and the details of procedure are described as follows. We take the PM skipping indication with 2 bits as example.

Figure1. The procedure of PDCCH monitoring skipping

Step 1: gNB sends the PM skipping indication in DCI at each PDCCH monitoring occasion when UE is triggered by power saving signal/channel to monitor PDCCH.

If there is traffic data transmission for UE, UE can be indicated by DCI to skip the number of PDCCH monitoring occasions after the last data arrival. If there is no DL data scheduled for the UE, it does not receive any assignment for its MAC entity’s C-RNTI from PDCCH and therefore cannot obtain a PM skipping indication in DCI pointing to the next PDCCH occasion to monitor after skipping the indicated number of PDCCH occasions. In this case, gNB would indicate UE, via PDCCH based power saving signal/channel to skip a number of PDCCH monitoring occasions and point UE to monitor the next PDCCH based power saving signal/channel.

In the slot where PDCCH monitoring skipping is triggered, the PM skipping indication in DCI or PDCCH based power saving signal can be set as one of ‘01’, ‘10’ or ’11’, otherwise, the PM skipping indication is set ‘00’. The setting of the number of PDCCH occasions skipping can be associated to characteristics of the UE traffic. For the delay insensitive traffic, the number of PDCCH occasions skipping can be set to large value; otherwise, the small value or default value can be set.

Step 2a: For the case without PDCCH monitoring skipping, UE receives the PM skipping indication in DCI or PDCCH based power saving signal.

For the slot with DL data scheduled, UE receives a corresponding assignment for its MAC entity’s C-RNTI from PDCCH and can obtain PM skipping indication in DCI; otherwise (if the assignment is lost), UE will continue monitoring PDCCH in next slot.

For the slot without DL traffic data, UE monitors the PDCCH at RRC configured monitoring occasion but does not receive any assignment for its MAC entity’s C-RNTI from PDCCH. UE would be indicated to detect the PDCCH based power saving signal/channel. If the power saving signal is detected, UE could obtain the PM skipping indication; otherwise, UE will monitor the PDCCH in next slot.

For received PM skipping indication in DCI or PDCCH based power saving signal/channel, if the indication is ‘00’, UE will monitor the PDCCH in next slot. If the indication is ‘01’, ‘10’ or ’11’, UE will go to micro sleep and wake up to monitor the Power saving signal/channel in 2 slot, 4 slots or 8 slots, respectively.

Step 2b: For the case where the PDCCH monitoring skipping was triggered, UE would wake up to monitor the PDCCH occasion at the configured slot according to the PM skipping indication.

If DL traffic data arrives at UE waking up occasion, UE would acquire the new PM skipping indication in the PDCCH with the scheduling information. If there is no DL traffic data for UE, the power saving signal may be found at UE waking up occasion and the new PM skipping indication can be obtained by UE.

If UE wakes up and doesn’t detect the PDCCH or power saving signal successfully, UE will monitor the PDCCH at the RRC configured monitoring periodicity and keep the default mode unless the PM skipping indication is received to indicate skipping the PDCCH occasions.

According to the above description, we found that UE can skip monitoring a flexible number of PDCCH occasions based on PM skipping indication. It should be noted that such scheme can be also used during the DRX Active Time, to further save power consumption for UE. UE can acquire the next monitoring occasion of PDCCH or power saving signal regardless any DL data reception or not, because of the introduction of PDCCH based power saving signal.

The short DRX is also designed to reduce the PDCCH monitoring along with long DRX.   The short DRX is triggered when the drx-InactivityTimer expires or a DRX Command MAC CE is received.   The short DRX would allow the UE to have semi-statically configured shorter DRX cycle to skip the PDCCH monitoring.   Comparing the PM skipping to short DRX configuration,  the short DRX does not have the flexibility in dynamically adjusting the PDCCH monitoring period based on the traffic arrival.

Proposal 1: The UE can skip monitoring a number of PDCCH occasions based on PM skipping indication received in a DL assignment or PDCCH based power saving signal/channel.  The PM skipping could dynamic adjust the PDCCH monitoring period based on traffic arrival comparing to semi-static PDCCH monitoring reduction with short DRX.

Proposal 2: Because of the introduction of PDCCH based power saving signal/channel, UE can acquire the next monitoring occasion of PDCCH or power saving signal/channel regardless any DL data reception or not.

Procedure of switching PDCCH monitoring periodicity

As the legacy PDCCH monitoring mechanism, UE monitors the PDCCH candidates in every slot, even during DRX Active Time. The periodic PDCCH monitoring can reduce the power consumption for UE. In this section, a scheme for switching PDCCH monitoring periodicity is shown in Figure 2 and the details of the procedure are given as follows.

Figure2. Procedure of switching PDCCH monitoring periodicity

Step 1: An indicator with 1 bit or 2 bits is introduced in DCI format 1_1 to indicate the PDCCH monitoring period.

For the indicator with 1 bit, the indication means whether PDCCH monitoring period changed. ‘0’ denotes the default RRC configured PDCCH monitoring periodicity, i.e. the PDCCH monitoring period is 1 slot, and ‘1’ denotes that PDCCH monitoring period is changed to a longer periodicity. The period of PDCCH monitoring can be configured by higher layer signaling. For the indicator with 2 bits, the indication means the index of PDCCH monitoring periodicity pre-configured by higher layer signaling. For instance, the period set of PDCCH monitoring in higher layer signaling is {1 slot, 2 slots, 4 slots, 8 slots}. For the indicator with 2 bits, the following example of code point mapping is considered:

- ‘00’ denotes that the period of PDCCH monitoring is 1 slot.

- ‘01’ denotes that the period of PDCCH monitoring is 2 slots.

- ‘10’ denotes that the period of PDCCH monitoring is 4 slots.

- ‘11’ denotes that the period of PDCCH monitoring is 8 slots.

For this case, indication ‘00’ is the default mode.

Step 2a: If UE detects the PDCCH successfully, the indicator of the period of PDCCH monitoring can be obtained in the DCI.

For the indicator with 1 bit, if the indicator is set to ‘0’, UE will toggle between the default periodic PDCCH monitoring and the additional new periodicity. If UE monitors the PDCCH in every slot, UE will continue to detect the PDCCH in each slot, when the indicator is detected as ‘0’. If UE monitors the PDCCH periodically in 4 slots, UE will still perform the PDCCH monitoring in every 4 slots. If the indicator is set as ‘1’, that means the period of PDCCH monitoring would be changed to the alternate (non-default) value. UE will switch the PDCCH monitoring periodicity to that configured in higher layer signaling.

For the indicator with 2 bit, the indicator may be one of ‘00’, ‘01’, ‘10’ or ’11’, that means the period of PDCCH monitoring may/may not be changed. UE will switching the period of PDCCH monitoring to that indicated by period index according to the period set pre-configured in higher layer signaling.

Step 2b: If UE fails to detect the PDCCH, e.g. there is no DL data scheduled or PDCCH decoding failure, the indicator of switching PDCCH monitoring periodicity cannot be obtained by UE. UE will keep the original periodicity of the PDCCH monitoring unless the different periodicity indicated, i.e. if UE monitors the PDCCH in every slot, UE will continue to detect the PDCCH in each slot; if UE monitors the PDCCH periodically in 4 slots, UE will still perform the PDCCH monitoring in every 4 slots.

Based on the above proposal, the PDCCH monitoring period can be dynamic switched by the indicator in DCI, which can trigger the adaptation of power consumption for UE.

Proposal 3: The PDCCH monitoring period can be dynamic switched by the indicator in DCI, which can trigger the adaptation of power consumption for UE.

Conclusion

The proposals are summarized as follows:

Proposal 1: The UE can skip monitoring number of the PDCCH occasions based on PM skipping indication the event while it stays at DRX on duration or before inactivity timer expires. The PM skipping could dynamic adjust the PDCCH monitoring period based on traffic arrival comparing to semi-static PDCCH monitoring reduction with short DRX.

Proposal 2: Because of the introduction of PDCCH based power saving signal/channel, UE can acquire the next monitoring occasion of PDCCH or power saving signal/channel regardless any DL data reception or not.

Proposal 3: The PDCCH monitoring period can be dynamic switched by the indicator in DCI, which can trigger the adaptation of power consumption for UE.

References

  - 3GPP TR38.840 V1.0.0, “NR; Study on UE Power Saving”

  - 3GPP TS 38.133 V15.3.0,” NR; Requirements for support of radio resource management”