3GPP TSG RAN WG1 #98	      R1-1908623

Prague, CZ, August 26th – 30th, 2019

Agenda Item:		7.2.2.1.3

Source: 	Intel Corporation

Title:	UL signals and channels for NR-unlicensed

Document for: 	Discussion/Decision

# Introduction

In 3GPP TSG RAN WG1 AdHoc 1901 meeting [1], the following working assumption was made regarding the design of wideband interlace for PUSCH/PUCCH for unlicensed operation:

Working assumption:

- For a given SCS, the following interlace design is supported at least for PUSCH:

  - Same spacing (M) between consecutive PRBs in an interlace for all interlaces regardless of carrier BW, i.e., the number of PRBs per interlace is dependent on the carrier bandwidth

  - Point A is the reference for the interlace definition

- For 15 kHz SCS, M = 10 interlaces and for 30 kHz SCS, M = 5 interlaces for all bandwidths

- FFS: Interlace design for PUCCH for bandwidths greater than 20 MHz

- FFS: Whether and how partial interlace allocation is supported

Later, in 3GPP TSG RAN WG1 #97 meeting [2], the following agreements were made regarding the enhancements of PUCCH formats for unlicensed operation:

Agreement:

Support enhancement of Rel-15 PUCCH formats PF0 and PF1 as follows:

- Mapping to physical resources of one full interlace in 20 MHz.

  - FFS: Sequence type and mapping considering the following alternatives:

    - Alt-1: Repetition of the length-12 Rel-15 PF0 and PF1 sequence in each PRB of an interlace with mechanism to control PAPR/CM considering the following alternatives

      - Alt-1a: Cycling of cyclic shifts across PRBs

      - Alt-1b: Phase rotation across PRBs of an interlace where the phase rotation is can be per RE or per PRB

    - Alt-2: Mapping of different length-12 Rel-15 PF0 and PF1 sequences to the PRBs of an interlace based on different group number u (range is 0 .. 29)

    - Alt-3: Mapping of a single long sequence to the PRBs of an interlace

  - FFS: Impact due to guardbands

- Note: Decisions on the above should be based on at least performance using the agreed MCL metric and specification impact

- Note: Interlaced PF2 and 3 are not enhanced to support 1-2 bit payloads

Agreement:

For enhanced Rel-15 PF3 supporting interlaced mapping, do not replace DFT-s-OFDM with CP-OFDM

In addition, several other physical layer aspects related to the design of uplink signals and channels for unlicensed operation have been specified within the scope of revised WID on NR-based access to Unlicensed Spectrum (RP-191575) [3], in conjunction with guidance on essential functionality for NR-U (RP-191581), as agreed in RAN#84 meeting [4].

In the above context, we will discuss the followings in this contribution:

- Design of PRB based interlace waveform for physical uplink signals/channels over bandwidth more than 20 MHz.

- Potential enhancements to NR UL data and control channels design required for NR-unlicensed operation.

- Potential enhancements to NR SRS design required for NR-unlicensed operation.

# Remaining issues of interlace design

According to the revised WID scope agreed in RAN #84 meeting [3], the following needs to be specified in NR-unlicensed:

10MHz operation for 5GHz band via NR-U/NR-U CA or NR/NR-U CA without air-interface optimizations specific to 10MHz.

NOTE:

- 10MHz Pcell or SpCell is not supported in NR-U.

- The absence of WiFi channels should be guaranteed.

Since 10 MHz BW operation for 5GHz band is to be supported, interlace design for PUSCH/PUCCH is to be extended for 10 MHz BW as well. Similarly, wideband operation with carrier bandwidth >20 MHz is also supported for NR-unlicensed operation, as noted in [3]:

Wide band operation (in integer multiples of 20MHz) for DL and UL for NR-U supported with multiple serving cells, and wideband operation (in integer multiples of 20MHz) for DL and UL for NR-U supported with one serving cell with bandwidth > 20MHz with potential scheduling constraint subject to input from RAN2 and RAN4 on feasibility of operating the wideband carrier when LBT is unsuccessful in one or more LBT subbands within the wideband carrier. For all wide-band operation cases, CCA is performed in units of 20MHz (at least for 5GHz).

In the above context, our view on the aforementioned remaining issues of interlace design is illustrated in the following subsections.

## Design of PRB based interlace waveform for 10 MHz BW

As mentioned in the previous section, 10 MHz operation for 5GHz band via NR-U/NR-U CA or NR/NR-U CA has been agreed in the last RAN meeting [3]. In order to support UL transmission in 10 MHz operation, the interlace design for PUSCH/PUCCH needs to be enhanced for 10 MHz BW as well. For LTE eLAA PUSCH, 10 MHz operation is supported for 15 KHz subcarrier spacing (SCS) with 5 interlaces and 10 PRBs/interlace for NRB = 50 (total number of available PRBs). For NR, we can consider the same baseline interlace structure (5 interlaces with 10 PRBs /interlace) for 10 MHz operation with 15 KHz SCS as well for PUCCH/PUSCH. However, unlike LTE, NR is targeting to support more diverse numerology with additional SCS of 30 and 60 KHz for BW≥ 10 MHz, while for each numerology, NRB may be the same or higher than what LTE eLAA was designed for, as shown in the following table

Table I. Maximum transmission bandwidth configuration NRB for frequency range 1 FR1 (450 – 6000 MHz) [7]

| SCS (kHz) | 5MHz | 10MHz | 15MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | [160] | 216 | 270 | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | [78] | 106 | 133 | 162 | 217 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | [38] | 51 | 65 | 79 | 107 | 135 |

Since NR-U allows allocation of non-uniform interlaces for efficient resource utilization, NRB can be non-uniformly distributed among M interlaces, with N or (N+1) PRBs per interlace, depending on the value of NRB. For example, NRB=52 for 10 MHz BW/15 KHz SCS, for which M interlaces with interlace indices #0, #1, .., #4 can be allocated with 11 PRBs/interlace for interlace #0 and #1 and 10 PRBs/interlace for interlaces #2, #3 and #4. Similarly, for 10 MHz BW/30 KHz SCS with NRB=24, 2 interlaces can be assigned with 12 PRBs/interlace for each of them. Since NRB=11 for 10 MHz BW/60 KHz SCS, interlace design that could meet OCB requirement is not possible for this numerology (to meet 80% OCB, at least 9 out of 11 PRBs have to be assigned to one interlace, which means more than one interlace is not possible to configure within 11 PRBs).

Proposal 1

For 10 MHz carrier bandwidth, support the following PRB based interlace design:

- 15 KHz SCS: M = 5 interlaces with N = 10 or 11 PRBs / interlace

- 30 KHz SCS: M = 2 interlaces with N = 12 PRBs / interlace

## Design of PRB based interlace waveform for wideband operation

As agreed in the past [1], PRB based interlace transmission for PUCCH/PUSCH over 20 MHz carrier bandwidth would be supported with N = 10/11 PRBs per interlace, leading to M = 10 interlaces at 15 KHz SCS and M = 5 interlaces at 30 KHz SCS. Further, it was agreed as a working assumption that, for carrier BW > 20 MHz, the interlace design at least for PUSCH is supported such that the same spacing (M) between consecutive PRBs in an interlace is maintained regardless of carrier BW, i.e. the number of PRBs /interlace (N) scales with bandwidth. Although keeping M the same with increasing bandwidth seems a straightforward way to scale the interlace design from 20 MHz to wider bandwidth, there are several drawbacks of this design from the perspective of resource utilization efficiency and UE multiplexing capacity, as noted below:

- By keeping M fixed, N increases proportionally with BW scaling, which leads to large number of PRBs allocated per interlace as BW increases. Allocation of one full interlace to an UE at wider bandwidth may not be efficient from resource utilization perspective, especially when the payload size to be carried using the interlace is relatively low. Also, from uplink transmission power perspective, interlaces with N greater than a threshold value, NTH (e.g. NTH = 19 PRBs/interlace for EIRP limit of -23 dBm and PSD limit of 10 dBm/1 MHz for frequency band 5150-5350 MHz as per ETSI regulation [5]) is not beneficial, since it cannot offer any additional power boost over N = NTH due to EIRP and PSD limitations.

- From the perspective of FDM, UE multiplexing capacity is severely restricted (to M) for wideband operation if one full wideband interlace is assigned to one UE.

One possible way to improve resource utilization and UE multiplexing capacity for wideband interlace based transmission is to have the scheduling flexibility at gNB to allocate partial interlace to an UE, in addition to the provision of allocating full interlace per UE. Depending on whether gNB would schedule an UE’s transmission over the entire wideband or over a sub-band (e.g. LBT sub-band of 20 MHz) within the wideband, partial interlace allocation schemes may be different. In the following subsection we will discuss partial interlace allocation for both of these scheduling scenarios.

### Partial interlace spanning over a wideband carrier

If multiple UEs are to be scheduled within a wideband carrier for interlace based transmission and each of the UEs are scheduled over the wideband carrier BW, a set of PRBs from each interlace can be assigned to an UE such that the number of PRBs assigned per UE remains nearly the same as the number of PRBs/interlace (N) for 20 MHz and at the same time, the partial interlace can meet the 80% OCB requirement (if mandated by regulation). By assigning partial interlace in this way, the UE multiplexing capacity (FDM) scales proportionally with BW, instead of remaining fixed as M when one full wideband interlace is allocated to an UE.

To illustrate further, when BW of 20 MHz is scaled by a factor n, n UEs can be FDM’d within one wideband interlace and the partial interlace allocation can be done in the following way:

If the PRBs allocated to one wideband interlace are indexed C(0), C(1), C(2), …, C(N), n UEs multiplexed in FDM manner within this one interlace would each be assigned a subset of PRBs from the set {C(0), C(1), …, C(N)} as:

UE(1): {C(0), C(n), C(2n), …}; UE(2): {C(1), C(n+1), C(2n+1), …}; …; UE(n) = {C(n-1), C(2n-1), C(3n-1), …}

Table 1 illustrates an example of partial interlace allocation for 40 MHz BW (i.e. n = 2) and 15 KHz SCS, for which the total number of available PRBs (NRB) is 216. Each row in the table consists of PRB indices corresponding to one interlace. Within one interlace, n=2  UEs can be FDM’d in this case, where the “black” entries in the j-th row of Table 1 denotes the PRBs assigned to UE(2j-1) and “red” entries in the row denotes the PRBs assigned to UE(2j). Therefore, corresponding to M=10, in total 2*10 = 20 UEs can be FDM’d within 40 MHz, 2 UEs being on each interlace. Note that, each of these partial interlaces satisfy the OCB requirement (80% of 40 MHz).

Table 1: Wideband partial interlace for 40 MHz BW and 15 KHz SCS (n = 2 UEs per interlace)

| Int. no. | Multiplexed UEs | PRB index over 40 MHz BW (for 15 KHz SCS) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | C0 | C1 | C2 | C3 | C4 | … | C19 | C20 | C21 |
| 1 | UE {1,2} | 1 | 11 | 21 | 31 | 41 | … | 191 | 201 | 211 |
| 2 | UE {3,4} | 2 | 12 | 22 | 32 | 42 | … | 192 | 202 | 212 |
| 3 | UE {5,6} | 3 | 13 | 23 | 33 | 43 | … | 193 | 203 | 213 |
| 4 | UE {7,8} | 4 | 14 | 24 | 32 | 44 | … | 194 | 204 | 214 |
| 5 | UE {9,10} | 5 | 15 | 25 | 35 | 45 | … | 195 | 205 | 215 |
| 6 | UE {11,12} | 6 | 16 | 26 | 36 | 46 | … | 196 | 206 | 216 |
| 7 | UE {13,14} | 7 | 17 | 27 | 37 | 47 | … | 197 | 207 | - |
| 8 | UE {15,16} | 8 | 18 | 28 | 38 | 48 | … | 198 | 208 | - |
| 9 | UE {17,18} | 9 | 19 | 29 | 39 | 49 | … | 199 | 209 | - |
| 10 | UE {19,20} | 10 | 20 | 30 | 40 | 50 | … | 200 | 210 | - |

As a second example, Table 2 illustrates partial interlace allocation for 60 MHz BW (i.e. n = 3) and 30 KHz SCS, for which the total number of available PRBs (NRB) is 162. In this case, within one interlace, n=3  UEs can be FDM’d, where the “black” entries in the j-th row of Table 2 denotes the PRBs assigned to UE(2j-1), “red” entries to UE(2j) and “green” entries to UE(2j+1) respectively.

Table 2: Wideband partial interlace for 60 MHz BW and 30 KHz SCS (n = 3 UEs per interlace)

| Int. no. | Multiplexed UEs | PRB index over 60 MHz BW (for 30 KHz SCS) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | C0 | C1 | C2 | C3 | C4 | … | C31 | C32 | C33 |
| 1 | UE{1,2,3} | 1 | 6 | 11 | 16 | 21 | … | 151 | 156 | 161 |
| 2 | UE{4,5,6} | 2 | 7 | 12 | 17 | 22 | … | 152 | 157 | 162 |
| 3 | UE{7,8,9} | 3 | 8 | 13 | 18 | 23 | … | 153 | 158 | - |
| 4 | UE{10,11,12} | 4 | 9 | 14 | 19 | 24 | … | 154 | 159 | - |
| 5 | UE{13,14,15} | 5 | 10 | 15 | 20 | 25 | … | 155 | 160 | - |

### Partial interlace spanning over 20 MHz LBT sub-band within a wideband carrier

While partial interlace allocation scheme illustrated in the last subsection is efficient from resource utilization perspective within a wideband carrier, there can be scenarios where not all UEs to be scheduled within a wideband carrier are capable of supporting >20 MHz BW. In order to give gNB sufficient scheduling flexibility, an alternative partial interlace allocation scheme can be adopted where each of the partial interlaces would span over a 20 MHz LBT sub-band within a wideband carrier. If an UE is scheduled with 20 MHz BW, the partial interlace needs to satisfy OCB over 20 MHz. Similar to the previous partial interlace design, n UEs can be multiplexed within one wideband interlace over n*20 MHz, where each UE would be assigned 20 MHz BW for transmission, over a partial interlace.

Table 3 illustrates this partial interlace assignment scheme for 40 MHz BW (n = 2) and 15 KHz SCS (NRB = 216), the same numerology as the example in Table 1. Within one interlace, n=2 UEs can be FDM’d, each over a 20 MHz sub-band. In Table 3, the “black” entries in the j-th row denotes the PRBs assigned to UE(2j-1) and “red” entries of the corresponding row denotes the PRBs assigned to UE(2j). Note that, here few PRBs near the band edges of each 20 MHz sub-band may be left unused due to guard band requirement (shown as grey PRB indices in Table 3)

Table 3: Sub-band partial interlace for 40 MHz BW and 15 KHz SCS (n = 2 UEs per interlace)

| Int. no. | Multiplexed UEs | PRB index over 40 MHz BW (for 15 KHz SCS) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | C0 | C1 | … | C10 | C11 | C12 | … | C20 | C21 |
| 1 | UE {1,2} | 1 | 11 | … | 101 | 111 | 121 | … | 201 | 211 |
| 2 | UE {3,4} | 2 | 12 | … | 102 | 112 | 122 | … | 202 | 212 |
| 3 | UE {5,6} | 3 | 13 | … | 103 | 113 | 123 | … | 203 | 213 |
| 4 | UE {7,8} | 4 | 14 | … | 104 | 114 | 124 | … | 204 | 214 |
| 5 | UE {9,10} | 5 | 15 | … | 105 | 115 | 125 | … | 205 | 215 |
| 6 | UE {11,12} | 6 | 16 | … | 106 | 116 | 126 | … | 206 | 216 |
| 7 | UE {13,14} | 7 | 17 | … | 107 | 117 | 127 | … | 207 | - |
| 8 | UE {15,16} | 8 | 18 | … | 108 | 118 | 128 | … | 208 | - |
| 9 | UE {17,18} | 9 | 19 | … | 109 | 119 | 129 | … | 209 | - |
| 10 | UE {19,20} | 10 | 20 | … | 110 | 120 | 130 | … | 210 | - |

Similarly, corresponding to Table 2, Table 4 illustrates the alternative sub-band partial interlace design for the same numerology, i.e. 60 MHz BW and 30 KHz SCS, where corresponding to j-th row, 3 multiplexed UEs are assigned the subsets of PRBs shown as “black”, “red” and “green” entries in the corresponding row of Table 4. Gain, few PRBs will remain unassigned in this case to meet guard band requirements of 20 MHz (shown as grey PRB indices in Table 4).

Table 4: Sub-band partial interlace for 60 MHz BW and 30 KHz SCS (n = 3 UEs per interlace)

| Int. no. | Multiplexed UEs | PRB index over 60 MHz BW (for 30 KHz SCS) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | C0 | C1 | ... | C10 | C11 | C12 | … | C21 | C22 | C23 | … | C33 |
| 1 | UE{1,2,3} | 1 | 6 | … | 51 | 56 | 61 | … | 106 | 111 | 116 | … | 161 |
| 2 | UE{4,5,6} | 2 | 7 | … | 52 | 57 | 62 | … | 107 | 112 | 117 | … | 162 |
| 3 | UE{7,8,9} | 3 | 8 | … | 53 | 58 | 63 | … | 108 | 113 | 118 | … | - |
| 4 | UE{10,11,12} | 4 | 9 | … | 54 | 59 | 64 | … | 109 | 114 | 119 | … | - |
| 5 | UE{13,14,15} | 5 | 10 | … | 55 | 60 | 65 | … | 110 | 115 | 120 | … | - |

Proposal 2

For carrier bandwidth > 20 MHz in NR-unlicensed band, support scheduling flexibility for interlace based transmission by enabling partial interlace based resource allocation.

- A partial interlace can either span over the carrier bandwidth or over a 20 MHz sub-band within the carrier bandwidth.

# Potential enhancements of NR UL data and control channels

In Rel-15 NR framework, uplink physical channels are designed based on CP-OFDM or DFT-s-OFDM waveforms and can span 1~14 symbol(s) in the time domain and ≥1 PRB(s) across the frequency domain (depending on which of the uplink physical channels, viz.  PUSCH or PUCCH is used). When a physical uplink channel in Rel-15 NR occupies more than 1 PRBs, the PRBs may be assigned in a contiguous manner, in contrast to the distributed resource allocation scheme described in the previous section.

As one of the primary objectives of NR-unlicensed WI phase for Rel-16 is to study the feasibility of inheriting NR physical channel frameworks as baseline, by avoiding any unnecessary divergence from the decisions made in NR WI phase for Rel-15, it is imperative to look into NR physical channels and identify the essential enhancements and modifications that need to be incorporated in order to enable the unlicensed operation.

Based on the above discussion, potential enhancements of various NR uplink physical channels are described in the following subsections.

## Enhancements of NR PUCCH formats to enable unlicensed operation

NR framework defines 5 PUCCH formats, which can be categorized into two classes based on their frequency domain span:

- Single-PRB transmission: three PUCCH formats, viz. formats 0/1/4 use single-PRB transmission.

- Multi-PRB transmission: two PUCCH formats, viz. formats 2/3 use multi-PRB transmission.

Potential enhancement scopes of each of these two categories of PUCCH formats are explored in the following subsections.

### NR PUCCH formats 0/1

NR PUCCH formats 0/1 allow UE multiplexing in the code domain (using cyclic shift and/or OCC) and can span across 1 PRB in frequency domain. While format 0 can span 1~2 OFDM symbol(s) within a slot, format 1 has a more dynamic timespan, with a range of 4~14 symbols within a slot. Formats 0/1 are intended to carry small payload size, e.g. 1~2 UCI bit(s). None of these formats, however, meet the OCB requirement of unlicensed domain, owing to their bandwidth limitation of less than 2 MHz. Moreover, due to regulatory PSD limitation of 10 dBm/1MHz, these formats in Rel-15 NR forms are severely restricted in transmit power due to 1 PRB frequency resource allocation, which is always less than 1 MHz in FR1.

In the above context, we made an agreement in the last RAN1 meeting [2] regarding enhancement of Rel-15 PUCCH formats 0/1 from single-PRB to multi-PRB format as follows:

Agreement:

Support enhancement of Rel-15 PUCCH formats PF0 and PF1 as follows:

- Mapping to physical resources of one full interlace in 20 MHz.

  - FFS: Sequence type and mapping considering the following alternatives:

    - Alt-1: Repetition of the length-12 Rel-15 PF0 and PF1 sequence in each PRB of an interlace with mechanism to control PAPR/CM considering the following alternatives

      - Alt-1a: Cycling of cyclic shifts across PRBs

      - Alt-1b: Phase rotation across PRBs of an interlace where the phase rotation is can be per RE or per PRB

    - Alt-2: Mapping of different length-12 Rel-15 PF0 and PF1 sequences to the PRBs of an interlace based on different group number u (range is 0 .. 29)

    - Alt-3: Mapping of a single long sequence to the PRBs of an interlace

  - FFS: Impact due to guardbands

- Note: Decisions on the above should be based on at least performance using the agreed MCL metric and specification impact

- Note: Interlaced PF2 and 3 are not enhanced to support 1-2 bit payloads

As per the above agreement, there are two main approaches for sequence type and mapping on interlaced PRBs for PF0/1: either one single long sequence, or repetitions of Rel-15 PF0/1 sequences of length-12. Since the correlation property of a long sequence may get degraded with distributed allocation across frequency (as in the case for interlaced PRBs), the performance of non-coherent detection based PF0 would be impacted especially under larger channel delay spread, compared to repeated short sequence mapped on non-contiguous PRBs, while each sequence is confined within one PRB. Therefore, from performance perspective, Alts-1/2 are more robust to channel delay spread and its impact to sequence correlation property than Alt-3. Alts-1 and 2 differ in the mechanisms applied for controlling PAPR/CM that’s essential for repeated transmission of sequence (which inherently exacerbates PAPR/CM). From specification impact perspective, Alt-1a is the most straightforward approach to adopt among the two alternatives.

Proposal 3

Support enhancement of Rel-15 PUCCH formats PF0 and PF1 by repetition of the length-12 Rel-15 PF0 and PF1 sequence in each PRB of an interlace, with cycling of cyclic shifts across PRBs, i.e. Alt-1a.

### NR PUCCH formats 2/3

NR PUCCH formats 2/3 can span up to 16 PRBs across frequency domain. While format 2 can occupy 1~2 OFDM symbol(s) within a slot, format 3 has a more dynamic timespan, similar to formats 1/4 (i.e. 4~14 symbols within a slot). Both formats 2 and 3 are intended to carry large UCI payload size (typically > 2 bits). In the Rel-15 NR specification for PUCCH formats 2/3, a maximum number of PRBs (which can be up to 16) is configured via RRC to the UE while PUCCH resources are configured by gNB, but there is no notion of minimum number of PRBs that PUCCH formats 2/3 can occupy. Depending on the UCI payload size and configured maximum code rate, number of PRBs used for PUCCH format 2/3 transmission can be as small as 1 PRB. Since unlicensed operation with temporal allowance of not meeting regular OCB (i.e. 80%-100% of nominal channel bandwidth) requires at least 2 MHz transmission bandwidth, resource configurations for PUCCH formats 2/3 in unlicensed operation require an additional configuration of minimum number of PRBs so as to ensure OCB to be minimum of 2 MHz, when temporal allowance can be used by regulation.  For 15/30/60 KHz SCS in FR1, the minimum number of PRBs required to meet 2 MHz OCB criteria is 12/6/3.

Proposal 4

For NR PUCCH formats 2/3, the minimum number of PRBs is included in the specification to enable unlicensed operation, when temporal allowance of 2 MHz OCB is allowed by regulation.

- For 15/30/60 KHz subcarrier spacing in FR1, minimum number of PRBs is 12/6/3.

While the resource allocation granularity is in units of PRB for Rel-15 PUCCH format 2/3, the interlace based enhanced version of these formats would have coarser resource allocation unit, in terms of interlace (i.e. at least one full interlace or 10 PRBs at 20 MHz). From the resource utilization perspective, it may not be efficient to allocate only one UE on one interlace, especially when the payload size is low (e.g. 1~2 bits).

In this context, the following subsections would look into potential scopes of enhancing UE multiplexing capability of interlace based PUCCH formats 2 and 3:

#### Multiplexing UEs on one interlace for enhanced PUCCH format 2

Unlike the TDM of DMRS and UCI in PUCCH format 3, the underlying structure of PUCCH format 2 is based on FDM of DMRS and UCI symbols, with 1/3 DMRS overhead (i.e. 8 UCI and 4 DMRS symbols on each PRB). For CP-OFDM based enhanced PUCCH format 2 (interlace based frequency domain resource allocation), UE multiplexing can be supported by applying OCC on DMRS and UCI symbols mapped on each PRB. UE multiplexing capability of maximum 4 (limited by min{#UCI symbols, #DMRS symbols}on each PRB) can be obtained on one interlace. In addition to frequency domain OCC, length-2 time domain OCC can be applied to further increase UE multiplexing capacity for 2-symbol PUCCH format 2.

#### Multiplexing UEs on one interlace for enhanced PUCCH format 3

Rel-15 NR PUCCH formats 3 and 4 have the same time domain structure (in terms of TDM pattern of DMRS and UCI symbols), while in frequency domain, the span of  PUCCH format 4 is restricted to be one PRB unlike PUCCH format 3 which can occupy more than one PRBs.  The key difference between these two formats is in terms of UE multiplexing capability. While NR PUCCH format 3 doesn’t allow UE multiplexing on the same PRB, PUCCH format 4 can multiplex up to 4 UEs within one PRB using pre-DFT blockwise spread OCC with SF = {2,4}. One unique property of pre-DFT blockwise spread OCC is its ability to split multiplexed UEs’ transmissions on different orthogonal combs (e.g. for SF = 2 the two multiplexed UEs use the odd and even combs respectively within a PRB), which in turn, provides frequency domain orthogonality that is resilient to adverse propagation conditions (delay spread, deep fade etc.).

Since PUCCH formats 3 and 4 are both based on DFT-s-OFDM, one straightforward way to enable UE multiplexing for interlace based PUCCH format 3 is to use similar pre-DFT OCC (as used in PUCCH format 4), but spread it blockwise across the entire interlace.  By extending pre-DFT OCC block-wise to all PRBs on one interlace, UCI symbols of each multiplexed UEs are mapped on orthogonal combs and hence, irrespective of channel delay spread the orthogonality between transmissions of different UEs retains.

Figure 1. Illustration of pre-DFT blockwise spread OCC of length-120 applied on PRB based interlace structure of PUCCH format 3 (10 PRBs/interlace) to multiplex 2 UEs on odd and even combs at SCS = 30 KHz

Figure 1 illustrates an example of pre-DFT blockwise spread OCC applied on UCI symbols mapped on one interlace with 10 PRBs/interlace and spreading factor = 2. For UE0, a block of data {d0, …, d59} is repeated in two blocks on which, OCC [1 1 … 1 | 1 1 … 1] is applied before DFT operation. The resulting DFT coded symbols are mapped on even combs (i.e. on subcarriers with indices 0, 2, 4, .., 118) on 10 PRBs or 120 REs. For UE1, a block of data {s0, …, s59} is repeated in two blocks on which, OCC [1 1 … 1 |-1 -1 …-1] is applied before DFT operation. The resulting DFT coded symbols are mapped on odd combs (i.e. subcarriers with indices 1,3,5,..,119) on the same 10 PRBs or 120 REs of the interlace.

On top of pre-DFT OCC, time domain OCC can also be applied to further enhance UE multiplexing capacity, although to what extent the additional multiplexing capacity boosting would be beneficial  may depend on other factors like the payload size, configured code rate, PUCCH duration in time etc.

Similar to Rel-15 NR-PUCCH format 4, cyclic shifts can be applied to DMRS symbols along with pre-DFT OCC on UCI symbols to assign orthogonal DMRS ports for different UEs in one interlace. The cyclic shifts for DMRS can be derived from the OCC index used on UCI symbols. If NCS denotes the number of available cyclic shifts for DMRS sequence and NOCC denotes the number of available OCC indices (depends on the spreading factor), then the cyclic shift index ICS of DMRS can be related to the OCC index IOCC applied on UCI symbols in a similar way as in Rel-15 NR PUCCH format4, i.e.  ICS=NCSNOCCIOCC.

Proposal 5

Support UE multiplexing on one interlace for enhanced PUCCH formats 2/3 for NR-unlicensed operation.

- Enable UE multiplexing for PUCCH format 2 by applying frequency domain OCC up to length-4 OCC on DMRS and UCI symbols.

- Enable UE multiplexing for PUCCH format 3 by applying pre-DFT blockwise spread OCC (spread across the entire transmission bandwidth) on UCI symbols and cyclic shifts on DMRS.

- Time domain OCC on UCI and DMRS symbols can be applied to PUCCH format 2 (2-symbol) and PUCCH format 3 to further enhance UE multiplexing capacity, on top of frequency domain OCC.

## Enhancements to PUCCH resource configuration during initial access

In NR, PUCCH resource sets before dedicated PUCCH resource configuration used for transmitting 1-bit HARQ-ACK in response to Msg. 4 transmission during initial access are given as follows:

- A PUCCH resource set before dedicated PUCCH resource configuration is provided by higher layer parameter pucch-ResourceCommon in system information block 1 (SIB1) through a 4-bit remaining minimum system information (RMSI) index.

- 4-bit RMSI indicates an entry into a 16-row table (Table 9.2.1-1 of TS 38.213).

- Each row in the table configures a set of 16 cell-specific PUCCH resources.

- A PUCCH resource is indicated by 5 parameters: a PUCCH format, a starting symbol, a duration, a PRB offset RBBWPoffset and a set of initial cyclic shift (CS) indices.

- Supported PUCCH durations and starting symbols (always aligned with slot boundary):

- Symbol duration 2/4/10/14  starting symbol index #12/#10/#4/#0.

- Supported PUCCH formats: only PUCCH format 0 and PUCCH format 1 (single PRB).

- Supported payload size: 1-bit only.

In NR-unlicensed, transmission of all signals/channels must abide by the regulatory requirements related to occupied channel bandwidth (OCB). Since a single PRB transmission in FR1 cannot meet this requirement, PUCCH resource sets are to be enhanced to support multi-PRB transmission in order to fulfil OCB related regulatory criteria. Since interlace based enhanced PUCCH formats 0/1 are already agreed to be supported in NR-unlicensed to carry 1~2 UCI bit(s), pucch-ResourceCommon configuration has to be enhanced as well in order to support interlace based PUCCH formats 0/1.

Several aspects of pucch-ResourceCommon configuration need further consideration in the context of interlace based transmission. For example, frequency hopping is always enabled for PUCCH resource sets before dedicated PUCCH resource configuration in Rel-15, which may not be essential for interlace based PUCCH, since frequency diversity gain is already achieved from distributed allocation of interlace PRBs and frequency hopping cannot possibly bring any additional gain. Similarly, the values of PRB offset as well as the formulae for PRB indices in the 1st and 2nd hop (for always enabled frequency hopping) may require repurposing/ modification depending on how interlace is being incorporated into the resource set configuration. Currently, in Rel-15, the combination of PUCCH starting symbols and durations are chosen in the pucch-ResourceCommon table such that the configured PUCCH is always aligned with slot boundary. In order to take into consideration the channel access and channel occupancy time (COT) sharing aspects for unlicensed operation, more flexibility in starting symbol configuration within a slot may be beneficial, which may not necessarily maintain the slot boundary alignment as is designed for Rel-15.

Proposal 6

To meet OCB requirements for NR-unlicensed operation, enhance PUCCH resource sets before dedicated PUCCH resource configuration (i.e. during initial access) to support interlace based transmission using enhanced PUCCH formats 0 and 1.

## Enhancements of NR-PUSCH for unlicensed operation

NR supports two different mapping types for PUSCH in frequency domain, viz. type-A and type-B. Depending on the mapping type, the starting symbol location/duration of PUSCH can be very flexible in NR. A PUSCH can start anywhere from OS#0 to OS#13 in a slot and can have a dynamic duration of 1~14 symbols. In TR38.889 [6], the following options were identified as potential candidates at least for the first PUSCH(s) transmitted in the UL transmission burst:

- Option 1: PUSCH(s) as in Rel-15 NR

- Option 2: Multiple starting positions in one or multiple slot(s) are allowed for PUSCH(s) scheduled by a single UL grant (i.e., not a configured grant) and one of the multiple PUSCH starting positions can be decided depending on LBT outcome.

PUSCH transmission within the UL transmission burst in NR-unlicensed should consider Rel-15 NR PUSCH (i.e. with similar scheduling flexibility as Rel-15 NR) as the baseline, since NR-PUSCH provides sufficient scheduling flexibility in terms of possible starting symbol locations within a slot that would be beneficial especially when PUSCH is scheduled within a gNB shared COT (where the gap between an UL burst and the preceding DL transmission is desired to be minimized in order to ensure easier channel access). On top of option 1, option 2 may potentially provide additional flexibility in terms of multiple PUSCH starting position options in a single UL grant, so that if UE fails LBT in the first scheduled starting location, it can perform CCA again in other symbol locations(s) based on the granted multiple starting position configurations and choose one based on its LBT outcome. While option 2 may potentially provide more transmission opportunities, it would come with an added cost of UE implementation complexity. Since the ending position of PUSCH transmission within an UL burst has been agreed to be fixed [6], a deferred PUSCH transmission without changing the granted TBS would result in either puncturing or rate matching of the PUSCH data. This, in turn, might result in degraded decoding performance at the receiver (when PUSCH is punctured) or may not meet UE processing time requirement (for rate matching). In addition, if CCA fails in a certain point, it is likely that the channel will be busy for comparatively long time duration since the device which acquires the channel will keep it for up to Max COT length. Therefore, the chance that CCA becomes successful in a very short time, e.g., a few OFDM symbol duration, is very low. Also, for PUSCH transmission within a gNB shred COT, the LBT congestion can be alleviated by proper scheduling (so as to reduce the gap between preceding DL and UL data burst transmissions) and added PUSCH starting position flexibility won’t be necessary.

Proposal 7

Support scheduling flexibility of PUSCH as in Rel-15 NR for NR-unlicensed operation, i.e. option 1.

NR supports two different time domain resource allocation types for PUSCH, viz. UL resource allocation type 0 (bitmap based) and UL resource allocation type 1 (RIV based). In LAA, RIV based time domain resource allocation was used for PUSCH for 20 MHz BW, whereas bitmap based approach was used for 10 MHz BW. In NR, even though the targeted numerologies (BW-SCS combinations) are much more diverse than LAA, the scheduling flexibility obtained from RIV based approach may be sufficient and additional flexibility achievable by bitmap based approach may not be necessary. In addition, RIV based approach needs less number of bits (for example, 6 bits for 20 MHz BW and 15 KHz SCS) than bitmap based approach (for example, 10 bits for 20 MHz BW and 15 KHz SCS. Also, Rel-15 Msg3 PUSCH transmission only supports type 1 (RIV based) PUSCH resource allocation, which implies less specification impact if type 1 resource allocation scheme is adopted for interlace based PUSCH resource frequency domain resource allocation.

Proposal 8

Support enhancements to Rel-15 type 1 (RIV based) PUSCH frequency domain resource allocation for interlace based PUSCH in NR-U.

# Enhancements of NR SRS to enable unlicensed operation

Rel-15 NR supports both periodic and aperiodic SRS transmissions. However, due to the possibility of LBT contention, periodic transmission is not preferred in unlicensed operation. Similar to eLAA, we propose to support only aperiodic SRS transmission, triggered by DCI in NR-unlicensed operation, to increase the opportunity of SRS transmission.

The baseline of SRS waveform for NR-unlicensed operation can be based on Rel-15 NR SRS, i.e. comb structure with comb-levels of 2 and 4. This is because regulatory requirements (in terms of OCB and PSD) can be met by proper configuration of the Rel-15 NR SRS waveform. Similar to Rel-15 NR, SRS can be of 1, 2 or 4 symbol duration in time, while time domain SRS resource configuration can be further enhanced to reduce LBT contention and increase SRS transmission opportunity, in case SRS transmission is blocked due to LBT. For example, when aperiodic SRS is triggered via DCI, gNB can configure multiple SRS time domain resource sets, each with different starting symbol locations, so that if the UE can’t transmit SRS due to LBT failure, UE can re-attempt to access the channel within the same subframe in a later symbol. To reduce the blind decoding burden at gNB side, gNB can configure UE with maximum 2~3 time domain SRS resources, each having a different starting symbol and same/different durations, so that if UE’s attempt of accessing channel fails at the earliest configured starting symbol, UE can perform another LBT at a later symbol within the same subframe and if succeeds, can still transmit SRS in the same subframe it was configured for aperiodic SRS transmission, triggered by DCI.

In Rel-15 NR, the minimum sounding bandwidth for SRS transmission can be 4 PRBs and for other sounding bandwidth configurations, it would always be a multiple of 4 PRBs. To enable unlicensed operation, however, the minimum sounding bandwidth needs to be such that it meets the OCB requirement. For example, assuming temporal 2 MHz OCB allowance, the minimum sounding bandwidth should be 12/8/4 PRBs (multiples of 4) for SCS of 15/30/60 KHz.

Proposal 9

Keep Rel-15 NR-SRS waveform (with comb level 2/4) in NR-unlicensed operation.

Proposal 10

Support only aperiodic SRS triggered by DCI to combat LBT contention in NR-unlicensed operation.

Proposal 11

Increase the minimum sounding bandwidth of SRS in the specification to enable unlicensed operation, when temporal allowance of 2 MHz OCB is allowed by regulation.

- For 15/30/60 KHz subcarrier spacing in FR1, minimum sounding bandwidth (in multiples of 4PRBs) is 12/8/4 PRBs.

Proposal 12

To increase SRS transmission opportunity in NR-unlicensed, support multiple starting symbol configurations within time domain resource configured by gNB for aperiodic SRS transmission.

# Conclusion

In this contribution, we have discussed the design of PRB based interlace waveform for physical uplink signals/channels over carrier bandwidth of 10 MHz and > 20 MHz, along with the potential enhancements to NR uplink data/control channels design and NR SRS design to enable NR-unlicensed operation. Based on our discussion, we make the following observations and proposals.

Proposal 1

For 10 MHz carrier bandwidth, support the following PRB based interlace design:

- 15 KHz SCS: M = 5 interlaces with N = 10 or 11 PRBs / interlace

- 30 KHz SCS: M = 2 interlaces with N = 12 PRBs / interlace

Proposal 2

For carrier bandwidth > 20 MHz in NR-unlicensed band, support scheduling flexibility for interlace based transmission by enabling partial interlace based resource allocation.

- A partial interlace can either span over the carrier bandwidth or over a 20 MHz sub-band within the carrier bandwidth.

Proposal 3

Support enhancement of Rel-15 PUCCH formats PF0 and PF1 by repetition of the length-12 Rel-15 PF0 and PF1 sequence in each PRB of an interlace, with cycling of cyclic shifts across PRBs, i.e. Alt-1a.

Proposal 4

For NR PUCCH formats 2/3, the minimum number of PRBs is included in the specification to enable unlicensed operation, when temporal allowance of 2 MHz OCB is allowed by regulation.

- For 15/30/60 KHz subcarrier spacing in FR1, minimum number of PRBs is 12/6/3.

Proposal 5

Support UE multiplexing on one interlace for enhanced PUCCH formats 2/3 for NR-unlicensed operation.

- Enable UE multiplexing for PUCCH format 2 by applying frequency domain OCC up to length-4 OCC on DMRS and UCI symbols.

- Enable UE multiplexing for PUCCH format 3 by applying pre-DFT blockwise spread OCC (spread across the entire transmission bandwidth) on UCI symbols and cyclic shifts on DMRS.

- Time domain OCC on UCI and DMRS symbols can be applied to PUCCH format 2 (2-symbol) and PUCCH format 3 to further enhance UE multiplexing capacity, on top of frequency domain OCC.

Proposal 6

To meet OCB requirements for NR-unlicensed operation, enhance PUCCH resource sets before dedicated PUCCH resource configuration (i.e. during initial access) to support interlace based transmission using enhanced PUCCH formats 0 and 1.

Proposal 7

Support scheduling flexibility of PUSCH as in Rel-15 NR for NR-unlicensed operation, i.e. option 1.

Proposal 8

Support enhancements to Rel-15 type 1 (RIV based) PUSCH frequency domain resource allocation for interlace based PUSCH in NR-U.

Proposal 9

Keep Rel-15 NR-SRS waveform (with comb level 2/4) in NR-unlicensed operation.

Proposal 10

Support only aperiodic SRS triggered by DCI to combat LBT contention in NR-unlicensed operation.

Proposal 11

Increase the minimum sounding bandwidth of SRS in the specification to enable unlicensed operation, when temporal allowance of 2 MHz OCB is allowed by regulation.

- For 15/30/60 KHz subcarrier spacing in FR1, minimum sounding bandwidth (in multiples of 4PRBs) is 12/8/4 PRBs.

Proposal 12

To increase SRS transmission opportunity in NR-unlicensed, support multiple starting symbol configurations within time domain resource configured by gNB for aperiodic SRS transmission.

# References

- Chairman’s note, 3GPP TSG RAN WG1 AdHoc 1901 meeting, Taipei, Taiwan, Jan. 2019

- Chairman’s note, 3GPP TSG RAN WG1 #97 meeting, Reno, USA, May 2019

- RP-191575, “Revised WID on NR-based Access to Unlicensed Spectrum”, 3GPP TSG RAN Meeting #84, Newport Beach, USA, Jun. 2019

- RP-191581, “Guidance on essential functionality for NR-U”, 3GPP TSG RAN Meeting #84, Newport Beach, USA, Jun. 2019

- ETSI EN 301.893, 5 GHz RAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU V2.1.1, 2017-05

- 3GPP TR 38.889, “Study on NR-based Access to Unlicensed Spectrum (Rel-16),” V 16.0.0, Nov. 2018

- 3GPP TS 38.101-1, User Equipment (UE) radio transmission and reception Part 1: Range 1 Standalone, V1.0.0, 2017-12.